The following is a connect sequence to WA3DSP-5 (Netrom) on 221.01,

connect to N3ET-1 (KPC-4) on 221.01, cross connect to KB3UD-1 (Netrom)

on 145.01, connect to KB3UD on 145.01.	Note the SX to do the cross

connect.  This would be SC if staying on the same frequency.  Also

note the R>###LINK MADE.  The > is the KPC-4 prompt that is not

followed by a carriage return.	You have to pick it up AFTER you

send the SX.  It will be in the input buffer.  For the KPC-2 the

prompt character is a ? instead of a > but the missing carriage

return means it must be handled the same way.  You may have to

modify the case of some of these statements.

CC WA3DSP-5

SC N3ET-1

RCONNECTED TO

R###CONNECTED TO NODE

RENTER COMMAND

SX KB3UD-1

R>###LINK MADE

SC KB3UD

RCONNECTED TO

FB0023C KB3UD

@C:\PKT\RLI\KB3UD.FWD

*** EOF

The following is a connect to WA3DSP-5 (Netrom) on 221.01, connect

to W3GJC-1 (KPC-2) on 221.01, connect to WB2GWD on 221.01.  The line

after R###CONNECTED TO NODE on a KPC-2 is user configurable.  Pick

a unique string from whatever is there.  In this case it's the

owners name.  The tricky part when I figured this out was how to

get by the KPC's hanging prompt line ie. > or ?.  You must have an

R line for each of the several lines sent by the KPC.

CC WA3DSP-5

SC W3GJC-1

RCONNECTED TO

R###CONNECTED TO NODE

RCUSICK

RENTER COMMAND:

SC WB2GWD

R?###LINK MADE

FB0023C WB2GWD

@C:\PKT\RLI\WB2GWD.FWD

*** EOF

I am using these scripts with the K3RLI supported version 4.6 of

W0RLI's "C" program.  I do not know if it works with W0RLI's new

8.xx version of the program.

73 John WA3TSW @ WA3TSW