+----------------------------------------------------------------+

|           WDCG -- Wake Digital Communications Group            |

|                         presents                               |

|          PTP -- Packet Terminal Program Version 2.20           |

|             with XPACKET binary transfer protocol              |

|                                                                |

|                 BY N4PY .... PUBLIC DOMAIN                     |

|          Split screen routine complements of AA4L              |

|                   Documentation by AB4S                        |

|                                                                |

+----------------------------------------------------------------+

PTP is a terminal program written for the application layer of a packet radio

station.  This program was developed specifically for use with the IBM PCjr and

the TAPR TNC, but it works with other versions of the IBM PC.  PTP requires IBM

DOS 2.1 or later, and will work with the monochrome adapter or the CGA adapter.

256K of RAM minimum is required to run this program.  Very little testing has

been done on any other combinations.  It was developed originally for private

use by Carl, N4PY, but at the urging of Ed, AB4S, Carl agreed to allow this one

out of his basement.  For use with the IBM PCjr, see the note in the Appendix.

Three modes of file transfer are supported:

1. ASCII file transfer using hardware flow control

2. XMODEM protocol (with application error correction)

3. XPACKET protocol (with AX.25 error correction)

There will be no discussion of the first two, as they are both well documented

elsewhere.  XPACKET, however, is the invention of N4PY, and therefore not too

widely known, so far.  XPACKET is a binary transfer method using speeds that

are the same as ASCII dumb transfers.

XPACKET, simply put, is nothing more than a binary transfer in transparent mode

so that all eight bits of each byte can be sent, and the TNC will not mistake

certain characters as commands to itself.  What is the reason for XPACKET if

that's all there is to it?  Well, the justification is to allow the orderly

start and end of a file transfer.  Without XPACKET, each operator has to enter

TNC transparent mode and coordinate the start of the transfer.  This can be

awkward, as you may know.  Also, without XPACKET, there is no way for the

receiver to cancel the operation when he realizes that he didn't get something

right.  XPACKET tries to account for the coordination problem, and allow the

"reverse channel interrupt" function to occur.

When the transfer is begun, the receiving station will automatically send a NAK

character every ten seconds until it has received some data from the sender.

At this time, the only character the receiver can send is a CAN (cancel) which

will cause the sending station to abort the transfer.  The CAN may be sent one

of two ways:

Automatically -- if a block is received which does not agree with the byte

count which was sent at the beginning.

Manually -- if the receiving operator presses the Ctrl-N key.

The sending station operator can also cause the transfer to abort by pressing

Ctrl-N while the transfer is taking place.  You should rarely encounter the

automatic abort, but if you do, you should try starting the transfer over

again.  It is likely that one of the two stations involved in the transfer has

his TNC set up wrong or a PCjr is being used at the receiving station and is

attempting to write his file directly to a physical diskette.  See the "Must

be" list and the PCjr note in the Appendix.

USER INSTRUCTIONS:

First of all, the only thing you should have to remember is to press the Home

key when you can't remember anything else about PTP options.  This will bring

up a help screen which you can customize to help with your own selections.

(See PTP.HLP in the FILES section of this document.)

Use a text editor (such as IBM Personal Editor II, or Professional Editor) to

create a file named PTP.DEF.  This file will consist of the numbers 11 through

40 and the information you want to be sent to the TNC when the appropriate

F-Key is pressed.  It will also contain information about how to route traffic

listed on the WDCG PBBS and whether or not to add line feed characters after

each CR during ASCII uploads.  All information in PTP.DEF is in pairs of lines

with the first of each pair describing an option, and the next defining your

choice for that option.

PTP.DEF might look like:

11

c wa5szl-1+cr  +----------------------------------------------+

| This file is optional.  The usage            |

12             | of the F-keys is: 11-20 = Shift -  F1 - F10  |

c k4iww+cr     |                   21-30 = Ctrl  -  F1 - F10  |

|                   31-40 = Alt   -  F1 - F10  |

.              +----------------------------------------------+

.

.

39

My name is Carl and the QTH is Cary, NC.+cr

40

73 and good luck on packet radio.+cr

CONNECT TEXT   Default text for Alt-M connect message.  This can be

xxxxx          up to 255 characters of text.

AUTOTIME       TNC-2 or Kantronics 1.6 only.  Sets TNC clock time from

YES            the system clock.  Default NO.

AUTOMSG        TNC-2 only.  If YES, sets CMSG off at

YES            program startup and on at exit.  Default NO.

HOLDBUF        This describes the size of the retrieve buffer lines.

128            (10 - 256) default is 128

CONNECT BELL   Connect alarm bell.  If YES, alarm will sound when connection

YES            is dectected.

FOREGROUND     This describes the color choice for text on the screen.

15             (0 to 15 -- See your BASIC manual for color numbers)

BACKGROUND     This describes the background color for text screen.

0

STATE          This should be the 2-letter Postal Code for the state

NC             in which you are located.

NEAR      _____________

VA                     |

|

NEAR                   |____ These are the nearby states that might be expected

SC                     |     to access the PBBS upon which you are placing the

|     NTS traffic generated with PTP.

NEAR      _____________|

GA

SENDLF         This will determine whether or not you want PTP to add

NO             LF characters after CR's during an ASCII upload.

DCD            This will determine whether or not you want PTP to use the DCD

YES            line to determine when a connection occurs. If NO, receipt of

*** CONNECTED will be used to detect connection.

CONNECT FILE   The full filename to which you want connected text to be

B:MYMSGS.TXT   saved when Qsave is enabled.  DCD YES is required.

NOTE:  All "keywords" used in this file (STATE, NEAR, etc.) must be in upper

case.  There must not be any blank lines in this file.

NOTE:  If you do not specify FOREGROUND and BACKGROUND, they will be set to 7

and 0 respectively.  Also, if FOREGROUND and BACKGROUND are set to the same

value, the defaults of 7 and 0 will be used.

NOTE:  There is no order dependence to the keyword pairs used in PTP.DEF.  You

may put those in any order you like as long as you keep the pairs together.

You do not have to define every possible F-key, but be sure that the number for

the key you want to use precedes the text for that key.  Also, if you do not

desire to set the other parameters, you do not need to include them in your

PTP.DEF file.  There is no order dependence on these pairs of lines.

If you assign a string of characters to an Fkey, whether it is Alt-Fkey,

Ctrl-Fkey, Shift-Fkey, or just Fkey 7 through 10, you have complete control of

the string.  If you desire a CR character to follow your string you simply type

"+cr" at the end of it.  If you desire an imbedded CR character, use "+cr+" at

the place where you desire the CR.  To start a string with the CR character,

you must put "cr+" before the string to be sent.  Also, if you want to have the

Ctrl-C character as a part of the Fkey definiton, you can do that by using

"cmd+", "+cmd+" or "+cmd" at the point where the Ctrl-C is to appear.

Also, you may want to create a file to be used as setup information for your

TNC.  This file is optional, and may have any name you like.  It is a command

list for the TNC and will be sent to the TNC when you press Alt-L from the

terminal communications screen.  An example of this file would look like:

MYCALL N4PY   +----------------------------------------------+

SCREENL 0     | This file is optional.  The commands are     |

.             | the ones listed in the owner's manual for    |

.             | your TNC.  Be sure they are set the way you  |

.             | want your TNC to be configured.              |

PACTIME 1     +----------------------------------------------+

A suggested method of building this file is to enable capture (via Qsave or

ASCII rcv mode) and enter DISP.  Your TNC should give you a list of all the

commands and the present settings for these commands.

PTP will be updated from time to time.  Check the source from which you

received this copy for updated versions.  The author does not plan to release

the source code, but comments and suggestions about future releases will be

appreciated.

When you first RUN the program, you will be given a menu screen that will allow

you to set up several operating parameters.  The numbered parameters are:

1  Line bit rate  [1200] (300..9600)

2  Data bits  [8] (7 or 8)

3  Parity type  [n] (e,o, or n)

4  Split Screen  [y] (y or n)

5  Comm Port  [1] (1 or 2)

6  QSave File Name  [B:CAPTURE.TXT]

7  User Defined  []

8  User Defined  []

9  User Defined  []

10  User Defined  []

11  Save Setup  [PTP]

12  Load Setup  [PTP]

These parameters can all be set by the user.  The procedure for each follows:

Line bit rate --- Type 1,9600<CR> if the desired baud rate is not shown.

The 1200 (or any other ABAUD speed you select) will become

the operating speed for the terminal program.  Must match

TNC.

Data bits ------- type 2,8<CR> if the desired word length is not shown.

Must match TNC.

Parity type ----- type 3,n<CR> if desired parity is not shown.

Even parity, Odd parity, or No parity.  Must match TNC.

Split Screen ---- type 4,y<CR> if you desire a split screen.  If you do not

select the split screen, a full screen mode will be used

with no keyboard buffer, and the TNC will provide echoing.

Comm Port ------- type 5,2<CR> if you need to use com2 port for TNC connection.

Use the comm port (1 or 2) to which you connected your TNC.

QSave File Name - type 6,c:message.sav<CR> if you want to have the Quick Save

buffer saved under a different name when you use it.  See the

explanation of CAPTURE.TXT under "FILES USED BY PTP

TERMINAL PROGRAM:."

User Defined ---- type 7,XXX<CR> if you want to use the F keys for frequently

sent commands, or sequences.  Note that items 8, 9 and 10 are

also used in the same way.  In addition to entering the se-

quence to be sent, you can type 7,,XXXX<CR> where XXXX will

be displayed on the terminal screen as a reminder of what you

have attributed to that key.  This is true for keys 8, 9 and

10 also.  Try it; you'll like it.

Save Setup ------ type 11,c:whatever<CR> to select the file name for this

parameter file.  If you type only "11," your parameter file

will be saved as "PTP.MOD"  All parameter files are .MOD

so don't try to assign an extender to them -- the pgm will do

that for you.  The file PTP.MOD will be automatically

loaded for you when the program is started.

Load Setup ------ type 12,c:whatever<CR> to select the file name of the

parameter file you wish to use.  The rules are the same as

for 11 (Save Setup).

After the menu is written to the screen, you will see a list of all .MOD files

present on your default disk.  You may make as many of these files (with

different names) as you like.  Each can have a different personality.

You will notice that when you type nothing and simply press the Enter key, you

will be placed in terminal mode and the previously selected commands will be

issued.  Take a few minutes to set up your parameters the way you like to

operate, then save the .MOD file before going into terminal mode.  The only

prompt shown on the main menu screen is F2=Exit.  When F2 is pressed, you must

also press the Enter key from this screen.

If you have selected the split-screen mode for terminal communications, you

will have an inverse video bar across the screen near the bottom.  This bar

contains some reference information about the status of the connect message and

status of the bell action.  If you use F5 to return to DOS, depending on what

you do while in DOS, this line might be gone when you return via EXIT.  If this

is the case, simply press Alt-C to restore the screen divider line.

The Alt-key definitions are:

Alt-A = Abort the sending of NTS traffic.  This key will also halt the loading

of TNC parameters via Alt-L.

Alt-B = Toggles the BEL character action.  This affects only whether or not the

BEL will cause the alarm to sound when received.  This has nothing to

do with the Connect alarm in PTP.DEF.

Alt-C = Clears all text from the communications screen.

Alt-D = Display the definitions for the extra 30 F-keys.  This only works if

you have the PTP.DEF file on your disk.

Alt-L = Load the TNC parameters.  You must make a file (using a text editor or

by capturing the results of DISP) which contains the setup you desire

for your TNC.  This option allows you to send the entire file of

commands to the TNC.

Alt-M = Connect message screen.  You are able to edit or remove the connect

message you have selected or set up via the PTP.DEF file.

Alt-S = Send traffic file(s).  (See Alt-W.)  When logged onto the WDCG PBBS

system, and when you are in the message section, you may send your NTS

traffic files automatically by pressing this key.  When the message is

entirely sent, the program will cause the message to be saved on the

PBBS.

Alt-T = Escape from transparent mode.  When you are in the split screen mode,

you cannot go back to command mode because the characters are buffered

until Enter is pressed.  This key will cause a one-second delay, send 3

^C characters, and delay for one more second.

Alt-W = Write traffic.  This key will take you to a full-screen editor which

has a template for message traffic.  The last message which was written

will be displayed again, and you can overwrite the message with a new

one.  If you want to enter a field that is completely different from the

one shown, you may press Alt-E to erase the field before typing.

See "NTS TRAFFIC GENERATION:."

In addition to the Alt-keys, from either communications screen you may enter

the following:

F1 = [Menu] This key will immediately take you to the main menu.

F2 = [Exit] This key will cause an end to the program.  When you press

F2, you will be asked if you are sure you want to exit.

F3 = [Rcvf] This key will allow you to receive a file from the PBBS or

from another packet station.  You are asked for the transfer

protocol desired, and the drive to store the file on.  Also

you are asked to provide a name for the file, unless it is

an XPACKET file.  You may optionally provide a name for the

XPACKET file to be received.  When you are receiving an

ASCII file, you will use F3 again to close the file.  Note

that the definition changes on the bottom of the screen when

you are receiving an ASCII file or using Qsave.

F4 = [Xmtf] This key will allow you to upload (send) a file from any

disk drive on your system.  When you are asked for the drive

be sure to include the colon (i.e. A:).  The protocol

choices are the same as for F3.

F5 = [DOS ] This key causes an immediate exit from the terminal program

and you will get the DOS prompt.  An XOFF is sent to the TNC

when you go to DOS via F5, and an XON is sent when you re-

turn.  You may do any DOS functions that your amount of mem-

ory will support.  Remember that the terminal program stays

in memory, so the usable memory available to you is reduced

by the amount required by the program.  When you are ready

to return to the terminal program, Enter "exit" at the DOS

prompt.

NOTE: If you leave the program altogether while you are in

DOS, you will have to send a CTRL-Q to the TNC to restore

normal operation.  If you return to PTP via "exit", it will

be done for you.  Each time PTP is started it issues an XON

to your TNC on the selected comm port.

NOTE: You must be running IBM PC DOS version 2.1 or higher to use F5.

Additionally, you must have COMMAND.COM somewhere that DOS can find it

or the program will abort.

F6 = [Qsve] This key will enable the "Quick Save" option.  On the main

menu you selected a file name for Quick Save.  When F6 is

pressed, the file will be appended with all that is received

until you press F6 again, or F3 to close the file.

The F6=[Qsve] will flash to let you know that the file is open.

You can, if you desire, capture any messages or monitored packets

which are being held in your TNC due to either XOFF or because of

the DTR line not being raised until the comm port is opened by

pressing F6 immediately after you do the null enter to go to the

communications screen.

F7 - F0     These keys are user defined.  See the main menu to set up

the definition and the 4-character description for these

keys.  Don't forget the rules for insertion of the CR character.

(See information under PTP.DEF for rules concerning CR character

and Ctrl-C character.)

Home        This key will cause the PTP.HLP file to be displayed, even

if you no longer have the PTP diskette installed.  When the

program is loaded, PTP.HLP is read from the diskette and

kept in storage for use when Home is pressed.

Cursor Up   These keys are used to recall previously entered text.  The

and      PTP.DEF file has an entry with the keyword HOLDBUF.  This

Cursor Dn   determines the length of the recall line.  You should set

this to the same length of your PACL in the TNC.  When you

press Cursor Up, you will step through the ten-step buffer

in one direction, and pressing Cursor Dn will step the

other direction.  Experiment with this and set the length

so it is proper for your operation.  This is most useful

for correcting typographical errors in connect attempts, or

for CQ messages when in CONVERSE mode.  This function is not

available in single screen mode.

NTS TRAFFIC GENERATION:

While in communications mode, if you desire to write a piece (or more) of

traffic, you can simply swap screens (by pressing Alt-W) and enter the traffic

on a full screen template.  The fields are defined and you can type the

required information in the highlighted fields.  There are several keys which

are used for NTS traffic entry.  They are:

Enter    Moves the cursor to the beginning of the next line, if multi-

line field.  If you are on the last line of a multi-line field

or if the field is a single line field, the cursor will move

to the beginning of the next field.

Tab      Moves cursor to the next field.

Shft-Tab Moves cursor to beginning of the previous field.

Alt-E    Erases to the end of the current field.

Alt-C    Returns to communications mode.

Esc      Saves the message under the name MSGnnn.MSG.

The nnn in the message name is the number field from the

message itself.  When this message has been uploaded to

the WDCG PBBS, the message file will be renamed to

MSGnnn.LOG.

Note that the check is not required in the message outline.  It will be

automatically generated when the message is saved.

The TIME field is special.  It is not usual that this field be used and it will

normally be left blank.  If you desire to use the time field, there are two

options:

* Put an "A" in the first position of the time field.  This will

cause PTP to enter the current time from the system clock.  There

will be a "Z" appended to the time.  Be sure your system clock is

set to UTC if you use this option.

* You may enter the time directly in the time field.  This time will

not be changed by PTP.

The DATE field will be automatically filled in for you from the system clock so

be sure that your clock is set correctly.

Later, when logged on the WDCG PBBS system, you can send your accrued traffic

by going to the <M>essage menu.  After the message menu prompt, you can send

messages from diskette by pressing Alt-S.  You will be shown all the unsent

messages from your default diskette, and you simply enter the number of the

message to be sent.  The rest is automatic (including the <S>ave function after

the message is complete.  You may now send another message by pressing Alt-S

again if desired.  Note that the sending of NTS message traffic can be halted

(aborted) at any time by pressing Alt-A.  When each MSGnnn.MSG file is sent to

the WDCG PBBS, it will be renamed to MSGnnn.LOG on your disk.

FILES USED BY PTP TERMINAL PROGRAM:

PTP.EXE       The terminal emulator program.  This file can be loaded with a

.bat file, or directly from the DOS prompt by typing "PTP".

PTP.MOD       This file is used to store your main menu data.  It contains the

the personality you assigned when you configured your PTP

program.  You may have as many .MOD files as your diskette space

allows, and each can be a new personality.

PTP.DEF       This file is the only one which requires a separate editor to

create.  There is a skeleton PTP.DEF file provided with PTP which

you may use to make your own PTP.DEF.  30 additional F-KEYs are

defined by this file (Alt-Fn, Ctrl-Fn, and Shift-Fn).  This is

an optional file.  If you do not create a PTP.DEF, your F-keys

will be usable, but the extra 30 definitions will not exist.

In addition, there is information included in the PTP.DEF file

which defines screen color and routing information for NTS

traffic to be uploaded to WDCG PBBS.  See PTP.DEF under "USER

INSTRUCTIONS:" for details on this file.

MSGnnn.MSG    These files are NTS messages which have been written and are

and        ready to be sent (.MSG) and logged messages which have already

MSGnnn.LOG    been sent (.LOG).  These files are generated as a part of the

NTS message process.

PTP.CKP       This file is automatically generated and contains information

about the last NTS message sent.  It is used as the default

text for beginning the next message.  Do not alter or erase this

file if you expect NTS message writing to work properly.

CAPTURE.TXT   This file is the default name which is used for QSAVE and may

be renamed at a later time, or may be changed in the main

menu.  From either full-screen terminal mode or split-screen

terminal mode, you can press F6 and this file (or other specified

file) will be opened for capture.  The file will continue to be

updated with everything received until F3 (Close file) or F6 is

again pressed.  NOTE: This file is appended each time it is

opened and the previous contents will remain.

PTP.SET       (Or other file name as desired.)  This file is used to prime your

TNC whenever you desire.  When used via Alt-L from the commun-

ications screen, the contents of this file will be sent to the

TNC in cmd: mode.  It is useful for setting, or restoring any

set of parameters you desire.  A suggested use is to capture (via

Qsave) the results of "disp" and use an editor to change this

file so that it will restore your TNC in the event of a loss of

data due to any unforeseen event.

PTP.HLP       This file is required if you desire to use the Home key for

online help.  You may, at your option, use a text editor to add

your own key definitions to the lower half of the .HLP screen.

NOTE: This file must remain the same size -- 23 lines.

PTP.SCN       This file MUST be present and MUST be unaltered in order for

PTP to run.  It must be on the default disk(ette) drive.

PTP.DOC       The documentation file (this file).  Print this file on your

system printer if you desire to have hard copy.  This file MUST

be copied along with PTP if you desire to share this program with

another Amateur Radio operator.

PTP.WRK       This is a temporary file which is generated by PTP when

downloading.  If you break, or end the program, before the down-

load is completed, you might see this file on the selected disk

drive.  You may erase it, or rename it if you desire.

MYMSGS.TXT    Or other name to your liking.  This is the file which you cause

to be created when you have CONNECT FILE in the PTP.DEF list.

This file will contain only messages that were received by you

while you were connected and F6 (Qsave) was active.  All

monitored traffic will be saved in CAPTURE.TXT.

When you are receiving an ASCII file via F3, the file will be automatically

closed when a BEL character is received.  The WDCG PBBS always ends downloads

with the BEL character.  Also, when PTP uploads a file, it will put a BEL

character as the last byte of the file.  This can be very handy when you are

transfering a file to a friend.  If he is not using PTP, have him to send you a

Ctrl-G when he has finished sending the file, and it will close your disk file,

thus ending the transfer.

SPLIT SCREEN CONSIDERATIONS:

For the most part, split screen operation is as simple as typing on the bottom

of the screen and seeing what is received on the upper part of the screen.

There are, however, a few things to be considered.

Because the split screen code is an assembly code routine, the BIOS calls are

not handled through DOS and are not the same as the single screen mode.  The

characters from the keyboard are buffered in PTP until you press the Enter key,

reach the size of your buffer (which you established in .DEF file).  Since you

sometimes want to send a character to the TNC without the CR, a function is

provided to allow this.  If you press the Tab key, the buffer will be sent to

the TNC without a trailing CR.  This is useful for entering command mode by

pressing Ctrl-C and following this with a Tab.  You will be unable to send the

HT character in split screen mode but this will be a very rare inconvenience.

If you desire to send the Tab character it will be necessary to use the single

screen mode.

Also, since characters will not normally be sent to the TNC until Enter is

pressed, a special key is defined to allow you to exit transparent mode from

the split screen.  See the description of Alt-T for this procedure.

Split-screen operation will seem a little unusual to you until you get used to

the benefits.  For instance, you can be typing to someone and he might send a

packet to you before you are finished.  You should see this information appear

on the upper part of your screen even as you type.  Additionally, it is very

nice to have all of your text on the lower part of the screen and the received

information above the divider line.  It will not scroll off your screen nearly

as fast this way.

APPENDIX:

XPACKET FLOW DIAGRAM

Example of an unsuccessful transfer.

SENDER                                                RECEIVER

------                                                --------

Ready to send file.

<---------------------------------------------- NAK (repeated every

10 seconds un-

til data trans-

fer begins)

filename -------------------------------------------->

128 bytes data -------------------------------------->

(listens)

128 bytes data -------------------------------------->

(listens)

128 bytes data -------------------------------------->

(listens)

128 bytes data -------------------------------------->

(listens)

<---------------------------------------------- CAN

< Both stations abort transfer and return to terminal mode. >

Example of a successful transfer:

SENDER                                                RECEIVER

------                                                --------

Ready to send file.

<---------------------------------------------- NAK (repeated every

10 seconds un-

til data trans-

fer begins)

filename -------------------------------------------->

128 bytes data -------------------------------------->

(listens)

128 bytes data -------------------------------------->

(listens)

128 bytes data -------------------------------------->

(listens)

128 bytes data -------------------------------------->

(listens)

128 bytes data -------------------------------------->

(listens)

128 bytes data -------------------------------------->

(listens)

128 bytes data -------------------------------------->

(listens)

109 bytes data -------------------------------------->

(listens)

End Of File ----------------------------------------->

< ----- Both stations return to terminal mode.------ >

NOTE: If nothing is received by the receiving station after 10 seconds, another

NAK is sent.  This is repeated every 10 seconds until the file transfer is

started.

NOTE: For details about the XPACKET protocol, see XPACKET.PRO, a brief

description of the protocol.

What's all this about "filename"?  Well, the XPACKET protocol sends the file

name in its first burst of data.  The receiving station can do with this what

he will.  The PTP program shows how this can be handled by allowing the file to

be saved under the name received, or under the name specified by the operator.

This program gives the receiver a chance to select a name under which he wants

the received file to be stored, or if you choose to let the sender provide the

name, you simply press Enter at the prompt for a file name.

The PTP program uses "hardware flow control" which means that it does not honor

XON or XOFF from the TNC.  Instead, it relies on the asynchronous port hardware

to control the sending of data to the TNC.  When the TNC buffer becomes full,

the TNC will drop CTS (Clear To Send) and the terminal will halt in its tracks

until CTS is again raised.  This is how the TNC keeps from being overrun with

data when the flow is otherwise continuous.

When the receiver XPACKET station is waiting to receive the first packet of

data it will send NAK every 10 seconds until the first packet is received.  The

last block of data will not necessarily be 128 bytes, but it will be followed

by an EOF character to signify the end of the transfer.  The receiving station

will then save the file according to the filespec received, or the one entered

by the operator.

TNC SETUP:

There are two catagories for the TNC command setup.  They are:

o  Must be     (for PTP to work as intended)

o  Recommended (Local area determined)

Must Be list:

============================================================================

AWLEN 8          DELETE OFF           PARITY 0 (TNC2)     CR ON

AX25 ON          FLOW ON              SENDPAC $0D         COMMAND $03

BKONDEL ON       PACTIME EVERY 1      XFLOW ON            8BITCONV ON (TNC2)

CMDTIME 1        PARITY 4 (TNC1,Kan)  XMITOK ON           TRFLOW OFF  (TNC2)

CONMODE CONVERS  START $11            STOP $13            TXFLOW OFF

============================================================================

Recommended list:

============================================================================

AUTOLF ON           BEACON EVERY 0         CONOK ON     LCOK ON

CWID OFF            HBAUD 1200 (TNC1,Kan)  LFADD OFF    NEWMODE ON (TNC2,Kan)

MCON OFF            RETRY 10               SCREENL 0    TXDELAY 6 (TNC1,Kan)

TXDELAY 24 (TNC2)

============================================================================

FINAL COMMENTS FROM WDCG:

The PCjr cannot write directly to a physical disk file during file reception.

If you have enough RAM installed to define a ramdisk, you can use this as the

target drive and should have no problems.

If you are using an IBM PCjr without the internal modem, you will need to run

COMPCJR.COM program which is included on this diskette.  The PCjr's RS232 port

is normally referred to as COM1 when the internal modem is not installed, and

as COM2 when the internal modem is there.  The hardware addresses are not

changed.  Be sure to select COM2 from the main menu screen if you use

COMPCJR.COM.

We hope you find the Packet Terminal Program as useful and as much fun as we

have.  Please feel free to pass this program along to other Amateur Radio

operators and Packeteers, but please DO NOT attempt to sell this program, or to

use it in any commercial way whatsoever.  This program is intended for the

private use of Radio Amateurs and your cooperation is greatly appreciated in

keeping it just as the author intended.  If you give a copy of PTP to someone

please be sure to include this document file also.

The author of PTP assumes no responsibility for the use of this program and no

guarantees are implied or intended.

Comments and suggestions are welcome, especially about the binary transfer

protocols.  Send comments to:

Carl Moreschi,N4PY

702 Madison Ave.

Cary, NC 27511

919-467-8146

or via K4IWW PBBS (145.010) in Cary, NC.