MAPPER .. A DX MAP DRAWING and Propagation Program

for IBM PC Compatibles with graphics capability.

by

Dennis P. Murray

11165 McDonald Street

Culver City, Ca. 90230

CompuServ 72770,2765

This program, written for BASIC compilers such as MICROSOFT QUICK

BASIC or BORLAND TURBO BASIC, displays a map of the world centered on

a user-selectable QTH, and does propagation predictions based upon a

modified MINIMUF method. It shows the regions of sunlight and darkness

around the world, and updates these regions automatically at user-

specified time intervals. It can also accept arbitrary times and dates

at the user option. The program accepts DXCC prefixes or

latitude/longitude values for the DX location, draws the path from the

home QTH to that location, and displays the range and azimuth from the

home QTH. The user can select either short or long paths. The sunspot

number used for propagation prediction can be changed either

temporarily, or be set as the new default value. The propagation

prediction gives the MUF (maximum usable frequency), LUF (lowest

usable frequency) based upon E-layer screening, and the minimum number

of F-layer hops for a 5 degree minimum launch angle.

This program is  written for IBM PC compatible computers with an

EGA Graphics capability and color display. The source code provided

could be modified by the user to use other graphics displays supported

by your BASIC compiler. The program runs quite quickly on an AT clone

with a hard disk and 640k of RAM, but it might be a 'dog' on a floppy-

only machine because it must read some long files for the map data.

The author does not intend to support this software, but is

simply releasing it into the public domain for non-commercial use by

HAMS who know what they are doing with their computer. The BASIC

source code is included for that reason.

The author encourages the distribution of this program to anyone

who might find it useful, or onto bulletin boards which deal with HAM

radio or shortwave listening. Please make sure to include ALL the

required files in any uploading however. These files are:

1- MAPPER.BAS       The Program Source Code

2- MAPPER.EXE       the Q-BASIC v3.0 executable code

3- MAPPER.ATL       An Atlas of DXCC countries and their

latitudes, longitudes, and DX prefix.

This is an ASCII file ,sorted by prefix,

which can be added onto by the user with

a word processor or text editor.

4- MAPPER.DEF       A short file used to store some default

values used by the program.

5- MAPPER.SCR       A large file which is used to store and

retrieve the screen data for a world map

centered on the user's location. This

file is rewritten whenever the user

changes the 'Home' location. ( It is

interesting to view the world as seen

from DX locations.) This data is read

into a large array in memory to permit

rapid redrawing of the map as the sun

moves. The file is written into the

integer array NSTORE by BLOAD and BSAVE

commands. Because the program uses a lot

of memory to speed things up, this array

is DYNAMIC and does not generally reside

in the normal BASIC data segment. The

version of the program supplied for

Q-BASIC uses the CALL PTR86 subroutine

supplied by MICROSOFT, and this module

must be specified during the linking

process. In TURBO-BASIC the VARSEG

command can be used to find the array

segment used in lines 2020 and 12140.

6- MAPPER.DOC       This instruction file.

7- WORLDMAP.DAT     A large ASCII file containing the

lat/lon of the world's borders. This is

used to recompute the map whenever the

map center point is redefined.

INSTRUCTIONS:

To run this program simply enter 'MAPPER'. After an initial

message explaining the program is displayed, and you press any key to

proceed, you will be presented with a 3-choice initial menu which

allows you to change the default values of sunspot number, Home QTH

latitude and longitude, or the time interval at which the

daylight/darkness areas are redrawn. Pressing the appropriate number

key will put up prompts to allow changes to these variables, and will

return you to this menu after a change is entered. Pressing <ENTER> at

the menu prompt will store these new values in the MAPPER.DEF file and

proceed on to the map drawing.

The map drawing will first put up a blank box, then calculate the

position of the sun at the time and date currently held by the DOS,

draw the solar terminator ( it looks like a satellite ground trace ),

and color the sun-lit portions of the world in red. It will then go on

to draw the country boundaries, color the oceans blue, and color the

USA green. The land areas which are in darkness will show up in black.

The program may miss coloring part of the oceans blue because of the

position of the solar terminator blocking the PAINT commands used in

the subroutine at line 8000 ... . I got tired of adding ocean points

to this routine to cover all the contingencies.

After the map is drawn the Main Menu will appear on the left of

the screen, with 8 choices:

1-   Select DX Prefix.

This is the default choice ( <enter> will select

it. ). You will be prompted to enter a DX prefix ( eg.

ZS for S. Africa ). The program will then search the

DX atlas for an occurrence of this prefix. It will

display all of the entries ( ie ZS1,ZS2,... ) that

contain this string and ask you to select one. If the

number of candidate entries is large you can simply

<ENTER> to continue the search.

When a selection is made the program will fetch

the location of the prefix, make a propagation

prediction for the currently selected date and time,

draw the path from your QTH to that location, and

display sundry useful information at the top of the

screen.

2-Specify Lat/Lon.

Same as the above option except you will be

prompted to enter the latitude and longitude of the

desired DX location.

3-Change Sunspot Number.

Prompts for a new value of sunspot number. The

program will use this new value until it is changed

again during this session, but it will not change the

value in the default file that is read when the

program starts.

4-Select Date/Time.

Prompts for a new date and time to be used to

calculate solar terminator and make propagation

predictions. <ENTER> responses to either of these

prompts will fetch the current DOS value of date or

time. This option will also turn off the automatic

updating of the terminator and map redraw.

5-Use Real Time.

This option will reset the above option to the

real ( DOS ) date and time and enable the auto update

feature.

6-Select Short Path.

This toggles the path from the long path option

back to short path.

7-Select Long Path.

This option toggles long path calculations.

8-Quit.

Exits the program and returns you to DOS.

The program is entirely menu-driven so you don't have to remember

any special commands to use it.

Good luck!!

Some Technical Information:

The propagation is based upon the MINIMUF method, but with some

modifications of the author.

1-I limit the ground range per F-layer hop to 3500km. This keeps

the minimum elevation angle to about 5 deg. for F-layer heights

of 300km. It is a rare HAM who has a high enough antenna or good

enough ground screen to get much energy below 5 degrees. For

those souls who want to be optimistic you can change line 40030

to some longer range per hop.

2-Rather than use the standard MINIMUF method of only looking at

the first and last ionospheric bounce points ( the two control

point method ), I calculate the propagation at every hop and

select the worst-case condition out of all of them. This results

in generally much more pessimistic MUF and LUF estimates than

MINIMUF.

3- I have included the E-layer screening frequency and lowest-

usable frequency ( LUF ) estimation technique given in chapter 4

of 'Amateur Radio Software ', by John Morris GM4ANB , RSGB

Books, Lambda House, Cranborne Road, Potters Bar, Hertfordshire

EN6 3JW. The subroutine starting at line 45000 contains this

calculation.

4-I have used what apparently is the HAM convention for longitudes

being positive-West for the input and output in this program, in

the MIMIMUF subroutine, and in the DX Atlas file MAPPER.ATL.

However, in the internals of the program I immediately reverse

the sign of the longitude. Being an engineer by training I think

in terms of right-handed coordinate systems! Therefore those who

want to modify the program should be wary of the sign of the

longitude being used.

5-Note that I have changed the names of the variables in the

MINIMUF subroutine from the standard values to names which are

descriptive of what the variable actually is.