=================================================================

X

X X

X X X

X  X  X

X   X   X

X    X    X

X     X     X

X      X      X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

XXX

XXXXX

XXXXXXX

THIS SHEET IS PROVIDED TO HELP YOU ALIGN YOUR PRINTER PAPER

ALIGN PAPER TO HAVE A SMALL MARGIN ABOVE THE TOP,

AND BELOW THE BOTTOM, ROW OF "=" CHARACTERS.

XXXXXXX

XXXXX

XXX

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X      X      X

X     X     X

X    X    X

X   X   X

X  X  X

X X X

XXX

X

=================================================================

==============  AUTORTTY - SAMPLE PROGRAM (AUTOSMPL.EXE)  ==============

======================= IMPORTANT NOTE =======================

THIS PROGRAM DOES -NOT- SUPPORT INTELLIGENT TERMINALS SUCH AS THE PK-232

ENCLOSED IS YOUR "AUTORTTY" SAMPLE PROGRAM (NAMED: "AUTOSMPL.EXE").  THE

DOCUMENTATION  FILE  IS LOCATED IN THE SUB-DIRECTORY  CALLED,  DOC-FILE.

THE  "SAMPLRTY.DOC"  FILE  CAN  BE PRINTED  USING  YOUR  MS-DOS  "PRINT"

PROGRAM.   THE FIRST PAGE HAS TOP AND BOTTOM EQUAL SIGNS "=" TO HELP YOU

ALIGN THE PAPER IN YOUR PRINTER.

PLEASE  NOTE:

A NUMBER OF FUNCTIONS HAVE BEEN DISABLED IN THE ENCLOSED SAMPLE PROGRAM.

THE  RECEIVE  AND TRANSMIT BUFFERS ARE LIMITED TO 5K  EACH,  THE  NORMAL

BUFFER  SIZE  IS  65,500 BYTE FOR EACH.  THE  INFO  SCREENS  WITHIN  THE

PROGRAM HAVE NOT BEEN CHANGED, HENCE, IT WILL GIVE THE VALUE 65,500.

YOU CAN -NOT- SAVE THE TRANSMIT BUFFER, WITHIN THE EDITOR, IN THE SAMPLE

PROGRAM.  AUTO-PIX (THE "PIXBOX" MODULE) IS NOT INCLUDED IN THIS  SAMPLE

VERSION.   AUTO-SAVE  (SENDING  RECEIVED TEXT TO  DISK)  HAS  ALSO  BEEN

DISABLED.  YOU CAN SAVE THE "LIMITED" RECEIVE BUFFER TO DISK VIA F4 KEY.

IMPORTANT MESSAGE:

YOU CAN ACTUALLY RECEIVE AND TRANSMIT WITH THE ENCLOSED SAMPLE  PROGRAM,

BE  AWARE THAT THE ENCODED CALLSIGN FOR "CWID" AND "OWNER  VERIFICATION"

HAVE  THE  LETTERS "S-A-M-P-L-E" (WITHOUT THE HYPHENS) EMBEDDED  IN  THE

PROGRAM.   WHAT THIS MEANS IS:  IF YOU PRESS CONTROL I TO SEND THE  CWID

(OR  YOU ARE IN TRANSMIT MODE OVER 9 MINUTES), IT WILL SEND THE  LETTERS

"S A M P L E".   IF  YOU TRANSMIT OVER  300  CHARACTERS AND  ATTEMPT  TO

RETURN TO RECEIVE, IT WILL INFORM YOU TO SEND PROPER ID.  THE  ENCLOSED,

SAMPLE,  PROGRAM  REQUIRES  YOU TO SEND THE LETTERS  "SAMPLE".   IN  THE

ACTUAL  PROGRAM, YOU WOULD BE REQUIRED TO SEND, AT LEAST, YOUR  CALLSIGN

BEFORE  YOU CAN EXIT TRANSMIT MODE (NORMALLY YOU WOULD SEND THE  STATION

YOU'RE WORKING AND YOUR CALL IN THE ID) EITHER WAY, YOU COULD THEN ENTER

THE <CTRL> E TO EXIT TRANSMIT MODE AND RETURN TO RECEIVE.

REMEMBER,  THE  RECEIVE AND TRANSMIT (PIX) BUFFERS ARE LIMITED  TO  5000

CHARACTERS IN THIS SAMPLE PROGRAM.

IF  YOU ENCOUNTER SCREEN INTERFERENCE WHEN USING A "CGA" VIDEO  ADAPTER,

YOU WILL HAVE TO USE THE /SNOW COMMAND LINE SWITCH WITH THE PROGRAM NAME

WHEN  YOU  BRING-UP THE PROGRAM.  IF YOU HAVE A  MONOCHROME  (COMPOSITE)

MONITOR  ON  YOUR  COLOR  VIDEO CARD, YOU WILL HAVE  TO  USE  THE  /COMP

(COMPOSITE MONOCHROME)  COMMAND LINE SWITCH.  YOU CAN USE EITHER OR BOTH

OF THESE SWITCHES IF NECESSARY.

EXAMPLE:  AUTOSMPL /SNOW /COMP<RETURN>

THE  ABOVE  EXAMPLE  INCLUDES  BOTH SWITCHES - USE  ONLY  THE  ONES  YOU

ACTUALLY NEED.

TO  PROPERLY  INSTALL  THE  PROGRAM  AND  SET  THE  PARAMETERS  IN   THE

CONFIGURATION SCREENS, PRINT-OUT THE DOC FILE "SAMPLRTY.DOC" AND  FOLLOW

THE INFORMATION UNDER INSTALLATION AND CONFIGURATION PAGES.  RS-232 PIN-

OUT INFORMATION (THE PROGRAM USES) IS ALSO INCLUDED IN "SAMPLRTY.DOC".

THE  OTHER PAGE, INCLUDED HERE, GIVES INFORMATION REGARDING  THE  SYSTEM

REQUIREMENTS NECESSARY FOR RUNNING "AUTOSMPL.EXE" PROGRAM, AS WELL AS  A

LIST OF MANY OF IT'S MAJOR FEATURES.

WALTER E. BESS, JR. (K8LC) -- PRECISION ELECTRONICS

1525 KARIN STREET -- IRONTON, OHIO   45638

** AUTORTTY/AUTO-PIX PROGRAM REQUIREMENTS AND ADDITIONAL INFORMATION **

THIS PROGRAM DOES -NOT- SUPPORT INTELLIGENT TERMINALS SUCH AS THE PK-232

REQUIREMENTS:  IBM OR COMPATIBLE COMPUTER - USING MS-DOS 2.1 OR HIGHER.

FREE MEMORY REQUIRED TO RUN:  391K FOR AUTORTTY - 410K FOR AUTO-PIX

=== ABOVE REQUIREMENTS MAY CHANGE WITH LATER VERSIONS OF PROGRAM/S ===

THIS  PROGRAM SUPPORTS MOST (BUT NOT ALL) OF THE POPULAR VIDEO  ADAPTERS

USED  IN IBM, AND COMPATIBLE, COMPUTERS.  THE /SNOW COMMAND LINE  SWITCH

IS PROVIDED TO HANDLE THE "SNOW" PROBLEM ENCOUNTERED WITH SOME CGA VIDEO

ADAPTERS.  THE /COMP COMMAND LINE SWITCH MUST BE USED IF YOU ARE USING A

COMPOSITE VIDEO MONITOR (MONOCHROME) ON YOUR COLOR VIDEO ADAPTER.

SOME OF THE FEATURES OF AUTORTTY:

* 45-75-110-135-300 BAUD - EITHER BAUDOT (5 LEVEL) OR ASCII (8/7 LEVEL)

* FULL USER CONFIGURATION - WHICH IS SAVED TO DISK AND USED AT STARTUP.

* RECEIVE BUFFER SIZE IS 65,500 BYTE - WHEN FULL WILL CONTAIN LAST 65.5K

* ABILITY TO SAVE THE RECEIVE BUFFER TO DISK - AT ANY TIME.

* AUTO-SAVE -ECHOES RECEIVED TEXT TO A DISKFILE, UP TO DISK CAPACITY.

* PRINT ECHO - <CTRL> P - RECEIVE TEXT (AND XMIT PIX) ECHO TO PRINTER.

* PROGRAM WILL -NOT- HANGUP IF PRINTER IS TAKEN OFF LINE FOR ANY REASON.

* SUPPORTS PARALLEL PRINTERS:  LPT1-2-3 -AND- SERIAL PRINTERS:  COM1-2-3

* USES A FULL 65,500 BYTE BUFFER TO:  LOAD - EDIT  AND TRANSMIT YOUR PIX

* THE ABOVE BUFFER CAN ALSO BE LOADED FROM THE RECEIVE BUFFER (OR BLOCK)

* THE "AUTOEDIT" MODULE, FOR EDITING THE XMIT (PIX) BUFFER, INCLUDES ALL

THE FEATURES NECESSARY TO CREATE OR MAINTAIN YOUR PIX FILES.

* THE ABILITY TO SEND PART (FILL) OF A PIX FROM THE XMIT (PIX) BUFFER.

* LINE ONE WILL DISPLAY FILENAME LOADED IN BUFFER - AND FILL LINES SET.

* 10 (1K BYTE) MESSAGE PORTS - 9 SAVABLE TO DISK - LOADED IN AT STARTUP.

* ABILITY TO NEST MESSAGE PORT CALLS - MAKES IT EASY TO "BUILD" YOUR ID.

* MESSAGE PORT #1 CAN BE MODIFIED WITHOUT ENTERING PORT EDIT MODE.

* TERMINAL UNIT CAN USE EITHER COM1 OR COM2 SERIAL PORT.

* 3000 BYTE TYPE AHEAD BUFFER - MUCH LARGER THAN ACTUALLY NEEDED.

* TRANSMIT "BREAK" MODE - ALLOWS YOU TO ANSWER A QUERY WITHOUT  SENDING

YOUR TYPE AHEAD BUFFER. (VERY HANDY)

* VIEW BUFFERS MODE - TO ALLOW YOU TO QUICKLY LOOK THROUGH THE BUFFERS

* BLOCK MODE - ALLOWS YOU TO "MARK" A BLOCK TO RESEND, SAVE OR DELETE IT

* HELP SCREENS AVAILABLE FOR MOST MODES - GIVES BRIEF DESCRIPTIONS.

* AUTOMATIC CW-ID - EVERY 9 MINUTES - OR CAN BE SENT WITH <CTRL> I.

* DISPLAY A DISK DIRECTORY FROM WITHIN PROGRAM (USES YOUR PATH INPUT).

YOU  WILL  STILL HAVE 6 LINES OF RECEIVE TEXT TO VIEW.  YOU  CAN  ALSO

SELECT:   LOAD XMIT (PIX) BUFFER MODE -AND- DELETE A FILE  MODE  WHILE

YOU VIEW THE DIRECTORY.

* INPUT OF A SEPARATE PATH FOR EACH PROGRAM MODE (USING CONFIGURATION)

YOU CAN ALWAYS CHANGE THE DEFAULT PATH WHEN NECESSARY.

* ABILITY TO CLEAR:  TYPE AHEAD, RECEIVE AND XMIT (PIX) BUFFERS.

* DELETE A DISKFILE FROM WITHIN THE PROGRAM.

* PRINT MODE WHICH ALLOWS PRINTING FROM EITHER THE RECEIVE OR XMIT (PIX)

BUFFERS.   YOU CAN PRINT ALL OR PART (FILL) OF EITHER BUFFER AND  HAVE

THE OPTION TO "AUTO LINEFEED" WHICH WILL SPREAD OUT OVERLINES IN A PIX

* AUTO-PIX  VERSION  OF AUTORTTY - THE  AUTO-PIX  MODE IS  A  COMPLETELY

AUTOMATIC "PIXBOX" MODULE WITHIN  "AUTORTTY".  IT ALLOWS OTHERS (AFTER

GIVING  YOUR  SYSTEM  CALLUP)  TO LOG ON YOUR  SYSTEM,  GET  DISK  AND

PIXFILE  NUMBERS, READ YOUR PIX FILES, GET PART  (FILL) OF  A  PIXFILE

(AFTER  GIVING   START  AND END LINE NUMBERS), LEAVE  (SAVE)  A  SHORT

MESSAGE FOR YOU  -AND-  OTHER FUNCTIONS RELATED TO RTTY PIX GATHERING.

* "AUTORTTY" ALWAYS SENDS THE PROPER END-OF-LINE SEQUENCE FOR RTTY PIX.

* SEVERAL SPECIAL PIX FILE MAINTENANCE PROGRAMS INCLUDED WITH EACH PROG.

WALTER E. BESS, JR. (K8LC) -- PRECISION ELECTRONICS

1525 KARIN STREET -- IRONTON, OHIO   45638

============== AUTORTTY/AUTO-PIX VERSION OF PROGRAM ==============

THE  ENCLOSED  SAMPLE PROGRAM IS OF THE BASIC "AUTORTTY"  RTTY  PROGRAM,

ALTHOUGH WRITTEN FROM THE GROUND UP FOR USE WITH RTTY (PIX) PICTURES, IT

CONTAINS ABOUT ANY FUNCTION NECESSARY FOR YOUR AMATEUR RADIO,  STANDARD,

RTTY COMMUNICATION NEEDS.  NOTE: THIS ISN'T A CONTEST PROGRAM.

AS  MENTIONED  ON THE LAST PAGE, AUTORTTY/AUTO-PIX IS  A  "PIXBOX"  TYPE

PROGRAM.  WHICH MEANS IT IS SIMILAR TO A "MAILBOX" PROGRAM (IN THAT  YOU

LOG  ON AND SEND IT COMMANDS) BUT CAN'T BE DIRECTLY USED AS  A  MAILBOX.

ALTHOUGH  IT DOES HAVE THE ABILITY TO HAVE SOMEONE SEND YOU  A  MESSAGE,

AND  SAVE IT TO DISK, THERE ARE NO FUNCTIONS TO SEARCH FOR "MAIL".   THE

MESSAGE SAVE, IN THIS PROGRAM, IS ONLY THERE SO YOUR "PIXBOX" USERS  CAN

LEAVE YOU A MESSAGE.

ONCE  CALLED  UP, BY SENDING THE REQUIRED CALL-UP CODE, A USER  WILL  BE

PROMPTED TO GIVE THEIR CALLSIGN THREE TIMES, SEPARATE EACH WITH A  SPACE,

AND  IF  EVERYTHING GOES OKAY, THEY WILL ENTER "AUTO-PIX"  MODE.   AFTER

GETTING  LOGGED  ON,  THE USER CAN GIVE VARIOUS  COMMANDS  WHICH  ALLOWS

HIM/HER TO RECEIVE PIX SENT FROM THE PIXFILE DISK DRIVE AS GIVEN IN  THE

PROGRAM CONFIGURATION.

THE  FULL  AUTORTTY/AUTO-PIX PROGRAM IS REALLY DESIGNED FOR  THOSE  THAT

HAVE SIZABLE RTTY PIX LIBRARIES AND PLAN ON HAVING A SYSTEM THAT  OTHERS

CAN  ACCESS AND RECEIVE PIX FROM THEIR PIX LIBRARY.  THIS  PROGRAM  ALSO

ALLOWS USERS TO ISSUE A "//FILL" COMMAND AND RECEIVE PART OF A PIX FILE.

THE USER ENTERS BOTH THE START AND END LINE NUMBERS FOR THE FILL.  THERE

ARE MANY COMMANDS WITHIN AUTO-PIX, ALL DESIGNED WITH THE USER'S EASE  OF

USE IN MIND.

========================================================================

THE  "AUTORTTY/AUTO-PIX"  PROGRAMS  ARE NOW READY FOR SHIPMENT.

THE  INTRODUCTORY PRICE OF "AUTORTTY" AND  "AUTORTTY/AUTO-PIX"  PROGRAMS

WILL BE AS FOLLOWS (NOTE: THE PRICES ARE IN U.S FUNDS):

AUTORTTY (COMPLETE RTTY WITHOUT AUTO-PIX "PIXBOX" MODULE) - $ 65.00

AUTORTTY/AUTO-PIX (COMPLETE RTTY WITH "PIXBOX" PROG) ------ $125.00

YOU MUST INCLUDE PRIORITY MAIL POSTAGE FOR THE ABOVE PROGRAMS

POSTAGE PAYMENT REQUIRED:

U.S.:

AUTORTTY AND AUTO-PIX PROGRAMS ----------------------------- $ 3.50

CANADA:

AUTORTTY AND AUTO-PIX PROGRAMS ----------------------------- $ 4.00

TO AVOID THE TWO TO THREE WEEK "PERSONAL CHECK" DELAY, IN SHIPPING  YOUR

PROGRAM, SEND YOUR ORDER WITH FUNDS BY CASHIER'S CHECK OR MONEY ORDER.

U.S. ORDERS:

THE  U.S.  POSTAL  SERVICE OFFERS A "COD" OPTION, BUT IT HAS  A  FEE  OF

$4.00,  THIS  IS  IN  ADDITION TO THE NORMAL  POSTAGE  FOR  SENDING  THE

PROGRAM. THE POSTAL SERVICE ALSO CHANGES YOU FOR A MONEY ORDER FOR  YOUR

RETURN.  I FEEL THIS IS JUST TOO HIGH AND WOULD PREFER NOT TO USE IT

TO ORDER EITHER OF THE PROGRAMS, USE THE ORDER FORM BELOW.

------------------------------ CUT HERE --------------------------------

PRECISION ELECTRONICS

1525 KARIN STREET - P.O. BOX 764

IRONTON, OHIO   45638   USA

PHONE: 1-614-532-6111

========================================================================

FULL NAME:

------------------------------------------------------------------------

AMATEUR CALL:                                 DATE:

------------------------------------------------------------------------

ADDRESS:

------------------------------------------------------------------------

ADDITIONAL ADDRESS

(IF NECESSARY):

------------------------------------------------------------------------

CITY, STATE, ZIP:

------------------------------------------------------------------------

=====  THE AMOUNTS BELOW ARE IN U.S. FUNDS  =====

===== NON U.S AMATEURS - MAKE APPROPIATE MONETARY CONVERSION =====

MARK DISK SIZE YOU PREFER FOR PROGRAM DISK(S).

____ 5 1/4 360K  ____ 5 1/4 1.2M(HD)  ____ 3 1/2 720K  ____ 3 1/2 1.44M

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

_____ AUTORTTY (BASIC RTTY PROGRAM WITHOUT AUTO-PIX) --- $______________

----- AUTORTTY/AUTO-PIX (FULL RTTY AND PIXBOX PROG.) --- $______________

OHIO RESIDENTS -MUST- INCLUDE 6% SALES TAX ON ORDER ---- $_____________

YOU -MUST- INCLUDE SHIPPING WITH YOUR ORDER.

U.S. RATE: $ 3.50 PER PROGRAM

CANADIAN RATE: 4.00 PER PROGRAM       POSTAGE INCLUDED:  $_____________

TOTAL ENCLOSED:    $______________

AVOID THE 2 TO 3 WEEK PERSONAL CHECK DELAY

SEND CASHIER'S CHECK OR MONEY ORDER

SORRY, NO COD'S AT PRESENT - U.S. POSTAL SERVICE COD JUST TO HIGH TO USE

SEND YOUR REMITTANCE TO:

WALTER E. BESS, JR.  K8LC

PRECISION ELECTRONICS

1525 KARIN STREET - P.O. BOX 764

IRONTON, OHIO   45638   USA

------------------------------ CUT HERE --------------------------------

SAMPLE PROGRAM OF "AUTORTTY" - RTTY PIX PROGRAM BY K8LC

A NUMBER OF FUNCTIONS, OF THE FULL PROGRAM, HAVE BEEN DISABLED IN

THIS SAMPLE VERSION "AUTOSMPL.EXE"

THE  FOLLOWING  MANUAL  IS  THE  COMPLETE  MANUAL  PROVIDED  WITH

"AUTORTTY"

THE MANUAL FOR THE "AUTO-PIX", AUTOMATED "PIXBOX" MODULE, HAS NOT

BEEN PROVIDED IN THIS DOCUMENTATION

==============  CONTENTS ==============

INTRODUCTION ------------------------------------- SEC 1  P --- 1

SERIAL PORT INFORMATION -------------------------- SEC 1  P --- 2

PROGRAM INSTALLATION ----------------------------- SEC 1  P --- 3

PROGRAM CONFIGURATION ---------------------------- SEC 1  P - 4-6

CREATING AND EDITING MESSAGE PORTS --------------- SEC 1  P --- 7

"FUNNY" CHARACTERS IN COL 80 --------------------- SEC 1  P --- 8

RECEIVE MODE (INFO) ------------------------------ SEC 1  P --- 9

TYPE AHEAD BUFFER USE ---------------------------- SEC 1  P --- 8

"BREAK" TRANSMIT MODE OPERATION ------------------ SEC 1  P --- 8

SEC 2  P --- 1

AUTO-SAVE (RECV TEXT TO DISK) MODE --------------- SEC 1  P -- 11

BLOCK MODE OPERATION ----------------------------- SEC 1  P -- 11

KEYS  ------- SEC 2  P - 6-7

PRINT MODE USE ----------------------------------- SEC 1  P -- 12

(TO HAVE INCOMING TEXT ECHO TO PRINTER - SEE PRINT ECHO PAGE III)

RTTY BAUD RATE AND MODE (IN BAUD INSTEAD OF WPM) - SEC 1  P -- 12

SEC 2  P --- 5

TOGGLING BETWEEN BAUDOT AND ASCII MODE ----------- SEC 1  P -- 12

SEC 2  P --- 5

DATE/TIME INPUT FOR XMIT FROM TYPE AHEAD --------- SEC 1  P -- 12

SEC 2  P --- 6

SENDING FILES ENTERED IN THE TYPE AHEAD BUFFER --- SEC 1  P -- 13

(BEST EXPLAINED IN SECTION 2)   SEC 2  P - 5-6

LOADING AND SENDING THE PIX XMIT BUFFER ---------- SEC 1  P -- 13

SEC 2  P --- 2

EDITING TRANSMIT (PIX) BUFFER -------------------- SEC 1  P 14-17

PREPARING AND SENDING PART (FILL) OF A PIX FILE -- SEC 1  P -- 13

SEC 2  P --- 3

USING "SHELL" MODE (TEMPORARY PRG EXIT TO "DOS") - SEC 1  P -- 12

SEC 2  P --- 4

EXIT THE AUTORTTY PROGRAM ------------------------ SEC 1  P -- 13

SEC 2  P --- 5

CORRECTING A MESSED UP SCREEN (SEC-1 P-9) -------- SEC 1  P --- 9

<ALT> F1 - SCREEN REDRAW)              SEC 2  P --- 1

AUTO-PIX MODE - ALL INFO FOR THIS MODE IS IN SECTION 3 OF MANUALS

(SECTION 3 HAS ITS OWN CONTENTS)

(I)

====== LIST OF FUNCTIONS AND THEIR KEY ASSIGNMENTS ======

=================================================================

FUNCTION --------------------- ASSIGNED KEY/S ---- LOCATION -----

=================================================================

HELP/INFO ------------------------- F1 ----------- SEC 2  P --- 1

BREAK XMIT MODE -------------- <SHIFT> F1 -------- SEC 1  P --- 8

SEC 2  P --- 1

PROGRAM CONFIGURATION -------- <CTRL> F1 --------- SEC 1  P --- 4

SEC 2  P --- 1

REDRAW RECEIVE SCREEN -------- <ALT> F1 ---------- SEC 1  P --- 9

(READ NOTES IN SEC 1 PAGE 9 ABOUT REDRAW)       SEC 2  P --- 1

FORCE LETTERS IN BAUDOT RECV ---- F2 / <CTRL> R -- SEC 2  P - 1-2

BREAK (IMMEDIATE) FROM XMIT TO RECV - F2/<CTRL>R - SEC 2  P - 1-2

TYPE AHEAD TRANSMIT MODE ------ <SFT> F2/<CTL> T - SEC 1  P --- 8

SEC 2  P --- 2

LOAD XMIT (PIX) BUFFER -------------- F3 --------- SEC 1  P -- 13

SEC 2  P --- 2

SENDING PIX FROM LOADED BUFFER ----- <SHIFT> F3 -- SEC 1  P -- 13

SEC 2  P --- 2

EDITING THE TRANSMIT (PIX) BUFFER --- <CTRL> F3 -- SEC 1  P 14-17

("AUTOEDIT" MODULE)                               SEC 2  P --- 2

SENDING PIX ENTERED IN TYPE AHEAD ---- <CTRL> F -- SEC 1  P -- 12

SEC 2  P - 4-5

FILL (PART PIX) LINE ENTRY ------------ F7 ------- SEC 1  P -- 13

SEC 2  P --- 3

SENDING FILL ---------------------- <SHIFT> F7 --- SEC 1  P -- 13

SEC 2  P --- 3

SAVING RECEIVE BUFFER TO DISK ---------- F4 ------ SEC 1  P --- 9

SEC 2  P --- 2

CREATING AND EDITING MESSAGE PORTS - F5 (THEN "M") SEC 1  P --- 7

SEC 2  P - 2-3

NESTING MESSAGE PORTS ------------ <ALT> 1 - 0 --- SEC 1  P --- 7

SEC 2  P --- 8

DISPLAY OF DISK DIRECTORY ------- F5 (THEN "D") -- SEC 1  P -- 10

EDIT/VIEW INFORMATION ------------------ F5 ------ SEC 1  P -- 10

SEC 2  P --- 2

BLOCK MODE (MARKING AND OPTIONS) ------- F8 ------ SEC 1  P -- 11

SEC 2  P --- 3

(CONTINUED ON NEXT PAGE)

(II)

===== LIST OF FUNCTIONS AND THEIR KEY ASSIGNMENTS (CONT.) ======

=================================================================

FUNCTION --------------------- ASSIGNED KEY/S ---- LOCATION -----

=================================================================

PRINTING MODE (NOT PRINT ECHO) --------- F9 ------ SEC 1  P -- 12

SEC 2  P --- 3

PRINT ECHO (RX ALSO GOING TO PRN) ---- <CTRL> P -- SEC 1  P --- 9

SEC 2  P --- 5

AUTO-SAVE (RX ALSO GOING TO DISK) ---- <CTRL> F9 - SEC 1  P -- 11

SEC 2  P --- 3

CLOSING AUTO-SAVE --------------------- <ALT> F9 - SEC 1  P -- 11

SEC 2  P --- 4

AUTO-PIX (AUTO-PIX VER. PROGRAM) ------- F10 ----- SEC 2  P --- 4

(AUTO-PIX INFO IN SECTION 3 - HAS ITS OWN CONTENTS)SEC 3  ( ALL )

AUTO-PIX (PRINT LOGFILE) (A-P ONLY) - <SHIFT> F10 -SEC 2  P --- 4

(AUTO-PIX INFO IN SECTION 3 - HAS ITS OWN CONTENTS)SEC 3  ( ALL )

EXIT PROGRAM (TEMPORARY) TO DOS ------ <ALT> F10 - SEC 1  P -- 12

SEC 2  P --- 4

EXIT PROGRAM (FINAL) --------- <CTRL> O-<CTRL> U - SEC 1  P -- 13

SEC 2  P --- 5

BAUD RATE (CHANGE) ------------------ <CTRL> B --- SEC 1  P -- 12

SEC 2  P --- 5

RTTY MODE (CHANGE BAUDOT-ASCII) ----- <CTRL> V --- SEC 1  P -- 12

SEC 2  P --- 5

ASCII (TOGGLE 8 BIT/7 BIT) ---------- <CTRL> A --- SEC 1  P -- 12

SEC 2  P --- 5

PIX SENDING FROM TYPE AHEAD ENTRY --- <CTRL> F --- SEC 1  P -- 13

(BEST EXPLAINED IN SECTION 2)  SEC 2  P - 5-6

"BELL" CHARACTER (SENDING) ---------- <CTRL> G --- SEC 2  P --- 6

DATE/TIME (SENDING) ----------------- <CTRL> D --- SEC 2  P --- 6

CWID (SENDING) ---------------------- <CTRL> I --- SEC 2  P --- 6

CLEAR A LINE FROM TYPE AHEAD OR MSG - <CTRL> Y --- SEC 2  P --- 6

MARKING THE END OF YOUR TYPE ENTRY -- <CTRL> E --- SEC 1  P --- 8

(USE THIS AT THE VERY END OF YOUR CURRENT XMISSION)SEC 2  P --- 6

ADDING A CALL TO MESSAGE PORT #1 ----- <ALT> A --- SEC 1  P --- 7

SEC 2  P --- 7

REMOVING A CALL FROM MESSAGE PORT #1 - <ALT> R --- SEC 1  P --- 7

SEC 2  P --- 7

(CONTINUED ON NEXT PAGE)

(III)

===== LIST OF FUNCTIONS AND THEIR KEY ASSIGNMENTS (CONT.) ======

=================================================================

FUNCTION --------------------- ASSIGNED KEY/S ---- LOCATION -----

=================================================================

CLEARING ALL CALLS FROM MSG PORT #1 --- <ALT> C -- SEC 1  P --- 7

SEC 2  P --- 7

==================================================================

KEYS USED IN  "AUTOEDIT" ARE GIVEN ON PAGES 14 - 17 OF THIS MANUAL

==================================================================

= THE FOLLOWING ARE KEYS USED WITHIN EDIT/VIEW AND BLOCK MODES =

^Q/HOME - ^Z/END - ^W/PAGEUP - ^X/PAGEDOWN ------- SEC 2  P - 6-7

======== THE FOLLOWING ARE KEYS USED WITHIN BLOCK MODE ========

<CTRL> B - <CTRL> E - ALL <ARROW KEYS> ----------- SEC 2  P - 6-7

(IV)

AUTORTTY/AUTO-PIX

INFORMATION  MANUAL

SECTION 1

============= INTRODUCTION TO AUTORTTY/AUTO-PIX ==============

THIS  PROGRAM  WAS DEVELOPED, FROM THE GROUND UP,  FOR  RTTY  PIX

ENTHUSIASTS.   MOST  PROGRAMS  WRITTEN IN THE  PAST  HAVE  EITHER

IGNORED RTTY PIX ALTOGETHER, OR HAVE ONLY COVERED PART, OF  MANY,

SPECIAL  DEMANDS  RTTY PIX SENDING, AND RECEIVING,  IMPOSE  ON  A

PROGRAM.  AUTORTTY HANDLES THE SPECIAL NEEDS FOR OVERLINED PIX  -

AND  THE  ABILITY TO "EXPLODE" THOSE LINES IN  A  PRINTOUT.   THE

PROGRAM WILL NOT "HANG-UP" IF YOUR PRINTER RUNS OUT OF PAPER, YOU

TAKE  IT  OFF LINE OR ANY ERROR CONDITION THAT MAY  OCCUR.   THIS

PROGRAM NOT ONLY TAKES OUT ANY EXTRA CARRIAGE RETURNS SENT WITH A

SINGLE LINEFEED, IT REMOVES ALL UNNECESSARY CARRIAGE RETURNS.  IF

YOU  RECEIVE 10 CR/LF PAIRS - THIS PROGRAM WILL STORE ONE CR  AND

10  LFS.  YOU DO HAVE THE AN OPTION WHICH WILL RETAIN  CR'S  SENT

WITH  MULITPLE CR/LF PAIRS, IF THEY ARE TRANSMITTED.   WHEN  THIS

PROGRAM  SENDS  A  PIX FILE - IT ALWAYS  SENDS  A  THREE  CONTROL

CHARACTER  END-OF-LINE  SEQUENCE.   ON  A  FULL  "NEW-LINE"  THIS

PROGRAM SENDS (1) CR/LF PAIR - THEN THE CONTROL CHARACTER OF  THE

FIRST  PRINTABLE CHARACTER IN THE LINE (A LTRS FOR A  <SPACE>  OR

OTHER "LETTER" CHARACTER  - A FIGS FOR A "FIGURE" CHARACTER).  ON

AN  OVERLINE,  THIS PROGRAM SENDS TWO CARRIAGE  RETURNS  AND  THE

CONTROL  CHARACTER OF THE FIRST PRINTABLE CHARACTER IN THE  LINE,

AS  MENTIONED ABOVE.  THIS WILL KEEP THE OLD MECHANICAL  MACHINES

FROM STAYING IN FIGS, IF THEY GET OUT OF STEP WITH THE PIX  BEING

SENT,  WHICH CONTAIN  ONLY "LETTERS" CHARACTERS.  IF YOU  DO  GET

OUT OF STEP, IT WILL ONLY BE FOR THE REMAINDER OF THAT LINE.

TRANSMIT (PIX) BUFFER EDITING IS ALSO AVAILABLE IN BOTH  AUTORTTY

AND THE AUTORTTY/AUTO-PIX COMBO PROGRAMS.  THE "AUTOEDIT"  MODULE

CONTAINS  ALL OF THE FUNTIONS NECESSARY FOR EDITING AND  CREATING

RTTY PIX - OR ANYTHING ELSE THAT REQUIRES AN ASCII TEXT EDITOR.

THIS  PROGRAM IS NOT ONLY A FINE PROGRAM FOR RTTY PIX - IT  IS  A

VERY NICE PROGRAM TO USE FOR GENERAL RAG-CHEWING QSO'S.  AUTORTTY

CONTAINS  MOST OF THE MAJOR FEATURES OF OTHER RTTY  PROGRAMS  AND

MANY THE OTHERS DON'T.  THERE ARE FUNCTIONS THIS PROGRAM  DOESN'T

INCLUDE SUCH AS CW AND COMMANDS FOR A "SMART" INTERFACE (TU), BUT

FOR THE AVID PIX LOVER THIS PROGRAM SHOULD FILL MANY VOIDS OTHERS

LEAVE.   THERE ARE MANY OTHER FEATURES AVAILABLE  WITH  AUTORTTY,

THOSE ARE BEST LEARNED BY ACTUALLY USING THE PROGRAM AND  READING

ABOUT THEM IN THE REST OF THIS MANUAL.

IF YOU PURCHASED THE FULL AUTORTTY/AUTO-PIX PROGRAM, YOU HAVE THE

FINEST AUTOMATED "PIXBOX" PROGRAM AVAILABLE TODAY - BAR NONE...

PLEASE NOTE:

YOU DO NOT HAVE TO REGISTER YOUR NEW AUTORTTY (OR  AUTORTTY/AUTO-

PIX  COMBO) PROGRAM.  I HAVE RETAINED ALL  INFORMATION  REGARDING

YOUR NAME, AMATEUR CALL, FULL ADDRESS, VERSION NUMBER AND DATE OF

YOUR PROGRAM.

HAPPY PIX'N

WALTER E. BESS, JR.,  K8LC

PRECISION ELECTRONICS

1525 KARIN STREET - P.O BOX 764

IRONTON, OHIO   45638

PHONE: 1-614-532-6111

1

========= INTERFACE (SERIAL) PORT INFO FOR AUTORTTY ==========

THIS   PROGRAM   SUPPORTS  EITHER  COM1 OR COM2 AS   THE   SERIAL

PORT  FOR CONNECTION TO YOUR INTERFACE (TU).  (DEFAULT IS COM2)

IMPORTANT NOTE:  THIS PROGRAM DOES -NOT- SUPPORT SPECIAL COMMANDS

REQUIRED  BY MOST "SMART" TERMINALS.  UNLESS  YOUR  "INTELLIGENT"

TERMINAL  UNIT  HAS  A  WAY  TO  PASS  DATA  BOTH  WAYS,  WITHOUT

MODIFICATION,  AND  YOU CAN USE THE THE RS-232  LINES  AS  LISTED

BELOW, THIS PROGRAM MAY GIVE UNSATISFACTORY RESULTS.

PINOUTS FOR YOUR RS-232 ARE AS FOLLOWS:

DB-9 PLUG       DB-25 PLUG

INPUT FROM RECEIVER -------------------    2    -------     3

OUTPUT TO TRANSMITTER -----------------    3    -------     2

SIGNAL GROUND -------------------------    5    -------     7

PTT (PUSH TO TALK LINE) ---------------    7    -------     4

CWID LINE -----------------------------    4    -------    20

IF  YOU  ENCOUNTER PROBLEMS OF SIGNALS BEING  REVERSED  (SUCH  AS

HAVING   THE XMITTER  KEYED WHEN THE PROGRAM IS SHOWING  RECEIVE)

YOU  CAN   INVERT   THE SIGNAL  VOLTAGE BY ADDING  EITHER  AN  IC

INVERTER OR A  SIMPLE  TRANSISTOR CIRCUIT.  IF YOU HAVE  PROBLEMS

ALONG  THESE LINES - CONTACT ME -  I  HAVE SEVERAL  CIRCUITS  YOU

MAY BE ABLE TO USE.  IF YOU ARE USING  A  COMPUTER WITH 2  SERIAL

PORTS  THAT  USE THE DB-25 PIN CONNECTOR, YOU  MAY  EXPERIENCE  A

PROBLEM OF THE INPUT AND OUTPUT LINES BEING REVERSED.  THEY   USE

TWO  FORMATS   FOR  THIS PLUG (MAY ALSO EXIST WITH THE  9  PIN  -

ALTHOUGH  I   HAVE  NOT ENCOUNTERED IT), THEY  USUALLY  HAVE  ONE

FEMALE  AND  ONE MALE CONNECTOR FOR THEM.  THE  TWO  FORMATS  ARE

"DCE"  (DATA COMMUNICATIONS EQUIPMENT) AND "DTE"  (DATA  TERMINAL

EQUIPMENT).   THE  ONLY SOLUTION HERE IS TO OPEN YOUR  CABLE  (OR

CONNECTOR) AND ACTUALLY SWITCH THE WIRES ON PINS 2 AND 3.

IF YOUR TERMINAL UNIT DOES NOT SUPPORT "CW ID" YOU WILL  PROBABLY

HAVE TO BUILD A SMALL (SIMPLE) CIRCUIT TO HAVE THE CWID LINE  KEY

YOUR  SPACE TONE, OR SPACE SHIFT (IF YOU ARE USING FSK).   I  CAN

SUPPLY  YOU  WITH SEVERAL CIRCUITS THAT SHOULD TAKE CARE  OF  THE

PROBLEM, IF NECESSARY.

2

============== INSTALLATION ==============

FIRST,   MAKE  A  BACKUP COPY OF THE ORIGINAL PROGRAM  DISK   AND

PUT  THE ORIGINAL IN A SAFE PLACE.  FOLLOW INSTRUCTIONS GIVEN  IN

THE "DOS" MANUAL REGARDING YOUR COPY -OR- DISKCOPY COMMANDS.

IMPORTANT NOTE:  IF YOU ARE USING A COMPOSITE VIDEO  (MONOCHROME)

MONITOR  ON  YOU COLOR VIDEO ADAPTER, YOU -MUST- USE  THE   /COMP

COMMAND  LINE  SWITCH WHEN YOU WANT TO BRING UP  AUTORTTY.   THIS

WILL  ENABLE  THE  PROGRAM  TO SELECT  THE  PROPER  VIDEO  MEMORY

ATTRIBUTES  FOR  A MONOCHROME MONITOR AND STILL USE  THE  "COLOR"

VIDEO MEMORY AREA.  THIS SWITCH WILL BE IN ADDITION TO THE  /SNOW

SWITCH (EXPLAINED BELOW).  IF YOU ONLY NEED THE COMPOSITE  SWITCH

YOUR  COMMAND LINE IS:  AUTORTTY /COMP<RETURN>.  IF  YOU  REQUIRE

BOTH:  AUTORTTY /SNOW /COMP<RETURN>.  THE SWITCHES NEED NOT BE IN

THIS ORDER.

TO   INSTALL,  JUST  COPY  THE  "AUTORTTY.EXE"  FILE   INTO   THE

SUBDIRECTORY  YOU WANT  TO  RUN THE PROGRAM FROM.  THEN (WHEN  IN

THE   AUTORTTY   DIRECTORY) ENTER:  AUTORTTY<RETURN>,  THIS  WILL

BRING-UP YOUR NEW AUTORTTY  PROGRAM.  AUTORTTY COMES UP WITH CAPS

LOCK = ON, YOU DON'T HAVE TO USE A UTILITY FOR THAT PURPOSE.  THE

FIRST  TIME THE PROGRAM IS "BOOTED" IT CREATES TWO  DISKFILES  IN

YOUR DEFAULT DIRECTORY PATH (THE PATH YOU SHOULD HAVE THE PROGRAM

IN),  ONE  IS CALLED  "RTYCONFG.DAT"  AND THE  SECOND  IS  CALLED

"MSG-PORTS.DAT".   THE  FIRST   FILE WILL  CONTAIN  A  NUMBER  OF

DEFAULTS  FOR THE  PROGRAM  OPTIONS, THE  SECOND HAS ALL  OF  THE

INFO FOR YOUR 9 (USER  CONFIGURABLE) MESSAGE PORTS.

IF YOU HAVE AN EARLY CGA VIDEO ADAPTER AND ARE EXPERIENCING SNOW,

OR OTHER INTERFERENCE, ON YOUR MONITOR - YOU WILL HAVE TO USE THE

/SNOW   COMMAND LINE SWITCH WHENEVER YOU BRING-UP AUTORTTY.   USE

THE  FOLLOWING,  IF  THE /SNOW  SWITCH  IS  NECESSARY:   AUTORTTY

/SNOW<RETURN>.  SEE ABOVE REGARDING A COMPOSITE VIDEO MONITOR (IF

IT APPLIES TO YOU).

NOTE:  YOU  DO  NOT  HAVE  TO  REGISTER  YOUR  NEW  AUTORTTY   OR

AUTORTTY/AUTO-PIX  PROGRAM.   I HAVE ALREADY FILED  ALL  OF  YOUR

PROGRAM  INFORMATION, AS WELL AS YOUR NAME, FULL ADDRESS AND  HAM

CALL.   IF  YOU  CHANGE YOUR AMATEUR CALL - I  WILL  UPDATE  YOUR

PROGRAM  AT NO CHARGE (IF YOU HAVE THE CURRENT VERSION).  IF  YOU

HAVE  A PREVIOUS VERSION, YOU WILL BE CHARGED THE  UPDATE  CHARGE

AND RECEIVE THE LATEST VERSION WITH YOUR NEW CALL SIGN IN IT.

3

============== CONFIGURATION ==============

PRESS,   AND   HOLD,  THE  <CTRL> KEY  WHILE  YOU  PRESS  THE  F1

FUNCTION   KEY,  THIS BRINGS UP THE FIRST OF  FIVE  CONFIGURATION

SCREENS.

NOTE: CONFIGURATION SCREENS #4 AND #5 ARE FOR USE WITH "AUTO-PIX"

MODULE ONLY.  IF YOU PURCHASED THE BASIC "AUTORTTY" PROGRAM,  YOU

DO NOT GIVE INPUT ON SCREENS 4 AND 5.

YOU SHOULD RUN THROUGH ALL OF THE CONFIGURATION THAT PERTAINS  TO

YOUR  PREFERENCE  AND HARDWARE.  YOU SHOULD GO THROUGH  ALL  FIVE

SCREENS (USING PAGEUP OR PAGEDOWN) AND SELECT THE THE OPTIONS YOU

WANT FROM  THE "TOGGLE"  OPTIONS ON THE FIRST TWO SCREENS.  INPUT

YOUR   PREFERENCE  FOR THE VARIOUS "PATH" OPTIONS, ON  SCREENS  3

AND  4.   SCREEN 5 HAS TWO INPUTS FOR  OPTIONAL  MESSAGES  TO  BE

SENT  DURING WELCOME AND EXIT  ROUTINES  IN AUTO-PIX  MODE.    IF

YOU DON'T WANT TO USE A MESSAGE,  JUST  LEAVE  THEM BLANK.

THE FOLLOWING IS A LIST OF CONFIGURATION OPTIONS.

1.   BAUD RATE USED FOR PROGRAM  STARTUP - SELECT THE  BAUD  RATE

THAT  YOU WANT THE PROGRAM TO USE WHEN FIRST "BOOTED".

2.   MODE USED FOR PROGRAM STARTUP - SELECT THE RTTY MODE  TO  BE

USED WITH ABOVE BAUD RATE AT STARTUP.

3.   YOUR TIME ZONE - SELECT THE TIME ZONE YOU LIVE IN,  THIS  IS

USED   FOR SENDING  DATE/TIME  IN TYPE TRANSMIT AND WITH THE  LOG

ON/OFF  INFO  FOR AUTO-PIX.

4.  TYPE OF TIME - SELECT EITHER DAYLITE-SAVING OR STANDARD.

5.  RETAIN  A CARRIAGE RETURN WITH EACH LINE-FEED  RECEIVED   (IF

SENT).  THIS  PROGRAM  DEFAULTS TO  REMOVING  "ALL"  UN-NECESSARY

CARRIAGE  RETURNS RECEIVED.  WHAT THIS MEANS IS:  IF YOU  RECEIVE

CR/CR/CR/LF  -  THE PROGRAM SAVES  ONLY -ONE- CARRIAGE  RETURN  -

CR/LF.    BY  THE  SAME TOKEN,  IF   THE  PROGRAM   RECEIVES  TEN

CARRIAGE  RETURN   LINE-FEED PAIRS:  10 CR/LF'S, IT  DEFAULTS  TO

SAVE ONLY THE FIRST CARRIAGE RETURN AND THEN  10 LINE-FEEDS.

IF  YOU PREFER TO SAVE A CARRIAGE RETURN WITH EACH   LINE-FEED  -

PRESS THE SPACE BAR TO TOGGLE YOUR SELECTION FROM "NO" TO "YES".

6.   SERIAL  PORT USED FOR INTERFACE - SELECT THE  SERIAL   (COM)

PORT   YOU ARE  USING WITH YOUR RTTY INTERFACE  (TERMINAL  UNIT).

EITHER COM-1 OR COM-2 IS SUPPORTED.

7.   PRINTER  PORT - SELECT THE PORT YOU WANT ANY PRINTER  OUTPUT

TO   GO.  THIS  IS  USUALLY  LPT1  (FOR PARALLEL PRINTERS).    IF

YOU  HAVE  OTHER PARALLEL PRINTERS AND WANT TO USE ONE OF THOSE -

JUST  SELECT THE  PROPER PORT.   IF  YOU HAVE A  SERIAL  PRINTER,

SELECT THE COM PORT YOU HAVE IT CONNECTED TO.

IMPORTANT:   DO  -NOT-  HAVE THE SAME "SERIAL" PORT SELECTED  FOR

BOTH  THE  RTTY INTERFACE (ITEM 6 ABOVE) AND A  "SERIAL"  PRINTER

(ITEM 7 ABOVE).

4

8.  HAVE  BORDER AROUND DISPLAY SCREEN -  (FOR EGA AND VGA  ONLY)

-   THE  PROGRAM FIRST BOOTS WITH A BORDER - FOR  THOSE  DISPLAYS

THAT  WILL SUPPORT IT.  IF YOU PREFER NOT TO HAVE A BORDER,  JUST

PRESS <SPACE> TO TOGGLE IT TO "NO".

NOTE:    WHEN  USING BORDER ON - SOME  SCREEN  SAVERS  (BLANKERS)

WILL  ONLY  REMOVE  THE  VISIBLE  PROGRAM FROM THE  DISPLAY   AND

LEAVES THE SCREEN ENTIRELY A DIM WHITE (GRAY).  IF YOU HAVE  SUCH

A SCREEN SAVER - YOU WILL HAVE TO SET THE BORDER TO OFF TO  BLANK

THE  SCREEN.   SOME SCREEN SAVERS WILL  NOT  PERMIT  TRANSMITTING

WHILE BLANKED.

9.   DISPLAY OPTIONS MENU FOR SHIFT KEY - AS YOU  PROBABLY   HAVE

NOTICED,  THIS  PROGRAM DISPLAYS THE OPTIONS ASSOCIATED WITH  THE

UNSHIFTED  STATE, <SHIFT>, <CTRL> AND <ALT> KEYS.  IF YOU USE THE

SHIFT  KEY A LOT AND ARE DISTRACTED  BY THE CHANGING STATUS  LINE

(LINE  25),  YOU  CAN  CHANGE THIS OPTION TO  "NO"  -  THIS  WILL

DISABLE THE CHANGE FOR THE <SHIFT> KEY.

10.  ECHO  TYPED CHARACTERS  AND  MESSAGE PORTS TO RECV BUFFER  -

MANY   PEOPLE  LIKE TO HAVE  WHAT THEY TYPE TO BE ECHOED  TO  THE

RECEIVE BUFFER AND RECEIVE SCREEN.  IF YOU WANT THIS, TOGGLE  THE

SELECTION TO "YES".

THE   ABOVE OPTIONS ARE ALL "TOGGLE" SELECTIONS.  THIS MEANS  YOU

SET  THE CURSOR  ON THE LINE YOU WANT TO CHANGE AND PRESS <SPACE>

KEY TO TOGGLE THROUGH YOUR CHOICES.

THE FOLLOWING ARE OPTIONS WHICH REQUIRE INPUT FROM YOU.

11.   DISK  TO XMIT BUFF PATH - THIS WILL BE THE  "DEFAULT"  PATH

USED   WHEN  YOU  WANT TO LOAD THE XMIT (PIX) BUFFER  FROM  DISK.

YOU  ARE NOT  "BOUND" BY THIS SELECTION - AFTER PRESSING  F3  AND

GETTING  THE  PROMPT ON LINE 17, YOU  CAN USE THE  BACKSPACE  (OR

DELETE)  KEY TO BACKUP AND  ENTER  ANOTHER PATH, WHEN  NECESSARY,

THEN THE DISKFILE NAME.

NOTE: YOU CAN USE WILDCARDS WITHIN THE FILENAME YOU INPUT ABOVE.

12.   RECV   BUFF  TO  DISK  PATH -  AGAIN,   THIS  WILL  BE  THE

"DEFAULT"  PATH DISPLAYED WHEN YOU PRESS F4 TO SAVE  THE  RECEIVE

BUFFER TO DISK.  YOU CAN BACKSPACE AND ENTER ANOTHER IF YOU WISH.

NO WILDCARDS HERE - PLEASE.

13.   AUTO_SAVE  DRIVE PATH - THIS WILL BE THE   "DEFAULT"   PATH

DISPLAYED  WHEN  YOU PRESS <CTRL> F9 (AUTO_SAVE MODE) - TO  ENTER

THE  FILENAME  USED TO SAVE ALL RECEIVED TEXT TO.  AGAIN YOU  CAN

BACKUP AND CHANGE IT.

14.   DIRECTORY  SEARCH PATH - THIS IS THE  "DEFAULT"  PATH  USED

WHEN  YOU  ENTER F5 (EDIT/VIEW MODE) AND SELECT "D"  FOR  DISPLAY

DIRECTORY.  IT CAN BE CHANGED TO.

15.   DELETE  DISK  FILE PATH - THE "DEFAULT" PATH  USED  IN  THE

PROMPT  WHEN  YOU SELECT <CTRL> F6 (DELETE FILE MODE).   THIS  IS

CHANGEABLE ALSO.

= THE FOLLOWING ARE FOR USE WITH  AUTO-PIX PROGRAM VERSION ONLY =

====== STARTING ON NEXT PAGE ======

= READ INFORMATION FOLLOWING THE AUTO-PIX CONFIGURE OPTIONS =

5

= THE FOLLOWING ARE FOR USE WITH  AUTO-PIX PROGRAM VERSION ONLY =

16.   AUTO_PIX ON ERROR PATH - THIS  WILL BE THE PATH THAT   WILL

CONTAIN FILES  THAT  FOR  SOME REASON COULDN'T BE SAVED IN  THEIR

PROPER  PATH, DURING AUTO-PIX MODE.  THIS CAN BE CAUSED BY A DISK

FULL  ERROR  OR  OTHER ERROR  CONDITION.  AT THIS TIME  ONLY  THE

LOGFILE SAVE USES THIS PATH IN AN EMERGENCY.

17.  AUTO_PIX LOGFILE PATH - THIS IS THE PATH USED WHEN  AUTO-PIX

SAVES LOGGING  INFORMATION  TO DISK - FOR SYSTEM USERS.  ALL   OF

THE  LOGFILE DISK FILENAMES WILL HAVE THE FORM:   MM-DD-YY     NO

EXTENSION.

18.   AUTO_PIX  USER_MSG  PATH  - THIS   IS  THE  PATH  ANY  USER

MESSAGES, SENT TO YOU, WILL BE STORED UNDER.

19.   AUTO_PIX  XMIT DRV PATH - THIS IS THE PATH  WHICH  AUTO-PIX

WILL USE TO ACCESS THE "PIX" FILES SELECTED BY YOUR SYSTEM USERS.

IT  CAN  BE JUST A FLOPPY  DRIVE  SUCH  AS   B:\ - OR  AN  ENTIRE

PATH  TO  YOUR HARD DRIVE SUBDIRECTORY WHERE YOU  WANT  USERS  TO

ACCESS LIKE  E:\XMAS-PIX\.

20.   AUTO_PIX CALL-UP CODE - YOU ENTER THE SUFFIX YOU WANT  USED

FOR  YOUR AUTO-PIX CALL-UP CODE - ALL - AUTO-PIX PROGRAM  CALL-UP

CODES WILL BEGIN WITH "PIX" - LIKE MINE - PIX8LC.

21.   SPECIAL OR HOLIDAY MESSAGE TO BE SENT DURING THE  "WELCOME"

MESSAGE  IN AUTO-PIX.  LEAVE COMPLETELY BLANK IF YOU DO NOT  WANT

ANYTHING SENT.

22.   SPECIAL OR HOLIDAY MESSAGE TO BE SENT DURING THE   SIGN-OFF

(EXIT) MESSAGE IN AUTO-PIX.  LEAVE COMPLETELY BLANK IF YOU DO NOT

WANT ANYTHING SENT.

ALL   OF   THE USER INPUT OPTIONS CAN BE CHANGED  BY  RE-ENTERING

THEM  AND  EDITING  -  THEN PRESS <RETURN> TO  GO  BACK  TO  LINE

SELECTION.

WHEN   YOU ARE FINISHED WITH YOUR CONFIGURATION,  PRESS  <RETURN>

AND   YOUR  SELECTIONS  WILL BE SAVED TO DISK.   YOU'LL  THEN  BE

ASKED  IF  YOU  WANT TO USE THE OPTIONS YOU  JUST  SELECTED,  YOU

PROBABLY  WILL WANT TO USE THEM - SO PRESS "Y" TO  INDICATE  YES.

UNLESS  YOU CHANGE YOUR SYSTEM IN SOME WAY OR  YOUR TIME  CHANGES

(DAYLITE/STANDARD)  - THIS SHOULD BE THE ONLY TIME YOU WILL  NEED

TO  RUN  THE  "COMPLETE"  CONFIGURATION.   THE  VALUES  YOU  HAVE

SELECTED  WERE  SAVED TO DISK AND WILL BE INPUT INTO  THE  PROPER

VARIABLES  EACH  TIME THE PROGRAM STARTS-UP. YOU DO NOT  HAVE  TO

CHANGE  EVERYTHING  WHEN  YOU  MODIFY  YOUR  CONFIGURATION,  JUST

CHANGE THE ONE OR TWO ITEMS THAT  NEED TO BE CHANGED, THEN  PRESS

<RETURN> AND THOSE CHANGES WILL BE SAVED  TO  DISK.   IF YOU WANT

TO  HAVE  A CHANGE ONLY USED DURING THE  CURRENT  SESSION,  PRESS

ESCAPE   KEY   <ESC>  INSTEAD  OF  <RETURN>,   THIS   EXITS   THE

CONFIGURATION  SCREENS  AND  ASKS WHETHER YOU  WANT  TO  USE  THE

CHANGES YOU  MADE DURING THE CURRENT SESSION - JUST PRESS "Y" AND

YOU WILL USE THOSE CHANGES ONLY DURING THE CURRENT SESSION.

6

=========== CREATING AND EDITING MESSAGE PORTS ===========

PRESS   F5,  THIS WILL BRING UP A MENU FOR THE  EDIT/VIEW   MODE.

SELECT THE EDIT MESSAGE PORTS OPTION, BY PRESSING "M".  YOU  WILL

THEN  BE PROMPTED FOR THE MESSAGE  PORT NUMBER.  YOU USE  ANY  OF

THE  NUMBER KEYS LOCATED ABOVE THE NORMAL ALPHABET KEYS (NOT  THE

ONES ON YOUR KEYPAD).  IT WILL DISPLAY A WINDOW FOR YOU TO  INPUT

YOUR  INFO  INTO  THAT MESSAGE  PORT.   WHEN  YOU  FINISH  EITHER

CREATING OR EDITING THAT MESSAGE PORT, PRESS <ESC> KEY, YOU  WILL

BE  ASKED IF YOU WANT THOSE CHANGES SAVED TO DISK - YOU  PROBABLY

WILL  - SO PRESS "Y" FOR YES.  FOLLOW THE SAME PROCESS TO  CREATE

OR EDIT ANY MESSAGE PORTS YOU WILL BE USING.

YOU   CAN   ALSO  NEST  A ANOTHER MESSAGE PORT CALL  WITHIN  YOUR

INPUT  TO THE CURRENT MESSAGE PORT.  YOU  INPUT A NESTED  MESSAGE

PORT BY PRESSING, AND HOLDING, THE  <ALT> KEY WHILE PRESSING  ANY

OF THE MESSAGE PORT NUMBER KEYS. (ONES ABOVE THE NORMAL  "LETTER"

KEYS - NOT THE KEYPAD)

THIS VERSION  OF "AUTORTTY"  SUPPORTS ONLY A "BACKSPACE" TYPE  OF

EDITING.   THE DELETE  KEY HAS THE SAME ACTION AS  THE  BACKSPACE

WHEN EDITING YOUR TYPE AHEAD AND MESSAGE PORTS.

PLEASE NOTE:  THE TRANSMIT (PIX) BUFFER EDITING SUPPORTS COMPLETE

EDITING USING ALL EDITING AND ARROW KEYS.

YOU CAN DELETE A WHOLE LINE BY PRESSING THE <CTRL> AND  "Y"   KEY

COMBINATION.    TO   INPUT  A MESSAGE PORT  IN  THE  TYPE   AHEAD

BUFFER,   YOU PRESS (AND HOLD) THE <ALT> KEY WHILE  PRESSING  THE

NUMBER  KEY OF THE PORT YOU WANT TO INPUT.  (AGAIN - DO  NOT  USE

KEYPAD KEYS)

IMPORTANT:  NEVER HAVE A MESSAGE PORT CALL ITSELF - OR ANY  OTHER

THAT  MAKES A CALL TO A MESSAGE PORT THAT WILL CREATE  A  "LOOP".

THE MAXIMUM SIZE OF A MESSAGE PORT IS 1000 CHARACTERS.

NOTE:   MESSAGE  PORT  ZERO (0) WILL  ALWAYS  BE  INITIALIZED  TO

CONTAIN  40 RY'S AND A CR/LF PAIR.    AT PROGRAM  START-UP.   YOU

CAN EDIT PORT 0 BUT IT WILL NEVER BE SAVED  TO     DISK,  AS  THE

OTHERS ARE.

NOTE:   MESSAGE PORT #1 IS A  SPECIAL PURPOSE MESSAGE PORT,  READ

BELOW.

NESTING   A  CALL TO MESSAGE PORT #1 MAKES IT EASY  TO  CONSTRUCT

THE   SEQUENCE  OF  PORTS  CALLED  FOR  STATION  ID  SENT  DURING

EXCHANGES IN A QSO.  MESSAGE PORT #1 SHOULD BE RESERVED FOR  THIS

PURPOSE.   YOU CAN CLEAR MES-SAGE PORT  #1 BY PRESSING <ALT> C  -

IT WILL PROMPT TO VERIFY YOU WANT TO CLEAR MESSAGE PORT  #1.   TO

ADD A CALL TO MESSAGE PORT #1 - PRESS <ALT> A -  IT  WILL  PROMPT

(ON LINE 17) FOR THE CALL SIGN TO ADD, DO NOT ADD A  LEADING   OR

TRAILING   SPACE   -  ONE  TRAILING SPACE IS ADDED  BY   PROGRAM.

YOU   CAN  REMOVE A CALL SIGN FROM MESSAGE PORT  #1  BY  PRESSING

<ALT> R, YOU WILL BE PROMPTED  (AGAIN  ON  LINE 17) FOR THE  CALL

SIGN  TO REMOVE -  IF  IT  IS UNABLE  TO FIND THE CALL - IT  WILL

DISPLAY  A  MESSAGE  TO THAT  EFFECT  ON      LINE  17.   I  CALL

MESSAGE  PORT #1 FROM WITHIN MESSAGE PORT #2 - THEN  ADD      "DE

K8LC,  WALT,  IRONTON, OHIO"(CR/LF).

(CONTINUED ON NEXT PAGE)

7

THIS  WAY  YOU ONLY HAVE TO  GIVE     MESSAGE  PORT  #2   IN  THE

TYPE  AHEAD BUFFER.  WHEN MESSAGE  PORT  #2  IS      CALLED,  THE

FIRST  THING  IT DOES IS SEND MESSAGE PORT #1 -  THEN  SENDS  THE

REMAINDER OF MESSAGE PORT #2.

IMPORTANT  NOTE:  ALTHOUGH AUTORTTY SAVES, AND USES, THE  DEFAULT

PATH DURING PROGRAM START-UP AND WHEN RETURNING FROM A "SHELL" TO

YOUR  DOS, IT MAY BE SAFER TO RETURN TO THE RTTY PROGRAM  DEFAULT

DIRECTORY  BEFORE  ENTERING:   EXIT<RETURN>   TO  RETURN  TO  THE

PROGRAM.

======= WHAT ARE THOSE FUNNY CHARACTERS IN COLUMN 80? =======

A   NOTE   ABOUT  THOSE  "FUNNY" CHARACTERS IN COLUMN   80:    AS

YOU   HAVE PROBABLY NOTICED BY NOW, I DO NOT DISPLAY THE  END  OF

LINE  SEQUENCE  RIGHT AFTER ANY TEXT YOU HAVE RECEIVED  OR  TYPED

INTO  VARIOUS  BUFFERS.   I  HAVE  ALWAYS  USED,  AND  RECOMMEND,

"WORDSTAR BY MICROPRO CORP." AS A FIRST RATE PIX FILE EDITOR, AND

HAVE  TRIED  TO KEEP THE SCREEN LOOKING  THE  SAME, FOR  EASE  OF

USE.    IT   ALSO  MAKES  FOR A  "CLEAN"   PIX   WHEN   YOU   ARE

RECEIVING  ONE.  JUST REMEMBER A "-" INDICATES A CARRIAGE RETURN,

A "J" INDICATES  A LINEFEED AND A "<" INDICATES A CARRIAGE RETURN

/ LINEFEED PAIR.

=========== TYPING INTO THE XMIT TYPE AHEAD BUFFER ===========

YOU   ENTER   TEXT  TO TRANSMIT IN THE SAME  FORM  YOU  USED   TO

CREATE YOUR MESSAGE PORTS.  IT SUPPORTS THE SAME "BACKSPACE" TYPE

EDITING AND WHOLE LINE DELETE WITH THE <CTRL> Y  KEY COMBINATION.

THIS  TYPE  AHEAD  BUFFER  HAS   A   MAXIMUM  CAPACITY  OF   3000

CHARACTERS.    THIS  SHOULD  BE  MORE  THAN  ENOUGH    FOR    ANY

CONVERSATION  QSO.  IF  YOU  TRANSMIT  MORE  THAN  300 CHARACTERS

WITHOUT  SENDING  YOUR CALL (FOR  IDENTIFICATION)   THE   PROGRAM

PROMPTS   YOU  TO  INPUT PROPER  ID AND THEN TO  PRESS  <CTRL>  E

KEY.   THE <CTRL>  E KEY SHOULD ALWAYS BE USED AT THE VERY END OF

A  BUFFER YOU ARE PLANNING TO XMIT, (AFTER ANY MESSAGE PORT  INFO

USED  FOR  ID AND THE  K'S WE USUALLY SEND).   WHEN  THE  PROGRAM

ENCOUNTERS <CTRL> E, IT WILL SCROLL UP THE TYPE AHEAD WINDOW  AND

PLACE  THE CURSOR IN COLUMN 1 - IT ALSO SENDS A  CARRIAGE  RETURN

LINEFEED  PAIR, HOLDS FOR A SHORT DELAY AND THEN   SWITCHES  YOUR

RIG  (AND  PROGRAM) BACK TO RECEIVE MODE.  TO  ACTUALLY  TRANSMIT

THE  BUFFER  YOU JUST CREATED, PRESS <CTRL> T OR <SHIFT>  F2  KEY

COMBINATIONS  THIS  WILL KEY YOUR TRANSMITTER AND  BEGIN  SENDING

YOUR  TYPE AHEAD BUFFER.  WHEN  THE TRANSMIT CATCHES UP  TO  YOUR

TYPING  -  YOU WILL STILL  HAVE  THE OPPORTUNITY  TO   CORRECT  A

TYPO (IN THE WORD YOU ARE ENTERING)  BY  JUST PRESSING  BACKSPACE

OR  DELETE TO GO BACK TO THE ERROR  THEN  ENTER  THE  CORRECTION.

THIS IS CALLED "XMIT BY THE WORD" WHICH MEANS  THE  WORD YOU  ARE

ENTERING  WILL NOT BE SENT UNTIL YOU PRESS THE <SPACE> AFTER  IT.

MESSAGE  PORTS  ARE  ENTERED BY PRESSING AND  HOLDING  <ALT>  AND

PRESSING  THE  NORMAL   NUMBER KEY (NOT KEYPAD  NUMBERS)  OF  THE

MESSAGE PORT YOU WANT TO SEND.

8

============== USING "BREAK" TRANSMIT MODE ==============

THERE  WILL  BE TIMES, AFTER TYPING IN A LOT OF TEXT TO  USE  FOR

YOUR  NEXT TRANSMISSION,  THAT  SOMEONE  WILL ASK YOU A  QUESTION

THAT DEMANDS AN IMMEDIATE  ANSWER.  INSTEAD OF HAVING TO SEND THE

WHOLE  BUFFER YOU HAVE TYPED, YOU CAN  ELECT TO USE  THE  "BREAK"

TRANSMIT  MODE.   WHEN YOU  PRESS <SHIFT>  F1  THE   TYPE   AHEAD

WINDOW  IS "SAVED" AND CLEARED.  THE TRANSMITTER  IS  KEYED ON  -

AND  YOU ARE READY TO ENTER THE TEXT TO BE SENT.  AT THE END  YOU

CAN  ENTER  YOUR ID INFO AND PRESS THE NORMAL <CTRL> E   KEY   TO

RETURN TO RECEIVE - OR YOU MAY ELECT TO GO DIRECTLY TO THE NORMAL

TYPE AHEAD MODE WITHOUT GOING BACK THROUGH RECEIVE - YOU CAN   DO

THIS BY PRESSING <CTRL> T OR <SHIFT> F2.

============== NORMAL RECEIVE MODE ==============

WHEN  THE PROGRAM IS "BOOTED" IT COMES UP IN THE  NORMAL  RECEIVE

MODE, AT WHATEVER  BAUD RATE AND MODE YOU SELECTED DURING PROGRAM

CONFIGURATION.  WHILE  YOU  ARE IN QSO - MANY  TIMES YOU WILL  BE

BUSY TYPING AHEAD FOR YOUR NEXT TRANSMISSION - SOMETHING IS  SENT

AND  BY THE TIME YOU LOOK UP, ITS SCROLLING OFF THE  SCREEN.   IF

THIS HAPPENS, YOU CAN PRESS THE F5 KEY TO  BRING-UP THE EDIT/VIEW

MODE  MENU.  BY PRESSING "R" YOU WILL SELECT THE RECEIVE  BUFFER,

THE  SCREEN THEN  DISPLAYS YOUR RECEIVE BUFFER STARTING  AT   THE

BEGINNING.   YOU CAN PRESS PAGEDOWN OR PAGEUP  TO   MOVE  THROUGH

THE  BUFFER, OR HOME/END KEYS TO JUMP TO THE BEGINNING  OR   END,

RESPECTIVELY,   OF  THAT BUFFER.  IN THIS WAY YOU CAN  SEE   WHAT

HAD   BEEN SENT.  WHEN YOU ARE FINISHED LOOKING  PRESS:  <ESCAPE>

AND YOU WILL RETURN TO  THE  EDIT/VIEW MENU, THEN PRESS  <ESCAPE>

AGAIN  TO RETURN  TO  NORMAL RECEIVE MODE.  YOU CAN ALSO USE  THE

"BLOCK"  MODE  TO  LOOK  THRU THE BUFFER AND   MARK  A  BLOCK  TO

TRANSMIT,  SAVE  TO DISK OR DELETE (FROM  THE   RECEIVE  BUFFER).

YOU ALSO HAVE THE OPTION TO SAVE THE RECEIVE BUFFER TO  DISK,  TO

DO  THIS  PRESS: F4 - A PROMPT IS DISPLAYED, ON LINE  17,  FOR  A

FILENAME  TO  BE USED TO SAVE THE BUFFER UNDER.   ANOTHER  OPTION

AVAILABLE  TO  YOU, IN  THE BLOCK MODE, IS THE ABILITY TO MARK  A

"BLOCK"  OF RECEIVE  BUFFER, YOU  CAN THEN PRINT  (PRINT MODE  IS

SELECTED  BY PRESSING F9 FROM  NORMAL RECEIVE  MODE),  DELETE  OR

TRANSMIT  THAT BLOCK.  YOU  CAN  BEGIN  "PRINT ECHO"   MODE,   TO

SEND RECEIVED CHARACTERS TO YOUR PRINTER,  BY  PRESSING <CTRL>  P

- IT WILL DISPLAY THE PRINT ECHO MESSAGE ON 16.  IF YOU DON'T GET

THE PRINT ECHO MESSAGE AND INSTEAD YOU HEAR A BEEP,  YOUR PRINTER

IS  PROBABLY  OFF  LINE   -  NOT TURNED ON -  OR  IN  SOME  ERROR

CONDITION.

THERE'S  AN OPTION IN THE CONFIGURATION SCREENS THAT  WILL  ALLOW

ALL TYPE AHEAD  CHARACTERS, MESSAGE PORTS AND BLOCK SENDING TO BE

ECHOED  TO  THE RECEIVE  SCREEN AND, AT THE SAME TIME,  INTO  THE

RECEIVE  BUFFER.   IF YOU DESIRE THIS - BE SURE  TO  SELECT  THAT

OPTION IN THE CONFIGURATION.

IMPORTANT NOTE:  THERE ARE, RARE, OCCASIONS THE PROGRAM MAY  MISS

AN  INTERRUPT  OR SOME OTHER TIMED EVENT, WHEN THIS  HAPPENS  ANY

NUMBER OF WEIRD THINGS CAN BEFALL THE PROGRAM.  ANYTIME SOMETHING

HAPPENS THAT CAUSES THE SCREEN TO GET MESSED UP - ALWAYS TRY  THE

<ALT> F1 REDRAW - MOST OF THE TIME THE NEW SCREEN WILL BE REDRAWN

CORRECTLY.   IT MAKE TAKE SEVERAL TRYS TO ACCOMPLISH THE  CORRECT

REDRAW.  IF THE REDRAW DOESN'T CORRECT YOUR SCREEN PROBLEM -  TRY

GOING  INTO  OTHER  MODES AND  RETURNING,  BEFORE  REBOOTING.

9

============== USING EDIT/VIEW MODE ==============

PRESSING F5 WILL BRING-UP THE EDIT/VIEW MENU IN THE UPPER AREA OF

THE RECEIVE WINDOW.  YOU HAVE SEVERAL OPTIONS AVAILABLE TO YOU IN

THIS  MODE.  THE FIRST IS "EDIT MESSAGE PORT", ENTERING "M"  WILL

TAKE YOU INTO THIS MODE.  THE NEXT WINDOW WILL PROMPT YOU FOR THE

MESSAGE  PORT  NUMBER YOU WISH TO EDIT (OR  CREATE).   USING  THE

NORMAL  NUMBER  KEYS  (NOT THOSE ON THE KEYPAD)  ENTER  THE  PORT

NUMBER  YOU NEED TO WORK WITH. AFTER MAKING YOUR SELECTION AS  TO

PORT  NUMBER TO USE - YOUR DISPLAY WILL NOW HAVE A  WINDOW  WHICH

YOU  WILL  USE  TO INPUT YOU MESSAGE.   ENTERING  CHARACTERS  AND

EDITING  USES THE SAME -BACKSPACE ONLY- FORMAT AS YOU USE IN  THE

TYPE  AHEAD AND BREAK MODES.  FULL EDITING WILL BE PROVIDED IN  A

LATER VERSION OF AUTORTTY.  AFTER YOU HAVE COMPLETED YOU MESSAGE,

PRESS <ESC> - YOU ARE THEN PROMPTED IF YOU WANT THE MESSAGE  PORT

SAVED TO DISK - SELECT "Y" OTHERWISE PRESS "N" OR <ESC>.

YOUR SECOND OPTION IN EDIT/VIEW IS TO VIEW EITHER THE RECEIVE  OR

TRANSMIT  (PIX)  BUFFER.  USING EITHER "R" OR "T"  (FOR  RECEIVE,

TRANSMIT  RESPECTIVELY)  YOU SELECT THE BUFFER YOU WANT  TO  USE.

YOU CAN MOVE THROUGH THE BUFFER USING ANY OF THE FOLLOWING  KEYS:

HOME  - TAKES YOU TO THE BEGINNING OF BUFFER, END - TAKES YOU  TO

THE END OF BUFFER, PAGEUP - TAKES YOU UP A PAGE (SCREEN),  PAGEDN

- TAKES YOU DOWN A PAGE (SCREEN).  WHEN YOU ARE FINISHED  LOOKING

THROUGH  THE  BUFFER - PRESS <ESC>, THIS TAKES YOU  BACK  TO  THE

EDIT/VIEW MENU.

THE THIRD OPTION IS TO DISPLAY A DISK DIRECTORY.  PRESS "D"  WILL

BRING  UP  A WINDOW WHICH PROMPTS FOR A  PATH  (DRIVE\SUB-DIRS\).

AFTER  ENTERING  YOUR  DESIRED PATH (ENTERING  NOTHING  WILL  NOT

DISPLAY  DEFAULT)  PRESS <RETURN> AND THE "SORTED"  DIRECTORY  IS

DISPLAYED  IN  THE  SAME WINDOW YOU HAVE  BEEN  USING.   YOU  USE

PAGEUP/PAGEDN TO MOVE THROUGH THE LIST OF FILES DISPLAYED, AGAIN,

HOME/END KEYS ARE SUPPORTED.  PRESS <ESC> WILL RETURN YOU TO  THE

EDIT/VIEW MENU.

TO   RETURN  TO  NORMAL RECEIVE MODE - PRESS:  <ESC>   OR   SHIFT

F5.  THE RECEIVE SCREEN IS COMPLETELY UPDATED.

10

============== USING THE BLOCK MODE OPTION ==============

PRESSING  F8  BRINGS  UP BLOCK MODE MENU,  HERE  YOU  CAN  SELECT

BETWEEN XMIT SENT  BUFFER (THIS BUFFER HAS THE LAST 1120 BYTE  OF

TEXT  YOU  HAVE SENT FROM  THE KEYBOARD) OR  THE  RECEIVE  BUFFER

(THIS BUFFER HAS A MAXIMUM OF 65,500  CHARACTERS). THE OTHER  TWO

OPTIONS ON THIS MENU ARE THE DELETE BLOCK FROM RECEIVE BUFFER AND

SAVING  THE  BLOCK TO DISK.  TO MARK A BLOCK TO BE USED  -  FIRST

SELECT  THE  BUFFER YOU WANT TO USE USING EITHER THE "T"  OR  "R"

KEY.   THE  XMIT  SENT  BUFFER IS  COMPLETELY  DISPLAYED  ON  ONE

SCREEN,  THE   RECEIVE BUFFER CAN CONTAIN  MANY  DISPLAY  SCREENS

(PAGES).   USE THE ARROW  KEYS  TO MOVE THE  CURSOR   AROUND  THE

SCREEN  AS NEEDED.  TO MARK THE BEGINNING OF A BLOCK - PLACE  THE

CURSOR  ON THE CHARACTER THAT WILL BE  THE  STARTING   POINT   OF

THE BLOCK - THEN  PRESS  <CTRL>  B.  THAT CHARACTER  WILL THEN BE

DISPLAYED IN REVERSE VIDEO.  MOVE THE CURSOR TO THE  VERY END  OF

THE  BLOCK YOU WANT TO SELECT AND PRESS <CTRL> E IF YOU  ARE   IN

THE XMIT SENT BUFFER - THE ENTIRE BLOCK IS DISPLAYED IN   REVERSE

VIDEO,   OTHERWISE  THE END CHARACTER WILL ALSO BE DISPLAYED   IN

REVERSE  VIDEO.   PRESSING EITHER <CTRL> B OR <CTRL>  E  ACTUALLY

TOGGLES  THE  BLOCK  POINTER  ON AND OFF.   IF,  AFTER  YOU  HAVE

SELECTED A BEGINNING OR END, YOU WANT  TO  CHANGE YOUR  SELECTION

-  JUST  PRESS  THE KEY THAT SELECTS THE POINTER   YOU   WANT  TO

CHANGE  (<CTRL>  B = START OR <CRTL> E  =  END) AND  PRESS   THAT

KEY  AGAIN.  THE FIRST KEY PRESS WILL CLEAR THE ORIGINAL  POINTER

(EVEN   IF YOU ARE NOT ON IT WITH THE CURSOR), PRESSING  IT   THE

SECOND   TIME WILL SELECT THAT POINTER AGAIN AT  WHATEVER  CURSOR

POSITION  YOU  MAY BE ON AT THE TIME.  YOU ARE NOW READY TO  EXIT

TO THE MENU,  YOU MUST  PRESS THE F8 KEY (PRESSING <ESCAPE>  WILL

RETURN  YOU TO THE MENU - BUT  CLEARS ANY BLOCK MARKERS THAT  YOU

MAY  HAVE SELECTED).  USING F8 TO EXIT THE BUFFER WILL  SAVE  THE

POINTERS  YOU HAVE SELECTED.  THIS BLOCK IS NOW READY FOR YOU  TO

DO WITH AS YOU WISH (SAVE,DELETE,PRINT OR RE-SEND).

NOTE:  ALL KEY ASSIGNMENTS FOR BLOCK MODE ARE GIVEN ON PAGES: 5 &

6  OF:  SECTION 2 - PROGRAM KEY ASSIGNMENTS.

============== USING AUTO-SAVE MODE ==============

SINCE  THE  RECEIVE  BUFFER  IS  LIMITED  TO  HOLDING  THE   LAST

65,500  CHARACTERS,   SAVING  EVERYTHING  THAT MAY  BE  SENT  FOR

LONG PERIODS IS IMPOSSIBLE  USING IT.  THIS IS WHERE  "AUTO-SAVE"

COMES  IN,  BY  SELECTING  THIS   MODE,  EVERYTHING  RECEIVED  IS

"ECHOED"  TO  A DISKFILE.  BY  PRESSING <CTRL> F9 - YOU  WILL  BE

PROMPTED  (ON  LINE  17)  FOR A FILENAME  TO  SAVE  THE  RECEIVED

CHARACTERS  UNDER.  THE AMOUNT OF RECEIVED TEXT IS ONLY   LIMITED

BY  THE  DISK SIZE.

IMPORTANT  NOTE:  FILES SAVED USING AUTO-SAVE MAY BE MUCH  LARGER

THAN CAN BE LOADED INTO PIX BUFFER TO EDIT.  IF THIS IS THE CASE,

YOU WILL HAVE TO USE AN OUTSIDE WORD PROCESSOR TO EDIT THE FILE.

NOTE:

IF YOU HAVE A COLOR MONITOR, LINE 25 (THE OPTION/INFO LINE)  WILL

HAVE  A  CYAN (AQUA) COLOR BACKGROUND.  MONO  USERS  SHOULD  HAVE

"NORMAL"  VIDEO.  YOU CAN ALSO CHECK BY PRESSING  <CTRL>   F9  TO

SEE  IF  YOU ARE IN "AUTO-SAVE" MODE -  BY  PRESSING  THOSE  KEYS

(WHEN YOU ARE IN AUTO-SAVE) A MESSAGE IS DISPLAYED INDICATING YOU

ARE ALREADY IN "AUTO-SAVE" MODE.  TO CLOSE FILE, PRESS <ALT> F9.

11

============== USING PRINT MODE ==============

PRESSING   F9  - DISPLAYS THE PRINT MENU (IF YOUR PRINTER  IS  ON

LINE   AND NOT  IN  SOME ERROR CONDITION).  HERE YOU  CAN  SELECT

BETWEEN   PIX   XMIT BUFFER, RECEIVE BUFFER OR A  RECEIVE  BLOCK.

AFTER  SELECTING  THE   BUFFER, YOU HAVE  THE  OPTION  OF  EITHER

PRINTING THE WHOLE FILE OR PART OF A  FILE (FILL).   YOU CAN ALSO

TOGGLE  BETWEEN AUTO-LINEFEED OR NORMAL  PRINTING.  USING   AUTO-

LINEFEED WILL SPREAD OUT AN OVERLINE TO MAKE SEEING  "DINGS" IN A

PIX  EASIER  TO  SPOT.  USING THIS PRINT MODE  YOU  CAN  PRINT  A

DISKFILE  FROM  THE XMIT (PIX) BUFFER WHILE  VIEWING  THE  NORMAL

RECEIVE  MODE SCREEN.  YOU WILL NOT LOSE ANY RECEIVE  OR  PRINTER

CHARACTERS BY DOING THIS.

============== USING "SHELL" MODE ==============

BY  PRESSING <ALT> F10 YOU CAN SHELL OUT OF THE PROGRAM  TO  YOUR

OPERATING  SYSTEM (THE FILE STORED UNDER THE COMSPEC  ENVIRONMENT

VARIABLE).   IF  YOU  LIMIT  YOUR  "OUT OF PROGRAM  TIME"  TO  20

MINUTES OR LESS, YOU  WILL NOT LOSE ANY RECEIVE CHARACTERS.  WHEN

YOU ARE FINISHED USING THE SHELL JUST ENTER:  EXIT<RETURN>  - AND

YOU  WILL BE RETURNED TO THE PROGRAM WITH THE  SCREEN  COMPLETELY

UPDATED.   THIS SELECTION IS AVAILABLE IN EITHER  NORMAL  RECEIVE

MODE, OR IN THE -TYPE AHEAD- XMIT MODE.

IMPORTANT NOTE:  ALTHOUGH AUTORTTY SAVES THE DEFAULT PATH  DURING

PROGRAM START-UP, FOR INTERNAL FILE SAVES (LIKE MSG PORTS, ETC.),

IT  MAY BE SAFER HAVE THE PATH SET TO THE PROGRAM DEFAULT  BEFORE

RETURNING   FROM  YOUR  "SHELL"  MODE,  ALTHOOGH  I  HAVE   NEVER

ENCOUNTERED  A PROBLEM BY RETURNING TO THE PROGRAM  FROM  ANOTHER

PATH.

IMPORTANT  NOTE:   NEVER CALL-UP A PROGRAM FROM THIS  SHELL  THAT

WILL MODIFY THE SERIAL PORT YOU ARE USING FOR YOUR INTERFACE.

============== BAUD RATE - RTTY MODE SELECTION ==============

TO  TOGGLE BETWEEN FIXED BAUD RATES (45-75-110-135-300) YOU  JUST

PRESS  <CTRL>  B.  TO TOGGLE BETWEEN ASCII AND BAUDOT JUST  PRESS

<CTRL>  V.  IF YOU  ARE  IN ASCII MODE - YOU CAN  TOGGLE  BETWEEN

ASCII8  (8  LEVEL)  AND ASCII  7 (7 LEVEL) BY PRESSING <CTRL>  A.

THESE SELECTIONS WORK ONLY IN RECEIVE MODE.

============== SENDING DATE/TIME AND CWID ==============

PRESS  <CTRL>  D  TO SEND THE CURRENT  DATE/TIME.   THIS  CAN  BE

ENTERED  INTO THE  TYPE AHEAD BUFFER AS WELL - AND WILL  BE  SENT

WHEN  THE  XMIT ROUTINE COMES TO IT.  PRESS <CTRL> I  TO  SEND  A

CWID.   THIS  IS  NOT STORED IN THE TYPE AHEAD, SO  IT   IS  ONLY

AVAILABLE WHEN THE TYPE AHEAD BUFFER IS EMPTY (CAUGHT UP  TO YOUR

TYPING).    AUTORTTY  WILL AUTOMATICALLY SEND  YOUR  CWID   EVERY

NINE MINUTES, WHETHER IN PIX OR FILL MODE OR JUST SENDING TEXT IN

A QSO.  IT WILL  EVEN  ID IF YOU ARE ON THE PHONE AND THE RIG  IS

SITTING  THERE  IN "MARK" TONE.

12

======= SENDING FILES ENTERED IN THE TYPE AHEAD BUFFER =======

WHEN  YOU  FIRST ENTER <CTRL> F  THE NEXT INPUT MUST BE  A  VALID

FILENAME.  YOU CAN'T HAVE ANY  SPACES BETWEEN THE  CHARACTERS  IN

THE    FILESPEC  (DRIVE/PATH/FILENAME).  YOU  END  YOUR  FILESPEC

ENTRY  BY  PRESSING <CTRL> F - A  SECOND TIME. (NEVER  ALLOW  THE

FILENAME  BETWEEN  THE CONTROL F CHARACTERS TO  WRAP  TO  ANOTHER

LINE)    SEE  AN EXAMPLE OF THIS AND FURTHER INFORMATION  IN  THE

PROGRAM  KEY  ASSIGNMENTS AREA OF THIS MANUAL, SECTION2.

========== LOADING AND TRAMSMITTING THE PIX XMIT BUFFER =========

BY   PRESSING  F3 - YOU WILL BRING UP A MENU IN THE UPPER  WINDOW

GIVING YOU THE OPTIONS AVAILABLE TO YOU FOR LOADING THE  TRANSMIT

(PIX)  BUFFER.  THOSE OPTIONS ARE:  1. LOAD BUFFER FROM DISK,  2.

LOAD  BUFFER FROM THE RECEIVE BUFFER (THE CURRENT  CONTENTS),  3.

LOAD BUFFER FROM A "BLOCK" SELECTED FROM THE RECEIVE BUFFER.

WHEN  LOADING  FROM DISK, YOU MUST INPUT  THE  COMPLETE  FILESPEC

(DRIVE:\PATH\FILENAME)  OF   THE DISKFILE TO BE LOADED  INTO  THE

PIX   XMIT BUFFER.  THE FILENAME WILL BE DISPLAYED ON  LINE  ONE.

IF YOU NEED TO EDIT THE FILE BEFORE SENDING, PRESS <CTRL> F3  AND

YOU WILL ENTER "AUTOEDIT", THE EDITOR MODULE.

WHEN LOADING FROM EITHER OF THE RECEIVE BUFFER OPTIONS, THE  NAME

DISPLAYED ON LINE ONE WILL REFLECT WHICH OPTION YOU USED.  IF YOU

PLAN  ON SAVING AND/OR EDITING THE TRANSMIT BUFFER, PRESS  <CTRL>

F3.  IF YOU PREVIOUSLY LOADED THE BUFFER FROM THE RECEIVE  BUFFER

OR  BLOCK  -  YOU  WILL  BE  PROMPTED  FOR  A  COMPLETE  FILESPEC

(DRIVE:\PATH\FILENAME)  YOU WANT THE BUFFER SAVED UNDER.    AFTER

GIVING AN ACCEPTABLE FILESPEC, YOU WILL ENTER THE TRANSMIT  (PIX)

BUFFER EDITOR "AUTOEDIT".

NOTE:   FOR INFORMATION ON USING "AUTOEDIT", THE  TRANSMIT  (PIX)

BUFFER EDITING MODULE, SEE NEXT PAGE.

WHEN  YOU   ARE   IN  XMIT  MODE, YOU CAN  TRANSMIT  THE  PIX  BY

PRESSING <SHIFT> F3,  THIS  WILL HALT  YOUR TYPE AHEAD SENDING  -

SO  IT SHOULD ONLY BE SELECTED WHEN  YOUR TYPE  AHEAD  BUFFER  IS

EMPTY (AS INDICATED BY THE TYPE BUFF SIZE ON  LINE 18).

========== PREPARING AND SENDING PART (FILL) OF A PIX ===========

WHEN  THE XMIT BUFFER IS LOADED, PRESS F7 - YOU WILL BE  PROMPTED

(ON  LINE 17)  FOR BOTH THE START AND END FILL LINES.  AFTER  YOU

HAVE GIVEN YOUR INPUT, IT WILL BE DISPLAYED BESIDE THE PIX NUMBER

ON  LINE ONE.  YOU  CAN TRANSMIT  THE  "FILL"  FROM  YOUR  NORMAL

TYPE AHEAD  MODE  BY  PRESSING <SHIFT> F7.  AGAIN YOUR TYPE AHEAD

BUFFER SHOULD BE EMPTY WHEN ENTERING THIS THIS MODE.

============== EXITING YOUR AUTORTTY PROGRAM ==============

TO   EXIT  AUTORTTY, PRESS <CTRL> O - A MESSAGE WILL  APPEAR   ON

LINE  25 INFORMING YOU TO PRESS <CTRL> U TO EXIT OR ANY OTHER  TO

CONTINUE IN THE PROGRAM.

13

=======  USING  "AUTOEDIT" TRANSMIT (PIX) BUFFER EDITOR  =======

AFTER  LOADING THE TRANSMIT (PIX) BUFFER, USING THE  F3  COMMAND,

YOU CAN ENTER THE EDIT BUFFER MODE BY PRESSING <CTRL> F3.  IF YOU

PREVIOUSLY  LOADED THE BUFFER FROM EITHER OF THE  RECEIVE  BUFFER

OPTIONS,  YOU  WILL  FIRST BE PROMPTED FOR  A  COMPLETE  FILESPEC

(DRIVE:\PATH\FILENAME) TO SAVE THE BUFFER UNDER.

THE EDITOR WILL COMPLETELY FILL YOUR COMPUTER SCREEN.  NOTE:  ALL

RECEIVED DATA IS STILL BEING SAVED IN THE RECEIVE BUFFER AND WILL

BE DISPLAYED WHEN YOU EXIT (BY PRESSING <ESCAPE>) THE EDITOR.

LINE  (1) HAS THE EDITOR NAME "AUTOEDIT" ALONG WITH THE  FILENAME

OF  THE FILE WHICH IS DISPLAYED.  LINE (24) SHOWS THE  AMOUNT  OF

"FREE" BUFFER SPACE - THE ACTUAL ROW OF THE FILE THE CURSOR IS ON

(OVERLINES  ARE  NOT  COUNTED) - THE COLUMN WITHIN  THE  ROW  THE

CURSOR IS ON (EACH ROW CAN BE AS LONG AS 225 CHARACTERS, ALTHOUGH

NOT  RECOMMENDED, ITS THERE SO YOU CAN USE BACKSPACE AND  DELETE

MODES WHICH APPEND LINES TOGETHER).  ADDITIONALLY, LINE (24) HAS

AN  INDICATOR TO INFORM WHETHER YOU ARE IN INSERT (OPENS A  LINE

UP  TO  "INSERT" CHARACTERS TYPED WITHIN IT) OR  OVERWRITE  (NEW

CHARACTERS  ARE ENTERED "OVER" THE OLD TEXT) MODE.  THE  LETTERS

"INS"  STAND FOR INSERT MODE - "OVR" INDICATES  OVERWRITE  MODE,

THE  CURSOR ALSO INDICATES WHICH MODE YOU ARE IN.  AN  UNDERLINE

CURSOR  INDICATES  "INSERT"  MODE -  A  BLOCK  CURSOR  INDICATES

"OVERWRITE"  MODE.  YOU TOGGLE BETWEEN THESE MODES  BY  PRESSING

<INSERT>  KEY.  LASTLY, LINE (24) DISPLAYS THE SIZE OF THE  FILE

YOU ARE EDITING, THIS IS UPDATED (AS IS THE BUFFER FREE) -  WHEN

YOU ENTER OR DELETE CHARACTERS.  LINE (25) DISPLAYS THE  OPTIONS

AVAILABLE WITHIN THE EDITOR.

HELP - PRESS F1 - WILL DISPLAY A SCREEN OF KEY ASSIGNMENTS USED.

PRESSING  F3  (IN  NORMAL  EDIT MODE) - WILL  PROMPT  YOU  FOR  A

COMPLETE FILESPEC (DRIVE:\PATH\FILENAME) OF THE DISKFILE YOU WANT

TO "INSERT" AT THE CURRENT CURSOR POSITION.

PRESSING F4 (IN NORMAL EDIT MODE) - WILL SAVE THE BUFFER TO DISK,

USING   THE  FILESPEC  YOU  ENTERED  PREVIOUSLY.    THE   PROGRAM

AUTOMATICALLY  RENAMES THE CURRENT FILE WITH A ".BAK"  EXTENSION,

THEN SAVES THE CURRENT BUFFER CONTENTS USING THE NORMAL FILENAME.

IF,  FOR  SOME REASON, IT CAN NOT SAVE THE FILE  AT  THE  CURRENT

FILESPEC - YOU WILL BE PROMPTED FOR ANOTHER FILESPEC TO SAVE  THE

BUFFER UNDER.

PRESSING  F4  (IN BLOCK MODE) - WILL PROMPT YOU  FOR  A  COMPLETE

FILESPEC  (DRIVE:\PATH\FILENAME) WHICH WILL BE USED  WHEN  SAVING

THE "MARKED" BLOCK.

PRESSING  F8 TOGGLES BLOCK MODE (ON/OFF).  WHEN ON, YOU CAN  MARK

THE  BEGINNING AND END OF A "BLOCK" OF TEXT TO EITHER  DELETE  OR

COPY TO ANOTHER LOCATION.  YOU SELECT THE "BLOCK START" BY  FIRST

PLACING  THE  CURSOR AT THE POINT YOU WANT THE BLOCK TO  START  -

THEN  PRESS  <CTRL> B. BY THE SAME TOKEN, YOU SELECT  THE  "BLOCK

END" BY PLACING THE CURSOR AT THE POINT YOU WANT THE BLOCK TO END

- THEN PRESS <CTRL> E.  WHEN YOU MAKE THESE SELECTIONS, RIGHT AND

LEFT  POINTING  TRIANGLES  ARE  "INSERTED"  IN  THEIR  RESPECTIVE

POSITIONS  (EVERYTHING  WITHIN  THESE  "TRIANGLES"  ARE  IN   THE

SELECTED  BLOCK - INCLUDING ANY END-OF-LINE CHARACTERS).  IF  YOU

(CONTINUED ON NEXT PAGE)

14

===  USING  "AUTOEDIT" TRANSMIT (PIX) BUFFER EDITOR (CONT.)  ===

MAKE A MISTAKE IN SELECTING A BLOCK MARKER - JUST PRESS THAT  KEY

SEQUENCE  AGAIN AND THE MARKER WILL BE CLEARED FROM  THE  SCREEN.

YOU  DO NOT HAVE TO BE "ON" THE MARKER TO CLEAR IT - EVEN IF  THE

MARKER  HAS SCROLLED OFF THE SCREEN, IT WILL BE CLEARED,  IF  YOU

HAVE THE CURSOR AT THE NEW POSITION - JUST PRESS THE PROPER BLOCK

SELECTING KEYS AGAIN AND IT WILL BE MARKED.  LINE 25 SHOWS IF YOU

HAVE  A BLOCK MARKER SET OR CLEARED.  PRESSING F8 A  SECOND  TIME

WILL EXIT BLOCK MODE.

YOU ONLY HAVE THREE OPTIONS AVAILABLE IN BLOCK MODE:  1. <CTRL> D

-  TO  DELETE  THE "MARKED" BLOCK,   2. <CTRL> O -  TO  COPY  THE

"MARKED"  BLOCK,  STARTING AT THE CURRENT CURSOR  POSITION.   YOU

CAN'T  HAVE THE CURSOR INSIDE THE BLOCK WHEN YOU  SELECT  "COPY".

3. F4 - SAVES CURRENT MARKED BLOCK (PROMPTS FOR FILESPEC).

IMPORTANT NOTE:

IF  YOU ATTEMPT TO DELETE A BLOCK THAT IS LARGER THAN  10K  BYTE,

YOU  WILL GET A MESSAGE WINDOW WHICH EXPLAINS THAT THE BLOCK  YOU

HAVE SELECTED IS OVER 10K AND YOU WILL NOT BE ABLE TO  "UNDELETE"

IT.   THE WINDOW THEN PROMPTS, ASKING IF YOU WANT TO  DELETE  THE

BLOCK ANYWAY.

----  KEYS SUPPORTED WITHIN "AUTOEDIT"  ----

F1  -  HELP  - DISPLAYS A SCREEN WHICH CONTAINS ALL  OF  THE  KEY

ASSIGNMENTS USED WITHIN THE EDITOR.

F3  (NORMAL  EDIT  MODE)  -  PROMPTS  FOR  A  COMPLETE   FILESPEC

(DRIVE:\PATH\FILENAME) OF THE FILE TO BE INSERTED AT THE  CURRENT

CURSOR POSITION.

F4 (NORMAL EDIT) - SAVES BUFFER - SEE PREVIOUS PAGE.

F4 (BLOCK MODE) - PROMPTS FOR A COMPLETE FILESPEC (DRIVE:\PATH\

FILENAME) FOR SAVING CURRENT BLOCK TO DISK.

F8  - TOGGLES ENTER/EXIT OF BLOCK MODE (LINE 24 WILL SHOW  IF  IN

BLOCK MODE OR NORMAL EDIT MODE).

<CTRL>B - TOGGLES SET/CLEAR OF START OF BLOCK (IN BLOCK MODE).

<CTRL>E - TOGGLES SET/CLEAR OF END OF BLOCK (IN BLOCK MODE).

<BACKSPACE>  WILL "DELETE" AS THE CURSOR MOVES BACK TO THE  LEFT.

IF  YOU  ARE IN COLUMN ONE - PRESSING BACKSPACE  WILL  CAUSE  THE

CURRENT LINE TO BE APPENDED TO THE LINE ABOVE (AFTER REMOVING THE

PREVIOUS LINE'S END-OF-LINE CHARACTER/S).

<DELETE>  WILL "DELETE" CHARACTERS TO THE RIGHT OF THE CURSOR  AS

THEY  ARE "PULLED" TO THE CURSOR.  IF YOU HAVE THE CURSOR AT  THE

END OF A LINE - IT WILL "PULL" THE NEXT LINE UP AND APPEND IT  TO

THE  CURRENT LINE (AFTER REMOVING THE CURRENT LINE'S  END-OF-LINE

CHARACTER/S).

<ARROW>  KEYS - ALL ARROW KEYS ARE SUPPORTED.  LINES LONGER  THAN

THE  CURRENTLY DISPLAYED 79 CHARACTERS (AS INDICATED BY A "+"  IN

COLUMN 80) WILL BE DISPLAYED, AS THE CURSOR REACHES COLUMN 80, BY

(CONTINUED ON NEXT PAGE)

15

===  USING  "AUTOEDIT" TRANSMIT (PIX) BUFFER EDITOR (CONT.)  ===

HAVING  THE  SCREEN "SCROLL" TO THE RIGHT.  IT WILL  JUMP  IN  20

CHARACTER INCREMENTS AS YOU MOVE THE CURSOR FARTHER TO THE  RIGHT

(OR  LEFT  - IF YOU ARE MOVING BACK TO THE BEGINNING).   YOU  CAN

SCROLL THROUGH THE FILE BY HOLDING DOWN EITHER THE "UP" OR "DOWN"

ARROW  KEYS.   THE  SCREEN WILL SHIFT RIGHT  AS  THE  CURSOR,  OR

CHARACTERS, ARE ENTERED IN COLUMN 80 - IT WILL SHIFT LEFT AS  YOU

MOVE THE CURSOR (OR BACKSPACE) LEFT FROM COLUMN 1 (UNLESS YOU ARE

AT THE BEGINNING OF THAT LINE).

<HOME> KEY - MOVES THE CURSOR TO THE BEGINNING OF ITS LINE.

<CTRL><HOME>  MOVES THE CURSOR TO THE UPPER LEFT OF  THE  CURRENT

SCREEN.

<END> KEY - MOVES THE CURSOR TO THE END OF ITS LINE.

<CTRL><END>  MOVES THE CURSOR TO THE END OF THE LAST LINE ON  THE

CURRENT SCREEN.

<PAGEUP> KEY - MOVES THE CURRENT DISPLAY SCREEN "UP" BY 20 LINES.

<CTRL><PAGEUP> KEY - TAKES YOU TO THE BEGINNING OF THE FILE.

<PAGEDOWN>  KEY - MOVES THE CURRENT DISPLAY SCREEN "DOWN"  BY  20

LINES.

<CTRL><PAGEDOWN> KEY - TAKES YOU TO THE END OF THE FILE.

<RETURN>  -  WILL INSERT A CARRIAGE RETURN/LINEFEED PAIR  AS  THE

END-OF-LINE CHARACTERS.

<CTRL> M - WILL INSERT A CARRIAGE RETURN ONLY AS THE  END-OF-LINE

CHARACTER.

<CTRL>  J - WILL INSERT A LINE WHICH HAS ONLY A LINEFEED  AS  THE

END-OF-LINE CHARACTER.

<CTRL>  Y - DELETES THE "WHOLE" LINE THE CURSOR IS  ON.   HOLDING

THESE  KEYS  DOWN WILL KEEP SCROLLING UP THE  FILE  AND  DELETING

LINES  AS IT GOES.  IF YOU USE THIS FEATURE, YOU SHOULD HAVE  THE

CURSOR ON LINE ONE - THIS WAY YOU HAVE THE REST OF THE SCREEN  TO

VIEW AS IT DELETES.  ON FAST COMPUTERS, YOU CAN GET DELETE  LINES

VERY QUICKLY - SO BE VARY CAREFUL.

<CTRL>  R  - WILL RESTORE THE "LAST" LINE  WHICH  WAS  PREVIOUSLY

DELETED WITH THE COMMAND ABOVE.

<ALT>  R - WILL RESTORE THE LAST BLOCK DELETED (IF  THAT  DELETED

BLOCK WAS LESS THAN 10K) AT THE CURRENT CURSOR POSITION.

<CTRL> N - INSERTS A LINE AT THE CURRENT CURSOR POSITIONS - WITH

A CARRIAGE RETURN/LINEFEED END-OF-LINE.

<ESCAPE>  KEY  - WILL EXIT THE EDIT MODE AND RETURN  YOU  TO  THE

NORMAL  RECEIVE RTTY SCREEN.  IF THE FILE HAS CHANGED  SINCE  YOU

LAST  SAVED,  YOU  WILL BE PROMPTED WITH THE  OPTIONS  TO  EITHER

"SAVE AND EXIT" OR "EXIT WITHOUT SAVE".

(CONTINUED ON NEXT PAGE)

16

THE DISPLAY OF EACH LINE'S END-OF-LINE CHARACTER/S IS THE SAME IN

THE EDITOR AS IT IS IN THE REST OF THE PROGRAM.  A "<" IN  COLUMN

80 INDICATES A CARRIAGE RETURN/LINEFEED PAIR.  A "-" IN COLUMN 80

STANDS FOR A CARRIAGE RETURN ONLY.  A "J" IN COLUMN 80  INDICATES

A LINEFEED ONLY.  A "+" INDICATES A LINE THAT IS LONGER THAN, THE

DISPLAYED, 79 CHARACTERS.

IF  YOU HAVE A COLOR MONITOR, THE LINE WHICH HAS THE CURSOR  WILL

BE DISPLAYED IN "YELLOW".

17

AUTORTTY/AUTO-PIX

KEY ASSIGNMENTS

SECTION 2

FUNCTION  KEY ASSIGNMENTS FOR K8LC AUTORTTY PROGRAM  VERSION 1.X

------  <CTRL> & <ALT> KEY ASSIGNMENTS ALSO INCLUDED ------

F1 - - - - - HELP/INFO --- OFFERS MULTIPLE SCREENS WHICH  EXPLAIN

HELP/KEYS    ALL KEY  ASSIGNMENTS USED BY THIS PROGRAM.  THE INFO

GIVEN IS JUST A BRIEF DESCRIPTION OF THEIR USE.

<SHIFT> F1 - FROM RECEIVE MODE, THIS WILL ALLOW YOU TO GO INTO  A

"BREAK"      BREAK-XMIT  TRANSMIT MODE.  YOUR NORMAL  TYPE  AHEAD

WINDOW WILL BE SAVED AND THEN CLEARED.  YOU ARE  NOW

IN  TRANSMIT BREAK MODE AS THE LINE ABOVE  THE  TYPE

BUFFER  WINDOW  WILL INDICATE. YOU CAN  GO  DIRECTLY

FROM  THIS  TRANSMIT MODE TO THE NORMAL  TYPE  AHEAD

XMIT  MODE BY PRESSING <SHIFT> F2 OR <CTRL> T.   THE

NORMAL  EXIT WILL BE BY USING THE CTRL E (AT END  OF

TEXT  YOU WANT SENT) OR WITH F2 OR <CTRL>  R  (PRESS

TWICE  FROM TYPE XMIT GOES TO RECEIVE - PRESS  THREE

TIMES TO ABORT PIX BUFFER SENDING - SUCH AS  SENDING

A PIX OR FILL - AND RETURNS TO TYPE XMIT MODE).

<CTRL> F1 -- BRINGS UP THE  FIRST, OF FIVE, PROGRAM CONFIGURATION

OPTION SCREENS.  THIS FIRST SCREEN COVERS:  THE BAUD

RATE AND RTTY MODE THE PROGRAM WILL STARTUP IN. YOUR

TIME  ZONE, WHETHER ITS DAYLIGHT SAVING OR  STANDARD

TIME AND LASTLY IF YOU WANT TO RETAIN A CR SENT WITH

A  CR/LF PAIR (IF SENT).  THE NEXT  SCREEN  INCLUDES

INPUT  OFTHE  SERIAL PORT YOU  ARE  USING  FOR  YOUR

INTERFACE   AND OTHER  VARIABLES  NEEDED FOR  PROPER

PROGRAM  OPERATION.  THE NEXT TWO SCREENS  HAVE  THE

DEFAULT   "PATHS"  USED  BY  THE   VARIOUS   PROGRAM

FUNCTIONS.   SCREEN 5 HAS TWO INPUTS WHICH ARE  USED

TO  SEND   SPECIAL INFO OR HOLIDAY GREETINGS  -  THE

FIRST IS SENT DURING THE WELCOME MESSAGE, THE SECOND

IS SENT DURING LOG-OFF.

IMPORTANT:   <CTRL>F1 SHOULD BE THE VERY FIRST THING YOU DO AFTER

INSTALLING "AUTORTTY" ON YOUR COMPUTER.

<ALT> F1 - - REDRAWS  THE RECEIVE  WINDOW AND STATUS  LINES.  IF,

FOR SOME REASON, YOUR RECEIVE WINDOW OR STATUS LINES

BECOME  MESSED UP, PRESSING <ALT> F1 SHOULD  CORRECT

THE PROBLEM. EVEN IF THE SCREEN GETS MESSED UP, YOUR

RECEIVE BUFFER SHOULD CONTAIN THE CORRECT DATA.

ALWAYS TRY THIS FUNCTION, ALONG WITH TRYING TO GO TO

SOME  OTHER  MODE AND RETURN, BEFORE  REBOOTING  THE

PROGRAM.  MOST OF THE TIME THIS FUNCTION SHOULD GIVE

SATISFACTORY RESULTS.

F2 - - - - - IN RECEIVE MODE,  THIS COMMAND PUTS THE  PROGRAM  IN

(OR CTRL R)  "LTRS" RECEIVE, JUST IN-CASE YOUR "28" GETS STUCK IN

IN  RECV IT  "FIGS".   IN  TRANSMIT MODE  (IF  CURRENTLY  SENDING

FORCES LTRS  YOUR  TYPE  AHEAD  BUFFER,  SENDING A PIX  OR  FILL)

THIS SELECTION WILL GIVE YOU  IMMEDIATE  BREAKOUT OF

TRANSMIT MODE.

(F2 INFO CONTINUED ON NEXT PAGE)

1

=== F2 INFORMATION CONTINUED ===

IN XMIT IT   NOTE:  YOU  WILL HAVE TO  PRESS F2  TWICE  TO  ABORT

FORCES END   NORMAL TYPE TRANSMIT  AND THREE TIMES TO ABORT A PIX

XMIT  AND    OR FILL.   (A SAFE GUARD AGAINST  ACCIDENTAL  ABORT)

RETURNS TO   ANY CHARACTERS LEFT IN THE XMIT BUFFER WILL BE  SENT

RECEIVE      THE NEXT TIME YOU USE  XMIT  MODE.  TO AVOID SENDING

THE  REMAINDER  OF  A BUFFER - YOU  SHOULD  USE  THE

NORMAL  RETURN TO RECEIVE <CTRL> E.   USING THIS  AT

THE  END  OF  "EVERYTHING"  YOU PLAN TO  XMIT,   THE

PROGRAM WILL AUTOMATICALLY  SEND  A CARRIAGE  RETURN

LINEFEED PAIR,  THEN SCROLL UP THE TYPE AHEAD SCREEN

AND  PLACES  CURSOR.  YOU ARE THEN  READY  TO  START

ENTERING YOUR TEXT AGAIN.

<SHIFT> F2 - GOES FROM RECEIVE MODE TO TRANSMIT MODE  -  OR  FROM

(OR CTRL T)  TRANSMIT BREAK MODE TO  NORMAL TYPE  AHEAD  TRANSMIT

RX TO TX     MODE.  THERE IS A CONFIGURATION OPTION ALLOWING YOUR

TYPE & MESSAGE PORT XMIT TO BE ECHOED TO THE RECEIVE

SCREEN AND PUT IN THE RECV BUFF.

F3 - - - - - DISPLAYS  A  MENU  (ON  UPPER  WINDOW)   GIVING  THE

LOAD PIX     OPTIONS FOR LOADING THE TRANSMIT (PIX) BUFFER.    IF

XMIT BUFFER  LOADING  FROM  A  DISK  FILE,  YOU  -MUST- ENTER THE

COMPLETE FILESPEC (DRIVE\PATH\FILENAME) OF THE  FILE

YOU WANT LOADED IN THE TRANSMIT BUFFER.  IF YOU WANT

TO USE SOMETHING OTHER THAN THE DISPLAYED   DEFAULT,

JUST  USE  EITHER   BACKSPACE OR  DELETE  (DELETE  =

BACKSPACE IN RTTY PRG) TO CLEAR THE DEFAULT AND TYPE

IN  YOUR  INPUT.  THIS FUNCTION IS  AVAILABLE  WHILE

VIEWING  A DIRECTORY BROUGHT UP BY F5 -  (EDIT  VIEW

MODE)  WITH  "D" SELECTED.  YOUR OTHER  OPTIONS  ARE

LOADING   THE  TRANSMIT  BUFFER  WITH  THE   CURRENT

CONTENTS  OF THE RECEIVE BUFFER, OR FROM A  SELECTED

"BLOCK" OF THE RECEIVE BUFFER.

<SHIFT> F3 - WHEN IN TRANSMIT MODE, THIS WILL SEND THE  DISKFILE

XMIT PIX     THAT WAS LOADED WITH THE ABOVE F3 COMMAND.

<CTRL> F3 -- ENTERS "AUTOEDIT" MODULE -- FOR EDITING THE TRANSMIT

AUTOEDIT     (PIX) BUFFER.   THIS FUNCTION ALLOWS "FULL"  EDITING

OF THE BUFFER.  FOR COMPLETE DETAILS, SEE SECTION  1

MANUAL - PAGES 14-16.

F4 - - - - - PROMPTS (ON LINE 17) FOR A  FILESPEC  (DRIVE:\PATH\-

SAVE RECV    FILENAME)  THAT  WILL  BE USED TO  SAVE THE  RECEIVE

BUFFER TO    BUFFER UNDER.  IF THE FILE  EXISTS,  IT WILL  ASK IF

DISK         YOU WANT TO OVER-WRITE THE EXISTING FILE.

<SHIFT> F4 - USE THIS WHEN YOU HAVE FINISHED ENTERING ALL OF YOUR

(OR CTRL E)  INPUT IN THE TYPE AHEAD (OR BREAK) BUFFER.

SEE <CTRL> E FOR COMPLETE INFORMATION ON THIS ENTRY.

F5 - - - - - DISPLAYS  THE EDIT/VIEW  MENU IN A  WINDOW  THAT  IS

EDIT OF MSG  OPENED  UP IN  THE UPPER RECEIVE WINDOW.   YOU  WILL

PORTS AND    STILL HAVE 6 LINES OF RECEIVE TEXT DISPLAYED.    YOU

VIEW TX &    HAVE  THE  OPTIONS OF  EDITING YOUR  MESSAGE  PORTS,

RX BUFFERS   VIEWING  THE  RECEIVE  AND XMIT  (PIX)  BUFFERS  AND

& DIR        DISPLAYING A DIRECTORY.

2

F6 - - - - - DISPLAYS  INFO  (ON  LINE  25)  ABOUT  CLEARING  THE

CLRS RX,TX,  VARIOUS BUFFS.  PRESS: "R" TO CLEAR RECEIVE - "T" TO

PIX BUFFS    CLEAR TYPE AHEAD - "P" TO CLEAR THE PIX BUFFER.

<CTRL> F6 -- PROMPTS  (ON LINE 17) FOR A FILESPEC  (DRIVE:\PATH\-

DELETES  A   FILENAME)  WHICH WILL BE DELETED - IT WILL  ASK  FOR

DISKFILE     VERIFICATION  BEFORE  ACTUALLY  DELETING  THE  FILE.

THIS  FUNCTION IS AVAILABLE WHILE YOU ARE VIEWING  A

DIRECTORY BROUGHT UP BY F5 (EDIT/VIEW MODE) WITH "D"

SELECTED.

F7 - - - - - PROMPTS (ON LINE 17) FOR  BOTH THE  START  LINE  AND

INPUT  PIX   END LINE  NUMBERS  USED FOR SENDING PART OF  A  FILE

FILL LINES   (FILL).  TRANSMIT BUFFER MUST BE LOADED BEFORE  THIS

FUNCTION WILL OPERATE.

<SHIFT> F7 - TRANSMITS THE  "FILL"  SELECTED WITH THE F7 FUNCTION

XMIT ABOVE   ABOVE.  YOU  MUST BE IN  TRANSMIT  MODE  BEFORE THIS

PIX FILL     THIS SELECTION WILL WORK. IF YOU WANT TO SEND TO THE

END  OF A PIX - PICK A LINE NUMBER GREATER THAN  THE

NUMBER IN PIX - I USE 999.

F8 - - - - - BRINGS UP THE MENU FOR "BLOCK" MODE.   YOU HAVE  THE

BLOCK MODE   OPTIONS  TO  MARK  A BLOCK EITHER IN  YOUR  KEYBOARD

MARK BLOCK   (TYPE SENT) BUFFER OR RECEIVE BUFFER.   YOU CAN XMIT

IN  RECEIVE/ THE  "MARKED" BLOCK,  WHILE IN  NORMAL  XMIT   (TYPE

TYPE AHEAD   AHEAD)  MODE BY PRESSING <SHIFT> F8.  THIS XMIT WILL

BUFFERS      HALT  NORMAL TYPE AHEAD SENDING - SO USE  THIS  MODE

WHEN  YOUR  TYPE BUFFER  INDICATES  ZERO.    ANOTHER

OPTION  ALLOWS  YOU  TO DELETE THE  BLOCK  FROM  THE

RECEIVE  BUFFER - OR SAVE  THE BLOCK TO  DISK.   YOU

CAN  ALSO PRINT THE  BLOCK  BY CALLING UP THE  PRINT

MODE (F9) AND SELECT "PRINT BLOCK".

<SHIFT> F8 - TRANSMITS  THE SELECTED BLOCK -- THIS FUNCTION  ONLY

XMITS BLOCK  WORKS WHEN YOU ARE IN TRANSMIT (TYPE AHEAD) MODE.

F9 - - - - - DISPLAYS  THE PRINTING OPTION MENU.  YOU CAN  PRINT

PRINTING     ALL  OR PART OF EITHER THE XMIT (PIX) BUFFER OR  THE

OPTION       RECEIVE BUFFER.  IT ALSO HAS THE OPTION  TO  ENABLE

MENU         "AUTO LINEFEED" WHICH WILL SPREAD  OUT THE OVERLINES

SO  YOU  CAN BETTER SEE A PROBLEM THAT  MAY  REQUIRE

EDITING.   YOU  CAN ALSO PRINT THE   "MARKED"  BLOCK

FROM THE RECEIVE BUFFER.   PRINTER ERRORS SHOULD NOT

NOT "HANG-UP" THE PROGRAM.

<CTRL> F9 -- PROMPTS  (ON LINE 17)  FOR  THE FILESPEC  TO  ACCEPT

AUTO-SAVE,   THE  CHARACTERS RECEIVED.   IF  YOU ARE PLANNING  ON

SAVES  ALL   BEING  AWAY FOR A NUMBER OF HOURS - YOU  CAN  SELECT

CHARACTERS   THIS  MODE TO HAVE  "EVERYTHING" RECEIVED IN A  DISK

RECEIVED TO  FILE.   YOU  CAN  LATER GO THIS FILE  WITH   A  WORD

A  SELECTED  PROCESSOR  AND EXTRACT ANY TEXT,  OR PIX  FILE,  YOU

DISKFILE     WANT  TO  KEEP.  YOU WILL STILL BE  ABLE   TO   VIEW

THE  RECEIVED  TEXT  AND  USE  MOST  OF  THE   OTHER

FUNCTIONS  OF THIS PROGRAM, SUCH AS TRANSMIT,  WHILE

YOU ARE IN AUTO-SAVE.  COLOR MONITOR USERS:  LINE 25

(OPTION/INFO  LINE)   WILL HAVE  A  CYAN  BACKGROUND

INSTEAD OF WHITE WHEN IN AUTO-SAVE MODE.

MONO USERS:  LINE 25 SHOULD BE IN REVERSE VIDEO.

3

<ALT> F9 - - THIS  CLOSES THE FILE OPENED BY THE  ABOVE   COMMAND

CLOSES A-S   AND RETURNS TO THE NORMAL PROGRAM MODE.

F10 - - - -  IF YOU HAVE THE FULL AUTORTTY/AUTO-PIX PROGRAM:

AUTO-PIX     PRESSING THIS KEY THREE TIMES, YOU WILL ENTER "AUTO-

MODE         PIX" MODE. PRESSING THIS KEY THREE TIMES FROM WITHIN

"AUTO-PIX" WILL EXIT THAT MODE AND RETURN YOU TO THE

NORMAL PROGRAM MODE.  THIS MODE WILL NOT FUNCTION IF

YOU ARE IN AUTO-SAVE.

<SHIFT> F10 - IF YOU HAVE THE FULL AUTORTTY/AUTO-PIX PROGRAM:

PRINTS  LOG-  THIS  SELECTION WILL  PRODUCE A  PRINT-OUT  OF  THE

FILE FROM     LOGFILE CREATED BY THE "AUTO-PIX" MODE.  YOU SHOULD

AUTO-PIX      RUN THIS ONCE A MONTH  TO  KEEP A RECORD OF  SYSTEM

LOGFILES      USAGE.   COPY  THE  FILES TO  A   FLOPPY  FOR  SAFE

KEEPING,  THEN  DELETE THE FILES FROM THE   LOGFILE

DEFAULT  PATH.    THIS WILL SPEED-UP  THE  LOG  OFF

PROCESS,  ESPECIALLY IF YOU ARE USING  A FLOPPY FOR

THE DEFAULT LOGFILE PATH.

<ALT> F10 -- SHELLS  OUT  TO  THE PROGRAM THAT IS  NAMED  IN  THE

SHELL TO     ENVIRONMENT  VARIABLE  "COMSPEC" - THIS  IS  USUALLY

"COMSPEC"    YOUR  VERSION  OF MS-DOS  UNLESS YOU  ARE  USING  AN

ALTERNATE  OPERATING  SYSTEM.   IF  YOU  KEEP   YOUR

"SHELL"  TIME LESS THAN 20 MINUTES -  YOU  SHOULDN'T

LOSE  ANY  RECEIVE  CHARACTERS  (BASED  ON  100  WPM

BAUDOT).   WHEN  IN  THE SHELL  YOU  CAN  RUN  OTHER

PROGRAMS  (IF  THEY  WILL FIT IN  MEMORY)  OR  OTHER

COMMAND LINE OPERATIONS.

4

<CTRL> & <ALT> KEY  ASSIGNMENTS FOR "AUTORTTY" PRG VERSION 1.X

CTRL B - - - TOGGLES  THROUGH THE VARIOUS BAUD RATES   SUPPORTED

BY  THIS PROGRAM.  THOSE ARE 45 - 75 - 110 -  135  -

300.   YOU  MUST  BE IN RECEIVE  MODE  TO  USE  THIS

FUNCTION.

CTRL V - - - TOGGLES  BETWEEN BAUDOT AND ASCII MODES.   THE  DEF-

AULT ASCII MODE USES THE  8 BIT  DATA FORMAT.    YOU

CAN  SWITCH BETWEEN ASCII8 AND ASCII7 BY  USING  THE

CTRL A SHOWN BELOW.

CTRL A - - - IN  ASCII  MODE - THIS TOGGLES  BETWEEN  ASCII8  AND

ASCII7.

CTRL O - - - PRESS TO EXIT PROGRAM -- YOU ARE PROMPTED  (ON  LINE

25) TO EITHER PRESS CONTROL U  (TO EXIT PROGRAM  AND

RETURN  TO DOS) OR TO PRESS ANY OTHER KEY TO  RETURN

TO PROGRAM.

CTRL P - - - TOGGLES PRINTER ECHO ON/OFF.   THIS CAN BE SELECTED

DURING  NORMAL RECEIVE  MODE, AS WELL AS,  WHEN  YOU

ARE  IN ANY XMIT MODE.   WHEN ON, ALL RECEIVED  TEXT

IS ECHOED TO THE PRINTER (USING ANY PORT -SERIAL  OR

PARALLEL-    )   YOU   SELECTED    DURING    PROGRAM

CONFIGURATION.  IF YOU TAKE THE PRINTER OFF LINE  OR

IT RUNS OUT OF PAPER, OR ANY OTHER MALFUNCTION,  THE

PROGRAM WILL -AUTOMATICALLY- TURN OFF THE PRINT ECHO

FEATURE.  THIS KEEPS THE PROGRAM FROM "HANGING UP".

NOTE:  IF  PRESSING  THIS KEY DOES  -NOT-  GIVE  THE  PRINT  ECHO

MESSAGE  ON EITHER LINE 16 OR 18 (RECV OR XMIT) YOUR PRINTER  MAY

BE OFF LINE (OR NOT TURNED ON) OR MAYBE OUT OF PAPER OR ANY OTHER

ERROR THAT WILL NOT ALLOW PRINTING.  YOU WILL GET A "BEEP" TO LET

YOU KNOW SOMETHING IS WRONG.

CTRL F - - - ALLOWS LOADING AND SENDING OF A DISKFILE (PIX)  FROM

WITHIN  THE  NORMAL  TRANSMIT  TYPE  AHEAD   BUFFER.

TO USE THIS FUNCTION:  PRESS CTRL F  (THIS  DISPLAYS

)  THEN ENTER  THE  FILESPEC  (DRIVE\PATH\FILENAME)

PRESS CTRL F  (THIS DISPLAYS ANOTHER ).   NOTE:  DO

-NOT- HAVE ANY SPACES BETWEEN THE CTRL F CHARACTERS.

YOU  MUST  INCLUDE DRIVE AND ANY  SUBDIRS.   IF  THE

TRANSMIT HAS CAUGHT UP TO YOU AND YOU BEGIN ENTERING

DATA AFTER THE FIRST CTRL F  --  THEN YOU DECIDE YOU

DO  NOT WANT TO SEND A FILE AFTER ALL - JUST  BACKUP

TO  THE  FIRST CTRL F AND ENTER THE SECOND  CTRL  F.

WHEN  THE PROGRAM ENCOUNTERS THE SECOND CTRL F -  IT

TRYS  TO OPEN THE FILE TO SEND - NOT FINDING  ONE  -

IT, HARMLESSLY, EXITS BACK TO THE NORMAL TYPE  AHEAD

MODE, JUST LIKE BEFORE YOU ENTERED THE FIRST CTRL F.

IMPORTANT:   THEY MUST BE USED IN PAIRS.   DON'T WRAP A  FILENAME

TO ANOTHER ROW, ENTER <RETURN> THEN THE COMPLETE ENTRY

YOU CAN ENTER ANY NUMBER OF DISKFILES IN THIS MANNER,  EACH  WILL

BE  LOADED AND SENT AS IT COMES UP IN THE TYPE  AHEAD BUFFER.

EXAMPLE:  B:1234-1P.PIXB:1235-1P.PIX - SENDS TWO FILES.

(<CTRL> F INFO CONTINUED ON NEXT PAGE)

5

CTRL F - - - (CONTINUED):

IF YOU ARE GETTING NEAR COLUMN 60,  ENTER <RETURN> TO GO TO

NEXT ROW BEFORE ENTERING ANOTHER SELECTION.

EXAMPLE:  THIS IS A TYPE AHEAD LINE BEFORE 1ST CRTL F<RETURN>

C:\NEW-PIX\GUD-NUDE.PIX

NOTE:  NEVER ALLOW A LINE WRAP BETWEEN THE TWO CTRL F'S

CTRL G - - - SENDS THE "BELL" - IT IS DISPLAYED WITH THE  "MUSIC

NOTE".

CTRL D - - - SENDS THE DATE/TIME - IT IS DISPLAYED  WITH  "SUN"

CHARACTER.

CTRL I - - - TRANSMITS A CWID - IN TRANSMIT MODE.

CTRL R - - - SEE INFORMATION FOR F2.

CTRL T - - - SEE INFORMATION FOR <SHIFT> F2

CTRL Y - - - CLEARS A LINE FROM EITHER TYPE AHEAD OR MESSAGE PORT

EDIT.

CTRL E - - - ENTER  THIS AT -END- OF  ALL TEXT YOU WANT  TO  SEND

(OR<SFT>F4)  (INCLUDING  ANY  MESSAGE  PORTS, YOU  ARE  USING  TO

INDICATE  THE END OF THE CURRENT  XMISSION.    AFTER

THIS  CHARACTER  IS ENTERED THE  PROGRAM  WILL   NOT

ALLOW  ANYMORE  CHARACTERS  TO  BE  ENTERED  IN   HE

KEYBOARD  BUFFER.   YOU CAN USE THE  <BACKSPACE>  OR

<DELETE>   KEYS  TO REMOVE  THIS   "CHARACTER"   AND

AGAIN  INPUT MORE TEXT OR MESSAGE PORTS IN THE  TYPE

AHEAD BUFFER.  WHEN  THE  XMIT  ROUTINE   ENCOUNTERS

THIS CHARACTER, THE PROGRAM WILL SCROLL UP THE  TYPE

AHEAD BUFFER WINDOW, PLACE CURSOR  AT THE  BEGINNING

OF  THE LINE (READY FOR YOUR NEXT INPUT) AND  RETURN

TO THE RECEIVE MODE.

CTRL M - - - ENTERING <CTRL> M  TRANSLATES TO A CARRIAGE RETURN/-

LINEFEED PAIR IN TYPE AHEAD, BREAK AND MESSAGE  PORT

BUFFER INPUT.

CTRL J - - - ENTERING <CTRL> J  ALSO  TRANSLATES  TO  A  CARRIAGE

RETURN/LINEFEED  PAIR  IN  TYPE  AHEAD,  BREAK   AND

MESSAGE PORT BUFFER INPUT.

========== KEYS USED INSIDE EDIT/VIEW AND BLOCK MODES ==========

HOME/CTRL Q --- TAKES YOU BACK TO THE  BEGINNING  OF THE  BUFFER

YOU ARE IN. THIS KEY HAS NO EFFECT WHEN IN  TYPE

SENT BUFFER IN BLOCK MODE.

END/CTRL Z ---- TAKES YOU TO THE END OF THE BUFFER YOU  ARE  IN.

THIS  KEY HAS EFFECT WHEN IN TYPE SENT BUFFER  IN

BLOCK MODE.

(CONTINUED ON NEXT PAGE)

6

PAGEUP/CTRL W - MOVES UP ONE  SCREEN (PAGE) IN THE BUFFER.   THIS

KEY  HAS NO EFFECT IN TYPE SENT BUFFER  IN  BLOCK

MODE.

PAGEDN/CTRL X - MOVES DOWN ONE SCREEN (PAGE)  IN  THE  BUFFER.

AGAIN, THIS KEY HAS NO EFFECT IN TYPE SENT BUFFER

IN BLOCK MODE.

<ESC> ESCAPE -- PRESSING THIS KEY WILL EXIT YOU FROM THE  BUFFER

YOU ARE IN BACK TO EITHER EDIT/VIEW OR BLOCK MODE

MENU.

NOTE:  USING THIS KEY TO LEAVE A BUFFER IN BLOCK MODE WILL  -NOT-

CREATE A MARKED, GLOBAL, BLOCK YOU CAN USE.

============== KEYS USED EXCLUSIVELY IN BLOCK MODE ==============

<ARROW KEYS> - USE THE ARROW KEYS  TO PLACE  THE  CURSOR  ON  THE

CHARACTER  THAT IS USED FOR THE START AND  END  OF

YOUR BLOCK.

CTRL  B  --  TOGGLES  THE -BEGINNING-  (TEMPORARY)  BLOCK  MARKER

WITHIN A BUFFER.  IF THE MARKER IS CLEAR - IT  WILL

MARK  (SET)  THE BEGINNING OF THE BLOCK.    IF  THE

MARK  IS  SET - IT WILL BE CLEARED  (REGARDLESS  OF

ITS LOCATION IN THE BUFFER).

CTRL  E - - TOGGLES THE -END- (TEMPORARY) BLOCK MARKER  WITHIN  A

BUFFER.   IF  THE  MARKER IS CLEAR -  IT  WILL  MARK

(SET)  THE  END OF A BLOCK.  A SET  MARKER  WILL  BE

CLEARED BY PRESSING IT.

HAVING   THE  ABOVE KEYS TOGGLE IS HANDY IF YOU  WISH  TO  CHANGE

A   MARKER  AND DON'T WANT TO HAVE TO MOVE OFF TO  ITS   LOCATION

TO  CLEAR  IT.  IF YOU SEE A  PLACE THAT SHOULD BE  THE  USED  AS

MARKER,  BUT  YOU  ALREADY   HAVE  ONE  SET,  JUST  PRESS   BLOCK

MARKER  KEY  (EITHER  <CTRL> B OR <CTRL> E) THE  ORIGINAL  MARKER

IS  CLEARED.    IF  THE CURSOR IS ON THE SPOT YOU  WANT  THE  NEW

MARKER  SET  - JUST  PRESS WHICH-EVER MARKER KEY THAT HAS  TO  BE

SET.  IF THE CURSOR ISN'T ON THE NEW LOCATION, MOVE IT THERE THEN

PRESS THE TEMPORARY BLOCK MODE MARKER KEYS <CTRL> B  OR <CTRL> E.

============ SPECIAL COMMANDS FOR MESSAGE PORT #1 ============

<ALT> C - - CLEARS   MESSAGE  PORT  #1 -  CONFIRMS  YOUR   CHOICE

BEFORE ACTUALLY CLEARING THE PORT.

<ALT> A - - ADDS  A  CALL  SIGN  TO  MESSAGE  PORT #1 -  DO   NOT

HAVE   ANY SPACES  IN  YOUR INPUT.  (PROMPTS ON  LINE

17)  THE  PROGRAM ADDS A TRAILING SPACE.

<ALT> R - - REMOVES  A  CALL FROM MESSAGE PORT #1 - AGAIN DO  NOT

HAVE  ANY SPACES IN YOUR INPUT. (PROMPTS ON LINE  17)

IF THE CALL SIGN CAN NOT BE FOUND, IT WILL INFORM YOU

ON LINE 17.

(MESSAGE PORT INFORMATION ON NEXT PAGE)

7

--- MESSAGE PORTS ---

ALT 1  THRU  ALT 0 -- (DO NOT USE KEYS ON KEYPAD)  USE TO  SELECT

MESSAGE  PORT,  TO  SEND, IN TYPE AHEAD OR  BREAK  BUFFER.   WHEN

PROMPTED  FOR MESSAGE PORT NUMBER IN "EDIT MESSAGE PORT" MODE  --

JUST  USE THE NORMAL 1 THRU 0.  AGAIN,  DO NOT USE <ALT>  OR  THE

KEYPAD KEYS, FOR EDITING.  YOU  CAN  NOW GIVE,  AS  PART OF  YOUR

INPUT,  ANOTHER MESSAGE PORT  NUMBER  (USING  THE <ALT> KEY  WITH

THE NORMAL NUMBER KEYS (NOT KEYPAD KEYS).

---------------------

AS YOU WILL SEE, I HANDLE END-OF-LINE DISPLAY A BIT  DIFFERENTLY.

COLUMN  80  WILL HAVE:  "<" FOR A CR/LF, "-" FOR A CR  ONLY,  "J"

FOR   A   LINEFEED AND  A "+" FOR AN EXTENDED LINE (ONE  THAT  IS

LONGER THAN 79 COLUMNS.  I CHOSE THIS METHOD SO YOUR RECEIVED PIX

WOULD BE AS CLEAN AS POSSIBLE, ON THE SCREEN.

WALTER E. BESS, JR.  K8LC

PRECISION ELECTRONICS

1525 KARIN STREET

IRONTON, OHIO   45638

PHONE:  1-614-532-6111

8