ADDITIONAL NOTES FOR THE "RU" RTTY UTILITY

This program is a slightly modified from the original RTTYU program

by KF4NB.  It's operation is mentioned in the documentation for his

RTTY program on this system.

Changes to look for are:

1) Prompt for drive that has pix; with "B" being the default.  Simply

hitting enter will save having to type the "B:" every time when

printing pix. If you want to override the default type "d:", with

"d" being the desired drive.

2) Request for pix size - with "Small" being the default.  This results

in a much better picture. The matrix printers used are rather thin

in putting ink on paper so the pix that look OK on a Teletype

machine look weak and washed out on the IBM Grafix printer.  In the

Small (default) mode the pix look great!

This change simply sends a printer modification command before the

pix is printed.  The command will set the printer to compressed

print and 6/72 " line spacing (I think).  This makes for a nice

tight pix that looks fantastic. If you have another type of printer

that can't handle this, use Full for full size pix or modify the

basic RU program to suit your printer, and recompile.  The basic

version runs very slow with out compiling.

3) When the quote (') is sent I suppressed the corresponding "BELL" or

annoying beep.  Some pix have thousands of 's in them and running

thru a pix can make the computer more annoying than the old noisy

TTY machines we just got rid of!

4) Not a change - just a note:  When you save a file you will be

prompted for additional info regarding compression.  If you say

Yes then the file will be saved as compressed data.  I have found

that most pix will shrink to 50% of the original size.  This depends

how many redundant characters there are in a pix, of course.

Another note, you can send the compressed pix right from your RTTY

program as it will un-compress it as it is sent out. NOT necessary

to un-compress with RU and then send it with RTTY.

5) For this program to operate correctly, a received RTTY pix must have

been received with the RTTY program in "ART" mode.  This is done with

the ALT-A key while the RTTY program is running, but before a pix

has started to be received. Of course, you will have hit F3 to

receive a file to disk!

A file that has been properly received for RTTY ART will have special

characters added to each line for carriage control and line feed

control.  Those characters are listed below.

Backwards "P" or paragraph symbol will cause a LINE FEED

The "vertical arrows" will cause a carriage return.

Note that if the pix has overline printing there will be the

vertical arrows without the backwards P.  This of course, will

cause a carriage return with no line feed. The result is over-

striking capability.

6) File sizes are listed below.  Use to verify that the download

is OK. This size should show up on a directory list on your PC.

RU.BAS       - 5338  bytes

RU.EXEBIN    - 22784

PARMS.RTY    - 175

GARFIELD.PIX - 7629

TUMBLWEE.PIX - 6779

"Happy Pix"   - de W9HLQ