LAB.BAS  is  a  basic  program written for the  IBM  or  IBM

compatible with 128k of RAM,  two disk drives and a printer.   It

may  be  run  on a one drive system by  removing  the  "B:"  from

several lines in the program files LAB.BAS, LAC.BAS AND SEM.BAS.

The  program  is  written  in  using  double  precision  for

accuracy.   If  accurate  station locations and  current  orbital

elements  are used angle accuracy is within .05 degrees and range

to about 50 meters.

This  is  a menu driven program using overlays  to  minimize

core requirements.  The following files are required:

LAB.BAS   (saved in ASCII)

LAC.BAS   (saved in ASCII)

SEM.BAS   (saved in ASCII)

ELSET.FIL

STATION.FIL

The  basic files must be stored on disk using the /A  switch

in  order  for  the  overlay technique  to  be  used.   The  disk

containing  the  above  files must be placed in drive  B  with  a

SYSTEM  disk  containing  BASIC,BASICA or  GWBASIC  in  drive  A.

Output is to the printer therefore the printer must be on line.

The  file "STATION.FIL" is the file containing the  location

of the station for which data are to be computed.  The parameters

are  latitude,  longitude,  height above the  spheroid,  meridian

deflection and prime deflection.  Height, meridian deflection and

prime  deflection  may  be  entered as zero  if  unknown  without

serious degradation of output.   Longitude is positive east.  The

following is an example of a location near the Tech Lab at PAFB:

28 13 34.8742 LAT

279 24  2.0120 LONG

-14.94 METERS above spheroid

.84 meridian deflection  (seconds of arc)

1.17 prime deflection  (seconds of arc)

ELSET.FIL  contains  the orbital elements of the  object  of

interest. Samples are included in the file.

With the default drive A, type BASIC B:LAB, BASICA B:LAB or

GWBASICA B:LAB.  Make menu selections and away you go.

When computing angles you will be prompted for the station

name, object number and object name.  Give names stored in

STATION.FIL and ELSET.FIL.  You will then be prompted for the

date of the desired look angles. The form is Julian (yydoy) or

year and day of year.  Example 86001 is the first of Jan. 1986.

The stop date is in the same format. The next prompt is for the

interval between points.  Enter the number of seconds (1 to

86400).  The computation begins.

The display will show U(3), (the number of kilometers above

or below the tangent plane), WD (the vertical velocity in

kilometers/sec, the current revolution the target is in and the

elevation angle in degrees.  These are displayed to show you the

computer is working and the printer will receive only those

points with a positive elevation.

Happy computing.########################################################################################################