Amateur Satellite Frequency Guide

Including UO-9, AO-10, UO-11, FO-12, RS-10, RS-11 and P3C

Produced for AMSAT North America by WA2LQQ

Rev 2.0  26Jun87

*******************************************************************************

*** UoSAT OSCAR 9 ***

Summary:  Downlinks on 2 meters, 70 cm and 13 cm; no uplinks since this is

primarily an educational and research spacecraft.

Other downlinks at 7.050, 14.002, 21.002 and 29.502 MHz which are

part of the HF propagation experiment.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

VHF downlink:     145.825 MHz

UHF downlink:     435.025 MHz

UHF downlink:    2401.    MHz

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Source: UoSAT Spacecraft Data Booklet, May 1986, University of Surrey, England

*******************************************************************************

*** UoSAT OSCAR 11 ***

Summary:  Downlinks on 2 meters, 70 cm and 13 cm; no uplinks since this is

primarily an educational and research spacecraft.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

VHF downlink:     145.825 MHz

UHF downlink:     435.025 MHz

UHF downlink:    2401.5   MHz

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Source: UoSAT Spacecraft Data Booklet, May 1986, University of Surrey, England

*******************************************************************************

*** AMSAT OSCAR 10 ***

Summary:

Mode B:  70 cm up; 2 meters down

Mode L:  24 cm up; 70 cm down

Beacons: Mode B: General Beacon 145.810 MHz; Engineering Beacon 145.987 MHz

Mode L: (No longer operational)

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode B: 435.030 - 435.180 up yields 145.825 - 145.975 MHz down (inverting)

Uplink         Downlink

=======================

145.987 MHz - Engineering Beacon

435.030 ------ 145.975 ----- Passband limit, upper

435.045 ------ 145.960 ----- General use limit, upper

435.050 ------ 145.955

435.060 ------ 145.945

435.070 ------ 145.935

435.080 ------ 145.925

435.090 ------ 145.915

435.100 ------ 145.905

435.105 ------ 145.900 ----- Nominal passband center

435.110 ------ 145.895

435.120 ------ 145.885

435.130 ------ 145.875

435.140 ------ 145.865

435.150 ------ 145.855

435.160 ------ 145.845 ----- General use limit, lower

435.180 ------ 145.825 ----- Passband limit, lower

145.810 ----- General Beacon

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode L:  (No longer operational)

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Source: Amateur Satellite Report 60/61, August 1, 1983 as amended

*******************************************************************************

*** FUJI OSCAR 12 ***

Summary:

Mode JA: 2 meter uplink; 70 cm downlink (analog mode for voice and CW)

Mode JD: 2 meter uplink; 70 cm downlink (digital mode for packet radio)

Beacon: 435.795 MHz

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode JA: 145.900-146.000 MHz up yields 435.900-435.800 MHz down (inverting)

Uplink         Downlink

=======================

145.900 ------ 435.900 MHz - Passband limit, upper

145.910 ------ 435.890

145.920 ------ 435.880

145.930 ------ 435.870

145.940 ------ 435.860

145.950 ------ 435.850 ----- Passband center

145.960 ------ 435.840

145.970 ------ 435.830

145.980 ------ 435.820

145.990 ------ 435.810

146.000 ------ 435.800 ----- Passband limit, lower

435.795 ----- Beacon

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode JD: (Digital) Any of four 2 meter channels up yields a 70 cm channel down

Uplink         Downlink

=======================

145.850 ------ 435.910 MHz - Channel 1

145.870 ------ 435.910 ----- Channel 2

145.890 ------ 435.910 ----- Channel 3

145.910 ------ 435.910 ----- Channel 4

(Downlink for all 4 channels is 435.910)

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Source: Amateur Satellite Report 119, March 12, 1986 and

Amateur satellite Report 130, August 18, 1986

JAS-1 Handbook, JARL, August 1985

*******************************************************************************

*** Radio Sputnik 10 ***

Summary:

Mode A :  2 meters up and 10 meters down.

Mode K :  15 meters up and 10 meters down.

Mode T :  15 meters up and 2 meters down.

Mode KT:  15 meters up and both 10 and 2 meters down.

Mode KA:  15 and 2 meters up and 10 meters down.

Beacons can carry telemetry or ROBOT downlink.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode A:  145.860 - 145.900 MHz up yields 29.360 - 29.400 MHz down

Uplink         Downlink

=======================

29.357 MHz -- Beacon

145.860 ------ 29.360 ------ Passband limit, lower

145.870 ------ 29.370

145.880 ------ 29.380 ------ Passband center

145.890 ------ 29.390

145.900 ------ 29.400 ------ Passband limit, upper

29.403 ------ Beacon

Robot A:

Uplink         Downlink

===================================================

145.820  ----- 29.357 or 29.403 MHz

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode K:  21.160 - 21.200 MHz up yields 29.360 - 29.400  MHz down

Uplink         Downlink

========================

29.357 MHz -- Beacon

21.160 ------- 29.360 ------ Passband limit, lower

21.170 ------- 29.370

21.180 ------- 29.380 ------ Passband center

21.190 ------- 29.390

21.200 ------- 29.400 ------ Passband limit, upper

29.403 ------ Beacon

Robot K:

Uplink         Downlink

===================================================

21.120 ? ----- 29.357 or 29.403 MHz

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode T: 21.160 - 21.200 up yields 145.860 - 145.900 MHz down

Uplink         Downlink

=======================

145.857 MHz -- Beacon

21.160 ------ 145.860 ------ Passband limit, lower

21.170 ------ 145.870

21.180 ------ 145.880 ------ Passband center

21.190 ------ 145.890

21.200 ------ 145.900 ------ Passband limit, upper

145.903 - ---- Beacon

Robot T:

Uplink         Downlink

===================================================

21.120 ? ----- 145.857 or 145.903 MHz

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode KT: 21.160-21.200 up yields 29.360-29.400 and 145.860-145.900 MHz down

KT Uplink     K Downlink    T Downlink

=======================================

29.357 ------ 145.857 MHz -- Beacon

21.160 ------ 29.360 ------ 145.860 ------ Passband limit, lower

21.170 ------ 29.370 ------ 145.870

21.180 ------ 29.380 ------ 145.880 ------ Passband center

21.190 ------ 29.390 ------ 145.890

21.200 ------ 29.400 ------ 145.900 ------ Passband limit, upper

29.403 ------ 145.903 ------ Beacon

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode KA: 21.160-21.200 and 145.860-145.900 up yields 29.360-29.400 MHz down

K Uplink      A Uplink       KA Downlink

========================================

29.357 MHz -- Beacon

21.160 ------ 145.860 ------ 29.360 ------ Passband limit, lower

21.170 ------ 145.870 ------ 29.370

21.180 ------ 145.880 ------ 29.380 ------ Passband center

21.190 ------ 145.890 ------ 29.390

21.200 ------ 145.900 ------ 29.400 ------ Passband limit, upper

29.403 ------ Beacon

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Source: Amateur Satellite Report 136, November 17, 1986 and

Amateur satellite Report 140, January 12, 1987

Radio Magazine, May, 1987

UA3CR, W4KM, G3IOR, VE1SAT

*******************************************************************************

*** Radio Sputnik 11 ***

Summary:

Mode A :  2 meters up and 10 meters down.

Mode K :  15 meters up and 10 meters down.

Mode T :  15 meters up and 2 meters down.

Mode KT:  15 meters up and both 10 and 2 meters down.

Mode KA:  15 and 2 meters up and 10 meters down.

Beacons can carry telemetry or ROBOT downlink

-----------------------------------------------------------------------------

Mode A: 145.910 - 145.950 MHz up yields 29.410 - 29.450 MHz down

Uplink         Downlink

=======================

29.407 MHz -- Beacon

145.910 ------ 29.410 ------ Passband limit, lower

145.920 ------ 29.420

145.930 ------ 29.430 ------ Passband center

145.940 ------ 29.440

145.950 ------ 29.450 ------ Passband limit, upper

29.453 ------ Beacon

Robot A:

Uplink         Downlink

===================================================

145.830 ------ 29.407 or 29.453 MHz

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode K: 21.210 - 21.250 up yields 29.410 - 29.450 MHz down

Uplink         Downlink

=======================

29.403 MHz -- Beacon

21.210 ------- 29.410 ------ Passband limit, lower

21.220 ------- 29.420

21.230 ------- 29.430 ------ Passband center

21.240 ------- 29.440

21.250 ------- 29.450 ------ Passband limit, upper

29.453 ------ Beacon

Robot K:

Uplink         Downlink

===================================================

21.130 ------- 29.403 or 29.453 MHz

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode T: 21.210 - 21.250 up yields 145.910 - 145.950 MHz down

Uplink         Downlink

=======================

145.907 MHz -- Beacon

21.210 ------ 145.910 ------ Passband limit, lower

21.220 ------ 145.920

21.230 ------ 145.930 ------ Passband center

21.240 ------ 145.940

21.250 ------ 145.950 ------ Passband limit, upper

145.953 ------ Beacon

Robot T:

Uplink        Downlink

====================================

21.130 ------ 145.907 or 145.953 MHz

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode KT: 21.210-21.250 up yields 29.410-29.450 and 145.910-145.950 MHz down

KT Uplink     K Downlink    T Downlink

=======================================

29.407 ------ 145.907 MHz -- Beacon

21.210 ------ 29.410 ------ 145.910 ------ Passband limit, lower

21.220 ------ 29.420 ------ 145.920

21.230 ------ 29.430 ------ 145.930 ------ Passband center

21.240 ------ 29.440 ------ 145.940

21.250 ------ 29.450 ------ 145.950 ------ Passband limit, upper

29.453 ------ 145.953 ------ Beacon

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode KA: 21.210-21.250 and 145.910-145.950 up yields 29.410-29.450 MHz down

K Uplink      A Uplink       KA Downlink

========================================

29.407 MHz -- Beacon

21.210 ------ 145.910 ------ 29.410 ------ Passband limit, lower

21.220 ------ 145.920 ------ 29.420

21.230 ------ 145.930 ------ 29.430 ------ Passband center

21.240 ------ 145.940 ------ 29.440

21.250 ------ 145.950 ------ 29.450 ------ Passband limit, upper

29.453 ------ Beacon

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Source: Amateur Satellite Report 136, November 17, 1986 and

Amateur satellite Report 140, January 12, 1987

Radio Magazine, May, 1987

UA3CR, W4KM, G3IOR, VE1SAT

*******************************************************************************

*** Phase 3C ***

Estimated Launch Date: Early 1988

(P R E L I M I N A R Y     E S T I M A T E S)

As of April 1987

Summary:

Mode B:  70 cm up; 2 meters down

Mode JL: 24 cm and 2 meters up; 70 cm down

Mode S:  70 cm up; 13 cm down

RUDAK:   24 cm up; 70 cm down

Beacons: Mode B:  General Beacon 145.8125; Engineering Beacon 145.975

Mode JL: General Beacon 435.650; Engineering Beacon 435.675

Mode S:  2400.640

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode B:

Uplink         Downlink

=======================

145.975 MHz - Engineering Beacon

435.425 ------ 145.975 ----- Passband limit, upper

435.435 ------ 145.965

435.445 ------ 145.955

435.455 ------ 145.945

435.465 ------ 145.935

435.475 ------ 145.925

435.485 ------ 145.915

435.495 ------ 145.905

435.505 ------ 145.895 ----- Passband center

435.515 ------ 145.885

435.525 ------ 145.875

435.535 ------ 145.865

435.545 ------ 145.855

435.555 ------ 145.845

435.565 ------ 145.835

435.575 ------ 145.825 ----- Passband limit, lower

145.8125 ---- General Beacon

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode JL:

L  Uplink    J1 Uplink   Downlink    J2 Uplink

==============================================

1269.325 --------------- 435.975 MHz ------------- L Passband limit, upper

1269.330 --- 145.820 --- 435.970 --- 144.440 ----- J Sub-band limit, upper

1269.340 --- 145.830 --- 435.960 --- 144.450

1269.350 --- 145.840 --- 435.950 --- 144.460 ----- J Sub-band center

1269.360 --- 145.850 --- 435.940 --- 144.470

1269.370 --- 145.860 --- 435.930 --- 144.480 ----- J Sub-band limit, lower

1269.380 --------------- 435.920

1269.390 --------------- 435.910

1269.400 --------------- 435.900

1269.410 --------------- 435.890

1269.420 --------------- 435.880

1269.430 --------------- 435.870

1269.440 --------------- 435.860

1269.450 --------------- 435.850 ----------------- L Passband Center

1269.460 --------------- 435.840

1269.470 --------------- 435.830

1269.480 --------------- 435.820

1269.490 --------------- 435.810

1269.500 --------------- 435.800

1269.510 --------------- 435.790

1269.520 --------------- 435.780

1269.530 --------------- 435.770

1269.540 --------------- 435.760

1269.550 --------------- 435.750

1269.560 --------------- 435.740

1269.570 --------------- 435.730

1269.575 --------------- 435.725 ----------------- L Passband limit, lower

435.675 ----------------- Engineering Beacon

435.650 ----------------- General Beacon

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Mode S:

Uplink          Downlink

========================

2400.640 ---- Beacon

435.610 ------ 2400.695 MHz  Passband limit, lower

435.615 ------ 2400.700

435.620 ------ 2400.705

435.625 ------ 2400.710 ---- Passband center

435.630 ------ 2400.715

435.635 ------ 2400.720

435.640 ------ 2400.725 ---- Passband limit, lower

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

RUDAK:            Uplink         Downlink

=======================

1269.675 ----- 435.675 MHz     Single channel

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Source:  AMSAT DL, DJ5KQ, DJ0HC/KE6MN, K0RZ

*******************************************************************************