���������������������������������ͻ

�                                 �

�  ������������������������       �

�    � ��� ��� �   ��� ��� �����  �

�    � �     � � � �     �  �     �

�    � �   ��� ��� ��� ���  �     �

�    � �   ��� � � ��� ���  �     �

�    �                            �

�    �                            �

�           Version 2.45          �

���������������������������������ͼ

�                         �

�     Paul E. Traufler    �

�     111 Emerald Dr.     �

�     Harvest, Al 35749   �

�     (205) 726-5511      �

�                         �

�������������������������ͼ

Satellite Tracking Program

12 August, 1990

TRAKSAT

(C)opyright 1990

All Rights Reserved

������������������������������������������������������������������ͻ

�                                                                  �

�             TRAKSAT is free for NON-COMMERCIAL use only.         �

�                                                                  �

������������������������������������������������������������������͹

�                                                                  �

�  If you find TRAKSAT useful and would like to use it in a        �

�  commercial operation please call or write for more information. �

�                                                                  �

�������������������ͻ                         ��������������������ͼ

�                         �

�   Paul E. Traufler      �

�   111 Emerald Dr.       �

�   Harvest, Al.  35749   �

�   205-726-5511 (work)   �

�   205-830-8450 (home)   �

�                         �

�������������������������ͼ

����������������������������������������������������������������

****************************************************************

TRAKSAT and its companion files is being distributed as

shareware. You are encouraged to share this software with others

provided that it is distributed complete with documentation and

in unmodified form and that no fee or other consideration is

charged or accepted.

****************************************************************

����������������������������������������������������������������

TRAKSAT makes no warranty of any kind, either express or implied,

including but not limited to implied warranties of

merchantability and fitness for a particular purpose, with

respect to this software and accompanying documentation.

Paul E. Traufler, author of TRAKSAT, SHALL NOT BE LIABLE FOR ANY

DAMAGES (INCLUDING DAMAGES FOR LOSS OF BUSINESS PROFITS, BUSINESS

INTERRUPTION, LOSS OF BUSINESS INFORMATION) ARISING OUT OF THE

USE OF OR INABILITY TO USE TRAKSAT.

TRAKSAT  Satellite Tracking Program                           Page  2

TABLE OF CONTENTS

-----------------------------------------------------------

INTRODUCTION ...........................................  4

THEORY OF SATELLITE MOTION .............................  5

HARDWARE REQUIRED TO RUN THE PROGRAM ...................  6

RUNNING THE PROGRAM ....................................  7

ADVANCED FEATURES (MAIN MENU OPTION ZERO) ..............  8

READ SATELLITE DATA (MAIN MENU OPTION ONE) .............  9

TRACKING STATIONS (MAIN MENU OPTION TWO) ............... 11

REAL-TIME MODE (MAIN MENU OPTION THREE) ................ 13

DELTA-TIME MODE (MAIN MENU OPTION FOUR) ................ 14

GRAPHICS (MAIN MENU OPTION FIVE) ....................... 16

SUN TERMINATOR ......................................... 18

TABULAR OUTPUT (MAIN MENU OPTION SIX) .................. 25

VISIBILITY (MAIN MENU OPTION SEVEN) .................... 29

MULTI-TRACK MODE (MAIN MENU OPTION EIGHT) .............. 31

MULTI-TRACK TABULAR OUTPUT MODE ........................ 33

QUITTING THE PROGRAM (MAIN MENU OPTION NINE) ........... 35

USER DEFINED SATELLITE PLOTTING COLOR .................. 35

ADVANCED FEATURES OPTION DESCRIPTION ................... 36

REVERSE SOLUTION (ADVANCED FEATURES MENU OPTION ZERO) .. 37

USER DEFINED/ALL SATELLITES ............................ 40

ANALYTICAL SOLUTION (LOS/OPTICAL) ...................... 41

NORAD/NASA 2-LINE SATELLITE DATA ....................... 46

WHAT ARE THE MEAN CLASSICAL ELEMENTS ................... 48

MODELS FOR PROPAGATION OF NORAD ELEMENT SETS ........... 53

THE PROPAGATION MODELS ................................. 53

COMPATIBILITY WITH NORAD ELEMENT SETS .................. 54

PROGRAM LIMITATIONS AND ASSUMPTIONS .................... 55

USER DEFINED STAR DATA ................................. 56

ACCURACY OF TRAKSAT .................................... 58

A BRIEF EDITORIAL ...................................... 60

SPECIAL THANKS ......................................... 61

QUESTIONS AND COMMENTS ................................. 62

FUTURE UPGRADES ........................................ 64

OBTAINING NORAD SATELLITE DATA SETS .................... 65

FILES REQUIRED FOR TRAKSAT ............................. 66

BIBLIOGRAPHY ........................................... 67

Trademarks used in this document

-----------------------------------------------------------------

IBM,PS/2 PC DOS are registered trademarks of International Business

Machines Corporation.

Microsoft MS, MS-DOS, QuickC, are registered trademarks of

Microsoft Corporation.

Epson FX,LQ, is a registered trademarks of Epson American Inc..

Hercules is a registered trademark of Hercules Computer Technology.

TRAKSAT  Satellite Tracking Program                           Page  3

INTRODUCTION

Ever since college I have been interested in satellites and

tracking methods. I have often looked up into the night sky and

thought, "I know satellites are up there but can I predict when

and where to look to see one".

I have several small programs to calculate different satellite

related quantities but there was not one program available to do

all the things I felt a satellite tracking program should do.

After several years of working in the aerospace field, I decided

that I could take on such a programing task.

I started it all with a program called STS, it was geared towards

tracking the space shuttle, but used the same basic orbital

calculations.

STS, version .95, is available on several BBS around the country,

see the references at the end of this document.

For a first attempt at such a satellite tracking program I was

some-what pleased with the results. But I felt there is room for

improvement and that is where TRAKSAT steps in.

TRAKSAT is a general purpose satellite tracking program, by that

I mean any satellite that has a NORAD, NASA 2-Line element set

can be used. There are some limitations in the program along with

some assumptions, the reader is directed to the section on limits

and assumptions for further study.

The solution to the satellite motion which is used by TRAKSAT is

completely analytic and therefore requires no numerical

integration. This makes the program fast, even faster when a

coprocessor is used, since the solutions can be evaluated at

arbitrarily large, or small, time intervals.

The purpose of this program is to provide the user with a means

of propagating NORAD element sets in time to obtain tracking

information of the space object.

TRAKSAT  Satellite Tracking Program                           Page  4

THEORY OF SATELLITE MOTION

A complete development of the theory required to predict

the position of an artificial satellite about the earth is not

presented here because this is not proper place for it. Such a

development would require a volume in itself and would be more

of a distraction than an aid to the potential user. Only enough

of the concepts required for a general understanding plus the

final results are given. References to detailed works from which

these results are derived are provided for the more than

casually interested reader.

At the end of the TRAKSAT operating instructions is a brief

overview of the fundamentals used in this program and is included

to help the reader understand the motion of an artificial

satellite about the earth.

TRAKSAT  Satellite Tracking Program                           Page  5

HARDWARE REQUIRED TO RUN THE PROGRAM

In order to run the program the user will need the following

hardware;

IBM or compatible PC,XT,AT,PS/2,386, 640K Ram

(510K FREE RAM IS REQUIRED), Floppy or Hard Disk,

Text mode display (25x80), Hercules, CGA, EGA,

or VGA graphics (used for plotting only),

Math coprocessor is NOT required for TRAKSAT,

(IF A COPROCESSOR IS PRESENT IT WILL BE USED *),

Epson FX-80 printer or compatible (for graphic print

out only), PC DOS or MS DOS version 2.1 or above.

* It should be noted that a coprocessor will be 3 to 4 times

faster than the emulator version. If the user plans on using the

real-time tracking mode, a coprocessor will "smooth out" the time

steps to such a small delta as to appear instantaneously. At any

rate the real-time mode runs as fast as the host computer can

calculate the data and update the screen.

******************

* IMPORTANT NOTE *

******************

The user will not be able to run TRAKSAT on a 360K floppy drive,

The size of the executable and the earth map data plus a

satellite data file will simply not fit on the 360K floppy disk.

The best solution to the problem would be run TRAKSAT from a hard

disk! The prices of hard disks have come down to a point where

practically all computers have them. If the user needs a "good

reason" to buy a hard disk, perhaps TRAKSAT can convince them to

do so.

******************

* IMPORTANT NOTE *

******************

To print out the document, TRAKSAT.DOC use the DOS copy command.

The syntax to use would be "COPY TRAKSAT.DOC PRN", without the

quotation marks.

TRAKSAT  Satellite Tracking Program                           Page  6

RUNNING THE PROGRAM

To start TRAKSAT you type "TRAKSAT", without the quotation marks,

at the DOS prompt. After a few moments the default tracking

station name will be displayed at the bottom of the screen. After

the opening screen has been displayed the TRAKSAT main menu will

appear. The main menu is the core of the program, i.e. from this

menu the user can setup satellite data, tracking station data,

and output selections.

Here is an main menu example;

���������������������������������������������������������������ͻ

�    Date: 3/26/1990                          Time: 18:30:00    �

���������������������������������������������������������������͹

�                  �������������������������ͻ                  �

�                  �         TRAKSAT         �                  �

�                  �������������������������͹                  �

�                  �        MAIN MENU        �                  �

�                  �������������������������͹                  �

�                  �  (0) Advanced Features  �                  �

�                  �  (1) Read Elements      �                  �

�                  �  (2) Tracking Stations  �                  �

�                  � #(3) Real Time Tracking �                  �

�                  �  (4) Delta Time Mode    �                  �

�                  �  (5) Graphics           �                  �

�                  �  (6) Output Data        �                  �

�                  �  (7) LOS Visibility     �                  �

�                  �  (8) Multi-Track        �                  �

�                  �  (9) QUIT               �                  �

�                  �   Enter Option (0 - 9)  �                  �

�                  �������������������������ͼ                  �

�                                                               �

���������������������������������������������������������������ͼ

the date and time will be the current system values. The date

format used is mm/dd/yyyy, while the time format is hh:mm:ss,

based on a 24 hour clock, i.e. 14:00:00 is the same as 2 PM. From

this menu the input data and output data can be directed.  If the

user enters other than the listed options numbers an error

message will appear at the bottom left of the screen. If the file

TRAKSAT.DEF is present the default tracking station from that

file will be displayed to remind the user of the default tracking

station. To change the default tracking station data see section;

TRACKING STATIONS, main menu option one.

******************

* IMPORTANT NOTE *

******************

All error messages are displayed for 3 SECONDS, then depending on

what the error was, program control will return to the user to

correct the problem. It is recommended that the user NOT press

any keys while the error message is being displayed, any key

presses may cause other error messages to appear.

TRAKSAT  Satellite Tracking Program                           Page  7

ADVANCED FEATURES (MAIN MENU OPTION ZERO)

The advanced features option will control access to the "Advanced

Features" menu. The term advanced features should NOT frighten

the potential user off, as the options in this section are NOT

hard to use just that some people will never need to use them.

For a full description of these menu options see the section;

ADVANCED FEATURES MENU.

TRAKSAT  Satellite Tracking Program                           Page  8

READ SATELLITE DATA (MAIN MENU OPTION ONE)

The main menu option 1 will call the read satellite data menu.

This program uses the NASA, or NORAD 2-line satellite element

data file format to read data into the program, (in this text the

use of NORAD refers to NASA 2-Line or NORAD satellite element

data sets). For a full explanation of the NASA 2-line satellite

element data sets see section; NASA 2-Line Satellite Data.

The read satellite data screen will appear;

���������������������������������������������������������������������ͻ

�      �������������������������������������������������������ͻ      �

�      �                                                       �      �

�      �           READ NASA 2-LINE SATELLITE FILE             �      �

�      �������������������������������������������������������͹      �

�      �                                                       �      �

�      �        Enter Satellite Filename: [NASA668.TXT ]       �      �

�      �        Enter Search String:      [            ]       �      �

�      �������������������������������������������������������Ķ      �

�      �                                                       �      �

�      �������������������������������������������������������Ķ      �

�      �                                                       �      �

�      �                                                       �      �

�      �                                                       �      �

�      �                                                       �      �

�      �                                                       �      �

�      �                                                       �      �

�      �                                                       �      �

�      �������������������������������������������������������ͼ      �

�                                                   Press Esc to Exit �

���������������������������������������������������������������������ͼ

the cursor will be placed at the satellite filename position. The

program will display the current satellite filename, if this

choice is acceptable for the user just press RETURN. If a

different satellite data file is desired the user will type in

the satellite data filename. To exit this option press Escape.

The next line requires the name of the satellite to track, a

maximum length of 12 characters is allowed. The program will

check if the file is present and display an error message if the

data file is NOT found.

To help the new user a NORAD satellite date file is included with

TRAKSAT, see section; Satellite Data Sets.

The search method used by the program will locate the first

occurrence of what was typed in for a search string when compared

to the satellite names, i.e. typing in "mi" could locate the

satellite named "Mir". The search is NOT upper/lower case

sensitive. If a match is found the full name is displayed and the

user is asked to accept this data or read for the next occurrence.

If the user does not know ANY satellite names they can enter a

carriage return, (Return or Enter), and ALL of the satellite names

will be displayed one at a time.

TRAKSAT  Satellite Tracking Program                           Page  9

******************

* IMPORTANT NOTE *

******************

TRAKSAT is limited to the first 2000 satellites in any data file.

If the user has more than 2000 satellites in a data file they will

need to remove, using a text editor, satellite data sets as to

include the desired data set in the 2000 limit. This may not prove

to be a limitation for most users as most satellite data sets have

less than 150 data points.

If no match is found a error message is displayed and the user

will try another match.

If the file is found and the satellite name has been located the

screen will appear like;

���������������������������������������������������������������������ͻ

�      �������������������������������������������������������ͻ      �

�      �                                                       �      �

�      �           READ NASA 2-LINE SATELLITE FILE             �      �

�      �������������������������������������������������������͹      �

�      �                                                       �      �

�      �        Enter Satellite Filename: [NASA668.TXT ]       �      �

�      �        Enter Search String:      [Mir         ]       �      �

�      �������������������������������������������������������Ķ      �

�      �        Found    MIR                                   �      �

�      �������������������������������������������������������Ķ      �

�      �                                                       �      �

�      �                                                       �      �

�      �               Satellite Name [MIR         ]           �      �

�      �                                                       �      �

�      �        Keep Reading Satellite File (y/n) [N]          �      �

�      �������������������������������������������������������ͼ      �

�                                                   Press Esc to Exit �

���������������������������������������������������������������������ͼ

the next step would be for the user to press return to stop reading

the satellite data file and return to the main menu.

The routine that reads the NASA 2-line satellite data does a check-

sum on the data to insure that the data is correct. If the check-

sum fails the user is notified with only a warning message, the

data may NOT be correct. THE USER CAN STILL USE THIS DATA BUT THE

RESULTS IT PRODUCES MAY NOT BE ACCURATE. For a full explanation

of NASA 2-line satellite element sets see section; NASA 2-Line

Satellite Elements.

TRAKSAT  Satellite Tracking Program                           Page 10

TRACKING STATIONS (MAIN MENU OPTION TWO)

The next option, number 2, will only need to be run once, unless

a different tracking station is used, by the user. The program

defaults to using Huntsville, Al. as the tracking station, if the

user does not want to use the default option they can select a

city from the city data file. The cite data file has over 720 of

the larger U.S. cities latitude and longitudes in it.

The tracking station search works very much like the satellite

name search. The user is asked for a search string and the first

occurrence is displayed, then the next one and so on, until no

more matches are found.

If the user accepts a match some additional data is asked for by

the program. The altitude above mean sea level in meters, hours

from Greenwich, daylight savings flag (1 = daylight savings, 0 =

standard time) , and time zone name, are required for the

tracking station. (Some examples of the hours from Greenwich; EDT

= -4, CDT = -5, CST = -6). If the altitude of the tracking

station are not known the user can enter zero with out to much

loss in accuracy.

If the user can not find a match to the city data then they will

need to use a text editor to add the city data in the file

TRAKSAT.CTY or use the closest city in the file.

Below is an example for Huntsville, Al..

���������������������������������������������������������������ͻ

�   �������������������������������������������������������ͻ   �

�   �            READ TRACKING SITE DATA FILE               �   �

�   �������������������������������������������������������͹   �

�   �                                                       �   �

�   �    Enter Search String:    [Hun                 ]     �   �

�   �                                                       �   �

�   �������������������������������������������������������Ķ   �

�   �        Found:   Hun                                   �   �

�   �������������������������������������������������������Ķ   �

�   �                                                       �   �

�   �                                                       �   �

�   �        Tracking Station Name [HUNTSVILLE, AL       ]  �   �

�   �        Keep Reading Tracking Data File (y/n) [n]      �   �

�   �                     HUNTSVILLE, AL                    �   �

�   �      Enter Altitude Above Sea Level (M) [228.6  ]     �   �

�   �      Enter Hours From UT, i.e. CST = -6 [-5]          �   �

�   �      Enter Daylight Savings, i.e. 1 = Daylight [1]    �   �

�   �      Enter 3 Character Timezone Name, i.e. CST [CDT]  �   �

�   �������������������������������������������������������ͼ   �

���������������������������������������������������������������ͼ

the program pauses for a few seconds to allow the user to review

the data for any errors.

TRAKSAT  Satellite Tracking Program                           Page 11

******************

* IMPORTANT NOTE *

******************

The daylight/standard time flag, (and time zone name), are NOT

used in ANY CALAULATIONS for TRAKSAT. IT IS ONLY TO REMIND THE

USER OF ANY TIME CHANGES. All of the calculations are done in UTC

times.

******************

* IMPORTANT NOTE *

******************

If the tracking station changes from the default values the file

TRAKSAT.DEF will hold the last saved tracking station data. While

running the program if a new tracking station is selected the

user will be asked if the old tracking station data should be

overwritten or not.

If the user saves the current data then the next time TRAKSAT is

run that new data will be the default else the old TRAKSAT.DEF

will be used.

A text editor can be used to change the TRAKSAT.DEF data also,

the user will need to use some caution with this method. The TWO

EXCEPTIONS are the multi-track options, see the section on its

recommended use (MAIN MENU OPTION EIGHT) and the user defined

area, see section; Analytical Solution.

After the tracking station has been chosen the main menu will

appear waiting for the next user choice.

TRAKSAT  Satellite Tracking Program                           Page 12

REAL-TIME MODE (MAIN MENU OPTION THREE)

If the user would like to track in real-time, press 3, the program

defaults to this mode at start-up. The screen will display the

current time mode (i.e., real or delta modes) by changing the

main menu background colors on the time modes and placing an arrow

on the current option.

The real-time mode will update the screen as fast as the hardware

will allow. For an XT class machine with no coprocessor, the

update time may be 1 to 2 seconds. An AT class computer with a

coprocessor can whip along at about 0.4 seconds per update. The

powerful and fast 386 coprocessor equipped machine can sing along

at 0.1 seconds per update. The average user will not require this

great of detail but it is included for the advanced user.

******************

* IMPORTANT NOTE *

******************

The time is read from the system clock, and as such is only as

accurate as the setting of this clock. The software date and time

can be set before running TRAKSAT to insure the correct time.

Refer to your DOS manuals to use the time and date functions.

A brief note about tracking satellites.

The accuracy of the data is the most important part of the

prediction process. NORAD does track some 8000+ objects in orbit

around the earth, and maintains a data base of the objects. The

earth modeling and perturbations are the most important factors

in satellite tracking. This program uses the NORAD element sets

mainly because they are available and have reasonably good

accuracy.

If the user would like to "see" a satellite in the night sky the

precision of 1 or 2 seconds is not important, several minutes may

not even be that important. This is not to say that the average

person can not locate the satellite, it is going to pass over

some site sooner or later, its the time of the passing that is of

importance.

It could be said that if you tell me where to look for the

satellite and tell me about when I should be looking for it the

chances are it will be spotted. The sky is a big place and it

would be almost impossible to locate a satellite without any help

from programs such as TRAKSAT.

TRAKSAT  Satellite Tracking Program                           Page 13

DELTA-TIME MODE (MAIN MENU OPTION FOUR)

If the user would like to track a satellite from say todays date

to some future date, the delta time mode is the choice to use.

The basic idea is track from some starting date to some stopping

date. At any rate the user will see the screen below with the

delta time mode screen;

����������������������������������������������������������ͻ

�                                                          �

�                                                          �

�     �����������������������������������������������ͻ    �

�     �                DELTA TIME MODE                �    �

�     �����������������������������������������������Ķ    �

�     �          STARTING DATE AND TIME (UT)          �    �

�     �                                               �    �

�     �     YEAR   [1990]                             �    �

�     �     MONTH  [  ]                               �    �

�     �     DAY    [  ]                               �    �

�     �     HOUR   [  ]                               �    �

�     �     MINUTE [  ]                               �    �

�     �     SECOND [  ]                               �    �

�     �     TIME STEP (MIN) [         ]               �    �

�     �                                               �    �

�     �              Press Esc to QUIT                �    �

�     �����������������������������������������������ͼ    �

�                                                          �

�                                                          �

����������������������������������������������������������ͼ

the user will need to "fill in the blanks". The program will

display "defaults" which the user will press return to accept. If

the default is not correct the user will be required to enter in

a new default. At the start of this section the default date and

time are the current local date and zero hours UTC, however, if

the user changes the data the next time this section is called

the "new" default data will be used. At any time during the

delta mode data entry the user presses the Esc key the program

will stop the data entry and the Main Menu will appear.

It is noted that the maximum length, that is from the starting

date to some future time, of the simulation is 99 days 99 hours

59 minutes 59 seconds.

An example is included for the user to "get the idea" on entering

the starting date information. This example starts on 26

december, 1989 at 0 hours UTC, and uses a 1 minute time step. The

user can enter smaller or larger time steps depending on the

requirements of the user.

TRAKSAT  Satellite Tracking Program                           Page 14

����������������������������������������������������������ͻ

�                                                          �

�     �����������������������������������������������ͻ    �

�     �                DELTA TIME MODE                �    �

�     �����������������������������������������������Ķ    �

�     �          STARTING DATE AND TIME (UT)          �    �

�     �                                               �    �

�     �     YEAR   [1989]                             �    �

�     �     MONTH  [12]                               �    �

�     �     DAY    [26]                               �    �

�     �     HOUR   [00]                               �    �

�     �     MINUTE [00]                               �    �

�     �     SECOND [00]                               �    �

�     �     TIME STEP (MIN) [1.0  ]                   �    �

�     �                                               �    �

�     �                                               �    �

�     �����������������������������������������������ͼ    �

�                                                          �

����������������������������������������������������������ͼ

An approach most people use is to pick a 2-3 minute time step and

check the output for any passes near the tracking station for

that day. Then return back to the delta time mode and use a

smaller time step to obtain a better estimate of the satellite

visibility. Another method is to use the analytical solution

option, see the section on Advanced Features for more

information.

The next step for the user is the length of time for the

propagation. The format is days, hours, minutes, and seconds.

Below is an example for 0 days, 12 hours, 30 minutes, 0 seconds;

����������������������������������������������������������ͻ

�                                                          �

�     �����������������������������������������������ͻ    �

�     �                DELTA TIME MODE                �    �

�     �����������������������������������������������Ķ    �

�     �            LENGTH OF PROPAGATION              �    �

�     �                                               �    �

�     �     DAY    [00]                               �    �

�     �     HOUR   [12]                               �    �

�     �     MINUTE [30]                               �    �

�     �     SECOND [00]                               �    �

�     �                                               �    �

�     �                                               �    �

�     �                                               �    �

�     �                                               �    �

�     �                                               �    �

�     �����������������������������������������������ͼ    �

�                                                          �

����������������������������������������������������������ͼ

after the length of propagation is entered the program will

return to the main menu.

TRAKSAT  Satellite Tracking Program                           Page 15

GRAPHICS (MAIN MENU OPTION FIVE)

Option number five, from the main menu, controls the graphics

output. If the user selects the graphics output a new menu will

be displayed with several options for the user to choose from.

A graphics menu screen will appear such as;

��������������������������������������������������ͻ

�                                                  �

�          �����������������������������ͻ         �

�          �           TRAKSAT           �         �

�          �����������������������������͹         �

�          �        GRAPHICS MENU        �         �

�          �����������������������������͹         �

�          �                             �         �

�          �   (1) Ground Tracks         �         �

�          �   (2) Star Background       �         �

�          �   (3) 3D Projections        �         �

�          �   (4) Return To Main Menu   �         �

�          �     Enter Option (1 - 4)    �         �

�          �����������������������������ͼ         �

�                                                  �

��������������������������������������������������ͼ

The options available from this menu control the graphics output.

The user has the choice of satellite ground tracks or star

background plots. If the user selects option 1 from the graphics

menu a satellite ground track will be produced, while a option 2

will display a star background. The star background option will

display visible stars from a database of 58 navigational stars,

the Sun, the planets, and the moon.

The term "star background" will be used in this document to mean

the 58 navigational stars, the Sun, the planets (not including

the earth), and the moon.

******************

* IMPORTANT NOTE *

******************

The star data is internal to TRAKSAT, i.e., there are no

provisions for the user to MODIFY THE INTERNAL STAR DATA at this

time. Star positions are for Epoch J2000.0, from USNO Floppy

Almanac 1988, Version 2.11.88, file STAR1.CAT. HOWEVER THE USER

CAN SUPPLY ADDITIONAL STAR DATA TO THE PROGRAM, see the section

on USER DEFINED STAR DATA.

The star background is a view looking from the tracking site

towards the stars. This plot will be useful for producing a "star

map" to take outside with you to compare the night sky with the

satellite path.

The option to print out this "star map" is included in TRAKSAT,

as of version 1.80 and up. The user will require a Epson FX-80

or compatible printer to use this feature. Again this will prove

valuable to the user in determining where and when to look to

"see" the satellite, this option will work for LOS or Optical

TRAKSAT  Satellite Tracking Program                           Page 16

visibility modes. To print out a screen copy the user will only

have to press a "P" or "p". After the output is printed the

program will continue with the plot. The user can interrupt the

printer process by pressing any key, the printer output will stop

and the program will continue.

******************

* IMPORTANT NOTE *

******************

The printer MUST be connected to parallel printer port number 1,

LPT1. To stop the printer output the user can press any key. A

Epson FX-80 or graphics compatible printer must be used for any

output. The print out routines have been tested on several

different Epson compatible printers without any problems. Some

laser printers can emulate the Epson printers and will produce

good results also.

THE DEFAULT NUMBER OF PRINTER PINS (9 OR 24) IS READ FROM THE

FILE TRAKSAT.DEF. The user can change the "Printer Pins = 9" to

"Printer Pins = 24" as required for the printer being used.

******************

* IMPORTANT NOTE *

******************

If the user selects any graphic output the program will test for

a graphics adapter and based on the type of graphics hardware

will select the "highest" graphics mode supported. An example

would be;

VGA mode 640x480 pixels,

EGA mode 640x350 pixels,

CGA mode 640x200 pixels,

HGC mode 720x348 pixels (Monochrome).

******************

*  SPECIAL NOTE  *

******************

The Hercules graphics mode requires running the driver

MSHERC.COM, this is the driver supplied with several Microsoft

programing languages, before using the TRAKSAT program. Type

"MSHERC" and then "TRAKSAT" to start the program. (A TRUE

Hercules card works with TRAKSAT, some clone cards may NOT.)

I can not test this mode as I do not have any 100% Hercules

graphic cards. Dave Ransom (author of ASTRO CLOCK and good

friend) has tested TRAKSAT on a TRUE Hercules card and had NO

MAJOR PROBLEMS. A few problems with the screen cursor but NOT

serious. (TRAKSAT version 2.20 has corrected the cursor

problems.) Many thanks to Dave for his testing!

Do not use a Microsoft mouse AND the Hercules graphics cards

together, according to Microsoft, as this may cause some

"problems". Any mouse drivers that are installed should be

removed before running TRAKSAT with a Hercules card also.

If the hardware does NOT support graphics an error message will

TRAKSAT  Satellite Tracking Program                           Page 17

be displayed and the program will return to the main menu. All of

the text modes will still be available to the user however.

******************

* IMPORTANT NOTE *

******************

****************************************************************

If the user has a computer that is not 100% compatible or the

graphic card is not 100% VGA compatible the program can be forced

to use the EGA mode. To use this method invoke the program with

"TRAKSAT\EGA", without the quotes. This will set the display to

EGA modes only and MAY eliminate any graphic problems

encountered. NO OTHER GRAPHIC MODES CAN BE SET THIS WAY.

****************************************************************

(GRAPHICS MENU OPTION 1)

If the ground track option is entered (1) the program proceeds to

draw a Mercator projection map of the world. The upper left

corner is at latitude 90 degrees and longitude -180 degrees,

while the lower right corner is latitude -90 degrees and

longitude 180 degrees. The grid spacing is 30 degrees for both

the latitude and the longitude. A "+" will be plotted for the

tracking station coordinates, the coordinates from TRAKSAT.DEF or

the currently loaded data.

The plotting process may take a minute or two on a slow XT type

computer, something under 2 seconds on the particular computer I

use.

The user will be asked about displaying the sun terminator. The

sun terminator is not dependent on the satellite but rather the

tracking station. The terminator is plotted based on a zero sun

elevation angle. (That is to say the terminator is the line where

the sun is "just" below the local horizon.) The terminator is

accurate to about 10-15 minutes of actual. The effects of the

local horizon and atmospheric refraction can "shift" the

terminator a few minutes anyway so great accuracy may not be

achieved. The sun is plotted as an "+" in the center of the

terminator. This will tell the user on what side of the

terminator the tracking station is on. (Daylight or darkness.)

The sun terminator will be "refreshed" every 4 minutes in either

the real or delta time modes. The refresh may take several

seconds on the slower machines. (About 20 seconds.)

******************

* IMPORTANT NOTE *

******************

THE SUN TERMINATOR PLOTTING PROCESS MAY TAKE A MINUTE OR SO ON A

SLOW XT TYPE COMPUTER, something under 1 second on the particular

computer I use.

The ground track plot is the only option that will display the

sun terminator.

The file EARTH.DAT contains the world map data, some 8200 points

in all. This file is compressed to save space and reduce the

reading time. (TRAKSAT version 2.10 and up has improved the

TRAKSAT  Satellite Tracking Program                           Page 18

compression ratio (45%), older versions of the program can NOT be

used with this new data file, nor can TRAKSAT use the older data

file.)

If the EARTH.DAT file is not found an error message will be

displayed and the program will return to the main menu again.

After the world map is displayed the simulation begins. The

starting position for the satellite is marked as a yellow filled

(if that is the user defined color) circle, this was added to

help locate the starting position. The screen will plot the

orbital ground trace of the chosen satellite along with other

valuable data. The top line will have the UTC date and time,

while the second line will have the local date and time

displayed. The lower lines will have the tracking data displayed.

An example of the output screen would be;

-----------------------�   TRAKSAT Version 1.80  �--------------------

|                                                                      |

|   UTC   21:37:26.1  Date 12/26/1989  Satellite Name:   MIR           |

|   Local 15:37:26.1  Date 12/26/1989  Tracking Station: HUNTSVILLE,AL |

|                                                                      |

|        (The version number may be different in this display.)        |

|              (no world map drawn in this example)                    |

|                                                                      |

|  Lat     45.1635�  Azimuth    309.1281�  Range     7115.4 Km         |

|  Long  -175.3926�  Elevation  -30.1331�  Rev # 22120 NOT Visible     |

|                                                                      |

-----------------------------------------------------------------------

The Lat and Long are the satellite latitude and longitude. The

Azimuth and Elevation are as seen from the tracking station,

while the Range is the distance from the satellite to the

tracking station.

The azimuth is always between 0 and 360 degrees with north being

0, east 90 south 180 and so on. The elevation will be always be

between -90 and +90 degrees. If the elevation is less than zero

the satellite is below the horizon as seen from the tracking

station.

The Rev # is based on the input data starting revolution number

plus the number of revs per day times the days past the epoch

date, i.e. the formula;

rev = rev_epoch + (mean motion(rev/day) * (epoch date - date).

The epoch refers to the NORAD satellite data set, see section;

NORAD/NASA 2-Line Satellite Data, for a full explanation of the

input data.

The last item displayed is based on if the satellite is visible

from the tracking station. See main menu option seven for a

complete description of the methods used by TRAKSAT to test for

visibility.

TRAKSAT  Satellite Tracking Program                           Page 19

******************

* IMPORTANT NOTE *

******************

To stop the display the user can press any key and the screen

will "freeze". The user will need to press any key again to

continue the simulation. If the user presses Esc, escape key, the

simulation will stop and the user will be returned to the main

menu.

******************

* IMPORTANT NOTE *

******************

The ground track will continue until the user stops the

simulation by pressing Escape (Esc). After 8-9 ground tracks have

been plotted the screen will be "very busy", the user can re-draw

the screen by pressing Esc, than pressing main menu option five

and one again. The world map will be drawn again along with the

new orbital ground tracks. This will cut down on the screen

"clutter".

The user can print out the screen output with the ground track

option. To print the output of the screen press "P" or "p",

without the quotation marks. The print out may take a minute or

so depending on the type of printer being used. The print out

will start with the upper right side of the screen.

TRAKSAT  Satellite Tracking Program                           Page 20

(GRAPHICS MENU OPTION 2)

If the graphics menu option 2 has been chosen the program will

ask the user for some additional data. The user can display the

visible star names if so desired. (Only the internal star data

names will be displayed, not the user defined star data.) The

user will choose what direction to look, i.e., North, East,

South, or West. The field of view of the star background is 180

degrees in azimuth and 0 through 90 degrees in elevation. If the

user selects North the visible range of the azimuth will be 270

degrees (west) to 90 degrees (east). If the user selects East the

visible range of the azimuth will be 0 degrees (north) to 180

degrees (south) with 90 degrees being the center of the screen

(due east). The option South will display from 90 degrees (east)

to 270 degrees (west). The option for West will display from 180

degrees to 360 degrees with 270 degrees (west) being the center

of the screen (due west).

An example could be to see a satellite rise out of the west the

user would select W, that will display from due south through

west to the north. The screen will appear such as;

��������������������������������������������������ͻ

�          �����������������������������ͻ         �

�          �           TRAKSAT           �         �

�          �����������������������������͹         �

�          �        GRAPHICS MENU        �         �

�          �����������������������������͹         �

�          �                             �         �

�          �   (1) Ground Tracks         �         �

�          �   (2) Star Background       �         �

�          �   (3) 3D Projections        �         �

�          �   (4) Return To Main Menu   �         �

�          �     Enter Option (1 - 4)    �         �

�          �����������������������������ͼ         �

�       ����������������������������������ͻ       �

�       �  Display Star Names (y/n)? [Y]   �       �

�       �                                  �       �

�       �  User Star Database (y/n)? [N]   �       �

�       �                                  �       �

�       �  Looking N,E,S,W (N,E,S,W) [W]   �       �

�       ����������������������������������ͼ       �

��������������������������������������������������ͼ

By default TRAKSAT will display the star names, including the

planets and the moon, not use a user defined star database, and

look west. The user will only need to change these options by

typing in the correct response.

If the user selects the option to include a user defined star

database the program will ask for the file name. If the file is

not found the program will continue and display the internal star

database only. The FIRST 1500 stars will be read from the user

defined file and then the program will continue. The user will

have the responsibility to check the user defined star data for

duplicates of the TRAKSAT internal star data. (See the section on

TRAKSAT  Satellite Tracking Program                           Page 21

PROGRAM LIMITATIONS AND ASSUMPTIONS for a list of the internal

stars used in TRAKSAT.)

*****************************************************************

** REMEMBER TO USE EPOCH J2000 FOR ANY USER DEFINED STAR DATA. **

*****************************************************************

An example of a star background would be;

UTC   14: 2:12.2  Date  3/26/1990  Az   211.3715�  Rev# 23519

Local  8: 2:12.2  Date  3/26/1990  El   -43.8641�  Vis  NOT Visible

----------------------------------------------------------------

|    Satellite: Mir       TRAKSAT 1.85                         |

|                                                              |

75-                                                              |

|                                                              |

|       O Moon                                                 |

60-                                                              |

|                                                              |

|                           EXAMPLE                            |

45-                       (NO PLOT SHOWN)                        |

|                  (This is only an example, the               |

|                   DATA displayed is NOT correct)             |

|                                                              |

|                                                              |

30-                       o Venus                                |

|                                                              |

|                                                              |

15-                                               . Alpharatz    |

|                                                              |

|                            west                              |

----------------------------------------------------------------

210        240         270        300        330

the side axis is the elevation while the bottom axis is the

azimuth. (Observer coordinates are the only output type at this

time). In this example star names were displayed and the option

to look west was selected. The "sky track" of the satellite will

also be plotted on the screen. The top two lines will display the

local and UTC time and dates, azimuth, elevation, rev, and if the

satellite is visible (based on the setting of option 7 LOS or

Optical). The star map background will remain black even if the

sun has risen. (The version number may be different in this display.)

******************

* IMPORTANT NOTE *

******************

If the delta time mode is selected time steps are marked on

the screen and labeled for the user. It is recommended that the

delta time mode be used for star backgrounds so a "time tag" can

be placed on the screen. The real time mode will not place "time

tags" on the screen. This approach was used to help reduce screen

clutter, i.e., to many time tags. This method was chosen because

few people have laptop computers "out in the field" to use for

viewing aids. A printed "sky map" generated before the nights

viewing will be of much greater use to most people.

TRAKSAT  Satellite Tracking Program                           Page 22

******************

* IMPORTANT NOTE *

******************

It is noted that the star background will be refreshed EVERY 15

MINUTES, in either delta or real time modes. It is therefore wise

to select a starting time about 15 minutes PRIOR to the time of

interest and run the program in the delta mode until PAST the

time of interest. An example of this would be;

Time of interest 11:30:00 UTC,

Starting time 11:23:00 UTC,

Time span 00:14:00.

This will provide the user with the "sky map" from 11:23 to 11:37

UTC and avoid the screen refresh.

******************

* IMPORTANT NOTE *

******************

To stop the display the user can press any key and the screen

will "freeze". The user will need to press any key again to

continue the simulation. If the user presses ESC, escape key, the

simulation will stop and the user will be returned to the main

menu.

(GRAPHICS MENU OPTION 3)

If the user presses option 3 from the graphics menu a 3-D

orthographic projection of the earth and the satellite will be

drawn. The perspective will be centered on the tracking station

coordinates with the altitude above the earth being set by the

semi-major axis of the satellite to view. A small "X" will mark

the tracking station coordinates. The grid lines are drawn 10

degrees apart with the orthographic projections.

The 3-D graphic will plot the "orbital trace" above the planet,

i.e., not the ground trace, WHILE HOLDING THE EARTH STILL. The

earth will be non-rotating with the satellite going around it.

The close earth satellites (1000 Km altitude and less) will

produce a good quality plot while near geosync satellites will

display the earth as a small hard to see "ball". The highly

eccentric orbits (ecc. > .3) will not display the complete

orbital trace due to some limitations of the methods used in

TRAKSAT. This should not pose much of a concern to most users.

****************************************************************

THE 3-D PROJECTION WILL BE SLOW ON XT TYPE COMPUTERS WITHOUT A

COPROCESSOR SO IF THE USER DOES NOT WANT TO WAIT FOR THE COMPLETE

EARTH TO PLOT OUT PRESS ANY KEY AND THE EARTH LAND MASS PLOT WILL

STOP AND THE SATELLITE VIEW WILL START. (Only the grid lines will

be displayed.)

****************************************************************

The screen can be stopped and started the same way as any other

graphic modes, i.e., any key to freeze and Esc to stop.

TRAKSAT  Satellite Tracking Program                           Page 23

At this time the altitude above the earth is not a user changed

option. If the user would like a polar (either north or south)

projection select Main Menu option 2 and select either the north,

south or even the equator tracking station coordinates.

******************

* IMPORTANT NOTE *

******************

It is recommended that the user edit the TRAKSAT.CTY file and

change the first three tracking station coordinates to his or her

LOCAL LONGITUDE. The user will answer the questions about

tracking station altitude and time zone information based on the

local conditions.

Below is an example of the TRAKSAT.CTY file with the first three

"extra" tracking stations included. The format of the tracking

station is;

City Name     Long. (deg)   Lat. (deg).

'North Pole ',-86.5867,90.0

'Equator ',-86.5867,.0

'South Pole ',-86.5867,-90.0

'Rancho Palos Verdes CA ',-118.403334,33.767501

'Calaveras County, CA ',-120.566667,38.15

'Washington (USNO), DC ',-77.06575,38.920556

'Auburn, AL ',-85.4833,32.6067

'Birmingham, AL ',-86.81,33.5169

'Gadsden, AL ',-86.0114,34.0158

'Huntsville, AL ',-86.5867,34.7317

The TRAKSAT.CTY file can hold a maximum of 1000 tracking stations

in it. (The file included with TRAKSAT has 723 "cities" in it).

TRAKSAT  Satellite Tracking Program                           Page 24

TABULAR OUTPUT (MAIN MENU OPTION SIX)

TRAKSAT can also produce a tabular output of the satellite

tracking data, the output is in a text mode not graphics. If the

user picks main menu option six, the program will display another

menu asking if the output is to go to a file or the screen.

I chose NOT to include the option for a printer mainly because of

all the different printers and the problems that go along with

different hardware. However, the file option output can be edited

and printed out by the user if so desired.

At any rate, the screen will look like;

������������������������������������������������������ͻ

�                                                      �

�       �������������������������������������ͻ        �

�       �      OUTPUT DATA TO SCREEN/FILE     �        �

�       �������������������������������������Ķ        �

�       �                                     �        �

�       �       S = Output to Screen          �        �

�       �       F = Output to File            �        �

�       �                                     �        �

�       �         Choice (S,F) [S]            �        �

�       �                                     �        �

�       �        A = All Passes               �        �

�       �        V = Visible Passes           �        �

�       �                                     �        �

�       �         Choice (A,V) [A]            �        �

�       �                                     �        �

�       �������������������������������������͹        �

�       �       Ra & Dec (J2000 Epoch)        �        �

�       �������������������������������������͹        �

�       �   Do you want to display the        �        �

�       �   Ra & Dec coordinates (Y/N)? [Y]   �        �

�       �������������������������������������ͼ        �

������������������������������������������������������ͼ

the user has to enter S or F. If the user presses any other keys

than the S or F the program will default to using the screen for

the output.

If the user presses the S key the program will display a header

with some of the tracking station data and the units of the data.

Below is an example of the screen output without the Ra & Dec;

Tracking Station:  HUNTSVILLE, AL           Satellite:  Mir

Date    Time (UTC)   Azim      Elev   Range      Lat       Long     Rev

V

HR:MN:Sec    Deg       Deg    Km         Deg       Deg

Thr 25Jan90 01:27:5.550  268.7678 -43.87 9384.23883  -1.65793 -176.815 22575

Thr 25Jan90 01:27:5.600  268.7689 -43.87 9384.00198  -1.65535 -176.813 22575

Thr 25Jan90 01:27:5.660  268.7702 -43.87 9383.72069  -1.65229 -176.811 22575

Thr 25Jan90 01:27:5.710  268.7712 -43.87 9383.48382  -1.64971 -176.809 22575

this example was run using the real-time mode and the default

tracking station, Huntsville, Al..

In this case the LONG is the satellites longitude, positive (+)

TRAKSAT  Satellite Tracking Program                           Page 25

means EAST longitude while negative (-) means WEST longitude.

The column "V" is the visibility flag, i.e., if the satellite is

visible the flag is set to "Y" else it is blank.

The other output quantities are the same as in main menu option

five.

If the user had chosen to include the Ra & Dec the output would be

like;

Tracking Station:  HUNTSVILLE, AL           Satellite:  Mir

Date    Time (UTC)   Azim      Elev   Range      Ra        Dec      Alt

V

HR:MN:Sec    Deg       Deg    Km         HH:MM:SS  DD:MM:SS Km

Thr 25Jan90 01:27:15.38  268.9799 -43.57 9337.76175  21:58:19 -01:05:53 388

Thr 25Jan90 01:27:15.44  268.9811 -43.56 9337.47902  21:58:20 -01:05:42 388

Thr 25Jan90 01:27:15.55  268.9835 -43.56 9336.95618  21:58:21 -01:05:22 388

the user will notice that the latitude and longitude have been

replaced by the Ra & Dec.

******************

* IMPORTANT NOTE *

******************

The Ra & Dec are based on the J2000 epoch, 1,1.5,2000 UTC date,

and are NOT user selectable, perhaps in the next version of

TRAKSAT. The J2000 epoch is the "current" epoch on most star

charts.

The user will notice that the header is stationary just the data

is scrolling. This option is useful for a quick view of tracking

data, since no graphics are used.

If the user was in delta-time mode the step between outputs would

be the delta time step value set in main menu option four.

******************

* IMPORTANT NOTE *

******************

To stop the display the user can press any key and the screen

will "freeze". The user will need to press any key again to

continue the simulation. Pressing ESC will return the user to the

main menu.

The other option, F, will place the tracking data output into a

file. The file name will consist of the first 8 characters of the

satellite name with the extension ".PRT" added to the end. The

name of the output file will be displayed for the user.

An example could be the satellite Mir, the filename for output

would be "Mir.PRT".

******************

* IMPORTANT NOTE *

******************

The program will produce the file xxxxxxx.PRT, the x being the

current satellite name, if one is not found, but will OVERWRITE

TRAKSAT  Satellite Tracking Program                           Page 26

an old one if found. The user will have the responsibility to re-

name the file after completing a run if they would like to save

the output.

The output in the file is very similar to the screen output

option. Below is an example of the file output mode;

TRAKSAT  Version 1.90

Tracking Station:  HUNTSVILLE, AL

[ Line Of Sight (LOS) Visibility ]

Satellite: Mir

Satellite Data Set:

1 16609U 86 17  A 90 91.75081924  .00058269  00000-0  66933-3 0  5052

2 16609  51.6174 355.6250 0013230 341.6181  18.4560 15.61365027236165

Date      Time (UTC)   Azim     Elev     Range    Lat     Long     Rev   V

HR:MN:SEC    Deg      Deg      Km       Deg     Deg

Thr  25Jan90  01:26:42.04   268.26  -44.61   9494.42   -2.86  -177.67  22575

Thr  25Jan90  01:26:42.21   268.26  -44.60   9493.63   -2.86  -177.66  22575

Thr  25Jan90  01:26:42.32   268.27  -44.60   9493.12   -2.85  -177.66  22575

Thr  25Jan90  01:26:42.43   268.27  -44.60   9492.60   -2.84  -177.65  22575

again this example used the real-time mode. This output is a

standard 80 columns, for printers or the 25x80 text screen. In

this example a line of sight (LOS) mode was chosen. If the

visibility mode was optical the header, the line under the

tracking station name, would display; [ Optical visibility ].

If the file mode and the real-time mode are chosen the screen

will display the number of records that have been written to

file. The program DOES check the remaining disk space and stops

the program if the record space exceeds available disk space. The

data prior to exceeding the disk space is written and an error

message is displayed, no data will be lost.

It is recommended that the real-time mode NOT be used for file

output, mainly because of the large files that could be produced.

If the file mode and delta-time mode are chosen the screen will

display the same record count as above, but also the total number

of records to calculate. This method produces the smallest file

size the user requires.

The total number of records to calculate would be;

total_records = (stop_time - start_time)/delta_time.

The size of the file is approximately 81 bytes per record,

therefore 1440 records, one day at one minute intervals, will

produce a file size of about 116K. (ALL TIME STEPS DISPLAYED).

TRAKSAT  Satellite Tracking Program                           Page 27

******************

* IMPORTANT NOTE *

******************

To stop the display the user can press any key and the screen

will "freeze". The user will need to press any key again to

continue the simulation. Pressing ESC will return the user to the

main menu.

The option has been added to TRAKSAT version 1.5, and up, to

display only the visible passes, based on the setting of the flag

for main menu option 7, or ALL passes. The program will default

to ALL if a return is pressed.

The output, in the tabular modes, can display Ra & Dec of the

satellite also. The coordinate used is based on J2000 epoch, this

was chosen to be used with "current" star charts. The default is

to include Ra & Dec in the output.

Remember the default is to include the Ra & Dec in the output,

but by answering No to the question about Ra & Dec, that will

place the satellite latitude and longitude in the output file.

TRAKSAT  Satellite Tracking Program                           Page 28

VISIBILITY (MAIN MENU OPTION SEVEN)

There are two different methods used by TRAKSAT to determine

visibility. The first method is simply when the elevation is

greater than zero degrees the satellite will be visible to the

tracking station. This method is called line of sight (LOS) in

the program. This method would be suitable for monitoring

satellite radio transmissions, interesting RF signals no doubt.

It should be noted that at most tracking sites zero degrees

elevation is not visible due to ground based obstructions, i.e.

trees buildings, and other such objects. A rule of thumb is if

you hold out your arm straight and stick out your thumb

horizontal to the ground so it appears to touch the horizon the

upper edge of your thumb is about two degrees elevation, while

your closed fist is about ten degrees elevation.

The second method, optical visibility, requires the satellite to

be above zero degrees elevation also, however the satellite must

be sun-lit while the tracking station is in darkness. This

method would be used for viewing satellites with the aid of say

binoculars. It is of interest to note that some satellites are

NOT visible even if the elevation angle is above the horizon,

because they are in the earth's shadow. It is difficult to

observe a satellite "coming out" of the earth's shadow, it is

easier to see the entrance into the shadow.

If the lighting conditions are favorable a "bright" satellite can

be seen with the naked eye also. The best time for these

favorable lighting conditions usually occur an hour before sun

rise or an hour after sun set, as seen at the tracking site. The

best type of satellite is low, about 250 - 400 kilometer

altitude, ones for naked eye observations. (Mir, Hubble, Shuttle

are a good examples.)

******************

* IMPORTANT NOTE *

******************

TRAKSAT version 1.60 and above use NAUTICAL twilight, i.e. the

sun is greater than or equal to -12 degrees BELOW the local

horizon, to determine the lighting conditions. The user can not

change the type of twilight used in TRAKSAT, i.e., civil,

nautical, or astrodynamic.

The type of visibility is set from the main menu, the default is

to use the LOS method. If the user would like to change the

visibility method, select main menu option number seven. The

main menu will always print the type of visibility test that

will be performed by the program. This menu option is a toggle

function, i.e. selecting option 7 changes from one method to the

other.

******************

* IMPORTANT NOTE *

******************

With either method the visual magnitude is NOT calculated. Such

a calculation would require knowledge about the emissivity of the

TRAKSAT  Satellite Tracking Program                           Page 29

satellite, and atmospheric conditions, neither of which is readily

available to the user.

TRAKSAT  Satellite Tracking Program                           Page 30

MULTI-TRACKING (MAIN MENU OPTION EIGHT)

TRAKSAT version 1.7 and above have the capability to track

several satellites at the same time. The main menu option number

eight will bring up another menu, this menu is used for the

multi-track option. The multi-track menu will appear asking the

user to select ground track or tabular output modes. After that

selection the program will ask for a real time or delta time

mode.

Below is an example of the multi-track options menu.

�������������������������������������������������������������ͻ

�            ������������������������������������ͻ           �

�            �              TRAKSAT               �           �

�            ������������������������������������͹           �

�            �          MULTI-TRACK MENU          �           �

�            ������������������������������������͹           �

�            �                                    �           �

�            �    (1) Ground Tracks               �           �

�            �    (2) Tabular Output              �           �

�            �    (3) Return To Main Menu         �           �

�            �                                    �           �

�            �        Enter Option (1 - 3)        �           �

�            ������������������������������������Ķ           �

�            �        Satellites To Track         �           �

�            �        File: NASA668.TXT           �           �

�            �    Satellite #1 [MIR         ]     �           �

�            �    Satellite #2 [SALYUT 7    ]     �           �

�            �    Satellite #3 [IRAS        ]     �           �

�            �    Satellite #4 [SEASAT 1    ]     �           �

�            �    Satellite #5 [EGP         ]     �           �

�            �    Satellite #6 [NOAA 9      ]     �           �

�            �                                    �           �

�            ������������������������������������ͼ           �

�������������������������������������������������������������ͼ

The satellite data file is displayed, this can be changed from

the main menu option number one, along with the default satellite

names. The user can return to the main menu by pressing option

number three.

If the satellite data file name is NOT correct then return to the

main menu, press 3, and run option number one. Enter the correct

file name and any search string, this search string will not be

used in the multi-track modes. If the data file is found return

to the main menu and run option eight again, this time the

correct file will be read. The default file name used for both

the single and multi-tracking modes is stored in TRAKSAT.DEF,

and can be edited and changed. It is not the best way to change

the default data but can be used.

The program will read the default data at startup and if any

changes are made DURING A RUN before the program terminates the

user will be asked to save this new data or keep the old data.

TRAKSAT  Satellite Tracking Program                           Page 31

******************

* IMPORTANT NOTE *

******************

The user can ONLY change the satellite names by editing the

TRAKSAT.DEF file and changing the default names. The format for

the satellite names is as follows;

COLUMN

1  - through -   16 CAN BE ANYTHING,

COLUMN 16 STARTS THE SATELLITE NAME,

THE NEXT 12 CHARACTERS USED ARE THE

NAMES OF THE SATELLITES TO TRACK.

Below is an example, this is the data

included in the TRAKSAT.DEF file.

Satellite # 1: MIR

Satellite # 2: SALYUT 7

Satellite # 3: IRAS

Satellite # 4: SEASAT 1

Satellite # 5: EGP

Satellite # 6: NOAA 9

It is recommended that the FULL name of the satellite be used,

the search routine will use the first match found and not look

for any other matches. If the name of a satellite is not found

the output will display a NO DATA, i.e., no data for the

requested satellite has been loaded.

Six satellites MAXIMUM can be tracked, if the TRAKSAT.DEF has

fewer than TWO the program will issue a error and return to the

main menu. (Use the single tracking mode.)

The user has the option of either ground tracks or a tabular

output. If either option, ground tracks or tabular, is chosen

then program will ask the user to use real time or the delta time

modes next.

If the ground track option is chosen the program will proceed to

plot the tracks on the world map. The plots are very similar to

the main menu option five, except that for every satellite a

different colored line is drawn. The order of the colors are;

1st satellite = yellow

2nd satellite = cyan

3rd satellite = green

4th satellite = lite red

5th satellite = lite magenta

6th satellite = lite green.

(The colors can NOT be changed by the user)

******************

* IMPORTANT NOTE *

******************

Users with monochrome monitors will have trouble identifying the

ground tracks in the multi-track mode as no difference in the

colors will be seen. As of TRAKSAT version 1.90 and above a

number (1 - 6) along with the "starting circle" is plotted to

help identify the satellite on the monochrome screen.

TRAKSAT  Satellite Tracking Program                           Page 32

Below is an example of the multi-track ground tracks. It is noted

that no actual graphics plot is included due to the limits of

storing graphics and text together.

-------------------�    TRAKSAT   Version 1.80    �------------------

|   UTC   17:17:36.3  Date  2/20/1990  Satellite Name: MULTI-TRACKING  |

|   Local 11:17:36.3  Date  2/20/1990  Tracking Station: HUNTSVILLE, AL|

|                                                                      |

|        (The version number may be different in this display.)        |

|                (no world map drawn in this example)                  |

|                                                                      |

|    Mir         Salyut 7    IRAS        SeaSat 1    EGP         NOAA 9|

|Azi 32.079      227.66      274.10      220.47      4.9673      2.2960|

|Ele 14.499      -60.75      -50.48      -57.27      33.954      -15.43|

----------------------------------------------------------------------

The output will display the ground tracks, the azimuth as seen

from the tracking station, and the elevation. The elevation is

the angle above or below the tracking station horizon. The output

is in degrees, with the time and date formats the same as option

five. (The version number may be different in this display.)

******************

* IMPORTANT NOTE *

******************

No visibility methods, LOS or optical, are used in the multi-

track GROUND TRACK modes. THE TABULAR OUTPUT MODES WILL DETERMINE

THE VISIBILITY BASED OF THE SETTING OF THE MAIN MENU OPTION

SEVEN, (LOS or Optical). (The user can determine the LOS

visibility by looking at the elevation angles displayed when

using the ground track option.)

MULTI-TRACKING TABULAR OUTPUT MODE

As of TRAKSAT version 2.30 and above a tabular multi-tracking

mode has been included. The output is in text rather than a

graphic display mode. Below is an example of the multi-tracking

tabular output mode. The different satellites will be displayed

in different colors also, the colors are the same as the ground

track options.

TRAKSAT  Satellite Tracking Program                           Page 33

���������������������͵    TRAKSAT  Version 2.30    �����������������������ͻ

�                       Tabular Multi-Tracking Mode                         �

�  Tracking Station: HUNTSVILLE, AL          Input Data File: NASA707.TXT   �

�                  UTC   00:10:00.0  Date  6/19/1990                        �

�                  Local 00:10:00.0  Date  6/18/1990                        �

�����������������������������Ĵ Mir           �����������������������������Ķ

�       Latitude   -12.0376�  Azimuth    283.3060�  Range 11839.43 Km       �

�       Longitude   42.4516�  Elevation  -63.4864�  NOT Visible             �

�����������������������������Ĵ Salyut 7      �����������������������������Ķ

�       Latitude    -5.4339�  Azimuth    249.6659�  Range  7706.61 Km       �

�       Longitude  209.8406�  Elevation  -33.6718�  NOT Visible             �

�����������������������������Ĵ HST           �����������������������������Ķ

�       Latitude    21.5907�  Azimuth    350.3650�  Range 11762.57 Km       �

�       Longitude  102.0782�  Elevation  -60.0983�  NOT Visible             �

�����������������������������Ĵ SeaSat 1      �����������������������������Ķ

�       Latitude    59.6848�  Azimuth    328.8212�  Range  7653.05 Km       �

�       Longitude   20.8611�  Elevation  -29.3966�  NOT Visible             �

�����������������������������Ĵ EGP           �����������������������������Ķ

�       Latitude    -2.4094�  Azimuth    106.5518�  Range  8414.05 Km       �

�       Longitude  338.9396�  Elevation  -27.4664�  NOT Visible             �

�����������������������������Ĵ IRAS          �����������������������������Ķ

�       Latitude    57.6857�  Azimuth    354.3749�  Range  9450.31 Km       �

�       Longitude   82.8507�  Elevation  -39.6712�  NOT Visible             �

�                                                                           �

���������������������������������������������������������������������������ͼ

The output of the multi-tracking mode is similar to the bottom

two lines of a ground track plot. The output will display only

satellites found in the element file.

TRAKSAT  Satellite Tracking Program                           Page 34

QUITTING THE PROGRAM (MAIN MENU OPTION NINE)

This option will stop the TRAKSAT program and return the user to

DOS. If the tracking station data was changed during the program

execution, the user will be asked if the new data should replace

the old default data. That choice is up to the user to decide.

The old data will be displayed along with the current data to help

the user with the choice. THE DEFAULT IS NOT TO SAVE THE NEW

DATA.

USER DEFINED SATELLITE PLOTTING COLOR

TRAKSAT version 2.40 and above allows the user to choose what

color to plot the satellite in. The color is changed in the file

TRAKSAT.DEF. Below is an example of a TRAKSAT.DEF file with the

default satellite plotting color as light yellow (14). To change

the color edit the file TRAKSAT.DEF and change the Color = XX to

any of the colors allowed. (If a incorrect color is chosen the

light yellow will be used as a default.)

COLUMN NUMBER

123456789012345678901234567890123456789012345678901234567890123456

2447836.50000000  Color =  14 ed read.me

Printer Pins = 9

HUNTSVILLE, AL         34.7317000  273.4033000    228.60 -5 0 CDT

NASA678.TXT

Multi-Track Satellite Names

Satellite # 1: MIR

Satellite # 2: SALYUT 7

Satellite # 3: IRAS

Satellite # 4: SEASAT 1

Satellite # 5: EGP

Satellite # 6: NOAA 9

User Defined Satellite Names

IRAS

SALYUT 7

COSMOS 1686

COSMOS 1766

MIR

The user can choose from the pallette of;

2 = GREEN                          9 = LIGHT BLUE

3 = CYAN                          10 = LIGHT GREEN

4 = RED                           11 = LIGHT CYAN

5 = MAGENTA                       12 = LIGHT RED (sun term.)

6 = BROWN                         13 = LIGHT MAGENTA

7 = WHITE (map land mass color)   14 = LIGHT YELLOW

8 = GRAY                          15 = BRIGHT WHITE

TRAKSAT  Satellite Tracking Program                           Page 35

ADVANCED FEATURES MENU OPTIONS

The advanced features option will control access to the "Advanced

Features" menu. If the user selects the zero option the advanced

features menu will appear;

���������������������������������������������������������������ͻ

�                                                               �

�                                                               �

�                                                               �

�          �����������������������������������������ͻ          �

�          �         ADVANCED FEATURES MENU          �          �

�          �����������������������������������������͹          �

�          �                                         �          �

�          �   (0) Reverse Solution                  �          �

�          �   (1) All Satellites In Data File       �          �

�          �   (2) Analytical Rise & Set (LOS)       �          �

�          �   (3) Analytical Rise & Set (Optical)   �          �

�          �   (4) Return To Main Menu               �          �

�          �       Enter Option (0 - 4)              �          �

�          �                                         �          �

�          �����������������������������������������ͼ          �

�                                                               �

�                                                               �

�                                                               �

���������������������������������������������������������������ͼ

The user has five options from this menu, the Reverse Solution,

All or User Defined Satellites, Analytical Rise & Set (LOS)

Solution, Analytical Rise & Set (Optical) Solution, and finally

the return to Main Menu options. If the user selects any other

options than 0 - 4 the program will return to the Main Menu.

TRAKSAT  Satellite Tracking Program                           Page 36

REVERSE SOLUTION (ADVANCED FEATURES MENU OPTION ZERO)

The main menu option 0 will perform the "reverse solution", i.e.,

given a right ascension & declination (Ra & Dec) and time (UTC)

determine what satellite was observed. For this method to work the

user must take accurate right ascension, declination and UTC time

measurements. If the user selects this option a opening warning

screen will appear before entering into the reverse solution

menu. This warning is to remind the user that accurate

measurements (THREE ARE REQUIRED) are needed to obtain acceptable

results. The accuracy of the data should be within the abilities of

most users with a telescope that has Ra & Dec digital read out

or mechanical measurement devises. The observed data set WILL

NEED TO BE FROM THE SAME REVOLUTION NUMBER, i.e., A SINGLE PASS

NEAR THE TRACKING SITE.

TRAKSAT has been tested with several satellites, changing the

observed Ra & Dec and times to better determine the accuracy

requirements of the reverse solution option. The Ra & Dec can be off

about 2 minutes and still produce good results. If the Ra & Dec

is off more then 2 minutes the error grows linearly with time.

Remember that the closer the satellite element data set is to the

observed data the better the match will be. A brief description

of the methods used for the reverse solution is located in

section; (Program Limitations and Assumptions).

The user is given a choice to return to the Main Menu or continue

with the reverse solution from this warning message screen.

At any rate the reverse solution menu will appear as such;

�������������������������������������������������������������ͻ

�  �������������������������������������������������������ͻ  �

�  �                 REVERSE SOLUTION MENU                 �  �

�  �������������������������������������������������������͹  �

�  �                                                       �  �

�  �   Observation #1   Observation #2   Observation #3    �  �

�  �������������������������������������������������������Ķ  �

�  �   Month      [  ]  Month      [  ]  Month      [  ]   �  �

�  �   Day        [  ]  Day        [  ]  Day        [  ]   �  �

�  �   Year     [    ]  Year     [    ]  Year     [    ]   �  �

�  �   Hour (UTC) [  ]  Hour (UTC) [  ]  Hour (UTC) [  ]   �  �

�  �   Minute     [  ]  Minute     [  ]  Minute     [  ]   �  �

�  �   Second     [  ]  Second     [  ]  Second     [  ]   �  �

�  �   Ra Hour    [  ]  Ra Hour    [  ]  Ra Hour    [  ]   �  �

�  �   Ra Minute  [  ]  Ra Minute  [  ]  Ra Minute  [  ]   �  �

�  �   Ra Second  [  ]  Ra Second  [  ]  Ra Second  [  ]   �  �

�  �   Dec Degree[   ]  Dec Degree[   ]  Dec Degree[   ]   �  �

�  �   Dec Minute [  ]  Dec Minute [  ]  Dec Minute [  ]   �  �

�  �   Dec Second [  ]  Dec Second [  ]  Dec Second [  ]   �  �

�  �                                                       �  �

�  �������������������������������������������������������ͼ  �

�������������������������������������������������������������ͼ

the user will "fill in the blanks". Data entered will be checked

for "correct" values, i.e., no 13th month or any other such

errors. The "full" year is used, i.e., 1990 NOT 90.

TRAKSAT  Satellite Tracking Program                           Page 37

After all of the data has been entered the program will display

the search mode screen. The satellite data filename is displayed,

the current filename will be used. The screen will appear as;

���������������������������������������������������������������ͻ

�   �������������������������������������������������������ͻ   �

�   �          READING NASA 2-LINE SATELLITE FILE           �   �

�   �������������������������������������������������������͹   �

�   �              Looking For Matching Data                �   �

�   �������������������������������������������������������Ķ   �

�   �                   File: NASA668.TXT                   �   �

�   �      Possible Match                     Data Fit      �   �

�   �                                                       �   �

�   �                                                       �   �

�   �                                                       �   �

�   �                                                       �   �

�   �                                                       �   �

�   �                                                       �   �

�   �                                                       �   �

�   �                                                       �   �

�   �                                                       �   �

�   �������������������������������������������������������ͼ   �

���������������������������������������������������������������ͼ

As the program finds any "matches", i.e., if the satellite could

be the observed one, the satellite name is displayed along with a

data fit. The data fit is simply the Root Sum Square (RSS) values

of the difference between the measured right ascension and

declination and the calculated values. The RSS value is then

converted into a percent and displayed. A data fit of 100% means

the observation data matched a particular satellite, i.e., the

observed data fit the predicted satellite position almost

perfectly. The data fit is displayed so the user can determine

any possible satellites that could have been seen.

******************

* IMPORTANT NOTE *

******************

If the user would like to change the default filename, FIRST run

main menu option 1 and select the new satellite filename and ANY

satellite data set in that file. This is important because the

reverse solution will search the default satellite data file, the

default data file being the currently loaded in memory or the

filename in the TRAKSAT.DEF file. For more information on

changing the default data file see main menu option one.

An example is included below to help user with the reverse

solution option. (This example is for a UNKNOWN satellite

using elements from the NASA668.TXT file.) The tracking station

coordinates were for Huntsville, Al..

TRAKSAT  Satellite Tracking Program                           Page 38

�������������������������������������������������������������ͻ

�  �������������������������������������������������������ͻ  �

�  �                 REVERSE SOLUTION MENU                 �  �

�  �������������������������������������������������������͹  �

�  �                                                       �  �

�  �   Observation #1   Observation #2   Observation #3    �  �

�  �������������������������������������������������������Ķ  �

�  �   Month      [3 ]  Month      [3 ]  Month      [3 ]   �  �

�  �   Day        [27]  Day        [27]  Day        [27]   �  �

�  �   Year     [1990]  Year     [1990]  Year     [1990]   �  �

�  �   Hour (UTC) [3 ]  Hour (UTC) [3 ]  Hour (UTC) [3 ]   �  �

�  �   Minute     [1 ]  Minute     [3 ]  Minute     [6 ]   �  �

�  �   Second     [0 ]  Second     [0 ]  Second     [0 ]   �  �

�  �   Ra Hour    [2 ]  Ra Hour    [8 ]  Ra Hour    [13]   �  �

�  �   Ra Minute  [38]  Ra Minute  [12]  Ra Minute  [10]   �  �

�  �   Ra Second  [51]  Ra Second  [22]  Ra Second  [4 ]   �  �

�  �   Dec Degree[46 ]  Dec Degree[56 ]  Dec Degree[-21]   �  �

�  �   Dec Minute [47]  Dec Minute [29]  Dec Minute [17]   �  �

�  �   Dec Second [4 ]  Dec Second [44]  Dec Second [22]   �  �

�  �                                                       �  �

�  �������������������������������������������������������ͼ  �

�������������������������������������������������������������ͼ

After all of the observed data was entered the next screen

will appear such as;

���������������������������������������������������������������ͻ

�   �������������������������������������������������������ͻ   �

�   �          READING NASA 2-LINE SATELLITE FILE           �   �

�   �������������������������������������������������������͹   �

�   �              Looking For Matching Data                �   �

�   �������������������������������������������������������Ķ   �

�   �                   File: NASA668.TXT                   �   �

�   �      Possible Match                     Data Fit      �   �

�   �      Name: Salyut 7                       100%        �   �

�   �                                                       �   �

�   �                End Of Data File                       �   �

�   �                Number Of Matches = 1                  �   �

�   �                                                       �   �

�   �                                                       �   �

�   �                                                       �   �

�   �                                                       �   �

�   �                                                       �   �

�   ȵ Press Return ���������������������������������������ͼ   �

���������������������������������������������������������������ͼ

In this example the observed satellite was the Salyut 7, matching

the observed data 100 percent.

******************

* IMPORTANT NOTE *

******************

Due to the fact that about 140 of the more than 8000 satellite

data sets are included in a typical data file, most of the

observed data sets will not find a matching satellite. The only

cure would be to have ALL 8000 satellite data sets, this seems

HIGHLY unlikely for the average user let alone the advanced user.

TRAKSAT  Satellite Tracking Program                           Page 39

ALL/USER DEFINED SATELLITES (ADVANCED FEATURES MENU OPTION ONE)

The user has the choice with the analytical solution options to

select all of the satellites in the data file or some user

defined satellites. The default will be to read ALL of the

satellites for these options. The user can define his or her

"favorite" satellites, up to 25 satellites can be included. The

user will need to use a word processor to edit the TRAKSAT.DEF

file and add the satellites names to it. The word processor used

will need to save the file in PLAIN ASCII format, i.e., NO

SPECIAL CONTROL CHARACTERS EXCEPT THE END OF FILE MARKER. The

edlin or PC-Write word processors will do the job nicely for the

user. An example of the TRAKSAT.DEF file with user defined

satellites is included below;

column number

123456789012

2447836.50000000 Color = 14

HUNTSVILLE, AL         34.7317000  273.4033000    228.60 -5 0 CDT

NASA678.TXT

Multi-Track Satellite Names

Satellite # 1: MIR

Satellite # 2: SALYUT 7

Satellite # 3: IRAS

Satellite # 4: SEASAT 1

Satellite # 5: EGP

Satellite # 6: NOAA 9

User Defined Satellite Names

IRAS

SALYUT 7

COSMOS 1686

COSMOS 1766

MIR

******************

* IMPORTANT NOTE *

******************

Remember to start the user defined satellite name in column 2.

The next 12 characters make up the satellite name, upper or lower

case makes no difference. If a user defined satellite is NOT

located in the satellite data file a warning message will be

displayed. The user defined satellites can be ANY of the

satellites included in the data file, however, 25 satellites is

the maximum number allowed. If the user has more than 25

satellites then select the All Satellite Data Sets option.

The user defined option is a toggle switch, i.e., if the menu

shows "All Satellites In Data File" and the user presses option 1

the menu will reappear displaying the "User Defined Satellite"

message. The user can toggle back and forth between these two

options.

TRAKSAT  Satellite Tracking Program                           Page 40

ANALYTICAL RISE & SET (ADVANCED FEATURES OPTION 2/3)

TRAKSAT version 2.00 and above has included a very powerful

option, Analytical Rise & Set. Many people have asked "why use

this analytical approach ?". Three reasons come to mind speed,

speed, and speed! The analytical approach used is a closed form

solution to the problem of determining when a satellite can be

seen (either LOS #2 or Optical #3) by a ground tracking station.

In effect, this problem usually involves the calculation of the

rise-and-set time (UTC) of a given satellite from a specific

ground tracking station.

In the past, it has been the custom to solve the problem by

letting the satellite run through its ephemeris, and checking at

each instant to see whether the elevation angle of the satellite

was greater than some minimum value. However, by attacking the

problem from a different point of view, that is, with the

eccentric anomaly taken to be the independent variable, it is

possible to obtain a closed-form solution to the satellite

visibility problem. Specifically, the closed-form solution is a

single transcendental equation in the eccentric anomalies

corresponding to a rise-and-set time for a given orbital pass of

a satellite. It is more difficult to solve the controlling

equation than the standard Keplerian equation. However, the

method offers the advantage that the controlling equation is

solved only once per orbit period as contrasted with the hundreds

of times the Keplerian equation must be solved with the standard

step-by-step technique.

"How much faster is the analytical solution ?" Several

"benchmarks" were run using the same satellite data sets and

starting times to determine the speed of each method. If the user

selects the Delta Time Mode and then the Analytical Solution the

speed difference will be obvious.

******************

* IMPORTANT NOTE *

******************

The analytical solution methods will use 10 DEGREES AS THE

MINIMUM ELEVATION ANGLE that the viewer can "see". In practice

that "assumption" is very well founded. The minimum is NOT user

selectable in this case.

An example of the speed comparison is in order to back-up these

claims. The user is invited to try this "test" to better

determine the speed advantage the analytical solution will

provide. The steps to complete this "test" are:

1. Edit the TRAKSAT.DEF file and place ONLY ONE satellite

name under the User Defined area.

2. Run TRAKSAT and select Main Menu option 1 (read in

satellite data) and select the satellite name.

3. The next step is to run Main Menu option 4 (delta time

mode) from say 4-20-1990 0 UTC for 24 hours by 1 minute

steps. (Any date can be substituted.)

TRAKSAT  Satellite Tracking Program                           Page 41

4. Answer the prompts to place output into a File of only

the Visible passes and Yes to the Ra & Dec question.

5. Use a watch, or a stopwatch to time how long the

simulation takes and make a note of the time.

6. Now return to the Main Menu and select the Advanced

Features option 0, and press option 1 (User Defined

Satellites) and then option 2 or 3 (use the same LOS

or Optical visibility test as before). Answer Yes to the

question about the same times. The output MUST be

directed to a FILE. The program will display how long

the analytical solution method took to solve the SAME

TIME PERIOD as the delta time mode.

The conclusions will be easy to figure out; the Analytical

Solution will be some 20+ TIMES FASTER than the Delta Time Mode,

(OVER THE SAME TIME PERIOD).

On the machine used for testing (coprocessor installed) the

analytical solution runs about 4 seconds per satellite per 24

hours of simulation time vs. the 120 seconds per 24 hours in the

delta time mode. (This was on a 386/16 with a 287 coprocessor.)

(A 386/33 computer with a 387 can crank out 160 satellites per

minute in this "benchmark"!!!!)

Use the analytical solution for "rough" estimates and the delta

time mode for the detailed analysis. The term "rough" implies

that the analytical solution is not as accurate as the Delta Time

mode, and in fact that is true. The error (Delta Time mode vs.

Analytical Solution) is usually LESS then 1 minute for predicted

rise or set times. (Remember that the analytical solution will

display results for a satellite elevation ABOVE 10 degrees.) The

error is the price to pay for the speed advantage.

******************

* IMPORTANT NOTE *

******************

The analytical solution DOES INCLUDE THE DRAG EFFECTS in the

calculations, (as of TRAKSAT version 2.30). By including the

drag effects in the solution a long term prediction can be done

without a loss of accuracy.

It has proved helpful for several TRAKSAT users to predict

several weeks into the future any visible passes and make a note

of the dates. As the date approaches and newer elements become

available the user can "improve" the viewing times to finally

obtain a accurate prediction of the satellites. Using this

approach the user can mark "special" days when a particular

satellite should be seen.

The analytical solution will save the user from "looking" for

satellites that will not be seen, or unfavorable passes.

******************

* IMPORTANT NOTE *

******************

If the user selects the analytical solution AND the OPTICAL

TRAKSAT  Satellite Tracking Program                           Page 42

visibility test then only satellites with a mean motion of

greater than 2.5 revs per day will be calculated. That is any

satellite that is "near geosync" will not be calculated. This

approach was necessary due to some of the limits of the

analytical solution. This should NOT be of to great importance to

most users as seeing ANY near geosync satellites is VERY

DIFFICULT. The line of sight (LOS) method will still work for the

near geosync satellites. If the user needs to "see" a near

geosync satellite than the delta time mode will be required.

That is enough "horn blowing" let us look at an example.

Below is and example output from the analytical solution using

the screen output option. The visibility method in this example

was LOS (line of sight) while the starting date was 4-20-1990 @ 0

UTC hours. The end time was 24 hours. The starting times are

entered through the same type method as used in the Delta Time

mode. (The time step is NOT entered.) The user is directed to the

section on Delta Time mode for further study on entering starting

times.

The "header" at the top of the display will show some vital

information to the user. The tracking station name and satellite

data file name along with the visibility method will be displayed

to remind the user of the current settings.

******************

* IMPORTANT NOTE *

******************

To change the input file name the user will need to return to the

Main Menu and select option 1, and enter the new satellite data

file name along with ANY satellite name in that file. (This name

will not have any special meaning to the analytical solution.)

(SCREEN OUTPUT OPTION)

TRAKSAT Version 2.00   Analytical Solution

Tracking Station: HUNTSVILLE, AL       File: NASA678.TXT   Visibility: LOS

Satellite    UTC     TIME     LOCAL   TIME      AZIMUTH  MAX   MIN    DURATION

DATE    HR:MN:SC DATE    HR:MN:SC           ELE   RANGE  HR:MN:SC

Alouette 1   20Apr90 01:08:32 19Apr90 20:08:32  N  TO SE 25    1905   00:10:32

20Apr90 02:54:12 19Apr90 21:54:12  NW TO S  43    1387   00:12:12

Cosmos 398   20Apr90 01:19:02 19Apr90 20:19:02  NW TO E  33    1970   00:16:02

20Apr90 03:34:48 19Apr90 22:34:48  NW TO SE 74    2069   00:26:48

20Apr90 05:54:02 20Apr90 00:54:02  W  TO S  21    4272   00:23:02

20Apr90 20:54:16 20Apr90 15:54:16  NW TO N  10    1100   00:01:16

21Apr90 01:27:19 20Apr90 20:27:19  NW TO E  39    1879   00:18:19

Starlette    20Apr90 02:27:41 19Apr90 21:27:41  NW TO E  35    1582   00:13:41

Working ...

Press Esc to Quit

If the Optical method was selected the header message will

display so.

******************

* IMPORTANT NOTE *

******************

The user can NOT stop/start the screen as in the other modes,

TRAKSAT  Satellite Tracking Program                           Page 43

i.e., pressing Esc will STOP the display and terminate the

analytical solution. This method was chosen to avoid

inadvertently waiting for the screen to update while in a pause

mode. The screen update can be slow on a Optical visibility test

and a NON-coprocessor equipped machine. (There may not be any

satellites optical visible at all!)

If a file output was selected the header placed in the file has

the same information as the screen header. Below is an example of

the analytical solution file output. (The same times as above

were used but this example used the optical visibility test.)

Below is the screen or file output option menu.

�����������������������������������������������������������ͻ

�           ������������������������������������ͻ          �

�           �              TRAKSAT               �          �

�           ������������������������������������͹          �

�           �       ANALYTICAL RISE & SET        �          �

�           ������������������������������������͹          �

�           �                                    �          �

�           �      (F) Output To File            �          �

�           �      (S) Output To Screen          �          �

�           �          LOS Visibility            �          �

�           �       Enter Option (F,S) [F]       �          �

�           �                                    �          �

�           �                                    �          �

�           �                                    �          �

�           ������������������������������������ͼ          �

�����������������������������������������������������������ͼ

******************

* IMPORTANT NOTE *

******************

The default is to write output to the file TRAKSAT.OUT. This is

NOT user selectable, ANY OLD FILES WITH THIS NAME WILL BE

OVERWRITTEN.

(FILE OUTPUT OPTION)

TRAKSAT  Version 2.00

Analytical Solution

Tracking Station: HUNTSVILLE, AL

[ Optical Visibility ]

Satellite    UTC     TIME     LOCAL   TIME      AZIMUTH  MAX  MIN    DURATION

DATE    HR:MN:SC DATE    HR:MN:SC           ELE  RANGE  HR:MN:SC

Alouette 1   20Apr90 02:54:12 19Apr90 21:54:12  NW TO S   43   1387  00:12:12

Cosmos 398   20Apr90 01:19:02 19Apr90 20:19:02  NW TO E   33   1970  00:16:02

20Apr90 03:34:48 19Apr90 22:34:48  NW TO SE  74   2069  00:26:48

21Apr90 01:27:19 20Apr90 20:27:19  NW TO E   39   1879  00:18:19

Starlette    20Apr90 02:27:41 19Apr90 21:27:41  NW TO E   35   1582  00:13:41

20Apr90 02:30:13 19Apr90 21:30:13  NW TO E   55   1241  00:13:13

LAGEOS       20Apr90 04:57:34 19Apr90 23:57:34  NE TO W   47   6830  00:51:34

20Apr90 08:22:09 20Apr90 03:22:09  NE TO NW  27   8006  00:40:09

21Apr90 03:34:44 20Apr90 22:34:44  NE TO SW  70   6108  00:56:44

GPS-0001     20Apr90 02:33:52 19Apr90 21:33:52  W  TO SE  65  20491  06:47:52

TRAKSAT  Satellite Tracking Program                           Page 44

GPS-0002     20Apr90 05:05:15 20Apr90 00:05:15  NW TO NW  13  24419  01:15:15

GPS-0005     21Apr90 02:37:23 20Apr90 21:37:23  W  TO SE  65  20533  06:47:23

GPS-0006     20Apr90 00:16:22 19Apr90 19:16:22  NW TO SW  57  20854  04:27:22

SME          21Apr90 02:15:21 20Apr90 21:15:21  S  TO NW  29    812  00:05:21

Salyut 7     20Apr90 09:56:11 20Apr90 04:56:11  SW TO NE  71    413  00:06:11

Cosmos 1383  20Apr90 02:53:40 19Apr90 21:53:40  N  TO S   50   1293  00:12:40

IRAS         20Apr90 01:52:33 19Apr90 20:52:33  S  TO SW  23   1793  00:08:33

The screen below will be displayed when the file output mode is

selected and the simulation is over. This case took 423 seconds

to complete 112 satellite predictions for 24 hours.

�����������������������������������������������������������ͻ

�           ������������������������������������ͻ          �

�           �              TRAKSAT               �          �

�           ������������������������������������͹          �

�           �       ANALYTICAL RISE & SET        �          �

�           ������������������������������������͹          �

�           �                                    �          �

�           �      (F) Output To File            �          �

�           �      (S) Output To Screen          �          �

�           �          LOS Visibility            �          �

�           �       Enter Option (F,S) [F]       �          �

�           �       Input File: NASA678.TXT      �          �

�           �       Record# 112   of  112        �          �

�           �       Total Time: 423   Sec        �          �

�           ������������������������������������ͼ          �

�����������������������������������������������������������ͼ

TRAKSAT  Satellite Tracking Program                           Page 45

ADVANCED FEATURES OPTION 4

To return to the Main Menu the user can press return or option 4.

The default will return the user to the Main Menu.

NORAD/NASA 2-LINE SATELLITE DATA

NORAD maintains general perturbation element sets on all resident

space objects.  These element sets are periodically refined so as

to maintain a reasonable prediction capability on all space

objects. In turn, these element sets are provided to users.

The input file of current orbital elements can be obtained form

several BBS around the country. One such BBS is the Celestial BBS

at (513) 427-0674 in Fairborn, Ohio, the SYSOP is T.S. Kelso.

See section; Obtaining Satellite Data, for more information on

obtaining satellite data.

I have included a file of the latest elements for over 130

orbiting satellites. See section; Files Required To Run TRAKSAT.

******************

* IMPORTANT NOTE *

******************

The following was downloaded from Celestial BBS, T.S. Kelso SYSOP.

Effective January 1986, this system began posting the most recent

element sets received from NASA/Goddard Space Flight Center for

several categories of satellites:  Amateur Radio, Earth

Resources, Manned Spacecraft, Navigation, Weather, and NASA's 30

Day Specials (which contain objects launched within the last 30

days and are often easy to spot visually).  More specifically,

these include the following satellites or satellite series:

OSCAR, Radio Sputnik, UOSAT, Cosmos, LandSat, SeaSat 1, SPOT,

Mir, Salyut 7, Soyuz, LDEF, US Space Shuttle, NAVSTAR (GPS),

GOES, Meteor, and NOAA.

These elements will be maintained in ASCII format in the file.

Data for each satellite will consist of three lines in the

following format:

AAAAAAAAAAA

1 NNNNNU NNNNNAAA NNNNN.NNNNNNNN +.NNNNNNNN +NNNNN-N +NNNNN-N N NNNNN

2 NNNNN NNN.NNNN NNN.NNNN NNNNNNN NNN.NNNN NNN.NNNN NN.NNNNNNNNNNNNNN

Line 1 is a eleven-character name. Lines 2 and 3 are the standard

Two-Line Orbital Element Set Format identical to that used by

NASA and NORAD.  The format description is:

Line 2

Column     Description

01-01     Line Number of Element Data

03-07     Satellite Number

TRAKSAT  Satellite Tracking Program                           Page 46

10-11     International Designator (Last two digits of launch year)

12-14     International Designator (Launch number of the year)

15-17     International Designator (Piece of launch)

19-20     Epoch Year (Last two digits of year)

21-32     Epoch (Julian Day and fractional portion of the day)

34-43     First Time Derivative of the Mean Motion (rev/day^2)

or Ballistic Coefficient (Depending of ephemeris type)

45-52     Second Time Derivative of Mean Motion (Blank if N/A)

54-61     BSTAR drag term if GP4 general perturbation theory was used.

Otherwise, radiation pressure coefficient.

63-63     Ephemeris type

65-68     Element number

69-69     Check Sum (Modulo 10)

(Letters, blanks, periods = 0; minus sign = 1;

plus sign = 2)

Line 3

Column     Description

01-01     Line Number of Element Data

03-07     Satellite Number

09-16     Inclination [Degrees]

18-25     Right Ascension of the Ascending Node [Degrees]

27-33     Eccentricity (decimal point assumed)

35-42     Argument of Perigee [Degrees]

44-51     Mean Anomaly [Degrees]

53-63     Mean Motion [Revs per day]

64-68     Revolution number at epoch [Revs]

69-69     Check Sum (Modulo 10)

All other columns are blank or fixed.

Example:

NOAA 6

1 11416U          86 50.28438588 0.00000140           67960-4 0  5293

2 11416  98.5105  69.3305 0012788  63.2828 296.9658 14.24899292346978

For a description of the mean orbital elements see section; What

Are The Mean Classical Elements.

Note that the International Designator fields are usually blank,

as issued in the NASA Prediction Bulletins.  All epochs are UTC.

Satellites will be ordered by their NASA Catalog Number.  The

data file will be updated as soon as possible after receipt of

new element sets or whenever element sets are received for the

Space Shuttle.

TRAKSAT  Satellite Tracking Program                           Page 47

The following pages contain a brief overview of the methods used

in TRAKSAT and are included to help the reader understand the

mechanics of a orbiting satellite about the earth.

WHAT ARE THE MEAN CLASSICAL ELEMENTS

Five independent quantities called "orbital elements" are

sufficient to completely describe the size, shape and orientation

of an orbit. A sixth element is required to pinpoint the position

of the satellite along the orbit at a particular time. The

classical set of six orbital elements are defined as:

1. a, semi-major axis, a constant defining the size of

the conic orbit.

2. e, eccentricity, a constant defining the shape of the

conic orbit.

3. i, inclination, the angle between the Z axis, i.e.

like the North Pole, and the angular momentum vector,

h = R X V, i.e. the vector R crossed with the vector V.

4. �, longitude of the ascending node, the angle, in the

fundamental plane, between the direction of the

vernal equinox and the point where the satellite

crosses the fundamental plane in a northerly

direction, (ascending node).  This angle is measured

counterclockwise when viewed from the north side of

the fundamental plane.

5. w, argument of periapsis, the angle, in the plane of

the satellite's orbit, between the ascending node and

the periapsis point, measured in the direction of the

satellite's motion.

6. T, time of periapsis passage, the time when the

satellite was at periapsis.

6a. Sometimes the time of periapsis passage is replaced

by the true anomaly, v, the angle, in the plane of

the satellite's orbit, between perigee and the

position of the satellite at the particular time, t0,

called the epoch.

* (To convert from T to v)

v = (360 deg) * t0 / T

TRAKSAT  Satellite Tracking Program                           Page 48

The sharp reader will notice that the NORAD elements do NOT

include the semi-major axis, a. It is possible to calculate the

semi-major axis with the data in a NORAD elements set. The

approach would be;

1. Convert the mean motion into degrees per second.

and calculate the time to complete one orbit, this

will be called the period.

2. Using the period and the earth's gravitational

constant, mu, the semi-major axis can be calculated.

(equations used)

xn_s = (mean motion * 360)/86400

per  =  360/xn_s

a    = ((per^2 * mu)/(4*�^2))^(1/3)

mu = 3.986012d+14 km^3/sec^2.

The starting point for the study of motion of one body orbiting

another, such as an artificial satellite about the earth, is

always the two-body problem; i.e., two point masses attracted to

each other according to Newton's Law of Universal Gravitation,

the inverse square law. The solution is well-known; the two

bodies move about each other in conic sections. For bounded

motions, such as those of an earth satellite, this conic is

either a circle or an ellipse.

The problem can be formulated in different ways, but is always

convenient to chose a coordinate system with the origin centered

at one of the bodies. The position of the second body then can

be specified, for example, by giving its initial cartesian

position and velocity coordinates and then integrating the

equations of motion to find the future positions and velocities.

The cartesian system is not the most convenient one in which to

represent the motion because an analytic solution cannot be

obtained and the integrations must be done numerically.

By adopting a polar coordinate system, one is able to effect an

analytic solution referred to above which can be specified in

terms of six constants of motion; five orbital elements,

a,e,i,w,� and the time of pericenter passage T. The last

constant can be, and usually is, replaced by the mean anomaly M

which is a linear function of time. This is a very convenient way

to specify the initial position and velocity of a satellite and

it also allows an easy visualization of the motion. The position

and velocity of the satellite at any future time can be

specified in terms of these six constants, a,e,i,w,�,M and

time.

In realistic applications, such as artificial satellites about

the earth, there are forces acting on the satellite in addition

to the inverse square force although this is the dominate one.

Other gravitational forces are due to distant bodies such as the

moon and sun but the principal additional gravitational forces

are due to the non-sphericity of the earth. All of the

gravitational forces are conservative and can be represented by

a potential function. In addition to these extra gravitational

TRAKSAT  Satellite Tracking Program                           Page 49

forces, there are non-conservative forces such as atmospheric

drag. All of these forces other than the inverse square force

are called perturbations. The prediction of motion considering

these additional forces is called Perturbation Theory.

The orbital elements, constant for pure two-body motion, become

slowly varying functions of time when the perturbations are

considered. Differential equations describing the time rates of

change of the elements are called the Lagrange Planetary

Equations, LPE and can be found in any standard book on

celestial mechanics. Considering conservative forces only, which

can be represented by a potential function, the part of the

potential other than the two-body part is conventionally called

the disturbing function, represented by R, and the LPE are:

.

a = 2 / n a * ( �R / �M )

.

e = (-(1-e�)^� / na�e)*�R/�w+(1-e�/na�e)*�R/�M

.

i = cot i/(na�(1-e�)^� * �R/�w - �R/(��na�(1-e�)�)

.

w = (1-e�)^� * �R / na�e�e - cot i * �R/(na�(1-e�)^�)*�i)

.

� = �R/(na� sin i *(1-e�)^�) * �i)

M = n - 2�R/na�a - 1-e� * �R/(na�e * �e).

* where � is the partial derivative

starting from the very simple representation of the gravitational

potential between two point masses of magnitude m0 and mi

separated by distance r as;

V = -G * (m0 * mi)/r

one can, by applying this to a satellite of mass m0 and to every

infinitesimal mass point mi of the earth and integrating over the

whole earth, arrive at the following potential function for the

earth;

�                   n �              n    m

V=-�/r(1-� JnPn (sin�)(re/r)^ +� � Jnm (re/r)^  Pn^ (sin�)cos(m(�-�))).

n=2                     n=2m=1                             mn

The first term is the one giving pure two-body motion and the

additional terms are the perturbing terms. The first sum, zonal

harmonics, represents the flattening and other distortions

relative to the equator and the second sum, tesseral harmonics,

represents the non-uniformity of the earth in longitude. If, as

is frequently done, one assumes that the earth possesses

rotational symmetry, then the second sum vanishes. The neglect of

the second sum usually produces no noticeable effects except in

the case of geosynchronous satellites. Then one must consider

those terms which cause slow long-period drifts of the

geosynchronous position.

TRAKSAT  Satellite Tracking Program                           Page 50

For close earth satellites one can usually take about three terms

from the first sum and get very accurate results; even the first

term alone will produce very satisfactory results in most cases

for short-time periods.

The Jn are constants which depend on the mass distribution in the

earth and are deduced from analysis of observed satellite

motions. The currently accepted values of J2, J3, and J4, which

are used in TRAKSAT, are;

-3

J2 = 1.082616 X 10

-6

J3 = -2.53881 X 10

-6

J4 = -1.65597 X 10  .

The Pn (sin �) are Legendre polynomials of index n and are even

functions of sin � for n even and odd functions for n odd. The J2

term describes the flattening of the earth and the J3 term the

so-called pear shape. J2, which is three orders of magnitude

larger than J3, gives rise to secular changes in the elements w,

�, and M while J3 gives rise to long_period oscillations in e and w.

In general, even harmonics cause long-period and secular changes

in the elements, and odd harmonics cause long-period

oscillations.

Short-period oscillations can result from all terms; but since J2

is so much larger than the other coefficients, generally only the

J2 short-period terms are considered. Secular terms are those

which monotonically increase or decrease with time. For first

order solutions this change with time is linear. Long-period

terms are those which oscillate with a period of typically one to

two months, and short-period terms are those which oscillate with

a period of one orbital period or some rational fraction of it.

To finish formulating the problem, the disturbing function is

expressed in terms of the orbital elements and then the

appropriate partial derivatives are taken and substituted into

the LPE. One then has a coupled set of first order non-linear

ordinary differential equations. Because they are non-linear, they

can be solved only by various approximation methods. The usual

method is to assume that the solutions can be represented in some

type of power series expansion in a small parameter and arrive at

sets of approximation equations which can be a close

representation of the real motion, at least over short-time

periods.

The complete solution consists of the sum of the secular terms,

short-period terms, and the long-period terms; i.e.,

a    = a  + a  + a  .

osc    s    sp   lp

TRAKSAT  Satellite Tracking Program                           Page 51

******************

* IMPORTANT NOTE *

******************

I have NOT included the actual equations used in the program in

this document for obvious reasons, i.e. they are long and hard to

type in with a text based word processor. If you have an interest

in these equations they are in several references I have listed.

TRAKSAT  Satellite Tracking Program                           Page 52

MODELS FOR PROPAGATION OF NORAD ELEMENT SETS

NORAD maintains general perturbation element sets on all

resident space objects.  These element sets are periodically

refined so as to maintain a reasonable prediction capability on

all space objects.  In turn, these element sets are provided to

users.

The most important point to be noted is that not just any

prediction model will suffice.  The NORAD element sets are "mean"

values obtained by removing periodic variations in a particular

way.  In order to obtain good predictions, these periodic

variations must be reconstructed (by the prediction model) in

exactly the same way they were removed by NORAD.  Hence,

putting NORAD element sets into a different model (even though

the model may be more accurate or even a numerical integrator)

will result in degraded predictions.

All space objects are classified by NORAD as near-Earth (period

less than 225 minutes) or deep-space (period greater than or

equal 225 minutes). Depending on the period, the NORAD element

sets are automatically generated with the near-Earth or deep-

space model.

The PROGRAM will calculate the satellite period and know which

PREDICTION MODEL TO USE.

THE PROPAGATION MODELS

Two mathematical models for prediction are used by TRAKSAT. The

first of these, SGP4, was developed by Ken Cranford in 1970 (see

Lane and Hoots 1979) and is used for near-Earth satellites.  This

model was obtained by simplification of the more extensive

analytical theory of Lane and Cranford (1969) which uses the

solution of Brouwer (1959) for its gravitational model and a

power density function for its atmospheric model (see Lane, et al

1962).

The next model, SDP4, is an extension of SGP4 to be used for

deep-space satellites.  The deep-space equations were developed

by Hujsak (1979) and model the gravitational effects of the moon

and sun as well as certain sectoral and tesseral Earth harmonics

which are of particular importance for half-day and one-day

period orbits.

TRAKSAT  Satellite Tracking Program                           Page 53

COMPATIBILITY WITH NORAD ELEMENT SETS

The NORAD element sets are currently generated with either SGP4

or SDP4 depending on whether the satellite is near-Earth or deep-

space.

For SGP4 and SDP4 users, the mean motion is first recovered from

its altered form and the drag effect is obtained from the SGP4

drag term (B*) with the pseudo-drag term being ignored.  The

value of the mean motion can be used to determine whether the

satellite is near-Earth or deep-space (and hence whether SGP4 or

SDP4 was used to generate the element set).  From this

information the program will decide whether to use SGP4 or SDP4

for propagation and hence be assured of agreement with NORAD

predictions.

TRAKSAT  Satellite Tracking Program                           Page 54

PROGRAM LIMITATIONS AND ASSUMPTIONS

The ephemeris equations DO include the zonal harmonics, through

2nd order, of the gravitational potential.  This implies a

gravitational field produced by an oblate spheroidal earth

unsymmetrical with respect to the equator, pear-shaped. In other

words, the ephemeris equations contain J2, J3, and J4 terms. The

currently accepted values of J2, J3, and J4, which are used in

TRAKSAT, are;

-3

J2 = 1.082616 X 10

-6

J3 = -2.53881 X 10

-6

J4 = -1.65597 X 10  .

The earth equatorial radius used by TRAKSAT is; 6378.135 Km,

while the flattening factor used is 1/298.257 (both are from the

1972 WGS models).

The program TRAKSAT models only ELLIPTICAL orbital motion about

the earth. That is, the orbital eccentricity MUST BE LESS THAN

ONE and GREATER THAN ZERO. Very small eccentricities are

acceptable, i.e., such as 1.0E - 5.

STARS USED IN TRAKSAT

The star background option will use the following list of stars

for the display. The star data values are from USNO Floppy

Almanac 1988, Version 2.11.88, file STAR1.CAT using Epoch J2000

coordinates.

Bayer Name            Proper Name

----------            -----------

Ursae Minoris         Polaris

Andromedae            Alpheratz

Phoenicis             Ankaa

Cassiopeiae           Schedar

Ceti                  Diphda/Deneb Kaito

Eridani               Achernar

Arietis               Hamal

Eridani               Acamar

Ceti                  Menkar

Persei                Mirfak

Tauri                 Aldebaran

Orionis               Rigel

Aurigae               Capella

Orionis               Bellatrix

Tauri                 Elnath

TRAKSAT  Satellite Tracking Program                           Page 55

Orionis               Alnilam

Orionis               Betelgeuse

Carinae               Canopus

Canis Majoris         Sirius

Canis Majoris         Adhara

Canis Minoris         Procyon

Geminorum             Pollux

Carinae               Avior

Lambda Velae          Suhail

Carinae               Miaplacidus

Hydrae                Alphard

Leonis                Regulus

Ursae Majoris         Dubhe

Leonis                Denebola

Corvi                 Gienah

Crucis                ACrux

Crucis                GaCrux

Ursae Majoris         Alioth

Virginis              Spica

Ursae Majoris         Alkaid

Centauri              Hadar

Centauri              Menkent

Bo�tes                Arcturus

Centauri A            Rigil Kentaurus

Librae                Zubenelgenubi

Ursae Minoris         Kochab

Coronae Borealis      Alphecca

Scorpii A             Antares

Triangulii            Atria

Ophiuchi              Sabik

Lambda Scorpii        Shaula

Ophiuchi              Rasalhague

Draconis              Eltanin

Sagittarii            Kaus Australis

Lyrae                 Vega

Sagittarii            Nunki

Aqilae                Altair

Pavonis               Peacock

Cygni                 Deneb

Pegasi                Enif

Gruis                 Al Na ir

Piscis Austrini       Fomalhaut

Pegasi                Markab

USER DEFINED STAR DATA

The option of using your own star data in TRAKSAT is

included. Up to 1500 stars can be included in a data file.

** ALL STAR DATA WILL NEED TO BE FOR EPOCH J2000. **

The following description of the data base format is included

for the user. All star data files are in standard ASCII code. See

section; Files Required To Run TRAKSAT for a sample star data

file.

TRAKSAT  Satellite Tracking Program                           Page 56

User defined star data has the following format:

GGGCCC HHMMD+DDMM +M.M +C.CC

----------------------------

GAMCAS 00567+6043 +2.4 -0.15

GA2AND 02039+4220 +2.3 +1.37

39 ARI 02479+2915 +4.5 +1.11

HR961  03203+7744 +5.5 +0.19

where GGG is the first three characters of the Greek name of the

star OR the first two characters and a superscript number OR the

Flamsteed number of the star. Alpha is ALP, beta is BET, gamma is

GAM, and so on. Omicron and Omega are abbreviated OI and OE to

distinguish them. CCC is the standard three-letter abbreviation

of the constellation name -- ORI for Orion, UMA for Ursa Major,

CVN for Canes Venatici. If the star has neither Greek letter nor

Flamsteed number, then GGGCCC becomes is "HR" plus the four-digit

Yale Bright Star Catalog number.

HHMMD+DDMM is the star's location in right ascension and

declination coordinates for Epoch J2000. HH is hours, MM is

minutes, and D is decimal minutes of RA. +DD is declination

degrees and MM is declination minutes. The coordinates, as given,

are precise enough for virtually any computer graphics

application. +M.M is the star's visual magnitude in yellow light,

essentially how bright the star appears to the eye. The sign is

necessary because some of the brightest stars, like Sirius, have

negative magnitudes.

Finally +C.CC is the B-V "color index" of the star. This number

ranges from -0.25 to +2.5. The "bluest" stars have the lower or

negative B-V color, while red stars have color indices over 1.6

or so. The Sun has a B-V of +0.62, and is a yellow-white star.

TRAKSAT does not use the color data for any calculations. We thus

decode the examples as: Gamma Cassiopeiae, a medium-bright blue

star at 0 hours 56.7 minutes right ascension and +60 (degrees)

43' declination; Gamma-2 Andromedae as a reddish star of

approximately the same brightness; 39 Arietis doesn't have a

Greek letter name; and HR 961 is the 961st entry in the Yale

Bright Star Catalog, barely visible to the naked eye.

The user defined star data option will NOT display the star

names. (There would not be any room on the screen to see the

orbital track!) The format used is popular with several star map

programs and should be available to the user. (A star file is now

included with TRAKSAT.)

METHOD DESCRIPTION FOR REVERSE SOLUTION

The reverse solution is a very powerful and useful option for the

satellite observer. May times I have looked up to the skies only

to see a unknown satellite fly-by. Determining which satellite I

saw was almost impossible. The approach I used was to run TRAKSAT

in the multi-track mode until I either ran out of satellite data

sets or I found a possible match. This could prove very tedious

and not very practical on a day to day basis.

This "lacking" provided the desire for the reverse solution

option in TRAKSAT. The next step was to determine a method to use

TRAKSAT  Satellite Tracking Program                           Page 57

and what would be required from the user. Several methods of

converting observed data into orbital elements are in use today,

such as Gauss, f & g series, Lambert, and true anomaly iteration

are just a few. It is difficult to be specifically precise in a

statement of which of the previous methods can be termed "the

best". Certainly a method which best decrees the problem at hand

should be chosen for the determination of a particular orbit.

There are, however, several points of interest that should be

known to the analyst before choosing a particular method. For

example, which method is the fastest from a computational point

of view? Which method has the least numerical error? Which method

experiences the least convergence difficulties? Numerical studies

upon many orbits of varying eccentricity and semi-major axis have

provided a very partial answer to these questions.

To answer the first question, that is, which is the fastest

computational method, several sources were consulted. The fastest

method proved to be the true anomaly with the Gauss method

following close behind. The f & g series was the slowest method.

The second question, which method has the least numerical error,

is difficult to answer, and, as might be expected, indicates that

every method is optimum in the computation of a particular

element. It is, however, possible to segregate the overall

results of the numerical studies into three categories: high,

medium, and low accuracy. The Gauss and Lambert methods display a

high accuracy rating while the true anomaly and f & g series have

only a medium accuracy rating.

The last question, ease of convergence, is not as difficult to

answer. The Gauss and f & g series methods are called "self-

perpetuating" in their convergence. That is, from a easily

estimated first guess an iterative loop is initiated which

converges automatically to the desired result. The Gauss method

suffers from instability of convergence for radial spreads of

greater than 90 degrees. This should NOT prove a problem because

most observed satellite data sets will be viewed in less than a

90 radial spread.

The natural conclusion to this analysis was to use the method of

Gauss. This document will NOT develop this method as MANY of the

references listed go into great detail on the subject. I would

suggest the reader to investigate further if interested.

ACCURACY OF TRAKSAT

Several people have asked the question; " How accurate is

TRAKSAT ?". To answer such a question one must define a set of

limits. The real "acid test" is to have a prediction from TRAKSAT

and then go outside and observe that satellite, taking note of

the time and position of the satellite. A comparison between

actual observed data and predicted can then be determined.

Another approach is to compare the output from TRAKSAT against

several other satellite tracking programs. Both the public domain

and the commercial markets have several good tracking programs

that the user could compare with.

The later approach, that is the comparison between tracking

TRAKSAT  Satellite Tracking Program                           Page 58

programs, has been carried out by several people including

myself. The output from TRAKSAT compares very well with many of

the "current" tracking programs (both commercial and public

domain). It could be concluded from a simple test of TRAKSAT that

it agrees with several other tracking programs. (Gee we all can't

be wrong!)

The next step is one of comparing predicted output and observed

data. The most popular use for TRAKSAT has been in the optical

sighting options. The optical sighting of a satellite will be the

"acid test" used for this accuracy test.

First a note about NASA 2-line elements, low earth satellites

(about 15 rev per day satellites) have larger disturbances from

the atmosphere than higher satellites. The drag on a satellite

can cause purtubations greater than the J2 terms therefore the

drag term is of great importance. The very latest elements for

the low earth satellite can greatly improve the prediction

process, while the higher satellites do not require as current of

elements. (The term low will be in the range of 250 - 375

kilometers altitude.)

Reports about TRAKSAT (and its predictions) have been made on the

MIR satellite along with several other low earth satellites. The

bottom line being LESS THAN 30 SECONDS ERROR (prediction vs.

actual) for 10 day old satellite element sets. If the satellite

elements are 20 days old the error is about 60 seconds. If the

elements a only a day or two old, errors of less than 10 seconds

are possible (several reports have been made about 2-10 seconds

of error).

The position data is on the money, it is the time at that

position that usually drives the accuracy of the observation.

The higher earth satellites generally have less than 30 seconds

of error for 20 day old elements.

A NOTE MUST BE MADE ABOUT THE ERROR ANALYSIS, THE ASSUMPTION IS

MADE THAT NO ORBITAL MANEUVERING WAS DONE TO THE SATELLITE DURING

THE "TEST" PERIOD. (The STS-31, the Hubble launch, was a prime

example of several orbital maneuvers changing the predicted

observed times).

In general it can be said that the most current elements are the

best ones for planning the evenings viewing. (Elements over 30

days old can have a very large error to them). Element sets 7 -14

days old will be acceptable for most users.

NORAD/NASA updates the satellite elements for this very reason,

to keep the prediction process accurate.

TRAKSAT  Satellite Tracking Program                           Page 59

A BRIEF EDITORIAL

One of the first decisions to be made when setting out to write a

program is the choice of a programming language. I'm an Aerospace

Engineer working for a company in Huntsville, Al. My job title

is; Trajectory Analysis Engineer. I work with NASA, mostly the

shuttle program, and design trajectories for several upcoming

shuttle missions. I have written large trajectory simulations

programs, for the most part they were written in FORTRAN.

I know FORTRAN is not the best language to use for programs that

use graphics, but Microsoft has come up with the ideal solution.

Microsoft FORTRAN, version 4.0 and higher, can call BASIC, C, and

PASCAL routines. Microsoft FORTRAN version 5.0 also contains graphic

routines that were used in TRAKSAT, these graphic routines are the

same as used in QuickC version 2.0.

Most (95%) of the TRAKSAT program is written in FORTRAN to get

the best speed and high precision mathematics and C, Microsoft C

version 6.0, for some useful utilities. I have found this

combination to be very powerful and useful for program

development.

TRAKSAT  Satellite Tracking Program                           Page 60

SPECIAL THANKS

I would like to take this opportunity to thank the many people

who helped me either directly or indirectly on this program.

First of all my wife, Anita, who understands why I have a hobbie

like computers and enjoy working with them. She has not

complained about the many hours, in excess of 500 hours, I have

spent working on TRAKSAT.  TRAKSAT version 2.45 has some 21,000

lines of code and IS STILL GROWING.

Dave Ransom Jr., of Rancho Palos Verdes, CA. has kept me going

when my interest in the program was slipping away. I did use the

city data from his excellent program ASTROCLOCK. I also used

several of the references Dave listed in his program. I would

highly recommend his program to any person interested in

astrodynamics.  The documentation supplied with ASTROCLOCK is in

itself very interesting reading and very well done. I could only

hope that someday TRAKSAT will have that level of

professionalism. My thanks to Dave and his wife Vicki.

John Williams and Dr. Jeff Wallach, from the Dallas DataLink BBS,

have been very helpful in this project also. They have offered

data and a helping hand with TRAKSAT.  The DataLink BBS has a

vast amount of satellite information along with other interests.

I would recommend it to others interested in satellite tracking.

The DataLink BBS is THE place to learn about obtaining satellite

images. My thanks to John and his family.

I would also like to thank T.S. Kelso, SYSOP of the Celestial BBS

where current satellite data can be downloaded. Several satellite

tracking programs are also available on his BBS along with a vast

amount of satellite information.

TRAKSAT  Satellite Tracking Program                           Page 61

QUESTIONS AND COMMENTS

I would very much like to hear from anyone interested in

this program and astrodynamics in general. I have not included

the source code to TRAKSAT mainly because most people do not have

a Microsoft FORTRAN compiler nor a working knowledge of FORTRAN.

The high cost ($500.00) of both the FORTRAN and C compilers makes

the cost of working on a project like TRAKSAT expensive also.

As for the choice of FORTRAN compiliers there are many fine

products out and I have used many of them. However, the only

compilier that supports mixed language is Microsoft. For this

reason alone I would recommend it to others interested in

programming.

At this time I feel that TRAKSAT is still going through some

"growing pains"  and I would like the chance to improve it and

add new features. The only way this can happen is if you, the

user, takes the time to leave me messages or mail on problems or

suggestions. I will try to answer your messages in a timely

manor.

One of the major goals of this project is to make the finest,

easiest to use, satellite tracking program available. A program

the average person can use and successfully track satellites.

I would suggest the user to OBTAIN A COPROCESSOR if they do not

have one already. A coprocessor speeds up math intensive

programs, such as TRAKSAT, to a level that was only dreamed about

a few years ago.

Please feel free to contact me to discuss TRAKSAT or other

computer problems. I can reached through the RPV BBS;

RPV BBS

Rancho Palos Verdes, Ca.

213-541-7299

24 hours, 2400/1200 baud.

This BBS is owned and operated by Dave Ransom Jr.. I call up the

BBS three or four times a week to check my mail and do some file

transfers. This BBS is geared towards Astronomical interests. The

latest version of ASTROCLOCK can be downloaded from this BBS

also.

Other BBS's I frequent are;

Celestial RCP/M                     DataLink RBBS System

Fairborn, Ohio                      Dallas, Texas

513-427-0674                        214-394-7438

24 hours, 2400/1200 baud,           24 hours, 9600/2400/1200 baud.

I can also be reached at work or home, please no calls after 10

PM Central Time. Please leave a phone number and the best time to

call on any messages that require by personal attention. My work

number has an answering machine pick-up if I can not be reached.

TRAKSAT  Satellite Tracking Program                           Page 62

The last, and slowest method to reach me is with the U.S. mail

service, I will respond with a phone call if at all possible.

Paul E. Traufler

111 Emerald Dr.

Harvest, Al.  35749

Phone (work) 205-726-5511

Phone (home) 205-830-8450

TRAKSAT  Satellite Tracking Program                           Page 63

FUTURE UPGRADES

To obtain the latest version of TRAKSAT, several BBSs around the

country keep it online. The BBSs listed under section; Questions

and Comments list several places to obtain the current version.

Compuserve has TRAKSAT and many other releated files also. If you

would like to save on the long distance charges, contact myself

and I will try to find a local BBS that I can upload TRAKSAT to.

For a small fee ($10.00) I will mail TRAKSAT on a disk, 360K,

1.2M, if that is the easiest way to obtain the latest version,

(1.2M is best for me). Please contact the author for more

information.

If you send a SELF ADDRESSED AND STAMPED DISK-MAILING PACKAGE,

with the proper disk format, I will return mail it free of

charge.

TRAKSAT and its companion files is being distributed as

shareware. YOU ARE ENCOURAGED TO SHARE THIS SOFTWARE WITH OTHERS

PROVIDED THAT IT IS DISTRIBUTED COMPLETE WITH DOCUMENTATION AND

IN UNMODIFIED FORM AND THAT NO FEE OR OTHER CONSIDERATION IS

CHARGED OR ACCEPTED.

*****************************************************************

*****************************************************************

THERE IS NO CHARGE FOR THIS PROGRAM, but in the spirit of the

shareware concept, if the end user finds this package useful, and

would like to help fund development of enhancements to this

package, any amount that the end user feels is appropriate for

the value received in using this package will be gladly accepted

(perhaps $15.00). Please mail any donations to the above address.

ANYONE WHO SENDS A DONATION WILL RECEIVE A PERSONALLY REGISTERED

COPY OF TRAKSAT AND BE PLACED ON THE UPDATE LIST FOR FUTURE

UPDATES TO THE PROGRAM. This copy will give the user credit for

supporting the shareware concept.

*****************************************************************

*****************************************************************

TRAKSAT  Satellite Tracking Program                           Page 64

OBTAINING NORAD SATELLITE DATA SETS

The following BBS's have the current satellite data files;

Celestial RCP/M

Fairborn, Ohio

SYSOP: T.S. Kelso

513-427-0674

24 hours, 2400/1200 baud,

8 bit NO parity 1 stop.

Datalink RBBS System

Dallas, Texas

SYSOP: Dr. Jeff Wallach

214-394-7438

24 hours, 9600/2400/1200 baud,

8 bit NO parity 1 stop.

RPV BBS

Rancho Palos Verdes, Ca.

SYSOP: Dave Ransom Jr.

213-541-7299

24 hours, 2400/1200 baud,

8 bit NO parity 1 stop.

To obtain the elements from the Celestial RCP/M BBS, (with the

least amount of trouble), just dial up the BBS and login. The

next step is to type "F" (without the quotes) to go to the Files

section, then to area #1. The next step is to type "D", for

download and then type "BULLETIN.ARC" as the file to download,

open an XMODEM file transfer mode with your telecommunications

software. This will transfer the NASA 2-line elements to a file

on the users computer. Log out of the BBS and then unarchive the

file using several of the unarchiving programs, (PAK, ARC,

PKZIP, etc.).

The downloaded file will have some characters at the

top of the file that will need to be removed with a word

processor (this MUST save the file in pure ASCII, i.e., PC-Write,

Edlin etc. works very well).

(This method assumes that an account is available to the user).

NASA can provide up to 20 satellite data sets (HARD COPY FORM

ONLY) if the user writes and requests them. This service is free

but the user will need to enter the elements into a file for use

with TRAKSAT. (This is done the old fashion way, you TYPE them!)

NASA Goddard Space Flight Center

Control Center Support Section

Code 513.2

Greenbelt, Md.  20771

TRAKSAT  Satellite Tracking Program                           Page 65

FILES REQUIRED TO RUN TRAKSAT

The following files should have been included in the archive

file;

TRAKSAT.EXE  The program.

TRAKSAT.DEF  The default data for the tracking station.

TRAKSAT.CTY  The city file for tracking stations.

TRAKSAT.DOC  TRAKSAT program documentation.

EARTH.DAT  World map data file.

NASA724.TXT  This is the latest NORAD satellite data set,

(as of 26 July, 1990, element set #724).

READ.ME   Latest notes about the program.

MSHERC.COM  This utility is used for Hercules graphics.

MODERN.FON  This is a font file used for the graphics.

STARS.DAT  This is a star data base that is NOT required

to run TRAKSAT, it is optional. This data file

can be used in the user defined star data

option.

STARS.DAT is almost identical to the star

database used with Richard Berry's (Editor of

ASTRONOMY) program STARS.BAS. For

compatibility, I have retained the same

database format. According to Mr. Berry's

documentation file STARS.DOC, his database is

essentially the same database used for the

StarDome map printed in the center spread of

ASTRONOMY every month.

TRAKSAT  Satellite Tracking Program                           Page 66

BIBLIOGRAPHY

THe following sources were used to prepare and test the

TRAKSAT program.

Meeus, Jean, ASTRONOMICAL FORMULAE FOR CALCULATORS, 3rd Edition,

Willmann-Bell, Inc., Richmond, VA. 1985.

Duffett-Smith, Peter, PRACTICAL ASTRONOMY WITH YOUR PERSONAL

COMPUTER, Cambridge University Press, New York, NY. 1986.

Danby, John, FUNDAMENTALS OF CELESTIAL MECHANICS, 2nd Edition,

Willmann-Bell, Inc., Richmond, VA. 1988.

Bate-Mueller-White, FUNDAMENTALS OF ASTRODYNAMICS, Dover

Publications, Inc. New York, NY. 1971.

Forsythe-Malcolm-Moler, COMPUTER METHODS FOR MATHEMATICAL

COMPUTATIONS, Prentice-Hall, Inc. Englewood Cliffs, NJ. 1977.

USAF-Ford Aerospace Corporation, ORBITAL MECHANICS, O&M Training

Section, Sunnyvale, CA. 1982.

Moulton, F. R., CELESTIAL MECHANICS, Macmillan Company, New York,

NY. 1960.

Brand, L., VECTOR ANALYSIS, John Wiley and Sons, New York, NY.

1957.

Geyling-Westerman, INTRODUCTION TO ORBITAL MECHANICS, Addison

Wesley, Whippany, NJ. 1971.

Brouwer, D., "Solution of the Problem of Artificial Satellite

Theory without Drag", Astronomical Journal 64, 378-397, November

1959.

Hilton, C.G. and Kuhlman, J.R., "Mathematical Models for the

Space Defense Center", Philco-Ford Publication No. U-3871, 17-28,

November 1966.

Hoots, F.R., "A Short, Efficient Analytical Satellite Theory".

AIAA Paper No. 80-1659, August 1980.

Hoots, F.R., "Theory of the Motion of an Artificial Earth

Satellite", accepted for publication in Celestial Mechanics.

Hujsak, R.S., "A Restricted Four Body Solution for Resonating

Satellites with an Oblate Earth", AIAA Paper No. 79-136, June

1979.

Hujsak, R.S. and Hoots, F.R., "Deep Space Perturbations Ephemeris

Generation", Aerospace Defense Command Space Computational Center

Program

Documentation, DCD 8, Section 3, 82-104, September 1977.

TRAKSAT  Satellite Tracking Program                           Page 67

Kozai, Y., "The Motion of a Close Earth Satellite", Astronomical

Journal 64, 367-377, November 1959.

Lane, M.H. and Cranford, K.H., "An Improved Analytical Drag

Theory for the Artificial Satellite Problem", AIAA Paper No. 69-

925, August 1969.

Lane, M.H., Fitzpatrick, P.M., and Murphy, J.J., "On the

Representation of Air Density in Satellite Deceleration Equations

by Power Functions with Integral Exponents", Project Space Track

Technical Report No. APGC-TDR-62-15, March 1962, Air Force

Systems Command, Eglin AFB, FL.

Lane, M.H. and Hoots, F.R., "General Perturbations Theories

Derived from the 1965 Lane Drag Theory", Project Space Track

Report No. 2, December 1979, Aerospace Defense Command, Peterson

AFB, CO.

Bellman, R. and Kalaba, R.E., "Modern Analytic Computational

Methods in Science and Mathematics", American Elsevier Publishing

Company, Inc. 1967.

Escobal, P.R., "Mehtods of Orbit Determination", John Wiley and

Sons, New York, NY. 1965.

Craig, C. John, "Microsoft QuickC Programmer's Toolbox",

Microsoft Press, Redmond Washington 1990.

El'yasberg, P. E., "Theory of Flight of Artificial Earth

Satellites", Israel Program for Scientific Translations Ltd.,

1967. (Translated from Russian)

King-Hele, Desmond, "Observing Earth Satellites", Van Nostrand

Reinhold Co. Inc., 1983.