(comments in brackets are by G.HARRISON)

Comments on BASIC Engineering Programs

Richard B. Kolbly, PE

Golden Rule Systems

5782 Turquoise Avenue

Alta Loma, California  91701

FILTER.ASC  was originally written to run on a CP/M system  under

Microsoft MBASIC version 5.2.  The formulas were derived from the

Burr-Brown "Handbook of Operational Amplifier Active RC Networks"

(1966).  This program is confined to the multiple-feedback active

filter.   The  performance  of  the  filter is based  on  the  s-

parameter response from standard servo theory.

(Program does lp, hp and bp. Enter gain as positive)

MFILTER.ASC  is  an  extension  of  FILTER.ASC.   it  allows  for

cascading of multiple-feedback filters and 'tuning' of the element

values.   In addition,  a routine has been incorporated to  allow

selection   of   standard  RETMA  values  for  circuit   elements

automatically.   The analysis portion allows multiple 'looks'  at

filter response and component changes.

(Enter the gain, cutoff, and damping for each section & the program

calculates the resistor and capacitor values for each section and

the cascaded response)

This  program  has  been used to design many active  filters  and

measured  results have agreed with performance predicted  by  the

program.  Note  that this program assumes an "ideal"  operational

amplifier. More precise results could be used by using NET85.ASC.

The  program  NET85.ASC  is an analysis  program,  not  a  design

program,  so  should  be  used after component values  have  been

determined.

NET85.ASC is an adaptation of a network analysis program  written

up  in a trade magazine (EDN,  February 4,  1981 pp 126-133)  and

published  in  a BASIC-like Hewlett-Packard language for  one  of

their  early desktop computers.  The program has been  translated

into  BASIC  and many "bells and whistles" added for storing  and

loading circuits and results in files for use by other  programs.

The fundemental program has not been changed.

(Hints to use:Number nodes starting from 1. Make the higest number

node be ground. Components can be changed by first removing the

component by putting a negative component across it.)

RFDESIGN.ASC  is an adaptation of a Smith Chart program published

in  RF Design magazine for June of 1985 and has been  adapted  to

run  on the IBM-PC or compatable.   This program does not use any

of the advanced BASIC features,  and is currently being run on  a

Zenith Z-150 with GWBASIC with excellent results.

(Runs ok on pc using basica)

SABIN.ASC  is an adaptation of a signal analysis program by  W.E.

Sabin  published  in EDN for June,1983 (page 243).  It  has  been

adapted  to  run  under Microsoft GWBASIC.   SABINFIX.ASC  is  an

extension of SABIN.ASC to allow more direct user controls,  and a

list  of  the things done,  as well as allowing  dual  arrays  to

transform  and  manipulate.   Also,  file  operations  have  been

extended.

(Removed direct statements in file. Seems to run. I do not have

Sabins original article so am unable to check further.)

(Disk.asc designs a parabolic dish and the sub-reflector for a

Cassegrain feed. Looks ok)

(Satellit.asc runs)

1