Page D-1

STANDARD.DOC

This suggests a set of standards for a simple two sided board with plated

through holes.  The symbols in ELEC.LIB follow this standard and will save some

time in getting started.

Level 1

Used for the component side of the board.  We look at the board from the

component side and see through to the solder side.  We show this side in

green/cyan depending on the Palette switch setting.

Level 2

Used for the solder side of the board.  We show this side in red/magenta

depending on the Palette switch setting.

Level 3

Used to show drill spots.  We show the drill spots in green/cyan.

Level 4

Used to show component outlines.  We show the components in brown/white.

Level 5

Used for the solder mask.  We would show the solder mask in green/cyan.  We

have not set up a solder mask on the demo symbols.

While you are free to set up your own standards, if you want us to plot a

demonstration file for you it will reduce the chance for confusion if you

follow the above.

We suggest that you name your plot files with names ending in 1,2,3,etc.. so

that they come out looking like MYPLOT1.PLT etc..  This way we will be less

likely to mirror the wrong plot if we plot for you.  We will normally plot side

1 mirrored, and side 2 direct.  This way the two sides will register emulsion

to emulsion for the least error.

We find that everyone has different choices for colors.  There seems to be

great variations in the way each of us perceive color.  When we can, we will

allow  multi-color selection for the EGA version of this program.  If you

hate the IBM color set choices this is one more reason to buy the licensed

version.  Note that you can switch between the two IBM color choices (1,1,3,5).

Page D-2