SECTION                    PAGE #

INTRODUCTION

Purpose of this software ..................  1

Current Capability ......................  1

Future Enhancements .....................  2

Support for Abel ......................  2

Support for Cupl ......................  2

Reduction of Boolean Equations ........  2

GETTING STARTED

Hardware Requirement ......................  3

Minimum Required System .................  3

Recomended Hardware .....................  3

Main Menu Commands ........................  4

Naming the pins .........................  4

Entering the signal list order ..........  4

Drawing the signals on the screen .......  4

Creating the PALasm 2 formated file .....  5

How to exit and save your work ..........  6

How to exit without saving your work ....  6

SAMPLE DESIGNS

4 bit counter .............................

with reset ..............................

with set, reset, and load ...............

8 bit ring counter ........................

with reset ..............................

with reset and hold .....................

ADVANCED FEATURES

More than one state machine in a PAL ......

Extended simulation capability ............

Designing a continuous counter ............

LIMITATIONS

*** CAUTION *** ...........................

WHO TO CONTACT WITH PROBLEMS ................

A. INTRODUCTION

1. Purpose of this software

a) Current Capability

The TiDE program is very nearly a finished product.  In

the near future there will be a few minor additions to the

software which will be discussed in the next paragraph.

Aside from these additions I will take sugestions from users

of the software on what they would like to see added or

changed.  In the present revision [ver 2.16] only registered

outputs are supported.  This will be changed in ver 2.17 and

latter versions.

The TiDE software will allow a design engineer to enter

a minimal amount of information about the actual device to

be used and then enter a description of what the state

machine should do by drawing the timing diagram on the

screen of the PC or PC clone.

b) Future Additions

1) Support for Abel

In the near future TiDE will ask you if you will be

using MMI's PALasm 2 or Data I/O's ABLE software.  It will

then generate the proper file for use with your software.

2) Support for Cupl

Due to the fact that I have not seen as many customers

using the Assisted Technology CUPL software I will be

waiting to see the number of requests that I get for this

feature.  If enough people want it I will do it.

3) Reduction of Boolean Equations

I had intended to include a boolean reduction program

for use with PALasm 2 but I now feel that it would be better

to release a working version of the program and then add the

reduction routine in a future release.

B. GETTING STARTED

1. Hardware Requirement

a) Minimum Required System

The minimum system that this software has been tested

on was a DATA GENERAL ONE laptop computer with 256 K of

memory.  This system is an MS Dos machine with a few

exceptions.  I feel that if it runs on this machine it

should run on any MS Dos computer.  If you find an MS Dos

machine that this will not run on please contact me with the

details.

Due to the method used for drawing the wave forms you

do not need to have any graphics capability at all.  The

program is small enough to only require a single floppy disk

to hold both the program and the files created by the

program.  With these limited requirements it should be

possible to run TiDE on allmost any MS Dos computer.

b) Recomended Hardware

There is only one piece of hardware that would make a

nice addition and that is an IBM graphics printer.  The

Epson printers and their clones will not work for screen

dumps of the timing diagram without the use of special

software such as Print Works by SoftStyle.  The reason for

this is that the graphics on the screen are only the

extended character set of the IBM PC and are not really

graphics at all.  On an Epson or equivalent the graphics

characters are not there and instead you will get italics

characters.  This is only a limitation if you wish to get a

hardcopy of the screen while in the drawing mode.

2. Main Menu Commands

a) Naming the pins - F1

This is the first option on the main menu and it is

used to enter the mneumonics that you wish to assign to the

inputs and outputs of the device to be designed.  To invoke

this Press the F1 key while the main menu is desplayed. This

is the first thing that must be done on any new design.

b) Entering the signal list order - F2

This option does not need to be used unless you wish to

have the signals shown in an order other than the pin number

order.  When you use this option you have the ability to

have the same signal displayed as many times as you wish.

The only warning is that a signal that is showing in two or

more places on the screen should not be edited.  You will

get very strange results if you try to do this.  Even with

this precaution you may wish to display the same signal more

than once to allow you to compare the timing of this signal

to another.

c) Drawing the signals on the screen - F3

This is the fun part.  Once you have entered the pin

names you may begin to draw the signals by using this

option.  A menu of the available commands is located at the

bottom of the screen.  This menu lists all of the keys that

may be used to enter data, edit signals, or control the

screen.

The first part that we will discuss is the method for

entering the data.  The valid input keys for this are the

"X", "H", and the "L" keys.  Upper or lower case will work.

The meaning of the keys are:

X : don't care - the signal is undefined at this point.

H : high -

on input signal - pin will be high during the next

rising edge of the clock.

on output signal - this signal is to go high after the

next rising edge of the clock if the

previous columns conditions are met.

L : low - same as for H except defines the low condition.

The character entered will be displayed directly above

the cursor after which the cursor will advance one position.

The four arrow keys will all move the cursor in the

indicated direction.  The cursor may not be moved off of the

screen nor will it scroll the screen when it hits the top or

bottom.  If you wish to view the next five signals press the

pgdn key on the cursor control keypad.  To view the previous

five signals use the pgup key.  Due to the fact that there

are seven signals displayed on the screen there will always

be a two signal overlap for continuity.

The last of the control keys are the ins and del keys

again located on the cursor control keypad.  These keys will

allow you to insert a spot into a signal which can then be

changed to the proper value.  The cursor should be placed

under the first location that is to be moved to the right to

make room for the new value.  The del key will delete the

value directly above the cursor and move everything to the

right of the cursor left to fill the hole.  The use of these

two keys will help when you wish to make a small change

after entering a long signal so as not to have to enter the

entire signal again.

d) Creating the PALasm 2 formated file - F4

After you have entered the timing diagram you are ready

to produce the text file which will contain the heading,

equations and the simulation in a PALasm 2 format.  If you

follow all the steps carefully in your design you should not

have to use an editor on the file at all.  It is usually

possible to generate a file which is complete and ready to

be assembled into the fuse pattern for your part.

When this section starts it will ask you for a file

name.  At this point you should enter the name for the

PALasm 2 file.  Standard convention dictates that the

extension to the file name should be ".PDS", this stands for

Pal Design Specification.  Care should be taken here not to

enter a filename that is already being used,  at present no

checking is done to insure that a file is not overwritten

when the new file is created.  This is a feature that will

be added to a future version.

After you have entered the filename you will be asked

several questions.  If you are familiar with PALasm 2 you

will recognize that each of these questions is for the

heading of the file.  If you are just testing a portion of

your design and this will not be the final version of your

design you may just press the ENTER key and leave the

heading blank.  The exception to this is the part type,  if

you do not enter a part type the software will declare the

part as a PALXXXX in the text file.  At this point the file

will not work with palasm and you will have to use an editor

to change the part type to a valid one.  This can be used to

your advantage if you wish to get the equations and then go

back to decide which Pal you wish to use.

The last question will ask you if you are using a part

which has the capability of programmable output polarity.

If you answer this question yes the software will determine

if an output should be written active low or active high

based on which will require the least number of product

terms.  If you are not using a part with programmable

polarity all equations will be written active low.  An

example of a part with programmable output polarity is the

PAL16P8 vs. the PAL16L8.  The P in the part number

designates that the part has programmable polarity.  Many of

the new parts being done by MMI have the added feature of

programmable polarity.

e) How to exit to Dos and save your work - F5

When you have finished entering information or if you

are interrupted you may save your work and continue it at a

latter time.  This will save all pin names and signal values

that have been entered up to this point.  This will also

allow you to come back into the screen editor and change the

timing diagram if you find a problem in the PAL at a latter

date.

f) How to exit to Dos without saving your work -F6

I hope that you will not need this option but if you do

here it is.  If you make a major mistake while editing your

design file and you decide that you would like to lose the

changes that have been made since the last save you may do

so by using this key.  A check is made to insure that you

really want to do this as there is no turning back once it

is done.

C. SAMPLE DESIGNS

1. 4 bit counter

a) with reset

b) with set, reset, and load

2. 8 bit ring counter

a) with reset

b) with reset and hold

D. ADVANCED FEATURES

1. More than one state machine in a PAL

Due to the way that this software generates the

equations to implement your design it is possible to define

multiple independent state machines in the same device.  In

order to do this all you will have to do is insure that the

seperate sections are not drawn under each other.  This

means that at least one column of nothing but don't cares

must seperate the sections.  You can use this method to

implement several different sections in the same device.

2. Extended simulation capability

When the simulation section of the output file is being

generated it works from the timing diagram.  The only signal

lines that it looks at are the input signals and the control

signals such as output enable.  By extending one of the

input signals such as a output enable type simulation will

continue.

3. Designing a continuous counter

In order to make a state machine that will loop back on

itself you will need to show the starting point for the loop

as the last column of the design.  For examples of this and

other special features look at the sample files on this

disk.  Files with an extension of ".SIG" are design examples

that you can use to experiment with.

E. LIMITATIONS

1. *** CAUTION ***

This section will be used to list bugs that have been

found and reported.  I will try to keep this section of the

manual current.  Since this is the first official release of

the program I do not have any reported bugs as yet.

F. Who to contact with problems

If you have a problem with the operation of this

software or if you decide that a feature needs to be added I

have a computer bulletin board which you may use to leave me

a comment.  The phone # for the board is (513) 866-5157.

This board will also contain the latest version of this

software along with the manual, as well as having other

utility programs to help you in designing with MMI Pal

devices.

G. Credits

Author :            Bill Hollon

Field Applications Engineer

Monolithic Memories Inc.

261 Trumpet Drive

Dayton, Ohio      45449

Concept and Ideas : Mike Wier

Regional Sales Manager

Monolithic Memories Inc.

Anyone who makes a contribution of a signifigant idea will

be added to this list.  So send in any and all ideas for

improvement.

Thanks;

William R. Hollon