***** PERFECT BOX ***** Version 4.1 *****

I.   INTRODUCTION:

PERFECT BOX is a Computer Aided Design (CAD) program for the

simulation  of  loudspeaker  performance.   PERFECT BOX is a very

powerful  tool for the amateur or professional  speaker  builder.

Loudspeaker   design   is  a  balance  of  compromises  involving

enclosure size, low frequency  cutoff,  and  transient  response.

This  program  will  allow you to model the loudspeaker-enclosure

combination entirely on  your  PC.   With proper use, you can  be

assured of the best (lowest in frequency)  bass  response  in the

smallest  enclosure.   You can then build the enclosure with full

confidence that the final  result  will  not  become  the new dog

house for Fido, but an accurate, natural sounding system.

PERFECT BOX does not deal with midrange or tweeter  drivers.

It only considers the low frequency driver in a multi-way  system

(or  the  only  driver  in a full-range system.)  Since enclosure

alignments, such as box size,  box  type, and box frequency  (now

on refered to as Alignments), effect the woofer response, PERFECT

BOX models systems covering low frequencies from 10 to 200 Hertz.

The mathematical model will hold for  higher  frequencies,  but I

have optimized the program for the most useful range.

II.  REQUIREMENTS:

PERFECT  BOX requires certain parameters that uniquely define

the driver's characteristics.  Some of these parameters have been

coined as the  Thiele/Small  (or T/S) parameters.  If you are new

to loudspeaker design, it isn't  important  that  you  understand

what  each  value  represents,  only  that you are able to obtain

these  values  from  the  driver manufacturer,  or  measure  them

yourself (more accurate.)

PARAMETER                             UNITS

Free air resonance    (Fs)                   Hertz

Electrical   (Qes)

Q Mechanical (Qms)

Q Total   (Qts)

Volume   Equivalent   Compliance(Vas)         Ft^3

D.C.  Voice  Coil Resistance (Re)             Ohms

Peak  Cone

Displacement  (Xmax)                          Inch

Effective Piston Diameter                     Inch

Thermal Power (P)                         Watts, RMS

NOTE:  Qts=(Qes*Qms)/(Qes+Qms)

PERFECT BOX allows you to store these driver parameters in a

data file ("BOXDATA4").   This file can store characteristics for

thousands of different drivers.   A  few sample drivers have been

included  so  that you can experiment with  PERFECT  BOX.   These

samples represent "real life" units which are currently available

at this time.

III. DEFINITIONS:

***SYSTEM ORDER***

PERFECT BOX uses a classical high-pass filter model to simulate a

loudspeaker mounted  in an enclosure.  The order of a loudspeaker

system represents the  same characteristic roll off as that of an

electrical filter.  The roll off rate (or  slope) is:

roll   off   rate=(6    *   system   order)   dB/octave

or   roll off rate=(20 * system order) dB/decade.

PERFECT BOX can simulate both  infinite baffle (sealed box),

and  vented  (ported)  systems.  Each of  these  systems  have  a

natural order in which they  resemble  an  electrical  high  pass

circuit.   For  infinite  baffle, order=2, or that is to say that

they are of 2nd order.  And  for  vented  systems,  order=4.   It

seems  that  each  kind of enclosure has pros and cons associated

with them.

In general, both  systems, if properly aligned, perform with

the same low frequency response.  In practice, an infinite baffle

system is easier to build,  and  possibly  has  better  transient

response.   The  vented  system offers slightly higher efficiency

(2.9 dB), and good cone dampening  near  the  vent frequency (but

less below Fb.)  My preference is for 4th and 6th  order systems.

The  sixth  order  system involves the use of a 4th order  system

tuned slightly lower  than  the 4th order ideal, and an 2nd order

active equalizer circuit in the amplifier path.

***SYSTEM CUTOFF FREQUENCY***

This is the frequency at which  the  system amplitude response is

exactly 3.0 dB less than at a mid-band reference frequency (where

response is basically flat at 0 dB.)  This is a standard point of

interest that can be used to compare with  another  system's  low

frequency  response  data.   This  point  on  the  frequency plot

represents  the  "half-power"  amplitude,  as  compared  to   the

amplitude at a mid-band reference frequency.

***OTHER SYSTEM ORDERS***

As  you  are  problably  aware,  a  Graphic  Equalizer can make a

tremendous  effect on the way your speaker  system  sounds.   The

bad news is that an equalizer (EQ) usually doesn't have a control

exactly centered  on  the proper frequency to properly extend low

frequency cutoff, and thus  will  typically  make  matters worst.

PERFECT  BOX  can  incorporate  the  effects of either a  1st  or

2nd order, ACTIVE EQ (you must build) into the loudspeaker model.

This mini EQ consists of a simple active high-pass filter, having

a low turn-over (cutoff) frequency (typically  20-60  Hz),  which

actually  has  a  slight  boost  (peak) in its amplitude response

above its cutoff frequency.  If the  amount  of  boost and cutoff

freq. are suitably chosen, the total system response typically is

extended  a  half-octave  or  more,  while  providing  infrasonic

(rumble)  filtering  to  boot!   In 2nd-order EQ's, the boost  is

controlled by the "D" parameter (D=1/filter  Q,  for  you  filter

buffs.)   Typical  values  of "D" range from 0.3 to 1.414, giving

most to least amount of amplitude  boost.   A  "D"  of 0.52 gives

approximately 6 dB of boost,  a typical starting value.   The  EQ

frequency  and  the "D" factor should be juggled for the flatest,

most extended low frequency response.

A word of caution:   using  large  amounts  of boost may tax

small  power  amplifiers  (less  than 50 Watts,RMS) beyond  their

limits.  A properly designed 3, 3.5,  5, or 6th order system will

not over tax your amplifier unless you listen to music either :

1.  at extremely high SPL levels,

2.  with heavy (double fortisimo) organ pedal notes, or

3.   use  other  signal  modification  equipment   such  as  an

extremely   low-end   boosted  Graphic  Equalizer  (Stupid),   or

Sub-Sonic Synthesizer (Harmonizer.)

The EQ can be inserted  into  the signal path of your system

at  either  a  "Preamp-out/Amp-in"  loop,   or   "tape   monitor"

connection.  A simple design for a 2nd order EQ is provided  with

PERFECT BOX.  Type EQ2  at the DOS prompt to execute this option.

You  must have (at least) a CGA graphics card in order to run EQ2

because  the electrical skematic is drawn on the screen after all

the circuit values are calculated.  You may execute a screen dump

(PrtSc) for a hardcopy, as with all screens in PERFECT BOX.

IV.  RUNNING PERFECT BOX:

PROGRAM EXECUTION:

If you  have  an  IBM  (or Epson graphics) printer, you can

print the graphic plots generated  by  PERFECT  BOX.   Before you

start  PERFECT  BOX,  you  must  execute  either your DOS command

GRAPHICS.COM, or the PERFGRAF.COM (included).  These TSR programs

set  aside memory for doing graphic dumps of  your  CRT  to  your

graphics  printer.   Use  GRAPHICS.COM  if  you  are  using a CGA

graphics  card.   If  you  have  EGA  capability,  you  can  take

advantage of the higher resolution by using PERFGRAF.COM.

(You might have to set your printer to do an "automatic" Linefeed

[LF] after a Carriage Return [CR], see your printer manual.)

I  have  created  a  simple  batch file called GO.BAT, which

will  execute GRAPHICS.COM for you.   Of  course  you  must  have

either  GRAPHICS.COM  on  the  program disk or in the current DOS

path.  Change this batch file to suit your particular needs.  The

actual program file is BOX4_1.EXE.   You  will not have to change

the  program  file  name  until you recieve a future  upgrade  of

PERFECT BOX.

Insert the PERFECT BOX  program  disk into your default disk

drive,  and type GO <Return>.  You should  see  the  PERFECT  BOX

opening screen.   Take  the time to read this screen and remember

that many, many hundreds  of  hours  of effort have been put into

this program.  If you use this program  more than five times, you

must be understanding the purpose so don't forget your obligation

to this author!

If  you  see  an  error  message  stating  "Bad  Command  or

Filename", check to see if you are in the  same  directory as the

PERFECT BOX files.  After about 5 seconds, you may  press  RETURN

to continue.

If all is well, you should see the following screen.

PERFECT BOX - Version 4.1            Current Time:

�͹ Boxdata Choices ������������ͻ   �����������������������������ͻ

�                                �   � Drivers on file :  145      �

� (B) Browse the database        �   � Boxdata path  : \qb4\       �

� (L) Load a driver by #         �   � Graphics mode : EGA         �

� (S) Store New Driver data      �   �����������������������������ͼ

� (E) Edit the database          �                       Quick Mode

� (B) Boxdata Reports       ����������������������͹ Utility �ͻ

�                           �                                  �

���������������������������͹ (C) Change Boxdata path          �

� (P) Sort/Pack Boxdata file       �

��������������������ͻ   � (A) Append new records           �

� REGISTERED TO:     �   � (W) Write defaults               �

� Shareware copy.    �   � (D) DOS Shell                    �

� Please! Your $$$   �   � (Q) Quit to DOS                  �

� help homeless      �   ����������������������������������ͼ

� woofers find a     �

� new home!          �

��������������������ͼ

Use PgUp - PgDn - Home - End - # - # - <RETURN>= Select - or HotKey

Each  of the menu choices may be executed by either pressing

the "key letter"  or  by using the cursor arrow keys to highlight

the choice and then pressing RETURN.

BOXDATA CHOICES:

(B)  Browse.  This option  will  allow  you  to  view the drivers

currently  in  the BOXDATA file.  You can move through  the  list

using the arrow, Home, End, Page Up, and Page Down Keys.  The end

key takes you to the bottom of the list, and Home will return you

to the first listing.   Use  these  keys  to  point the "flashing

arrows" at your driver choice.

Browse-Choose  the driver with the arrow keys and  press  RETURN.

�106  �PR 38 XM 150  �POLYDAX        �15",PRO,MONITOR,96dB     �

107   PR 38 XT 150   POLYDAX         15",PRO,FOAM EDGE,MONITOR

108   PR 38 XT 250   POLYDAX         15",PRO,99dB

109   PR 38 XT 350   POLYDAX         15",PRO,HI-POWER,100dB

110   TA-305 FSW     PRECISION       DUAL V.C. SUBWOOFER

111   TA-305F        PRECISION       OMEGA

112   TD-125F        PRECISION       ALPHA

113   TD-255F        PRECISION       EPSILON

114   TD-255F-SW     PRECISION       10",Alum Dual VC,90.7dB

115   TO-205F        PRECISION       INFINITE BAFFLE 8"

116   TP-165F        PRECISION       4 or 8 OHMS

117   TP-165R        PRECISION       6.5" , 88.5 dB

118   TX-205F        PRECISION

119   TX-205R        PRECISION       8",Mid-Bass,POLY, 90dB

120   TX-255F        PRECISION       4 or 8 OHMS

121   SPP-154        ROCKFORD-FOSGA  AUTOMOTIVE

122   11FGX          SEAS            PRO 4.5",CAST FR.,4/8,$32

123   CA17RCX        SEAS            WILSON WATT $32

124   P 25 REX       SEAS            10 " SEALED BOX

125    P14RCY           SEAS             5.25",WESTLAKE,POLY,$34

<F1>=Quit - <F2>=Edit - <F3>=Load defaults.   PgUp-PgDn-Home-End-

# - #  <INS>=Get #   <#��>= Get Record 106

Perfect Box Browse function.

After pointing to a particular  driver,  you  may LOAD the driver

parameters   by  pressing  RETURN.   You  can  EDIT  the   driver

parameters by  pressing the <F2> function key.  The <F3> function

key will load any  previous  Box Volume, Vent Frequency, Order, D

factor,  and  EQ  frequency  information   saved  in  a  previous

execution of PERFECT BOX, and initiate the calculation loop.  You

still can bypass the current selected driver  and  load any other

driver by pressing <INSERT> and entering the record  number.  The

<F3>  function  key  will take you back out to the opening  menu.

When you return to the  BROWSE function, the BOXDATA list will be

on the same page as the last time you were there.

(L)  LOAD:

This option allows you to  LOAD  driver  data  directly  from the

BOXDATA  file  using the record number.  On Floppy based systems,

this option might save you time by skipping the Browse feature.

(S)  STORE:

This option  allows  you  to  enter new driver data into the

BOXDATA file.  ALL parameters listed  are required, so don't skip

any unless you are told you may enter a  Zero.  If you don't have

all of the correct T/S parameters, you should  try to obtain them

from  the  manufacturer.  In a pinch, you can approximate  values

from other drivers  in the BOXDATA file.  Don't forget to add the

correct values when you  get  them.   Of  course, the accuracy of

this  loudspeaker  model  is  DIRECTLY proportional  to  the  T/S

parameter accuracy!  Which brings me to say this:

I claim no responsibility  for  the completeness or accuracy

of the T/S parameters included with  your  copy  of  PERFECT

BOX.   Many (very many!) of the current drivers listed  have

been appended  from  files sent to me by Registered users of

PERFECT BOX.  If you are  unsure  of  any  data,  it is your

responsiblity   to   verify  the  T/S  parameters  with  the

manufacturer or test them yourself!

(Soapbox off.)

The new entries are placed  at  the  bottom  of the file and

once entered, can be LOADED immediately.  It is for  this  reason

that  I  eliminated  the  "Manual  Parameter  Entry"  function of

previous  releases  of  PERFECT BOX.  I usually want to keep  new

parameters and never used  that function so I hope you don't miss

it!

(E)  EDIT:

You can edit any of the  T/S parameters in the BOXDATA file.

Use the current driver record number  to  call  up the driver and

follow the screen prompts for instructions.

(B)  REPORTS:

You can print the full BOXDATA file on your  printer  in two

forms.   The  first  form  contains  ALL of the driver parameters

including  comments.   The second form will  print  the  driver's

basic T/S parameters, Fs, Vas, and Qts, and use them to calculate

the "Ideal" 4th order Box  volume,  Vent frequency, and F3 cutoff

point.  This form is very handy to compare driver performance.

UTILITY

(C)  Change Boxdata Path:

The BOXDATA file containing all  driver  parameters,  can be

located  on  a  different  drive/directory  than  the current DOS

default.  Choose this option if you want to change  the  path  to

the data file.  Note:  The path is saved for future executions of

PERFECT BOX using the Write Defaults option.

(P)  Sort/Pack

Each  new  driver  that  you  enter into the BOXDATA file is

placed  at the bottom of the file.  You  can  use  the  Sort/Pack

option to  sort  the file alphabetically by Manufacturer.  If you

have marked a driver  for  deletion  in the EDIT driver menu, the

Sort/Pack  routine  will purge it from the  BOXDATA  file.   This

routine is the same sort  program  that was included with earlier

versions of PERFECT BOX, and thus (if  you  have  them)  you  can

delete the old BSORT.BAT and BOXSORT3.EXE program files.

(A)  Append new records

This  option is used to combine other BOXDATA files into the

current, active  BOXDATA file.  If you are upgrading from Version

3.xx, rename the OLD  data  file  to "DATAUP.DAT".  Start PERFECT

BOX and choose the Append option.   If  you  are  upgrading  from

Version 2.xx, you must run the 2to4.exe program first.  Version 3

uses  a  slightly different file structure, and thus the 2to4.exe

program will convert you old BOXDATA file into the new structure,

leaving a new file called "BOXDATA4".

Since this is the current name of the Version 4.x data file,

YOU MUST DO  THIS  CONVERSION  IN  ANOTHER DIRECTORY THAN THE ONE

CONTAINING  THE NEW BOXDATA4 FILE INCLUDED  IN  THE  VERSION  4.0

ARCHIVE!  Either  use  a  different  directory  or  copy the NEW,

original  BOXDATA4  to  another  directory  for safe keeping.   I

suggest that you make backup copies of EVERYTHING,  OLD  AND NEW,

before  attempting  the  conversion  process.   After running the

2to4.exe  program,  rename  the resulting file from  BOXDATA4  to

"DATAUP.DAT".   Copy  "DATAUP.DAT"  file  into  the  PERFECT  BOX

directory, choose the APPEND function, and watch it fly!

(W)  Write defaults.

Each time PERFECT BOX is loaded, a file named PERFECT.CTL is

read for startup defaults.   There  are four default values kept

in  this  file.   1.   Boxdata4  data file  drive/directory;   2.

Graphics mode;  3.  "Quick mode" toggle. and 4. Printer type

The  first  two  values have obvious  purposes.   The  third

parameter is used to hold  the  default  mode for the calculation

loop end point.  In normal mode, the loop  counts  up  to 200 Hz.

In Quick mode, the loop counts up to 100 Hz.  This is useful  for

making quick estimates on the slower "XT" type machines.  You can

toggle this mode within PERFECT BOX with the <F9> function key.

The  Write defaults option will over-write the configuration

file with current  values  for  BOXDATA  file  path, Graphics and

Quick Modes.

(D)  DOS Shell

You can spawn another copy of COMMAND.COM from  PERFECT  BOX

using  this  option.  Use this option to check directorys or file

space.  Type EXIT at the DOS prompt to return to PERFECT BOX.

(Q)  Quit to DOS

Just like  it  says!   This  choice  will  halt execution of

PERFECT BOX and immediately (without any verbose  advertisements)

return you to DOS.

OK, OK!, enough talk.  Let's see some action!

Now  that  you  know  what all of that stuff does on the  opening

menu, choose a driver from  the  BOXDATA  file.   Use  the BROWSE

option, point at an interesting driver, and press RETURN.

The parameters listed are the minimum that you will need  to know

about  any  driver simulated with PERFECT BOX.  For this example,

answer "Y" to the question "Do you want this driver?"  Now choose

"1" for a single driver simulation.

Record # ���������#  106

Manufacturer �����# POLYDAX

Model Number �����# PR 38 XM 150

Comment ����������# 15",PRO,MONITOR,96dB

Free air resonance=  19.5 hz.

Vas =  24.37 ft^3.

Qts =  .27

Qes =  .3

Qms =  2.7

Re =   5.8 �

Xmax=  .1377953

Piston Diameter=  14 in.

Power= 150 Watts.

DO YOU WANT TO USE THIS DRIVER (Y)?

Now, PERFECT  BOX  will  calculate the "Ideal" 4th Order Box

volume, the port frequency for  that  ideal  box  volume, and the

cutoff  frequency  (-3dB  point.)   The ideal box volume  is  the

smallest  enclosure  that  provides  the   flatest,  deepest  low

frequency  response.  The cutoff point will give  you  a  general

idea of how  low  the driver will perform in any box design (best

case.)   The  port frequency  (box  frequency)  must  be  set  as

calculated to  obtain the cutoff predicted.

In a 4th order system:       Ideal vented volume is  8.53  Ft^3.

System cut-off (-3dB) is  32.54  Hz.

If the Vent frequency is  26.61  Hz.

�������������������������������������������������������ͻ

� ORDER �             SYSTEM DESCRIPTION                �

�������������������������������������������������������͹

�   2.  �  CLOSED BOX���without Active Equalizer        �

�   3.  �             ��with 1st Order Active Equalizer �

�  3.5  �             ��with 2nd Order Active Equalizer �

� � 4.  �  VENTED BOX���without Active Equalizer        �

�   5.  �             ��with 1st Order Active Equalizer �

�   6.  �             ��with 2nd Order Active Equalizer �

�������������������������������������������������������ͼ

Efficiency Bandwidth Product..(Fs/Qes).. is 64 , which suggests a

4th  order system.

Enter System Order [ 4  =default] ?

Enter BOX VOLUME (Ft^3) [ 8.53 =default] ?

Enter BOX VENT FR. (Hz.) [ 26.61 =default] ?

Now you must  decide  what type of enclosure to use.  If the

driver has a Qts of 0.5 or greater,  I  would try infinite baffle

(order=2) systems first, otherwise, use Order=4,  with  the ideal

box size and frequency.  Save the other Order options for  later.

Since  PERFECT BOX will recommend the Order for you, use it as  a

default.  Press RETURN to accept the default Order.

Enter  the Box volume in cubic feet.  I suggest that you use

the ideal box  size as a starting point.  Then, on the next pass,

change one of the  box  alignments and check how the response was

affected.  Again, press RETURN  to accept the default Box volume,

or change it if you prefer to do so.

Enter the Vent Frequency in  Hertz.   Again,  since  this is

your  first  time  through,  press  RETURN  to accept the default

value.

PERFECT BOX will now calculate :

1.  Amplitude response in decibels;

2.  Maximum Power in Watts;

and  3.  Maximum Sound Pressure Level, at each frequency cycle

between 10 and 200 Hertz.  (100  Hz  in  Quick mode.)

Please wait ... Calculating data for the frequency of  xx  Hz.

Be patient here.  PERFECT  BOX  is running through a healthy

chunk of floating point calculations,  just  sit  back, watch the

counter change and reminisce about the good old slide  rule days.

When the calculations are complete, an Outputs/Changes Menu  will

be    presented.

PERFECT    BOX    -   Version   4.1

Current Time:

�ͺ Output Choices ������������ͻ ��������������������������������ͻ

�                               � � Record # :  106                �

� (F) Bode Plot (Frequency)     � � Driver   : PR 38 XM 150        �

� (L) Excursion Limit plot      � � Manufacturer: POLYDAX          �

� (V) View data points          � ��������������������������������ͼ

� (D) Print Data points         �                    Quick mode

� (P) Calculate Ports  �����������͹ Changes ��������������ͻ

� (W) Driver Worksheet �                                    �

�                      � (B) Change Box Volume or Vent freq.� �=2.857

����������������������͹ (E) Change EQ. Frequency or D      � h=1.365

� (A) Change All parameters          �

� (T) Toggle graphics mode (EGA)     �

� (C) Change QLoss    ���������������ʹ Done̻

�       (Qloss = 7 )  �                      �

���������������������͹ (S) Load/Save Align. �

� (R) Retrieve another �

� (Q) Quit to DOS      �

�                      �

����������������������ͼ

Use PgUp-PgDn-Home-End- # - # - <RETURN>= Select - or HotKey

Output/Changes Menu

OUTPUT CHOICES:

(F)  Bode Plot

This  choice  will  plot  the classical Bode (pronounced  BO

DEE; or Frequency Response) plot.   If  the  Quick  mode  was  on

during  calculations,  the  plot  will stop a 100 Hz.  If you had

loaded the GRAPHICS.COM or PERFGRAF.COM  before  loading  PERFECT

BOX, you may print a copy of the plot on your graphics printer by

pressing  a  "P".   This  pops  up  a  nice  time-date stamp, and

requests  an  interrupt 5 (Print Screen) to execute.   After  the

printer is finished,  you  can  send  a  Form  Feed  (FF) to your

printer by pressing the <F10> function key.  If you are using the

PERFGRAF.COM  in EGA mode, you will be able to fit two  plots  on

one page.

(L)  Excursion Limit Plot

This graph  of  SPL  & Power vs. Frequency, will give you an

picture of the system driven  at  maximum power.  This plot gives

the Sound Pressure Level (in dB) and  the input power required to

reach  the excursion limit (Xmax) of the  driver.   See  the  the

Design Notes  chapter of this document for more discusion of this

important graph.

(V)  View data points

This option  will  list  on the screen, all calculated data,

between the frequency limits of your choice.

(D)  Print data points

This option will print on your printer, all calculated data,

between the frequency limits of your choice.

(W)  Driver Worksheet

This  option produces on your  printer,  a  worksheet  which

gives you a  handy  reference  table  to  use  as  you change the

different alignments about your system.  Each block can be filled

from data printed on the Bode plot option.  I suggest that you do

this option first (only if you have a printer on line.)  and  log

each repetition of your progress.

(B)  Change Box Volume or Vent Frequency

(E)  Change EQ. Frequency or D

(A)  Change ALL parameters

At  this  point  you  may  choose  what parameter to change in an

attempt to optimize the frequency plot, or what ever else you are

trying to achieve.  If you are currently  working on a 2nd or 4th

order system, the "Change EQUALIZER Frequency  or D." choice will

not  be  allowed.   Thus if you want to change to an  "equalized"

order, you must use the "Change ALL Parameters" option.  Remember

that the previous value  for  each prompt in the CHANGE menu will

be the default value (just press RETURN.)

(T)  Toggle graphics mode

This choice toggles you between  EGA and CGA modes.  This is

really not needed anymore.  I left it in  for  those  of you whom

can't get the PERFGRAF.COM program to function properly with your

printer

(C)  Change Qloss  (New to version 4.x)

This  option  allows you to account for box leakage  losses.

Normal vented cabinents  are modeled at Ql=7.  If you do not have

the means to accurately measure Qloss, then leave Ql=7.

(S)  Load/Save Alignment

This  choice  allows  you   to   save  your  current  design

parameters to the BOXDATA file for future use.  This becomes very

handy if you want to keep track of your  best  designs  for  each

driver.

(R)  Retrieve another driver

This choice will dump all calculated data, and return you to

the opening menu.

(P)  Calculate Ports

After  you are satisfied with the current design parameters,

PERFECT BOX will  calculate the port lengths required to tune the

cabinet to the frequency  currently  chosen.   A  column  for two

identical length ports is also given.  Dual ports are useful when

the box volume or the MACH speed is too large for a single  port.

The  MACH  number  is the percent of the speed of sound that will

occur at the largest port output.

POLYDAX             PR 38 XM 150

BOX VOLUME=  8.53  Ft^3   BOX FREQUENCY=  26.61 Hz.

ORDER :  4

HEIGHT=  39.72  in.   WIDTH=  24.52  in.   DEPTH=  15.2  in.

VENT LENGTHS FOR  TYPICAL  INTERNAL  DIAMETERS  OF  PVC  PIPE

(Inches)

DIAMETER      LENGTH       MACH       DUAL VENTS    MACH

-------       ------       ----       ----------    ----

1           -.38        1.243 NO !    |   -.03       .311  NO!

1.5         -.31        .553  NO !    |    .48       .138

2           -.06        .311  NO !    |   1.35       .078  Best

2.5          .37        .199  Use Dual|   2.57       .05   Best

3            .97        .138  O.K.    |   4.13       .035  Best

3.5         1.75        .101  O.K.    |   6.05       .025  Best

4           2.7         .078  Best    |   8.32       .019  Best

4.5         3.83        .061  Best    |  10.95       .015  Best

5           5.13        .05   Best    |  13.92       .012

Vent Calculations.

Stay  away  from large (speed> 0.2) port MACH speeds as they

will cause unmusical  whistle  noises  from the port.  Ports that

are longer than the box depth, can be curved inside the enclosure

with PVC swept elbows.

DESIGN HINTS:

If  the ideal box volume is too large  to  be  realistically

built, PERFECT BOX will allow you to see just how the loudspeaker

performance will suffer, as you down size box volume.  It is this

iterative  design  process  that PERFECT BOX is designed for.  By

changing either box volume, port  frequency,  or  in EQ cases, EQ

frequency and boost, a "perfect" frequency curve can be obtained.

At  this point you can tell how good a driver really  is  because

sometimes  another  driver  will  out  perform others in half the

enclosure volume.

The  port frequency is set by the area  and  length  of  the

port.  PERFECT  BOX  will  calculate  the  port  for you, but the

length  is  typically  10-15%  off,  so add an half-inch  to  the

calculated  value, and measure the box  frequency  for  yourself.

The system will  not  perform  as calculated if you miss the port

frequency by as little as 1 Hz,  so  try  to  hit  it as close at

possible.   Also,  after  you  have  reached  a  final box  size,

increase  the  box volume by as much as required to  account  for

volume used by the  drivers,  bracing, and crossover network.  It

is  much better to error on the  high  side  of  Box  Volume.   I

typically  add  10% to the final size as a fudge factor.  You can

always add braces to account for any overage.

To find box  diminsions from the box volume, take the volume

in cubic feet, multiply by 1728 cubic in/ft^3, to get in^3.  Then

take the cube root  (or raise it to the 0.3333334 th power.)  Now

this number is the width  in  inches.  Multiply the width by 1.62

for the height, and multiply the  width  by  0.62  for the depth.

volume (ft^3) * 1728 = volume (in^3)

width     (in.) =  (volume   (in^3))^0.3333334

height (in.)    =   width * 1.62

depth  (in.)    =   width * 0.62

NOTE: All diminsions are internal, not external!

This will give the box a "typical speaker look" and also set

the diminsions up for minimum internal  reflections.  PERFECT BOX

calculates these diminsions for you and prints  them  in the port

length table.

VI.  REFERENCES:

This document and the program PERFECT BOX are only tools for

the  speaker  designer.   One  must  know  how to use these tools

properly before judging the usefulness of PERFECT BOX.  I suggest

that you read any of the following publications  on the finer art

of speaker building.

SPEAKER BUILDER MAGAZINE (6 times a year)

Edward T. Dell, Jr., Publisher

PO Box 494 Peterborough, NH 03458

"The Loudspeaker Design Cookbook"

by Vance Dickason

Distributed by:

Old Colony Sound Lab

PO Box 243

Peterborough, NH 03458-0243

"How to Design, Build, & Test Complete Speaker Systems"

by David B. Weems

Library of Congress #TK9968.W43

Published by:

TAB Books

Blue Ridge Summit, PA  17214

No. 1064

See the PERFECT1.DOC for more details on new additions.

VIII.  ERRATA:

Your support helps!  PERFECT BOX  is  supported  by  you, the

user.   Without  user support, Shareware such as PERFECT BOX  can

not continue to exist!  Use this program!, give a copy of PERFECT

BOX to your friends  (the   BBS  Archive  file is the best way to

make  sure all the PERFECT BOX  files are present.)   Above  all,

don't forget your part in the deal!  If you are using PERFECT BOX

on a trial basis, that's fine.

If you  are  using  PERFECT  BOX  in  a  business or commercial

environment,  you must Register, PERIOD.  Many  registered  users

are using PERFECT  BOX  for  professional  sound reinforcement or

automotive  sound applications.  If this is true  for  you,  just

don't forget the concepts under which shareware functions.

PERFECT BOX is under constant scrutany for improvement.  If you

have any ideas for new features/functions, or problems in program

operation, send  them to me at AUSTIN ACOUSTICS.  Your suggestion

could be included in the next revision of PERFECT BOX.

By registering your copy, you will be sent (if requested) a disk

with the current Version  installed  in your name.  You will also

receive a copy of the next MAJOR PERFECT  BOX  revision,  and new

BOXDATA file parameters.  Registered users can have their current

copy  updated  at  any time.  Also, Registered users can download

the future releases  of  PERFECT BOX directly from my 24 hour BBS

at FIDONET 363/507 (407-281-5522:2400-N-8-1),  including unlimited

"Online  support" through BBS E-Mail (See Perfect1.doc  for  more

information.)

If you want  to others to share in your BOXDATA resources, please

send a COPY of  your BOXDATA file (5.25" or 3.5") in a U.S.  Post

Office approved disk  envelope  (or safe equivalent.)  Or, upload

your BOXDATA file to my BBS.

TO REGISTER:

Send $40.00 in U.S. funds (personal  check  or  money  order)  to

cover registration and return postage.  IT'S THAT SIMPLE!

The un-registered copy is NOT "crippled" in any way.  All PERFECT

BOX functions are available on the un-registered program.

Please make checks payable to:

WARREN MERKEL

248 Whiterapids, Ct.

ORLANDO, FL  32828

*** End of Document ***