*** Perfect1.doc ***

I  want  to comment on the BOXDATA records that I  have been

receiving   on  your disks.  First, I think that  many  users are

confusing the  definition  of  the X-max parameter.  The  typical

ranges for X-max range from 0.1 inches to 0.25 inches with 0.5 on

the JBL 18" 2245H being the largest I've seen (Have you ever seen

one,  they  are  huge.)   If  you  don't  know  the  X-max value,

approximate it with 0.1 to 0.15, 0.2 for large suspensions.

Note  that   X-max   is  NOT  the   total   possible   cone

movement  allowed   by  the suspension, which  on  large  drivers

could   be  at least   an   inch.   I   have  seen  specification

sheets  from  a  few manufacturers (Credence is one) that give an

"X-max" and a "X-lin" (for X-linear?) value. As far as I can tell

from these,  the   X-lin is really the value of X-max as normally

defined.  And the  "X-max"  value  is  the  maximum  cone  travel

possible.     Contact the manufacturer  if you are confused about

their   specifications.    I   hope   that  a  standard  for  "X"

measurements will surface soon.

The X-max measurement defines the peak (not to be  confused

with  peak-to-peak)  distance  that  the voice  coil  can  travel

without extending beyond the magnetic  field  as  determined   by

the  pole  piece  length.  The voice coil may be longer in length

than  the  pole  piece.    This   is  defined  as   "voice   coil

overhang."    The  PB  program  uses  the  X-max  measurement  in

calculating the Maximum Sound Pressure  Level  (SPLmax), and thus

is a key factor in the Excursion Limit Plot.  Theoretically,  the

driver  is  "linear"  only if the cone travel is within the X-max

limits!  Think  about  this  the next time you watch your low-end

drivers move 3/4 " peak, from the rest position!

In general, (not always,)  low  Q  drivers  are intended for

4th  order   systems,  and  typically  have  what  is  coined  as

"long-throw  suspension."  Also, remember that PERFECT BOX  model

is valid up to Qts of 0.7, and the Marquis 12G38BPP1H is slightly

over that value at  0.72.  Of course, each individual driver will

vary from the manufacturers published specs, so you might want to

measure them yourself to be sure.

Good luck  in your new designs, thanks again for all of your

generous support.   If  you  have any specific questions or would

like  chat about loudspeaker topics, drop me a letter, or call my

BBS, "UCF Health Topics" at (407)-281-5522, 2400 N-8-1.

You will find the latest release of Perfect Box posted here.

My BBS is an node  in  FidoNet,  (1:363/507), and thus

will  accept  "Electronic Mail" 24 Hours a day.   If  you  are  a

FidoNet  SysOp   or   know   a   friendly   one   with   a  "FIDO

compatible   Mailer",  you  can  file  request  PB with the magic

filename  "PERFECT",   for   the   current  MS-DOS  version,   or

MACPERF    for  Macintosh   (when  available.)   Of  course,  you

could   always   File  Request  by   the   current  archive  name

"PERF410.ARC."    Also,   I  host  a  local  echo-mail   (E-mail)

roundtable called, "Audio Topics".   You  can find many questions

regarding Loudspeakers, etc. answered there.

You now can save the "RAW data" to an ASCII comma-delimited file

for use with other graphics programs.  Press F7 at the Output

Menu, to Create a new file, and F8 to re-load a previously

saved file.  You can "jump" to the Output Menu from the Opening

Menu by pressing F6.  Thanks to Registered User, Bob Schiers, of

Ames, IA, for this very helpful suggestion.

Warren A. Merkel

*** End of Document ***