Note: Print using WS 3.3

Multiple-tuned Amplifier Program

Jack Porter

Cubic Corporation

P. O. Box 85587

San Diego, CA 92138

(619) 277-6780

Permission to publish the above information is granted.

The program was developed using a TRS-80 Model 4 computer with the DOSPLUS

IV operating system and Model 4 Basic, a version of Microsoft Basic which

is very similar to the one used on the IBM PC.  The program should run

with very minor changes using any version of Microsoft Basic.  The effects

of any statements which are peculiar to Model 4 Basic, DOSPLUS, or the

Gemini 10 printer are explained by remarks in the program at those lines.

The program requires 16K of memory and a monochrome display. The display

formats in the program use 80 columns and 24 lines.

Abstract:

This program is used to design tuned amplifier coupling networks which

consist of one to eight tuned circuits each.  These filter networks may

be of any conventional filter type, such as Butterworth, Chebyshev or

Bessel.  The program calculates capacitor, inductor and resistor values,

and transformer turns ratios required to form a filter network with a

specified shape, center frequency, and bandwidth which will provide the

desired mismatch ratios for the transistors which are coupled by it.

The program also calculates low-pass prototype element values for

Butterworth and Chebyshev filters. For other types of filters these

must be entered from the keyboard.

Variable Descriptions:

Am: Chebyshev filter passband ripple in dB.  Am=0 for Butterworth filters.

Ap: Power gain in dB.

BS, BL: Imaginary parts of source and load admittances.

BW: Bandwidth in MHz.

Ck: Tuning capacitance of k'th tuned circuit in pF.

Ck,k+1: Coupling capacitance between tuned circuits k and k+1.

Ct: Total capacitance at each node in pF.

F0: Design center frequency in MHz.

Fc: Corrected center frequency used in calculations.

GS, GL: Real parts of source and load admittances.

K: Stern stability factor.

L: Tuned circuit inductance in nH.

Mia, Moa: Input and output mismatch ratios for transistor a.

Mib, Mob: Mismatch ratios for transistor b.

MS, ML: Source and load mismatch ratios.

N: Number of tuned circuits.

N1, N2: Turns ratios at filter input and output.

Q0: Reciprocal of filter fractional bandwidth.  Q0=Fc/BW.

Q1, Q2: Q's of the first and last tuned circuits in the filter.

QL: Loaded Q of a single-tuned circuit.

Qu: Unloaded circuit Q; approximately equal to inductor Q.

r: Low-pass prototype filter termination resistance for N odd,

or conductance for N even.

R1, R2: Loading resistance values at filter input and output.

S: Alignment sensitivity.

Y11, Y21, Y12, Y22: Transistor short circuit admittance parameters.

Enter these values in data statements in that order,

real part of each Y parameter first.

Yi, Yo: Transistor input and output admittances.

Sample program run showing text which is displayed on the screen:

RUN "TUNEDAMP/BAS"

TUNED AMPLIFIERS                                           05/12/86

N, Am(dB)? 3, -1

Prototype element values

1 ? 2.2034

2 ? .97051

3 ? .33742

Prototype element values:

1  2.2034       2  .97051       3  .33742

r= 1

Print element values? Y

TUNED AMPLIFIERS   N= 3    Am=-1                           05/12/86

Transistors:

1. MRF904 10V 2.5mA  180 MHz            2. 2N4957 10V 2.0mA  180 MHz

Coupling network types:

1. Interstage  2. Input  3. Output? 1

Transistors? 1, 2

Mia, Moa, Mib, Mob? 10, 20, 8, 12

Ka= 2.833   Sa= .64357                   Kb= 5.0985   Sb= .454846

Are K and S OK? N

Mia, Moa, Mib, Mob? 10, 50, 10, 20

Ka= 6.88015   Sa= .378057                Kb= 10.0663   Sb= .270341

Are K and S OK? Y

TUNED AMPLIFIERS   N= 3    Am=-1                           05/12/86

Transistors:

1. MRF904 10V 2.5mA  180 MHz            2. 2N4957 10V 2.0mA  180 MHz

Mia= 10            Moa= 50             Mib= 10             Mob= 20

Ka= 6.88015        Sa= .378057         Kb= 10.0663         Sb= .270341

F0(MHz), BW(MHz), Qu? 180, 15, 120

Options:  1. Select C12  2. Select Ct  3. N1=1  4. N2=1? 1

C12(pF)? .5

Fc= 179.844 MHz                 Ap= 16.5514 dB

Q1= 26.4178     Q2= 4.04552     Qu= 120

R1= 6207.32     R2= 679.082     N1= 2.53424     N2= 2.54096

L= 89.3367 nH  Ct= 8.76639 pF

C 1 = 7.80128   C 2 = 6.98868   C 3 = 5.48921

C 12 = .5       C 23 = 1.27771

Print results? Y

New C? N

New M's? N

TUNED AMPLIFIERS   N= 3    Am=-1                           05/12/86

Transistors:

1. MRF904 10V 2.5mA  180 MHz            2. 2N4957 10V 2.0mA  180 MHz

Coupling network types:

1. Interstage  2. Input  3. Output? 0

TUNED AMPLIFIERS                                           05/12/86

N, Am(dB)? 2, .1

Specify:  1. Ripple BW   2. 3 dB BW? 2

Prototype element values:

1  1.6545       2  1.22071

r= .737811

Print element values? N

TUNED AMPLIFIERS   N= 2    Am= .1                          05/12/86

Transistors:

1. MRF904 10V 2.5mA  180 MHz            2. 2N4957 10V 2.0mA  180 MHz

Coupling network types:

1. Interstage  2. Input  3. Output? 2

Transistor? 1

GS, BS, MS, Mib, Mob? .02, 0, 2, 10, 50

K= 6.88015      S= .378057

Are K and S OK? Y

TUNED AMPLIFIERS   N= 2    Am= .1                          05/12/86

Transistor:

1. MRF904 10V 2.5mA  180 MHz

MS= 2                                 Mib= 10             Mob= 50

K= 6.88015          S= .378057

F0(MHz), BW(MHz), Qu? 180, 15, 120

Options:  1. Select C12  2. Select Ct  3. N1=1  4. N2=1? 1

C12(pF)? .5

Fc= 179.844 MHz                 Ap(Transducer)=-5.13807 dB

Q1= 19.8367     Q2= 19.8368     Qu= 120

R1= 12263.2     R2= 5653.23     N1= 7.86234     N2= 4.82723

L= 91.9257 nH  Ct= 8.51949 pF

C 1 = 8.01949   C 2 = 7.50881

C 12 = .5

Print results? Y

New C? N

New M's? N

TUNED AMPLIFIERS   N= 2    Am= .1                          05/12/86

Transistors:

1. MRF904 10V 2.5mA  180 MHz            2. 2N4957 10V 2.0mA  180 MHz

Coupling network types:

1. Interstage  2. Input  3. Output? 0

TUNED AMPLIFIERS                                           05/12/86

N, Am(dB)? 1, 0

Prototype element values:

1  2

r= 1

Print element values? N

TUNED AMPLIFIERS   N= 1    Am= 0                           05/12/86

Transistors:

1. MRF904 10V 2.5mA  180 MHz            2. 2N4957 10V 2.0mA  180 MHz

Coupling network types:

1. Interstage  2. Input  3. Output? 3

Transistor? 2

Mia, Moa, ML, GL, BL? 10, 20, 1, .02, 0

K= 10.0663      S= .270341

Are K and S OK? Y

TUNED AMPLIFIERS   N= 1    Am= 0                           05/12/86

Transistor:

2. 2N4957 10V 2.0mA  180 MHz

Mia= 10            Moa= 20               ML= 1

K= 10.0663         S= .270341

F0(MHz), BW(MHz), Qu? 180, 25, 120

Options:  1. Select C12  2. Select Ct  3. N1=1  4. N2=1? 3

Fc= 179.565 MHz                 Ap= 20.4109 dB

QL= 7.18262     Qu= 120

R1= 1033.43                     N1= 1           N2= 3.50168

L= 37.8275 nH  Ct= 20.7677 pF

C 1 = 18.8655

Print results? N

New C? Y

Options:  1. Select C12  2. Select Ct  3. N1=1  4. N2=1? 2

Ct(pF)? 8

Fc= 179.565 MHz                 Ap= 20.4109 dB

QL= 7.18262     Qu= 120

R1= 2682.74                     N1= 1.6112      N2= 5.6419

L= 98.1986 nH  Ct= 8 pF

C 1 = 7.26726

Print results? Y

New C? N

New M's? N

TUNED AMPLIFIERS   N= 1    Am= 0                           05/12/86

Transistors:

1. MRF904 10V 2.5mA  180 MHz            2. 2N4957 10V 2.0mA  180 MHz

Coupling network types:

1. Interstage  2. Input  3. Output? -1

Ready

Printer output from sample program run:

TUNED AMPLIFIERS         05/23/86

N= 3

Prototype element values:

1  2.2034          2  .97051           3  .33742

r= 1

Transistors:

MRF904 10V 2.5mA  180 MHz              2N4957 10V 2.0mA  180 MHz

Y11= +1.730E-03 +5.063E-03j            Y11= +2.585E-03 +6.348E-03j

Y21= +6.583E-02 -2.481E-02j            Y21= +5.396E-02 -2.084E-02j

Y12= -2.305E-05 -5.248E-04j            Y12= -7.264E-08 -4.480E-04j

Y22= +7.946E-05 +1.590E-03j            Y22= +1.397E-04 +1.296E-03j

Mia= 10            Moa= 50             Mib= 10             Mob= 20

Ka= 6.88015        Sa= .378057         Kb= 10.0663         Sb= .270341

Yia= +5.317E-03 +1.345E-02j            Yib= +5.769E-03 +1.459E-02j

Yoa= +8.434E-04 +3.375E-03j            Yob= +4.682E-04 +2.146E-03j

N= 3

F0= 180 MHz        BW= 15 MHz          Fc= 179.844 MHz

Ap= 16.5514 dB

Q1= 26.4178        Q2= 4.04552         Qu= 120

R1= 6207.32 6      R2= 679.082 6

N1= 2.53424        N2= 2.54096

L= 89.3367 nH     Ct= 8.76639 pF

C1= 7.80128 pF

C12= .5 pF

C2= 6.98868 pF

C23= 1.27771 pF

C3= 5.48921 pF

TUNED AMPLIFIERS         05/23/86

Transistor:

MRF904 10V 2.5mA  180 MHz

Y11= +1.730E-03 +5.063E-03j

Y21= +6.583E-02 -2.481E-02j

Y12= -2.305E-05 -5.248E-04j

Y22= +7.946E-05 +1.590E-03j

MS= 2                                 Mib= 10             Mob= 50

K= 6.88015          S= .378057

GS= .02            BS= 0              Yib= +5.317E-03 +1.345E-02j

Yob= +8.434E-04 +3.375E-03j

N= 2              Am= .1 dB          3 dB BW

F0= 180 MHz        BW= 15 MHz          Fc= 179.844 MHz

Ap(Transducer)=-5.13807 dB

Q1= 19.8367        Q2= 19.8368         Qu= 120

R1= 12263.2 6      R2= 5653.23 6

N1= 7.86234        N2= 4.82723

L= 91.9257 nH     Ct= 8.51949 pF

C1= 8.01949 pF

C12= .5 pF

C2= 7.50881 pF

TUNED AMPLIFIERS         05/23/86

Transistor:

2N4957 10V 2.0mA  180 MHz

Y11= +2.585E-03 +6.348E-03j

Y21= +5.396E-02 -2.084E-02j

Y12= -7.264E-08 -4.480E-04j

Y22= +1.397E-04 +1.296E-03j

Mia= 10            Moa= 20               ML= 1

K= 10.0663         S= .270341

Yia= +5.769E-03 +1.459E-02j

Yoa= +4.682E-04 +2.146E-03j             GL= .02             BL= 0

N= 1              Am= 0 dB

F0= 180 MHz        BW= 25 MHz          Fc= 179.565 MHz

Ap= 20.4109 dB

QL= 7.18262                            Qu= 120

R1= 2682.74 6

N1= 1.6112         N2= 5.6419

L= 98.1986 nH     Ct= 8 pF

C1= 7.26726 pF

#################################################################################