YOU BUILD-IT SURGE PROTECTORS

FOR 117 VOLT HOUSE CURRENT AND YOUR MODEM

==============

Surge protectors are not really that mysterious and are easily constructed at

home.  There are many ways to protect a line from power surges, but the style

that I'll deal with here is the quickest acting form available (also the least

expensive).  This form won't protect your system, however, from direct lightning

strikes, but can be used in conjunction with the heavier (slower) type that is

capable of handling that type of load (i.e., a direct lightning strike to your

house line).

Metal-Oxide Varistors (MOV for short) can absorb the excess (or high surge)

voltages above the starting point designed for the particular MOV (130v for most

of our applications) up to about 50,000 volts (again depending upon design.

This information is available from the manufacturers' spec. sheets).

A MOV is a variable resistor that does nothing until line voltage (where it's

attached) exceeds it's specified threshold.  At this time, the MOV then absorbs

any voltage greater than the starting point for that particular MOV (the voltage

differences are detected between the two lines on your system).  Enough of this

quasi-technical stuff.  I'll just get into how to build your own surge

protectors.

There are several designs for using MOV's.  The one that works best will be

described here.  This design allows for total benefit of the MOV's.  For each

surge protector you wish to construct, you'll need three (3) MOV's.  You'll also

need some type of multiple outlet device that has enough room inside of it to

allow installation of these three MOV's.  And you'll also need to be able to

solder these electronic devices (it's really not that difficult).

Each MOV has two (2) leads extending from it and are soldered to two of the

three AC power lines inside your multiple outlet (see diagram).

O              __________________________

/ \            [                          ]

/   \           [    O        =    MOV     ]

|/  |  \|         [                          ]

|   |   |         [ / or \      =    leads   ]

| O |   |         [                          ]

|/ \|   |         [    |        =    AC lines]

|   | O |         [__________________________]

|   |/ \|

HOT --------------> |   |   |

NEUTRAL --------------> |   |

GROUND -------------------> |

I should explain the reason for using three (3) MOV's.  A MOV detects voltage

changes between the two lines (hot & neutral).  So, if you only use one MOV, and

both lines get zapped with the same surge of equal voltage, there is no differ-

ence detected between the lines (therefore, IT DON'T WORK!!!).  So, if you use

three (3) MOV's, as shown in the diagram, the difference will be detected.

After soldering, check your work for shorts, reassemble the multiple outlet,

and check AGAIN for shorts.

Before leaving you may be wondering, "How much each MOV will set you back?".

These devices are very inexpensive, generally anywhere from $ 1.50 to $ 5.00 ea.

I would also like to say that due to my inability to control your parts purchase

AND your ability to solder components, I assume NO LIABILITY for the information

contained in this article.  I do personally use the construction as described,

but non-the-less!

Chemical Kidd

Houston, Tx.

*****************************************************************************

One for the Phone

*****************************************************************************

To build a protector for a phone line is basically the same as building one for

your AC line.  The biggest difference is the maximum voltage the MOV should

begin to activate,. i.e., the AC line voltage is about 120 VAC & the MOV swings

in at 130 VAC.  For the phone lines the MOV should swing in about 7-8 VDC.

The diagram for the protection of all 4 wires entering the modem is as follows:

O

O   / \

/ \ /   \               ___________________________

NO connection here ----->  /   /     \             [                           ]

between the two MOV's     (   ( \     |            [    O        =    MOV      ]

(see below)               |   |  \|   |            [                           ]

|   |   |   |            [ / or \      =  MOV leads  ]

| O |   | O |            [                           ]

|/ \|   |/ \|            [    |        =  PHONE lines]

|   | O |   |            [___________________________]

|   |/ \|   |

RED --------------> |   |   |   |

GREEN ----------------> |   |   |

YELLOW -------------------> |   |

BLACK ------------------------> |

_O_

/   \

/     \

(       )

|   |   |   |

to further clarify --->  |-O-|-O-|-O-|

the connections          |   |   |   |

(       )

\     /

\_ _/

O

Judging from the size of the $19.95 phone surge protectors, the diagram being

used is as follows:

___________________________

[                           ]

[    O        =    MOV      ]

|   |                    [                           ]

|   |                    [ / or \      =  MOV leads  ]

| O |                    [                           ]

|/ \|                    [    |        =  PHONE lines]

|   |                    [___________________________]

|   |

RED --------------> |   |

GREEN ----------------> |

As stated in the previous section, this last diagram works fine of only one (1)

line gets the surge.  But, if both lines get zapped, you can forget it ever

working.  Red and Green wires are your carrier lines but, with today's new rules

on do-it-yourself installations, double-check to be sure.

The case for using five (5) MOV's runs somewhere in the neighborhood of $5-7.

The cases are available from various hardware sources & electronic supply firms.

AND, once again due to my inability to control the construction of, choice of

materials,....etc. , etc., etc., I can assume NO responsibility for the previous

information nor its use.

Thank you,

Chemical Kidd

Houston, Tx

#################################################