----- ELNEC Example Antenna File Notes -----

1 February 1991

Several example antenna description files are included on your

ELNEC disk.  Following are some notes about each one.  All files

have the extension ".EN" added to the name shown here.

Rather than having the examples all show ELNEC in its best light,

some were chosen specifically to illustrate weaknesses or

limitations you need to be aware of when using ELNEC or any

MININEC-based program.

DEMO USERS NOTE: The antennas marked '*' are too complex for the

DEMO program due to its 15-pulse limit, so they're not on the DEMO

disks.  Also, some of the other antennas differ slightly from the

descriptions given here because the total number of segments was

reduced.

* 15MQUAD

The dimensions for this 15 meter quad, modeled in free space, come

from the ARRL ANTENNA BOOK, 15th ed., p. 12-2.  The design is

attributed to W7ZQ.  As it does on most quads, ELNEC (and MININEC)

reports a poor front/back ratio -- only 2 dB for this quad.  This

is a result of MININEC's "cutting corners" where wires are

connected.  A much more accurate analysis is obtained by tapering

the segment length using ELNEC's automated segment tapering

capability (see the manual for information).  The result of

tapering is file 15MQUADT, described next.  If you delete the

parasitic loop and look at the pattern of the driven element by

itself, you'll find that very few segments are necessary to obtain

an accurate pattern.

* 15MQUADT

If you take the 15 meter quad, above, and taper the segments using

ELNEC's automated segment tapering with default values, the result

is this model.  The indicated front/back has improved to 8.3 dB

from 2 dB and the forward gain has increased about 0.8 dB.  I

believe that this is an accurate representation of the operation of

this quad.  You probably can get sufficiently accurate results with

fewer total segments by experimenting with the maximum and minimum

segment length when doing the tapering.  Note that very many

segments per side are necessary to approach the accuracy of the

results achievable by using the segment tapering technique.

4SQUARE

This popular phased array was invented by Dana Atchley, W1CF.  It

has several desirable properties.  Because of its symmetry, it's

easy to switch in four directions.  The forward lobe is broad

enough that four-direction switching gives good coverage to all

directions.  Good rejection of signals occurs over a broad region

to the rear.  The small rear nulls can be eliminated and the

forward gain increased slightly by increasing the element spacing.

However, this may make the physical spacing too far to conveniently

use some feed methods.  Feeding of the four-square array is covered

in detail in Chapter 8 of the ARRL ANTENNA BOOK, 15th Edition.  One

interesting feature of this array is that one element has a

negative feedpoint resistance if loss is low enough.  This is a

valid result; the element is absorbing power from the others by

mutual coupling and feeding power back INTO the feed system.  There

is still some lingering belief that the fields from the elements of

an array are proportional to the powers delivered to them.  Element

1 of this array has a field equal to that from the other elements,

and it's FURNISHING power, showing the error in this belief.

Observing the currents in the elements reveals the truth: The

element CURRENTS determine the field strength, and they're equal in

all four elements.

CARDIOID

A popular phased array, the 90-degree phased, quarter-wavelength

spaced, two-element array is effective and forgiving, and gives an

honest 3 dB gain over a single element.  It's modeled here over

perfect ground to point out the small reverse lobe which doesn't

usually show up in the textbooks.  This is due to the change in

current distribution on the elements from mutual coupling effects.

See July 1990 QST, page 39, for more information on this phenomenon

and its consequences.  For information on how to design a feed

system for this array see the ARRL ANTENNA BOOK, 15th Ed., Chapter

8, and "The Simplest Phased Array Feed System -- That Works" in the

second ARRL ANTENNA COMPENDIUM.

DIPOLE1

This is a plain dipole in free space, about the simplest antenna

you're likely to model.  The frequency is 299.7+ MHz (selected by

entering '0' for the frequency), at which a wavelength is one

meter.  The antenna dimensions, in meters, are therefore also the

dimensions in wavelengths.  Note that this antenna, exactly a half-

wavelength long, is greater than a resonant length, as indicated by

the positive feedpoint reactance (seen by selecting 'SD' in the

Main Menu).  A resonant "half-wave" antenna is somewhat shorter

than 0.5 wavelength, the amount depending on its diameter.

FDSP

A personal favorite, the "Field Day Special" has been built on

several bands and accounts well for itself from the home QTH as

well as on Field Day.  It was described in June, 1984 QST.  The

elements are folded dipoles made from twinlead, connected by a

half-twisted twinlead "phasing line".  Folded dipoles are difficult

to model with ELNEC, so the elements are modeled as ordinary

dipoles with a diameter equivalent to the effective diameter of the

two-conductor twinlead.  This is valid since the radiation

properties of ordinary and folded dipoles are identical -- only the

feedpoint impedance is affected by the "folding" process.  The

program used to design the original antenna wasn't entirely

accurate so the element currents reported in QST weren't quite

correct.  The source currents in the model are the currents

actually measured on the elements of a Field Day Special built to

the dimensions shown in the QST article.  This antenna has a

respectable gain at low angles, a good f/b ratio, and a broad

forward lobe.  It's also quite forgiving.  Constructed from

twinlead, the input SWR is near unity.

* N4PCLOOP

This multiband horizontal loop antenna was created by Paul Carr,

N4PC, and described in December, 1990 CQ Magazine.  A unique

feature is that it's driven at opposite corners by out-of-phase

signals as in the W8JK antenna.  This results in an overhead null

on all bands.  If you look at the Wires Menu description, you'll

see two sources shown IN phase.  This is necessary due to a

combination of the way MININEC assigns pulses to wires and the

direction current is assumed to flow in the wires.  When multiple

sources are placed in connected wires as done here, you must check

the resulting currents to make sure they're flowing in the

directions you thought.  If not, you must reverse one or more

sources.  Note also the relatively large number of segments for

each wire.  On 20 meters, the 51-foot sides are approaching a full

wavelength long, so an appropriately large number of segments must

be used.  See the manual for more information on multiband antennas

and on using multiple sources in an antenna.

NBSYAGI

This is a good test of program operation.  The National Bureau of

Standards carefully built and measured several Yagi antennas.  This

is a 50 MHz one, with dimensions from the ARRL ANTENNA BOOK, p. 18-

7.

VERT1

A "plain-vanilla" resonant vertical over average ground.

VERT2

Place VERT1 at the top of a 45-degree sloping hillside, and this is

what you get.  Note the large improvement in low-angle radiation.

This is an example of use of the multiple ground media capability

of ELNEC.  A linear boundary is used in this case.

W8JK

Originally designed by John Kraus, W8JK in about 1940, this antenna

has some interesting properties.  It's characterized by two

closely-spaced elements driven out of phase.  Although the fields

from the elements don't fully reinforce in any direction, gain is

nonetheless achieved because of lowering of the radiation

resistance due to mutual coupling.  And lower it is -- note the

feedpoint impedance of only 3.73 - j24.48 ohms -- compare this to a

single element.  The lower resistance results in heavier current,

hence greater field strength, for a given power input.  The

difficulty is that system losses can quickly eat up the gain.

Making this antenna from #12 copper wire (try it -- and include

wire loss) drops the gain about 0.65 dB, not too bad.  But great

attention must be paid to losses in matching networks.  And losses

rapidly increase in significance as the spacing is made closer than

the 0.1 wavelength of the example.  When mounted low (0.25

wavelength for the example), W8JK-type antennas have a lower

radiation angle than many other horizontal antennas due to the

inherent lack of high-angle radiation.  As an interesting exercise,

save the pattern for later comparison.  Then delete the second

source, making the antenna into a Yagi.  Note the increased gain.

Even though the takeoff angle is higher, the Yagi gain is as good

or better even at lower angles.  In addition, the feedpoint

impedance has increased to a much more manageable value.  On the

other hand, the W8JK will retain its performance over a wide

frequency range, while the Yagi won't.