WEFAX.EXE - Weather facsimile receiver (Version 1.0)

This weather facsimile program was written in order to provide free software

to those who wish to be capable of recieving pictures without the price of the

professional wefax programs.

** This program is freeware!!! ** and may not be distributed for any price, but

should be distributed for free.

Thisa program does require a special device that is capable of recieving

weather facsimile images.

This program cannot be decompiled or anything like that either.

It was written by Jason Johnson and Todd Hansen, KD6YPS, for use by those wish-

ing to monitor wefax transmissions on HF frequencies.

If any portion of this documentation or the acompanying programs are

changed in any way, then it is illegal to distribute the software.

A user of this software should have knowledge in computers, TNCs, radio

recievers and WEFAX transmissions.

File Listing

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

WEFAX.DOC - This file

WEFAX.EXE - The program

VIEWFAX.EXE   - A quick viewer for saved wefax images. (This is not

required and has many errors) [Usage: VIEWFAX <FileName> <CR>]

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

*** Special notice ***

To use the print fax option, the TSR (terminate and stay resident) program

called GRAPHICS.COM (included with most versions of MS-DOS) must be loaded.

COMMAND LINE PARAMETERS:

~~~~~~~~~~~~~~~~~~~~~~~~

The program accepts command line parameters as follows:

Usage:

From the command line: WEFAX [PORT#] [BAUDRATE]

Where: [BAUDRATE] = Speed to access the comport at.

[PORT#]    = # of comport to use.  (i.e. 1 for COM1, 2 for COM2,

etc.)

[/? or /HELP] = Show command line help/usage.

Note: If a baudrate or comport number are not given in the command line,

the program defaults to COM2 at 2400 Baud.

Also: Set data sample rate of TNC to 1280 bit samples/sec.

On the Kantronics KPC-3, type the following at the "cmd:" prompt:

WEFAX 1280 [CR]

------------------------------------------------------------------------------

This program was written for non-commercial distribution as freeware.

Since we made this for free, any problems encountered should be reported, but

please note that we are busy and may not be available to help you.

Send any comments, questions, descriptions of problems, or suggestions to:

Todd Hansen, KD6YPS

shansen@cello.gina.calstate.edu

(Don't use this address after 1/96)

Program Usage

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The program starts out in terminal mode with your TNC (Terminal

node controller). After you set your TNC to send you wefax data then

press F1. This will begin to display the graphic. After you have gotten

the image you want on the screen, then you can press any key. This will

bring you to a self explanitory menu. This allows you to save the image,

load another image and view it, print the current image (using

GRAPHICS.COM) and to exit back to terminal. When you are in the terminal

mode, you can press esc, and it will exit you from the program. This is

not a very good terminal program, and should only be used in order to

get your TNC setup to recieve a WEFAX image.

A couple of suggestions for receiving WEFAX images are to read

you owners manual of your TNC and reciever. The receiver must be able to

receive USB (Upper Side Band) transmissions. If your computer makes to

much interferance to your reciever, then turn your computer off, and

record the sound into a tape recorder and play it back into your TNC.

Some possible WEFAX frequencies are 6.630 kHz, 8.080 kHz, 8.646 kHz,

9.158 kHz, 4.346 kHz, 17.151 kHz, 12.730 kHz and 10.185 kHz. Also

remember to use a 1280 bits per second sample rate.

Errors found so far

~~~~~~~~~~~~~~~~~~~~~~~~~~~

After the screen scrolls up in terminal mode, you seem to lose the first

character in every line. This is solved by pressing F1 and then a key

and then pressing D and it will return you back to where you were.

One minor limitation, that can be by passed many ways, is that the

program uses ^C to take the TNC out of WEFAX mode.