G8BPQ Host Mode

From Version 4.00, a new application interface to the switch is provided.

This replaces the existing interfaces (TNC2, PK Host and KISS). New

software should if possible use the new interface - interface routines

supporting the old modes, and WA8DED Host mode, will be provided as TSRs

to support existing application software.

The interface is normally called using INT 7Fh. From version 4.03 the interrupt

number may be changed - The value is specified in BPQCFG.TXT as INTERRUPT=nnn,

where nnn is in decimal. The function required is in AH, and the stream in AL.

The system supports 64 streams, or sessions, numbered from 1 - 64.

Changes in Version 4.05.

1.	Additional option on funtion 6 - allows connects using BBS Call, even

if APPLMASK is not set (say to prevent connects).

2.	Additional info returned by function 8.

3.	Format of frame returned by function 11 added to documentation.

4.	Function 15 added. Simplifies access to timing info.

5. 	Broadcast option added. If function 2 is called with AL=0, the

data will be sent as a UI frame to all ports which have an UNPROTO

address set.

6. 	Function 12 added to update Beacon text.

Changes in Version 4.07b

1. 	Function 0 added to check if loaded, and get version number. This

is because the old method of checking (looking at the bytes before

the interrupt routine) isn't compatible with some versions of

DesqView, or the Windows version of the switch (coming soon!)

2.	Function 12 extended to allow application to request a NODES

broadcast.

3.	Function 13 added to allow application to request a free stream and

to free a stream after use. This will simplify configuration when

several applications are used.

Note that in all cases where a string is returned, ES:DI must point to

a buffer supplied by the user, and the switch will copy the data to

this buffer. The code does NOT return pointers to its own data.

AH = 0	Get node/switch version number and description.  On return

AH='B',AL='P',BH='Q',BL=' '

DH = major version number and DL = minor version number

AH = 1	Set application mask to value in DL.

Set application flags to value in CL.

If top bit of CL is set, monitored frames will be available.

Use function 11 to receive them.

AH = 2	Send frame in ES:SI (length CX)

AH = 3	Receive frame into buffer at ES:DI, length of frame returned

in CX.  BX returns the number of outstanding frames still to

be received (ie. after this one) or zero if no more frames

(ie. this is last one).

Note that CX is NOT an input param - the supplied buffer

must be big enough to receive a full AX.25 frame, including

up to 8 digi callsigns. 340 should be enough.

AH = 4	Get stream status.  Returns:

CX = 0 if stream disconnected or CX = 1 if stream connected

DX = 0 if no change of state, or DX = 1 if

the connected/disconnected state has changed.

AH = 5	ACK stream status change. Must be called to allow another

status change to be reported.

AH = 6	Session control.

CX = 0 Connect to Node. Session uses BBS callsign if bottom

bit of DL=1, otherwise uses Node Call .

CX = 1 Connect to Node. Session uses BBS Call if APPLMASK=1,

otherwise Node Call.

CX = 2 Disconnect

CX = 3 Return user to node

AH = 7	Get buffer counts for stream.  Returns:

BX = number of frames queued for receive

CX = number of un-acked frames to be sent

DX = number of buffers left in node

AH = 8	Port control/information.  Called with a stream number

in AL returns:

ES:DI = CALLSIGN (10 bytes space padded)

The following are now supported (4.05 onwards)

AL = Radio port on which channel is connected (if level 2)

AH = Session Type (If Connected). Session type bits are:

L2LINK	EQU	1

SESSION	EQU	10B

UPLINK	EQU	100B

DOWNLINK	EQU	1000B

BPQHOST	EQU	100000B

BX = L2 paclen for the session

CX = Maxframe. Taken from PORT maxframe if L2 session,

Switch maxframe if L4.

DX = L4 window size (if L4 circuit, or zero)

AH = 10	RAW (KISS) transmit frame.  Data pointed to by ES:SI, of

length CX, is transmitted as a HDLC frame on the radio

port (not stream) in AL.

AH = 11 Get Trace (RAW Data) Frame into ES:DI, Length to CX. Data

is as received off air (ie with calls in AX.25 format, etc)

See note on CX under function 3.

The returned buffer includes a 5 byte header:

Bytes 0-1  - Internal control info (ignore)

Byte  2    - Port number. Top bit set if transmitted frame.

Bytes 3-4  - Frame length (including this header)

AH = 12 Update Switch Info. This will be used for a number of

operations. At the moment the only ones implemented are to

update the Beacon Text field, and initiate a NODES broadcast.

DX = Function Number.

DX = 1  Update Beacon Text.

ES:SI Data to send in Beacons

CX    Length of Data

DX = 2  Initiate NODES broadcast

AH = 13 Allocate/deallocate stream

If AL=0, return first free stream in AL. If AL=255 on

return, there are no free streams

If AL>0, CL=1, Allocate stream. If aleady allocated,

return CX nonzero, else allocate, and return CX=0

If AL>0, CL=2, Release stream

Note that for compatiblity with old software, certain other

functions also set the allocated bit (Functions 1,2,6). So

an application has a reasonable chance of getting an unused

stream, even if some applications dont use the allocate function.

AH = 15 Get time marker in AX. Value is number of timer interrupts

since switch was loaded, modulo 65536. Timer interrupts

normally occur at approx 18.2 per second.

PACLEN.

Frames are transmitted as received from the application, so the application

is responsible for the transmitted PACLEN. Function 8 will return the

recommended value for the current session. I think the value is always OK,

but I suggest you treat a value of zero as 'undefined', and use your own

default. The value may change during a session, if for instance, you connect

to the switch, and then out on a port with a different PACLEN. Also at

some time in the future I hope to adjust PACLEN to suit conditions on the

channel. So I recommend that the value is re-read from time to time during

a session.

The following sample of code shows how to find the Interrupt Number to use

to call the switch, and check that the switch is loaded.

NOTBPQERROR	DB	'Switch not found$'

VERSERROR	DB	'Version not compatible with Node Software$'

BADCONFIGMSG	DB	'Configuration file read error',0DH,0AH,'$'

CONFIGFILENAME	DB	'BPQCFG.BIN',0

NOCONFIGMSG	DB	'Configuration file BPQCFG.BIN not found',0DH,0AH,'$'

;

;	BPQCFG FIRST 128 BYTES

;

CONFIGTABLE	LABEL	BYTE

;

;	CONFIGURATION DATA STRUCTURE

;

;	DEFINES LAYOUT OF CONFIG RECORD PRODUCED BY CONFIGURATION PROG

;

;	LAYOUT MUST MATCH THAT IN CONFIG.C SOURCE

;

C_NODECALL	DB	10 DUP (0)	 ; OFFSET = 0

C_NODEALIAS	DB	10 DUP (0)	 ; OFFSET = 10

C_BBSCALL	DB	10 DUP (0)	 ; OFFSET = 20

C_BBSALIAS	DB	10 DUP (0)	 ; OFFSET = 30

;

C_OBSINIT	DW	0		 ; OFFSET = 40

C_OBSMIN	DW	0		 ; OFFSET = 42

C_NODESINTERVAL	DW	0		 ; OFFSET = 44

C_L3TIMETOLIVE	DW	0		 ; OFFSET = 46

C_L4RETRIES	DW	0		 ; OFFSET = 48

C_L4TIMEOUT	DW	0		 ; OFFSET = 50

C_BUFFERS	DW	0		 ; OFFSET = 52

C_PACLEN	DW	0		 ; OFFSET = 54

C_TRANSDELAY	DW	0		 ; OFFSET = 56

C_T3		DW	0		 ; OFFSET = 58

DW	0		 ; OFFSET = 60

DW	0		 ; OFFSET = 62

C_IDLETIME	DW	0		 ; OFFSET = 64

C_EMSFLAG	DB	0		 ; OFFSET = 66

DB	0

C_BBS		DB	0		 ; OFFSET = 68

C_NODE		DB	0		 ; OFFSET = 69

C_HOSTINTERRUPT	DB	0		 ; OFFSET = 70

C_DESQVIEW	DB	0		 ; OFFSET = 71

C_MAXLINKS	DW	0		 ; OFFSET = 72

C_MAXDESTS	DW	0

C_MAXNEIGHBOURS	DW	0

C_MAXCIRCUITS	DW	0		; 78

C_TNCPORTLISTO	DB	16 DUP (0)	 ; OFFSET = 80

C_IDINTERVAL	DW	0		; 96

C_XXXXXXXX	DW	0		; 98    ; SPARE (WAS DIGIFLAG)

C_MINQUAL	DW	0		; 100

C_HIDENODES	DB	0		; 102

C_L4DELAY	DW	0		; 103

C_L4WINDOW	DW	0		; 105

C_BTINTERVAL	DW	0		; 106

X_UNPROTO	DB	9 DUP (0)	; 108	; NOW SPARE

C_BBSQUAL	DW	0		; 117

DB	(CONFIGTABLE+128-$) DUP (0)

CONFIGHANDLE	DW	0

NODE:

INT	7FH

RET

INIT:

;

;	GET NODE INTERRUPT NUMBER FROM CONFIG FILE

;

MOV	DX,OFFSET CONFIGFILENAME

MOV	AH,3DH

MOV	AL,0			; READ ONLY

INT	21H			; OPEN IT

JC	NOCONFIGFILE

MOV	CONFIGHANDLE,AX

MOV	BX,AX

MOV	DX,OFFSET CONFIGTABLE

MOV	CX,128

MOV	AH,3FH

INT	21H			; READ

CMP	AX,CX

JNE	SHORTCONFIG

JMP SHORT PROCESSCONFIG

NOCONFIGFILE:

MOV	DX,OFFSET NOCONFIGMSG

JMP SHORT CONFIGERR

SHORTCONFIG:

MOV	DX,OFFSET BADCONFIGMSG

CONFIGERR:

MOV	AH,9

INT	21H

MOV	AX,4C01H

INT	21H			; EXIT

PROCESSCONFIG:

MOV	BX,CONFIGHANDLE

MOV	AH,3EH

INT	21H			; CLOSE IT

MOV	AL,C_HOSTINTERRUPT	; INTERRUPT

MOV	BYTE PTR NODE+1,AL	; PATCH NODE CALL INSTRUCTION

;

;	GET NODE VERSION

;

PUSH	DS

MOV	AH,0

ADD	AX,AX			; VECTORS ARE 4 BYTES LONG

ADD	AX,AX

MOV	SI,AX

XOR	AX,AX

MOV	DS,AX

;

;	DS:SI POINTS OUR VECTOR

;

;	FIRST SEE IF VECTOR POINTS TO CODE - CALLING AN UNINITIALISED

;	INTERRUPT WILL CRASH THE SYSTEM

;

CMP	WORD PTR DS:2[SI],0	; INTERRUPT ROUTINE SEGMENT ADDRT

POP	DS

JNE	SWITCHOK		; ROUTINE PRESENT

NOSWITCH:

MOV	DX,OFFSET NOTBPQERROR

MOV	AH,9

INT	21H

JMP SHORT EXIT

SWITCHOK:

MOV	AH,0

CALL	NODE			; SET INSTALLED FLAGS AND VERSION

CMP	AX,'PB'			; AX,BX SHOULD BE 'BPQ '

JNE	NOSWITCH

CMP	BX,' Q'

JNE	NOSWITCH

CMP	DH,MAJORVERSION

JNE	BADVERSION

CMP	DL,MINORVERSION

JE	VERSIONOK

BADVERSION:

POP	DS

MOV	DX,OFFSET VERSERROR

MOV	AH,9

INT	21H

EXIT:

MOV	AX,4C00H

INT	21H

VERSIONOK:

POP	DS

;

..........

The release disk also contains a demonstration terminal program, TERM4,

written by G8IMB (in PASCAL) to demonstate the use of the new interface from

a high level language. Note, however, that it was written for version 4.02,

and therefore only works with the Host Mode interrupt configured at 7Fh

John Wiseman, G8BPQ @ GB7BAD

20/12/91

Revised 5/9/92 and 26/3/94