Hostmode information on Kantronic TNC

------------------------------------------------------------------

: This file describes the HostMode on the  Kantronics TNC.  This :

: file is issued by LA6GHA, Bob, but unfortunately the author is :

: unknown. Many thanks to SM7OYP for his help.                   :

:                                                73's de LA6GHA  :

------------------------------------------------------------------

KANTRONICS HOST MODE OPERATION

--------------------------------

In order to operate in the Host mode with Kantronics TNC, you must first

set the Intface command to Host.  After this accomplished,  it  will  be

necessary to perform a soft reset in order to enter the Host mode.  This

may be accomplished by typing reset at the cmd: prompt.  If you want the

TNC to always operate in the Hostmode, be sure to give the command PERM.

(Unless you have battery-backup).

You will also need to set the abaud command to the appropriate baud rate

for your terminal. If the abaud command is not set, the TNC will run its

normal autobaud routine,  looking for an asterisk (*) from the keyboard.

When the asterisk is  entered,  the  TNC  will  then  immediately  enter

Hostmode. While operating Hostmode,  your program must use hardware flow

control  (RTS/CTS).  Software flow control is not possible in Host mode.

In the KAM, Host mode only applies to the packet mode (v 3.05 and lower)

COMMUNICATION FORMAT - HOST COMPUTER TO TNC

Communication from the Host to the TNC must occur in blocks.  The  block

of data is delimited with a FEND character  ($C0)  at the beginning  and

end.  If the FEND character appears within the block as valid data,  the

Host must replace this character with a special sequence,  consisting of

a FESC  ($DB)  by a  TFEND  ($DC).  One other special  sequence  may  be

required in the event a  FESC  ($DB)  characters is required in the data

field.  This is accomplished by the special sequence  of  a  FESC  ($DB)

followed by a TFESC ($DD).  These special sequences are the same used in

the kiss code, as implemented by Phil Karn, KA9Q.

After the opening FEND,  the next character is the command byte and will

indicate the type off command being given to the  TNC.  The  permissible

characters in the command byte are C, D or Q.  For the KAM  only,  there

are some other combinations used for non-packet HF mode operation. These

will be covered later.

A "C" command byte indicates a command that the TNC will interpret as if

it were in the command mode.  If the command mode byte is a "D", the TNC

will consider the data as data to be transmitted on the  specified  port

and stream.  The letter "Q" in the command byte will cause  the  TNC  to

exit the Host mode and return to terminal mode.

The next byte is the port byte.  This byte must be used with every block

of  type  "D"  to  signify  which  port  (1 or 2)  is  to  be  used  for

transmission of the data.  Type  "C"  blocks must always specify this as

either 1 or 2, but this is only used on those commands that are specific

to a port. This would include the connect and disconnect commands.  When

using KAM,  the VHF port is selected by a port byte of  "1"  and the  HF

port is "2". Single port units use port byte "1".

The fourth byte is the stream byte.  This byte determines  which  stream

(A-Z) the TNC will use for the data.  If the stream byte is 0 for a data

packet  (command byte D),  the data will be sent out unproto.  (See  the

section on the KAM HF modes for more information).  For commands that do

not not involve a specific port or stream, the port and stream bytes are

ignored, but must be specified.  In the cases, you should address port 1

stream A.

After these four header bytes,  the structure of the block for a command

is exactly the same as if you were entering the  command  from  terminal

mode of the TNC.  If entering data to be transmitted,  simply place  the

data in the following bytes.  Note that commands do not need a  carriage

return included in the data portion of the packet.

After the data or command,  terminate the information from the Host with

a FEND ($C0) character.

KAM HF MODES

When operating non-packet modes  (i.e., RTTY, ASCII, AMTOR, or CW)  with

the KAM, a few additional Host to TNC frames are required.  First select

the mode with the standard command structure as described  above  (e.g.,

FEND C2ARTTY 75 FEND).  Data that you  wish  to  transmit to  the  other

station should have command byte "D", port byte "2", and stream byte "0"

Special Host frames for these modes are:

FEND T FEND          Enter transmit mode

FEND E FEND          Enter receive mode when buffer empty

FEND R FEND          Enter receive mode immediately

FEND X FEND          Return HF port to packet mode

FEND CTRL-X FEND     Clear the transmit buffer in KAM HF

TNC TO HOST COMPUTER

Communication from the  TNC  to the Host also occurs in blocks which are

delimited at beginning and end with FEND characters ($C0).

After the beginning  FEND,  the next character is  the  status  byte.  A

status byte  "C"  is a response to a command  from  the  Host  with  the

command byte "C".  A status byte of  "D"  indicates that  the  data  was

received on a connected stream  (see the section  below  on  the  KAM HF

operation).  "M" in the status byte means that the data in this block is

the result of the monitor commands.  A status byte of  "S"  is a message

caused by a  change  in  the  link  state.  Such  messages  include  the

***DISCONNECTED, and FRMR sent.  A special "S" block of data consists of

two FEND characters, the characters S00 and another FEND character. This

indicates that the TNC has performed a  soft  reset,  and  all  existing

connections (if any) are not longer valid. This is equivalent to the TNC

having just been turned on.

A data block with the  status  byte  ***connect request.  A  block  with

status byte  "T"  is the result of the trace command.  Port  and  stream

bytes  (defined below)  are valid for "D" and "S" blocks,  but only  the

port byte is valid for "T", "M",and "R" blocks.  The port  byte  follows

the  status  byte,  and  will  contain  the  port  number  the  specific

information is from.  This will be a  "1"  if the  TNC  is  single  port

operation,  or a "1" or "2" if in dual port operation.  When  using  the

KAM,  the VHF port is indicated by a port byte of "1".  The stream  byte

follows the port byte.  The stream byte will be "A"-"Z" for data on  the

connected streams.

Data being sent to the  Host  which is not connected data will have  the

stream byte set to "0".  If the TNC returns a "C" status block  with  no

data, this indicates that the command was accepted.  This will occur  on

connect and disconnect  commands.  A "T"  block  from  the  Host  (trace

information) is raw data, and not a hex dump of the received packet. The

kiss transparency  (FESC, FEND, and TFESC)  described  above  is  always

applied to all blocks.

HF MODE OPERATION

When using a non-packet mode on the HF port of the KAM, received HF data

will be sent to the Host with status "D", port byte "2", and stream byte

"0".  Each received character is sent from the  KAM  to the  Host  in  a

separate block.

When operating AMTOR mode A,  another KAM to  Host  block  is  possible,

indicating the current status of your station  (IRS or ISS).  The  block

has status byte of "I", port byte "2", and the stream byte indicates IRS

("0") or ISS ("1").  This block will be sent whenever there is a change.

When operating in the AMTOR mode A (ARQ) the KAM will send a  "S"  block

containing information pertinent to the link.  For  instance,  when  the

link is established, the KAM sends a "S" frame with the message <LINKED>

or <LINKED TO xxxxxx>.  "S" frames  are  also  sent  indicating  a  link

failure,  or anytime the KAM returns to amtor standby.  This  can  occur

when the other station breaks the link, or if the link fails due to poor

propagation.

When receiving NAVTEX, the KAM does not send any information to the Host

until it has received a proper  NAVTEX  header  for a  message  that  is

vaild to be printed  (see the NAVTEX section of the operations  manual).

The header will be sent to the Host in a single block. After the closing

NNNN of the message if the number of errors received exceeds the  NAVERR

setting,  an error message will  be  sent  from  the  KAM  to  the  Host

indicating ***too many errors*** and the message number.  This  will  be

sent in a  "D20"  frame and not as a  status  frame.  This  allows  this

message, to be stored in the file, if you are capturing the  HF  channel

to disk.  This message will also be sent if the link  fails  before  the

closing of the current message.

<EOT - End Of Text>

/EX