The KISS TNC: A simple Host-to-TNC communications

protocol

Mike Chepponis, K3MC

Phil Karn, KA9Q

ABSTRACT

The KISS[1] TNC provides direct computer  to  TNC

communication  using a simple protocol described here.

Many TNCs now implement it, including the  TAPR  TNC-1

and TNC-2 (and their clones), the venerable VADCG TNC,

the AEA PK-232/PK-87 and all TNCs  in  the  Kantronics

line.   KISS has quickly become the protocol of choice

for TCP/IP operation and multi-connect BBS software.

1.  Introduction

Standard TNC software was written with human users in mind;

unfortunately,  commands and responses well suited for human use

are ill-adapted for host computer use, and vice versa.  This  is

especially  true  for multi-user servers such as bulletin boards

which must  multiplex  data  from  several  network  connections

across  a  single  host/TNC  link.  In addition, experimentation

with new link level protocols is greatly hampered because  there

may  very well be no way at all to generate or receive frames in

the desired format without reprogramming the TNC.

The KISS TNC solves these problems by eliminating  as  much

as possible from the TNC software, giving the attached host com-

plete control over and access to the contents of the HDLC frames

transmitted  and  received over the air.  This is central to the

KISS philosophy: the host software should have control over  all

TNC functions at the lowest possible level.

The AX.25 protocol is removed entirely from the TNC, as are

all  command interpreters and the like.  The TNC simply converts

between synchronous HDLC, spoken on  the  full-  or  half-duplex

radio  channel,  and  a  special asynchronous, full duplex frame

"Keep It Simple, Stupid"

July 14, 1990

format spoken on the host/TNC link.  Every frame received on the

HDLC  link  is  passed  intact  to  the  host  once  it has been

translated to the asynchronous  format;  likewise,  asynchronous

frames  from  the host are transmitted on the radio channel once

they have been converted to HDLC format.

Of course, this means that the bulk of  AX.25  (or  another

protocol)  must  now  be implemented on the host system. This is

acceptable, however, considering the greatly increased flexibil-

ity  and reduced overall complexity that comes from allowing the

protocol to reside on the same machine with the applications  to

which it is closely coupled.

It should be stressed that the KISS TNC was  intended  only

as  a  stopgap.   Ideally, host computers would have HDLC inter-

faces of their  own,  making  separate  TNCs  unnecessary.  [15]

Unfortunately,  HDLC  interfaces  are  rare,  although  they are

starting to appear for the  IBM  PC.   The  KISS  TNC  therefore

becomes  the  "next  best thing" to a real HDLC interface, since

the host computer only needs an ordinary asynchronous interface.

2.  Asynchronous Frame Format

The "asynchronous packet protocol" spoken between the  host

and  TNC  is  very simple, since its only function is to delimit

frames. Each frame is both preceded and followed  by  a  special

FEND  (Frame  End) character, analogous to an HDLC flag.  No CRC

or checksum is provided.  In addition,  no  RS-232C  handshaking

signals are employed.

The special characters are:

Abbreviation            Description                    Hex value

FEND                 Frame  End                         C0

FESC                 Frame  Escape                      DB

TFEND                Transposed Frame End               DC

TFESC                Transposed Frame Escape            DD

The reason for both preceding and ending frames with  FENDs

is  to  improve  performance  when  there is noise on the asynch

line.  The FEND at the beginning of a  frame  serves  to  "flush

out"  any  accumulated garbage into a separate frame (which will

be discarded by the upper layer protocol) instead of sticking it

on  the  front of an otherwise good frame.  As with back-to-back

flags in HDLC, two FEND characters in a row should not be inter-

preted as delimiting an empty frame.

3.  Transparency

Frames are sent in 8-bit binary; the asynchronous  link  is

set  to  8 data bits, 1 stop bit, and no parity.  If a FEND ever

appears in the data, it is translated into the two byte sequence

FESC  TFEND  (Frame Escape, Transposed Frame End).  Likewise, if

the FESC character ever appears in the user data, it is replaced

July 14, 1990

with  the two character sequence FESC TFESC (Frame Escape, Tran-

sposed Frame Escape).

As characters arrive at the receiver, they are appended  to

a  buffer  containing the current frame.  Receiving a FEND marks

the end of the current  frame.   Receipt  of  a  FESC  puts  the

receiver  into "escaped mode", causing the receiver to translate

a following TFESC or TFEND back to FESC or  FEND,  respectively,

before adding it to the receive buffer and leaving escaped mode.

Receipt of any character other than  TFESC  or  TFEND  while  in

escaped  mode is an error; no action is taken and frame assembly

continues.  A TFEND or TESC received while not in  escaped  mode

is treated as an ordinary data character.

This procedure may seem somewhat  complicated,  but  it  is

easy  to implement and recovers quickly from errors. In particu-

lar, the FEND character is never sent over the channel except as

an  actual end-of-frame indication. This ensures that any intact

frame (properly delimited by FEND  characters)  will  always  be

received  properly  regardless  of  the  starting  state  of the

receiver or corruption of the preceding frame.

This asynchronous framing protocol is identical  to  "SLIP"

(Serial Line IP), a popular method for sending ARPA IP datagrams

across asynchronous links. It could also form the  basis  of  an

asynchronous  amateur packet radio link protocol that avoids the

complexity of HDLC on slow speed channels.

4.  Control of the KISS TNC

Each asynchronous data frame sent to the TNC  is  converted

back  into "pure" form and queued for transmission as a separate

HDLC frame.  Although removing the human interface and the AX.25

protocol  from the TNC makes most existing TNC commands unneces-

sary (i.e., they  become  host  functions),  the  TNC  is  still

responsible  for keying the transmitter's PTT line and deferring

to other activity on the radio channel. It is  therefore  neces-

sary  to  allow the host to control a few TNC parameters, namely

the transmitter keyup delay, the transmitter  persistence  vari-

ables and any special hardware that a particular TNC may have.

To distinguish between  command  and  data  frames  on  the

host/TNC link, the first byte of each asynchronous frame between

host and TNC is a "type" indicator.  This type indicator byte is

broken into two 4-bit nibbles so that the low-order nibble indi-

cates the command number (given in  the  table  below)  and  the

high-order  nibble indicates the port number for that particular

command.  In systems with only one HDLC port, it is  by  defini-

tion  Port  0.  In multi-port TNCs, the upper 4 bits of the type

indicator byte can specify one of up to sixteen ports.  The fol-

lowing commands are defined in frames to the TNC  (the "Command"

field is in hexadecimal):

July 14, 1990

Command        Function         Comments

0           Data frame       The  rest  of the frame is data to

be sent on the HDLC channel.

1           TXDELAY          The next  byte  is  the  transmitter

keyup  delay  in  10 ms units.

The default start-up value is 50

(i.e., 500 ms).

2           P                The next byte  is  the  persistence

parameter,  p, scaled to the range

0 - 255 with the following

formula:

P = p * 256 - 1

The  default  value  is  P  =  63

(i.e.,  p  =  0.25).

3           SlotTime         The next byte is the slot interval

in 10 ms units.

The default is 10 (i.e., 100ms).

4           TXtail           The next byte is the time to hold

up the TX after the FCS has been

sent, in 10 ms units.  This command

is obsolete, and is included  here

only for  compatibility  with  some

existing  implementations.

5          FullDuplex        The next byte is 0 for half duplex,

nonzero  for full  duplex.

The  default  is  0

(i.e.,  half  duplex).

6          SetHardware       Specific for each TNC.  In the

TNC-1, this command  sets  the

modem speed.  Other implementations

may use this function  for   other

hardware-specific   functions.

FF         Return            Exit KISS and return control to a

higher-level program. This is useful

only when KISS is  incorporated

into  the TNC along with other

applications.

The following types are defined in frames to the host:

Type			Function		Comments

0                 Data frame       Rest of frame is data from

the HDLC channel.

No other types are defined; in particular, there is no pro-

vision for acknowledging data or command frames sent to the TNC.

KISS implementations must ignore any unsupported command  types.

All  KISS implementations must implement commands 0,1,2,3 and 5;

the others are optional.

5.  Buffer and Packet Size Limits

One of the things that makes the KISS  TNC  simple  is  the

deliberate lack of TNC/host flow control. The host computers run

a higher level protocol (typically TCP, but AX.25  in  the  con-

nected  mode  also  qualifies)  that  handles flow control on an

end-to-end basis.  Ideally,  the  TNC  would  always  have  more

buffer  memory  than  the sum of all the flow control windows of

all of the logical connections using it  at  that  moment.  This

would  allow  for the worst case (i.e., all users sending simul-

taneously).  In practice, however, many (if not most) user  con-

nections are idle for long periods of time, so buffer memory may

be safely "overbooked".  When the occasional "bump" occurs,  the

TNC  must  drop  the  packet gracefully, i.e., ignore it without

crashing or losing packets already  queued.   The  higher  level

protocol   is   expected   to   recover  by  "backing  off"  and

July 14, 1990

retransmitting the packet at a later time, just as it does when-

ever  a  packet is lost in the network for any other reason.  As

long as this occurs infrequently, the performance degradation is

slight;  therefore the TNC should provide as much packet buffer-

ing as possible, limited only by available RAM.

Individual packets at  least  1024  bytes  long  should  be

allowed.   As  with  buffer  queues,  it  is recommended that no

artificial limits be placed on packet size.   For  example,  the

K3MC  code  running  on  a  TNC-2  with  32K of RAM can send and

receive 30K byte packets, although  this  is  admittedly  rather

extreme.   Large  packets reduce protocol overhead on good chan-

nels. They are essential for good performance when operating  on

high speed modems such as the new WA4DSY 56 kbps design.

6.  Persistence

The P and SlotTime parameters are used  to  implement  true

p-persistent CSMA.  This works as follows:

Whenever the host queues data  for  transmission,  the  TNC

begins  monitoring  the carrier detect signal from the modem. It

waits indefinitely for this signal  to  go  inactive.  When  the

channel  clears, the TNC generates a random number between 0 and

1.[2] If this number is less than or equal to the parameter p,

the  TNC  keys the transmitter, waits .01 * TXDELAY seconds, and

transmits all queued frames. The TNC then unkeys the transmitter

and  goes  back  to  the  idle  state.   If the random number is

greater than p, the TNC delays  .01  *  SlotTime  seconds  and

repeats the procedure beginning with the sampling of the carrier

detect signal. (If the carrier detect signal has gone active  in

the  meantime,  the  TNC again waits for it to clear before con-

tinuing).  Note that p = 1 means  "transmit  as  soon  as  the

channel clears"; in this case the p-persistence algorithm degen-

erates into the 1-persistent CSMA generally used by conventional

AX.25 TNCs.

p-persistence causes the TNC to wait for an  exponentially-

distributed  random  interval after sensing that the channel has

gone clear before attempting to transmit. With proper tuning  of

the  parameters  p and SlotTime, several stations with traffic

to send are much less likely to collide  with  each  other  when

they  all see the channel go clear.  One transmits first and the

others see it in time to prevent a collision,  and  the  channel

To conform to the literature, here p  takes  on values between 0

to 1. However, fractions are difficult to use in a fixed point

microprocessor so the KISS  TNC actually works with P values that

are rescaled to the range 0 to  255.   To  avoid  confusion,  we

will  use lower-case  p to mean the former (0-1) and upper-case

P whenever we mean the latter (0-255).

July 14, 1990

remains stable under heavy load.   See  references  [1]  through

[13] for details.

We believe that optimum p and SlotTime  values  could  be

computed  automatically.  This could be done by noting the chan-

nel occupancy and the length of the frames on the  channel.   We

are  proceeding with a simulation of the p-persistence algorithm

described here that we  hope  will  allow  us  to  construct  an

automatic algorithm for p and SlotTime selection.

We added p-persistence to the KISS TNC  because  it  was  a

convenient  opportunity to do so.  However, it is not inherently

associated with KISS nor with  new  protocols  such  as  TCP/IP.

Rather,  persistence is a channel access protocol that can yield

dramatic performance improvements regardless of the higher level

protocol  in  use;  we urge it be added to every TNC, whether or

not it supports KISS.

7.  Implementation History

The original idea for a simplified host/TNC protocol is due

to Brian Lloyd, WB6RQN.  Phil Karn, KA9Q, organized the specifi-

cation and submitted an initial version on 6 August 1986.  As of

this writing, the following KISS TNC implementations exist:

TNC  type		  Author                  Comments

TAPR TNC-2 & clones    Mike  Chepponis,  K3MC  First implementation,

most widely used.

Exists in both

downloadable and

dedicated  ROM.

TAPR TNC-1 & clones    Marc Kaufman, WB6ECE    Both download and

dedicated ROM.

VADCG  TNC             Mike Bruski, AJ9X       Dedicated ROM.

AEA PK-232 & PK-87     Steve Stuart, N6IA      Integrated into

standard AEA firmware

as  of  21 Jan 1987.

The special commands "KISS ON" and "KISS OFF" (!) control entry

into KISS mode.

Kantronics             Mike  Huslig             Integrated  into

standard Kantronics

firmware as of July

1987.

The AEA and Kantronics implementations  are  noteworthy  in

that  the  KISS  functions  were  written  by  those vendors and

integrated into their standard  TNC  firmware.  Their  TNCs  can

operate  in  either  KISS  or  regular  AX.25  mode  without ROM

changes.  Since the TNC-1 and TNC-2 KISS versions  were  written

by  different  authors  than  the  original  AX.25 firmware, and

because the original source code for those  TNCs  was  not  made

available,  running KISS on these TNCs requires the installation

of nonstandard ROMs. Two ROMs are available for the  TNC-2.  One

contains "dedicated" KISS TNC code; the TNC operates only in the

KISS  mode.  The  "download"  version  contains  standard   N2WX

firmware with a bootstrap loader overlay. When the TNC is turned

on or reset, it executes the loader. The loader  will  accept  a

memory  image  in Intel Hex format, or it can be told to execute

the standard N2WX firmware  through  the  "H"[3]  command.   The

July 14, 1990

download version is handy for occasional KISS  operation,  while

the  dedicated  version is much more convenient for full-time or

demo KISS operation.

The code for the TNC-1 is also available in  both  download

and  dedicated  versions.  However,  at present the download ROM

contains only a bootstrap; the original ROMs must be put back in

to run the original TNC software.

8.  Credits

The combined "Howie + downloader" ROM  for  the  TNC-2  was

contributed  by  WA7MXZ.  This document was carefully typeset by

Bob Hoffman, N3CVL.

9.  Bibliography

1.   Tanenbaum, Andrew S., "Computer  Networks"    pp.  288-292.

Prentice-Hall  1981.

2.   Tobagi, F. A.: "Random Access Techniques for Data Transmis-

sion  over  Packet  Switched Radio Networks," Ph.D. thesis,

Computer Science Department, UCLA, 1974.

3.   Kleinrock, L., and Tobagi, F.:  "Random  Access  Techniques

for Data Transmission over Packet-Switched Radio Channels,"

Proc. NCC, pp. 187-201, 1975.

4.   Tobagi, F. A., Gerla, M., Peebles, R.W., and Manning, E.G.:

"Modeling  and  Measurement Techniques in Packet Communica-

tions Networks," Proc.  IEEE, vol. 66, pp. 1423-1447,  Nov.

1978.

5.   Lam, S. S.: "Packet Switching in  a  Multiaccess  Broadcast

Channel",  Ph.D. thesis, Computer Science Department, UCLA,

1974.

6.   Lam, S. S., and Kleinrock, L.: "Packet Switching in a  Mul-

tiaccess  Broadcast  Channel:  Dynamic Control Procedures,"

IEEE Trans. Commun., vol COM-23, pp. 891-904, Sept. 1975.

7.   Lam, S. S.: "A Carrier Sense Multiple Access  Protocol  for

Local  Networks,"  Comput. Networks, vol 4, pp. 21-32, Feb.

1980

8.   Tobagi, F. A.: "Multiaccess Protocols in Packet  Communica-

tions  Systems,"  IEEE Trans. Commun., vol COM-28, pp. 468-

488, April 1980c.

July 14, 1990

9.   Bertsekas, D., and Gallager, R.: "Data Networks", pp.  274-

282 Prentice-Hall 1987.

10.  Kahn, R. E., Gronemeyer, S. A., Burchfiel, J., and  Kungel-

man,  R.  C.   "Advances in Packet Radio Technology," Proc.

IEEE.  pp. 1468-1496.  1978.

11.  Takagi, H.: "Analysis of Polling  Systems,"  Cambridge,  MA

MIT Press 1986.

12.  Tobagi, F. A., and Kleinrock, L. "Packet Switching in Radio

Channels: Part II - The Hidden Terminal Problem in CSMA and

Busy-Tone Solution," IEEE Trans. Commun.  COM-23 pp.  1417-

1433.  1975.

13.  Rivest, R. L.: "Network Control  by  Bayessian  Broadcast,"

Report MIT/LCS/TM-285.  Cambridge, MA.  MIT, Laboratory for

Computer Science.  1985.

14.  Karn, P. and Lloyd, B.: "Link Level  Protocols  Revisited,"

ARRL  Amateur  Radio  Fifth Computer Networking Conference,

pp. 5.25-5.37, Orlando, 9 March 1986.

15.  Karn, P., "Why Do We Even Need TNCs Anyway", Gateway,  vol.

3 no. 2, September 5, 1986.

July 14, 1990