SMACK - Protocol Description

----------------------------

Version 1.0, Dated 27.02.92

by Jan Schiefer, DL5UE and Dieter Deyke, DK5SG/N0PRA

1. Introduction

The end of 1990 was the first time that the Stuttgart Packet-Radio-Amateurs

thought about the factual certainty of Data Security between TNC and

WAMPES(*)- Node Computer. Already with other Packet-Node Systems Data losses

had appeared, we considered which sort and in what manner a compatible

development of the KISS-Protocols with a Checksum could be considered

possible. From the Result of this consideration came the name SMACK

(Stuttgart Modified Amateur radio CRC-KISS. This Document should explain

the difference between SMACK and KISS and make possible the implementation

of other Systems.

2. What is KISS?

KISS had been recommended by Phil Khan KA9Q [1]. Because his TCP/IP-Software

it needed a Protocol, that enabled a simple entry to Packet-Radio below

the AX.25-Protocol level. KISS offers a layer 2a-Entry. Assignments of the

TNC are basically, only the Access Control of the Frequency (Channel-Busy-

Recognition, P-Persistence-Action) and the conversion of the synchronous

HDLC-Data on the PR-Channel into the asynchronous Format of the  RS232-Port.

The KISS-Protocol regulates the demarcation of individual Packets with

Delimiters, the eventual handling n the Data Stream of the appearing

Delimiter and defines a simple Command structure for the installation of

TNC-Parameters. Also, it has met the provision, to be able to operate TNCs

with more Packet-Channels.

3. Modification of the KISS-Protocol

The Host Computer communicates with KISS in the Form of Packets. The

beginning of such a Packet will be recognised through the FEND. Then

follows the so called Command Byte. It issues, whether it is handling Data-

or a Command Packet and what Command is meant. With one exception (Reset-

Command = 255) all Commands employ only the lower 4 Bits of this Command

Byte. This means, that the upper 4 Bits with Multi-Channel-TNCs can issue

the Channel. So 16 Channels can be given Parameters individually.

Because we do not know either 16- or 8-Channel-TNCs, we have used a purpose

other than originally intended, the topmost Bit of this Command Byte. If

it is set, it handles itself with the doubtful Packet as a Data Packet with

CRC (only with Data Packets are to found a Checksum calculation instead!).

With such Packets, the Checksum will be attached at the End of the Frame,

and the lower value Byte first. Both of the following illustrations show the

layout of a Data Packet once with and once without Checksum.

+------+------+------+------+-----+------+------+

| FEND | 0x00 | DATA | DATA | ... | DATA | FEND |

+------+------+------+------+-----+------+------+

KISS-Frame without Checksum

+------+------+------+------+-----+------+---------+----------+------+

| FEND | 0x80 | DATA | DATA | ... | DATA | CRC LOW | CRC HIGH | FEND |

+------+------+------+------+-----+------+---------+----------+------+

Smack-Frame with Checksum

It should be once again reiterated here, that only Data Packets will be

CRC-secured. With that it will be prevented, that a Command can be sent to

the TNC, when themselves Host and TNC with CRC/not CRC are in disagreement.

4. Switching from KISS onto SMACK

A SMACK-TNC works after switching into the KISS-Modus, therefore generates

no Checksum. As soon as the first Packet is received with CRC, it switches

its Send Routines likewise to CRC. This Operation condition can be

abandoned only through a Reset. The Host behaves analogously.  If however a

KISS-TNC is connected, so CRC-Frames from the Host by reason of their

unknown Command Byte will be discarded and working continued.

This Method has the advantage, that both KISS- as also SMACK-TNCs can be

operated alternately to a Host; without that, a reconfiguration is

necessary. That should be illustrated by a Display of both cases.

---------------------------------------------------------------------------

Case 1: KISS-TNC

Host                                    TNC

- Sends an individual Packet with

CRC, then switches its Sender

on Normal-KISS again.

- Receives a frame which for

it is an unknown Command-

byte 0x80 and abandons it.

- Dispatches KISS-Data without

Checksum.

- Dispatches KISS-Data without

Checksum.

---------------------------------------------------------------------------

Fall 2: SMACK-TNC

Host                                    TNC

- Sends an individual Packet with

CRC, then switches its Sender

onto Normal-KISS again.

- Receives a Frame with CRC-

Recognition, switches its

Sender on CRC.

- Dispatches KISS-Data without

Checksum.

- TNC sends the first Frame

with CRC.

- Receives a Frame with CRC-

Recognition, switches its

Sender on CRC.

- Dispatches SMACK-Data with

Checksum.

- Dispatches SMACK-Data with

Checksum.

---------------------------------------------------------------------------

Dependent upon Send condition (CRC/not CRC) the received Frames will always

be handled as follows:

Receiver Frame                 | Action

-------------------------------+------------------------

No CRC                         | Frame progressed further

With CRC, Correct Checksum     | Frame progressed further

With CRC, False Checksum       | Frame abandoned

This Protocol sets in advance, that a KISS-Implementation abandons its

unknown Frame. This will be promoted in the KISS-Specification [1].

4. CRC-Calculation and Implementation Tips

This is not the correct place, to explain the Theory of the cyclical

Redundancy-check CRC. Reference should be made to the work of Michael

Rhner, DC4OX [2]. This extract depicts only important Details for an

implementation.

As Check Polynomial the CRC16-Polynomial will be used. This has the form

16    15   2

X   + X  + X  + 1

The CRC-Generator will be pre-set with 0. The calculation of the CRC will be

of the CRC over all Data Bytes inclusive of the Command Byte 0x80.

As is well known in the KISS-Protocol the demarcation of the Frame will be

performed with the FEND-Character (0xc0). In the case where this Character

occurs in the Data Stream, it will be handled otherwise. This occurrence

will be named SLIP-Encoding.

The CRC must be calculated, before the SLIP-Encoding takes place, and will

be checked, after the SLIP-Decoding has taken place. Instead it gives more

Grounds:

- The CRC Bytes could contain FESC, TFEND, FEND etc.

- The SLIP En/Decoder will be used in most Host-Implementations (e.g.

WAMPES) also dependent upon KISS, in order to make the Connection to the

Unix-Kernel as an example. Additionally, it is admitted that in this case

CRC-Checking would be desirable, but will not be understood by the other

side.

The CRCs logically belong therefore, to the KISS Layer.

Instead the Calculation is to be found as follows:

- CRC-Generator pre-set with 0.

- All Data Bytes one behind the other added into the Algorithm, including

both CRC-Bytes.

- At the End a 0 must stand again in the CRC-Generator. If the value is not

0, so  a Transfer Error has occurred and the Frame must be discarded.

Different Algorithms for the CRC-Generator will be described in [2].

Here is a simple table of a controlled Algorithm issued in the C

Programming-Language, which the CRC of a Buffers (buf) calculates the

Length n

/*---------------------------------------------------------------------------*/

static int  calc_crc(char *buf, int n)

{

static int  crc_table[] = {

0x0000, 0xc0c1, 0xc181, 0x0140, 0xc301, 0x03c0, 0x0280, 0xc241,

0xc601, 0x06c0, 0x0780, 0xc741, 0x0500, 0xc5c1, 0xc481, 0x0440,

0xcc01, 0xcc0,  0x0d80, 0xcd41, 0x0f00, 0xcfc1, 0xce81, 0x0e40,

0x0a00, 0xcac1, 0xcb81, 0x0b40, 0xc901, 0x09c0, 0x0880, 0xc841,

0xd801, 0x18c0, 0x1980, 0xd941, 0x1b00, 0xdbc1, 0xda81, 0x1a40,

0x1e00, 0xdec1, 0xdf81, 0x1f40, 0xdd01, 0x1dc0, 0x1c80, 0xdc41,

0x1400, 0xd4c1, 0xd581, 0x1540, 0xd701, 0x17c0, 0x1680, 0xd641,

0xd201, 0x12c0, 0x1380, 0xd341, 0x1100, 0xd1c1, 0xd081, 0x1040,

0xf001, 0x30c0, 0x3180, 0xf141, 0x3300, 0xf3c1, 0xf281, 0x3240,

0x3600, 0xf6c1, 0xf781, 0x3740, 0xf501, 0x35c0, 0x3480, 0xf441,

0x3c00, 0xfcc1, 0xfd81, 0x3d40, 0xff01, 0x3fc0, 0x3e80, 0xfe41,

0xfa01, 0x3ac0, 0x3b80, 0xfb41, 0x3900, 0xf9c1, 0xf881, 0x3840,

0x2800, 0xe8c1, 0xe981, 0x2940, 0xeb01, 0x2bc0, 0x2a80, 0xea41,

0xee01, 0x2ec0, 0x2f80, 0xef41, 0x2d00, 0xedc1, 0xec81, 0x2c40,

0xe401, 0x24c0, 0x2580, 0xe541, 0x2700, 0xe7c1, 0xe681, 0x2640,

0x2200, 0xe2c1, 0xe381, 0x2340, 0xe101, 0x21c0, 0x2080, 0xe041,

0xa001, 0x60c0, 0x6180, 0xa141, 0x6300, 0xa3c1, 0xa281, 0x6240,

0x6600, 0xa6c1, 0xa781, 0x6740, 0xa501, 0x65c0, 0x6480, 0xa441,

0x6c00, 0xacc1, 0xad81, 0x6d40, 0xaf01, 0x6fc0, 0x6e80, 0xae41,

0xaa01, 0x6ac0, 0x6b80, 0xab41, 0x6900, 0xa9c1, 0xa881, 0x6840,

0x7800, 0xb8c1, 0xb981, 0x7940, 0xbb01, 0x7bc0, 0x7a80, 0xba41,

0xbe01, 0x7ec0, 0x7f80, 0xbf41, 0x7d00, 0xbdc1, 0xbc81, 0x7c40,

0xb401, 0x74c0, 0x7580, 0xb541, 0x7700, 0xb7c1, 0xb681, 0x7640,

0x7200, 0xb2c1, 0xb381, 0x7340, 0xb101, 0x71c0, 0x7080, 0xb041,

0x5000, 0x90c1, 0x9181, 0x5140, 0x9301, 0x53c0, 0x5280, 0x9241,

0x9601, 0x56c0, 0x5780, 0x9741, 0x5500, 0x95c1, 0x9481, 0x5440,

0x9c01, 0x5cc0, 0x5d80, 0x9d41, 0x5f00, 0x9fc1, 0x9e81, 0x5e40,

0x5a00, 0x9ac1, 0x9b81, 0x5b40, 0x9901, 0x59c0, 0x5880, 0x9841,

0x8801, 0x48c0, 0x4980, 0x8941, 0x4b00, 0x8bc1, 0x8a81, 0x4a40,

0x4e00, 0x8ec1, 0x8f81, 0x4f40, 0x8d01, 0x4dc0, 0x4c80, 0x8c41,

0x4400, 0x84c1, 0x8581, 0x4540, 0x8701, 0x47c0, 0x4680, 0x8641,

0x8201, 0x42c0, 0x4380, 0x8341, 0x4100, 0x81c1, 0x8081, 0x4040

};

int  crc;

crc = 0;

while (--n >= 0)

crc = ((crc >> 8) & 0xff) ^ crc_table[(crc ^ *buf++) & 0xff];

return crc;

}

/*---------------------------------------------------------------------------*/

Saving of the Table requires 512 Bytes. If the EPROM in the TNC should

be too full for the saving of this Table, so it can be constructed as

follows for the Runtime:

/*---------------------------------------------------------------------------*/

unsigned short Table[256];

const int Rest[8] = { 0xC0C1, 0xC181, 0xC301, 0xC601, 0xCC01, 0xD801,

0xF001, 0xA001 };

main()

{

int i, j;

unsigned short value;

for (i = 0; i < 256; i++) {

value = 0;

for (j = 0; j < 8; j++)

if (i & (1 << j))

value ^= Rest[j];

Table[i] = value;

}

}

/*---------------------------------------------------------------------------*/

If this Algorithm is to be coded in Assembler, so it requires distinctly

less space than the Saving of the Table itself.

The Theory is to be found again in [2].

5. Implementation

Previously this Protocol has been implemented in the following Systems:

- WAMPES

- SMACK, Version 1.3. This Software has been developed further by Jan

Schiefer, DL5UE, from TNC2-KISS-Implementation written by K3MC. It will

be installed especially in the TNCs of the WAMPES-NODES DB0ID (Digipeater

Stuttgart) and DB0SAO (Mailbox Stuttgart) there it contains also,

individual WAMPES-specific adaptations. - In the KISS of the NORD><LINK-

Firmware, from TheFirmware Version 2.4 - For the TCP/IP-Software NOS it

gives from Thommy Osterried, DL9SAU, a 'a rundown', of the NOS developed

from the SMACK-capabilities.

7. Outlook

A  desirable Implementation of this Protocol would be especially, a Packet-

Driver from the 'Packet driver specification' [3]. With it would be given an

uninhibited possibility for all existing NET- and NOS-Versions without

alteration of the Source Codes. But also every other Packet-Radio-Software,

that installs the KISS-TNCs could profit from the Error Protection of the

CRC.

This Protocol Description has a preliminary Character. The Authors

(dl5ue@db0sao, dk5sg@db0sao) are thankful for Improvement recommendations,

Advice on Error or other Comments.

(*) WAMPES = Wrttemberg Amateurraadio Multi-Protocol-Experimental-Software.

A software packet under UNIX, that implements multiple Protocols from

AX.25-, TCP/IP and NET/ROM-Protocol Families and, will be installed

mainly in the Stuttgart Area, as Net Nodes. Compare [4].

Literature

[1] Karn, Phil, KA9Q; Proposed "Raw" TNC Functional Spec, 6.8.1986;

published in the USENET-News;

[2] Rhner, Michael, DC4OX; Was ist CRC?; distributed in the Packet-Radio

Mailbox-Net, May 1988

[3] FTP Software, Inc.; PC/TCP Version 1.09 Packet Driver Specification;

Wakefield, MA 1989

[4] Schiefer, Jan, DL5UE; WAMPES - Weiterentwicklung; Vortrags-Skriptum

des 5. berregionalen Packet-Radio-Treffens; Frankfurt 1989;

Translation by G0KIU