de DG9EP @ DB0IZ.#NRW.DEU.EU

to IW2GKO @ IW2GKO.ITA.EU

EXTENDED HOSTMODE COMMAND    by DG3DBI

The main purpose of the XHOST-Mode is the reduction of unnessecary

polling of the TNC and so an increment of speed on the seriell line.

There are only very few changes necessary in existing Hostmode-Terminal-

programs. E.g. GP support this mode.

The PC transmit the 'G'-Kommando to channel 255 of the TNC

An old-fashioned TNC without XHOST will answer with the errormessage

"INVALID CHANNEL NUMBER".

A TNC using the Extended Hostmode will transmit a nul-terminated string

(ASCIIZ) back to the PC. This string contains a list of channel

which have already some kind of Info in the buffer. This is also true

for the Monitor-channel (channel 0) and the statusinformation (Statusdaten).

The channelnumber inside the string are incremented by one, since the

null-terminated string wouldn't allow data of channel 0 otherwise.

Example for the answer of the TNC:

0xFF 0x01 0x00   - No Data available.

0xFF 0x01 0x01 0x00 - Some Data are waiting in the Monitor-Buffer.

0xFF 0x01 0x01 0x03 0x04 0x00 - Data in Monitor and channel 2 and 3

regards, Walter