Documentation for

NORAD SGP4/SDP4 Units

Developed by

Dr TS Kelso

Version 2.50

1992 October 01

Copyright (C) 1992.  All rights reserved.

PURPOSE

The enclosed Pascal source code implements the NORAD SGP4/SDP4 orbital models

for use with the standard two-line orbital element sets to determine earth-

centered inertial (ECI) and topocentric coordinates of earth-orbiting objects.

This code implements both the near-earth and deep-space portions of the NORAD

SGP4 orbital model.  These units are designed to make the development of

programs based on the NORAD orbital models straightforward and standardized.

INTRODUCTION

In order to properly determine the position of any earth-orbiting object using

the standard NORAD two-line element sets, it is necessary that the proper

orbital model be used.  Since the observations taken by NORAD for each earth-

orbiting object are reduced to orbital elements using the SGP4 (Simplified

General Perturbations) model, the SGP4 model *must* be used to get the most

accurate determination possible of an object's position and velocity.  The

primary reason for this requirement is that each orbital model handles

perturbations (due to atmospheric drag, solar and lunar gravitational effects,

irregularities of the earth's gravitational field, etc.) in a different

manner.  The NORAD two-line element sets incorporate these perturbations using

the SGP4 orbital model and that model is required to accurately reconstruct

the magnitudes of these effects.

The SGP4 orbital model takes into account perturbations due to atmospheric

drag (based on a static, non-rotating, spherically-symmetric atmosphere whose

density can be described by a power law), fourth-order zonal geopotential

harmonics (J2, J3, and J4), spin-orbit resonance effects for synchronous and

semi-synchronous orbits, and solar and lunar gravitational effects to first

order.  The two portions of the SGP4 model are SGP4 (for objects in orbits

with periods less than 225 minutes) and SDP4 (for objects in orbits greater

than or equal to 225 minutes).  The reason for breaking the model into two

parts is that for low-earth orbits the effects of spin-orbit resonance and

lunar and solar gravity are not significant.  This result allows the

development of an analytical model (SGP4) for determining an object's position

and velocity, thereby reducing the computational burden.  For deep-space

orbits, a semi-analytical model (SDP4) is required.

Models which implement the older SGP model should be accurate for low-earth

orbits but really won't be adequate for deep-space objects, particularly those

in resonance with the geopotential.

COMPUTER IMPLEMENTATION

The enclosed Pascal source code was developed in Turbo Pascal Version 6.0 to

fully implement the NORAD SGP4 orbital model.  There are now twelve units

provided:

SGP4SDP4     Full implementation of NORAD SGP4/SDP4 models

SGP_OBS      Observer-dependent routines for calculating topocentric

information

MINMAX       Minimum/maximum functions

SGP_MATH     Various trigonometric and mathematical routines

SGP_TIME     Time-based routines for converting among time systems

SGP_INIT     Code and variables needed to initialize SGP4SDP4

SGP_INTF     Interface between SGP4SDP4 and SGP_CONV (and some special-

purpose programs)

SGP_CONV     Routines for converting two-line data and SGP4 state vectors

SGP_IN       Routines to simplify input of data (with error checking)

SGP_OUT      Routines for outputting program results in standard formats

SOLAR        Routines for calculating the position of the sun and whether

a satellite is in earth umbral eclipse

SUPPORT      General support routines for machine-dependent features

These units are structured to make development of software as simple as

possible and to reduce the time needed for validating results.  A certain

amount of the development is also tailored toward the development of a similar

set of units in C, which is now underway.  A more complete description of

these Pascal units is included in SGP4-PLB.ITF.

Of the units provided, the most complex is the implementation of the NORAD

orbital models (SGP4SDP4).  The development of this unit was done to follow as

closely as possible the implementation contained in Spacetrack Report Number

3, "Models for Propagation of NORAD Element Sets," (a copy of the LaTeX

documentation and complete FORTRAN source code is available on this system).

No attempt has been made at this point to optimize the code to run faster.  A

future release will provide a more streamlined implementation.  To ensure the

validity of these units, no changes should be made to the units themselves

which have been extensively tested and validated.  LaTeX documentation to

support this validation should be available on this system soon (look for

SGP4-VAL.TEX).

A test program (SGP4TEST.PAS) is included to implement the sample cases

included in the Spacetrack Report Number 3 documentation.  It should provide a

reasonably good idea of how to use these Turbo Pascal units.  To determine an

object's position and velocity, the object's two-line orbital element set is

read into the array {sat_data} [Note:  Variable names will be enclosed within

braces to set them apart in this document].  A call is then made to the

procedure Convert_Satellite_Data passing the satellite's index {satnumber}.

In the call to this procedure, a determination is automatically made as to

whether the object is in a near-earth or deep-space orbit; the result is

returned in {ideep}.  If {ideep} is 0, then the object is in a near-earth

orbit; if {ideep} is 1, it is in a deep-space orbit.  A call is then made to

either SGP4 or SDP4 depending on the value of {ideep}.

The time passed in this call, {tsince}, represents the time since (before or

after) the satellite epoch in the two-line element set.  The four-dimensional

vectors {pos} and {vel} are returned containing the x, y, and z ECI

coordinates (referenced to the true equator and mean equinox of date) and

vector magnitude of the object's position and velocity, respectively, at the

specified time.  Units are earth radii and earth radii per minute,

respectively; appropriate conversions must be made to get the proper units (as

demonstrated in the test program using Convert_Sat_State).  The variable

{iflag} is used internally for keeping track of initialization conditions for

the deep-space portion of the model.

Running this test program should provide results quite close to those provided

in the Spacetrack Report Number 3 documentation.  The minor differences that

do occur are due to two primary reasons.  The first is the result of

differences in the internal precision of the FORTRAN compiler used to generate

the report and that of the Turbo Pascal compiler.  The second is due to the

choice to implement a consistent numerical precision in the Turbo Pascal code.

Examination of the FORTRAN source code will reveal inconsistent use of single

and double-precision variables and trigonometric functions; the Turbo Pascal

code uses double-precision variables and functions throughout.

Because it is not very straightforward to perform calls to the SGP4 units

based on the time since each satellite's element set epoch, a procedure is

also provided which interfaces between a standard time system and the SGP4 and

SDP4 calls.  This procedure, SGP, requires only that the user pass the Julian

Date of interest; the procedure Julian_Date_of_Epoch converts from a date in

the format used in the two-line element set to a Julian Date.  The SGP call

also takes care of making the determination of whether the object is in a

near-earth or deep-space orbit and calls the appropriate model.  The SGP_TIME

unit contains these procedures as well as routines to convert Julian Dates to

more recognizable time formats.

The main advantage of using the Julian Date in SGP is that there is no problem

calculating the time interval between the time of interest and the satellite

element set epoch, regardless of whether this interval spans the beginning of

a year.  There is also a function to convert Julian Dates to calendar dates

(Calendar_Date) of the form "1992 Jan 30."  Epoch_Time transforms the Julian

Dates to the form used in the two-line element sets.

The test program SGP4TST2.PAS gives an example implementation of this

approach.  It is designed to produce the same results as SGP4TEST.PAS.  The

main difference is that the Julian Date is calculated to be the time of the

epoch for each element set and then the offset (in minutes) is added.  Be sure

to note that times outside the units are now all in *days* not in minutes (as

they are inside the units).

CONCLUSION

These Turbo Pascal units should make it easy to implement the official NORAD

orbital models in developing any number of applications.  It is now very

straightforward to select input files (of satellite and observer data) and

time conditions (with full error checking) and output data ranging from

spacecraft ECI position and velocity to spacecraft ground tracks (latitude,

longitude, and altitude) to look angles (azimuth, elevation, range, and range

rate) to right ascension and declination.  Each of these outputs uses the WGS

'72 geoid (nonspherical) and takes into account atmospheric refraction, where

appropriate.  These results can be easily output as text or incorporated into

advanced graphical applications.

For a full-blown example of how to implement these functions, look for the

application TrakStar/SGP4, also available on this system.  It allows output

for up to 250 satellites (or element sets for a single satellite) in the form

of spacecraft ECI position and velocity, spacecraft ground tracks (latitude,

longitude, and altitude), look angles (azimuth, elevation, range, and range

rate), and right ascension and declination.  A separate data file is created

for each object/element set -- a great tool for all sorts of analyses.

Future releases of these units will include routines to efficiently determine

crossing phenomena (e.g., satellite rise/set, sun rise/set, and satellite

entry into/exit from earth eclipse) and local optima (e.g., closest approach).

And, of course, the existing units will be converted to C with an eye toward

portability.

In a continuing effort to promote standardization of orbital calculations, I

will endeavor to continue to improve this package and will gratefully accept

user feedback or contributions to this effort.

- Dr TS Kelso

Internet:  tkelso@afit.af.mil

anonymous ftp at archive.afit.af.mil

in the directory pub/space

SYSOP, Celestial BBS

513/427-0674 (modem)

Operating 24 hours/day

No parity, 8 data bits, 1 stop bit

300, 1200, 2400, 4800, and 9600 bps

v.32/32bis, v.42/42bis

2340 Raider Drive

Fairborn, OH, USA  45324-2001