NOS/BPQ Link - NODEDRV.COM

I have now produces a 'Packet Driver' to allow NOS to communicate with

the switch. It uses the KISS mode application interface (See KISSMODE.DOC

for details). One copy of the packet driver is needed for each radio

port to be used by NOS, plus one for the loopback port if you want NOS

to be able to talk to the switch. Experience with the system is rather

limited, but I suggest that if you are using TCP/IP over NETROM that

you use the lookback port, and let the switch handle all the radio links.

Only if you are using IP in datagram or ax.25 virtual circuit mode will

you need to give NOS direct access to the radio ports.

Please note that you need at least version 3.57 of BPQCODE.EXE

The following is an extract from my system. You will need to tailor

it to your own requirements.

NODEDRV must be loaded in the same window as NOS - if you put it

under desqview the system will probably crash. It takes two parameters,

the software interrupt number used by NOS to communicate with it (in

the range 60-7f hex), and the virtual COM port used to talk to the

switch.

STARTNOS.BAT

nodedrv 0x61 10

nodedrv 0x62 11

nodedrv 0x63 12

nos

This is the relevent TNCPORT bit of BPQCFG.TXT. I have two radio ports

(Port 1 on 2m, and port 2 on 70 cms). Port 3 is the loopback port.

TNCPORT

COM=10

TYPE=KISS

KISSMASK=4		; PORT 3 (100B) - Loopback port

ENDPORT

TNCPORT

COM=11

TYPE=KISS

KISSMASK=2		; PORT 2 (10B) - 70cms

ENDPORT

TNCPORT

COM=12

TYPE=KISS

KISSMASK=1		; PORT 1 (1B) - 2m

ENDPORT

This is the 'attach' bit of AUTOEXEC.NET. I can't see where the parameter

following the interface name is used, but the code suggests it is the

maximum length of the transmit queue. The next param is the MTU.

#

attach packet 61 node 5 236

attach packet 62 ax2 5 256

attach packet 63 ax1 5 256

#

#	MYCALL doesnt seem to be applied to packet driver interfaces, so

#	    set up a callsign (or it sends garbage)

#

if node link g8bpq-5

if ax1 link g8bpq-5

if ax2 link g8bpq-5

#

This is a new facility, and there is very little local IP activity for

me to try it with. Any feedback would be most welcome, both on problems

and suggestions on how best to set up the system.

73,

John Wiseman, G8BPQ

76, Haywood Road,

Mapperley,

Nottingham.

NG3 6AE

6th June, 1990

##########