Monitor Heard List Ver 1.02 for BPQ Nodes V3.2 or above

P.M.Hounslow G4YFE 1989

Install the MH facility in the BPQ Node, by entering MH

in the applications description section of BPQCFG.TXT.

e.g. for a single window, all radio ports:-

APPLICATIONS=BBS,HOST,RSYS,MH

for 2 windows on radio ports 1 and 2

APPLICATIONS=BBS,HOST,RSYS,MH1,MH2

For each copy of MH.EXE to be run enter the command

MH c a p l

Where c is the COM port number; 1 for COM1:, 2 for COM2:, 3 for COM3: etc

a is the application number (in decimal). i.e. 8 and 8,16 for the above

examples, see the APPL $nn command for more details.

p is the radio PORTS to monitor (in decimal) i.e. 1=1, 2=2, 3=1 & 2,

4=3, 8=4,....255=ALL. This permits seperate MH windows to monitor

seperate radio ports, as in the second example, see the MMASK $nn

command for more details.

l is the length of the heard list, max=20, min=5, default=15.

The program checks for BPQCODE to be present and warns you before exiting if

it is not present.

The monitor status line shows whether it is currently/has been connected to a

station, also whilst the list is being sent a line sent count appears.

To quit from the program before closing the DESQview window press 'ESC'.

The current wishlist includes a 'multi-monitor' version so that additional

copies of the code do not have to be run, this will ask for the Port/Band

when the user connects.