CONFIG.DOC   Documentation on ARESDATA.CFG for Version 1.4 for SYSOP

ARES/Data will not run correctly unless you have properly edited

ARESDATA.CFG!  In addition, to set your callsign, you must edit

STARTUP.xxx for the TNC type that you are using.  Please read this file

carefully, and see the example on the distribution disk for a good config file.

Multiple comment lines starting with a semicolon are allowed anywhere in

the file.  Each non-comment line begins in column 1 with a command

identifier followed by several optional parameter values that can be in

the form "value" or "identifier=value".  Follow each parameter

definition with at least one space.  Look at the example ARESDATA.CFG

for illustrations.  Below is a description of each configuration

command.  Replace the square brackets and the parameter name inside with

the value you desire for that parameter.  For string values, remove the

double quotes when you enter the string in the config file.

journal [journalpath] [jrninterval]

journalpath     is the drive:path for the ARESDATA.JRN file.

jrninterval     is the number of new database transactions

between updates of the journal file. max=25.

backup [backuppath] [backupint]

backuppath      is the drive:path for the .BAK files.

backupint       is the backup interval in number of new database

transactions since the last backup. 0 means

no backup.

DBpath [dbpath]

dbpath          is the drive:path for the database files

Indexpath [ndxpath]

ndxpath         is the drive:path for the index files

Publicpath [pubpath]

pubpath         is the drive:path for public downloading of your

choice.  Be sure to put the "info" file in

this subdirectory so users can type "get info".

printer [printerno]

printerno       is the printer number.  Put 1 for lpt1:, etc.

startup ["auto" or "prompted"]

"auto"          means that the program will attempt to synchronize

with all interfaces automatically.

"prompted"      means you want to be asked to turn on each TNC

logall                  If present, causes all incoming packets to be logged

to the sysop screen.

attach [name] type=[xxxx] port=[num] baud=[bnum] users=[nuser]

autoexec=[filename] ["deleteoff" or "deleteon"]

name            is a three-letter name for this interface that

you choose, i.e., "ax0", "ax1", for asynch ports,

"dr0" for the DRSI port, etc.

xxxx            is the interface type: must be "ded" for any TNC

with WA8DED firmware, or "drsi" if there are

DRSI boards present.   Note no matter how many

DRSI boards you might have, use only ONE attach

command!  But you must have one attach command

for each non-DRSI tnc.

num             is the COM port number.  Use "0" for DRSI.

For all others, use the port number you have

selected with MBBCONFG.EXE.  Most often, you

will use "1" for COM1: with address 3F8 and

int4, and "2" for COM2: with address 2F8 and

int3.

bnum            is the baud rate.  Not used for DRSI (see example in

ARESDATA.CFG).  For serial ports, we recommend 4800 or

lower.

nuser           is the maximum number of users allowed on this

interface at one time.  Set this to be equal

to or less than the maximum number of connects

allowed on this particular TNC.  For DRSI,

this should be less than or equal to the maximum

number of connects allowed by the TNCTSR you are

using, with all "sub-ports" taken together.

filename        is the filename of a startup file that contains

DED commands to be sent to this interface

when ARES/Data starts.  NOTE! THIS IS WHERE THE

DATABASE's CALLSIGN IS SET - EDIT TO SET YOURS!!

See the STARTUP.XXX

files on the distribution disk for examples.

Note that every command in the startup file

must END with a semicolon ";".

"deleteoff"     if this parameter is present, this interface

starts with remote delete disabled.  (see

SYSOP.DOC for an explanation)

"deleteon"      if this parameter is present, this interface

starts with remote delete enabled.

retry [ntimes]

ntimes          is the serial port retry count.  Adjust this

upward if you are having TNC timeout errors.

pcparetry [ntimes]

ntimes          is the DRSI PC*PA adapter retry count.  Adjust this

upward if you are having DRSIin timeout errors.

beacon [interval] [beacontext]

interval        is the beacon (unproto) interval in MINUTES.

Please do not beacon with a short interval!!!!!!

Zero ("0") disables beacon function.

beacontext      is the beacon text

labels = [label1],[label2],[label3],[label4],[msglabel]

label1          is the label for field 1 (20 chars max)

label2          is the label for field 2

label3          similar

label4          similar

msglabel        similar

widths = [wid1],[wid2],[wid3],[wid4]

wid1            is the printing width for field 1

wid2            is the width for field 2

wid3            similar

wid4            similar

end CONFIG.DOC