PMP -- Poor Man's Packet

Version 1.1

A software TNC for the IBM PC.

by

Andrew C. Payne, N8KEI

April, 1990

(last revised July, 1991)

Disclaimer:

No guarantees  or warranties  of any kind are expressed

or implied.  This program is  supplied "AS  IS" without

any warranty  of any kind, either expressed or implied,

including, but not limited  to, the  implied warranties

of  merchantability   and  fitness   for  a  particular

purpose.  Neither the author  nor  anyone  else  who as

been involved  in the creation, production, or delivery

of  this  program  shall  be  liable  for  any  direct,

indirect,   or   consequential  or  incidental  damages

arising  out  of  the  use  or  inability  to  use this

program.

Copyright (c) 1990, 1991 Andrew C. Payne

All rights reserved.

TABLE OF CONTENTS

=================

Introduction . . . . . . . . . . . . . . . . . . . . . . . .    3

Getting Started  . . . . . . . . . . . . . . . . . . . . . .    5

Hardware Requirements . . . . . . . . . . . . . . . . .    5

Installing, Configuring, and Running PMP  . . . . . . .    5

Configuration Commands  . . . . . . . . . . . . . . . .    5

Using PMP  . . . . . . . . . . . . . . . . . . . . . . . . .   10

Keyboard Commands . . . . . . . . . . . . . . . . . . .   10

Modem Interface  . . . . . . . . . . . . . . . . . . . . . .   12

PMPTEST  . . . . . . . . . . . . . . . . . . . . . . . . . .   13

Commands  . . . . . . . . . . . . . . . . . . . . . . .   13

Getting your modem to work with PMP . . . . . . . . . .   14

Introduction

============

What is PMP?

PMP  is  a  packet  radio  program  that  uses  the software

approach.  In a  nutshell, this  means that  the things that

are  normally  done  with  hardware  in  a  conventional TNC

(Terminal Node Controller) are instead done with software in

your computer.  The advantage of this method is that most of

the hardware of a normal TNC  is  no  longer  required.   In

simplifying the  hardware, you  lose some of the flexibility

of a TNC.

Who is PMP intended for?

PMP is intended for anyone with an IBM PC type  computer who

wants to get into packet very inexpensively.  PMP is perfect

for someone who wants to get his/her feet wet before sinking

a lot  of money  into equipment.   PMP is also ideal for the

scanner listener who wants  to  monitor  packet  but doesn't

want to  spend another scanner's worth of money.  Because of

the minimal hardware required, PMP is also good for portable

setups.   Modems can  be constructed  to actually fit inside

VHF/UHF radios, eliminating the need for  a separate  TNC or

modem box.

What do I need to run PMP?

PMP  requires  an  IBM  PC/AT/386/486  or compatible with at

least 256K of memory and a parallel printer port (many video

cards have  a parallel port built in).  You also need a Bell

202  type  modem,  VHF/UHF  radio,  and  cables  to  connect

everything up.  Finally, you need the PMP software.

What are the advantages of PMP over a "regular" TNC?

The advantages  of PMP  are basically  those outlined above:

minimal hardware and low price.  The only  hardware required

is a  1200 baud  Bell 202 modem and cables to connect to the

computer and radio.  We have a single-chip  design that fits

on a  2 inch  square board  and costs  less than $40.  Also,

this crystal controlled modem  is a  much better  modem than

those in many of the TNCs.

What do I lose by using PMP instead of a TNC?

PMP doesn't  have all  of the flexibility of a TNC.  It runs

on PCs ONLY.  Also, since PMP is a program  on your  PC, you

can't leave  PMP on  (to act  as a  digipeater, for example)

without tying  up your  entire computer.   Also,  all of the

packet software  for the PC designed to work with TNCs (such

as YAPP, the W0RLI BBS, and KA9Q's TCP/IP software) will not

work with PMP.

How was PMP developed?

DIGICOM>64  for  the  Commodore  64  (which  uses  the  same

software approach) has been  around for  a while.   With the

popularity  of  DIGICOM>64  and  the  growing  popularity of

inexpensive IBM PC clones, we asked ourselves:   "Has anyone

done a  DIGICOM>64-like program  for the  PC?"  Some queries

about such a program resulted in answers like:  "Nope, never

heard of  such a  thing.  Sure would be a good idea though."

At that point, we decided to start the PMP project.   Kevin,

WB2EMS, concentrated on the hardware design and Andy, N8KEI,

developed the software.    Within  a  few  weeks,  the first

prototype  modem  was  built,  and  two weeks after that PMP

sending and receiving its first packets.

I have some more questions.  How can I get in touch with you?

For software  and general  questions, problems, suggestions,

or comments, write to:

Andrew C. Payne, N8KEI

Route 3, Box 78-Q

Berkeley Springs, WV  25411

Or, if you have access to Internet e-mail, write to:

payne@tcgould.tn.cornell.edu

For hardware  questions, problems,  suggestions, or comments

write to:

Kevin Feeney, WB2EMS

468 Hines Road

Ithaca, NY  14850

Also, write to Kevin for kit availability.  Currently (July,

1991)  we   have  TCM3105   modem  chips,   PC  boards,  and  the

miscellaneous parts needed to make a complete modem.

Getting Started

===============

Hardware Requirements

---------------------

The following hardware is required to run PMP:

- an IBM PC/XT/AT/386/486 or compatible with at least 256K RAM

and a parallel printer port

- a Bell 202 type 1200 baud modem

- A VHF/UHF radio

- cables  to  connect  the  printer  port to the modem and the

modem to the radio

Installing, Configuring, and Running PMP

----------------------------------------

Installing PMP is very  straightforward: only  two files are

required, one is optional:

PMP.EXE        The PMP program

PMP.CFG        The Configuration file

PMPTEST.EXE    (optional)  An alignment/test program

To install  PMP on your system, copy these files to a floppy

disk or a subdirectory on your  hard disk.   To  start, just type

'PMP' from  the DOS  prompt and  you should see the opening title

screen in a few seconds.

The  configuration  file,  PMP.CFG,   contains  options  and

parameters  specific  to  your  packet  station:   your callsign,

timing parameters, and other items described in detail below.

The PMP.CFG file is  a  an  ASCII  text  file  which  can be

modified with  any standard text editor (such EDLIN supplied with

DOS).  Lines that begin with  a pound  sign (#)  are comments and

are ignored.  All other lines are configuration commands for PMP,

listed below:

Configuration Commands

----------------------

mycall <callsign>

Sets your callsign in the form of N8KEI-XX, where XX  is the

SubStation  Id  (SSID).    The SSID is used to differentiate

between  multiple  stations  with  the  same  callsign (e.g.

N8KEI, N8KEI-1,  N8KEI-2, etc).   The  SSID ranges from 0 to

15, and 'N8KEI' and 'N8KEI-0' are equivalent.  Your callsign

MUST be correct before transmitting.

txdelay <milliseconds>

Sets  the  transmitter  delay.    This is the time PMP waits

after keying the transmitter and before sending data.   Slow

transmitters  (synthesized  rigs,  rigs  with  relays in the

transmit path, etc) will  require longer  to stabilize after

the transmit  line is  keyed.  These slow rigs need a larger

'txdelay'.

retry <#>

Number of retries attempted  before  dropping  a connection.

If you  are on a busy channel, you may want to increase this

value  from  its  default  value  (10)   to  prevent  losing

connections.

frack <#>

Number of seconds used to compute the round trip time before

resending packet.  If  you are  on a  busy channel,  you may

want to increase this value.

pthresh <#>

Sets I-packet  size threshold.  This is an optimization that

speeds up the AX.25 protocol for  small packets.   I-packets

with data  fields of smaller than 'pthresh' are resent (with

no polling) when no ACK is  received in  the timeout period.

Longer packets are polled a la standard AX.25 Level 2.

check <#>

Specifies the time-out value (in seconds) for the link keep-

alive timer.  After this  many  seconds  of  inactivity, PMP

will poll the remote node.

maxframe <#>

Specifies  the  maximum  number  of  frames  allowed  to  be

outstanding (e.g. un-acknowledged).  This parameter  must be

between 1 and 7.  On busy channels, a lower value is better.

paclen <#>

Specifies  the   maximum  packet  length  in  bytes.    This

parameter must be between 1 and 256.

normal  <#>

bright  <#>

inverse <#>

status  <#>

message <#>

These commands set the attributes (colors)  for normal text,

bright  text,  inverse  text,  and  the status line.  If not

specified, the  attributes default  to the  attributes for a

monochrome video  system.   An attribute  consists of a code

for a background color and a code for a foreground color:

Color          Foreground          Background

Black               0                   0

Blue                1                   16

Green               2                   32

Cyan                3                   48

Red                 4                   64

Magenta             5                   80

Brown               6                   96

Lightgray           7                   112

Darkgray            8                   -

Lightblue           9                   -

Lightgreen          10                  -

Lightcyan           11                  -

Lightred            12                  -

Lightmagenta        13                  -

Yellow              14                  -

White               15                  -

To form the color  attribute,  just  add  the  code  for the

foreground color to the code for the background color.  Note that

not all colors can be backgrounds.  For  example, for  a white on

red  status  line  (attribute  15+64=79),  use  the configuration

command:

status 79

sound <#>

Enables or  disables sound.   If  <#> is  non-zero, sound is

enabled.  If <#> is zero, sound is disabled.

ctext <string>

Specifies the welcome message to be sent when other stations

connect to you.  If no CTEXT is specified in  the .CFG file,

no  welcome   message  is  sent  during  incoming  connects.

Multiple CTEXT  lines can  be specified:   all  will be sent

during  incoming  connects  in  the order they appear in the

.CFG file.

capture <#>

Specifies the size (in bytes) of the capture  memory buffer.

Capture data  is held  in this  buffer, then flushed to disk

when the buffer fills.  Smaller capture buffers will require

more disk  writes.  On a floppy based systems, a large value

is recommended to minimizes disk writes.  This  value should

always be a multiple of 512 (the MS-DOS sector size).

define <key> <definition>

Defines a  string associated  with a function key.  <key> is

one of the function keys  F1  through  F8  (F9  and  F10 are

currently used  for debug  functions).   <definition> is any

string.  Control  characters   may   be   imbedded   in  the

<definition>  string  with  the  '^' character (e.g. '^M' is

interpreted as a carriage return).  ALT key sequences may be

imbedded with the '~' character (e.g. '~c' is interpreted as

ALT-C, the connect command).

beacon <#>

Specifies the beaconing interval, in seconds.   Beacons (see

'btext', below)  will be  sent at the specified interval.  A

'0' value disables beaconing.

btext <string>

Specifies the text sent during beacons.

baddr <string>

Specifies the address (and path) to which beacons  are sent.

Digipeaters are allowed (e.g. "QST VIA W2CXM-1").

autowrap <#>

Specifies  the  column  number  at which autowrap is active.

Autowrap is a feature similar to most word  processors where

it is  not necessary for the user to hit return after typing

beyond a certain column; the line  is automatically wrapped.

A '0' value disables autowrap.

pttport <port#> <bit> <level>

txport <port#> <bit>

cdport <port#> <bit> <level>

rxport <port#> <bit>

init <port#> <value>

These configuration commands specify the ports and bits that

PMP  uses  for   the   modem   interface.      The  standard

configuration file  assumes values appropriate for the first

parallel port, LPT1.   You should  not have  to change these

commands  unless   you  are  using  PMP  with  LPT2  or  are

experimenting  with  a  different   modem  interface.     To

reconfigure PMP for LPT2:, use these configuration commands:

pttport 0x278 2 0

txport 0x278 1

cdport 0x279 0x80 0

rxport 0x279 8

<port#>  specifies  the  port  number in the PC's I/O space.

<bit> is a bit  mask to  specify the  particular bit  of the

port (e.g.  1 for  bit 0,  2 for  bit 1,  4 for bit 2, etc).

<level> specifies the active  level for  the signal:   1 for

active high,  0 for  active low.  'pttport' (output) defines

the port PMP  uses  to  make  the  radio  transmit. 'cdport'

(input) defines  the carrier detect line.  'txdata' (output)

and 'rxdata' (input) define the output and input data lines.

Note that  no active levels are specified for these signals:

the data lines are edge-sensitive, not level-sensitive.

The 'init' command specifies any port  initializations (e.g.

to enable  mini-modems powered  from spare output bits).  At

start up, PMP writes the specified <value> to  the specified

<port#>.

Sample PMP.CFG file:

#

#    This is a sample PMP.CFG file

#

mycall    N8KEI-2

txdelay   300

retry     12

frack     3

pthresh   15

sound     1

ctext     Hello from Andy in Berkeley Springs, WV.

ctext     If I'm not around, please leave a message.

btext     N8KEI, QTH Berkeley Springs, WV

baddr     QST VIA W2CXM-1

beacon    300

define f1 ~cwb2ems-1^m

#

#    End of sample file

#

All options  in PMP.CFG default to reasonable values if they

are not specified in the file.  However, a MYCALL  parameter must

always be present to set your callsign correctly.

Using PMP

=========

After starting  PMP (by typing 'PMP' at the DOS prompt), you

should see an opening title screen (with current version number).

Press any key to advance from this screen to the main screen.

The main  screen is  divided into  three areas.  Most of the

upper part of the  screen is  the incoming  data area;   incoming

data and packets are displayed here.   The second to last line on

the screen is  the  data  entry  line;  everything  you  type for

transmission is  entered here.   The bottom line of the screen is

the highlighted status line, which has the following  items (from

left to right):

-    The version number

-    Your   callsign   (make   sure   this   is   correct  before

transmitting)

-    The current connect path (blank if currently not connected)

-    Outstanding packet bar, there will  be  an  asterisk  (*) on

this bar for each outstanding packet (a packet that has been

sent but not acknowledged  by the  other station)   This bar

will be blank if currently not connected.

-    TX/RX status lights.  This status indicators will light when

PMP is transmitting or receiving packets.

Keyboard Commands

-----------------

Most commands in PMP are  ALT  key  combinations  (formed by

holding down the ALT key and some other key), listed below:

ALT-C     Connect to  another station.   You will be prompted for

the connect station and path in the form:

WB2EMS VIA W2CXM-1,WB2TVB

Press ESC to abort the connect attempt.

ALT-D     Starts a disconnection.   If  the  remote  node  is not

responding to the disconnect request, press ALT-D again

to force a disconnect.

ALT-B     Sends a beacon packet (with the  text specified  in the

'btext' parameter in the configuration file).

ALT-H     Shows a help screen, listing all keyboard commands.

ALT-J     Screen snapshot.   Writes  the current screen to a text

file.

ALT-N     Nodes recently heard.   Nodes are  listed starting with

the most  recently heard.   Information listed for each

node includes the callsign of the  node, the  time last

heard, the  number of  packets heard,  and the protocol

types used by the node.

ALT-P     Pause (hold) the screen.  After pausing, press  any key

to resume.

ALT-S     Shows the  status of  PMP:   the number of packets sent

and received, error counts, number of bytes free, etc.

ALT-X     Exits to DOS.

ALT-U     Text file upload.  You will be prompted for the name of

an  ASCII  file  to  send  across a currently connected

link.

ALT-L     Toggle  text  file  capture/download.    You   will  be

prompted for  a file  to log  all received data.  "Cap"

will appear  on  the  status  line,  indicating  that a

capture  is  in  progress.    Press ALT-L again to stop

capturing and close the file.

UP & DOWN The [Up] and [Down] arrows can be used  to move forward

and backward  to review  data that has scrolled off the

top of the screen.  [PgUp] and [PgDn] will page through

the scrollback buffer a screenfull at a time.

F1-F4     User defined  function keys.   Sends the string defined

for  the  key  (see   'define'  in   the  configuration

commands).

Modem Interface

===============

PMP interfaces to the external modem (and radio) through the

PC's parallel port.  PMP uses two output pins and  two input pins

which are connected as follows:

Parallel

Port

Pin     Signal    Dir. Description

-----------------------------------------

2       TXD      Out  Transmitted data.

3       PTT      Out  Push-to-talk,  active-high.    Keys  the

radio to transmit.

11      CD       In   Carrier  detect,   active-high.     Goes

active when a packet signal is detected.

15      RXD      In   Receive data.

25      Gnd      Gnd  Signal ground.

The PMPTEST  utility is very useful for testing the parallel

port and modem interface (see next section for more information).

On most PCs,  the  parallel  port  is  a  female  DB-25 type

connector and uses standard TTL levels.

We have  developed a small, inexpensive, two-chip modem that

works well with PMP.  For details, see the August,  1991 issue of

73 Magazine (pages 8-14).

PMPTEST Utility

===============

PMPTEST  is  a  utility  program  that  aids  in testing the

parallel port and modem interface (see MODEM INTERFACE).  PMPTEST

is started  by typing  'PMPTEST' from the DOS prompt.  At present

PMPTEST works only with modems interfaced to LPT1:.

Across the bottom of PMPTEST's screen are four indicators:

Carrier Detect (CD)      Indicates the current input level of the

Carrier Detect line.

Receive Data (RXD)       Indicates the current input level of the

Receive Data line.

Transmit Data (TXD)      Indicates the  current  output  level of

the Transmit Data line.

Push To Talk (PTT)       Indicates  the  current  output level of

the Push To Talk line.

Commands

--------

PMPTEST has several commands:

[ESC]     Exits program.

[SPACE]   Toggles transmit  (PTT)  line.    The  current transmit

status will be indicated with the PTT indicator.

[F2]      On-air modem alignment mode.  (see below)

[F3]      Toggle transmit data level.  The current transmit level

is indicated with the TXD indicator.

[F5]      Transmits a 600hz test signal.   PTT will  be set high,

and the  transmit data  will be  toggled at 600hz.  Any

key will cancel the test.

[F6]      Loopback alignment (currently not implemented).

Getting your modem to work with PMP

-----------------------------------

This section describes some simple tests using PMPTEST.

Carrier Detect

Adjust your radio  and  modem  so  that  the  Carrier Detect

accurately reflects  channel activity.   The CD status light

should indicate channel activity.

Receive Data

The RD indicator should  flicker (on  and off)  when data is

being received.

PTT

Use the [SPACE] bar to toggle the transmit on and off.  Your

radio should start transmitting when PTT goes high  and stop

transmitting when PTT goes low.

Transmit Data

Use the  [SPACE] bar to toggle your radio into transmit mode

and use the [F2] key to  toggle  the  transmit  data.   Your

radio should  transmit a  high or low tone, depending on the

transmit data level.

Receive Alignment

Many modems  (including our  design) have  a receive section

that must  be properly  aligned.   Tune your radio to a busy

packet channel, and make  sure that  the Carrier  Detect and

Receive  Data  lines  are  functioning (as described above).

Use [F1] to enter  the alignment  mode.   As each  packet is

received,  it's  timing  will  be  analyzed  and  two  error

percentages will be displayed  on the  screen.   Adjust your

modem  so  that  these  two  numbers are as close to zero as

possible:  between -5% and +5% is ideal.   Use  long packets

to make your adjustments; short packets or noise bursts will

give inaccurate error readings.