@TRTY AND THE PK-232 by KG6YY, Peter Ellena         1/2/92

-------------------------------------------------------------

Configuring TRTY for baudot on the PK-232:

If you have connected your PK-232 to Com-1 and

have established contact with your PK-232 using TRTY as

explained elsewhere in this document.

1) Log On Message:

#AEA PK-232M Data Controller

Copyright (C) 1986, 1987, 1988, 1989 by

Advanced Electronic Applications, Inc.

Release 31.OCT.89

Checksum $E3

2) Display MORSE/RTTY/AMTOR parameters:    (cmd:disp r)

cmd: disp r RTN PK232 responds with important baudot

parameters:  (some of these parameters have been changed to

accommodate TRTY.  Others are in their default status.)

ABAUD    110

ACRRTTY  0

ADELAY   4 (40 msec.)

ALFRTTY  OFF

ARQTMO   60 (60 sec.)

BITINV   $00

CCITT    ON

CODE     0 (International)

CRADD    OFF

DIDDLE   OFF

EAS      ON

MARSDISP OFF

MSPEED   20

MWEIGHT  10

NAVMSG   all

NAVSTN   all

RBAUD    45

RFEC     ON

RXREV    OFF

TDBAUD   96

TDCHAN   0

TXREV    OFF

USOS     OFF

WIDESHFT OFF

WORDOUT  ON

From the cmd: prompt type BAdot

Your  SELCAL,  MYCALL  AND  ALTCAL also should  be installed if you

plan AMTOR or MORSE.

If  you  plan  to  use  AMTOR and Packet with TRTY as your primary

software, you will need to make additional changes to some default

parameters in the PK-232.  These are covered in the PK-232 manual.

3)  Following is a copy of those strings used in session 1 for

baudot.  These strings of course can be put in any Session.

Strings other than O,  P  &  R (menu 8;  A, B & C) are shown as

examples.

( 2 )

4) TRTY.CFG File

STUPSTR "O"

EOTSTR "P"

WRUSTR "R"

SENSITIVE 4  10

SQUELCH

ALTERNATE

BELLMIND

STRING(A) "ZCZC\m\j\zZA\c\g"         (Strings A thru K are strings I

STRING(B) "ZCZC\m\jDE \c\m\j\zZB\c"   use.  There are numerous others

STRING(C) "\zZC\m\j\zZC\m\j"          that can be written and placed

STRING(D) "ZCZC\m\j\z DE \c\m\j\zZD\c\g"  in any locations you desire)

STRING(E) "ZCZC\m\jDE \c\L"

STRING(F) " "

STRING(G) " "

STRING(H) "ZCZC\m\j\z DE YOURNAME \c\m\j     "

STRING(I) "\z DE \c\m\j\g"

STRING(J) "ZCZC\m\j\z DE \c\m\j    "

STRING(K) "\z DE \c\j\g"

STRING(L) " "

STRING(M) " "

STRING(N) "#r\m"<---The PK-232 sees this as CTL-CR RTN.

STRING(O) "#X\M\J\SV\m"<---(The PK-232 sees this as:

CTL-C CR LF send string V RTN

CTL-X at the keyboard sends this string)

STRING(P) "#"<--The PK-232 sees this as a CTL-D.  When you send a CTL-G

from the keyboard TRTY will respond with NNNN)

STRING(Q) " "

STRING(R) "\M\JZCZC \J    :::: \U\C\K\Y ::::\JNNNN"<---(personal

STRING(S) " "                                           WRU String)

STRING(T) " "

STRING(U) " "

STRING(V) "###################"<---(These are embedded CTL-O's, LTRS

STRING(W) " "                       in PK-232 Language.  Embed them

STRING(X) " "                       using the F6 in your trty.cfg)

STRING(Y) " "

STRING(Z) "ZCZC\M\JDE \C # # # \M\J\ZZW\M\J\G" <--This string is

merely a convenience string for calling someone's WRU.  It

contains three bells (embedded ctl-g).  Use bells sparingly, the

old men on the net take long naps.  Also note that when you embed

a CTL-C using TRTY it appears as a heart,  the CTL-D appears as

a diamond.  My word processor shows them all as a single dot.

( 3 )

5)  Strings  O,P  have  been set up to emulate the commands in the

TRTY program as closely as possible.  When configuring menu 8:

Menu 8 |   TRTY.CGF  |    My    |   Function        | From your

strings|     Name    |  strings |                   | keyboard

------------------------------------------------------------

A    |   STUPSTR   |    "O"   | (START UP STRING) | CTL-X

B    |   EOTSTR    |    "P"   | (EOT STRING)      | CTL-G

C    |   WRUSTR    |    "R"   | (WRU STRING)      | CTL-W

Some  operators  may  wish  to turn diddle on.  If so you will not

need \sv" in String O.  The \sv" portion  of  this  string  merely

calls  about  12  embedded Ctl-O's (letters) in string V, prior to

the  transmission  of  any  data  by  the  PK-232.   This  assures

synchronization  at  the receiving station before transmit data is

sent.  It may  also  open  any  hardware  auto-start  features  on

various T.U. equipment. Diddle's (BLANKS) are ignored by the newer

systems.

Strings  A  and  B  (menu  8)  are  required for the PK-232.  They

correspond to Ctl-X and Ctl-G at the TRTY  keyboard  respectively.

String C may be personalized, as I have shown above (String R), or

may  be left in default.  If you personalize this string make sure

it corresponds to your Relay and Log options (TRTY Menu 6 & 8).

6) Although you will initially set up TRTY dialog (menu 6) in full

duplex, in actual operation you will be in Receive while receiving

data  from the PK-232 and in Full Duplex when transmitting data to

the PK-232.  This was done in order  to  retain  the  basic  Ctl-X

Transmit  and  Ctl-G  NNNN Receive parameters of the TRTY program.

In order to change parameters in the PK-232 testing procedures,  I

have  included  String  N.  From the full duplex dialog (transmit)

run String N and a return.  The PK-232 cmd: prompt should reappear.

7)  The  ON-LINE  editing  feature  of  TRTY is only available in

Receive mode.  If you continue to type  into  the  pretype  buffer

after  hitting a CTL-X, which dumps previously typed data to the

PK-232, you will need to add your own carriage  returns  to  typed

text.   In  other  words  you  do  not  have the luxury of on line

editing while you are transmitting data.  Because of this  I  have

turned  the  PK-232  parameter WORDOUT ON.  This is limited to one

word but has not posed a real problem.