O R B S

Orbital Prediction Program

for

Earth  Orbiting Satellites

Version  1.4

USER REFERENCE

11/16/87

Ron Dunbar, W0PN

=========

O R B S

=========

INTRODUCTION:

ORBS is a menu-driven system which produces printed listings showing

an earth-orbiting satellite's azimuth, elevation, range as well as

other approprate data at specified intervals of time as seen from an

observer's location.  Perhaps the easiest way to give you an idea of

ORB's features is to start with the opening menu:

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

=                                                                   =

=                             O R B S                               =

=                                                                   =

=          ORBITAL PREDICTION PROGRAM WITH TIME WINDOWS             =

=                            8/22/1987                              =

=                                                  Ron Dunbar W0PN  =

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

1.  Generate orbital predictions.

2.  Update Elements.

3.  Modify beacon frequency file.

4.  Modify station call/location file.

5.  Exit to operating system.

Your choice..

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Let's investigate the menu items one at a time...

1.  GENERATE ORBITAL PREDICTIONS:

If you choose this function, the next screen you'll see will look

something like this:

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

This program prints tracking data for earth-orbiting satellites

SATELLITE SELECTION MENU

1.   oscar-9           ID = 12888   Set 1066    8/17/87

2.   oscar-10          ID = 14129   Set 305     8/7/87

3.   oscar-11          ID = 14781   Set 245     8/6/87

4.   oscar-12          ID = 16909   Set 55      7/28/87

5.   rs-5              ID = 12999   Set 426     8/13/87

6.   rs-7              ID = 13001   Set 337     8/17/87

7.   rs-10/11          ID = 18129   Set 60      8/17/87

8.   meteor 2-14       ID = 16735   Set 126     5/31/87

9.   meteor 2-15       ID = 17290   Set 87      8/16/87

10.  meteor 3-1        ID = 16191   Set 689     8/16/87

11.  noaa-9            ID = 15427   Set 186     8/7/87

12.  noaa-10           ID = 16969   Set 70      8/14/87

13.  mir               ID = 16609   Set 782     8/17/87

14.  salyut-7          ID = 13138   Set 732     8/17/87

15.  ajisai            ID = 16908   Set 50      8/8/87

SELECT SATELLITE?

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Now choose the satellite you're interested in, answer a few questions,

and the printed listing is produced in record speed, thanks to Turbo

BASIC's nimble compiled code and Tom Clark's QUICKSTEP algorithm

trickery (that's the infamous W3IWI, you know) which speeds up the

calculations when the satellite is below the horizon.

As you answer the questions mentioned above, you'll note that another

unique feature has been added; TIME WINDOWS.  You have the option of

ignoring all satellite passes outside a selected timeframe.. after all,

there's little value in spending time calculating and producing a

listing covering times when you can't be there to see/hear/work that

satellite, is there?

TIME WINDOWS will allow you to span 2400 UTC with your window time;

you can specify a window such as 2200 to 0400, and it will be handled

properly.

The TIME WINDOW feature is particularly useful for listing the 'good'

visual passes of MIR and COSMOS 1870, for instance, because you can

only see them during the period between local civil twilight and the

time when they are enveloped in the earth's shadow.

Now let's see what a typical listing looks like.  We'll use Oscar-9:

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

*** O R B S ***

OSCAR-9 Orbital Predictions                 Page # 1

for W0PN at Lat: 46.860  Lon: 92.459

Element set 1066 is  7.5 days old

Listing covers the period 237.0000 to 237.5000 or  12.00 hrs.

This listing run at 00:40:06 on 08-25-1987.

PARAMETER           REFERENCE           STARTING

---------           ---------           --------

Catalog number       12888

Epoch               87229.50723967      87237.00000000

Inclination          97.6439             [ No change ]

R. A. A. N.         248.5757            256.2849

Eccentricity        0.00029260           [ No change ]

Arg. Perigee         33.8666              7.4516

Mean Anomaly        326.2793            195.4792

Mean Motion         15.29943109         15.29988380

Drag Correction      3.021E-005          [ No change ]

Orbit number         32611               32726

S.M.A.,(km)          6854.0761           6853.9409

Perigee Ht, km        473.9106            473.7754

Apogee  Ht, km        477.9216            477.7863

Freq (MHz)           145.825  (F1)       435.025  (F2)

*************************************************************

Printing only the window between 0130:00 and 0500:00 UTC.

*************************************************************

U T C    AZ  EL  145.825  435.025  RANGE HEIGHT   LAT  LONG PHASE  F2 TRACK

hhmm:ss  deg deg  DPLR Hz  DPLR Hz    km    km     deg  deg  <256>   435.025

- - - Orbit # 32728 - - - Day # 87237 - - - Tuesday, 8/25/87 - - -

0226:00  263   1    +1320    +3938   2417   474   40.6 119.5   24    435.021

0228:00  285   2     +206     +613   2366   474   48.1 122.0   29    435.024

0230:00  306  -1     -972    -2901   2606   474   55.6 125.2   35    435.028

End of OSCAR-9 listing.

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

As you can see by the information between the lines of asterisks, we

specified a 'time window' of 0130 to 0500 UTC, which is late enough for

most of us here in the Midwest, and prior to that time, we're probably

eating supper.  You can further see (in the 5th printed line) that we

specified a 12 hour period, and only one pass was visible (and print-

ed) which occurred in that 12 hour period AND was within our specified

time window.  You will also note from the ELevation column that it was

a marginal pass, just barely peeking up over the horizon to the west of

this QTH.

Note that two 'beacon' frequencies were present for Oscar-9 in the

FREQS.ORB file.. 145.825 and 435.025 MHz.  The doppler associated with

each frequency is listed in the 4th and 5th columns of the printout...

the column heading shows the frequency the doppler is computed for.

The RANGE, HEIGHT, LAT and LONG all refer to the satellite.  The LAT

and LONG are the point on the earth over which the satellite is

currently positioned (the Sub-Satellite Point, or SSP).  The RANGE is

the distance from the observer in meters, and the HEIGHT is the

satellite's altitude above the earth in meters.  LAT and LONG are in

degrees, of course.

Note the rightmost column labeled "F2 TRACK".  The numbers in this

column represent the frequency you must transmit on to 'hit' the F2

TRACK frequency (435.025) as seen at the spacecraft.. this listed

frequency takes the doppler into consideration and is a handy feature

when you need to uplink at a specific frequency.  One of the last

questions you are asked before the program goes off to compute all

this stuff is whether you would like to manually enter an alternate

frequency for F2.  This lets you temporarily override the frequencies

contained in the FREQS.ORB default file.  (More on that later).

Ok, now you've run your listing, and you've just downloaded the AMSAT

Orbital Data Bulletin and captured this week's latest Kepler elements

into your STUFF.LOG file.  Use your favorite word processor in the NON-

DOCUMENT MODE (please) to edit STUFF.LOG and get rid of everything

except the elements.  You can leave in all the carefully constructed

headers, asterisks, etc., but clean out all the stuff not relating to

the Bulletins.

Be sure to check the bulletin for typos.. sometimes the ':' following

the keywords gets lost, or 'e' (as in  "1.687e-04") turns up missing.

The program will catch the first error, but not the second one.  It

can show up as subsurface apogee and perigee heights in the parameter

listing on the first page.. that's why that listing is there!  If you

come up with anything that looks weird, check and correct the ELEMENTS

file, then redo the update operation.

All bulletins should be combined into one file which MUST be named

ELEMENTS  (no .EXTension, please).  Now move that file into the same

directory that ORBS resides in (I call it \ORBS, for want of a better

name.. I have trouble remembering obscure names, birthdays, etc.).

If you spell ELEMENTS wrong or forget to move it into the \ORBS direc-

tory, you'll get a RUNTIME ERROR 53: which says you spelled ELEMENTS

wrong or forgot to move it to the \ORBS directory.. don't DO that!

(See HELPFUL HINTS at the end for a .BAT program to do the moving.)

2.  UPDATE ELEMENTS:

Now, we're ready to choose item (2) on the ORBS menu.

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

=                                                                   =

=               ORBITAL ELEMENT UPDATE PROGRAM                      =

=                                                                   =

=                          8/23/1987                                =

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Scanning input file... please wait.

Total number of satellites read from the ELEMENTS file = 16

Total number of satellites written to the KEPLER.ORB file = 16

The frequency data in FREQS.ORB will be inserted into the

records contained in KEPLER.ORB.

FREQS.ORB contains 5 records for input.

5 insertions completed.

End of update..

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Now, wasn't that a lot easier than typing in 14 numbers for 16 dif-

ferent satellites?  You didn't have to do anything but sit there and

wonder what the computer was doing (especially when it was messing

around with your hard disk).  You've just updated all the Keplerian

elements for all the satellites contained in the bulletin in about

one minute or less!

What actually happened was that (1) the program ORBUPDAT.TBC was called

by the menu program.  (2) The file ELEMENTS was read in, (3) the beacon

frequency data from the file FREQS.ORB was merged with the ELEMENTS

data, and (4) the combined data was written to the KEPLER.ORB file

which is used by the tracking program ORBP.TBC.

KEPLER.ORB is the source of the data that is displayed as the list of

satellites from which you choose when you do menu item (1), "Generate

orbital predictions".

Let's suppose that a new satellite has been placed in orbit, and it

contains a beacon or two which you want to monitor.  You're ready for

the next menu choice...

3.  MODIFY BEACON FREQUENCY FILE:

When you make this choice from the menu, you'll see this....

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

=                                                                   =

=                    Modify FREQS.ORB Entries                       =

=                                                                   =

=                            8/23/1987                              =

=                                                                   =

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Existing entries will be presented for possible modification, followed

by blank entry(s) to allow you to add to the list of satellite beacon

frequencies; the frequency specified can be any you wish to track; it

doesn't have to be strictly a beacon.  Make certain you enter the

satellite's CATALOG number accurately.. the record is keyed on it.

Satellite name:  Oscar-9

Catalog number:  12888

Frequency 1:  145.825

Frequency 2:  435.025

Accept, Delete, Modify this entry (A/d/m):

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Note the prompt line.. the 'A' is in UPPER CASE.  This is the

convention I use to indicate the DEFAULT you will get if you just hit

the ENTER key.  You may also type an 'a' if you wish to follow

directions.  If you Accept this entry, the next satellite contained in

the FREQS.ORB file will be presented in order, and so on until the

entries are exhausted.

If you choose to DELETE the entry, it goes away.. simple.  If you

choose to MODIFY it, BE VERY CAREFUL that you get the "Catalog number"

right!  You can misspell the name, but the catalog number is the key to

matching this data up with it's counterpart contained in the ELEMENTS

file.

If you choose MODIFY, the cursor moves to the field following

"Satellite name:" and waits for your action.  If you merely hit ENTER,

the data in that field remains unchanged and the cursor moves on to the

next field.

If you modify a field, any data remaining in the field beyond the cursor

is erased before the cursor advances to the next field.  When you ENTER

following the last field (enter '0' if you don't want one of the beacon

frequencies anymore), you are asked "Is data correct (Y/n):".  If you

made an error, you can redo it by answering 'n', otherwise, a 'y' or an

ENTER will accept the modifications. When you have responded to the last entry and hit the ENTER key to

accept the data, the following appears:

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

=                                                                   =

=                    Modify FREQS.ORB Entries                       =

=                                                                   =

=                            8/23/1987                              =

=                                                                   =

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Existing entries will be presented for possible modification, followed

by blank entry(s) to allow you to add to the list of satellite beacon

frequencies; the frequency specified can be any you wish to track.  It

does not have to be strictly a beacon.  Make certain you enter the

satellite's CATALOG number accurately.. the record is keyed on it.

Satellite name:

Catalog number:

Frequency 1:

Frequency 2:

Make an entry (y/N):

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

You have noticed, of course, that all the fields are empty, and the

prompt is asking whether you wish to make a new entry.  Remember to get

that "Catalog number" right!

I suppose that you'd like to get your call letters and location stuck

in here somewhere.. that brings us to the next main menu function..(4):

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

=                                                                   =

=                     Modify ORB.CFG Entries                        =

=                                                                   =

=                            8/24/1987                              =

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

The ORBS.CFG file contains 'observer specific' data used by the prediction

program.  The existing entry will be displayed for possible modification.

Latitude is entered as decimal degrees (nn.nnn); South latitudes are

preceeded by a '-' (-nn.nnn).  Longitude is entered as decimal degrees

West longitude.  Observer Altitude is entered in meters, not feet.

Minimum elevation is that elevation (in degrees) above which you wish the

printout to commence (usually 0 or -1).

Observer name/call:  W0PN

Observer latitude:  46.860

Observer W. longitude:  92.459

Observer altitude:  435

Min sat elevation (deg): -1

Accept or Modify this entry (A/m):

Most of the fields are pretty self-explanatory.  The Station latitude

and longitude must be in decimal degrees, not degrees, minutes, etc.

The longitude is specified as WEST longitude.. none of that '-179.999'

stuff allowed.  The altitude is in METERS, not feet or furlongs.

If you don't want to forever run listings with my call and coordinates

on them, I suggest you choose to MODIFY this entry.  When you have

completed your task (operates the same as the previous function), you

will receive this screen:

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

=                                                                   =

=                     Modify ORB.CFG Entries                        =

=                                                                   =

=                            8/24/1987                              =

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

The ORBS.CFG file contains 'observer specific' data used by the prediction

program.  The existing entry will be displayed for possible modification.

Latitude is entered as decimal degrees (nn.nnn); South latitudes are

preceeded by a '-' (-nn.nnn).  Longitude is entered as decimal degrees

West longitude.  Observer Altitude is entered in meters, not feet.

Minimum elevation is that elevation (in degrees) above which you wish the

printout to commence (usually 0 or -1).

Observer name/call:  W3IWII

Observer latitude:  66.871

Observer W. longitude:  74.103

Observer altitude:  250

Min sat elevation (deg): -1

Is data correct (Y/n):

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Again, you have a chance to correct any typos.

The only remaining main menu function is (5) Exit to operating system.

That one ALWAYS works exactly as it's supposed to, even after extensive

and rigorous testing conditions (read: exiting to fix another bug!).

Please forward your comments, suggestions and abuse to me via AMSAT

Telemail (RDUNBAR), or you can try to reach me where I now live (but I

don't expect to be here long.. hi!):

Ron Dunbar, W0PN         (Res) 218-729-6355

4696 Hwy 33 N.           (Bus) 218-720-2093

Saginaw, MN  55779

..and I thank you for your support!

Supplemental:

The ARChived file of which this document file is a part, contains

the following files:

ELEMENTS          ORBP    .TBC

FREQS   .ORB      ORBS    .CFG

KEPLER  .ORB      ORBS    .DOC

MODCFG  .TBC      ORBS    .EXE

MODFREQ .TBC      ORBUPDAT.TBC

ELEMENTS:

Contains a recent version of the AMSAT Orbital Data Bulletin which

is prepared weekly for AMSAT NA by Ralph Wallio, W0RPK.  This file is

normally captured directly from a bulletin board system (BBS).  ORBS

allows you to input this unedited data file directly, thereby

avoiding the formidable task of entering the data manually.  You can

also download current Bulletins by calling W0RPK's BBS at 515-961-

3325.  This service is also being provided by others; just ask

around, or leave a query on Ralph's BBS when you call in.

A sample of the first few lines of a typical Orbital Data Bulletin:

*******************************************************************

*      A M S A T   O R B I T A L   D A T A   B U L L E T I N      *

*                         ORBS227O  15AUG87                       *

*                  Prepared for AMSAT-NA by W0RPK                 *

*******************************************************************

Orbital Elements for OSCARs

SAT.   UO-9  AO-10  UO-11  FO-12  RS-5  RS-7  RS-10/11

SET#     62    305    245     55   425   335        52

NEW?      Y      Y      Y      N     N     Y         Y

Satellite: oscar-9                          |Yes, you can create a file

Catalog number: 12888                       |with your favorite word pro-

Epoch time:      87218.25756376             |cessor in NON-DOCUMENT mode

Element set:     1062                       |IF you take particular care

Inclination:       97.6454 deg              |to duplicate the KEYWORDS

RA of node:       237.0035 deg              |exactly as shown at left;

Eccentricity:    0.0003676                  |the upper-case letters MUST

Arg of perigee:    58.3920 deg              |be upper-case and the colons

Mean anomaly:     301.7665 deg              |MUST be positioned as shown.

Mean motion:   15.29888737 rev/day          |There MUST be at least one

Decay rate:      2.007e-05 rev/day^2        |space between the keyword

Epoch rev:           32439                  |and the value following.

|The words following the value

Satellite: oscar-10                         |are NOT required; i.e.; 'deg',

Catalog number: 14129                       |'rev/day', etc.

.....etc.................

FREQS.ORB:

Contains user defined beacon frequencies for selected satellites.

When you choose "Update Elements." from the ORBS menu, the ORBUPDAT

routine merges the data from the FREQS.ORB file with the data from

the ELEMENTS file to produce the file KEPLER.ORB, which is used

directly by the ORBS tracking program, ORBP.TBC.

As supplied, FREQS.ORB contains the following data:

"Oscar-9", 12888,145.825,435.025

"Oscar-10",14129,145.810,145.987

"Oscar-11",14781,145.826,435.026

"Oscar-12",16909,435.795,145.850

"rs-10/11",12999,145.857,145.907

Each line (or record) is terminated in a <CR/LF> to indicate the end

of the record.  You need not worry about constructing this file

yourself, because ORBS provides you with a menu selection which does

the hard work for you.

ORBS.CFG:

Contains observer specific data.  As supplied, ORB.CFG contains the

following:

"W0PN",46.860,92.459,435,-1

This entry is followed by a <CR/LF> to indicate the end of the record.

LATE NOTES & HELPFUL HINTS:

--------------------------

Julian search time:

When you're running a listing, you will notice a highlighted 6 digit

number spinning in the upper right corner of your screen.. the 6 digit

number is followed by a single '0' or '1' which appears to change

randomly.  The label to the left of the number says "Julian search

time:".  This is the instantaneous day of the year and fractional part

of the day that is being used in the determination of the satellite's

position.  The 0/1 following indicates whether the satellite is below

or above the observer's horizon.  It's just a 'fuzzy blanket' to let

you know the computer is actually doing something between printouts.

Sidereal Time table:

I neglected to mention that the sidereal time tables have been

eliminated because that figure is now calculated internally by ORBP.TBC.

Moving the ELEMENTS file from your communications subdirectory to the

ORBS subdirectory:

I use the following batch file to perform the relocation of the ELEMENTS

file from my communication subdirectory over to the ORBS subdirectory.

Note that the previous ELEMENTS file is renamed ELEMENTS.OLD, just in case

something is drastically wrong with the new data and I want to get the old

stuff back (simply a matter of deleting the defective ELEMENTS file and

renaming ELEMENTS.OLD to ELEMENTS).

= = = = = =

REM Move ELEMENTS to \ORBS directory from \PROCOMM

echo off

cd\procomm

if NOT EXIST ELEMENTS goto nofile

cd\orbs

del elements.old

ren elements elements.old

cd\procomm

copy elements \orbs\elements

del  elements

goto exit

REM

:nofile

echo No ELEMENTS file found.. update aborted.

pause

REM

:exit

echo on

= = = = = =