USERREF.NOS

Network Operating System

User Reference Manual

Phil Karn, KA9Q

and

Gerard van der Grinten, PA0GRI

1.  The NOS.EXE Program

The MS-DOS executable file  nos.exe  provides Internet (TCP/IP), NET/ROM and

AX.25 facilities. Because it has an internal multitasking operating system,

nos.exe can act simultaneously as a client, a server and a packet switch for

all three sets of protocols. That is, while a local user accesses remote

services, the system can also provide those same services to remote users

while also switching IP, NET/ROM and AX.25 packets and frames between other

client and server nodes.

The keyboard and display is used by the local operator to control both host

and gateway level functions, for which a number of commands are provided.

1.1.  Startup

When  nos.exe  is executed without arguments, it attempts to open the file

autoexec.nos  in the root directory of the current drive. If it exists, it is

read and executed as though its contents were typed on the console as

commands. This feature is useful for attaching communication interfaces,

configuring network addresses, and starting the various services. If  nos.exe

is invoked with an argument, it is taken to be the name of an alternate

startup file to be read instead of  autoexec.nos.

Four command-line options are accepted:

1.1.1.  -s

The -s option specifies the size of the socket array to be allocated within

nos. This limits the number of network connections that may exist

simultaneously; the default is 40.

1.1.2.  -d

The -d option allows the user to specify a "root"  directory for the

configuration and spool files; it defaults to the root directory of the

system.

1.1.3.  -m

The -m option tells nos how much RAM to grab from MS-DOS for its internal

heap. It defaults to 200K. On a 640K system, this is usually enough to run a

subshell and some smaller commands. If you don't need to run subcommands and

prefer to allocate as much memory as possible to nos, you can specify a large

number (e.g., 1000) to the -m option and nos will grab as much memory as it

can.

1.1.4.  -t

The -t option allows the user to trace command execution during the startup of

nos. It lets the commands read from autoexec.nos echo before they are

executed. This is a nice help if nos stops (hangs) during initialization.

2.  Console modes

The console may be in one of two modes: command mode and converse mode. In

command mode, the prompt net> is displayed and any of the commands described

in the next section may be entered. In converse mode, keyboard input is

processed according to the current session. Sessions come in eight types:

Telnet, FTP, AX25, NETROM, Ping, More, Hopcheck and Tip.

In a Telnet, AX25, NETROM, or Tip session, keyboard input is sent to the

remote system and any output from the remote system is displayed on the

console. In an FTP session, keyboard input is first examined to see if it is a

known local command; if so it is executed locally. If not, it is "passed

through" to the remote FTP server. (See the section titled FTP Subcommands).

In a Ping session the user may test the path to a remote site, and in a More

session, the user may examine a local file. A Hopcheck session is used to

trace the path taken by packets to reach a specified destination.

The keyboard also has cooked and raw states. In cooked state, input is

line-at-a-time; the user may use the line editing characters ^U, ^R and

backspace to erase the line, redisplay the line and erase the last character,

respectively. Hitting either return or line feed passes the complete line up

to the application. In raw mode, each character is immediately passed to the

application as it is typed. The keyboard is always in cooked state in command

mode. It is also cooked in converse mode on an AX25, FTP or NET/ROM session.

In a Telnet session it depends on whether the remote end has issued (and the

local end has  accepted) the Telnet WILL ECHO option. (See the echo command).

On the IBM-PC, the user may escape back to command mode by hitting the F10

key. On other systems, the user must enter the escape character, which is by

default control-] (hex 1d, ASCII GS). (Note that this is distinct from the

ASCII character of the same name). The escape character can be changed (see

the escape command).

In the IBM PC version, each session (including the command "session") has its

own screen. When a new session is created, the command display is saved in

memory and the screen is cleared. When the command escape key (usually F10) is

hit, the current session screen is saved and the command screen is restored.

When a session is resumed, its screen is restored exactly as it appeared when

it was last current.

3.  Commands

This section describes the commands recognized in command mode. These are

given in the following notation:

command

command literal_parameter

command subcommand <parameter>

command [<optional_parameter>]

command a|b

Many commands take subcommands or parameters, which may be optional or

required. In general, if a required subcommand or parameter is omitted, an

error message will summarize the available subcommands or required parameters.

(Giving a '?' in place of the subcommand will also generate the message. This

is useful when the command word alone is a valid command.) If a command takes

an optional value parameter, issuing the command without the parameter

generally displays the current value of the variable. (Exceptions to this rule

are noted in the individual command descriptions.)

Two or more parameters separated by vertical bar(s) denote a choice between

the specified values. Optional parameters are shown enclosed in [brackets],

and a parameter enclosed in <angle brackets> should be replaced with an actual

value or string. For example, the notation <hostid> denotes an actual host or

gateway, which may be specified in one of two ways: as a symbolic name listed

in the file /domain.txt, or as a numeric IP address in dotted decimal

notation, e.g., 44.0.0.1. If a domain name server is available on the network

and configured into nos, it will be queried if the name isn't found in

/domain.txt.

All commands and many subcommands may be abbreviated. You only need type

enough of a command's name to distinguish it from others that begin with the

same series of letters. Parameters, however, must be typed in full.

Certain FTP subcommands, e.g., put, get, dir, etc, are recognized only in

converse mode with the appropriate FTP session; they are not recognized in

command mode.

3.1.  <cr>

Entering a carriage return (empty line) while in command mode puts you in

converse mode with the current session. If there is no current session, nos

remains in command mode.

3.2.  !

An alias for the shell command.

3.3.  #

Commands starting with the hash mark (#) are ignored. This is mainly useful

for comments in the autoexec.nos file.

3.4.  abort [<session #>]

Abort a FTP get, put or dir operation in progress. If issued without an

argument, the current session is aborted. (This command works only on FTP

sessions.) When receiving a file, abort simply resets the data connection;

the next incoming data packet will generate a TCP RST (reset) response to

clear the remote server. When sending a file, abort sends a premature

end-of-file. Note that in both cases abort will leave a partial copy of the

file on the destination machine, which must be removed manually if it is

unwanted.

3.5.  arp

Display the Address Resolution Protocol table that maps IP addresses to their

subnet (link) addresses on subnetworks capable of broadcasting. For each IP

address entry the subnet type (e.g., Ethernet, AX.25), subnet address and time

to expiration is shown. If the link address is currently unknown, the number

of IP datagrams awaiting resolution is also shown.

3.5.1.   arp add <hostid> ethernet|ax25 <ethernet address>|<ax25_address>

Add a permanent entry to the table. It will not time out as will an

automatically-created entry, but must be removed with the ax25 drop command.

3.5.2.   arp publish <hostid> ethernet|ax25 <ethernet address>|<ax25_address>

The  arp publish  command is similar to the  arp add  command, but system

will also respond to any ARP request it sees on the network that seeks the

specified address. (Use this feature with great care.)

3.5.3.  arp drop <hostid> ax25|ethernet

Remove the specified entry from the ARP table.

3.5.4.  arp flush

Drop all automatically-created entries in the ARP table; permanent entries are

not affected.

3.5.5.  asystat

Display statistics on attached asynchronous communications interfaces (8250 or

16550A), if any. The display for each port consists of three lines. The first

line gives the port label and the configuration flags; these indicate whether

the port is a 16550A chip, the trigger character if any, and whether CTS flow

control is enabled. (The trigger character when received causes the driver to

signal upper layer software that data is ready; it is automatically set to the

appropriate frame end character for SLIP and NRS lines.)

The second line of the status display shows receiver (RX) event counts: the

total number of receive interrupts, received characters, receiver overruns

(lost characters) and the receiver high water mark. The high water mark is the

maximum number of characters ever read from the device during a single

interrupt. This is useful for monitoring system interrupt latency margins as

it shows how close the port hardware has come to overflowing due to the

inability of the CPU to respond to a receiver interrupt in time.  8250 chips

have no FIFO, so the high water mark cannot go higher than 2 before overruns

occur. The 16550A chip, however, has a 16-byte  receive FIFO which the

software programs to interrupt the CPU when the FIFO is one-quarter full. The

high water mark should typically be 4 or 5 when a 16550A is used; higher

values indicate that the CPU has at least once been slow to respond to a

receiver interrupt.

When the 16550A is used, a count of FIFO timeouts  is  also displayed on the

RX status line. These are generated automatically by the 16550A when three

character intervals go by with more than 0 but less than 4 characters in the

FIFO. Since the characters that make up a SLIP or NRS frame are normally sent

at full line speed, this count will usually be a lower bound on the number of

frames received on the port, as only the last fragment of a frame generally

results in a timeout (and then only when the frame is not a multiple of 4

bytes long.)

The third line shows transmit (TX) statistics, including a total count of

transmit interrupts, transmitted characters, the length of the transmit queue

in bytes, and the number of CTS interrupts (this latter value will be zero

unless CTS flow control has been enabled.)

3.6.  attach <hw type> ...

Configure and attach a hardware interface to the system. The details are

highly interface dependent. Drivers are available for the following hardware

types:

3.6.1.  asy

Standard PC asynchronous interface (com port) using the National 8250 or

16550A chip.

3.6.2.  drsi

N6TTO driver for the DRSI PCPA 8530 card.

3.6.3.  eagle

WA3CVG/NG6Q driver for the Eagle Computer card (Zilog 8530).

3.6.4.  hapn

KE3Z driver for the Hamilton Amateur Packet Network adapter board (Intel

8273).

3.6.5.  hs

Special "high speed" 8530 driver for the WA4DSY 56kb/s modem.

3.6.6.  packet

Driver for use with separate software "packet drivers" meeting the FTP

Software, Inc, Software Packet Driver specification.

3.6.7.  pc100

Driver for the PACCOMM PC-100 (Zilog 8530) card.

3.6.8.  scc

PE1CHL driver for generic 8530 cards.

Detailed instructions for each driver are in the section Attach Commands. An

easy way to obtain a summary of the parameters required for a given device is

to issue a partial attach command, e.g., attach packet. This produces a usage

message giving the complete command format.

3.7.  ax25 <subcommand>

Ax25 subcommands are:

3.7.1.  ax25 blimit [<val>]

Display or set the AX25 retransmission backoff limit. Normally each successive

AX25 retransmission is delayed by twice the value of the previous interval;

this is called binary exponential backoff. When the backoff reaches the blimit

setting it is held at that value, which defaults to 30. To prevent the

possibility of "congestion collapse" on a loaded channel, blimit should be set

at least as high as the number of stations sharing the channel.

3.7.2.  ax25 digipeat [on|off]

Display or set the digipeater enable flag.

3.7.3.  ax25 flush

Clear the AX.25 "heard" list (see ax25 heard).

3.7.4.  ax25 heard

Display the AX.25 "heard" list. For each interface that is configured to use

AX.25, a list of all callsigns heard through that interface is shown, along

with a count of the number of packets heard from each station and the

interval, in hr:min:sec format, since each station was last heard. The local

station always appears first in the listing; the packet count actually

reflects the number of packets transmitted. This entry is always present even

if no packets have been sent.

3.7.5.  ax25 irtt [<val>]

Display or set the initial value of smoothed round trip time to be used when a

new AX25 connection is created. The value is in milliseconds. The actual round

trip time will be learned by measurement once the connection has been

established.

3.7.6.  ax25 kick <axcb>

Force a retransmission on the specified AX.25 control block.

3.7.7.  ax25 maxframe [<val>]

Establish the maximum number of frames that will be allowed to remain

unacknowledged at one time on new AX.25 connections. This number cannot be

greater than 7.

3.7.8.  ax25 mycall [<call>]

Display or set the local AX.25 address. The standard format is used, e.g.,

KA9Q-0 or WB6RQN-5. This command must be given before any attach commands

using AX.25 mode are given.

3.7.9.  ax25 paclen [<val>]

Limit the size of I-fields on new AX.25 connections. If IP datagrams or

fragments larger than this are transmitted, they will be transparently

fragmented at the AX.25 level, sent as a series of I frames, and reassembled

back into a complete IP datagram or fragment at the other end of the link. To

have any effect on IP datagrams, this parameter should be less than or equal

to the MTU of the associated interface.

3.7.10.  ax25 pthresh [<val>]

Display or set the poll threshold to be used for new AX.25 Version 2

connections. The poll threshold controls retransmission behavior as follows.

If the oldest unacknowledged I-frame size is less than the poll threshold, it

will be sent with the poll (P) bit set if a timeout occurs. If the oldest

unacked I-frame size is equal to or greater than the threshold, then a RR or

RNR frame, as appropriate, with the poll bit set will be sent if a timeout

occurs.

The idea behind the poll threshold is that the extra time needed to send a

"small" I-frame instead of a supervisory frame when polling after a timeout is

small, and since there's a good chance the I-frame will have to be sent anyway

(i.e., if it were lost previously) then you might as well send it as the poll.

But if the I-frame is large, send a supervisory (RR/RNR) poll instead to

determine first if retransmitting the oldest unacknowledged I-frame is

necessary; the timeout might have been caused by a lost acknowledgement. This

is obviously a tradeoff, so experiment with the poll threshold setting. The

default is 128 bytes, one half the default value of paclen.

3.7.11.  ax25 reset <axcb>

Delete the AX.25 connection control block at the specified address.

3.7.12.  ax25 retry [<val>]

Limit the number of successive unsuccessful retransmission attempts on new

AX.25 connections. If this limit is exceeded, link re-establishment is

attempted. If this fails retry times, then the connection is abandoned and all

queued data is deleted.

3.7.13.  ax25 route

Display the AX.25 routing table that specifies the digipeaters to be used in

reaching a given station.

3.7.14.  ax25 route add <target> [digis ... ]

Add an entry to the AX.25 routing table. An automatic ax255 route add is

executed if digipeaters are specified in an AX25 connect command, or if a

connection is received from a remote station via digipeaters. Such automatic

routing table entries won't override locally created entries, however.

3.7.15.  ax25 route drop <target>

Drop an entry from the AX.25 routing table.

3.7.16.  ax25 status [<axcb>]

Without an argument, display a one-line summary of each AX.25 control block.

If the address of a particular control block is specified, the contents of

that control block are dumped in more detail. Note that the send queue units

are frames, while the receive queue units are bytes.

3.7.17.  ax25 t3 [<val>]

Display or set the AX.25 idle "keep alive" timer. Value is in milliseconds.

3.7.18.  ax25 version [1|2]

Display or set the version of the AX.25 protocol to  attempt to use on new

connections. The default is 1 (the version that does not use the poll/final

bits).

3.7.19.  ax25 window [<val>]

Set the number of bytes that can be pending on an AX.25 receive queue beyond

which I frames will be answered with RNR (Receiver Not Ready) responses. This

presently applies only to suspended interactive AX.25 sessions, since incoming

I-frames containing network (IP, NET/ROM) packets are always processed

immediately and are not placed on the receive queue. However, when an AX.25

connection carries both interactive and network packet traffic, an RNR

generated because of backlogged interactive traffic will also stop network

packet traffic from being sent.

3.8.  bbs

Enter the local bbs port (same as a telnet session to your own station).

3.9.  cd [<dirname>]

Change the current working directory, and display the new setting. Without an

argument, cd simply displays the current directory without change. The pwd

command is an alias for cd.

3.10.  close [<session #>]

Close the specified session; without an argument, close the current session.

On an AX.25 session, this command initiates a disconnect. On a FTP or Telnet

session, this command sends a FIN (i.e., initiates a close) on the session's

TCP connection. This is an alternative to asking the remote server to initiate

a close ("QUIT" to FTP, or the logout command appropriate for the remote

system in the case of Telnet). When either FTP or Telnet sees the incoming

half of a TCP connection close, it automatically responds by closing the

outgoing half of the connection. Close is more graceful than the reset

command, in that it is less likely to leave the remote TCP in a "half-open"

state.

3.11.  connect <iface> <callsign> [<digipeater> ... ]

Initiate a "vanilla" AX.25 session to the specified call sign using the

specified interface. Data sent on this session goes out in conventional AX.25

packets with no upper layer protocol. The de-facto presentation standard

format is used, in that each packet holds one line of text, terminated by a

carriage return. A single AX.25 connection may be used for

terminal-to-terminal, IP and NET/ROM traffic, with the three types of data

being automatically separated by their AX.25 Level 3 Protocol IDs.

Up to 7 optional digipeaters may be  given; note that the word  via  is NOT

needed. If digipeaters are specified, they are automatically added to the AX25

routing table as though the ax25 route add command had been given before

issuing the connect command.

3.12.  detach <iface>

Detach a previously attached interface from the system. All IP routing table

entries referring to this interface are deleted, and forwarding references by

any other interface to this interface are removed.

3.13.  dir [<dirname>]

List the contents of the specified directory on the console. If no argument is

given, the current directory is listed. Note that this command works by first

listing the directory into a temporary file, and then creating a more session

to display it. After this completes, the temporary file is deleted.

3.14.  disconnect [<session #>]

An alias for the close command (for  the  benefit  of  AX.25 users).

3.15.  domain addserver <hostid> [<hostid> ...]

Add one or more domain name server(s) to the  list  of  name servers.   If

the <hostid> is the ip address of this system the domain name server is

started to listen for queries from other hosts. Note that in the /autoexec.nos

file the ip address command should be before this command.

3.16.  domain dropserver <hostid> [<hostid> ...]

Remove one or more domain name server(s) from  the  list  of name  servers.

You are warned when you delete the last name server.

3.17.  domain listservers

List the currently configured  domain  name  servers,  along with  statistics

on  how many queries and replies have been exchanged with each one, response

times, etc.

3.18.  domain suffix [<domain suffix>]

Display or specify the default domain name suffix to be appended to a host

name when it contains no periods. For example, if the suffix is set to

ampr.org. and the user enters telnet ka9q, the domain resolver will attempt to

find ka9q.ampr.org. If the host name being sought contains one or more

periods, however, the default suffix is NOT applied; e.g., telnet foo.bar

would NOT be turned into foo.bar.ampr.org. Note that a trailing dot (.) is

required for the suffix.

3.19.  domain trace [on|off]

Display or set the flag controlling the  tracing  of  domain

server  requests  and responses. Trace messages will be seen

only if a domain name being sought is not found in the local

cache file, /#/#/#/d#d#d#do#o#o#om#m#m#ma#a#a#ai#i#i#in#n#n#n.#.#.#.t#t#t#tx#x#x#xt#t#t#t.

3.20.  domain debug [on|off]

Display or set the flag controlling the extended tracing  of

the domain software internals.

3.21.  domain loopback [on|off]

Display or set the flag controlling  the  internal  loopback

capability of the domain software. This is only usefull when

the local system is an domain server.

3.22.  domain load [<filename>]

Loads the file <filename> or if  <filename>  not  given  the

standard  /#/#/#/d#d#d#do#o#o#om#m#m#ma#a#a#ai#i#i#in#n#n#n.#.#.#.t#t#t#tx#x#x#xt#t#t#t  file  into memory as a fast database.

Note that if too many records are read into memory you  will

get  into  problems  with  memory shortage soner or later. A

good compromise I found is around 400 records. That is prob-

ably more stations as you can reach.

3.23.  domain bootp <server> [<filename>]

This is for "mountain top" stations where no disk  space  is

available  for  keeping  a#9  o#o#o#om#m#m#ma#a#a#ai#i#i#in#n#n#n.#.#.#.t#t#t#tx#x#x#xt#t#t#t  file.  It sends out a

request to <server> to send all records in his  <file>.  (by

default /domain.txt).  All records received will be added to

the memory database. No disk access for  a  domain  file  is

made.   The   <server>   address   needs   to  be  given  in

[xx.xx.xx.xx] format as the software assumes  it  could  not

resolve a symbolic name yet.

3.24.  domain nslookup <server> <type> <record>

A domain query is send  out  to  <server>  for  record  type <type> and name

<record>.  The <type> can be a standard type (A , CNAME, WKS) or an integer

value (1..255) where  255  is TYPE_ANY.  (all  types referencing <record>.

Record <record> is where is sought for. Note that the answer to  this  query

is NOT added to the database (domain.txt or memory based).

3.25.  domain retries value

Sets the number of domain  query  retries  for  a  paticular request.

Default  is  0.  This means after an request to a server is timed out no

further requests will be send out. If you can reach more than one server set

this parameter acord- ingly so that more than one server  can  be  queried

before giving up.

3.26.  domain timeout value

This sets a timeout value (1 to 255 seconds) to a  query  or domain  name

server.  This is not set for a already defined server but will be used for a

newly  defined  name  server.

Also the value is used for domain nslookups.  Note that name servers can have

(PC based) trouble finding  records  in  an large database. The default is set

to 30 seconds.

3.27.  domain save <filename>

This command  saves  all  records  (except  A  records  with 0.0.0.0)  onto

file  <filename>  from the memory data base.  Records are grouped  by  origin.

If  there  where  answered queries , this is a way to save them.

3.28.  echo [accept|refuse]

Display or set the flag controlling client Telnet's response to a remote WILL

ECHO offer.

The Telnet  presentation  protocol  specifies  that  in  the absence  of  a

negotiated agreement to the contrary, neither end echoes data received from

the other.  In  this  mode,  a Telnet  client  session  echoes  keyboard

input locally and nothing is actually sent until a carriage return  is  typed.

Local  line editing is also performed: backspace deletes the last character

typed, while  control-U  deletes  the  entire line.

When communicating from keyboard to  keyboard  the  standard

local  echo  mode  is used, so the setting of this parameter

has no effect.  However,  many  timesharing  systems  (e.g.,

UNIX)  prefer to do their own echoing of typed input.  (This

makes screen editors work right, among other  things).  Such

systems  send  a  Telnet  WILL  ECHO  offer immediately upon

receiving an incoming Telnet  connection  request.  If  e#e#e#ec#c#c#ch#h#h#ho#o#o#o

a#a#a#ac#c#c#cc#c#c#ce#e#e#ep#p#p#pt#t#t#t  is in effect, a client Telnet session will automati-

cally return a DO ECHO response. In this mode, local echoing

and  editing  is  turned  off  and  each  key stroke is sent

immediately (subject to  the  Nagle  tinygram  algorithm  in

TCP).   While  this mode is just fine across an Ethernet, it

is clearly inefficient and painful across  slow  paths  like

packet  radio  channels.  Specifying  e#e#e#ec#c#c#ch#h#h#ho#o#o#o  r#r#r#re#e#e#ef#f#f#fu#u#u#us#s#s#se#e#e#e causes an

incoming WILL ECHO offer to be answered with  a  DONT  ECHO;

the  client  Telnet  session remains in the local echo mode.

Sessions already in the remote  echo  mode  are  unaffected.

(Note:  Berkeley  Unix  has a bug in that it will still echo

input even after the client has refused the WILL ECHO offer.

To  get around this problem, enter the s#s#s#st#t#t#tt#t#t#ty#y#y#y -#-#-#-e#e#e#ec#c#c#ch#h#h#ho#o#o#o command to

the shell once you have logged in.)

3.29.  eol [unix|standard]

Display or set Telnet's end-of-line behavior when in  remote echo  mode.   In

standard  mode, each key is sent as-is. In unix mode, carriage returns are

translated  to  line  feeds.  This  command is not necessary with all UNIX

systems; use it only when you find that a particular system responds to line

feeds  but  not  carriage  returns.   Only SunOS release 3.2 seems to exhibit

this behavior; later releases are fixed.

3.30.  escape [<char>]

Display or set the current command-mode escape character  in hex.   (This

command  is not provided on the IBM-PC; on the PC, the escape char is always

F10.)

3.31.  etherstat

Display 3-Com Ethernet  controller  statistics  (if  config- ured).

3.32.  exit

Exit the nos program and return to MS-DOS (or CP/M).

3.33.  finger <user@hostid> [<user@hostid> ...]

Issue a network finger request for user u#u#u#us#s#s#se#e#e#er#r#r#r at host h#h#h#ho#o#o#os#s#s#st#t#t#ti#i#i#id#d#d#d.

This  creates  a  client  session  which may be interrupted,

resumed, reset, etc, just like a Telnet client session.

3.34.  ftp <hostid>

Open an FTP control channel to the specified remote host and

enter  converse mode on the new session.  Responses from the

remote server are displayed directly on the screen. See  the

chapter  F#F#F#FT#T#T#TP#P#P#P  S#S#S#Su#u#u#ub#b#b#bc#c#c#co#o#o#om#m#m#mm#m#m#ma#a#a#an#n#n#nd#d#d#ds#s#s#s  for  descriptions of the commands

available in an FTP session.

3.35.  help

Display a brief summary of top-level commands.

3.36.  hop check <hostid>

Initiate a h#h#h#ho#o#o#op#p#p#pc#c#c#ch#h#h#he#e#e#ec#c#c#ck#k#k#k session to the specified host. This uses

a  series  of  UDP  "probe"  packets  with increasing IP TTL

fields to determine the sequence of gateways in the path  to

the  specified destination. This function is patterned after

the UNIX t#t#t#tr#r#r#ra#a#a#ac#c#c#ce#e#e#er#r#r#ro#o#o#ou#u#u#ut#t#t#te#e#e#e facility.

ICMP message tracing should be turned off before  this  com-

mand is executed (see the i#i#i#ic#c#c#cm#m#m#mp#p#p#p t#t#t#tr#r#r#ra#a#a#ac#c#c#ce#e#e#e command).

3.37.  hop maxttl [<ttl>]

Display or set the maximum TTL value to be used in hop check sessions.  This

effectively bounds the radius of the search.

3.38.  hop maxwait [<time>]

Display or set the maximum interval, in seconds, that a hop- check  session

will wait for responses at each stage of the trace. The default is 5 seconds.

3.39.  hop queries [<count>]

Display or set the number of UDP probes that will be sent at each stage of the

trace. The default is 3.

3.40.  hop trace [on|off]

Display or set the flag that controls the display  of  addi- tional

information during a hop check session.

3.41.  hostname [<name>]

Display or set the local host's  name.  By  convention  this should  be  the

same as the host's primary domain name. This string is used only in the

greeting messages of the  various network  servers;  note that it does NOT set

the system's IP address.

3.42.  hs

Display statistics about the HS high speed HDLC  driver  (if configured and

active).

3.43.  icmp echo [on|off]

Display or set the flag controlling the asynchronous display

of  ICMP  Echo Reply packets.  This flag must be on for one-

shot pings to work (see the p#p#p#pi#i#i#in#n#n#ng#g#g#g command.)

3.44.  icmp status

Display statistics about the Internet Control Message Proto- col  (ICMP),

including  the number of ICMP messages of each type sent or received.

3.45.  icmp trace [on|off]

Display or set the flag  controlling  the  display  of  ICMP

error  messages.  These informational messages are generated

by Internet routers in  response  to  routing,  protocol  or

congestion problems. This option should be turned off before

using the h#h#h#ho#o#o#op#p#p#pc#c#c#ch#h#h#he#e#e#ec#c#c#ck#k#k#k facility because it relies on  ICMP  Time

Exceeded  messages,  and  the  asynchronous display of these

messages will be mingled with hop check command output.

3.46.  ip address [<hostid>]

Display or set the default local IP  address.  This  command must  be  given

before an attach command if it is to be used as the default IP address for the

interface.

3.47.  ip rtimer [<val>]

Display or set the IP reassembly timeout. The default is  30 seconds.

3.48.  ip status

Display Internet Protocol (IP)  statistics,  such  as  total packet counts and

error counters of various types.

3.49.  ip ttl [<val>]

Display or set the default time-to-live value placed in each outgoing  IP

datagram. This limits the number of switch hops the datagram will be allowed

to take. The idea is  to  bound the  lifetime  of  the  packet  should it

become caught in a routing loop, so make the value  somewhat  larger  than

the diameter of the network.

3.50.  log [stop|<file>]

Display or set the file name for logging server sessions. If

s#s#s#st#t#t#to#o#o#op#p#p#p  is  given  as the argument, logging is terminated (the

servers themselves are unaffected). If a file name is  given

as  an argument, server session log entries will be appended

to it.

3.51.  mbox

Display the status of the mailbox server system (if  config- ured).

3.52.  memory free

Display the storage allocator free list. Each entry consists of a starting

address, in hex, and a size, in decimal bytes.

3.53.  memory sizes

Display a histogram of storage allocator request sizes. Each histogram bin is

a binary order of magnitude (i.e., a factor of 2).

3.54.  memory status

Display a summary of storage allocator statistics. The first line shows the

base address of the heap, its total size, the amount of heap memory available

in bytes and as a percentage of  the  total heap size, and the amount of

memory left over (i.e., not placed on the  heap  at  startup)  and  therefore

available for shell subcommands.

The second line shows the total number of calls to  allocate and  free  blocks

of  memory,  the  difference of these two values (i.e., the number of

allocated  blocks  outstanding), the  number  of  allocation requests that

were denied due to lack of memory, and the  number  of  calls  to  free()

that attempted  to  free garbage, e.g., by freeing the same block twice or

freeing a garbled pointer.

The third line shows the number of calls to malloc and  free that  occurred

with  interrupts  off.  In normal situations these values should be zero.  The

fourth line shows  statis- tics  for  the  special  pool  of fixed-size

buffers used to satisfy requests for memory at interrupt time. The variables

shown are the number of buffers currently in the pool, their size, and the

number of requests that failed due to  exhaus- tion of the pool.

3.55.  mode <iface> [vc|datagram]

Control the default transmission mode on the specified AX.25

interface.  In d#d#d#da#a#a#at#t#t#ta#a#a#ag#g#g#gr#r#r#ra#a#a#am#m#m#m mode, IP packets are encapsulated in

AX.25 UI frames and transmitted without any other link level

mechanisms, such as connections or acknowledgements.

In v#v#v#vc#c#c#c (virtual circuit) mode, IP packets are encapsulated in

AX.25  I  frames  and  are  acknowledged  at  the link level

according to the AX.25 protocol.  Link level connections are

opened if necessary.

In both modes, ARP is used to map  IP  to  AX.25  addresses.  The  defaults

can  be  overridden  with the type-of-service (TOS) bits in the IP header.

Turning  on  the  "reliability" bit  causes  I  frames to be used, while

turning on the "low delay" bit uses UI frames.  (The effect of turning  on

both bits is undefined and subject to change).

In  both  modes,  IP-level  fragmentation  is  done  if  the

datagram is larger than the interface  MTU.  In virtual cir-

cuit mode, however, the resulting datagram (or fragments) is

further  fragmented  at  the AX.25 layer if it (or they) are

still larger than the AX.25 p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n parameter. In AX.25 frag-

mentation,  datagrams  are  broken into several I frames and

reassembled at the receiving end before being passed to  IP.  This  is

preferable  to  IP fragmentation whenever possible because of decreased

overhead (the IP header isn't  repeated in  each fragment) and increased

robustness (a lost fragment is immediately retransmitted by the link layer).

3.56.  more <file> [<file> ...]

Display the specified file(s) a screen at a time. To proceed

to  the  next  screen,  press  the  space bar; to cancel the

display, hit the 'q' key.  The m#m#m#mo#o#o#or#r#r#re#e#e#e command creates  a  ses-

sion  that  you  can  suspend and resume just like any other

session.

3.57.  nntp <subcommand>

Network news transfer protocol  has  the  following  subcom- mands:

3.57.1.  nntp addserver  <nntpserver>  <interval>  [<range>]

[<groups>]

Add a nntp news server to quiry every interval for new arti- cles  specified

by  the nntp groups command. Range can be a limit in time to query like nntp

addserver  someserver  600 22:00-23:00  To only query between 10 and 11pm

Multiple nntp add can be used co concatenate groups (up to 512 bytes)

3.57.2.  nntp directory <directory>

Show or set default directory for spooling  news.   This  is not the control

directory but the (alternate) place to spool articles to.

3.57.3.  nntp dropserver <nntpserver>

3.57.4.  nntp groups <group> [<group> ...]

Set or display currently set newsgroups.

3.57.5.  nntp  kick <nntpserver>

Kick the client to get in touch with the named server.

3.57.6.  nntp listservers

Lists the curently defined servers.

3.58.  nntp trace <level>

Sets or shows the current trace level for the  nntp  client.  0  No  tracing

1  (default)  displays  serious  errors.   2 displays as 1 and transient

errors.  3  displays  as  2  and session  progress.   4   displays  as  3 and

actual received articles.  5  displays errors.

3.59.  param <iface> [<param> ...]

Invoke a device-specific control routine.   On  a  KISS  TNC

interface,  this  sends  control  packets  to the TNC.  Data

bytes are treated as decimal.  For example, p#p#p#pa#a#a#ar#r#r#ra#a#a#am#m#m#m a#a#a#ax#x#x#x0#0#0#0 1#1#1#1  2#2#2#25#5#5#55#5#5#5

will  set  the  keyup timer (type field = 1) on the KISS TNC

configured as ax0 to 2.55 seconds (255 x  .01  sec).   On  a SLIP interface,

the param command allows the baud rate to be read (without arguments) or set.

The implementation of this command  for the various interface drivers is

incomplete and subject to change.

3.60.  ping <hostid> [<len> [<interval> [<incflag>]]]

Ping (send ICMP Echo Request packets to) the specified host.

By default the data field contains only a small timestamp to

aid in determining round trip time; if the  optional  l#l#l#le#e#e#en#n#n#ng#g#g#gt#t#t#th#h#h#h

argument  is  given,  the  appropriate  number of data bytes

(consisting of hex 55) are added to the ping packets.

If i#i#i#in#n#n#nt#t#t#te#e#e#er#r#r#rv#v#v#va#a#a#al#l#l#l is specified, pings will  be  repeated  indefin-

itely  at  the  specified  interval, in seconds; otherwise a

single, "one shot" ping  is  done.   Responses  to  one-shot pings  appear

asynchronously  on  the command screen, while repeated pings create a session

that may  be  suspended  and resumed.   Pinging  continues  until the session

is manually reset.

The i#i#i#in#n#n#nc#c#c#cf#f#f#fl#l#l#la#a#a#ag#g#g#g option causes a repeated ping to  increment  the

target  IP  address  for  each  ping;  it is an experimental

feature for searching blocks  of  IP  addresses  for  active hosts.

3.61.  pop mailbox [<name>]

Show or sets the name to <name> for POP mail queries to  the mailhost.

3.62.  pop mailhost [<name>]

Show or sets the IP address to the system named  <name>  for POP connections.

3.63.  pop kick

Start a POP connection now. (one shot connection)

3.64.  pop timer [<seconds>]

Show or start a POP connection every <seconds>  to  transfer mail.

3.65.  pop userdata <name> <password>

Sets up the userdata used for validation on the  POP  server system.  Note

that <name> and <password> are case sensitive.  When only pop userdata is

entered to show  the  values  only the  <name>  is  shown.  (ultimate in

security, if you realy want to see the password look in the "ether").

3.66.  ps

Display all current processes in the system. The fields  are as follows:

P#P#P#PI#I#I#ID#D#D#D - Process ID (the address of the process descriptor).

S#S#S#SP#P#P#P - The current value of the process stack pointer.

s#s#s#st#t#t#tk#k#k#ks#s#s#si#i#i#iz#z#z#ze#e#e#e - The size of the stack allocated to the process.

m#m#m#ma#a#a#ax#x#x#xs#s#s#st#t#t#tk#k#k#k - The apparent peak stack utilization  of  this  pro-

cess.  This  is done in a somewhat heuristic fashion, so the

numbers should be treated as  approximate.  If  this  number reaches  or

exceeds the stksize figure, the system is almost certain to crash; the nos

program should  be  recompiled  to give the process a larger allocation when

it is started.

e#e#e#ev#v#v#ve#e#e#en#n#n#nt#t#t#t - The event this task is waiting for,  if  it  is  not

runnable.

f#f#f#fl#l#l#l - Process status flags. There are  three:  I  (Interrupts

enabled),  W  (Waiting  for  event)  and  S (suspended - not

currently used). The I flag is set whenever a task has  exe-

cuted  a  pwait()  call (wait for event) without first disa-

bling hardware interrupts. Only tasks that wait for hardware

interrupt  events  will  turn off this flag; this is done to

avoid critical sections and missed interrupts.  The  W  flag

indicates  that  the  process  is  waiting for an event; the

e#e#e#ev#v#v#ve#e#e#en#n#n#nt#t#t#t column will be non-blank. Note that although there may

be  several  runnable processes at any time (shown in the p#p#p#ps#s#s#s

listing as those without the W flag  and  with  blank  event

fields)  only  one  process  is  actually running at any one

instant (The Refrigerator Light Effect says that the p#p#p#ps#s#s#s com-

mand  is  always  the  one running when this display is gen-

erated.)

3.67.  pwd [<dirname>]

An alias for the c#c#c#cd#d#d#d command.

3.68.  record [<filename>|off]

Append to f#f#f#fi#i#i#il#l#l#le#e#e#en#n#n#na#a#a#am#m#m#me#e#e#e all data received on the current session.

Data  sent  on  the current session is also written into the

file except for Telnet sessions in remote  echo  mode.   The

command r#r#r#re#e#e#ec#c#c#co#o#o#or#r#r#rd#d#d#d o#o#o#of#f#f#ff#f#f#f stops recording and closes the file.

3.69.  remote [-p <port>] [-k <key>] [-a  <kickaddr>]  <hos-

tid> exit|reset|kick

Send a UDP packet to the specified  host  commanding  it  to

exit  the  nos  program,  reset  the  processor,  or force a

retransmission on TCP connections.  For this command  to  be

accepted,  the  remote  system  must  be  running the r#r#r#re#e#e#em#m#m#mo#o#o#ot#t#t#te#e#e#e

server and the port number specified in the  remote  command

must match the port number given when the server was started on the remote

system.  If the port numbers do not match,  or if  the  remote  server is not

running on the target system, the command packet is  ignored.   Even  if  the

command  is accepted there is no acknowledgement.

The kick command forces a retransmission timeout on all  TCP connections  that

the  remote  node may have with the local node.  If the -a option is used,

connections to  the  speci- fied  host  are  kicked  instead. No key is

required for the kick subcommand.

The exit and reset subcommands are mainly  useful  for  res-

tarting  the nos program on a remote unattended system after

the configuration file has been updated.  The remote  system

should  invoke  the  nos program automatically upon booting,

preferably in an infinite loop.  For example,  under  MS-DOS

the boot disk should contain the following in a#a#a#au#u#u#ut#t#t#to#o#o#oe#e#e#ex#x#x#xe#e#e#ec#c#c#c.#.#.#.n#n#n#no#o#o#os#s#s#s:

:loop

nos

goto :loop

3.70.  -s <key>

The exit and reset subcommands of remote require a password.

The  password  is  set on a given system with the -#-#-#-s#s#s#s option,

and it is specified in a command to a remote system with the

-#-#-#-k#k#k#k  option.  If  no password is set with the -#-#-#-s#s#s#s option, then

the exit and reset subcommands are disabled.

Note that r#r#r#re#e#e#em#m#m#mo#o#o#ot#t#t#te#e#e#e is an experimental feature in  NOS;  it  is

not yet supported by any other TCP/IP implementation.

3.71.  rename <file1> <file2>

Rename f#f#f#fi#i#i#il#l#l#le#e#e#e1#1#1#1 to f#f#f#fi#i#i#il#l#l#le#e#e#e2#2#2#2.

3.72.  reset [<session>]

Reset the specified session; if no argument is given,  reset the  current

session.   This  command  should  be used with caution since it does not

reliably  inform  the  remote  end that the connection no longer exists.  (In

TCP a reset (RST) message will be automatically generated  should  the  remote

TCP  send  anything  after  a local reset has been done.  In AX.25 the DM

message performs a similar role.  Both are used to  get  rid  of  a  lingering

half-open connection after a remote system has crashed.)

3.73.  rip accept <gateway>

Remove the specified gateway  from  the  RIP  filter  table, allowing future

broadcasts from that gateway to be accepted.

3.74.  rip add <hostid> <interval> [<flags>]

Add an entry to the RIP  broadcast  table.  The  IP  routing

table  will  be  sent  to  h#h#h#ho#o#o#os#s#s#st#t#t#ti#i#i#id#d#d#d every i#i#i#in#n#n#nt#t#t#te#e#e#er#r#r#rv#v#v#va#a#a#al#l#l#l seconds. If

f#f#f#fl#l#l#la#a#a#ag#g#g#gs#s#s#s is specified as 1,  then  "split  horizon"  processing

will  be  performed  for  this  destination. That is, any IP

routing table entries pointing to the interface that will be used  to  send

this update will be removed from the update.  If split horizon processing is

not specified, then all rout- ing table entries except those marked "private"

will be sent in each update.  (Private entries  are  never  sent  in  RIP

packets).

Triggered updates are always done. That is,  any  change  in the  routing

table that causes a previously reachable desti- nation to become unreachable

will  trigger  an  update  that advertises  the  destination with metric 15,

defined to mean "infinity".

Note that for RIP packets to be sent properly to a broadcast

address,  there  must exist correct IP routing and ARP table

entries that will first steer the broadcast to  the  correct

interface  and  then  place the correct link-level broadcast

address in the link-level destination field. If  a  standard

IP broadcast address convention is used (e.g., 128.96.0.0 or

128.96.255.255) then chances are you already have the neces-

sary  IP routing table entry, but unusual subnet or cluster-

addressed networks may require special attention.   However,

an  a#a#a#ar#r#r#rp#p#p#p  a#a#a#ad#d#d#dd#d#d#d  command  will  be  required  to translate this

address to the appropriate  link  level  broadcast  address,

e.g.,

arp add 128.96.0.0 ethernet ff:ff:ff:ff:ff:ff

for an Ethernet network, and

arp add 44.255.255.255 ax25 qst-0

for an AX25 packet radio channel.

3.75.  rip drop <dest>

Remove an entry from the RIP broadcast table.

3.76.  rip merge [on|off]

This flag controls an experimental feature for consolidating redundant

entries in the IP routing table. When rip merging is enabled, the table is

scanned after processing  each  RIP update. An entry is considered redundant

if the target(s) it covers would be routed  identically  by  a  less

"specific" entry  already in the table. That is, the target address(es)

specified by the entry in question must also match the  tar- get addresses of

the less specific entry and the two entries must have the same interface and

gateway fields.  For  exam- ple, if the routing table contains

Dest            Len Interface    Gateway          Metric  P Timer  Use

1.2.3.4         32  ethernet0    128.96.1.2       1       0 0      0

1.2.3           24  ethernet0    128.96.1.2       1       0 0      0

then the first entry would be  deleted  as  redundant  since packets  sent  to

1.2.3.4 will still be routed correctly by the second entry. Note that  the

relative  metrics  of  the entries are ignored.

3.77.  rip refuse <gateway>

Refuse to accept RIP updates from the specified  gateway  by

adding  the gateway to the RIP filter table. It may be later

removed with the r#r#r#ri#i#i#ip#p#p#p a#a#a#ac#c#c#cc#c#c#ce#e#e#ep#p#p#pt#t#t#t command.

3.78.  rip request <gateway>

Send a RIP Request packet to the specified gateway,  causing it  to reply with

a RIP Response packet containing its rout- ing table.

3.79.  rip status

Display RIP status, including a count of the number of pack- ets sent and

received, the number of requests and responses, the number of unknown RIP

packet types, and  the  number  of refused  RIP  updates from hosts in the

filter table. A list of the addresses and intervals to which periodic RIP

updates are being sent is also shown, along with the contents of the filter

table.

3.80.  rip trace [0|1|2]

This variable controls the tracing of incoming and  outgoing RIP  packets.

Setting  it to 0 disables all RIP tracing. A value of 1  causes  changes  in

the  routing  table  to  be displayed, while packets that cause no changes

cause no out- put. Setting the variable  to  2  produces  maximum  output,

including tracing of RIP packets that cause no change in the routing table.

3.81.  rlogin host

Sets up an rlogin session via port 511 to an *NIX compatible station.   Defaut

terminal  is a vt100 (as defined with the fkeys) compatible terminal.

3.82.  route

With no arguments, r#r#r#ro#o#o#ou#u#u#ut#t#t#te#e#e#e displays the IP routing table.

3.83.   route   add   <dest_hostid>[/bits]|default   <iface>

[<gateway_hostid> [<metric>]]

This command adds an entry to the routing table. It requires

at  least two more arguments, the host_id of the target des-

tination and the name of the interface to which its  packets

should  be  sent.   If  the  destination  is  not local, the

gateway's host_id should also be specified. (If  the  inter-

face  is  a  point-to-point link, then g#g#g#ga#a#a#at#t#t#te#e#e#ew#w#w#wa#a#a#ay#y#y#y_h#h#h#ho#o#o#os#s#s#st#t#t#ti#i#i#id#d#d#d may be

omitted even if the target is non-local because  this  field

is  only used to determine the gateway's link level address,

if  any.   If  the  destination   is   directly   reachable,

g#g#g#ga#a#a#at#t#t#te#e#e#ew#w#w#wa#a#a#ay#y#y#y_h#h#h#ho#o#o#os#s#s#st#t#t#ti#i#i#id#d#d#d  is  also  unnecessary  since the destination

address is used to determine the interface link address).

The optional /#/#/#/b#b#b#bi#i#i#it#t#t#ts#s#s#s suffix to the destination host id  speci-

fies  how  many  leading  bits in the host id are to be con-

sidered significant in  the  routing  comparisons.   If  not

specified,  32  bits  (i.e.,  full significance) is assumed.  With this

option, a single routing table entry may refer  to many  hosts  all sharing a

common bit string prefix in their IP addresses. For example, ARPA Class A, B

and  C  networks would use suffixes of /8, /16 and /24 respectively; the com-

mand

route add 44/8 sl0 44.64.0.2

causes any IP addresses beginning with "44" in the  first  8 bits  to  be

routed to 44.64.0.2; the remaining 24 bits are "don't-cares".

When an IP address to be routed matches more than one  entry

in  the routing table, the entry with largest b#b#b#bi#i#i#it#t#t#ts#s#s#s parameter

(i.e., the "best" match) is  used.  This  allows  individual

hosts  or blocks of hosts to be exceptions to a more general rule for a larger

block of hosts.

The special destination d#d#d#de#e#e#ef#f#f#fa#a#a#au#u#u#ul#l#l#lt#t#t#t is used to  route  datagrams

to addresses not matched by any other entries in the routing

table; it is equivalent to specifying a /#/#/#/b#b#b#bi#i#i#it#t#t#ts#s#s#s suffix  of  /0

to  any destination hostid.  Care must be taken with default

entries since two nodes with  default  entries  pointing  at each  other will

route packets to unknown addresses back and forth in  a  loop  until  their

time-to-live  (TTL)  fields expire.   (Routing  loops for specific addresses

can also be created, but this is less likely to occur accidentally).

Here are some examples of the route command:

# Route datagrams to IP address 44.0.0.3 to SLIP line #0.

# No gateway is needed because SLIP is point-to point.

route add 44.0.0.3 sl0

# Route all default traffic to the gateway on the local Ethernet

# with IP address 44.0.0.1

route add default ec0 44.0.0.1

# The local Ethernet has an ARPA Class-C address assignment;

# route all IP addresses beginning with 192.4.8 to it

route add 192.4.8/24 ec0

# The station with IP address 44.0.0.10 is on the local AX.25 channel

route add 44.0.0.10 ax0

3.84.  route addprivate <dest hostid>[/bits]|default <iface>

[<gateway hostid> [<metric>]]

This command is identical to r#r#r#ro#o#o#ou#u#u#ut#t#t#te#e#e#e a#a#a#ad#d#d#dd#d#d#d except that  it  also

marks the new entry as private; it will never be included in

outgoing RIP updates.

_#3._#8_#5.  _#r_#o_#u_#t_#e _#d_#r_#o_#p <_#d_#e_#s_#t _#h_#o_#s_#t_#i_#d>

r#r#r#ro#o#o#ou#u#u#ut#t#t#te#e#e#e d#d#d#dr#r#r#ro#o#o#op#p#p#p deletes an entry from  the  table.  If  a  packet

arrives  for  the  deleted address and a default route is in

effect, it will be used.

_#3._#8_#6.  _#s_#e_#s_#s_#i_#o_#n [<_#s_#e_#s_#s_#i_#o_#n #>]

Without arguments, displays the list  of  current  sessions, including

session  number,  remote TCP or AX.25 address and the address of the TCP or

AX.25 control block.  An  asterisk (*)  is  shown next to the current session;

entering a blank line at this point puts you in converse mode with that  ses-

sion.   Entering a session number as an argument to the ses- sion command

will  put  you  in  converse  mode  with  that session.  If the Telnet server

is enabled, the user is noti- fied of an incoming request and a session number

is automat- ically  assigned.  The user may then select the session nor- mally

to converse with the remote user as though the session had been locally

initiated.

_#3._#8_#7.  _#s_#h_#e_#l_#l

Suspends _#n_#o_#s and executes a sub shell  ("command  processor"

under MS-DOS).  When the sub shell exits, _#n_#o_#s resumes (under

MS-DOS, enter the e#e#e#ex#x#x#xi#i#i#it#t#t#t command).  Background  activity  (FTP

servers, etc) is also suspended while the subshell executes.

Note that this will fail unless there is  sufficient  unused

memory  for the subshell and whatever command the user tries

to run (see the discussion of -#-#-#-f#f#f#f option to n#n#n#no#o#o#os#s#s#s.#.#.#.e#e#e#ex#x#x#xe#e#e#e).

_#3._#8_#8.  _#s_#m_#t_#p _#g_#a_#t_#e_#w_#a_#y [<_#h_#o_#s_#t_#i_#d>]

Displays or sets the host to  be  used  as  a  "smart"  mail relay.  Any  mail

sent to a host not in the host table will instead be sent to the gateway for

forwarding.

_#3._#8_#9.  _#s_#m_#t_#p _#k_#i_#c_#k

Run through the outgoing mail queue and attempt  to  deliver

any pending mail.  This command is periodically invoked by a

timer whenever _#n_#o_#s is running; this command allows the  user

to "kick" the mail system manually.

_#3._#9_#0.  _#s_#m_#t_#p _#m_#a_#x_#c_#l_#i_#e_#n_#t_#s [<_#v_#a_#l>]

Displays or sets the maximum number of simultaneous outgoing SMTP  sessions

that  will  be  allowed.  The default is 10; reduce it if network congestion

is a problem.

_#3._#9_#1.  _#s_#m_#t_#p _#t_#i_#m_#e_#r [<_#v_#a_#l>]

Displays or sets the interval, in seconds, between scans  of

the  outbound  mail  queue. For example, s#s#s#sm#m#m#mt#t#t#tp#p#p#p t#t#t#ti#i#i#im#m#m#me#e#e#er#r#r#r 6#6#6#60#0#0#00#0#0#0 will

cause the system to check for outgoing mail every 10 minutes

and  attempt to deliver anything it finds, subject of course

to the s#s#s#sm#m#m#mt#t#t#tp#p#p#p m#m#m#ma#a#a#ax#x#x#xc#c#c#cl#l#l#li#i#i#ie#e#e#en#n#n#nt#t#t#ts#s#s#s limit. Setting a value of  zero  dis-

ables  queue  scanning  altogether,  note  that  this is the

default!  This value is recommended for stand alone IP gate- ways that never

handle mail, since it saves wear and tear on the floppy disk drive.

_#3._#9_#2.  _#s_#m_#t_#p _#t_#r_#a_#c_#e [<_#v_#a_#l>]

Displays or sets the trace flag in the SMTP client, allowing you to watch

SMTP's conversations as it delivers mail.  Zero (the default) disables

tracing.

_#3._#9_#3.  _#s_#o_#c_#k_#e_#t [<_#s_#o_#c_#k_#e_#t #>]

Without an argument, displays  all  active  sockets,  giving

their index and type, the address of the associated protocol

control block and the and owner process ID and name. If  the

index  to  an  active socket is supplied, the status display

for the appropriate protocol is called.  For example, if the

socket  refers to a TCP connection, the display will be that

given by the t#t#t#tc#c#c#cp#p#p#p s#s#s#st#t#t#ta#a#a#at#t#t#tu#u#u#us#s#s#s command with  the  protocol  control

block address.

_#3._#9_#4.                                                  _#s_#t_#a_#r_#t

_#a_#x_#2_#5|_#d_#i_#s_#c_#a_#r_#d|_#e_#c_#h_#o|_#f_#i_#n_#g_#e_#r|_#f_#t_#p|_#n_#e_#t_#r_#o_#m|_#r_#e_#m_#o_#t_#e|_#p_#o_#p|_#s_#m_#t_#p|_#t_#e_#l_#n_#e_#t|_#t_#t_#y_#l_#i_#n_#k

Start the specified Internet server, allowing remote connec- tion requests.

_#3._#9_#5.                                                   _#s_#t_#o_#p

_#a_#x_#2_#5|_#d_#i_#s_#c_#a_#r_#d|_#e_#c_#h_#o|_#f_#i_#n_#g_#e_#r|_#f_#t_#p|_#n_#e_#t_#r_#o_#m|_#r_#e_#m_#o_#t_#e|_#p_#o_#p|_#s_#m_#t_#p|_#t_#e_#l_#n_#e_#t|_#t_#t_#y_#l_#i_#n_#k

Stop the specified Internet server,  rejecting  any  further remote connect

requests. Existing connections are allowed to complete normally.

_#3._#9_#6.  _#t_#c_#p _#i_#r_#t_#t [<_#v_#a_#l>]

Display or set the initial round trip time estimate, in mil- liseconds, to be

used for new TCP connections until they can measure and adapt to the actual

value.  The default is  5000 milliseconds  (5  seconds).   Increasing this

when operating over slow channels will avoid the flurry of  retransmissions

that  would otherwise occur as the smoothed estimate settles down at the

correct value. Note that this command should  be given  before  servers  are

started in order for it to have effect on incoming connections.

TCP also keeps a _#c_#a_#c_#h_#e of measured round trip times and mean

deviations (MDEV) for current and recent destinations. When-

ever a new TCP connection is opened, the system first  looks

in  this cache. If the destination is found, the cached IRTT and MDEV values

are used. If not,  the  default  IRTT  value mentioned  above  is  used,

along  with  a MDEV of 0.  This feature is fully automatic, and it can

improve  performance greatly  when  a series of connections are opened and

closed to a given destination (e.g., a series of FTP file transfers or

directory listings).

_#3._#9_#7.  _#t_#c_#p _#k_#i_#c_#k <_#t_#c_#b__#a_#d_#d_#r>

If there is unacknowledged data on the  send  queue  of  the specified  TCB,

this command forces an immediate retransmis- sion.

_#3._#9_#8.  _#t_#c_#p _#m_#s_#s [<_#s_#i_#z_#e>]

Display or set the TCP Maximum Segment Size  in  bytes  that will  be  sent on

all outgoing TCP connect request (SYN seg- ments).  This tells the remote end

the size of  the  largest segment  (packet)  it  may  send.  Changing MSS

affects only future connections; existing connections are unaffected.

_#3._#9_#9.  _#t_#c_#p _#r_#e_#s_#e_#t <_#t_#c_#b__#a_#d_#d_#r>

Deletes the TCP control block at the specified address.

_#3._#1_#0_#0.  _#t_#c_#p _#r_#t_#t <_#t_#c_#b__#a_#d_#d_#r> <_#r_#t_#t_#v_#a_#l>

Replaces the automatically computed round trip time  in  the

specified TCB with the r#r#r#rt#t#t#tt#t#t#tv#v#v#va#a#a#al#l#l#l in milliseconds.  This command

is useful to speed up recovery from a series of lost packets

since  it provides a manual bypass around the normal backoff retransmission

timing mechanisms.

_#3._#1_#0_#1.  _#t_#c_#p _#s_#t_#a_#t_#u_#s [<_#t_#c_#b__#a_#d_#d_#r>]

Without arguments, displays  several  TCP-level  statistics,

plus  a  summary  of all existing TCP connections, including

TCB address, send and receive queue sizes, local and  remote

sockets,  and  connection state. If t#t#t#tc#c#c#cb#b#b#b_#_#_#_a#a#a#ad#d#d#dd#d#d#dr#r#r#r is specified, a

more detailed  dump  of  the  specified  TCB  is  generated,

including send and receive sequence numbers and timer infor- mation.

_#3._#1_#0_#2.  _#t_#c_#p _#w_#i_#n_#d_#o_#w [<_#v_#a_#l>]

Displays or sets the default receive window size in bytes to be  used by TCP

when creating new connections. Existing con- nections are unaffected.

_#3._#1_#0_#3.  _#t_#e_#l_#n_#e_#t <_#h_#o_#s_#t_#i_#d> [<_#p_#o_#r_#t>]

Creates a Telnet session to the specified  host  and  enters converse  mode.

If  <port>  is  given that number is used.  Default port is 23.

_#3._#1_#0_#4.  _#t_#t_#y_#l_#i_#n_#k <_#h_#o_#s_#t_#i_#d> [<_#p_#o_#r_#t>]

Creates a Telnet session to the specified  host  and  enters converse  mode.

If  <port>  is  given that number is used.  Default port is 87.  This uses a

split screen interface  for easy conversation.

_#3._#1_#0_#5.  _#t_#i_#p <_#i_#f_#a_#c_#e>

Creates a t#t#t#ti#i#i#ip#p#p#p session that connects to the specified  inter-

face  in  "dumb  terminal"  mode.   The  interface must have

already been attached with the a#a#a#at#t#t#tt#t#t#ta#a#a#ac#c#c#ch#h#h#h command.   Any  packet

traffic  (IP  datagrams,  etc) routed to the interface while

this session exists will be discarded.  To close a t#t#t#ti#i#i#ip#p#p#p  ses-

sion,  use  the r#r#r#re#e#e#es#s#s#se#e#e#et#t#t#t command. It will then revert to normal

s#s#s#sl#l#l#li#i#i#ip#p#p#p,#,#,#, n#n#n#nr#r#r#rs#s#s#s or k#k#k#ki#i#i#is#s#s#ss#s#s#s mode operation.

This feature is primarily useful for  manually  establishing SLIP connections.

At present, only the built-in "com" ports can be used with this command.

_#3._#1_#0_#6.  _#t_#r_#a_#c_#e [<_#i_#f_#a_#c_#e> [<_#f_#l_#a_#g_#s> [<_#t_#r_#a_#c_#e_#f_#i_#l_#e>]]]

Controls packet tracing by the interface  drivers.  Specific

bits enable tracing of the various interfaces and the amount

of information produced.  Tracing is controlled  on  a  per-

interface  basis;  without  arguments, t#t#t#tr#r#r#ra#a#a#ac#c#c#ce#e#e#e gives a list of

all defined interfaces and their tracing status.  Output can

be  limited  to a single interface by specifying it, and the control flags can

be change by specifying them as well.  The flags are given as a hexadecimal

number which is interpreted as follows:

BTIO

||||--- Enable tracing of output packets if 1, disable if 0

|||---- Enable tracing of input packets if 1, disable if 0

||----- Controls type of tracing:

|       0 - Protocol headers are decoded, but data is not displayed

|       1 - Protocol headers are decoded, and data (but not the

|           headers themselves) are displayed as ASCII characters,

|           64 characters/line. Unprintable characters are displayed

|           as periods.

|       2 - Protocol headers are decoded, and the entire packet

|           (headers AND data) is also displayed in hexadecimal

|           and ASCII, 16 characters per line.

|------ Broadcast filter flag. If set, only packets specifically addressed

to this node will be traced; broadcast packets will not be displayed.

If t#t#t#tr#r#r#ra#a#a#ac#c#c#ce#e#e#ef#f#f#fi#i#i#il#l#l#le#e#e#e is not specified, tracing will be to  the  con-

sole.

_#3._#1_#0_#7.  _#u_#d_#p _#s_#t_#a_#t_#u_#s

Displays the status of all UDP receive queues.

_#3._#1_#0_#8.  _#u_#p_#l_#o_#a_#d [<_#f_#i_#l_#e_#n_#a_#m_#e>]

Opens f#f#f#fi#i#i#il#l#l#le#e#e#en#n#n#na#a#a#am#m#m#me#e#e#e and sends it on the current session as though

it were typed on the terminal.

_#3._#1_#0_#9.  _#w_#a_#t_#c_#h

Displays the current software stopwatch values, with min and

max  readings for each. This facility allows a programmer to

measure the execution time of critical sections of code with

microsecond  resolution.   This command is supported only on

the IBM PC, and the meaning of each stopwatch value  depends

on where the calls have been inserted for test purposes; the

distribution copy of _#n_#o_#s usually has no stopwatch calls.

_#3._#1_#1_#0.  ?

Same as the h#h#h#he#e#e#el#l#l#lp#p#p#p command.

_#4.  _#F_#T_#P _#S_#u_#b_#c_#o_#m_#m_#a_#n_#d_#s

During converse mode with an FTP server, everything typed on the  console  is

first  examined to see if it is a locally- known command. If not, the line  is

passed  intact  to  the remote  server  on  the control channel. If it is one

of the following commands, however, it is executed  locally.  (Note that  this

generally  involves other commands being sent to the remote server on the

control channel.)

_#4._#1.  _#d_#i_#r [<_#f_#i_#l_#e>|<_#d_#i_#r_#e_#c_#t_#o_#r_#y> [<_#l_#o_#c_#a_#l _#f_#i_#l_#e>]]

Without arguments, d#d#d#di#i#i#ir#r#r#r requests that a full directory  list-

ing  of the remote server's current directory be sent to the

terminal.  If one argument is given, this is passed along in the  LIST

command; this can be a specific file or subdirec- tory that is meaningful to

the remote file  system.  If  two arguments  are  given, the second is taken

as the local file into which the directory listing should be put  (instead  of

being sent to the console).  The PORT command is used before the LIST command

is sent.

_#4._#2.  _#g_#e_#t <_#r_#e_#m_#o_#t_#e _#f_#i_#l_#e> [<_#l_#o_#c_#a_#l _#f_#i_#l_#e>]

Asks the remote server to send the  file  specified  in  the first  argument.

The second argument, if given, will be the name of the file on the local

machine;  otherwise  it  will have  the  same name as on the remote machine.

The PORT and RETR commands are sent on the control channel.

_#4._#3.  _#l_#s [<_#f_#i_#l_#e>|<_#d_#i_#r_#e_#c_#t_#o_#r_#y> [<_#l_#o_#c_#a_#l _#f_#i_#l_#e>]]

l#l#l#ls#s#s#s is identical to the d#d#d#di#i#i#ir#r#r#r command except  that  the  "NLST"

command is sent to the server instead of the "LIST" command.

This results in an abbreviated directory listing, i.e.,  one showing  only

the  file  names themselves without any other information.

_#4._#4.  _#m_#k_#d_#i_#r <_#r_#e_#m_#o_#t_#e _#d_#i_#r_#e_#c_#t_#o_#r_#y>

Creates a directory on the remote machine.

_#4._#5.  _#p_#u_#t <_#l_#o_#c_#a_#l _#f_#i_#l_#e> [<_#r_#e_#m_#o_#t_#e _#f_#i_#l_#e>]

Asks the remote server to accept  data,  creating  the  file named  in the

first argument. The second argument, if given, will be the name of the file on

the remote  machine;  other- wise  it  will  have  the same name as on the

local machine.  The PORT and STOR commands are sent on the control channel.

_#4._#6.  _#r_#m_#d_#i_#r <_#r_#e_#m_#o_#t_#e _#d_#i_#r_#e_#c_#t_#o_#r_#y>

Deletes a directory on the remote machine.

_#4._#7.  _#t_#y_#p_#e [_#a|_#i|_#l <_#b_#y_#t_#e_#s_#i_#z_#e>]

Tells both the local client and remote server  the  type  of

file  that  is to be transferred.  The default is 'a', which

means ASCII (i.e., a text  file).   Type  'i'  means  _#i_#m_#a_#g_#e,

i.e.,  binary.   In  ASCII  mode,  files are sent as varying

length lines of text in ASCII separated by cr/lf  sequences;

in  IMAGE mode, files are sent exactly as they appear in the

file system.  ASCII mode should be used  whenever  transfer-

ring text between dissimilar systems (e.g., UNIX and MS-DOS)

because of their different  end-of-line  and/or  end-of-file

conventions.  When exchanging text files between machines of

the same type, either mode will work but IMAGE mode is  usu-

ally  faster.   Naturally,  when exchanging raw binary files

(executables, compressed archives, etc) IMAGE mode  must  be

used.   Type 'l' (logical byte size) is used when exchanging

binary files with remote servers having oddball  word  sizes

(e.g., DECSYSTEM-10s and 20s). Locally it works exactly like

IMAGE, except that it notifies the remote system  how  large

the byte size is. b#b#b#by#y#y#yt#t#t#te#e#e#es#s#s#si#i#i#iz#z#z#ze#e#e#e is typically 8.  The type command

sets the local transfer mode and generates the TYPE  command

on the control channel.

_#5.  _#A_#t_#t_#a_#c_#h _#C_#o_#m_#m_#a_#n_#d_#s

This section details the attach  commands  for  the  various

hardware  interface drivers. Not all of these drivers may be

configured into every nos.exe binary; a list of  the  avail-

able types may be obtained by entering the command a#a#a#at#t#t#tt#t#t#ta#a#a#ac#c#c#ch#h#h#h ?#?#?#?.

Some parameters are accepted by several drivers. They are:

_#5._#1.  _#b_#u_#f_#s_#i_#z_#e

For asynchronous devices (e.g., COM ports operating in  SLIP or  NRS  mode)

this  parameter  specifies  the  size of the receiver's ring buffer.  It

should be large enough  to  hold incoming  data  at full line speed for the

longest time that the system may be busy in MS-DOS or the BIOS  doing  a  slow

I/O  operation  (e.g., to a floppy disk). A kilobyte is usu- ally more than

sufficient.

For synchronous devices (e.g., the s#s#s#sc#c#c#cc#c#c#c,#,#,#, h#h#h#hs#s#s#s,#,#,#, p#p#p#pc#c#c#c1#1#1#10#0#0#00#0#0#0,#,#,#, h#h#h#ha#a#a#ap#p#p#pn#n#n#n  and

d#d#d#dr#r#r#rs#s#s#si#i#i#i interfaces operating in HDLC mode), the bufsize parame-

ter specifies the largest packet that may be received on the

interface.   This  should  be  set by mutual agreement among stations sharing

the channel. For standard AX.25 with a max- imum  I-frame  data size of 256

bytes, a value of 325 should provide an adequate safety margin. On higher

speed  channels (e.g.,  56kb/s)  larger values (e.g., 2K bytes) will provide

much better performance and allow full-sized Ethernet  pack- ets to be carried

without fragmentation.

_#5._#2.  _#i_#o_#a_#d_#d_#r

The base address of the interface's  control  registers,  in hex.

_#5._#3.  _#v_#e_#c_#t_#o_#r

The interface's hardware interrupt (IRQ) vector, in hex.

_#5._#4.  _#i_#f_#a_#c_#e

The name (an arbitrary character string) to be  assigned  to

this  interface.  It  is  used  to refer to the interface in

i#i#i#if#f#f#fa#a#a#ac#c#c#ce#e#e#e and r#r#r#ro#o#o#ou#u#u#ut#t#t#te#e#e#e commands and in t#t#t#tr#r#r#ra#a#a#ac#c#c#ce#e#e#e output.

_#5._#5.  _#m_#t_#u

The Maximum Transmission Unit  size,  in  bytes.   Datagrams

larger  than  this  limit will be fragmented at the IP layer

into smaller pieces. For AX.25 UI frames,  this  limits  the

size of the information field.  For AX.25 I frames, however,

the a#a#a#ax#x#x#x2#2#2#25#5#5#5 p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n parameter is also relevant.  If the datagram

or  fragment  is  still larger than p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n, it is also frag-

mented at the AX.25 level  (as  opposed  to  the  IP  level)

before  transmission.   (See  the  a#a#a#ax#x#x#x2#2#2#25#5#5#5  p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n  command for

further information).

_#5._#6.  _#a_#t_#t_#a_#c_#h _#3_#c_#5_#0_#0 <_#i_#o_#a_#d_#d_#r>  <_#v_#e_#c_#t_#o_#r>  _#a_#r_#p_#a  <_#i_#f_#a_#c_#e>  <_#q_#l_#e_#n>

<_#m_#t_#u> [<_#i_#p__#a_#d_#d_#r>]

Attach a 3Com 3C501 Ethernet interface.  q#q#q#ql#l#l#le#e#e#en#n#n#n is the maximum

allowable  transmit  queue length.  If the i#i#i#ip#p#p#p_#_#_#_a#a#a#ad#d#d#dd#d#d#dr#r#r#r parameter

is not given, the value associated with a prior  i#i#i#ip#p#p#p  a#a#a#ad#d#d#dd#d#d#dr#r#r#re#e#e#es#s#s#ss#s#s#s

command will be used.

The use of this driver is not recommended;  use  the  packet driver  interface

with  the  loadable  3C501  packet driver instead.

_#5._#7.  _#a_#t_#t_#a_#c_#h _#a_#s_#y  <_#i_#o_#a_#d_#d_#r>  <_#v_#e_#c_#t_#o_#r>  _#s_#l_#i_#p|_#a_#x_#2_#5|_#n_#r_#s  <_#i_#f_#a_#c_#e>

<_#b_#u_#f_#s_#i_#z_#e> <_#m_#t_#u> <_#s_#p_#e_#e_#d> [<_#f_#l_#a_#g_#s>]

Attach a "com port" (asynchronous  serial  port).   Standard

values  on  the  IBM PC and clones for i#i#i#io#o#o#oa#a#a#ad#d#d#dd#d#d#dr#r#r#r and v#v#v#ve#e#e#ec#c#c#ct#t#t#to#o#o#or#r#r#r are

0x3f8 and 4 for COM1, and 0x2f8 and 3 for COM2. If the  port

uses  a  16550A  chip, it will be detected automatically and the FIFOs

enabled.

Three operating modes are available:

_#5._#7._#1.  _#s_#l_#i_#p

Encapsulates IP datagrams directly in SLIP frames without  a link  header.

This is for operation on point-to-point lines and is compatible with 4.2BSD

UNIX SLIP.

_#5._#7._#2.  _#a_#x_#2_#5

Similar to s#s#s#sl#l#l#li#i#i#ip#p#p#p, except that an AX.25 header and a KISS  TNC

control header are added to the front of the datagram before

SLIP   encoding.    Either   UI   (connectionless)   or    I

(connection-oriented) AX.25 frames can be used; see the m#m#m#mo#o#o#od#d#d#de#e#e#e

command for details.

_#5._#7._#3.  _#n_#r_#s

Use the NET/ROM asynchronous framing technique for  communi- cation with a

local NET/ROM TNC.

_#5._#8.  _#a_#t_#t_#a_#c_#h _#d_#r_#s_#i <_#i_#o_#a_#d_#d_#r> <_#v_#e_#c_#t_#o_#r> _#a_#x_#2_#5  <_#i_#f_#a_#c_#e>  <_#b_#u_#f_#s_#i_#z_#e>

<_#m_#t_#u> <_#c_#h__#a__#s_#p_#e_#e_#d> <_#c_#h__#b__#s_#p_#e_#e_#d>

Attach a Digital Radio Systems PCPA card.  Since  there  are

two channels on the board, two interfaces are attached. They

will be named i#i#i#if#f#f#fa#a#a#ac#c#c#ce#e#e#e with 'a' and 'b'  appended.  b#b#b#bu#u#u#uf#f#f#fs#s#s#si#i#i#iz#z#z#ze#e#e#e  is

the  receiver  buffer  size in bytes; it must be larger than

the largest frame to be received. c#c#c#ch#h#h#h_#_#_#_a#a#a#a_#_#_#_s#s#s#sp#p#p#pe#e#e#ee#e#e#ed#d#d#d and  c#c#c#ch#h#h#h_#_#_#_b#b#b#b_#_#_#_s#s#s#sp#p#p#pe#e#e#ee#e#e#ed#d#d#d

are  the  speeds,  in  bits/sec,  for  the A and B channels,

respectively.

_#5._#9.  _#a_#t_#t_#a_#c_#h _#e_#a_#g_#l_#e <_#i_#o_#a_#d_#d_#r> <_#v_#e_#c_#t_#o_#r> _#a_#x_#2_#5 <_#i_#f_#a_#c_#e>  <_#b_#u_#f_#s_#i_#z_#e>

<_#m_#t_#u> <_#s_#p_#e_#e_#d>

Attach an Eagle Computer 8530 interface card.

_#5._#1_#0.  _#a_#t_#t_#a_#c_#h _#h_#a_#p_#n <_#i_#o_#a_#d_#d_#r> <_#v_#e_#c_#t_#o_#r> _#a_#x_#2_#5 <_#i_#f_#a_#c_#e>  <_#b_#u_#f_#s_#i_#z_#e>

<_#m_#t_#u> _#c_#s_#m_#a|_#f_#u_#l_#l

Attach a Hamilton Area  Packet  Network  adapter  using  the

Intel  8273 chip.  The c#c#c#cs#s#s#sm#m#m#ma#a#a#a|#|#|#|f#f#f#fu#u#u#ul#l#l#ll#l#l#l parameter specifies whether

the port should operate in  carrier  sense  multiple  access

(CSMA) mode or in full duplex.

_#5._#1_#1.  _#a_#t_#t_#a_#c_#h _#h_#s <_#i_#o_#a_#d_#d_#r> <_#v_#e_#c_#t_#o_#r>  _#a_#x_#2_#5  <_#i_#f_#a_#c_#e>  <_#b_#u_#f_#s_#i_#z_#e>

<_#m_#t_#u> <_#k_#e_#y_#u_#p__#d_#e_#l_#a_#y> <_#p>

Attach a DRSI PCPA or Eagle Computer interface card using  a special  "high

speed"  driver.  This  driver uses busy-wait loops to send and receive each

byte instead  of  interrupts, making  it  usable  with  high speed (56kb/s)

modems on slow systems. This does have the side effect  of  "freezing"  the

system whenever the modem transmitter or receiver is active.

This driver can operate only in CSMA mode, and it is  recom- mended  that  no

other interfaces requiring small interrupt latencies be attached to the same

machine.

The k#k#k#ke#e#e#ey#y#y#yu#u#u#up#p#p#p_#_#_#_d#d#d#de#e#e#el#l#l#la#a#a#ay#y#y#y parameter specifies  the  transmitter  keyup

delay  in  byte  time  intervals.  The p#p#p#p value specifies the

transmitter  persistence  value  in  the  range  1-255;  the corresponding

slot time is fixed at one hardware clock tick, about 55 ms on the PC.

As  with  the  other  8530  drivers,  this  driver  actually attaches two

interfaces, one for each 8530 channel.

_#5._#1_#2.  _#a_#t_#t_#a_#c_#h _#p_#a_#c_#k_#e_#t <_#i_#n_#t_#v_#e_#c> <_#i_#f_#a_#c_#e> <_#t_#x_#q_#l_#e_#n> <_#m_#t_#u>

Attach a packet driver conforming to the FTP Software,  Inc,

Packet  Driver  Specification.  The driver must have already

been installed before the attach command  is  given.  Packet

drivers  in  the  Ethernet, ARCNET, SLIP, KISS/AX25 and SLFP

classes are supported.

i#i#i#in#n#n#nt#t#t#tv#v#v#ve#e#e#ec#c#c#c is the software interrupt vector used for  communica-

tion  to the packet driver, and t#t#t#tx#x#x#xq#q#q#ql#l#l#le#e#e#en#n#n#n is the maximum number

of packets that will be allowed on the transmit queue.

_#5._#1_#3.  _#a_#t_#t_#a_#c_#h _#p_#c_#1_#0_#0 <_#i_#o_#a_#d_#d_#r> <_#v_#e_#c_#t_#o_#r> _#a_#x_#2_#5 <_#i_#f_#a_#c_#e> <_#b_#u_#f_#s_#i_#z_#e>

<_#s_#p_#e_#e_#d>

Attach a Paccomm PC-100 interface card. Only AX.25 operation is supported.

_#5._#1_#4.  _#a_#t_#t_#a_#c_#h _#s_#c_#c <_#d_#e_#v_#i_#c_#e_#s>  _#i_#n_#i_#t  <_#a_#d_#d_#r>  <_#s_#p_#a_#c_#i_#n_#g>  <_#A_#o_#f_#f>

<_#B_#o_#f_#f> <_#D_#a_#t_#a_#o_#f_#f> <_#i_#n_#t_#a_#c_#k> <_#v_#e_#c> [_#p]<_#c_#l_#o_#c_#k> [_#h_#d_#w_#e] [_#p_#a_#r_#a_#m]

Initialize a generic SCC (8530)  interface  board  prior  to actually

attaching it. The parameters are as follows:

_#5._#1_#4._#1.  _#d_#e_#v_#i_#c_#e_#s

The number of SCC chips to support.

_#5._#1_#4._#2.  _#a_#d_#d_#r

The base address of the first SCC chip (hex).

_#5._#1_#4._#3.  _#s_#p_#a_#c_#i_#n_#g

The spacing between the SCC chip base addresses.

_#5._#1_#4._#4.  _#A_#o_#f_#f

The offset from a chip's base address to its channel A  con- trol register.

_#5._#1_#4._#5.  _#B_#o_#f_#f

The offset from a chip's base address to its channel B  con- trol register.

_#5._#1_#4._#6.  _#D_#a_#t_#a_#o_#f_#f

The offset from each channel's control register to its  data register.

_#5._#1_#4._#7.  _#i_#n_#t_#a_#c_#k

The address of the INTACK/Read Vector port. If none, specify 0 to read from

RR3A/RR2B.

_#5._#1_#4._#8.  _#v_#e_#c

The CPU interrupt vector for all connected SCCs.

_#5._#1_#4._#9.  _#c_#l_#o_#c_#k

The clock frequency (PCLK/RTxC) of all SCCs in hertz.   Pre- fix  with  'p'

for  PCLK,  'r' for RTxC clock (for baudrate gen).

_#5._#1_#4._#1_#0.  _#h_#d_#w_#e

Optional hardware type. The following values  are  currently supported:   1 -

Eagle card, 2 - PACCOMM PC-100, 4 - PRIMUS- PC card (DG9BL), 8 - DRSI PCPA

card.

_#5._#1_#4._#1_#1.  _#p_#a_#r_#a_#m

Optional extra parameter. At present, this is used only with the  PC-100  and

PRIMUS-PC cards to set the modem mode. The value 0x22 is used with the PC-100

and 0x2 is used with  the PRIMUS-PC card.

The a#a#a#at#t#t#tt#t#t#ta#a#a#ac#c#c#ch#h#h#h s#s#s#sc#c#c#cc#c#c#c .#.#.#..#.#.#..#.#.#. i#i#i#in#n#n#ni#i#i#it#t#t#t command must  be  given  before  the

interfaces are actually attached with the following command.

_#5._#1_#5.  _#a_#t_#t_#a_#c_#h _#s_#c_#c <_#c_#h_#a_#n>  _#s_#l_#i_#p|_#k_#i_#s_#s|_#n_#r_#s|_#a_#x_#2_#5  <_#i_#f_#a_#c_#e>  <_#m_#t_#u>

<_#s_#p_#e_#e_#d> <_#b_#u_#f_#s_#i_#z_#e> [_#c_#a_#l_#l]

Attach an initialized SCC port to the system. The parameters

are as follows:

_#5._#1_#5._#1.  <_#c_#h_#a_#n>

The SCC channel number to attach,  0  or  1  for  the  first chip's  A  or  B

port,  2 or 3 for the second chip's A or B port, etc.

_#5._#1_#5._#2.  _#s_#l_#i_#p|_#k_#i_#s_#s|_#n_#r_#s|_#a_#x_#2_#5

The operating mode of the interface. s#s#s#sl#l#l#li#i#i#ip#p#p#p,#,#,#, k#k#k#ki#i#i#is#s#s#ss#s#s#s and n#n#n#nr#r#r#rs#s#s#s  all

operate  the  port  hardware  in  asynchronous mode; s#s#s#sl#l#l#li#i#i#ip#p#p#p is

Internet-standard serial line IP mode, k#k#k#ki#i#i#is#s#s#ss#s#s#s  generates  SLIP

frames  containing  KISS  TNC commands and AX.25 packets and

n#n#n#nr#r#r#rs#s#s#s uses NET/ROM local serial link  framing  conventions  to

carry  NET/ROM  packets. Selecting a#a#a#ax#x#x#x2#2#2#25#5#5#5 mode puts the inter-

face into synchronous HDLC mode that is suitable for  direct

connection to a half duplex radio modem.

_#5._#1_#5._#3.  _#s_#p_#e_#e_#d

The interface speed in bits per second, e.g., 1200.   Prefix with  'd'  when

an external divider is available to generate the TX clock. When the clock

source is PCLK, this can  be  a /32  divider  between  TRxC  and  RTxC. When

the clock is at RTxC, the TX rate must be supplied at TRxC. This  is  needed

only for full duplex synchronous operation. When this arg is given as 'ext',

the transmit and receive clocks  are  exter- nal,  and the internal baud rate

generator (BRG) and digital phase locked loop (DPLL) are not used.

_#5._#1_#5._#4.  _#A_#t_#t_#a_#c_#h _#E_#x_#a_#m_#p_#l_#e_#s

Here are some examples of the attach command:

# Attach a 3Com Ethernet controller using the standard 3Com address and

# vector (i.e., as it comes out of the box) to use ARPA-standard encapsulation.

# The receive queue is limited to 5 packets, and outgoing packets larger

# than 1500 bytes will be fragmented

attach 3c500 0x300 3 arpa ec0 5 1500

# Attach the PC asynch card normally known as "com1" (the first controller)

# to operate in point-to-point slip mode at 9600 baud, calling it "sl0".

# A 1024 byte receiver ring buffer is allocated. Outgoing packets larger

# than 256 bytes are fragmented.

attach asy 0x3f8 4 slip sl0 1024 256 9600

# Attach the secondary PC asynch card ("com2") to operate in AX.25 mode

# with an MTU of 576 bytes at 9600 baud with a KISS TNC, calling it "ax0".

# By default, IP datagrams are sent in UI frames

attach asy 0x2f8 3 ax25 ax0 1024 576 9600

# Attach the packet driver loaded at interrupt 0x7e

# The packet driver is for an Ethernet interface

attach packet 0x7e ethernet 8 1500

_#6.  _#T_#h_#e /_#f_#t_#p_#u_#s_#e_#r_#s _#F_#i_#l_#e

Since MS-DOS is a single-user operating system  (some  might say  it  is  a

glorified  bootstrap loader), it provides no access control; all files can be

read, written or deleted by the local user.  It is usually undesirable to give

such open access to a system to remote network users.   Net  therefore

provides its own access control mechanisms.

The file /#/#/#/f#f#f#ft#t#t#tp#p#p#pu#u#u#us#s#s#se#e#e#er#r#r#rs#s#s#s controls remote FTP and  mailbox  access.

The  FTP  default is _#n_#o access; if this file does not exist,

the FTP server will be unusable.  A remote user  must  first

"log in" to the system with the USER and PASS commands, giv-

ing a valid name and password listed in /#/#/#/f#f#f#ft#t#t#tp#p#p#pu#u#u#us#s#s#se#e#e#er#r#r#rs#s#s#s, before he

or she can transfer files.

Each entry in /#/#/#/f#f#f#ft#t#t#tp#p#p#pu#u#u#us#s#s#se#e#e#er#r#r#rs#s#s#s consists of a  single  line  of  the

form

username password /path permissions

There must be exactly four fields, and there must be exactly one  space

between each field.  Comments may be added after the last field. Comment lines

begin with '#' in column one.

u#u#u#us#s#s#se#e#e#er#r#r#rn#n#n#na#a#a#am#m#m#me#e#e#e is the user's login name.

p#p#p#pa#a#a#as#s#s#ss#s#s#sw#w#w#wo#o#o#or#r#r#rd#d#d#d is the required password.  Note  that  this  is  in

plain  text; therefore it is not a good idea to give general

read permission to the root directory.  A password of '*' (a single asterisk)

means that any password is acceptable.

/#/#/#/p#p#p#pa#a#a#at#t#t#th#h#h#h is the allowable prefix on accessible  files.   Before

any  file  or directory operation, the current directory and

the user- specified file name are joined to form an absolute path name in

"canonical" form (i.e., a full path name start- ing at the root, with "./" and

"../" references, as well  as redundant  /'s,  recognized  and  removed).  The

result MUST begin with the allowable path prefix; if not, the  operation is

denied.   This field must always begin with a "/", i.e., at the root

directory.

p#p#p#pe#e#e#er#r#r#rm#m#m#mi#i#i#is#s#s#ss#s#s#si#i#i#io#o#o#on#n#n#ns#s#s#s is a  decimal  number  granting  permission  for

read,  create  and  write  operations.  If the low order bit

(0x1) is set, the user is allowed to read a file subject  to the  path name

prefix restriction.  If the next bit (0x2) is set, the user is allowed to

create a new file if it does not overwrite  an existing file.  If the third

bit (0x4) is set, the user is allowed to write a file even if it overwrites an

existing  file, and in addition he may delete files.  Again, all operations

are allowed subject to the path  name  prefix restrictions.  Permissions  may

be combined by adding bits, for example, 0x3 (= 0x2 + 0x1) means that the user

is  given read and create permission, but not overwrite/delete permis- sion.

For example, suppose /#/#/#/f#f#f#ft#t#t#tp#p#p#pu#u#u#us#s#s#se#e#e#er#r#r#rs#s#s#s on  machine  pc.ka9q.ampr.org

contains the line

friendly test /testdir 7

A session using this account would look like this:

net> ftp pc.ka9q.ampr.org

Resolving pc.ka9q.ampr.org... Trying 128.96.160.1...

FTP session 1 connected to pc.ka9q.ampr.org

220 pc.ka9q.ampr.org FTP version 900418 ready at Mon May 7 16:27:18 1990

Enter user name: friendly

331 Enter PASS command

Password: test [not echoed]

230 Logged in

ftp>

The  user  now  has  read,  write,  overwrite   and   delete privileges  for

any  file under /testdir; he may not access any other files.

Here are some more sample entries in /#/#/#/f#f#f#ft#t#t#tp#p#p#pu#u#u#us#s#s#se#e#e#er#r#r#rs#s#s#s:

karn foobar / 7         # User "karn" with password "foobar" may read,

# write, overwrite and delete any file on the

# system.

guest bletch /g/bogus 3 # User "guest" with password "bletch" may read

# any file under /g/bogus and its subdirectories,

# and may create a new file as long as it does

# not overwrite an existing file. He may NOT

# delete any files.

anonymous * /public 1   # User "anonymous" (any password) may read files

# under /public and its subdirectories; he may

# not create, overwrite or delete any files.

This last entry is the standard  convention  for  keeping  a repository  of

public  files;  in  particular, the username "anonymous" is an established

ARPA convention.

_#7.  _#T_#h_#e /_#d_#o_#m_#a_#i_#n._#t_#x_#t _#F_#i_#l_#e

_#N_#e_#t translates domain names (e.g., "pc.ka9q.ampr.org") to IP

addresses  (e.g., 128.96.160.3) through the use of an Inter-

net  Domain  Name  resolver  and  a  local   "cache"   file,

/#/#/#/d#d#d#do#o#o#om#m#m#ma#a#a#ai#i#i#in#n#n#n.#.#.#.t#t#t#tx#x#x#xt#t#t#t.  Whenever  the  user  specifies  a domain name,

/#/#/#/d#d#d#do#o#o#om#m#m#ma#a#a#ai#i#i#in#n#n#n.#.#.#.t#t#t#tx#x#x#xt#t#t#t is first searched for the desired entry.  If  it

is present, it is used; if not, and if domain name server(s)

have been configured, a query is sent over  the  network  to

the  current  server.  If the server responds, the answer is

appended to the /#/#/#/d#d#d#do#o#o#om#m#m#ma#a#a#ai#i#i#in#n#n#n.#.#.#.t#t#t#tx#x#x#xt#t#t#t file  for  future  use.  If  the

server  does not respond, any additional servers on the list

are tried indefinitely in a round-robin  fashion  until  one

responds.  If /#/#/#/d#d#d#do#o#o#om#m#m#ma#a#a#ai#i#i#in#n#n#n.#.#.#.t#t#t#tx#x#x#xt#t#t#t does not contain the desired entry

and there are no configured domain name  servers,  then  the

request immediately fails.

If a domain name server is available, and if all  references

to hostids in your /a#a#a#au#u#u#ut#t#t#to#o#o#oe#e#e#ex#x#x#xe#e#e#ec#c#c#c.#.#.#.n#n#n#no#o#o#os#s#s#s file are in IP address for-

mat, then it is possible to start with  a  completely  empty

/#/#/#/d#d#d#do#o#o#om#m#m#ma#a#a#ai#i#i#in#n#n#n.#.#.#.t#t#t#tx#x#x#xt#t#t#t  file  and  have _#n_#o_#s build it for you.  However,

you may wish to add your own entries to /#/#/#/d#d#d#do#o#o#om#m#m#ma#a#a#ai#i#i#in#n#n#n.#.#.#.t#t#t#tx#x#x#xt#t#t#t,  either

because  you  prefer  to  use  symbolic domain names in your

/a#a#a#au#u#u#ut#t#t#to#o#o#oe#e#e#ex#x#x#xe#e#e#ec#c#c#c.#.#.#.n#n#n#no#o#o#os#s#s#s file or you don't  have  access  to  a  domain

server  and  you need to create entries for all of the hosts

you may wish to access. Each entry takes one line,  and  the

fields are separated by tabs. For example:

pc.ka9q.ampr.org.       IN      A       128.96.160.3

I#I#I#IN#N#N#N is the _#c_#l_#a_#s_#s of the record. It  means  _#I_#n_#t_#e_#r_#n_#e_#t,  and  it

will  be found in all entries.  A#A#A#A is the _#t_#y_#p_#e of the record,

and it means that this is an _#a_#d_#d_#r_#e_#s_#s  record.   Domain  name

p#p#p#pc#c#c#c.#.#.#.k#k#k#ka#a#a#a9#9#9#9q#q#q#q.#.#.#.a#a#a#am#m#m#mp#p#p#pr#r#r#r.#.#.#.o#o#o#or#r#r#rg#g#g#g     therefore    has    Internet    address

128.96.160.3.

Another possible entry is the C#C#C#CN#N#N#NA#A#A#AM#M#M#ME#E#E#E (Canonical Name) record,

e.g.,

ka9q.ampr.org.          IN      CNAME   pc.ka9q.ampr.org.

This says that domain name "ka9q.ampr.org"  is  actually  an

alias  for  the  system  with (primary, or _#c_#a_#n_#o_#n_#i_#c_#a_#l) domain

name "pc.ka9q.ampr.org." When a domain name having  a  C#C#C#CN#N#N#NA#A#A#AM#M#M#ME#E#E#E

record is given to _#n_#o_#s, the system automatically follows the

reference to the canonical name and returns the  IP  address

associated with that entry.

Entries added automatically by _#n_#o_#s will have  an  additional

field  between  the  domain  name  and the class (I#I#I#IN#N#N#N) field,

e.g.,

pc.ka9q.ampr.org.       3600    IN      A       128.96.160.3

This is the _#t_#i_#m_#e-_#t_#o-_#l_#i_#v_#e value, in seconds, associated  with

the  record  received from the server. Clients (such as _#n_#o_#s)

caching these records are supposed to delete them after  the

time-to-live  interval  has expired, allowing for the possi-

bility that the information in the record may become out  of

date.  Cache expiration is not currently implemented in _#n_#o_#s,

but TTLs are recorded in /#/#/#/d#d#d#do#o#o#om#m#m#ma#a#a#ai#i#i#in#n#n#n.#.#.#.t#t#t#tx#x#x#xt#t#t#t in anticipation of this

feature.  Since  /#/#/#/d#d#d#do#o#o#om#m#m#ma#a#a#ai#i#i#in#n#n#n.#.#.#.t#t#t#tx#x#x#xt#t#t#t is a plain text file, it may be

easily edited by the user to delete old records.

Additional types of records, include NS  (name  server)  and

SOA  (start  of  authority)  may  appear in /#/#/#/d#d#d#do#o#o#om#m#m#ma#a#a#ai#i#i#in#n#n#n.#.#.#.t#t#t#tx#x#x#xt#t#t#t from

remote server responses. These are not currently used by _#n_#o_#s

but  are retained for future development (such as the incor-

poration of a domain name server into _#n_#o_#s  itself).   Server

responses  indicating that a given record does not exist are

also recorded in /#/#/#/d#d#d#do#o#o#om#m#m#ma#a#a#ai#i#i#in#n#n#n.#.#.#.t#t#t#tx#x#x#xt#t#t#t. These entries have empty value

fields, e.g.,

foobar.ampr.org.        500     IN      A

_#8.  _#S_#e_#t_#t_#i_#n_#g _#B_#u_#f_#s_#i_#z_#e, _#P_#a_#c_#l_#e_#n, _#M_#a_#x_#f_#r_#a_#m_#e, _#M_#T_#U, _#M_#S_#S _#a_#n_#d _#W_#i_#n_#d_#o_#w

Many _#n_#o_#s users are confused by these parameters and  do  not

know  how  to  set  them  properly.  This section will first

review these parameters  and  then  discuss  how  to  choose

values  for  them.  Special  emphasis  is  given to avoiding

interoperability problems that may appear when communicating

with non-_#n_#o_#s implementations of AX.25.

_#8._#1.  _#H_#a_#r_#d_#w_#a_#r_#e _#P_#a_#r_#a_#m_#e_#t_#e_#r_#s

_#8._#1._#1.  _#B_#u_#f_#s_#i_#z_#e

This parameter is required by most of  _#n_#o_#s's  built-in  HDLC

drivers  (e.g.,  those for the DRSI PCPA and the Paccomm PC-

100). It specifies the size of the buffer  to  be  allocated for  each

receiver port. HDLC frames larger than this value cannot be received.

There is no default b#b#b#bu#u#u#uf#f#f#fs#s#s#si#i#i#iz#z#z#ze#e#e#e; it must  be  specified  in  the

a#a#a#at#t#t#tt#t#t#ta#a#a#ac#c#c#ch#h#h#h command for the interface.

_#8._#2.  _#A_#X_#2_#5 _#P_#a_#r_#a_#m_#e_#t_#e_#r_#s

_#8._#2._#1.  _#P_#a_#c_#l_#e_#n

Paclen limits the size of the data  field  in  an  AX.25  I-

frame. This value does _#n_#o_#t include the AX.25 protocol header

(source, destination and digipeater addresses).

Since unconnected-mode (datagram) AX.25 uses UI frames, this

parameter has no effect in unconnected mode.

The default value of p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n is 256 bytes.

_#8._#2._#2.  _#M_#a_#x_#f_#r_#a_#m_#e

This parameter controls the number of I-frames that _#n_#o_#s  may

send on an AX.25 connection before it must stop and wait for

an acknowledgement.  Since the  AX.25/LAPB  sequence  number field is 3 bits

wide, this number cannot be larger than 7.

Since unconnected-mode (datagram) AX.25 uses UI frames  that

do  not have sequence numbers, this parameter does _#n_#o_#t apply

to unconnected mode.

The default value of m#m#m#ma#a#a#ax#x#x#xf#f#f#fr#r#r#ra#a#a#am#m#m#me#e#e#e in _#n_#o_#s is 1 frame.

_#8._#3.  _#I_#P _#a_#n_#d _#T_#C_#P _#P_#a_#r_#a_#m_#e_#t_#e_#r_#s

_#8._#3._#1.  _#M_#T_#U

The MTU (Maximum Transmission Unit) is an interface  parame-

ter  that limits the size of the largest IP datagram that it

may handle.  IP datagrams routed to an  interface  that  are

larger  than  its  MTU are each split into two or more _#f_#r_#a_#g_#-

_#m_#e_#n_#t_#s.  Each fragment has its own IP header and  is  handled

by  the  network  as  if it were a distinct IP datagram, but

when it arrives at the destination it  is  held  by  the  IP layer until all

of the other fragments belonging to the ori- ginal datagram have arrived. Then

they are reassembled  back into the complete, original IP datagram. The

minimum accept- able interface MTU is 28 bytes: 20 bytes for the  IP  (frag-

ment) header, plus 8 bytes of data.

There is no default MTU in _#n_#o_#s; it must be explicitly speci-

fied for each interface as part of the a#a#a#at#t#t#tt#t#t#ta#a#a#ac#c#c#ch#h#h#h command.

_#8._#3._#2.  _#M_#S_#S

MSS (Maximum Segment Size) is  a  TCP-level  parameter  that

limits the amount of data that the _#r_#e_#m_#o_#t_#e TCP will send in a

single TCP packet. MSS values are exchanged in the SYN (con-

nection  request) packets that open a TCP connection. In the

_#n_#o_#s implementation of TCP, the MSS actually used by  TCP  is

further reduced in order to avoid fragmentation at the local

IP interface. That is, the local TCP asks IP for the MTU  of the interface

that will be used to reach the destination. It then subtracts 40 from the MTU

value to allow for the  over- head  of  the TCP and IP headers. If the result

is less than the MSS received from the remote TCP, it is used instead.

The default value of M#M#M#MS#S#S#SS#S#S#S is 512 bytes.

_#8._#3._#3.  _#W_#i_#n_#d_#o_#w

This is a TCP-level parameter that controls  how  much  data

the  local  TCP  will allow the remote TCP to send before it

must stop and wait for an acknowledgement. The actual window

value  used  by TCP when deciding how much more data to send

is referred to as the _#e_#f_#f_#e_#c_#t_#i_#v_#e _#w_#i_#n_#d_#o_#w.  This is the smaller

of two values: the window advertised by the remote TCP minus

the unacknowledged data in flight, and the  _#c_#o_#n_#g_#e_#s_#t_#i_#o_#n  _#w_#i_#n_#-

_#d_#o_#w,  an automatically computed time-varying estimate of how

much data the network can handle.

The default value of W#W#W#Wi#i#i#in#n#n#nd#d#d#do#o#o#ow#w#w#w is 2048 bytes.

_#8._#4.  _#D_#i_#s_#c_#u_#s_#s_#i_#o_#n

_#8._#4._#1.  _#I_#P _#F_#r_#a_#g_#m_#e_#n_#t_#a_#t_#i_#o_#n _#v_#s _#A_#X._#2_#5 _#S_#e_#g_#m_#e_#n_#t_#a_#t_#i_#o_#n

IP-level fragmentation often makes it possible to  intercon-

nect  two  dissimilar networks, but it is best avoided when-

ever possible.  One reason is that when a single IP fragment

is  lost, all other fragments belonging to the same datagram

are effectively also lost and the entire  datagram  must  be

retransmitted  by  the source.  Even without loss, fragments

require the allocation of temporary  buffer  memory  at  the

destination, and it is never easy to decide how long to wait

for missing fragments before giving up and discarding  those

that have already arrived.  A reassembly timer controls this

process.  In _#n_#o_#s it is (re)initialized with  the  i#i#i#ip#p#p#p  r#r#r#rt#t#t#ti#i#i#im#m#m#me#e#e#er#r#r#r

parameter  (default 30 seconds) whenever progress is made in

reassembling a datagram (i.e., a new fragment is  received).  It is not

necessary that all of the fragments belonging to a datagram arrive within a

single timeout interval, only  that the interval between fragments be less

than the timeout.

Most subnetworks that carry IP have MTUs  of  576  bytes  or

more,   so  interconnecting  them  with  subnetworks  having

smaller values can result in considerable fragmentation. For

this  reason,  IP implementors working with links or subnets

having unusually small packet size limits are encouraged  to

use _#t_#r_#a_#n_#s_#p_#a_#r_#e_#n_#t _#f_#r_#a_#g_#m_#e_#n_#t_#a_#t_#i_#o_#n, that is, to devise schemes to

break up large IP datagrams into a sequence of link or  sub-

net frames that are immediately reassembled on the other end

of the link or subnet into the original, whole  IP  datagram

without  the use of IP-level fragmentation. Such a scheme is

provided in AX.25 Version 2.1.  It can break a large  IP  or

NET/ROM  datagram  into  a series of p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n-sized AX.25 seg-

ments (not to be confused with TCP segments), one per  AX.25

I-frame,  for transmission and reassemble them into a single

datagram at the other end of the link before handing  it  up

to  the  IP or NET/ROM module.  Unfortunately, the segmenta-

tion procedure is a new feature in  AX.25  and  is  not  yet

widely  implemented;  in  fact, _#n_#o_#s is so far the only known

implementation. This creates some interoperability  problems

between  _#n_#o_#s  and  non-_#n_#o_#s  nodes,  in  particular, standard

NET/ROM nodes being used to carry IP datagrams. This problem

is discussed further in the section on setting the MTU.

_#8._#4._#2.  _#S_#e_#t_#t_#i_#n_#g _#p_#a_#c_#l_#e_#n _#a_#n_#d _#b_#u_#f_#s_#i_#z_#e

The more data you put into an AX.25 I frame, the smaller the

AX.25  headers  are  in relation to the total frame size. In

other words, by increasing p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n, you lower the AX.25  pro-

tocol overhead. Also, large data packets reduce the overhead

of keying up a transmitter, and this  can  be  an  important

factor  with  higher  speed modems. On the other hand, large

frames make bigger targets for noise and interference.  Each

link  has an optimum value of p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n that is best discovered

by experiment.

Another thing to remember when setting p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n  is  that  the

AX.25  version  2.0  specification  limits  it to 256 bytes.

Although _#n_#o_#s can handle much larger values, some other AX.25

implementations  (including digipeaters) cannot and this may

cause interoperability problems. Even _#n_#o_#s may  have  trouble

with  certain  KISS  TNCs because of fixed-size buffers. The

original KISS TNC code for the  TNC-2  by  K3MC  can  handle

frames  limited in size only by the RAM in the TNC, but some

other KISS TNCs cannot.

_#N_#e_#t's built-in HDLC drivers (SCC, PC-100, DRSI,  etc)  allo-

cate receive buffers according to the maximum expected frame

size, so it is important that these  devices  be  configured

with the correct b#b#b#bu#u#u#uf#f#f#fs#s#s#si#i#i#iz#z#z#ze#e#e#e. To do this, you must know the size

of the largest possible frame  that  can  be  received.  The

p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n parameter controls only the size of the data field in

an I-frame and not the total size of the frame as it appears

on  the  air.  The AX.25 spec allows up to 8 digipeaters, so

the largest possible frame is (p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n + 72)  bytes.  So  you

should make b#b#b#bu#u#u#uf#f#f#fs#s#s#si#i#i#iz#z#z#ze#e#e#e at least this large.

Another important consideration is that the more recent ver-

sions  of  NOS  improve  interrupt response by maintaining a

special pool of buffers for use  by  the  receive  routines.

These  buffers are currently fixed in size to 2048 bytes and

this can be changed only by editing config.h and recompiling

NOS.   This  limits  b#b#b#bu#u#u#uf#f#f#fs#s#s#si#i#i#iz#z#z#ze#e#e#e;  in  fact, attempting to set a

larger value may cause the driver not to work at  all.  This

situation  can be detected by running the m#m#m#me#e#e#em#m#m#mo#o#o#or#r#r#ry#y#y#y s#s#s#st#t#t#ta#a#a#at#t#t#tu#u#u#us#s#s#s com-

mand and looking for a non-zero count  of  I#I#I#Ib#b#b#bu#u#u#uf#f#f#ff#f#f#fa#a#a#ai#i#i#il#l#l#l  events,

although  these  events  can  also occur occasionally during

normal operation.

One of the drawbacks of AX.25 that there is no way  for  one

station  to tell another how large a packet it is willing to

accept.  This requires the stations  sharing  a  channel  to

agree  beforehand  on  a  maximum  packet size.  TCP is dif-

ferent, as we shall see.

_#8._#4._#3.  _#S_#e_#t_#t_#i_#n_#g _#M_#a_#x_#f_#r_#a_#m_#e

For best performance on a half-duplex  radio  channel,  m#m#m#ma#a#a#ax#x#x#x-#-#-#-

f#f#f#fr#r#r#ra#a#a#am#m#m#me#e#e#e  should  always be set to 1. The reasons are explained

in the paper _#L_#i_#n_#k _#L_#e_#v_#e_#l _#P_#r_#o_#t_#o_#c_#o_#l_#s _#R_#e_#v_#i_#s_#i_#t_#e_#d by  Brian  Lloyd

and Phil Karn, which appeared in the proceedings of the ARRL

5th Computer Networking Conference in 1986.

_#8._#4._#4.  _#S_#e_#t_#t_#i_#n_#g _#M_#T_#U

TCP/IP header overhead considerations similar  to  those  of

the  AX.25  layer when setting p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n apply when choosing an

MTU.  However, certain subnetwork  types  supported  by  _#n_#o_#s

have  well-established MTUs, and these should always be used

unless you know what you're doing: 1500 bytes for  Ethernet,

and 508 bytes for ARCNET. Other subnet types, including SLIP

and AX.25, are not as well standardized. SLIP has  no  offi-

cial  MTU, but the most common implementation (for BSD UNIX)

uses an MTU of 1006 bytes.  Although _#n_#o_#s has no  hard  wired

limit on the size of a received SLIP frame, this is not true

for other systems.  Interoperability problems may  therefore

result if larger MTUs are used in _#n_#o_#s.

Choosing an MTU for an AX.25 interface is more complex. When

the  interface  operates  in  datagram  (UI-frame) mode, the

p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n parameter does not apply. The MTU effectively becomes

the  p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n  of  the  link.   However, as mentioned earlier,

large packets sent on AX.25  _#c_#o_#n_#n_#e_#c_#t_#i_#o_#n_#s  are  automatically

segmented into I-frames no larger than p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n bytes.  Unfor-

tunately, as also mentioned earlier, _#n_#o_#s is so far the  only

known  implementation  of  the  new  AX.25 segmentation pro-

cedure. This is fine as long as all  of  the  NET/ROM  nodes

along  a path are running _#n_#o_#s, but since the main reason _#n_#o_#s

supports NET/ROM is to allow use of  existing  NET/ROM  net-

works, this is unlikely.

So it is usually important to avoid AX.25 segmentation  when

running IP over NET/ROM.  The way to do this is to make sure

that packets larger than p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n are never handed  to  AX.25.

A  NET/ROM  transport  header  is 5 bytes long and a NET/ROM

network header takes 15 bytes, so 20 bytes must be added  to

the  size  of  an  IP datagram when figuring the size of the

AX.25 I-frame data field. If p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n is 256, this leaves  236

bytes  for  the  IP datagram. This is the default MTU of the

n#n#n#ne#e#e#et#t#t#tr#r#r#ro#o#o#om#m#m#m pseudo-interface, so as long as p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n  is  at  least

256  bytes,  AX.25 segmentation can't happen. But if smaller

values of p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n are used,  the  n#n#n#ne#e#e#et#t#t#tr#r#r#ro#o#o#om#m#m#m  MTU  must  also  be

reduced with the i#i#i#if#f#f#fc#c#c#co#o#o#on#n#n#nf#f#f#fi#i#i#ig#g#g#g command.

On the other hand, if you're running IP directly on  top  of

AX.25, chances are all of the nodes are running _#n_#o_#s and sup-

port AX.25 segmentation.  In this case there  is  no  reason

not  to  use  a larger MTU and let AX.25 segmentation do its

thing. If you choose an MTU on the order of 1000-1500 bytes,

you  can  largely  avoid  IP-level  fragmentation and reduce

TCP/IP-level header overhead on file transfers to a very low

level.  And you are still free to pick whatever p#p#p#pa#a#a#ac#c#c#cl#l#l#le#e#e#en#n#n#n value

is appropriate for the link.

_#8._#4._#5.  _#S_#e_#t_#t_#i_#n_#g _#M_#S_#S

The setting of this TCP-level  parameter  is  somewhat  less critical than the

IP and AX.25 level parameters already dis- cussed, mainly because it is

automatically lowered according to  the  MTU  of  the  local  interface when a

connection is created. Although this is,  strictly  speaking,  a  protocol

layering   violation  (TCP  is  not  supposed  to  have  any knowledge of the

workings of lower  layers)  this  technique does  work well in practice.

However, it can be fooled; for example, if a routing change occurs after the

connection has been  opened  and  the new local interface has a smaller MTU

than the previous one, IP fragmentation  may  occur  in  the local system.

The only drawback to setting a large MSS is  that  it  might cause avoidable

fragmentation at some other point within the network path if it includes a

"bottleneck"  subnet  with  an MTU  smaller  than  that  of  the  local

interface.  (Unfor- tunately, there is presently no way to know when this is

the case.  There is ongoing work within the Internet Engineering Task Force on

a "MTU Discovery" procedure to  determine  the largest  datagram that may be

sent over a given path without fragmentation, but it is not yet complete.)

Also, since  the MSS  you  specify  is sent to the remote system, and not all

other TCPs do the MSS-lowering  procedure  yet,  this  might cause  the

remote  system to generate IP fragments unneces- sarily.

On the other hand, a too-small MSS can result in a consider- able  performance

loss, especially when operating over fast LANs and networks that can handle

larger  packets.  So  the best  value for MSS is probably 40 less than the

largest MTU on your system, with the 40-byte margin allowing for the TCP and

IP  headers.  For example, if you have a SLIP interface with a 1006 byte MTU

and an Ethernet interface with  a  1500 byte  MTU, set MSS to 1460 bytes. This

allows you to receive maximum-sized Ethernet packets, assuming the  path  to

your system  does  not  have  any bottleneck subnets with smaller MTUs.

_#8._#4._#6.  _#S_#e_#t_#t_#i_#n_#g _#W_#i_#n_#d_#o_#w

A sliding window protocol like TCP cannot transfer more than

one  window's worth of data per round trip time interval. So

this TCP-level parameter controls the ability of the  remote

TCP to keep a long "pipe" full. That is, when operating over

a path with many hops, offering a large TCP window will help

keep  all those hops busy when you're receiving data. On the

other hand, offering too large a window can congest the net-

work  if  it  cannot  buffer all that data. Fortunately, new

algorithms for dynamic controlling the  effective  TCP  flow

control  window  have been developed over the past few years

and are now widely deployed. _#N_#e_#t includes them, and you  can

watch  them  in  action  with the t#t#t#tc#c#c#cp#p#p#p s#s#s#st#t#t#ta#a#a#at#t#t#tu#u#u#us#s#s#s <#<#<#<t#t#t#tc#c#c#cb#b#b#b>#>#>#> or s#s#s#so#o#o#oc#c#c#ck#k#k#ke#e#e#et#t#t#t

<#<#<#<s#s#s#so#o#o#oc#c#c#ck#k#k#kn#n#n#no#o#o#o>#>#>#> commands.  Look at the  c#c#c#cw#w#w#wi#i#i#in#n#n#nd#d#d#d  (congestion  window)

value.

In most cases it is safe to set the TCP window  to  a  small

integer  multiple  of the MSS, e.g., 4x, or larger if neces-

sary to fully utilize a high bandwidth*delay  product  path.

One  thing  to  keep in mind, however, is that advertising a

certain TCP window value declares that the system  has  that

much  buffer space available for incoming data. _#N_#e_#t does not

actually preallocate this space; it keeps  it  in  a  common

pool  and  may  well "overbook" it, exploiting the fact that

many TCP connections are idle for long periods and  gambling

that  most  applications  will  read  incoming  data from an

active connection as soon as  it  arrives,  thereby  quickly

freeing  the  buffer memory.  However, it is possible to run

_#n_#o_#s out of memory if excessive TCP window sizes  are  adver-

tised  and  either the applications go to sleep indefinitely

(e.g., suspended  Telnet  sessions)  or  a  lot  of  out-of-

sequence  data  arrives.   It  is wise to keep an eye on the

amount of available memory and to decrease  the  TCP  window

size (or limit the number of simultaneous connections) if it

gets too low.

Depending on the channel access method and link level proto- col,  the  use

of a window setting that exceeds the MSS may cause an increase in channel

collisions. In particular, col- lisions  between data packets and returning

acknowledgements during a bulk file transfer may become common. Although this

is,  strictly  speaking,  not TCP's fault, it is possible to work around the

problem at the TCP level by  decreasing  the window  so that the protocol

operates in stop-and-wait mode.  This is done by making the window value equal

to the MSS.

_#8._#5.  _#S_#u_#m_#m_#a_#r_#y

In most cases, the default values provided by _#n_#o_#s  for  each

of  these parameters will work correctly and give reasonable

performance. Only in special circumstances such as operation over  a  very

poor  link or experimentation with high speed modems should it be necessary to

change them.

NOSDOC.DOC

Date: Tue, 22 Nov 88 00:35:09 EST

From: karn@ka9q.bellcore.com (Phil Karn)

To: tcp-group@ucsd.edu

Subject: nos.arc on flash.bellcore.com

I'm uploading the file nos.arc to flash.bellcore.com under the /pub/ka9q

directory. This contains the current source of the new version I've been

working on. Although it is *NOT* yet complete, I thought I should make it

available so people can see how it's shaping up.

Go ahead and give it a try on the Internet. I have NOT yet finished the

implementation of AX.25 sockets, so don't bug me about that part yet.

I know, I know, there's no documentation yet. The main thing you need to know

is that \hosts.net has been replaced with \domain.txt, and the format is that

of a domain server database, not UNIX /etc/hosts.

Phil

Unfortunately, I just haven't had the time to update the documentation for

NOS. Actually, the user interface isn't that much different; the main

difference is the use of a domain resolver. As part of this work I got rid of

hosts.net in favor of a local domain cache called domain.txt.

A word of warning about the domain stuff in NOS: the domain.txt file is

intended only as a local CACHE for the resolver. I have not yet done a domain

SERVER, so the content of domain.txt is not necessarily the same as you would

put in a server database.

This particular version tries to grab 200K from the system at startup time for

the heap; if less is available, it'll get whatever it can. If you want to use

more, modify the call to grabcore() in pc.c. Eventually this will probably be

a easily configurable value.

1. I've made the final switch to Turbo. You need Turbo C 2.0 Professional to

compile and assemble this package from scratch.

2. The command line now accepts several arguments that are best illustrated by

example:

net /m200 /s40 /d\ /autoexec.net

In this case, net is told to grab 200K of memory from MS-DOS for the heap, to

allow a maximum of 40 active sockets, and to make the directory prefix for all

files (configuration, domain.txt, mail spool files, etc) the root.  These

particular values all happen to be the defaults for their corresponding

parameters, so of course you'd get the same effect by just typing "net".

I decided that command line parameters were a better way to go than

environment variables. The latter don't necessarily exist on systems other

than MS-DOS and UNIX.  You can always create a batch (shell script) file to

invoke net with the parameters you want.

If you call net with the -d option, you can specify the parent directory of

all the various files used by net.

Interesting. Well, one thing you might try is giving NOS less memory than the

default, which is 200K for the heap. On my 640K system with no major TSRs

running (just MS-DOS, COMMAND.COM and TRW PC-2000 packet driver) this leaves

about 155K unused. (I also have a disk cache and ram disk, but they all run in

extended memory). Depending on what else you might have loaded, the problem

might well be as simple as running out of memory, e.g., if your driver tries

to request memory from DOS when it is all taken but doesn't bother checking

(or behaving rationally) when this happens.

To specify less memory, invoke NOS like this:

net /m100

That would cause it to grab only 100K.

NOS does not deliberately "barf" if it requests more memory space than was

available. It will simply use whatever is available. The only problem this

creates is when you try to shell out to run a subcommand; there won't be any

memory left to run command.com.

2. As you can probably tell from the foregoing example, NET is now large model

by default. The code will still compile and run under small model if you want.

In large model you can use as much memory as the system has available for

buffering, but the downside of this is that there may be none left for other

activities, e.g., running under a multitasker or shelling to a subprogram.

Hence the option of specifying how much memory should be used. You can now run

many more sessions, run much larger windows, etc, and it all works!

2. The domain receiver task is now started immediately after the very first

"domain addserver" command is executed. This allows subsequent lines in the

autoexec.net file to refer to domain names that aren't yet in domain.txt -- IF

sufficient routing entries are already in place to reach the domain server.

Formerly, the use in autoexec.net of any domain name that wasn't already in

domain.txt caused net to hang at startup. Note, however, that domain.txt still

needs a few "seed" entries if you use domain names for the "ip address" and

"domain addserver" commands, or for "route add" commands necessary to reach

the domain server.

The ip address must be set before the first "domain addserver" command.

1. A finger server has been added. It works similarly to the pre-NOS finger

server by KA7AXD *except* that the files under /finger do NOT have ".txt"

appended to them. If you finger "foo@machine.name", finger will send back the

contents of "/finger/foo".

2. I finally got tired of the schizophrenic MS-DOS use of either backslash (\)

or forward slash (/) as the path separator in file names. The code now deals

strictly with forward slashes just as though it's running under UNIX; MS-DOS

system calls that return pathnames now have any backslashes in their return

values immediately converted to forward slashes for consistency.

This requires ONE IMPORTANT CHANGE -- you must edit your /ftpusers file to

change all instances of backslash in the allowable prefix field to forward

slash, or you will not be able to access any files. The symptoms of not doing

this are that you will be allowed to log in via FTP, but every operation you

try will be rejected with a "permission denied" message -- even if you are

supposed to have full access to the system.

3. The session summary command ("session" without arguments) now shows any

active data connections that are associated with FTP sessions.

3. Added the "delete" and "rename" top-level commands.

Lately I've been receiving an increasing number of messages asking questions

about the UNIX ports of my code, particularly Xenix and AT&T Unix System V

(e.g., Microport).

I need to point out a few things:

1. I did not do these ports. In fact, all of my TCP/IP development work is

done on an AT clone running MS-DOS 3.3. That is the ONLY version of NET about

which I can answer questions.

2. As a matter of personal principle, I avoid all AT&T-based UNIX systems.

All of the UNIX systems I use run Berkeley or Berkeley-derived code (e.g.,

SunOS, Ultrix). Since all BSD-based UNIX systems already have full TCP/IP

support, there is little reason to run my code on those systems.

3. Bob Hoffman, hoffman@vax.cs.pittsburgh.edu, has graciously agreed to act as

coordinator of those working with UNIX ports of my code. Please direct any

questions about these versions to him, not to me.

1. The "remote" server and client are completely reworked. The "reset" and

"exit" functions no longer depend on the secrecy of the control port number,

but use a password text string instead. (Not that this makes things any more

secure against someone merely watching the channel, of course. Someday this

will be replaced with a cryptographic authenticator.)

I did this because I just added a "remote kick" command and I wanted to make

it available to everyone without also making the more disruptive "exit" and

"reset" functions also freely available. I therefore recommend that people use

the default port number (0x1234) whenever possible.

As before, there are no explicit acknowledgements to any of the "remote"

commands.

Here are a few sample commands:

net> remote pc.ka9q.ampr.org kick

Tell pc.ka9q.ampr.org to kick all TCP connections it has with me.

If there aren't any, do nothing.

net> remote -p 555 pc.ka9q.ampr.org kick

As above, but send the command to UDP port 555 on the remote system.

(You won't have to use this if everybody uses the default port

when starting the remote server.)

net> remote -a ka9q.bellcore.com pc.ka9q.ampr.org kick

Tell pc.ka9q.ampr.org to kick all TCP connections it may have with

ka9q.bellcore.com.

net> remote -k key pc.ka9q.ampr.org exit

Tell the copy of net.exe on pc.ka9q.ampr.org to exit. (If the

specified key is incorrect, the remote system will log that

fact and take no other action, giving you half a chance to detect

nasty people before they do actual damage.) "reset" may be

substituted for "exit"; this causes a cold reboot of the remote

system (MS-DOS only).

net> remote -s mykey

Set the key for the local system's "remote" server. (This command

does not generate any network packets.) NB: Until a key

is set for the local system, it will refuse ALL remote requests

except those for TCP kicks. I.e., you can allow remote kicks

while disallowing remote resets and exits altogether by not

setting a key.

The protocol formats for the new "remote" command are reasonably backward

compatible with the old. The commands are sent in UDP datagrams, as before,

with the first byte of the data field containing the command (1 == reset, 2 ==

exit, 3 == kick). In the case of reset or exit, the variable length key must

immediately follow the command byte.  For the kick command, an optional 4-byte

target IP address to kick follows the command byte; there is no key field. If

the target IP address is missing, the system will default to kicking the

source address in the UDP command packet.

This version includes the NET/ROM layer 3 and 4 protocols as integrated into

NOS by SM0RGV. I had to fix a few ANSI function prototypes and delint a few

other places, but I've adopted Anders' code pretty much as is. It seems to

work, although I did notice one occasion that the NET/ROM routing table got

garbled. I would like reports from others on how well this code works before

we put it into wide circulation.

I just fixed a typo in nr3.c that kept IP-over-NET/ROM from working, and I

also added a remote kick feature to SMTP. This latter Dfeature is coupled with

the remote tcp kick server I added the other week, in that now when you kick a

remote station it will not only do a TCP kick on any existing TCP connections

it may have with you, but it will also run through the SMTP queue and start up

a session for anything that's waiting for you.

This feature should be very useful to those who want to run mail gateways with

manually dialed SLIP links (with Tomcat being the prime example).  All you'll

have to do is dial the connection to Tom's machine and send it the remote kick

command (which is encapsulated in a UDP datagram).  His system will then

immediately send you any mail it has waiting for you.

I've updated the version of nos on flash. I incorporated some bug fixes to the

NET/ROM socket interface code from Anders, and I added a new command to the ip

layer: "ip rtimer". This allows you to set the IP level reassembly timeout

interval. This is the length of time that IP will keep the fragments of an

incomplete datagram around while waiting for additional fragments to arrive;

the reassembly timer is restarted each time a new fragment arrives.

The timeout value was formerly hard-coded at 30 seconds. This is now the

default for the new command, so if you do nothing the system will continue to

behave exactly as before. NB: the value for rtimer, as with other timer

values, is in MILLISECONDS.

I added this in response to some observations by WB0MPQ that indicated that IP

reassembly timeouts were occurring before subsequent fragments could be

delivered successfully over a NET/ROM network (yes, it's really that slow).

But be careful: if you set the timeout too long and you get a lot of

fragmentary datagrams that never complete, you'll tie up a lot of memory.

I'm uploading the version of NOS on flash.bellcore.com. This version supports

RIP, the interior routing protocol made popular by Berkeley UNIX.  To enable

the reception of RIP broadcasts, merely say

start rip

If you're not acting as a gateway to anyone, this is all you need do -- your

system will begin to passively monitor its interfaces for broadcast routing

packets and it will automatically add routes to the routing table. It may take

up to 30 seconds on an ethernet for the table to be built (this assumes a

broadcast rate of 30 seconds, which is standard on Ethernet). If you want to

get started faster, you can give an optional IP address to the command:

start rip [128.96.160.0]

This broadcasts a RIP "request" packet on the local subnet, which triggers

each gateway within earshot to send you its routing tables. (This requires

knowledge of the local IP broadcast address, plus an ARP entry for same. Read

on to find out about both of these things.)

If you are also acting as a gateway, then you'll want to enable the transmit

routines. First add ARP entries that map the local broadcast address for each

of your networks to the correct hardware address. For example, the IP

broadcast address on my shack Ethernet is 128.96.160.0, so I include the line

arp add [128.96.160.0] ethernet ff:ff:ff:ff:ff:ff

in my /autoexec.net file. Then I enable the broadcasting of routing info

with the line

rip add [128.96.160.0] ethernet 30 6

This means "broadcast your routing tables every 30 seconds on the interface

named 'ethernet', using IP destination address 128.96.160.0. Generate

triggered updates as necessary, and use the split horizon method."

On a packet radio channel, I might use the following lines:

arp add [44.64.0.0] com1 qst-0

rip add [44.64.0.0] com1 600 7

The last parameter in the "rip add" command is the flag parameter. It is the

sum of the following possible flag values:

1 - Include a host-specific route to yourself in each update. (Not needed

if you're already advertising a route to the network you're on.)

2 - Use split horizon updating; that is, omit all routing entries that

point to the interface being used for the broadcast. (This reduces the

chances of routing loops forming).

4 - Generate triggered updates as necessary on this interface, i.e.,

whenever a metric changes in the routing table, immediately generate

a broadcast on this interface with the changed entries. If split horizon

(bit 2) is also set, use "poisoned reverse", i.e., for any routing

table entries that point to this interface, include them with an

infinite metric (RIP defines 16 to be infinity) instead of leaving them

out as happens during a normal routing broadcast when split horizon

is set.  Triggered updates helps spread the word faster when links

fail, reducing the chances of a temporary loop forming.

For Ethernet, I recommend a flag value of 6 (triggered updates and split

horizon). On SLIP links, use 6 or 7, depending on whether you need to emit a

host-specific route to yourself. On a packet radio channel where everyone is

not fully connected (the usual case), use 7 and DON'T advertise any routes to

the subnet unless you can reach everyone in that subnet.

(A packet radio channel is best modeled as a collection of point-to-point

links since it rarely fits the model of a fully interconnected subnet like

Ethernet.) Note that you don't have to broadcast your routes; you can direct

them to a specific set of stations by using their names or addresses in the

"rip add" command. You can have any number of such commands per interface,

with the only limit being channel traffic overhead.

An experimental parameter is available to control something I call "route

merging". If you say "rip merge on", then an incoming route that is more

specific than one you already have in your table is ignored if they both point

to the same gateway. For example, if you already have a default route that

points to gateway "foobar", then any route that arrives from gateway foobar

will be ignored because to put it in the table would not cause any change in

the routing of packets -- they'd still go to foobar anyway.

Properly used, this should save a lot of routing table space.

You can trace the automatic routing messages and controls by the "rip trace"

command; it takes a numeric parameter. "rip trace 0" disables tracing, "rip

trace 1" generates messages only when routes change, and "rip trace 2" shows

you everything, even when things are stable.

If you want to ignore routing broadcasts from a certain gateway (e.g., because

it can't hear you), say

rip refuse hostname

where 'hostname' is the domain name or [ip address] of the offending gateway.

To reverse this, just say

rip accept hostname

This manual filtering will probably do until I'm able to devise some sort of

link connectivity algorithm and protocol that can analyze the quality of the

path between your station and each of your neighbors so routing packets can be

filtered automatically. I have an idea of how to do this -- by broadcasting

periodic status packets containing received and transmitted packet count

statistics and using such packets to determine how well you're being heard.

Ooops, I think I made a minor error in my discussion of RIP over radio.  The

flag setting for RIP broadcasts over a radio channel should probably be 5, not

7. You don't want to use split horizon over a radio subnetwork unless it is

internally fully connected, since there may well be relay routes that enter

and leave an intermediate station by the same interface. Split horizon would

suppress such routes, which would preclude same-channel relay hops.

I've updated the NOS code on flash. I've been doing a lot of work on the RIP

code to fix bugs and make things more stable. I changed the command syntax for

a couple of the rip commands; look at the help messages or read the source for

details until I can document things further.

Note! This version REQUIRES that you set the default local IP address (i.e.,

issue the "ip addr" command) BEFORE you attach any interfaces. So edit your

autoexec.net file appropriately.

The reason for this requirement is the major change in this version: each

interface can now have its own IP address. An interface's IP address can be

specified in the attach command, but if it is omitted the IP address given

earlier in the "ip address" command will be used by default. (Hence the

requirement given above. I considered doing away with the "ip address" command

altogether and making the ip address argument on each attach command required,

but this would have been a more disruptive change.)

This architectural change brings the NET package into line with standard

Internet practice. This is mainly for the benefit of those who use NET as a

router between Ethernets in a regular Internet environment, where the other

hosts and routers lack NET's extensions to the standard Internet routing and

subnetting facilities. Although it was possible to make the earlier code work

in such situations through a combination of proxy arp and static routes, it

was a little unclean. With this version, the machine can be set up to act just

like a conventional Internet router and multi-homed host.

I.e., if you have NET running with several interfaces, each with its own

unique IP address, the system will accept packets addressed to any of its

interfaces' addresses. When the system originates packets, the local address

appropriate for the sending interface will be used, all in accordance with

standard Internet practice. Note that when you do a "tcp stat" the local IP

addresses for connections in LISTEN state are 0.0.0.0; this is normal. The

appropriate IP address is used when the connection is actually opened.  The

BSD constant "INADDR_ANY" is provided in a header file, and it can be used

just as it is in BSD when binding local socket addresses in an application.

Note that you are definitely NOT required to have a unique IP address for each

interface. If you specify the same IP address for each interface (or allow

each interface to use the global default set with the "ip address" command),

then the package behaves just as it did before.  Therefore this change is

largely irrelevanft to most AMPRNET users. It is also irrelevant whose NET

configurations have only a single interface (i.e., all end-user systems that

don't operate as gateways).

I've added a new command, "iface", that lists the interfaces on the system

along with their parameters (e.g., IP address, send and receive packet counts,

etc.)  There's also a "domain listservers" command that displays the list of

domain servers along with various statistics measured for each.

Subject: lost first characters

I haven't investigated Tom's specific problem, but I've seen similar things in

the past and it's almost always been due to null characters inbedded in the

stream being sent to the terminal. The ANSI.SYS and NANSI.SYS drivers both

display a null (0) character exactly as though it were a space, and this I

consider a bug. Every other terminal I've ever seen treats null as a no-op.

One of these days I'll dig into the NANSI.SYS source and fix it.

1. KY3B's port of PE1CHL's generic 8530 driver. Ken did all the work of

porting Rob's driver to NOS. I took his code, reformatted and delinted it a

bit and added ANSI-style function prototypes, but I have not actually tested

it. Those of you using DRSI cards with slow speed modems, please give this

driver a try and let us know if it works. There's a flag in config.h for

configuring out this code if you don't need it and want to save the space.

2. I added an option in config.h for turning off the mailbox code.  When the

mailbox has been configured out, the following commands are removed:

mbox

start telnet/netrom/ax25

stop telnet/netrom/ax25

The "mbox y" command is gone. It didn't seem to make sense to keep it around

given the presence of a separate "ttylink" server that goes straight to the

console. So whenever the mailbox is configured into the system the telnet,

netrom and ax25 servers all get the mailbox automatically. One less thing you

have to remember to put in your autoexec.net files...

I'm currently updating the NOS on flash. This incorporates the latest mbox and

netrom patches by Anders, the latest DRSI driver from Stu, and the

vector-saving fix to ec.c suggested by Ron Henderson. The only change I made

myself was to add an #ifdef MAILBOX in fingerd.c around a reference to the

mailbox code.

ALIAS

#

# SMTP server ALIAS file. This is for resolving a given target address into

# a single or multiple entry mail list.

# Format:

#    mail_list_name call_1@host_1 [call_2@host_2]......# comments

#

bob     gb3xp@gb3xp.ampr

colinw  g3tne@g3tne.ampr

colinr  g8kwi@g8kwi.ampr

G1PCD   g1pcd%gb3kp@gb3xp

G1WKK   g1wkk@g1wkk.ampr

G3RRA   g3rra@g3rra.ampr

G3TNE   g3tne@g3tne.ampr

G8GGI   gb3xp@gb3xp.ampr

G8KWI   g8kwi%gb7wok@gb3xp.ampr

g8kwi   g8kwi@g8kwi.ampr

GB3XP   gb3xp@gb3xp.ampr

hilary  g1eml@g1emm.ampr.org

ian     g3rra@g3rra.ampr

jim     g1wkk@g1wkk.ampr

john    g5ds%gb3kp@gb3xp

kelvin  g1emm@g1emm.ampr.org

mike    g8atk%gb7wok@gb3xp

paul    g1pcd%gb3kp@gb3xp

world   g3rra@g3rra.ampr g3tne@g3tne.ampr gb3xp@gb3xp.ampr

g1wkk@g1wkk.ampr g8kwi@g8kwi.ampr g6kvk%gb7spv@gb3xp.ampr

g1ttg@g1ttg.ampr g6vug@g6vug.ampr g6kqz@g6kqz.ampr

g4uqe@g4uqe.ampr g8ogr@g8ogr.ampr g4xwv@g4xwv.ampr

g1plt@g1plt.ampr

locals  g3rra@g3rra.ampr g1wkk@g1wkk.ampr g8kwi@g8kwi.ampr

g4bio@g4bio.ampr g1plt@g1plt.ampr gb3xp@gb3xp.ampr

g4uqe@g4uqe.ampr g6vug@g6vug.ampr g6kqz@g6kqz.ampr

ted     gb3kp%gb3kp@gb3xp

ron     g6vug@g6vug.ampr

tim     g4uqe@g4uqe.ampr

gareth  g6kvk%gb7spv@gb3xp.ampr

bolton  g4xhs%gb7crg@gb3xp.ampr

AUTOEXEC.NET

#                          A U T O E X E C . N E T

#                          -----------------------

#

#  This is the configuration file for the NOS.EXE program...

#  this file *MUST* be in the root directory ( \ ) of your current disk

#  when NOS.EXE is run if you want NOS to use is as the default file.

#

#  Note: NOS.EXE ignores all lines beginning with a pound sign (#).

#

#    There are many commands which must be provided to NOS.EXE

#    each time it is started to configure the program.  To keep

#    from having to type them by hand each time, we put them in

#    this file, which is read each time NOS starts up.  Commands

#    in this configuration file are entered exactly as they would

#    be typed at the keyboard in the program.

#

#    We've tried to give you intelligent default values for each of

#    the commands below.  You need to read completely through this

#    file, changing things as you go.  If you're not sure what to

#    do with a command, try leaving it like it is!  Once you become

#    familiar with the package, you'll no doubt find things to change.

#    Once you're up and running and are all set, you can delete some or

#    all of the comment lines (the ones that start with '#') to save

#    disk space and make the program load faster, if you wish.

#

#-----------------------------------------------

#

#    Turn off the internal stopwatch functions as on the Archimedes they

#    don't half hassle the pc emulators speaker driver code..and it BEEPS!

#

watch off

#

#-----------------------------------------------

#

#    Firstly, load up the memory database with all the remote nodes

#    identifications. The default file to use is \domain.txt but this

#    is very slow to access if the memory database is not invoked.

#

domain load

#

#-----------------------------------------------

#

#    Just in case we don't have a host name entry in our domain file

#    let's ask our domain server at g1emm2 to try and resolve unknowns.

#    This is currently VERY dangerous as it may stall the command

#    interpreter or worse still - the timer process!! => dead in the H2O!

#

# domain addserver g1emm2

#

#-----------------------------------------------

#

# The Domain timer backoff can be either linear or binary exponential.

# Realisically, the linear backoff is vital for Amateur radio work.

#

domain timertype linear

#

#-----------------------------------------------

#

# The Domain default suffix for host names without a '.' is '.ampr.org.'

# and the domain.txt file has only names of the format:-

# <host>.ampr. (CNAME) and <host>.ampr.org. (A) in it.

#

domain suffix ampr.org.

#

#-----------------------------------------------

#

# This entry tells NOS.EXE the name of your machine. Your hostname will

# show up in mail headers, etc.

#

hostname g1emm.ampr.org

#

#-----------------------------------------------

#

# The next line does the same thing that "MYCALL" did in your AX.25

# TNC... it identifies the callsign you're using on the air in AX.25

# packets. It also defines the callsign of your net/rom node!!!

#

ax25 mycall g1emm-1

#

#-----------------------------------------------

#

# This one contains your IP address. This can either be in the form

# [44.131.x.y] where x and y are your alloted number values or it can

# use the domain file information and will then take a symbolic node

# name and convert it internally to your IP address. The [] are required

# if you wish to give an absolute address.

#

ip address g1emm

#

#-----------------------------------------------

#

# The buffers command sets the number of buffers and thier sizes in

# the interrrupt memory handler cache. Ax25 doesn't use them at all

# but just in case, here are a few.

#

buffers 5 256

#

#------------------------------------------------

#

# The attach command tells NOS.EXE about the interfaces in your computer

# that you will be using for TCP/IP.  These can include normal IBM-type

# serial "comm ports", Ethernet controllers, or dedicated packet cards.

#

# The syntax is:

# attach <hw type> <I/O address> <vector> <mode> <label> <bufsize>

#        <mtu> [<speed>] [IP addr]

#

# COM1: / 9600 baud KISS connection for use with KISS TNC's.

#

#attach asy 0x3f8 4 ax25 tnc 2048 256 19200

#

# COM2: / 1200 baud KISS connection for use with KISS TNC's.

#

attach asy 0x2f8 3 ax25 tnc 2048 256 19200

#

# COM1: / 9600 baud slip link to another computer

#

#attach asy 0x3f8 4 slip sl0 8092 576 9600

#

#-------------------------------------------------

#

# The following entry defines the broadcast address for

# the interface labeled 'tnc' in the attach command.

#

ifconfig tnc broadcast [44.131.0.0]

#

#-------------------------------------------------

#

# The KISS computer-to-tnc protocol includes a "command packet" that

# you can use to twiddle TNC parameters like TXDELAY, TXTAIL, PERSIST,

# SLOTTIME and DUPLEX.  Since most KISS implementations include good

# default values, you shouldn't have to use this feature.

# All TNC's were not created equal, so you *MUST* read the docs for the

# KISS implementation for your TNC before twiddling!

#

# The format is:  param <label> <hex value> <hex value> <hex value> ...

#

param tnc 1 20

param tnc 2 16

param tnc 3 10

param tnc 4 10

param tnc 5 0

#

#-------------------------------------------------

#

# The following entry tells the program to route all packets out

# the interface labeled 'tnc' in the attach command.

#

route add default tnc

#

#-------------------------------------------------

#

# Time To Live is the maximum number of hops a packet can take

# before it is thrown away. This command prevents an inadvertent

# infinite loop from occuring with packets in the network.

#

ip ttl 10

#

#-------------------------------------------------

#

# The Maximum Segment Size is the largest single transmission that

# you will send. An mss of 216 corresponds to the mtu of 256 set

# up in the attach command above, overhead bytes considered.

#

tcp mss 216

#

#-------------------------------------------------

#

# Initial Retry TimeouT (irtt) is the time between retries before

# the corrected timers start to function in units of seconds.

# The default is 65 seconds but may be adjusted as required.

#

tcp irtt 65000

#

#-------------------------------------------------

#

# The TCP timer backoff can be either linear or binary exponential.

# Realisically, the linear backoff is vital for Amateur radio work.

#

tcp timertype linear

#

#-------------------------------------------------

#

# The Window parameter establishes the maximum number of bytes

# that may be outstanding before your system expects an ack.

# If window is twice as big as mss, for example, there will be two

# active packets on the channel at any given time... large values of

# window provide improved throughput on full-duplex links, but are a

# problem on the air.  Keep  mss <= window <= 2*mss if you're on the air.

#

#

tcp window 432

#

#-------------------------------------------------

#

# This entry will open 'logfile' in the \spool directory and will

# record the server activity of your system.  If you don't want a log,

# comment out this line... if you do, make sure you have a \spool

# directory!

#

log \spool\logfile

#

#-------------------------------------------------

#

# Each of the servers (services you will provide) must be turned

# on before they will be active. The following entries turn all

# of them on. To turn any function off use the command 'stop' after

# NOS gets fired up, or just comment out the line here.

#

start ax25

start netrom

start echo

start discard

start telnet

start ftp

start smtp

start finger

start remote

start rip

start ttylink

#

#--------------------------------------------------

#

# Cause the SMTP server to look for new mail to send every 10 minutes

#

smtp timer 600

#

#--------------------------------------------------

#

# Set the remote server password field

#

remote -s zardoz

#

#--------------------------------------------------

#

# Cause the SMTP server to send undeliverable mail to gb3xp for forwarding

#

smtp gateway gb3xp

#

#--------------------------------------------------

#

# Cause the SMTP server to queue mail for SMTP mailer or ext. mailer

#

smtp mode route

#

#--------------------------------------------------

#

# These values come into play when you are operating in the

# AX.25 mode, along with your ax25 mycall entry above.

#

# If you want to operate as a digipeater (for those poor souls

# not operating TCP/IP!), change 'off' to 'on'.

#

ax25 digipeat on

#

#-------------------------------------------------

#

# Use AX25 protocol version 2 as most people have now migrated from 1.

#

ax25 version 2

#

#---------------------------------------------------

#

# Maxframe sets the max number of frames allowed to remain

# unacknowledged. Cannot be greater than 7. Low values are best.

#

ax25 maxframe 3

#

#---------------------------------------------------

#

# Maximum size of packet which will have its data resent with the POLL

# bit set on retry, else, just RR (p) instead of I (p)

#

ax25 pthresh 128

#

#---------------------------------------------------

#

# Paclen limits the size of I=fields.

#

ax25 paclen 256

#

#---------------------------------------------------

#

# Retry limits the number of unsuccessful retransmissions.

#

ax25 retry 5

#

#---------------------------------------------------

#

# Blimit limits the number of incremental retransmission timeouts.

#

ax25 blimit 5

#

#---------------------------------------------------

#

# Set up the AX25 initial round-trip timer in milliseconds.

# The old T1 value is now taken to be 2*irtt and is variable.

#

ax25 irtt 2500

#

#---------------------------------------------------

#

# Set up the AX25 Link Keep-Alive timer T3 in milli-seconds.

#

ax25 t3 65000

#

#---------------------------------------------------

#

# Set up the AX25 Link Redundancy timer T4 in seconds.

#

ax25 t4 300

#

#-------------------------------------------------

#

# The AX25 timer backoff can be either linear or binary exponential.

# Realisically, the linear backoff is vital for Amateur radio work.

#

ax25 timertype linear

#

#---------------------------------------------------

#

# ax25 window sets the number of bytes that can be pending on an

# AX.25 receive queue before sending RNRs back to sender.

#

ax25 window 2048

#

#---------------------------------------------------

#

# Start tracing the previously defined `tnc' line with 111 which is

# trace all input and output packets and dump in ASCII only.

# Value pattern is:-

#       B:D:I:O

#       | | | |__Output packets 0 or 1          1 = trace / 0 = don't

#       | | |____Input packets  0 or 1          1 = trace / 0 = don't

#       | |______Dump level     0 or 1 or 2     0=headers / 1=ascii / 2=hex

#       |________Broadcasts     0 or 1          0 = trace / 1 = don't

#

#

#trace tnc 0111

#

#---------------------------------------------------

#

# Set up the NET/ROM pseudo device so that NOS knows that netrom is used.

#

attach netrom

#

#---------------------------------------------------

#

# Tell NET/ROM what line to use and what we are called, ie. our alias.

# Also define a default quality factor for incoming NET/ROM broadcasts.

#

netrom interface tnc farnhm 10

#

#---------------------------------------------------

#

# Set NET/ROM broadcast reception to only specifically named nodes.

#

netrom nodefilter mode accept

#

#---------------------------------------------------

#

# Set the NET/ROM inter-broadcast time to 30 minutes

#

netrom nodetimer 1800

#

#---------------------------------------------------

#

# Set the NET/ROM obsolesence timer to 30 minutes. Each time it fires

# we will derate the nodes in our tables. A non-refreshed node broadcast

# will vanish in about 3 hours then.

#

netrom obsotimer 1800

#

#---------------------------------------------------

#

# Set NET/ROM mode to switch rather then endpoint. We will tell the world

# about the nodes we can hear and not just ourself.

#

netrom verbose yes

#

#---------------------------------------------------

#

# Set NET/ROM minimum acceptable incoming broadcast level before we will

# build the nodes into our tables. Keeps out longshots!

#

netrom minquality 10

#

#---------------------------------------------------

#

# NET/ROM Time To Live is the maximum number of hops a packet can take

# before it is thrown away. This command prevents an inadvertent

# infinite loop from occuring with packets in the netrom network.

#

netrom ttl 15

#

#---------------------------------------------------

#

# Set NET/ROM acknowledge timer value in milliseconds

#

netrom acktime 3000

#

#---------------------------------------------------

#

# Set NET/ROM maximum number of bytes on the netrom queues (I think!)

#

netrom qlimit 1024

#

#---------------------------------------------------

#

# Set NET/ROM maximum number of retries before disconnecting

#

netrom retries 5

#

#---------------------------------------------------

#

# Set NET/ROM initial round-trip timer in milli-seconds.

#

netrom irtt 60000

#

#---------------------------------------------------

#

# Set NET/ROM promiscuous acceptance of broadcasts at minimum quality.

#

netrom promiscuous y

#

#-------------------------------------------------

#

# The NET/ROM timer backoff can be either linear or binary exponential.

# Realisically, the linear backoff is vital for Amateur radio work.

#

netrom timertype linear

#

#---------------------------------------------------

#

# Let the world know that we are alive and kicking. Three broadcasts

# just to make sure that most people will be able to catch one of them!

#

netrom bcnodes tnc

netrom bcnodes tnc

netrom bcnodes tnc

#

#---------------------------------------------------

#

# Use I frames for ax25 traffic instead of UI frames.

#

mode tnc vc

#

#---------------------------------------------------

#

# Use I frames for NET/ROM traffic instead of UI frames. (Is there a choice?)

#

mode netrom vc

#

#---------------------------------------------------

#

# Initial state ot the system is that the is no manual operator present.

#

attended no

#

#---------------------------------------------------

#

# Bring in all the variable parameters relating to the outside world.

#

# First bring in all the remote NET/ROM nodes for filtering.

#

source \nodes.fil

#

# Next bring in all the remote Tcp/Ip nodes ARP definitions

#

source \nodes.arp

#

# Lastly bring in all the remote Tcp/Ip nodes route definitions.

#

source \nodes.rou

#

#---------------------------------------------------

#

# Define the RIP setup and request a broadcast to load the tables.

#

rip merge n

#

rip add [44.131.0.0] 900 6

#

rip refuse g4hpe

rip refuse g1maa

#

rip request [44.131.0.0]

rip request [44.131.0.0]

rip request [44.131.0.0]

#

#---------------------------------------------------

#

# Wake up the SMTP process to start shipping any outstanding mail.

#

smtp kick

#

#---------------------------------------------------

#

# Announce our start up is complete and the time we took to do it.

#

motd "Sorry, I'm not in at the moment. Please feel free to leave a message. 73, Kelv."

status

#

#---------------------------------------------------

#

# THE END

#

BBSFWD.INF

From wb0mpq@wb0mpq.ampr.org Sat Nov 17 03:01:55 1990

Received: from wb0mpq.ampr.org by n2mh.ampr.org with SMTP

id AA2277 ; Sat, 17 Nov 90 02:07:58 GMT

Date: Sat, 17 Nov 90 01:48:43 GMT

Message-Id: <19297@wb0mpq.ampr.org>

From: wb0mpq@wb0mpq.ampr.org (Al Shjarback - Warren,NJ 07060)

To: n2mh@n2mh.ampr.org

Subject: Forwarding 1

>From nn2z@nn2z Wed Oct 10 23:09:51 1990

Received: from pc.nn2z.ampr.org by wb0mpq.ampr.org with SMTP

id AA16293 ; Wed, 10 Oct 90 22:55:43 GMT

Received: from nn2z.ampr.org by pc.nn2z.ampr.org with SMTP

id AA6110 ; Wed, 10 Oct 90 18:30:16 EDT

Received: by nn2z.ampr.org (5.59/smail2.5/08-26-89)

id AA17857; Wed, 10 Oct 90 18:42:39 EDT

Received: from ucsd.edu by thumper.bellcore.com (4.1/4.7)

id AA10039; Wed, 10 Oct 90 18:44:55 EDT

Received: by ucsd.edu; id AA11758

sendmail 5.64/UCSD-2.1-sun

Wed, 10 Oct 90 13:55:35 -0700 for hpbbrd!db0sao!dg4scv-2!tcp-group

Received: from sics.se by ucsd.edu; id AA11743

sendmail 5.64/UCSD-2.1-sun via SMTP

Wed, 10 Oct 90 13:55:27 -0700 for /usr/lib/sendmail -oc -odb -oQ/var/spool/lqueue -oi -ftcp-group-relay tcp-group-list

Received: from isis.sics.se by sics.se (5.61-bind 1.5+ida/SiteCap-3.0)

id AA21441; Wed, 10 Oct 90 21:55:11 +0100

Received: from localhost by isis.sics.se (5.61-bind 1.5+ida/SiteCap-3.0)

id AA07844; Wed, 10 Oct 90 21:55:05 +0100

Message-Id: <9010102055.AA07844@isis.sics.se>

To: tcp-group@ucsd.edu

Cc: phealy@cs.tcd.ie

Subject: NOS Mbox forwarding

Date: Wed, 10 Oct 90 21:55:00 +0100

From: klemets@sics.se

I have been hacking on the NOS mailbox recently. Paul Healy, EI9GL, wrote some

good forwarding code for NOS. I started changing Pauls code, and I am not sure

if he will recognize it now.

The NOS mailbox now supports forwarding, both outgoing forwarding and reverse

forwarding. It keeps track of which bulletins have been forwarded by adding

"X-Forwarded-To:" lines to the messages. Messages in public "areas" are sent

as bulletins, with BID and everything. Messages from private areas are treated

as private mail and are deleted after forwarding.

The message areas can be checked for messages to forward at a regular

intervals, which is set with the "mbox timer" command. Forwarding can be

started immediately with the "mbox kick" command. There is also a "mbox motd"

and a "mbox attended" command, which I picked from the G1EMM code.

There is a new command in the mailbox too, "Verbose." It displays a message

with all it header lines, contrary to "Read" which only shows four lines. It

is also possible to read a message simply by typing its number.

The mailbox reads a forwarding file, "spool/forward.bbs". Here is a sample

file:

sk0tm 0006

ax25 ax0 sk0tm

sk0tm

amsat

all

------

sm0rgv

netrom #sth675

..c sm0rgv-2

sm0rgv

amsat

The first word on the first line in a forwarding record is the name of the BBS

to forward to. This should be the same type of name which is shown by the

"mbox status" command. The second word is optional. It specifies a range when

forwarding may take place. "0006" means that there will only be forwarding to

this station between midnight and 6am.

The second line specifies how to establish the connection. It should start

with the protocol (ax25, tcp or netrom) and be followed by all the parameters

which are necessary when NOS has to establish a network connection.

Directly after the second line there may be lines that start with a dot. What

follows after the dot will be sent to the remote BBS as soon as the connection

is established.

Then follows the names of a number of message areas, public or private.

Finally, there should be a couple of '-' signs to separate one forwarding

record from another.

The code seems to be working, although there could of course be bugs left.

Anyway, the source is available with ftp from sics.se. The filename is

archive/packet/ka9q/nos/mailbox.arc.

As usual, this message is the only documentation, so it could be worth saving.

Anders

From wb0mpq@wb0mpq.ampr.org Sat Nov 17 04:15:14 1990

Received: from wb0mpq.ampr.org by n2mh.ampr.org with SMTP

id AA2278 ; Sat, 17 Nov 90 03:16:45 GMT

Date: Sat, 17 Nov 90 01:49:06 GMT

Message-Id: <19299@wb0mpq.ampr.org>

From: wb0mpq@wb0mpq.ampr.org (Al Shjarback - Warren,NJ 07060)

To: n2mh@n2mh.ampr.org

Subject: Forwarding 2

>From nn2z@nn2z Mon Oct 15 15:08:12 1990

Received: from pc.nn2z.ampr.org by wb0mpq.ampr.org with SMTP

id AA16697 ; Mon, 15 Oct 90 14:57:51 GMT

Received: from nn2z.ampr.org by pc.nn2z.ampr.org with SMTP

id AA6493 ; Mon, 15 Oct 90 10:24:34 EDT

Received: by nn2z.ampr.org (5.59/smail2.5/08-26-89)

id AA01219; Mon, 15 Oct 90 10:35:34 EDT

Received: from ucsd.edu by thumper.bellcore.com (4.1/4.7)

id AA01720; Mon, 15 Oct 90 10:31:56 EDT

Received: by ucsd.edu; id AA00355

sendmail 5.64/UCSD-2.1-sun

Mon, 15 Oct 90 04:29:50 -0700 for hb9zz

Received: from sics.se by ucsd.edu; id AA00339

sendmail 5.64/UCSD-2.1-sun via SMTP

Mon, 15 Oct 90 04:29:38 -0700 for /usr/lib/sendmail -oc -odb -oQ/var/spool/lqueue -oi -ftcp-group-relay tcp-group-list

Received: from sarapis.sics.se by sics.se (5.61-bind 1.5+ida/SiteCap-3.0)

id AA09221; Mon, 15 Oct 90 12:29:26 +0100

Received: from localhost by sarapis.sics.se (5.61-bind 1.4+ida/SiteCap-3.0)

id AA06837; Mon, 15 Oct 90 11:56:45 +0100

Message-Id: <9010151056.AA06837@sarapis.sics.se>

To: "JAMES C MANKIN (814)-863-4348" <N5X@psuvm.psu.edu>

Cc: tcp-group@ucsd.edu

Subject: Re: NOS Mbox forwarding

In-Reply-To: Your message of Sun, 14 Oct 90 15:36:00 -0400.

<9010141936.AA05617@sics.se>

Date: Mon, 15 Oct 90 11:56:41 +0100

From: klemets@sics.se

James,

(I am Cc:ing this to tcp-group.)

>   Thanks, changing that line solved the problem.  Another thing I notice now

> is that the nos mailbox doesnt disconnect cleanly after forwarding, it seems

> to do a reset because the connection disappears from the ax status display

> without an ax25 disconnect packet being sent.

This is really odd, has anybody else noticed this?

>  Is there a way to send a message that is for a user at a bbs, rather than

> the bbs's owner?  I can do it if I edit the .txt file in the mail directory

> to have the To: address I want, so my problem is how does one get a message

> for kb3kj @ w3ya  to be placed in the w3ya  mail area.  There must be

> something I am overlooking.

When a message is entered to the mailbox, the address is rewritten according

to rules in the /spool/rewrite file. The first word on each line in the

rewrite file is an address using wildcards. If that address matches the

address you are trying to rewrite, it is rewritten according to the format of

the second word. If the third word is an 'r', the rewrite procedure will be

repeated, now using the new address as input.

Ok, so here is a sample /spool/rewrite file for a machine with the hostname

sk0we.ampr.org:

*!* $2@$1 r

*@sk0we.ampr.org $1 r

*@*.ampr.org $1@$2.ampr.org

sm0rgv@* sm0rgv

*@w3ya* w3ya

*@k* $1%k$2%w3iwi.ampr.org@tomcat.gsfc.nasa.gov

The first line means that addresses written in the "host!user" format should

be rewritten to "user@host" and then the rewrite procedure should be repeated.

The second line converts the address "user@sk0we.ampr.org" into the local

address "user" and then repeats the rewrite procedure.

The third line seems to do nothing. It just converts "user@host.ampr.org" to

"user@host.ampr.org". But it also exits the rewrite procedure since the line

does not end with an 'r'. Any address ending with ".ampr.org" will not be

converted further.

The line "sm0rgv@* sm0rgv" will keep mail for sm0rgv on the local machine even

if it is mistakenly addressed to another BBS, such as "sm0rgv@w3iwi". The line

will however fail to convert addresses such as "sm0rgv@w3iwi.ampr.org" because

of the line described in the previous paragraph.

The line "*@w3ya* w3ya" will leave mail for "user@w3ya" in the local message

area named "w3ya". The same goes for mail using some sort of hierarchical

routing designators such as "user@w3ya.state.na.us". The only exception is for

mail addressed to "user@w3ya.ampr.org" as described above.

The line "*@k* $1%k$2%w3iwi.ampr.org@tomcat.gsfc.nasa.gov" is an example of

how to establish an Internet "wormhole." Suppose I want to forward all mail to

stations with callsigns starting with K to W3IWI through the Internet. The

line quoted above would do it. But if the mail for "user@kxxxx" is received by

w3iwi.ampr.org with SMTP, there might be some problems because the rewrite

file is not scanned when mail is received with SMTP. This could easily be

changed however. Let me know if this is causing you any inconveniencies.

Anders

BM.RC

# configuration file for Bdale's Mailer... format is:

#        host <space> this_host_name

#        user <space> this_user_name

#        fullname <space> your full name for mail headers (optional)

#        reply <space> your reply address if not this machine (optional)

#                  useful for pc on large network off smart hosts

#        smtp <space> path to mailboxes          default /spool/mail

#        edit <space> path your editor (optional)

#        mbox <space> name of file used as default for s command (optional)

#        folder <space> name of directory for save mailby s command (optional)

#        record <space> name of file save a copy of sent mail (optional)

#        maxlet <space> max number of message in mbox ( optional default 300)

#        screen direct | bios video mode ( turbo C only default direct )

#

host g1emm.ampr.org

user g1emm

fullname Kelvin Hill - IO91oe - PSTN (0252) 725864

reply g1emm@g1emm.ampr.org

screen direct

edit \tcpip\qe.exe

smtp /spool/mail

mbox /folder/mbox

record /folder/outmail

folder /folder

maxlet 400

DOMAIN.TXT

#Phil Chapman, nr Manchester, ?

g4lhx.ampr.org. IN      A       44.131.1.1

g4lhx.ampr.     IN      CNAME   g4lhx.ampr.org.

#Nigel Watkinson, Altrincham, IO83uj

g1ula.ampr.org. IN      A       44.131.1.2

g1ula.ampr.     IN      CNAME   g1ula.ampr.org.

#Roger James, Morecambe, IO84nb

g4tot.ampr.org. IN      A       44.131.1.3

g4tot.ampr.     IN      CNAME   g4tot.ampr.org.

#Micheal Lord, Congleton, IO83ls

g3zau.ampr.org. IN      A       44.131.1.4

g3zau.ampr.     IN      CNAME   g3zau.ampr.org.

#David ?, Southport, IO83mp

g4tup.ampr.org. IN      A       44.131.1.5

g4tup.ampr.     IN      CNAME   g4tup.ampr.org.

#Chris McMahon, Blackpool, IO83lu

g6fci.ampr.org. IN      A       44.131.1.6

g6fci.ampr.     IN      CNAME   g6fci.ampr.org.

#Neil ?, Caton, IO87b

g3uvq.ampr.org. IN      A       44.131.1.7

g3uvq.ampr.     IN      CNAME   g3uvq.ampr.org.

#John Heaton, Bolton, IO83sn

g1yyh.ampr.org. IN      A       44.131.1.8

g1yyh.ampr.     IN      CNAME   g1yyh.ampr.org.

#Alban ?, ?, ?

g8nvw.ampr.org. IN      A       44.131.1.9

g8nvw.ampr.     IN      CNAME   g8nvw.ampr.org.

#Ian Marchant, Congleton, IO83ve

g1rsw.ampr.org. IN      A       44.131.1.10

g1rsw.ampr.     IN      CNAME   g1rsw.ampr.org.

#Brian Lewis, Ingleton,

g4vps.ampr.org. IN      A       44.131.1.11

g4vps.ampr.     IN      CNAME   g4vps.ampr.org.

#John Greenwood, Bolton, IO83vi

g4xhs.ampr.org. IN      A       44.131.1.12

g4xhs.ampr.     IN      CNAME   g4xhs.ampr.org.

#Mike Skyner, Sandbach, IO83ud

g4ght.ampr.org. IN      A       44.131.1.13

g4ght.ampr.     IN      CNAME   g4ght.ampr.org.

#North West Digipeater, Altrincham,

gb7nwp.ampr.org.        IN      A       44.131.1.14

gb7nwp.ampr.    IN      CNAME   gb7nwp.ampr.org.

#Martin Smith, Poyton, ?

g4fik.ampr.org. IN      A       44.131.1.15

g4fik.ampr.     IN      CNAME   g4fik.ampr.org.

#Alan Yates, Stockport, ?

g3wec.ampr.org. IN      A       44.131.1.16

g3wec.ampr.     IN      CNAME   g3wec.ampr.org.

#Ken Wood, Northwich, IO83rh

g3wcs.ampr.org. IN      A       44.131.1.17

g3wcs.ampr.     IN      CNAME   g3wcs.ampr.org.

#Ian Springle, Congleton, IO83vd

g1opb.ampr.org. IN      A       44.131.1.18

g1opb.ampr.     IN      CNAME   g1opb.ampr.org.

#Dave ?, Penrith, ?

g6bsk.ampr.org. IN      A       44.131.1.19

g6bsk.ampr.     IN      CNAME   g6bsk.ampr.org.

#Alan Abbott, ?, IO83vi

g1gop.ampr.org. IN      A       44.131.1.20

g1gop.ampr.     IN      CNAME   g1gop.ampr.org.

#Dave Wrigley, Rochdale, IO83vo

g6gxk.ampr.org. IN      A       44.131.1.21

g6gxk.ampr.     IN      CNAME   g6gxk.ampr.org.

#Steve Cumberlidge, Hoylake, ?

g7bbf.ampr.org. IN      A       44.131.1.22

g7bbf.ampr.     IN      CNAME   g7bbf.ampr.org.

#Neil ?, ?, ?

g4oar.ampr.org. IN      A       44.131.1.23

g4oar.ampr.     IN      CNAME   g4oar.ampr.org.

#Brian ?, Hoylake, ?

g7bbg.ampr.org. IN      A       44.131.1.24

g7bbg.ampr.     IN      CNAME   g7bbg.ampr.org.

#Phil ?, ?, ?

g0jsb.ampr.org. IN      A       44.131.1.25

g0jsb.ampr.     IN      CNAME   g0jsb.ampr.org.

#Chris ?, ?, ?

g0dvv.ampr.org. IN      A       44.131.1.26

g0dvv.ampr.     IN      CNAME   g0dvv.ampr.org.

#John ?, ?, ?

g8uzz.ampr.org. IN      A       44.131.1.27

g8uzz.ampr.     IN      CNAME   g8uzz.ampr.org.

#Richard ?, ?, ?

g8ndd.ampr.org. IN      A       44.131.1.28

g8ndd.ampr.     IN      CNAME   g8ndd.ampr.org.

#Paul ?, ?, ?

g4dly.ampr.org. IN      A       44.131.1.29

g4dly.ampr.     IN      CNAME   g4dly.ampr.org.

#Jeremy ?, ?, ?

g4noy.ampr.org. IN      A       44.131.1.30

g4noy.ampr.     IN      CNAME   g4noy.ampr.org.

#Julian Ross, Winsford, IO83re

g0lbo.ampr.org. IN      A       44.131.1.31

g0lbo.ampr.     IN      CNAME   g0lbo.ampr.org.

#Hans Field, Winsford, IO83re

g4xfd.ampr.org. IN      A       44.131.1.32

g4xfd.ampr.     IN      CNAME   g4xfd.ampr.org.

#Brian Heslop, Stockport, IO83??

g7ame.ampr.org. IN      A       44.131.1.33

g7ame.ampr.     IN      CNAME   g7ame.ampr.org.

#Andy ?, Bury, ?

g8lir.ampr.org. IN      A       44.131.1.35

g8lir.ampr.     IN      CNAME   g8lir.ampr.org.

#Ron ?, Bury, ?

g4shc.ampr.org. IN      A       44.131.1.36

g4shc.ampr.     IN      CNAME   g4shc.ampr.org.

#John Tumath, Wallasey, ?

g1dbc.ampr.org. IN      A       44.131.1.38

g1dbc.ampr.     IN      CNAME   g1dbc.ampr.org.

#Eddie Wilkinson, Urmston, IO83tk

g4xht.ampr.org. IN      A       44.131.1.39

g4xht.ampr.     IN      CNAME   g4xht.ampr.org.

#Alec Butterworth, Shaw, IO83wn

g4uqc.ampr.org. IN      A       44.131.1.40

g4uqc.ampr.     IN      CNAME   g4uqc.ampr.org.

#Tony Walker, Urmston, IO83tk

g4xdr.ampr.org. IN      A       44.131.1.41

g4xdr.ampr.     IN      CNAME   g4xdr.ampr.org.

#David Tait, Manchester, IO83uk

g0jvy.ampr.org. IN      A       44.131.1.42

g0jvy.ampr.     IN      CNAME   g0jvy.ampr.org.

#Chris Baron, Gt Harwood, IO83ts

g6pwy.ampr.org. IN      A       44.131.1.43

g6pwy.ampr.     IN      CNAME   g6pwy.ampr.org.

#Roger Bulcock, Bury, ?

g6fuq.ampr.org. IN      A       44.131.1.44

g6fuq.ampr.     IN      CNAME   g6fuq.ampr.org.

#Steven Tucker, Sale, ?

g6eld.ampr.org. IN      A       44.131.1.45

g6eld.ampr.     IN      CNAME   g6eld.ampr.org.

#Andrew ?, Staybridge, ?

g6efi.ampr.org. IN      A       44.131.1.46

g6efi.ampr.     IN      CNAME   g6efi.ampr.org.

#Peter ?, Bury, ?

g8aen.ampr.org. IN      A       44.131.1.47

g8aen.ampr.     IN      CNAME   g8aen.ampr.org.

#Gary ?, Radcliffe, ?

g6xky.ampr.org. IN      A       44.131.1.48

g6xky.ampr.     IN      CNAME   g6xky.ampr.org.

#Andy Anderson, Atherton, IO83sm

g6ybc.ampr.org. IN      A       44.131.1.49

g6ybc.ampr.     IN      CNAME   g6ybc.ampr.org.

#Duncan Berry, Leigh, IO83rl

g8jnf.ampr.org. IN      A       44.131.1.50

g8jnf.ampr.     IN      CNAME   g8jnf.ampr.org.

#Dave ?, Bolton, IO83sn

g3tmz.ampr.org. IN      A       44.131.1.51

g3tmz.ampr.     IN      CNAME   g3tmz.ampr.org.

#Dennis Freedman, Manchester, IO83uk

g3vsh.ampr.org. IN      A       44.131.1.52

g3vsh.ampr.     IN      CNAME   g3vsh.ampr.org.

#Brian Curran, Hale, IO83ti

g4oqe.ampr.org. IN      A       44.131.1.53

g4oqe.ampr.     IN      CNAME   g4oqe.ampr.org.

#Bernie ?, Southport, IO83lp

g7dem.ampr.org. IN      A       44.131.1.54

g7dem.ampr.     IN      CNAME   g7dem.ampr.org.

#Les ?, Blackpool, ?

g0ffd.ampr.org. IN      A       44.131.1.55

g0ffd.ampr.     IN      CNAME   g0ffd.ampr.org.

#Roy Morrison, Hyde, ?

g6ckn.ampr.org. IN      A       44.131.1.56

g6ckn.ampr.     IN      CNAME   g6ckn.ampr.org.

#Frank Beesley, Hyde, ?

g8cze.ampr.org. IN      A       44.131.1.57

g8cze.ampr.     IN      CNAME   g8cze.ampr.org.

#Mike Dent, Heysham, IO84nb

g6phf.ampr.org. IN      A       44.131.1.58

g6phf.ampr.     IN      CNAME   g6phf.ampr.org.

#John Greenwood, Bolton, IO83vi

g4xhs2.ampr.org.        IN      A       44.131.1.59

g4xhs2.ampr.    IN      CNAME   g4xhs2.ampr.org.

#Donald Appleton, Wigan, IO83pn

g4kcp.ampr.org. IN      A       44.131.1.60

g4kcp.ampr.     IN      CNAME   g4kcp.ampr.org.

#Dave Staniforth, Heysham, IO84nb

g6crv.ampr.org. IN      A       44.131.1.61

g6crv.ampr.     IN      CNAME   g6crv.ampr.org.

#Peter Jones, Widnes, ?

g1xtj.ampr.org. IN      A       44.131.1.62

g1xtj.ampr.     IN      CNAME   g1xtj.ampr.org.

#David Pryle, Wigan, IO83pm

g7eea.ampr.org. IN      A       44.131.1.63

g7eea.ampr.     IN      CNAME   g7eea.ampr.org.

#Steve Cumberlidge, Moreton, IO83kj

g7bbf2.ampr.org.        IN      A       44.131.1.64

g7bbf2.ampr.    IN      CNAME   g7bbf2.ampr.org.

#Alan ?, Moreton, IO83kj

g1xyp.ampr.org. IN      A       44.131.1.66

g1xyp.ampr.     IN      CNAME   g1xyp.ampr.org.

#Derek ?, ?, ?

g7cwe.ampr.org. IN      A       44.131.1.67

g7cwe.ampr.     IN      CNAME   g7cwe.ampr.org.

#Harold Heath, Ripon, IO94cd

g1oqq.ampr.org. IN      A       44.131.2.1

g1oqq.ampr.     IN      CNAME   g1oqq.ampr.org.

#John Kirk, Leyburn, IO94bg

g4lpq.ampr.org. IN      A       44.131.2.2

g4lpq.ampr.     IN      CNAME   g4lpq.ampr.org.

#Malcolm Newsome, Stillington, IO94kc

g6guw.ampr.org. IN      A       44.131.2.3

g6guw.ampr.     IN      CNAME   g6guw.ampr.org.

#Dave Lockwood, Wakefield, IO93fp

g4cli.ampr.org. IN      A       44.131.2.4

g4cli.ampr.     IN      CNAME   g4cli.ampr.org.

#Phil Green, Sheffield, ?

g4phl.ampr.org. IN      A       44.131.2.5

g4phl.ampr.     IN      CNAME   g4phl.ampr.org.

#Martin Taylor, Leconfield, IO93rv

g3vxn.ampr.org. IN      A       44.131.2.6

g3vxn.ampr.     IN      CNAME   g3vxn.ampr.org.

#Martin Taylor, Leconfield, IO93rv

test.g3vxn.ampr.org.    IN      A       44.131.2.7

test.g3vxn.ampr.        IN      CNAME   test.g3vxn.ampr.org.

#Steve Collins, Leeds, IO93fu

g0fcw.ampr.org. IN      A       44.131.2.8

g0fcw.ampr.     IN      CNAME   g0fcw.ampr.org.

#Mike Parks, Leeds,

g4upd.ampr.org. IN      A       44.131.2.9

g4upd.ampr.     IN      CNAME   g4upd.ampr.org.

#Keith ?, Leeds, ?

g4vrw.ampr.org. IN      A       44.131.2.10

g4vrw.ampr.     IN      CNAME   g4vrw.ampr.org.

#Wakefield Digipeater, Wakefield,

gb7yax.ampr.org.        IN      A       44.131.2.11

gb7yax.ampr.    IN      CNAME   gb7yax.ampr.org.

#Kevin Hawkins, Huddersfield, IO93cp

g8kio.ampr.org. IN      A       44.131.2.12

g8kio.ampr.     IN      CNAME   g8kio.ampr.org.

#Robert Brown, Grimsby, IO93wi

g1sco.ampr.org. IN      A       44.131.2.13

g1sco.ampr.     IN      CNAME   g1sco.ampr.org.

#Rob Bartholomew, Sheffield, IO93hi

g1brv.ampr.org. IN      A       44.131.2.14

g1brv.ampr.     IN      CNAME   g1brv.ampr.org.

#Jim Lodge, Leeds, IO93et

g6xpc.ampr.org. IN      A       44.131.2.15

g6xpc.ampr.     IN      CNAME   g6xpc.ampr.org.

#Ken Collins, Harrogate, IO94fa

g4fip.ampr.org. IN      A       44.131.2.16

g4fip.ampr.     IN      CNAME   g4fip.ampr.org.

#Mike Johnson, Northallerton, IO94gi

g0gck.ampr.org. IN      A       44.131.2.17

g0gck.ampr.     IN      CNAME   g0gck.ampr.org.

#Lloyd Delapp, Harrogate, IO93fx

g0lbu.ampr.org. IN      A       44.131.2.18

g0lbu.ampr.     IN      CNAME   g0lbu.ampr.org.

#Darley ARC, Darley, IO94da

g0fos.ampr.org. IN      A       44.131.2.19

g0fos.ampr.     IN      CNAME   g0fos.ampr.org.

#Ken ?, Leeds, ?

g4oww.ampr.org. IN      A       44.131.2.20

g4oww.ampr.     IN      CNAME   g4oww.ampr.org.

#Doug Shore, Huddersfield, IO93cp

g0eoj.ampr.org. IN      A       44.131.2.21

g0eoj.ampr.     IN      CNAME   g0eoj.ampr.org.

#GB7EXP Gateway, Huddersfield, IO93cp

gb7exp.ampr.org.        IN      A       44.131.2.22

gb7exp.ampr.    IN      CNAME   gb7exp.ampr.org.

#John Goodwin, Sheffield, IO93hi

g6cnx.ampr.org. IN      A       44.131.2.23

g6cnx.ampr.     IN      CNAME   g6cnx.ampr.org.

#Richard Horton, Harrogate, IO93fx

g3xwh.ampr.org. IN      A       44.131.2.24

g3xwh.ampr.     IN      CNAME   g3xwh.ampr.org.

#Bill Dunbar, Harrogate, IO93fx

g4puk.ampr.org. IN      A       44.131.2.25

g4puk.ampr.     IN      CNAME   g4puk.ampr.org.

#Mick Taylor, Allerton Bywate, ?

g0gya.ampr.org. IN      A       44.131.2.26

g0gya.ampr.     IN      CNAME   g0gya.ampr.org.

#John Goodwin, Sheffield, IO93hi

g6cnx.ampr.org. IN      A       44.131.2.27

g6cnx.ampr.     IN      CNAME   g6cnx.ampr.org.

#Andy McMullin, Edgbaston, ?

g8tqh.ampr.org. IN      A       44.131.3.1

g8tqh.ampr.     IN      CNAME   g8tqh.ampr.org.

#Neil Rimmer, Daventry, ?

g4jty.ampr.org. IN      A       44.131.3.2

g4jty.ampr.     IN      CNAME   g4jty.ampr.org.

#Adrian Booth, Tamworth, ?

g4jbx.ampr.org. IN      A       44.131.3.3

g4jbx.ampr.     IN      CNAME   g4jbx.ampr.org.

#?, Tamworth, ?

g8osx.ampr.org. IN      A       44.131.3.4

g8osx.ampr.     IN      CNAME   g8osx.ampr.org.

#R Evans, Lichfield, ?

g8khv.ampr.org. IN      A       44.131.3.5

g8khv.ampr.     IN      CNAME   g8khv.ampr.org.

#Andy Witts, Wolverhampton, IO82vo

g1dil.ampr.org. IN      A       44.131.3.6

g1dil.ampr.     IN      CNAME   g1dil.ampr.org.

#Hayden Bate, Sutton Coldfiel, IO92bn

g8amd.ampr.org. IN      A       44.131.3.7

g8amd.ampr.     IN      CNAME   g8amd.ampr.org.

#Martin Cole, Coventry, IO92gj

g0dkr.ampr.org. IN      A       44.131.3.8

g0dkr.ampr.     IN      CNAME   g0dkr.ampr.org.

#Paul Fisher, Coventry, IO92gk

g6zxd.ampr.org. IN      A       44.131.3.9

g6zxd.ampr.     IN      CNAME   g6zxd.ampr.org.

#Steve ?, Rugby, ?

g8lyb.ampr.org. IN      A       44.131.3.10

g8lyb.ampr.     IN      CNAME   g8lyb.ampr.org.

#Robin ?, Sutton Coldfied, IO92cn

g1ahn.ampr.org. IN      A       44.131.3.11

g1ahn.ampr.     IN      CNAME   g1ahn.ampr.org.

#Brian Lonnon, B'Ham, IO92dm

g3zum.ampr.org. IN      A       44.131.3.12

g3zum.ampr.     IN      CNAME   g3zum.ampr.org.

#Mike Martin, Nuneaton, IO92gl

g4vrq.ampr.org. IN      A       44.131.3.13

g4vrq.ampr.     IN      CNAME   g4vrq.ampr.org.

#Mike Tubby, Worcester, IO82vf

g8tic.ampr.org. IN      A       44.131.3.14

g8tic.ampr.     IN      CNAME   g8tic.ampr.org.

#Jonathan Naylor, Wirksworth, IO93fb

g4klx.ampr.org. IN      A       44.131.4.1

g4klx.ampr.     IN      CNAME   g4klx.ampr.org.

#Mike Wilson, Lincoln, IO93ve

g4gou.ampr.org. IN      A       44.131.4.2

g4gou.ampr.     IN      CNAME   g4gou.ampr.org.

#William Armitage, Nottingham, ?

g8rwk.ampr.org. IN      A       44.131.4.5

g8rwk.ampr.     IN      CNAME   g8rwk.ampr.org.

#Nottingham University, Nottimgham, ?

ux.g3unu.ampr.org.      IN      A       44.131.4.6

ux.g3unu.ampr.  IN      CNAME   ux.g3unu.ampr.org.

#Nottingham University, Nottimgham, ?

pc.g3unu.ampr.org.      IN      A       44.131.4.7

pc.g3unu.ampr.  IN      CNAME   pc.g3unu.ampr.org.

#Nottingham University, Nottimgham, ?

am.g3unu.ampr.org.      IN      A       44.131.4.8

am.g3unu.ampr.  IN      CNAME   am.g3unu.ampr.org.

#Mick Allen, Long Eaton, IO92iv

g4xmh.ampr.org. IN      A       44.131.4.9

g4xmh.ampr.     IN      CNAME   g4xmh.ampr.org.

#GB7LED Digipeater, Long Eaton, IO92iv

gb7led.ampr.org.        IN      A       44.131.4.10

gb7led.ampr.    IN      CNAME   gb7led.ampr.org.

#Andy Soars, Loughborough, IO92js

g4vcn.ampr.org. IN      A       44.131.4.11

g4vcn.ampr.     IN      CNAME   g4vcn.ampr.org.

#Len Fennelow, Wisbech, JO02cq

g0dwi.ampr.org. IN      A       44.131.5.1

g0dwi.ampr.     IN      CNAME   g0dwi.ampr.org.

#Ian Wade, nr Bedford, IO91sx

g3nrw.ampr.org. IN      A       44.131.5.2

g3nrw.ampr.     IN      CNAME   g3nrw.ampr.org.

#Tim Haynes, Luton, ?

g8ptp.ampr.org. IN      A       44.131.5.3

g8ptp.ampr.     IN      CNAME   g8ptp.ampr.org.

#David Brooke, Over nr Cambrid, ?

g6gzh.ampr.org. IN      A       44.131.5.4

g6gzh.ampr.     IN      CNAME   g6gzh.ampr.org.

#Bob Compton, nr Sandy, IO92vc

g1zpu.ampr.org. IN      A       44.131.5.5

g1zpu.ampr.     IN      CNAME   g1zpu.ampr.org.

#Dwight Ernest, Toddington, ?

g0kcu.ampr.org. IN      A       44.131.5.6

g0kcu.ampr.     IN      CNAME   g0kcu.ampr.org.

#Ian Bultitude, Biggleswade, IO92vc

g6khw.ampr.org. IN      A       44.131.5.7

g6khw.ampr.     IN      CNAME   g6khw.ampr.org.

#Dave Bailey, Ely, ?

g0gwo.ampr.org. IN      A       44.131.5.8

g0gwo.ampr.     IN      CNAME   g0gwo.ampr.org.

#Tony Stimson, Banbury, IO92ia

g1bje.ampr.org. IN      A       44.131.5.9

g1bje.ampr.     IN      CNAME   g1bje.ampr.org.

#Bob Lewin, Banbury, IO92ia

g1ydy.ampr.org. IN      A       44.131.5.10

g1ydy.ampr.     IN      CNAME   g1ydy.ampr.org.

#Paul ?, Towcester, ?

g4bki.ampr.org. IN      A       44.131.5.11

g4bki.ampr.     IN      CNAME   g4bki.ampr.org.

#Paul Young, Northampton, ?

g0hwc.ampr.org. IN      A       44.131.5.12

g0hwc.ampr.     IN      CNAME   g0hwc.ampr.org.

#Jon ?, Northampton, ?

g7cia.ampr.org. IN      A       44.131.5.13

g7cia.ampr.     IN      CNAME   g7cia.ampr.org.

#Chris Smith, Biggleswade, IO92vc

g1fef.ampr.org. IN      A       44.131.5.14

g1fef.ampr.     IN      CNAME   g1fef.ampr.org.

#Peter Robinson, Cambridge, JO02be

g3mrx.ampr.org. IN      A       44.131.5.15

g3mrx.ampr.     IN      CNAME   g3mrx.ampr.org.

#Richard Baker, Cambridge, ?

g6fvb.ampr.org. IN      A       44.131.5.16

g6fvb.ampr.     IN      CNAME   g6fvb.ampr.org.

#Phil ?, Earith, ?

g4bik.ampr.org. IN      A       44.131.5.17

g4bik.ampr.     IN      CNAME   g4bik.ampr.org.

#Mike ?, Godmanchester, ?

g3tej.ampr.org. IN      A       44.131.5.18

g3tej.ampr.     IN      CNAME   g3tej.ampr.org.

#Mike Hibbert, Milton, ?

g6coq.ampr.org. IN      A       44.131.5.19

g6coq.ampr.     IN      CNAME   g6coq.ampr.org.

#John Young, St Ives, IO92xi

g4bxn.ampr.org. IN      A       44.131.5.20

g4bxn.ampr.     IN      CNAME   g4bxn.ampr.org.

#Julian Tether, Towcester, IO92mc

g6loh.ampr.org. IN      A       44.131.5.21

g6loh.ampr.     IN      CNAME   g6loh.ampr.org.

#Hazel Tether, Towcester, IO92mc

g1nod.ampr.org. IN      A       44.131.5.22

g1nod.ampr.     IN      CNAME   g1nod.ampr.org.

#Julian Tether, Towcester, IO92mc

g7abi.ampr.org. IN      A       44.131.5.23

g7abi.ampr.     IN      CNAME   g7abi.ampr.org.

#Chris Osborn, Huntingdon, IO92vh

g4uxv.ampr.org. IN      A       44.131.5.24

g4uxv.ampr.     IN      CNAME   g4uxv.ampr.org.

#Mike Ferriday, Reading, IO91lk

g1nqw.ampr.org. IN      A       44.131.6.1

g1nqw.ampr.     IN      CNAME   g1nqw.ampr.org.

#Slough College, Slough, IO91qm

g6cgd.ampr.org. IN      A       44.131.6.2

g6cgd.ampr.     IN      CNAME   g6cgd.ampr.org.

#Paul Taylor, High Wycombe, IO91po

g1plt.ampr.org. IN      A       44.131.6.3

g1plt.ampr.     IN      CNAME   g1plt.ampr.org.

#Paul Shayler, Slough, IO91rm

g6tsf.ampr.org. IN      A       44.131.6.4

g6tsf.ampr.     IN      CNAME   g6tsf.ampr.org.

#Andy Chivers, Penn, IO91pp

g4bio.ampr.org. IN      A       44.131.6.5

g4bio.ampr.     IN      CNAME   g4bio.ampr.org.

#Kevin Quinlan, Hazlemere, IO91pp

g7dzd.ampr.org. IN      A       44.131.6.6

g7dzd.ampr.     IN      CNAME   g7dzd.ampr.org.

#Andy Scott, Chesham, IO91qr

g4tnu.ampr.org. IN      A       44.131.6.7

g4tnu.ampr.     IN      CNAME   g4tnu.ampr.org.

#Andrew ?, Milton Keynes, ?

g8ugc.ampr.org. IN      A       44.131.6.8

g8ugc.ampr.     IN      CNAME   g8ugc.ampr.org.

#Colin ?, Hazlemere, IO91pc

g1lla.ampr.org. IN      A       44.131.6.9

g1lla.ampr.     IN      CNAME   g1lla.ampr.org.

#Peter ?, Olney, IO92od

g1zqn.ampr.org. IN      A       44.131.6.10

g1zqn.ampr.     IN      CNAME   g1zqn.ampr.org.

#? ?, ?, ?

g6xqb.ampr.org. IN      A       44.131.6.11

g6xqb.ampr.     IN      CNAME   g6xqb.ampr.org.

#Mike ?, Hanslope, IO92oc

g1ert.ampr.org. IN      A       44.131.6.12

g1ert.ampr.     IN      CNAME   g1ert.ampr.org.

#Chris Sillence, Bletchley, IO91px

g6wzd.ampr.org. IN      A       44.131.6.13

g6wzd.ampr.     IN      CNAME   g6wzd.ampr.org.

#Paul Taylor, High Wycombe, IO91po

sys2.g1plt.ampr.org.    IN      A       44.131.6.14

sys2.g1plt.ampr.        IN      CNAME   sys2.g1plt.ampr.org.

#Clive Payne, High Wycombe, Bucks, IO91pp

g0gnh.ampr.org. IN      A       44.131.6.15

g0gnh.ampr.     IN      CNAME   g0gnh.ampr.org.

#?, ?, ?

g4pzk.ampr.org. IN      A       44.131.6.251

g4pzk.ampr.     IN      CNAME   g4pzk.ampr.org.

#New Malden Digipeater, New Malden, IO91vj

gb3xp.ampr.org. IN      A       44.131.7.1

gb3xp.ampr.     IN      CNAME   gb3xp.ampr.org.

#Bob Geddes, New Malden, IO91vj

relay.g8ggi.ampr.org.   IN      A       44.131.7.2

relay.g8ggi.ampr.       IN      CNAME   relay.g8ggi.ampr.org.

#Bob Geddes, New Malden, IO91vj

g8ggi.ampr.org. IN      A       44.131.7.3

g8ggi.ampr.     IN      CNAME   g8ggi.ampr.org.

#Edward Batts, Kingston, IO91uk

g8lwy.ampr.org. IN      A       44.131.7.4

g8lwy.ampr.     IN      CNAME   g8lwy.ampr.org.

#Kingston Digipeater, Kingston, IO91uk

gb3kp.ampr.org. IN      A       44.131.7.5

gb3kp.ampr.     IN      CNAME   gb3kp.ampr.org.

#Kelvin Hill, Farnham, IO91oe

g1emm.ampr.org. IN      A       44.131.7.6

g1emm.ampr.     IN      CNAME   g1emm.ampr.org.

#Paul Dicken, Cobham, IO91ti

g1pcd.ampr.org. IN      A       44.131.7.7

g1pcd.ampr.     IN      CNAME   g1pcd.ampr.org.

#John Danks, Surbiton,

g5ds.ampr.org.  IN      A       44.131.7.8

g5ds.ampr.      IN      CNAME   g5ds.ampr.org.

#Don Fields, Henley on Thame, IO91mm

g3xtt.ampr.org. IN      A       44.131.7.9

g3xtt.ampr.     IN      CNAME   g3xtt.ampr.org.

#John Baker, Reigate, IO91vf

g0hwo.ampr.org. IN      A       44.131.7.10

g0hwo.ampr.     IN      CNAME   g0hwo.ampr.org.

#Colin Wantling, Camberley, IO91ph

g3tne.ampr.org. IN      A       44.131.7.11

g3tne.ampr.     IN      CNAME   g3tne.ampr.org.

#Ian Matheson, Camberley, IO91ph

g3rra.ampr.org. IN      A       44.131.7.12

g3rra.ampr.     IN      CNAME   g3rra.ampr.org.

#Justin Brickwood, Camberley, IO91ph

g1ttg.ampr.org. IN      A       44.131.7.13

g1ttg.ampr.     IN      CNAME   g1ttg.ampr.org.

#?, Camberley, ?

g8kwi.ampr.org. IN      A       44.131.7.14

g8kwi.ampr.     IN      CNAME   g8kwi.ampr.org.

#Tim Fitzgerald, Camberley, IO91pi

g4uqe.ampr.org. IN      A       44.131.7.15

g4uqe.ampr.     IN      CNAME   g4uqe.ampr.org.

#John Holton, Farnham, IO91of

g8ogr.ampr.org. IN      A       44.131.7.16

g8ogr.ampr.     IN      CNAME   g8ogr.ampr.org.

#Mike Willis, Guildford, IO91QF

g6vdo.ampr.org. IN      A       44.131.7.17

g6vdo.ampr.     IN      CNAME   g6vdo.ampr.org.

#Kelvin Hill, Farnham, IO91oe

g1emm2.ampr.org.        IN      A       44.131.7.18

g1emm2.ampr.    IN      CNAME   g1emm2.ampr.org.

#Ignatius Tan, Catford, ?

g7gbq.ampr.org. IN      A       44.131.7.19

g7gbq.ampr.     IN      CNAME   g7gbq.ampr.org.

#Ignatius Tan, Catford, ?

pc.g7gbq.ampr.org.      IN      A       44.131.7.20

pc.g7gbq.ampr.  IN      CNAME   pc.g7gbq.ampr.org.

#Keith Brazington, Rochester, JO01gj

g4lzv.ampr.org. IN      A       44.131.8.1

g4lzv.ampr.     IN      CNAME   g4lzv.ampr.org.

#Ashford Digipeater, Ashford, ?

gb7ck.ampr.org. IN      A       44.131.8.2

gb7ck.ampr.     IN      CNAME   gb7ck.ampr.org.

#Keith Brazington, Rochester, JO01gj

g4lzv2.ampr.org.        IN      A       44.131.8.3

g4lzv2.ampr.    IN      CNAME   g4lzv2.ampr.org.

#Smudge Lundegard, Swanley, JO01di

g3gjw.ampr.org. IN      A       44.131.8.4

g3gjw.ampr.     IN      CNAME   g3gjw.ampr.org.

#Dave Turner, Ashford, ?

g4idx.ampr.org. IN      A       44.131.8.5

g4idx.ampr.     IN      CNAME   g4idx.ampr.org.

#Sue Squibb, Teynham, JO1ji

g1tzu.ampr.org. IN      A       44.131.8.6

g1tzu.ampr.     IN      CNAME   g1tzu.ampr.org.

#Geoff Squibb, Teynham, JO01ji

g4mpa.ampr.org. IN      A       44.131.8.6

g4mpa.ampr.     IN      CNAME   g4mpa.ampr.org.

#Tony Kempton, Bromley, JO01aj

g1bys.ampr.org. IN      A       44.131.8.7

g1bys.ampr.     IN      CNAME   g1bys.ampr.org.

#John Wellard, Canterbury, JO01oh

g6zaa.ampr.org. IN      A       44.131.8.8

g6zaa.ampr.     IN      CNAME   g6zaa.ampr.org.

#Phil Coull, Folkestone,

g3xvy.ampr.org. IN      A       44.131.8.9

g3xvy.ampr.     IN      CNAME   g3xvy.ampr.org.

#Quentin North, Brighton,

g1ikj.ampr.org. IN      A       44.131.8.10

g1ikj.ampr.     IN      CNAME   g1ikj.ampr.org.

#Rob ?, Dover, ?

g4fxe.ampr.org. IN      A       44.131.8.11

g4fxe.ampr.     IN      CNAME   g4fxe.ampr.org.

#Derek ?, Bromley, JO01ak

g1hsn.ampr.org. IN      A       44.131.8.12

g1hsn.ampr.     IN      CNAME   g1hsn.ampr.org.

#Bill Green, Biggin Hill, JO01ah

g3tdg.ampr.org. IN      A       44.131.8.13

g3tdg.ampr.     IN      CNAME   g3tdg.ampr.org.

#Tony Kempton, Bromley, JO01aj

g1bys-1.ampr.org.       IN      A       44.131.8.14

g1bys-1.ampr.   IN      CNAME   g1bys-1.ampr.org.

#Christopher Leigh, Beckenham, ?

g1vsl.ampr.org. IN      A       44.131.8.15

g1vsl.ampr.     IN      CNAME   g1vsl.ampr.org.

#Nick Lawes, Crayford, JO01dj

g8zhr.ampr.org. IN      A       44.131.8.16

g8zhr.ampr.     IN      CNAME   g8zhr.ampr.org.

#Ray Euden, Erith, ?

g6jha.ampr.org. IN      A       44.131.8.17

g6jha.ampr.     IN      CNAME   g6jha.ampr.org.

#Nigel Mundy, Burgess Hill, IO90wx

g1tdm.ampr.org. IN      A       44.131.8.18

g1tdm.ampr.     IN      CNAME   g1tdm.ampr.org.

#Mike Mundy, Burgess Hill, IO90wx

g0gnv.ampr.org. IN      A       44.131.8.19

g0gnv.ampr.     IN      CNAME   g0gnv.ampr.org.

#Burgess Hill Digipeater, Burgess Hill, IO90wx

gb7zzz.ampr.org.        IN      A       44.131.8.20

gb7zzz.ampr.    IN      CNAME   gb7zzz.ampr.org.

#Chris Kenny, Hove, IO90vu

g3ljk.ampr.org. IN      A       44.131.8.21

g3ljk.ampr.     IN      CNAME   g3ljk.ampr.org.

#Peter Meiring, Plymouth, IO70wj

g0bsx.ampr.org. IN      A       44.131.9.1

g0bsx.ampr.     IN      CNAME   g0bsx.ampr.org.

#Barry Cresswell, Plymouth, IO70wh

g1mrg.ampr.org. IN      A       44.131.9.2

g1mrg.ampr.     IN      CNAME   g1mrg.ampr.org.

#Simon Rodda, Penzance,

g4pem.ampr.org. IN      A       44.131.9.3

g4pem.ampr.     IN      CNAME   g4pem.ampr.org.

#Iain Dunsworth, Saltash, IO79vj

g4snl.ampr.org. IN      A       44.131.9.4

g4snl.ampr.     IN      CNAME   g4snl.ampr.org.

#Stuart Charles, Plymouth, IO70wj

g8nvw.ampr.org. IN      A       44.131.9.5

g8nvw.ampr.     IN      CNAME   g8nvw.ampr.org.

#John Veale, Plymouth, ?

g4sca.ampr.org. IN      A       44.131.9.6

g4sca.ampr.     IN      CNAME   g4sca.ampr.org.

#Major ?, Beer, ?

g1dii.ampr.org. IN      A       44.131.9.7

g1dii.ampr.     IN      CNAME   g1dii.ampr.org.

#Ray ?, Launceston,

g6ujb.ampr.org. IN      A       44.131.9.8

g6ujb.ampr.     IN      CNAME   g6ujb.ampr.org.

#Fred Bourne, Plymouth, IO70vj

g3yjq.ampr.org. IN      A       44.131.9.9

g3yjq.ampr.     IN      CNAME   g3yjq.ampr.org.

#Nick Smith, Plymouth, IO70wj

g7auq.ampr.org. IN      A       44.131.9.10

g7auq.ampr.     IN      CNAME   g7auq.ampr.org.

#Peter Meiring, Plymouth, IO70wj

pc.g0bsx.ampr.org.      IN      A       44.131.9.11

pc.g0bsx.ampr.  IN      CNAME   pc.g0bsx.ampr.org.

#Tony Glover, Plymouth, IO70wj

g0dwk.ampr.org. IN      A       44.131.9.12

g0dwk.ampr.     IN      CNAME   g0dwk.ampr.org.

#John Bewley, Plymouth, IO70vj

g1sqi.ampr.org. IN      A       44.131.9.13

g1sqi.ampr.     IN      CNAME   g1sqi.ampr.org.

#John Woods, Plymouth, ?

g3tgr.ampr.org. IN      A       44.131.9.14

g3tgr.ampr.     IN      CNAME   g3tgr.ampr.org.

#Peter ?, Exeter, ?

g7agm.ampr.org. IN      A       44.131.9.15

g7agm.ampr.     IN      CNAME   g7agm.ampr.org.

#Allan Baker, Plymouth, ?

g3kfn.ampr.org. IN      A       44.131.9.16

g3kfn.ampr.     IN      CNAME   g3kfn.ampr.org.

#Tony King, Plymouth, IO70wj

g7any.ampr.org. IN      A       44.131.9.17

g7any.ampr.     IN      CNAME   g7any.ampr.org.

#Dave Morgan, Exmouth, IO80hp

g7bwo.ampr.org. IN      A       44.131.9.18

g7bwo.ampr.     IN      CNAME   g7bwo.ampr.org.

#Ken ?, ?, ?

g4mpq.ampr.org. IN      A       44.131.9.19

g4mpq.ampr.     IN      CNAME   g4mpq.ampr.org.

#Tony Rider, Ipplepen, ?

g6glp.ampr.org. IN      A       44.131.9.20

g6glp.ampr.     IN      CNAME   g6glp.ampr.org.

#Phil ?, ?, ?

g0kdt.ampr.org. IN      A       44.131.9.21

g0kdt.ampr.     IN      CNAME   g0kdt.ampr.org.

#David Reed, Plymouth, ?

g4ydr.ampr.org. IN      A       44.131.9.22

g4ydr.ampr.     IN      CNAME   g4ydr.ampr.org.

#Mike Waterfall, Redruth, ?

g8nxd.ampr.org. IN      A       44.131.9.23

g8nxd.ampr.     IN      CNAME   g8nxd.ampr.org.

#Peter ?, St Tudy, ?

g4axc.ampr.org. IN      A       44.131.9.24

g4axc.ampr.     IN      CNAME   g4axc.ampr.org.

#Ray ?, Exeter, ?

g3ybk.ampr.org. IN      A       44.131.9.25

g3ybk.ampr.     IN      CNAME   g3ybk.ampr.org.

#Tim ?, Launceston, ?

g1zlh.ampr.org. IN      A       44.131.9.26

g1zlh.ampr.     IN      CNAME   g1zlh.ampr.org.

#Maurice ?, ?, ?

g3iue.ampr.org. IN      A       44.131.9.27

g3iue.ampr.     IN      CNAME   g3iue.ampr.org.

#Joe Jones, Plymouth, ?

g1rxr.ampr.org. IN      A       44.131.9.28

g1rxr.ampr.     IN      CNAME   g1rxr.ampr.org.

#Graham Jones, Swansea, ?

gw4uck.ampr.org.        IN      A       44.131.10.1

gw4uck.ampr.    IN      CNAME   gw4uck.ampr.org.

#Ray Gordon, Swansea, ?

gw1xrn.ampr.org.        IN      A       44.131.10.2

gw1xrn.ampr.    IN      CNAME   gw1xrn.ampr.org.

#Univ College Swansea, Swansea, ?

gw3uws.ampr.org.        IN      A       44.131.10.3

gw3uws.ampr.    IN      CNAME   gw3uws.ampr.org.

#Richard Artym, Swansea, ?

gw7exm.ampr.org.        IN      A       44.131.10.4

gw7exm.ampr.    IN      CNAME   gw7exm.ampr.org.

#Gavin Stewart, Belfast, IO74an

gi4tvv.ampr.org.        IN      A       44.131.15.1

gi4tvv.ampr.    IN      CNAME   gi4tvv.ampr.org.

#Ivan Andrews, Chelmsford, JO01mr

g6aal.ampr.org. IN      A       44.131.16.1

g6aal.ampr.     IN      CNAME   g6aal.ampr.org.

#Dirk Koopman, East Dereham, JO02lq

g1tlh.ampr.org. IN      A       44.131.16.2

g1tlh.ampr.     IN      CNAME   g1tlh.ampr.org.

#Richard ?, nr Harlow, ?

g6tcj.ampr.org. IN      A       44.131.16.3

g6tcj.ampr.     IN      CNAME   g6tcj.ampr.org.

#John Melton, Southminster, JO01jq

n6lyt.ampr.org. IN      A       44.131.16.4

n6lyt.ampr.     IN      CNAME   n6lyt.ampr.org.

#John Sager, Ipswich, JO02pa

g8onh.ampr.org. IN      A       44.131.16.5

g8onh.ampr.     IN      CNAME   g8onh.ampr.org.

#Ipswich Digipeater, Ipswich, JO02ob

gb7mx.ampr.org. IN      A       44.131.16.6

gb7mx.ampr.     IN      CNAME   gb7mx.ampr.org.

#John Lord, Bury St Edmonds, JO02ig

g4xrk.ampr.org. IN      A       44.131.16.7

g4xrk.ampr.     IN      CNAME   g4xrk.ampr.org.

#?, Mildenhall, JO02gj

g4zgy.ampr.org. IN      A       44.131.16.8

g4zgy.ampr.     IN      CNAME   g4zgy.ampr.org.

#Colin Ferryman, Colchester, JO01kw

g4diw.ampr.org. IN      A       44.131.16.9

g4diw.ampr.     IN      CNAME   g4diw.ampr.org.

#Peter ?, ?, JO01bk

g1uws.ampr.org. IN      A       44.131.16.10

g1uws.ampr.     IN      CNAME   g1uws.ampr.org.

#Neal ?, ?, JO02FI

g0brm.ampr.org. IN      A       44.131.16.11

g0brm.ampr.     IN      CNAME   g0brm.ampr.org.

#Nick Martin, Saffron Walden, ?

g6nhk.ampr.org. IN      A       44.131.16.12

g6nhk.ampr.     IN      CNAME   g6nhk.ampr.org.

#Nigel Hinderwell, Witham, JO01ht

g8ifn.ampr.org. IN      A       44.131.16.13

g8ifn.ampr.     IN      CNAME   g8ifn.ampr.org.

#Charles Brain, Chelmsford, JO01gr

g4guo.ampr.org. IN      A       44.131.16.14

g4guo.ampr.     IN      CNAME   g4guo.ampr.org.

#Malcolm Salmon, Witham, JO01ht

g3xvv.ampr.org. IN      A       44.131.16.15

g3xvv.ampr.     IN      CNAME   g3xvv.ampr.org.

#Dave Castle, Chelmsford, JO01fs

g6oqj.ampr.org. IN      A       44.131.16.16

g6oqj.ampr.     IN      CNAME   g6oqj.ampr.org.

#Alan Heaysman, Chelmsford, JO01fs

g1ibp.ampr.org. IN      A       44.131.16.17

g1ibp.ampr.     IN      CNAME   g1ibp.ampr.org.

#Nigel Hull, Chelmsford, JO01fr

g6zvv.ampr.org. IN      A       44.131.16.18

g6zvv.ampr.     IN      CNAME   g6zvv.ampr.org.

#Ted Hansen, Braintree, JO01gv

g8npf.ampr.org. IN      A       44.131.16.19

g8npf.ampr.     IN      CNAME   g8npf.ampr.org.

#John Ray, Loughton, JO01ap

g8dzh.ampr.org. IN      A       44.131.16.20

g8dzh.ampr.     IN      CNAME   g8dzh.ampr.org.

#SAMAIR Club, Braintree, JO01gv

g7dwb.ampr.org. IN      A       44.131.16.21

g7dwb.ampr.     IN      CNAME   g7dwb.ampr.org.

#Brian Wiseman, Basingstoke, IO91kf

g6kqz.ampr.org. IN      A       44.131.17.1

g6kqz.ampr.     IN      CNAME   g6kqz.ampr.org.

#Clive Sanders, Southampton, ?

g4kcm.ampr.org. IN      A       44.131.17.2

g4kcm.ampr.     IN      CNAME   g4kcm.ampr.org.

#Iain Dunsworth, Saltash, IO79vj

g4snl2.ampr.org.        IN      A       44.131.17.3

g4snl2.ampr.    IN      CNAME   g4snl2.ampr.org.

#Ed Harland, Weymouth, IO80ro

g3vpf.ampr.org. IN      A       44.131.17.4

g3vpf.ampr.     IN      CNAME   g3vpf.ampr.org.

#Lloyd Arrow, Southsea, IO90lt

g1jar.ampr.org. IN      A       44.131.17.5

g1jar.ampr.     IN      CNAME   g1jar.ampr.org.

#?, Bishops Waltham,

g4zrt.ampr.org. IN      A       44.131.17.6

g4zrt.ampr.     IN      CNAME   g4zrt.ampr.org.

#Jim ?, Basingstoke, IO91kf

g1wkk.ampr.org. IN      A       44.131.17.7

g1wkk.ampr.     IN      CNAME   g1wkk.ampr.org.

#Ron Collins, Farnborough, ?

g6vug.ampr.org. IN      A       44.131.17.8

g6vug.ampr.     IN      CNAME   g6vug.ampr.org.

#Penny Collins, Farnborough, ?

g1bpk.ampr.org. IN      A       44.131.17.9

g1bpk.ampr.     IN      CNAME   g1bpk.ampr.org.

#Mathew Walden, Alton, IO91md

g4xwv.ampr.org. IN      A       44.131.17.10

g4xwv.ampr.     IN      CNAME   g4xwv.ampr.org.

#Richard Simpson, Farnborough, IO91pg

g1dgl.ampr.org. IN      A       44.131.17.11

g1dgl.ampr.     IN      CNAME   g1dgl.ampr.org.

#Stan Pratt, Fareham, IO90jv

g7drd.ampr.org. IN      A       44.131.17.14

g7drd.ampr.     IN      CNAME   g7drd.ampr.org.

#? ?, ?, ?

g1maa.ampr.org. IN      A       44.131.17.100

g1maa.ampr.     IN      CNAME   g1maa.ampr.org.

#Andrew ?, Sunderland, IO94hu

g4nxh.ampr.org. IN      A       44.131.18.1

g4nxh.ampr.     IN      CNAME   g4nxh.ampr.org.

#Neil Imrie, Guisborough, IO94mm

g7doj.ampr.org. IN      A       44.131.18.2

g7doj.ampr.     IN      CNAME   g7doj.ampr.org.

#Gordon ?, Stockton on Tee, IO94in

g0bzc.ampr.org. IN      A       44.131.18.3

g0bzc.ampr.     IN      CNAME   g0bzc.ampr.org.

#Andrew ?, Sunderland, IO94hu

pc.g4nxh.ampr.org.      IN      A       44.131.18.4

pc.g4nxh.ampr.  IN      CNAME   pc.g4nxh.ampr.org.

#Neil Imrie, Guisborough, IO94mm

pc.g7doj.ampr.org.      IN      A       44.131.18.5

pc.g7doj.ampr.  IN      CNAME   pc.g7doj.ampr.org.

#Joe Gordon, Sunderland, IO94hv

g4lia.ampr.org. IN      A       44.131.18.6

g4lia.ampr.     IN      CNAME   g4lia.ampr.org.

#Terry Elliott, Sunderland, IO94hv

g0ehx.ampr.org. IN      A       44.131.18.7

g0ehx.ampr.     IN      CNAME   g0ehx.ampr.org.

#Jim Emerson, Sunderland, IO94hw

g0ban.ampr.org. IN      A       44.131.18.8

g0ban.ampr.     IN      CNAME   g0ban.ampr.org.

#Steve Green, Sunderland, IO94gu

g4ekm.ampr.org. IN      A       44.131.18.9

g4ekm.ampr.     IN      CNAME   g4ekm.ampr.org.

#Alan Johnson, Sunderland, IO95dw

g7aem.ampr.org. IN      A       44.131.18.10

g7aem.ampr.     IN      CNAME   g7aem.ampr.org.

#John Foster, Hartlepool, IO94jp

g0fzz.ampr.org. IN      A       44.131.18.11

g0fzz.ampr.     IN      CNAME   g0fzz.ampr.org.

#Bernie McIntosh, Stockton, IO94hn

g4wzg.ampr.org. IN      A       44.131.18.12

g4wzg.ampr.     IN      CNAME   g4wzg.ampr.org.

#Peter Przybyla, Washington, IO94fv

g0epp.ampr.org. IN      A       44.131.18.13

g0epp.ampr.     IN      CNAME   g0epp.ampr.org.

#Ken Robinson, Cramlington, IO95ea

g0lce.ampr.org. IN      A       44.131.18.14

g0lce.ampr.     IN      CNAME   g0lce.ampr.org.

#Peter Duncan, Sunderland, IO94gv

g4hix.ampr.org. IN      A       44.131.18.15

g4hix.ampr.     IN      CNAME   g4hix.ampr.org.

#Dail Przybyla, Gateshead, IO94fw

g1adp.ampr.org. IN      A       44.131.18.16

g1adp.ampr.     IN      CNAME   g1adp.ampr.org.

#Harry Whitfield, Consett, IO94bv

g6auc.ampr.org. IN      A       44.131.18.17

g6auc.ampr.     IN      CNAME   g6auc.ampr.org.

#Gareth Howell, Letchworth, IO91vx

g6kvk.ampr.org. IN      A       44.131.19.1

g6kvk.ampr.     IN      CNAME   g6kvk.ampr.org.

#Hugh Davies, St Albans, ?

g0cnr.ampr.org. IN      A       44.131.19.2

g0cnr.ampr.     IN      CNAME   g0cnr.ampr.org.

#Rod Hewitt, Ware, JO01at

g6ttd.ampr.org. IN      A       44.131.19.3

g6ttd.ampr.     IN      CNAME   g6ttd.ampr.org.

#Rod Hewitt, Ware, JO01at

g6ttd2.ampr.org.        IN      A       44.131.19.4

g6ttd2.ampr.    IN      CNAME   g6ttd2.ampr.org.

#Nigel Leaney, Buntingford, IO91xw

g1jkf.ampr.org. IN      A       44.131.19.5

g1jkf.ampr.     IN      CNAME   g1jkf.ampr.org.

#K Jones, London, JO01ao

g1sls.ampr.org. IN      A       44.131.19.6

g1sls.ampr.     IN      CNAME   g1sls.ampr.org.

#Phil Webber, St Albans, IO

g8klc.ampr.org. IN      A       44.131.19.7

g8klc.ampr.     IN      CNAME   g8klc.ampr.org.

#Bob Holton, Northwood, IO91to

g0bst.ampr.org. IN      A       44.131.19.8

g0bst.ampr.     IN      CNAME   g0bst.ampr.org.

#Ray Gordon, Edgware,

g1xrn.ampr.org. IN      A       44.131.19.9

g1xrn.ampr.     IN      CNAME   g1xrn.ampr.org.

#Keith Maton, Ealing, IO91um

g6nhu.ampr.org. IN      A       44.131.19.10

g6nhu.ampr.     IN      CNAME   g6nhu.ampr.org.

#Andy Adamson, Stevenage, IO91vw

g4spv.ampr.org. IN      A       44.131.19.11

g4spv.ampr.     IN      CNAME   g4spv.ampr.org.

#Mike Waterman, Letchworth, IO91vx

g6adx.ampr.org. IN      A       44.131.19.12

g6adx.ampr.     IN      CNAME   g6adx.ampr.org.

#Steve Richards, Reed, IO92xa

g4hpe.ampr.org. IN      A       44.131.19.13

g4hpe.ampr.     IN      CNAME   g4hpe.ampr.org.

#Steve McIver, East Ham,

g1sju.ampr.org. IN      A       44.131.19.14

g1sju.ampr.     IN      CNAME   g1sju.ampr.org.

#Mike Faithfull, Hatfield, ?

g6ubh.ampr.org. IN      A       44.131.19.15

g6ubh.ampr.     IN      CNAME   g6ubh.ampr.org.

#John Musgrave, Baldock, ?

g6kbs.ampr.org. IN      A       44.131.19.16

g6kbs.ampr.     IN      CNAME   g6kbs.ampr.org.

#Gareth Howell, Letchworth, IO91vx

ppc.g6kvk.ampr.org.     IN      A       44.131.19.17

ppc.g6kvk.ampr. IN      CNAME   ppc.g6kvk.ampr.org.

#Richard Wickham, Hitchin, IO91uw

g4jqb.ampr.org. IN      A       44.131.19.21

g4jqb.ampr.     IN      CNAME   g4jqb.ampr.org.

#S Blackmore, Hertford, IO91xu

g4tio.ampr.org. IN      A       44.131.19.22

g4tio.ampr.     IN      CNAME   g4tio.ampr.org.

#Andy Rutter, London, IO91wo

g8hck.ampr.org. IN      A       44.131.19.23

g8hck.ampr.     IN      CNAME   g8hck.ampr.org.

#The Instruction Set RC, London, IO91wn

g7bdh.ampr.org. IN      A       44.131.19.24

g7bdh.ampr.     IN      CNAME   g7bdh.ampr.org.

#Andy Rutter, London, IO91wo

g8hck2.ampr.org.        IN      A       44.131.19.25

g8hck2.ampr.    IN      CNAME   g8hck2.ampr.org.

#Mike Banahan, London, IO91vl

g4buh.ampr.org. IN      A       44.131.19.26

g4buh.ampr.     IN      CNAME   g4buh.ampr.org.

#Chris Rutt, Hatfield, IO91vr

g0amg.ampr.org. IN      A       44.131.19.27

g0amg.ampr.     IN      CNAME   g0amg.ampr.org.

#Martin Stubbs, ?, ?

g8imb.ampr.org. IN      A       44.131.20.1

g8imb.ampr.     IN      CNAME   g8imb.ampr.org.

#David Hough, Bristol, IO81sm

g4wrw.ampr.org. IN      A       44.131.20.2

g4wrw.ampr.     IN      CNAME   g4wrw.ampr.org.

#Mike Chace, Bath, IO81tj

g6dhu.ampr.org. IN      A       44.131.20.3

g6dhu.ampr.     IN      CNAME   g6dhu.ampr.org.

#Steve Gibbs, Vale, ?

gu3mbs.ampr.org.        IN      A       44.131.21.1

gu3mbs.ampr.    IN      CNAME   gu3mbs.ampr.org.

#Chris ?, ?, ?

gu4ymv.ampr.org.        IN      A       44.131.21.2

gu4ymv.ampr.    IN      CNAME   gu4ymv.ampr.org.

#Guernsey Digipeater, ?, ?

gb7gur.ampr.org.        IN      A       44.131.21.3

gb7gur.ampr.    IN      CNAME   gb7gur.ampr.org.

FTPUSERS

# F T P U S E R S

#

# Note: NET.EXE ignores all lines which start with a pound sign (#).

#

#

# The FTPUSERS file provides access control over the users of

# your host computer. It consists of 4 elements:

#        username    password    /path     permissions

#

# where

#      username is the users's login name for ftp

#       password is the password they are required to

#                use. A password of '*' allows anything

#                typed to be accepted.

#       /path is the allowable prefix on accessible files.

#                Think of this as the root of the directory

#                subtree this user is allowed to access.

#       permissions is a decimal number granting permission

#                for read, create, and write operations.

#                Bit 0x1 is read, bit 0x2 is create if not

#                overwriting, bit 0x4 allows overwriting an

#                existing file, and deleting files.

#                For example: a user with read AND create

#                would have a permission of 3 (1+2). A user

#                with full read/write/overwrite would have a

#                permission of 7 (1+2+4). Until such time as

#                an encryption system for passwords is

#                incorporated into TCP/IP caution should be

#                used in authorizing full access to your

#                system.

#

#----------------------------------------------

#

# This first entry is used to define a user "anonymous", who has read-only

# access to a directory which you have set up and arbitrarily named 'public',

# and all subdirectories of 'public'. In effect this is his root directory

# on your system.  This is the default entry everyone should have, so that

# new users who don't have a specific entry in this file can still get in

# and look at what's available, download a few things, etc.  When logging in

# to a machine using this entry, use your callsign or something else unique

# as the password, to identify yourself. Any password will work since there

# is a "*" in the password field.

#

anonymous * /public 3

#

bbs * /bbs 3

#

# The following entry for kelvin with a password of dummy has full

# access to your root directory and full read/write/overwrite/delete

# functions. You'll want to think twice before giving this access to

# others :-) .

#

kelvin dummy / 7

g1emm yy101179 / 255

#

# The following entry for guest with a password of guest has read and

# write access to /public and all its subdirectores.

# This is a good entry for someone

# you trust to upload files to your system, but who you don't want deleting

# things that are already there.

#

guest guest /public 3

colin g3tne / 3

ian g3rra / 3

bob g8ggi / 3

paul g1pcd / 3

ron g6vug / 3

brian g6kqz / 3

#

# THE END

NODES.ARP

#---------------------------------------------------

#

# Define the mapping of internal host names to the 'real world' names.

#

arp add [44.131.0.0] ax25 qst

arp add g0gnh ax25 g0gnh

arp add g0gnh netrom g0gnh

arp add g1bys ax25 g1bys

arp add g1bys netrom g1bys

arp add g1maa ax25 g1maa

arp add g1maa netrom g1maa

arp add g1plt ax25 g1plt

arp add g1plt netrom g1plt

arp add g1ttg ax25 g1ttg

arp add g1ttg netrom g1ttg

arp add g1wkk ax25 g1wkk-1

arp add g1wkk netrom g1wkk-1

arp add g3rra ax25 g3rra

arp add g3rra netrom g3rra

arp add g3tne ax25 g3tne

arp add g3tne netrom g3tne

arp add g4bio ax25 g4bio

arp add g4bio netrom g4bio

arp add g4tnu ax25 g4tnu

arp add g4tnu netrom g4tnu

arp add g4uqe ax25 g4uqe

arp add g4uqe netrom g4uqe

arp add g4pzk ax25 g4pzk

arp add g4pzk netrom g4pzk

arp add g4xwv ax25 g4xwv

arp add g4xwv netrom g4xwv

arp add g5ds ax25 g5ds

arp add g5ds netrom g5ds

arp add g6kqz ax25 g6kqz

arp add g6kqz netrom g6kqz

arp add g6kvk ax25 g6kvk-1

arp add g6kvk netrom g6kvk-1

arp add g6vug ax25 g6vug

arp add g6vug netrom g6vug

arp add g7gbq netrom g7gbq

arp add g8kwi ax25 g8kwi

arp add g8kwi netrom g8kwi

arp add g8ogr ax25 g8ogr

arp add g8ogr netrom g8ogr

arp add g8zhr ax25 g8zhr

arp add g8zhr netrom g8zhr

arp add g1emm2 ax25 g1emm-2

arp add g1emm2 netrom g1emm-2

arp add gb3xp ax25 g8ggi-1

arp add gb3xp netrom gb3xp-1

arp add g6xqb ax25 g6xqb

arp add g6xqb netrom g6xqb

#

#---------------------------------------------------

#

# The END

#

NODES.FIL

#---------------------------------------------------

#

# Define the NET/ROM view of the 'real world'.

#

netrom nodefilter add g1emm-2 tnc 255

netrom nodefilter add g1plt tnc 40

netrom nodefilter add g1wkk-1 tnc 90

netrom nodefilter add g3rra tnc 80

netrom nodefilter add g4bio tnc 100

netrom nodefilter add g6vug tnc 30

netrom nodefilter add g8kwi tnc 100

netrom nodefilter add g6xqb tnc 100

#

#---------------------------------------------------

#

# The END

#

NODES.ROU

#---------------------------------------------------

#

# Define the 'real world' routing to other remote nodes.

#

route add g1emm loopback

#

route add g4bio tnc

route add g1wkk tnc

#

# route addprivate g6xqb tnc

# route addprivate g6hpe tnc

#

route addprivate g3rra tnc

route addprivate g8kwi tnc

route addprivate g1ttg tnc

#

route addprivate g1bys netrom

route addprivate g1plt netrom

route addprivate g6kqz netrom

route addprivate g6kvk netrom

route addprivate g8zhr netrom

route addprivate gb3xp netrom

route addprivate g4pzk netrom

route addprivate g4lzv netrom

route addprivate g4tnu netrom

route addprivate g7gbq netrom

route addprivate g0gnh netrom

#

#---------------------------------------------------

#

# The END

#

PRIVS.DOC

/* Permission flag bits */

#define FTP_READ        1       /* Read files */

#define FTP_CREATE      2       /* Create new files */

#define FTP_WRITE       4       /* Overwrite or delete existing files */

#define AX25_CMD        8       /* Allow use of (G)ateway */

#define TELNET_CMD      16      /* Allow use of (T)elnet */

#define NETROM_CMD      32      /* Allow use of (N)etrom */

#define SYSOP_CMD       64      /* Allow use of @ command & sysop privs */