HOW TO USE THE NET/ROM(r) SUPPORT IN TCP/IP

by Dan Frank W9NK

We'll work from the bottom up in describing the facilities added

for the net/rom version of net.  It appeared that for some applications,

it might be worthwhile to be able to have a callsign per interface.

So far, that has only been done for the hapn and kiss attach code,

but the extension is obvious to the eagle and pc100 drivers.  Basically,

if you give an extra argument at the end of the attach line, that argument

will be taken as the callsign for that interface, overriding ax25 mycall.

mycall is still functional, and if you want to use just one callsign for

all your interfaces, just ignore the new feature.

No physical interface is completely dedicated to net/rom, which is as

it should be.  You attach all your AX.25 interfaces, of whatever sort.

Then you attach the net/rom pseudo-interface ("attach netrom").  Then

you identify to the net/rom software those interfaces you want to allow

it to use, with the "netrom interface" command.  The format of this

command is:

netrom interface ax0 #ipnode 192

The first argument is the name of the previously attached interface you

want to use.  The second argument is the alias of your node, to be used in

your routing broadcasts.  The alias is never used for anything else (as

you will see!).  The last number is the net/rom quality figure.  This is

used in computing the route qualities; it represents the contribution of

this interface to the overall computation.  For a 1200 baud half-duplex

connection, 192 is the right number.

You need a netrom interface command for every interface you're going

to use with net/rom.

Once you have set up your interfaces, you need to set some timers.

There are two:  the nodes broadcast interval timer, and the obsolescence

timer.  These are set in seconds, like the smtp timer.  They are usually

set to an hour.  You can set them to something different, if you want.

If your local net/rom nodes broadcast every hour, but you want to do so

every ten minutes, you can say:

netrom nodetimer 600

netrom obsotimer 3600

Every time the obsotimer kicks, the obsolescence counts for all non-permanent

entries are decremented by one.  When the count for an entry falls below

five, it is no longer broadcast.  When it falls to 0, it is removed.  The

count is initialized at 6.  These will eventually be settable parameters;

you can adjust them now by changing the initializers for the variables

in the source file.

When you first come on the air, you can send out nodes broadcasts to

tell the local nodes that you are available.  Use the command:

netrom bcnodes ax0

where ax0 is the interface on which you want to send the broadcast.  Do

this for every interface on which you want to do this.

Sometimes you can hear broadcasts from nodes that can't hear you.  If

your routing table gets filled with these unusable routes, your node will

grind to a halt.  The solution to this is node broadcast filtering, via

the netrom nodefilter command.  There is a filter list, which contains

a list of callsigns and interfaces.  Then there is a filter mode, which

indicates what to do with the list.

If the filter mode is "none", no filtering is done.  If it is "accept",

then only broadcasts from the indicated stations on the indicated

interfaces are accepted.  If it is "reject", then all broadcasts

except those from the listed stations on the listed interfaces are

accepted.

Because the net/rom code cannot at this time recognize unusable

routes and try alternates, I strongly recommend use of the filter

command to restrict broadcast acceptance to those nodes which you

know you can reach.

The last category of net/rom commands are those used for maintaining

the routing table.  They fall under the "netrom route" subcommand.

"netrom add" adds a permanent entry to the routing table.  Its format

is:

netrom route add #foo w9foo ax0 192 w9rly

This command adds an entry for w9foo, whose alias is #foo, route

quality 192, via w9rly on interface ax0.  Let's talk about what this

means.  w9foo is the *destination* node, the one to whom you want

the packets routed by the net/rom network.  w9rly is your *neighbor*,

the net/rom node to which you pass the packet to be forwarded.  Since

w9rly may appear on more than one interface (the callsign may be used

by more than one net/rom node on different bands), we specify that

we are to use ax0 to send the packet.

With net/rom, like IP, we don't know exactly what route a packet

will take to its destination.  We only know the name of a neighbor

which has indicated a willingness to forward that packet (of course,

the neighbor may be the destination itself, but that's unlikely in

our application).  Net/rom sends the packet to the neighbor, with a

network header specifying our callsign and that of the ultimate

destination (in this case w9foo).

We can use the netrom route add command to establish a digipeater

path to the neighbor.  For example:

netrom route add #foo w9foo ax0 192 w9rly wd9igi

This will cause us to use wd9igi as a digipeater in establishing our

connection to the net/rom node w9rly.

What if you have a net/rom node or nodes, and you'd like to attach

them to your computer via their serial interfaces, and use net as a

packet switch?  It's very easy:  you have to attach those interfaces,

using the "attach asy" command, but specifying type "nrs" instead of

"slip" or "kiss".  "nrs" is the net/rom serial framing protocol, which

is like KISS, but uses different framing characters and has an 8-bit

checksum.

When you attach an nrs interface, it can be used for passing IP

datagrams or AX.25 frames over serial lines or modems.  To use it

for net/rom, you have to identify it to the netrom code just like

any other interface, with the "netrom interface" command.

Now you know all the commands, but how do we actually use net/rom

for IP communications?  This takes two steps:

Step one:  update the routing table.  In all likelihood, you will

use net/rom to gateway two IP subnets.  So, you'll probably want to

identify a station on each end as a gateway.  Let's say we're on the

Milwaukee subnet, and we want to talk to someone in Madison.  If

we're not the gateway, we just have a routing table entry like this:

route add [44.92.0.0]/24 ax0 wg9ate-pc.ampr

This specifies that wg9ate should get all packets for the 44.92.0.x

subnet via interface ax0.

Wg9ate has this routing table entry:

route add [44.92.0.0]/24 netrom w9mad-pc.ampr

(presuming that w9mad is the Madison gateway).  Now, when the IP layer

at wg9ate gets datagrams for Madison, it knows that they have to go via

net/rom to w9mad.  Notice that we don't specify a "real" interface,

like ax1 or nr0, in the route entry.  The net/rom network layer will

pick the right interface based on its net/rom routing tables.

We're not done yet, though.  w9mad-pc.ampr is not an ax.25

callsign.  The net/rom send routine called by the IP layer needs

to map from the IP address to an ax.25 address.  It does this via

a manually added arp entry:

arp add w9mad-pc.ampr netrom w9mad

[We kind of fudged by using the arp table for this purpose, since

there is no way to do automatic address resolution for net/rom,

and arp messages are never sent or received for net/rom nodes.

However, the arp table does contain precisely what we have here:

mappings from IP addresses to callsigns, and it saved a lot of

code to do it this way.]

Notice also that no digipeaters are ever specified in the arp entry

for a net/rom node.  Also, the callsign to which we are mapping

is the final destination of the packet, not the non-destination

neighbor.  That neighbor will be picked based on the net/rom

routing tables.

So, as a summary, let's look at what happens to a packet that

reaches the IP layer on wg9ate, destined for Madison.  The IP

routing code looks the destination IP address up in the table,

and discovers that it should go via net/rom to w9mad-pc.ampr.

So, it passes the packet to the net/rom send routine.  That

routine uses the arp table to translate w9mad-pc's IP address

to the callsign "w9mad".  Then it passes the packet to the

net/rom routing code.  That code checks to see if the destination

callsign (w9mad) is the same as that of any of its assigned

net/rom interfaces.  Since it isn't, it puts a network layer

header (a.k.a. net/rom level 3 header) on it, and looks for

w9mad in its routing tables.  Presumably, it finds an appropriate

neighbor for the packet, and sends in out via ax.25.  The net/rom

network does the job of actually getting the packet to its

destination.

At w9mad, the packet's protocol ID causes it to be sent to

the same net/rom routing code that handled the outgoing packet

from wg9ate (running on a different computer, of course).  Now

the destination callsign matches, so the net/rom network layer

header is stripped off, and packet is passed up to the IP layer.

(Net/rom network headers don't have a protocol ID byte, so

we just hope for the best.  If a net/rom node addresses a

net/rom transport layer packet to us, it is likely to be dropped

by IP for any of a number of reasons.)

-------------------------------------------------------------------

Some notes, added November 1988:

1) As of version 871225.33 (alpha w9nk.1), the format of IP

datagrams in net/rom network packets has been changed.

This was for several purposes, including to eliminate

the bug in tracing that caused net/rom - ip packets to

appear garbled after the IP datagram sequence number

field incremented beyond 255.

2) As of version 871225.33 (alpha w9nk.2), the default mode

for routing broadcasts is *not* to send the contents of

the routing table, but instead to just broadcast the

callsign and alias of the station.  NET/ROM routing is

fragile enough without our help.  If you need to make

routing broadcasts, because you are really contributing

some useful routes to the network, "netrom verbose yes"

will turn on full broadcasts.

3) There have been some complaints that if station A is

making NODES broadcasts picked up by his local net/rom

B, and station C connects to B and then asks B to connect

to A, that the connection is not successful.  This is

because B thinks that A is a net/rom, and tries to open

a transport connection to A, which is not supported.

As of late versions of NET, we now explicitly reject

such attempted connections, rather than letting them

time out.  If C wishes to connect to A through B, C

should connect to B, then issue a connect to A using

a digipeater.  This will cause B to use AX.25 rather

than NET/ROM transport.

4) I can not stress enough that "route add" and "netrom route add"

are two different commands, with different purposes.  In general,

you only need a "netrom route add" if you need to add a route

to a net/rom node via a digipeater path.  If you find yourself

using this command, ask yourself, "Why am I doing this?"  Many

people do not understand that net/rom does automatic routing

(well, sort of :-)).

Good luck!

73, Dan Frank, W9NK

##########