FBN Software proudly presents Network. Network is an analysis tool used to

perform  an AC spectral analysis of an electrical network. The program does

this by using Kirchoff's laws to solve for the network node voltages and

currents.

What can I use Network for? Its primary application is to obtain the frequency

response of an electrical network. It can be used to analyze active and

passive filter implementations. It is a mini SPICE program without all of the

bells and whistles. With NETWORK you can prototype a circuit on your computer

and obtain insight on the actual operation of the real electrical network. You

can vary parameter values and determine their effect on the network's

performance.

Network presently allows a maximum of 30 NODES. All node voltages are

referenced to NODE 0 (electrical ground). The following components can be used

in the circuit : Resistors (60 max.), Capcacitors (60 max.), Inductors (60

max.), Amplifiers (10 max.), FET's (30 max.), and Transistors (30 max.).

Included on the disk is a sample network that has been saved to disk :

TEMP.DTA. This is a 4 node bandpass active filter made up of the following

components :

C1 - 0.1

---------------| |-----------------

|                       R2 - 10k   |

|                                  |

|              --------/\/\/\/\-----

|              |                   |

|    C2 - 10   |    | .            |

|              |    |   .          |

Input (1)--/\/\/\/\---(2)-----| |----(3)---| -   .  +     |

R1 - 1K     .                   |     . -------  (4)

>       ------------| + .

R3 - 100 <       |           | .

>       |           |     A1

|--------

----- (0)

---

-

The opening screen of network gives you the option of entering your network

components manually or from a saved file. If you type "D" (or "d") to this

response and provide the sample file name it will load. You then have the

option of editing component values and/or adding new components to the network.

The manual component entry asks you for the input and output node and then

asks for component values and node connections. After the last component has

been entered (component type), the program enters into the edit mode so you can

review and correct any entry errors you might have made.

The program then enters into the response calculation mode. This mode will

calculate the output You will be asked for a start frequency, a stop or end

frequency, and the frequency step size. The sweep can be linear (equal step

size) or logarithmic (equal steps on a log scale). If you want output to the

CRT or to a line printer. The program will then output the network response in

dB and phase angle in degrees for each frequency step. You also have the

option of saving the sweep output data to disk for input to a plotting

routine.

The frequency response (1 Hz - 10KHz) of the network in logarithmic frequency

steps has been saved in file TEMP.PLT. If you are using a CGA card for video

output, you can plot this response on your CRT using the program PLOTFL.EXE

that I have included on the disk. The program PLOTFL.EXE automatically adjusts

for linear or logarithmic frequency scale. The PLOTFL.EXE graph can also be

output to a graphics printer via the Shift-PrtSc key if you have loaded

GRAPHICS.COM (DOS Utility) before running the program.

After the frequency response has been output you will view a menu with the

option to run a new sweep, Edit/modify component values, or save the network to

disk. This feature lets you change circuit values to determine their effect on

the network's response or lets you expand a portion of the frequency sweep in

order to obtain higher output resolution.

The best way to learn how to use the program is to run it. View the file

TEMP.PLT in order to obtain a feel for what the network response is. Then run

NETWORK and load TEMP.DTA. Answer "N" (or "n") to all the edit questions and

try out the various sweep modes. Then try modifying some component values

(R3 is a good candidate) and see what effect the change has on the network

response. You can save the sweep outputs to disk if you want to or just

observe the changes on the screen and make a mental note of what happens.

If you save the sweeps to disk, use a different file name for each sweep. I

personally prefer to view the numbers rather than the graphics because it

gives me a better feel for what the actual values and response changes are.

The graphics plot is good for reports and for presentations.

Hints : For convenience capacitor values are input in micro farads. Diodes or

switches are not valid circuit components since the program is used for linear

network analysis. Diodes are assumed to be "Ideal" switches which are turned

on and have 0 resistance and can therefore be replaced by a short. If you want

to use a diode, approximate the diode with a small resistor which is equal to

the forward resistance of the diode (10 ohms or so).  The amplifiers are

provided with differential inputs and outputs. Be sure that you use the proper

output in feedback circuits (negative feedback is a network between +output

and -input). The amplifiers (as provided) have infinite bandwidth. You can set

their open circuit voltage gain and output short circuit impedance. If you

want to add a gain-bandwidth limitation to an amplifier place a capacitor to

node 0 from its output. The amplifier response will then be band limited by

the RC time constant of the capacitor and the amplifier's output resistance

(Gain-Bandwidth = A/(2*pi*RC), where A is the Open circuit voltage gain of the

amplifier). Capacitors can also be added in parallel with FET source resistors

and Transistor collector resistors to limit the frequency response of these

components.

Suggestions : Since the program uses matrix arithmetic to solve for the

response, much faster response is obtained for small networks than for larger

ones. If you have a large network try analyzing the network response by

breaking it into parts and determine how each part works before attempting the

final solution. NETWORK.EXE has been compiled using QB 4.0. QB 4.0 emulates

the 8087 chip, if one is not installed in your machine, which can slow down

your output.

Problems & Freeware Pitch : I offer this program to the general public as

"Freeware". By this I mean you can make copies of the ARC file and give them

to your friends. Just be sure to include all files (Including NETWORK.DOC) in

NETWORK.ARC. If you have problems and want support, I'll provide it to you for

a negotiable fee (depending on the difficulty of the problem). If you want to

modify NETWORK on your own (or you're just curious) I'll provide you with the

source code for NETWORK (and to PLOTFL) on a 360k 51/4 inch disk for a $10.00

fee.

Who am I and where do you send your money :

Send checks (no plastic) to :

William L. Gill

12713 Brunswick Lane

Bowie,Md. 20715

My qualifications : I am an Electrical Engineer with 27 years of experience.

My specialty is Signal Processing. I am a former applications engineer for

Westinghouse Integrated Circuits. I have approximately 15 years of part time

teaching experience (Application of Digital Integrated Circuits, and the

Application of Linear Integrated Circuits). So I have a strong background in

circuit analysis and performance.

Why am I doing this ? I don't really expect to make money from this program.

It is an engineering program and should therefore have a limited appeal. I

think it provides a tool that some of you will want to try. The fees that I am

imposing are to prevent "nuisance calls".