SUBPART E - PROVIDING EMERGENCY COMMUNICATIONS

97.401  Operation during a disaster.

(a) When normal communication systems are overloaded, damaged

or disrupted because a disaster has occurred, or is likely to

occur, in an area where the amateur service is regulated by the

FCC, an amateur station may make transmissions necessary to meet

essential communication needs and facilitate relief actions.

(b) When normal communication systems are overloaded, damaged

or disrupted because a natural disaster has occurred, or is likely

to occur, in an area where the amateur service is not regulated by

the FCC, a station assisting in meeting essential communication

needs and facilitating relief actions may do so only in accord with

ITU Resolution No. 640 (Geneva, 1979).  The 80 m, 75 m, 40 m, 30

m, 20 m, 17 m, 15 m, 12 m, and 2 m bands may be used for these

purposes.

(c) When a disaster disrupts normal communication systems in

a particular area, the FCC may declare a temporary state of

communication emergency.  The declaration will set forth any

special conditions and special rules to be observed by amateur

stations during the communication emergency.  A request for a

declaration of a temporary state of emergency should be directed

to the EIC in the area concerned.

(d) A station in, or within 92.6 km of, Alaska may transmit

emissions J3E and R3E on the channel at 5.176 MHz for emergency

communications.  The channel must be shared with stations licensed

in the Alaska-private fixed service.  The transmitter power must

not exceed 150 W.

97.403  Safety of life and protection of property.

No provision of these rules prevents the use by an amateur

station of any means of radiocommunication at its disposal to

provide essential communication needs in connection with the

immediate safety of human life and immediate protection of property

when normal communication systems are not available.

97.405  Station in distress.

(a) No provision of these rules prevents the use by an amateur

station in distress of any means at its disposal to attract

attention, make known its condition and location, and obtain

assistance.

(b) No provision of these rules prevents the use by a station,

in the exceptional circumstances described in paragraph (a), of

this section, of any means of radiocommunications at its disposal

to assist a station in distress.

97.407  RACES operation.

(a) No station may transmit in RACES unless it is an

FCC-licensed primary, club, or military recreation station and it

is certified by a civil defense organization as registered with

that organization, or it is an FCC-licensed RACES station.  No

person may be the control operator of a RACES station, or may be

the control operator of an amateur station transmitting in RACES

unless that person holds a FCC-issued amateur operator license and

is certified by a civil defense organization as enrolled in that

organization.

(b) The frequency bands and segments and emissions authorized

to the control operator are available to stations transmitting

communications in RACES on a shared basis with the amateur service.

In the event of an emergency which necessitates the invoking of the

President's War Emergency Powers under the provisions of Section

706 of the Communications Act of 1934, as amended, 47 U.S.C. 606,

RACES stations and amateur stations participating in RACES may only

transmit on the following frequencies:

(1) The 1800-1825 kHz, 1975-2000 kHz, 3.50-3.55 MHz,

3.93-3.98 MHz, 3.984-4.000 MHz, 7.079-7.125 MHz, 7.245-7.255 MHz,

10.10-10.15 MHz, 14.047-14.053 MHz, 14.22-14.23 MHz, 14.331-14.350

MHz, 21.047-21.053 MHz, 21.228-21.267 MHz, 28.55-28.75 MHz,

29.237-29.273 MHz, 29.45-29.65 MHz, 50.35-50.75 MHz, 52-54 MHz,

144.50-145.71 MHz, 146-148 MHz, 2390-2450 MHz segments;

(2) The 1.25 m, 70 cm and 23 cm bands; and

(3) The channels at 3.997 MHz and 53.30 MHz may be used

in emergency areas when required to make initial contact with a

military unit and for communications with military stations on

matters requiring coordination.

(c) A RACES station may only communicate with:

(1) Another RACES station;

(2) An amateur station registered with a civil defense

organization;

(3) A United States Government station authorized by the

responsible agency to intercommunicate with RACES stations;

(4) A station in a service regulated by the FCC whenever

such communication is authorized by the FCC.

(d) An amateur station registered with a civil defense

organization may only communicate with:

(1) A RACES station licensed to the civil defense

organization with which the amateur station is registered;

(2) The following stations upon authorization of the

responsible civil defense official for the organization in which

the amateur station is registered:

(i) A RACES station licensed to another civil

defense organization;

(ii) An amateur station registered with the same or

another civil defense organization;

(iii) A United States Government station authorized

by the responsible agency to communicate with RACES stations; and

(iv) A station in a service regulated by the FCC

whenever such communication is authorized by the FCC.

(e) All communications transmitted in RACES must be

specifically authorized by the civil defense organization for the

area served.  Only civil defense communications of the following

types may be transmitted:

(1) Messages concerning impending or actual conditions

jeopardizing the public safety, or affecting the national defense

or security during periods of local, regional, or national civil

emergencies;

(2) Messages directly concerning the immediate safety of

life of individuals, the immediate protection of property,

maintenance of law and order, alleviation of human suffering and

need, and the combating of armed attack or sabotage;

(3) Messages directly concerning the accumulation and

dissemination of public information or instructions to the civilian

population essential to the activities of the civil defense

organization or other authorized governmental or relief agencies;

and

(4) Communications for RACES training drills and tests

necessary to ensure the establishment and maintenance of orderly

and efficient operation of the RACES as ordered by the responsible

civil defense organizations served.  Such drills and tests may not

exceed a total time of 1 hour per week.  With the approval of the

chief officer for emergency planning the applicable State,

Commonwealth, District or territory, however, such tests and drills

may be conducted for a period not to exceed 72 hours no more than

twice in any calendar year.