Morse Code Transceiver Version 1.0 MS-DOS

INTRODUCTION

Morse Code Transceiver Version 1.0 MS-DOS is an assembly language

program for use on IBM PC, XT, AT and Compatibles running the MS-DOS Version 2

or above. Both monochrome and color monitors are supported. This program was

developed for use by Amateur Radio operators or others who need a program that

is capable of both sending and receiving morse code over a wide range of code

speeds and operating conditions. Many powerful convenience features are

incorporated enhancing station operation in the CW mode together with ease of

operation.

Morse Code Transceiver Version 1.0 MS-DOS is Shareware therefore under

the conditions that it is not modified and a fee is not charged it may be

copied and distribution to others is encouraged.

******* DISCLAIMER:--- No liability assumed. --- No warrenty implied. *******

AUTHOR

Robert R Anderson, K2BJG 69 Page Drive Oakland, New Jersey  07436

Phone: 201-337-7858 or 337-9644

STATION INTERFACE

Morse Code Transceiver receive input data, transmit output data and a

transmit control output signal are at RS 232C level via serial port 1 or 2.

External hardware interface equipment is required to demodulate and translate

CW audio signals from the station receiver to RS 232C level for input to the

computer and to translate RS 232C level TX CW keying and TX control signals

from the computer to that required by the station transmitter. A negative

level is 'off' and a positive level is 'on' for all signals.

Pin  1          Protective Ground

Pin  4  (rts)   R/T Transmitter Control

Pin  5  (cts)   CW Receive Input

Pin  7          Signal Ground

Pin 20  (dtr)   CW Transmit Output

Commercial computer interfaces such as the MFJ 1224 are currently

available and will work with this program. For those who want to "roll their

own" a suitable unit can easily be constructed using parts readily available

from such retail electronic parts suppliers as Radio Shack. To aid in the

design of a suitable CW computer interface unit the following guide lines

should be followed. Detail circuit design information and practical examples

can be found in the ARRL Radio Amateurs Handbook.

The transmit channels of the interface unit consisting of the transmit

CW data from pin 20 and the transmit control signal from pin 4 should be

translated from RS 232 level to TTL level by use two sections of a MC1489 RS

232 Quad Line Receiver. The CW keying TTL signal must then interface to the

station transmitter. The circuit required to accomplish this depends on the

requirements of the transmitter. Older transmitters using grid block keying of

tubes will probably require what is called negative keying where a key up

relative high negative voltage of as much as 50 to 150 volts is seen and key

down current of a few miliampers is required. Late model solid state

transmitters will probably require what is called positive keying where a key

up relative low positive voltage of about 20 volts is seen and key down

current of a few miliampers is required. A single transistor of suitable

voltage rating and polarity will do the job however if your interface needs to

be universal then a dual transistor circuit to handle both types of keying

should be used.

Interface of the TTL transmit control signal to the transmitter PTT

line is not required if transmitter VOX circuits will do the job for your

operating technique. It is however better to use this signal since VOX delays

can cause problems at low keying speeds or when quick break in is desired.

Also this control signal is necessary to hold the transmitter on during a

pause between characters if operating in manual TX to RX program mode. A

single transistor of suitable voltage rating and polarity and able to handle

the required current will do the job.

The receive channel of the interface unit should consist of an audio

bandpass filter, a demodulator with a low pass filter and a level translator

to furnish input RS 232 level receive data to pin 5.

The band pass filter function can be accomplished with one or more

operational amplifiers configured as active filters. The center operating

frequency should match your receivers CW beat frequency offset. This is

usually about 800 to 1000 Hz for most amateur receivers. The bandwidth should

be about 100 to 200 Hz.

The demodulator can be a simple diode detector followed by a low pass

active filter with a cutoff frequency of approximately 75 Hz. A better method

is the use of a phase lock loop decoder tuned to the same frequency as the

audio filter.

Level translation of the signal from the demodulator to RS 232 level

is best accomplished by using one section of a MC1488 RS 232 Quad Line

Driver.

PROGRAM INITIALIZATION

The Morse Code Transceiver program CW.EXE and if they exist it's

associated configuration or data files CW.CNF, CW_KCS.DAT, CW_MSG and

CW_FXS.OVL must to be successfully loaded be located in the same directory on

the default disk drive.

The program is started by typing CW at the DOS prompt. Initial checks

of the DOS version and current video mode are made. The program is terminated

with an error message if the DOS version is not 2.0 or above or if the video

mode is other than 80 X 25 TEXT - color mode 2 BW80, color mode 3 CO80 or mode

7 MONO.

At this point only the main program CW.EXE has been loaded. It

contains all required default configuration data.  Next if it is found the

configuration file CW.CNF is read from the default disk drive replacing the

default configuration data. If the configuration file is not found an error

message is displayed.

2

The default or config file configuration data mow in memory includes

color video attribute data to be used for color display. The current video

mode is again checked. If it is mode 3 CO80 no action is taken. If it is mode

2 BW80 or mode 7 MONO then black/white attributes for normal and the reverse

video used for the status line replace the existing video attribute data.

Note - The DOS mode command can be used prior to starting CW to select the

desired video mode.

The data files CW_KCS.DAT (key character sequences) and CW_MSG.DAT (10

messages) are loaded if they are found. If they are not found error messages

are displayed.

The fixed TX speed overlay file if found is loaded replacing default

fixed TX speed data. A message is displayed indicating that it is or is not

loaded.

At this point the operator is prompted to see if initialization should

continue. Pressing (ESC) will cause an immediate return to DOS. Any other key

will cause program initialization to continue. Upon initialization completion

the operating screen is displayed and the bell will sound indicating ready to

start.

DEFAULT CONFIGURATION and THE CONFIGURATION FILE (CW.CNF)

The main program CW.EXE contains default configuration data. It can

operate without reading the configuration file CW.CNF. It has the means to set

all variables contained in the configuration file which can be created or

updated as an option during program exit. The configuration file if it exist

on the default drive is read during program initialization replacing default

configuration data.

The following program variables are included in the configuration file

CW.CNF and the default values contained in the main program CW.EXE are

indicated:

Variable                        Default

I/O Port                        03F8h           #1

Auto TX flag                    0               off

Auto RX flag                    0               off

Speaker flag                    0               off

Upper screen color attribute    30              yel/blu

Lower screen color attribute    31              h-wht/blu

Review buffer color attribute   31              h-wht/blu

Window color attribute          112             blk/wht

Window border color attribute   116             red/wht

Status line color attribute     79              h-wht/red

Tx Dot length                   30

Tx Dash length                  90

Tx Element space                30

Tx Character space              90

Tx Word space                   120

Rx Dot average length           30

Rx Dash average length          90

Rx Element average length       30

Tx Variable dot                 30

Tx Variable element space       30

00

3

COMMAND SUMMARY

The following commands are available from CW

F1      Display Help # 1

F2      Toggle Output to Printer - on/off

F3      Toggle Capture function - on/off

F4      Display Disk Directory

F5      Toggle TX Source - Keyboard/Send File

F6      Transfer Selected Message to Xmt Buffer

F7      Toggle RX speed mode - Track/Manual

F8      Toggle TX speed mode - Manual/Lock RX speed

F9      Toggle Speaker - on/off

F10     Toggle Mode - TX/RX

Shf-F1  Display Help # 2

Shf-F6  Preview Selected Message on Lower Screen

Shf-F7  Select RX speed LOCK mode

Shf-F8  Toggle TX speed mode - Fixed/Manual

Alt-F1  Display Help # 3

Alt-F3  Toggle Capture File - open/close

Alt-F4  Change Drive\Path

Alt-F5  Toggle Send File - open/close or restart

Alt-F6  Toggle Selected Message - Load/Finish

Alt-F10 Toggle TX Control auto/manual

Ctl-F1  Display Help # 4

Ctl-F3  Save Review Buffer

Ctl-F4  Delete Disk File

Ctl-F5  Clear Keyboard Buffer

Ctl-F6  Save Messages to Disk

Ctl-F8  Create a Fixed Tx Speed Overlay File

Ctl-F10 Toggle RX Control auto/manual

Alt-1   Select Message # 1

Alt-2   Select Message # 2

Alt-3   Select Message # 3

Alt-4   Select Message # 4

Alt-5   Select Message # 5

Alt-6   Select Message # 6

Alt-7   Select Message # 7

Alt-8   Select Message # 8

Alt-9   Select Message # 9

Alt-0   Select Message # 10

Alt-C   Select Color Attributes

Alt-K   Program key sequences

Alt-P   Select CW I/O port

Alt-X   Exit

Up Arrow        Increase TX speed

Pg Up           Increase RX speed

Left Arrow      Decrease TX weight

Right Arrow     Increase TX weight

Down Arrow      Decrease TX speed

Pg Dn           Decrease RX speed

Insert          Display review buffer

Delete          Exit review buffer display

4

All characters that are displayed on the upper screen are saved in the

"review" buffer.  Once the screen is full and data has scrolled off the top of

the screen this buffer may be displayed by pressing the Insert key then

scrolling through it by using the Home, Up-Arrow, Pg-Up, End, Down-Arrow or

Pg-Dn keys. Upon entry the current (newest) page remains on the screen. The

Up-Arrow key will display the previous line and depending on the data

available in the buffer the Pg-Up will display all or part of the previous

page. The Home key will display one page starting with the oldest available

data in the buffer. The Down-Arrow key will display the next line and

depending on current position the Pg-Dn key will display all or part of the

next page. The End key will display one page ending with the newest data in

the buffer. The console bell will sound and entry will not be allowed if the

screen is not full or scrolling past buffer ends is attempted. The Del key

will exit the review mode.

F1 (Help) Key Functions: F1 will display Help menu # 1 on the lower screen.

This menu includes Help Subjects, Printer Control and Capture Functions. Shf-

F1 will display Help menu # 2 on the lower screen. This menu includes Disk

Functions, Send Data Functions and Message Functions. Alt-F1 will display Help

menu # 3 on the lower screen. This menu includes Rx and Tx Speed Mode and

Control Functions. Ctl-F1 will display Help menu # 4 on the lower screen. This

menu includes Speaker Control, Rx and Tx Control Functions and Other

Functions.

F2 (Toggle) will turn on data output to the Printer. In this mode all data

that is displayed on the upper screen will also output to the Printer. A

carriage return and a line feed ASCII character sequence is inserted in the

data stream causing printer response at the end of each upper screen line.

Pressing F2 again will output carriage return and a line feed ASCII character

sequence and cut off data output to the Printer.

F3 (Capture) Key Functions:

F3 (Toggle) will turn on the Capture function. In this mode all data that is

displayed on the upper screen is saved in the Capture buffer. The contents of

the Capture buffer will automatically be written to disk whenever the buffer

is full. Pressing F3 again will turn off the Capture function.

Alt-F3 (Toggle) will display a file name prompt to open a Capture file. A new

file name will cause a new file to be created. If the file name exist a prompt

will be displayed. Available options are: (ESC key) to abort, (E) to erase the

existing file and start a new file or (A) to append new data to the existing

file. Once this file is open the Capture function controlled by the F3 key is

allowed. Pressing Alt-F3 again will turn off the Capture function, write the

contents of the Capture buffer to the disk and close the Capture file.

Ctl-F3  will display a file name prompt to specify a file to write the entire

contents of the review buffer to a disk. A new file name will cause a new file

to be created. If the file name exist a prompt will be displayed. Available

options are: (ESC key) to abort, (E) to erase the existing file and start a

new file or (A) to append new data to the existing file.

F4 (Disk) Key Functions:

F4 will cause the current directory file names to be displayed on the lower

screen. If more are in the directory than can be displayed a prompt is

displayed indicating that F4 can be used again to display more. Any key will

return to the normal lower screen. Program operation can continue during

directory display.

5

Alt-F4 will cause the current drive and directory to be displayed on the lower

screen followed by a prompt to change the drive or the directory. To change

the current drive enter a valid drive letter followed by a colon and return.

The new drive and directory will be displayed. A non valid drive specification

will cause an error message. To change the directory on the current drive

enter a \ followed by the directory path and return. the new drive and

directory will be displayed. ESC may be used to abort.

Ctl-F4 will display a file name prompt to specify a file to delete a disk

file. (ESC) can be used to abort.

F5 (Send) Key Functions:

F5 (Toggle) will turn on the Send File function. In this mode data contained

in the Send buffer is transmitted instead of data from the keyboard buffer.

The disk will automatically be read as required whenever the buffer is empty

until the end of file is reached. Pressing F5 again will turn off the Send

File function returning to transmission from the keyboard buffer.

Alt-F5 (Toggle) will display a file name prompt to open a file for subsequent

transmission. Once this file is open the disk will automatically be read to

transfer initial file data to a buffer and TX source selection controlled by

the F5 key is allowed. Pressing Alt-F5 again will display a prompt. Options

are to turn off the end file function and close the send file, reset the send

file pointer to the beginning of the file for a restart or continue with the

send file open.

Ctl-F5 Clears the keyboard buffer.

F6 (Message) Key Functions:

F6 will transfer the current selected message (if ready) to the transmit

buffer.

Shf-F6  will display a preview of the current selected message (if ready) on

the lower screen.

Alt-F6 will allow the operator to enter a new message into the currently

Selected Message buffer. Upon pressing Alt-F6 a prompt is displayed on the

lower screen. Options are (ESC) to abort or (Enter) to start loading indicated

by the selected message status display changing from "Empty" or "Ready" to

"Load". The message may then be entered from the keyboard. All special

character sequence keys may be used. The console bell will sound if another

input character or character sequence causes the buffer to become full. Upon

completion the Alt-F6 key must be pressed again to end the load process. The

status display will change from "Load" to "Ready".  If nothing was entered it

will change to "Empty".

Ctl-F6 will write all ten message buffers to a disk file named "CW_MSG.DAT".

This disk file is read and loads all 10 message buffers upon program

initialization.

F7 (Rx Speed) Key Functions:

F7 (Toggle) will change the RX speed control mode from Track to Manual.

Pressing F7 again will return the Rx speed control mode from Manual to Track.

In track Rx speed mode Rx speed follows the speed of the received CW signal.

To prevent received noise from a receiver not tuned to a CW signal from

driving the track mode Rx speed out of range the mode will switch to manual

mode if the track Rx speed exceeds approximately 85 WPM. In manual Rx speed

6

mode Rx speed is controlled from the keyboard.

Shf-F7 will change the Rx speed control mode from Track or Manual to Lock.

This Rx speed mode performs Rx speed Lock function by measuring the next 16 CW

pulses then calculating and setting a new Rx speed. Upon completion of the Rx

speed Lock process the Rx speed mode is automatically returned to Track.

F8 (Tx Speed) Key Functions:

F8 (Toggle) will change the Tx speed control mode from Manual to Lock Receiver

Tx speed control mode. In this mode the transmit code speed will change to and

will follow the current receiver code speed. Pressing F8 again will return the

Tx speed control mode to Manual allowing the transmit code speed to be

controlled from the keyboard.

Shf-F8 (Toggle) will change the Tx speed control mode from Manual or Lock

Receiver to Fixed Tx speed control mode. In this mode the operator is prompted

to enter the desired fixed transmit code speed. Pressing Alt-F8 again will

return the Tx speed control mode to Manual allowing the transmit code speed to

be controlled from the keyboard. The default fixed speeds are: S5, f5, 7.5,

10, 13, 15, 18, 20, 25, 30, 35, 40, 45, 50, 55, 60, 65 and 70 WPM. Slow fixed

speeds use expanded spacing. As an example the dot, dash and element space of

s5 (slow 5) are at a 13 WPM rate with character and word spacing expanded to

produce an overall 5 WPM rate described as 13e5. Similarly f5 (fast 5) is

16e5, 7.5 is 16e7.5 and 10 is 16e10. Speeds from 13 to 70 WPM use normal

spacing.

Ctl-F8 will display a prompt on the lower screen. The operator may press ENTER

to create a fixed tx speed overlay file on the default drive or may abort by

pressing any other key. Upon initial creation this file, CW_FXS.OVL will

contain the default fixed tx speed control data used by the program when

operated in fixed speed mode. Upon CW program initialization a check on the

default drive is performed to see if CW_FXS.OVL exist. If it does not exist,

the operator is informed that the  fixed tx speed overlay is not loaded. In

this case the default fixed tx speed control data are used. If it does exist

then it is read from the disk replacing the default fixed tx speed control

data and the operator is informed that fixed tx speed overlay is loaded. This

program feature allows of the fixed tx speed data to changed by editing the

overlay file. See further discussion in the file CW_FXS.DOC.

F9 (Toggle) will toggle the speaker on and off. When the speaker is on both

receive and transmit morse code elements may be heard.

F10 Key Functions:

F10 (Toggle) will toggle the operating mode. In receive mode CW input is

decoded for display and input to buffers and the transmitter control output

signal is off. In transmit mode the transmitter control output signal is on

and data from the transmit buffer is encoded and transmitted until the buffer

is empty.

Alt-F10 (Toggle) will toggle the TX control mode. In Automatic the operating

mode is switched from receive to transmit upon data becoming available in the

transmit buffer. Manual TX mode allows the operator to pretype TX data prior

to transmission.

Ctl-F10 (Toggle) will toggle the RX control mode. In automatic the operating

mode is switched from transmit to receive upon the the transmit buffer

becoming empty. Manual RX mode allows the operator to hold the transmitter on

during pauses of CW data transmission.

7

Alt-1 thru Alt-9 and Alt-0 keys select the current of ten 256 maximum

character message buffers. The message status display will indicate the

selected message number followed by "Ready" if the selected message buffer

contains data or "Empty" if it has not been loaded.

Alt-C will display a line on the lower screen indicating the  six current

color attributes in use. (1) Upper screen, (2) Lower screen, (3) Review Buffer

screen, (4) Window, (5) Window Border and (6) Status Lines. A prompt is

displayed to enter changes in the format: N=xx. N is one of the six indicated

screen areas. The attribute xx can be any number 0 thru 127. Refer to the disk

file on the distribution disk "coloratt.doc" for a complete listing. The new

color is displayed on the line upon selection. If it is not the desired select

again. When finished press ENTER to reinitialize all screen display and return

to the program. Pressing ESC at any time will abort cause return to the

program with no changes. Alt-C will not operate in video mode 7 MONO.

Alt-K will prompt the operator to select a key for the purpose of entering a

new key sequence. Any of the following may be used. After a key is selected

pressing return will prompt the operator to enter a character sequence. The

maximum length of sequences may be up to 23 characters long. Another return

will prompt the operator for Yes/No to save the new sequence to disk. If Yes

is selected the sequence is written to a disk file CW_KCS.DAT. This file is

read to memory upon program initialization. If No is selected the sequence can

be used during the current session.

'&' Ampersand key            =  ES

''' Apostrophe key           =  CQ

'(' Open Parenthesis key     =  QTH

')' Close Parenthesis key    =  (STATION LOCATION)

'*' Asterisk key             =  DX

'+' Plus sign key            =

':' Colon key                =  73

';' Semicolon                =

'<' Less than key            =  DE

'=' Equal sign key           =  RST

'>' Greater than key         =  (CALL SIGN)

'@' At sign key              =  TEST

'[' Open brackets            =

'\' Reverse alash            =

']' Close brackets           =

'^' Up Arrow key             =  QSL

'_' Underscore               =

'`' Grave accent             =

'{' Left brace               =

'|' Vertical bar             =

'}' Right brace              =

'~' Tilde                    =

Alt-P will display the currently selected CW I/O port (1), (2) or (E) and a

prompt on the lower screen. (E) indicates an error condition. Options are (1)

to select serial port #1 (3F8h), (2) to select serial port #2 (2F8h) or

(ENTER) to return to the CW program with the current displayed port selected.

Alt-X will display the "Exit CW Program ?" prompt. Options are (Y) for yes,

(S) to save the config data first and (N) or any key to return to the CW

program.