The code speed standard used by the morse code tranceiver program is

based on a word length of 50 elements. An example is "PARIS" consisting of (10

dots x 1 elm. = 10) + (4 dash x 3 elm. = 12) + (9 dot or dash elm space x 1

elm = 9) + (4 ltr space x 3 = 12) + (1 word space x 7 = 7) = 50. One word per

minute is then 60 sec./ 50 elements = 1.2 sec/elm or 1200 mili-seconds/elm.

Therefore Element time in ms = 1200/WPM or  WPM = 1200/elm time in ms.

The default fixed speeds are: S5, f5, 7.5, 10, 13, 15, 18, 20, 25, 30,

35, 40, 45, 50, 55, 60, 65 and 70 WPM. Slow fixed speeds use expanded spacing.

As an example the dot, dash and element space of s5 (slow 5) are at a 13 WPM

rate (elm length = 92.3ms) with character and word spacings expanded to

produce an overall 5 WPM rate described as 13e5. Similarly f5 (fast 5) is

16e5, 7.5 is 16e7.5 and 10 is 16e10. Speeds from 13 to 70 WPM use normal

character and word spacings.

Speed   Dot     Dash    Elmsp   Chrsp   Wrdsp

WPM     Ms      Ms      Ms      Ms      Ms

x1      x3      x1        Expanded

13e5    92.3    276.9   92.3    1442.9  3366.8

16e5    75      225     75      1527.6  3564.5

16e7.5  75      225     75      896.1   2090.8

16e10   75      225     75      580.3   1354

x1      x3      x1      x3      x7

13      92.3    276.9   92.3    276.9   646.2

15      80      240     80      240     560

18      66.7    200     66.7    200     466.7

20      60      180     60      180     420

25      48      144     48      144     336

30      40      120     40      120     280

35      34.3    102.9   34.3    102.9   240

40      30      90      30      90      210

45      26.7    80      26.7    80      186.7

50      24      72      24      72      168

55      21.8    65.5    21.8    65.5    152.7

60      20      60      20      60      140

65      18.5    55.4    18.5    55.4    129.2

70      17.1    51.4    17.1    51.4    120

The main program CW.EXE contains the fixed tx speed control data

required to produce the 18 default fixed Tx speeds as listed above. Upon CW

program initialization the file CW_FXS.OVL if it found is read from the disk

replacing the default fixed tx speed control data. This procedure allows the

the fixed Tx speeds to be changed or calibrated by editing the file CW_FXS.OVL

with "debug" or any other hex editor. The file is structured as follows:

The first 54 bytes at offset 0 through 53 (35h) are the 18 three byte

ASCII speed values. Each value must be unique and right justified with leading

blanks as required for the three byte length. These ASCII speed values are

used to select and then display the selected fixed tx speed.

The next 180 bytes at offset 54 (36h) through 233 (E9h) are, in the

same sequence as the ASCII speed values, the 18 five word (10 bite) binary

speed control values. Each group of 5 words consist of, in sequence, the dot

length, dash length, element space length, character space length and the

word space length value. Please note that the word space length value is

equal to the required word space length less the character space length.

In fixed speed mode the resolution of all CW element lengths is 100

micro seconds. (0.1 Ms). The approximate CW element speed control value is

therefore equal to: required length in ms / 0.1.

CALIBRATION PROCEDURE

1. Equipment required: A high quality counter capable of time interval

measurment. Set resolution to read .001 or .1 ms as desired.  Connect to

selected output port pin 20 (DTR).

2. Load five message buffers with about one line of the following

sequences. 1. Series of E's.  2. Series of T's.  3. Series of 5's.  4. Series

of 0's.  5. Series of E's and spaces.

3. Set leading edge trigger positive (+) and trailing edge trigger

negative (-) to measure dot or dash length. Set leading edge trigger negative

(-) and trailing edge trigger positive (+) to measure Character, Element or

Word spacing.

Signal         Trig          Trig

+ -           - +

E's            Dot           Character space

T's            Dash          Character space

5's            Dot           Element space

0's            Dash          Element space

E & space      Dot           Word space

4. Adjust CW_FXS.OVL speed control values as required.

The default fixed tx speed control values were obtained by use of the

above calibration procedure with the CW program running on a Turbo XT machine

using a 8088 processor. Slightly faster (shorter CW element lengths) were

measured on a AT 286 machine.