AA4RE BBS and the G8BPQ Switch.

From version 3.22, the switch attempts to support the AA4RE BBS. This has

been done by extending the PK232 host mode support, originally written to

support the G8UFQ BBS. The system allows the first Virtual TNC port to

support up to 8 users. The configuration I used is detailed below:

In BPQCFG.TXT:

HOSTMODE=2	; Note HOSTMODE replaces UFQBBS - for UFQ, set HOSTMODE=1

In PARAM.BB:

port

port_type       = PK-232

com             = 2			; use the first number in TNCPORTLIST

max_pac         = 120

speed           = 4800		; not actually used

max_connect     = 8

max_channel     = 9

max_pending     = 3			; seems ok, but keep an eye on minimum free buffer

monitor         = yes

time_out        = 180

fwd_min         = 15

bbs_only        = N

remote_sysop    = Y

upload_download = Y

broadcast       = y			; sends mail beacon on all radio ports

port_name       = 144.650/432.675 NODE

broadcast_path  = MAIL		; is ignored - set in config.txt file

monitor_color   = 121

no_out_fwd      = N

endport

In ROUTE.BB

G8BPQ  LEAVE

@BLANK LEAVE

*      GB7DAD

In PATH.BB

PATH GB7DAD A GB7DAD FORCE

T 0000 2300

C SWITCH				; note initial C SWITCH

SC LRG7

R* to *

SC GB7DAD

R* to *

I haven't been able to test it very throughly - please let me know how you

get on with it.

Note that the remaining Virtual TNCs continue to run as normal. If you

are running under Desqview, you can still run PAC2 (or even MBL/RLI!) in

other windows. However, AA4RE needs a LOT of store - you need to use

all the tricks described in the DV manual to get a big enough window to

run it effectively.

John Wiseman, G8BPQ.

11/7/89.

#################################################