The following contains information extracted from AA4RE's MBBIOS

Documentation. It has been modified to conform to my COMBIOS

emulation.

MBBIOS supports the standard INT 14 calls plus the extended ones as

defined by Jeff Jacobsen, WA7MBL for his BBS program.  These are

explained below.  AH=0 to AH=3 are identical to the IBM BIOS definition.

DX must contain the port number, which start at zero, so for COM1 set

DX=0, COM2 set DX=1, etc.

AH = 0     Initialize

---------------------

This is provided for compatibility only. It has no effect except to

return the current (virtual) LSR in AH, bit zero of which is set if

any characters remain in the rx buffer, and virtual MSR in AL.

THE MSR BITS SUPPORTED ARE DCD=CONNECTED

CTS=NOT BUSY

AN ATTEMPT IS ALSO MADE TO MAINTAIN DELTA BITS

BIT 7 - Receive Line Signal Detect (DCD)

6 - Ring Indicator (Not supported)

5 - Data Set Ready (Always on)

4 - Clear To Send

3 - Delta  RLSD ( ie state has changed since last

access)

2 - Not Supported (Trailing Edge Ring Detect)

1 - Not Supported (Delta DSR)

0 - Delta CTS

AH = 1     Send Character

-------------------------

AL contains the character to send.  Return values are the same as above.

AH = 2     Receive Character

----------------------------

On return, AL contains the character received.  AH contains the

virtual LSR. Again, AH bit 0 indicates if there is additional characters

in the buffer.  This routine does NOT timeout while waiting for a character

but will hang indefinitely waiting for a character to appear.

AH = 3     Return Status

------------------------

Return values are the same as returned by the Initialize routine.

AH = 4     Inquiry

------------------

Returns AX and BX set to 0AA55H.  This call is used to see if the

particular port is defined in TNCPORTLIST. Note that COMBIOS/MBBIOS

set AX, but not BX

AH = 5     Drop RTS

-------------------

This drops (virtual) DTR and RTS.  Will stop higher level code sending

any more data into the receive buffer. No return value.

AH = 6     Raise RTS

--------------------

Raise DTR and RTS.  No return value.

AH = 7     Send Break

---------------------

Generates a "Break".  Returns the virtual TNC to Command mode from Converse

or Transparent mode. No return value.

AH = 8     Non-destructive Read

-------------------------------

Returns the next character in the buffer in AL without removing it from

the buffer.  AH value defined as in AH=2 so you can use the Data Ready

bit to check to see if you really did get a character.

AH = 9     Set/Get options

--------------------------

Not implemented.

The following functions have been implemented to support AA4RE BBS,

using PK Hostmode. They should work in normal mode, but have not been

tested very extensively!

AH = A     Write buffer

-----------------------

Sends CX characters starting at DI:ES.  Upon return, DI will be updated

and CX will contain the unsent character count.  Unsent characters could

result from either BUFFER FULL or hardware handshake is OFF.  AX will

be set as in AH=3 (Status) above.

AH = B Read buffer

------------------

Receives up to CX characters starting at DI:ES.  Upon return, CX set to

actual count read and DI is updated.  If the buffer is empty, return is

immediate with CX = 0. The function will return a 'logical' line, up

to a CR in normal mode, or 17H in Host Mode. On return, AL contains the MSR

and AH has the virtual LSR. Again, AH bit 0 indicates if there are additional

characters in the buffer.

The following additional 'COMBIOS' calls are supported in versions

3.50 and above.

AH = 1F - G8BPQ NODE-SPECIFIC COMBIOS EXTENTIONS

------------------------------------------------

AL = 0 - RETURN VERSION AND NODE DS

RETURNS: AX=NODE DATA SEGMENT

DH=MAJOR VERSION

DL=MINOR VERSION    - IE FOR VERSION 3.50, DH=3, DL=50

AL = 1 - RETURN ES:SI = CONNECTED CALL (IN AX25 FORM), OR SI=0 ZERO

IF NOT CONNECTED

AL = 2 = RETURN (IN AH) NUMBER OF UNACKED PACKETS ON CONNECTION

AL = 10H DISCONNECT USER FROM VIRTUAL TNC, AND RETURN USER TO NODE

The segment registers are saved, (unless retuning a value), but other

registers may be destroyed.

#########################################