The PORTS section of the BPQCFG file is now keyword driven, with one

entry per line. The string of about a dozen params on one line was getting

a bit out of hand!

The format is:

PORT

PARAM=VALUE

..

..

ENDPORT

There are typical entries in the supplied config file, but the full list

of PARAM values supported is given below

ID=text string *		; Up to 30 chars, appear on PORTS display

TYPE=ASYNC/DRSI/PC120/RLC100/QUAD *	; Hardware Type

PROTOCOL=HDLC/KISS/ASYNC *		; ASYNC is NETROM backend link

IOADDR=xxxH *	; in hex, H is needed

INTLEVEL=n *	; 2,3,4,5 or 7 - now supports shared interrupt cards

SPEED=nnnn *	; Radio speed for DRSI, etc, async link speed for KISS

CHANNEL=A/B *	; Selects port on DRSI/PC120 or dual port KISS (eg KPC4)

BBSFLAG=NOBBS	; Stops direct connects to the BBS on this port

QUALITY=nnn *	; Default quality for nodes heard on this port. Set to zero

; to suppress NODES broadcasts on this port.

MAXFRAME=n *	; Maximum outstanding frames

TXDELAY=nnn *	; TX Keyup delay in milliseconds

SLOTTIME=100 *	; CSMA interval timer

PERSIST=nnn *	; 'Probability to transmit' value (1-255)

FULLDUP=0/1	; currently only used by KISS driver

SOFTDCD=0/1	; not yet supported

FRACK=nnnn *	; Level 2 timout in milliseconds

RESPTIME=nnnn *	; Level 2 delayed ack timer in milliseconds

RETRIES=n *	; Level 2 maximum retry value

PACLEN=nnn *	; default maximum packet length for this port

CWID=CALL	; Currently only supported on DRSI/PC120/RLC100

PORTCALL=CALL	; Additional callsign/alias for level 2 connects to the

PORTALIAS=ALIAS ; node for this port. Should not be the same as BBSCALL

VALIDCALLS=CALLS; A set of up to 8 callsigns, separated by a single space.

; If present, only stations in the list will be allowed

; to connect to you on this port.

QUALADJUST=NNN	; Percentage to reduce the quality of a NODE in the NODES

; broadcast, if the best neighbour is on the same port as

; the broadcast. Designed to improve throughput if more than

; one channel is available, by discouraging routes which

; cause a node to send a packet on the same port that it

; received it on.

Those marked with a * must be included, the rest are optional. The validation

is not good, and defaults are generally zero, so if you miss out any of the

essential ones, odd things may well happen!

##########################