MINIMUF PROGRAM OPERATION

Version 1.00

by Keith Seabourn, 5N6SKD & KD5NZ

These program calculates the Maximum Usable Frequency (MUF)

for specified paths for a 24 hour period given in GMT (UTC).

The program gives the MUF for 1 path and draws a graph.  The

user can select from 15 pre-programmed paths or can enter

the latitude and longitude coordinates for any path desired.

The program can customized for any QTH by putting the

necesary information in a setup file.

The program operates the screen in the text mode as opposed

to the graphics mode.  The text mode is common to all IBM-

compatible equipment whereas the graphics mode is not.  By

using the text mode, all access is provided to greater

numbers of amateur radio operators.  Also, the text mode

allows any printer to be used to obtain a printed copy of

the screen graph.  The printers use widely different

standards for their graphics mode of operation.

SETUP FILE

A SETUP FILE called MUFSETUP.DAT customizes MINIMUF to your

exact callsign and QTH.  It also contains 15 path choices.

This disk is originally supplied with MUFSETUP.DAT

containing information for a QTH in Dallas, Texas.  You

should use any ASCII text file editor or a word processor

that can read/write ASCII text files to create a

MUFSETUP.DAT for your exact QTH.

The MUFSETUP.DAT file should be created as follows:

Line 1 would contain your callsign; your station latitude;

your station longitude (all separated by semicolons)

Line 2 to Line 16 would contain paths in the format path

name; destination latitude; destination longitude (again

separated by semicolons)

NOTE:  MINIMUF expects Choices 16 and 17 to always be

exactly as they are written.  The user must only enter 15

path options in MUFSETUP.DAT.  The program requests

additional data if Option 16 or 17 are chosen.  The user may

freely change path options 1-15.  The latitude and longitude

conventions used in this program are:

Latitude North is positive (+)

Latitude South is negative (-)

Longitude West is positive (+)

Longitude East is negative (-)

MINIMUF DOCUMENTATION                     Page 1

An example is:

KD5NZ;33.0;97.0

To USA-W.Coast(San Fran);38.0;122.0

To USA-E.Coast (Wash DC);39.0;73.0

To USA-Hawaii;21.0;158.0

To AFRICA-Zambia;-15.0;-28.0

|

|

v

To ASIA-Japan;36.0;-140.0

To EUROPE-London;51.0;0.0

The maximum lengths of the various items are:

Station Callsign              11 characters

Station Latitude,             7 characters maximum (-xxx.xx)

Longitude

Path Name                     24 characters

Destination Lat, Lon          7 characters (same as Station

Lat, Lon)

The best way to customize the MUFSETUP.DAT is to use your

word processor/text editor to read the MUFSETUP.DAT supplied

with the program disk, called MUFSETUP.DAT.  Then modify the

station call, station latitude/longitude and any path names

and latitude/longitude.

PROGRAM OPERATION

The user works through the requested information.

1. Choice.  The user must enter a choice from 1 to 17.

Press RETURN when ready to proceed.

2. Day and month.  Enter the day and month as day <space>

month.  The <space> separates the 2 entries.  Enter

RETURN to proceed.

3. Source of solar activity.  Enter either 's' for Sunspot

data or 'f' for Solar flux data.  RETURN to proceed.

4. Sunspot number or Solar Flux.  Enter the correct value.

RETURN to proceed.

5. Flag on MUF.  A 'y' will cause the graph to show a

special character when the MUF exceeds the frequency

entered.   A 'n' to omit flag.  RETURN to proceed. If

'y' selected, the user is asked to enter frequency in

MHz.  All points which exceed this frequency are

plotted with a different character.

The MUF is calculated for all times from 0000z to 2300z at

this time.  The screen shows the MUF information both

graphically and tabular.

MINIMUF DOCUMENTATION                     Page 2

6. Want Printout (Y/N)?  Enter 'y' to print a hard copy of

this screen data.  Make sure the printer is turned on

and paper inserted before proceeding. Type an 'n' to

omit hard copy.  RETURN to proceed.

7. Plot another (Y/N)?  Enter 'y' to calculate the MUF for

another path.  If 'y' is chosen, the 17 path choices

will appear again.  However, the items 2-5 are omitted

and the previous values are repeated for the new path.

Type an 'n' to cancel program.  RETURN to proceed.

PROGRAM OUTPUT

The program presents the MUF for the chosen path on the

screen both graphically and in tabular form.  By pressing

'y' when the question Want Printout (Y/N)? is asked, the

program will print a copy of the screen.  The program sends

no special printer control characters so any printer that

can print 80 columns will work satisfactorily.  The program

will use the default state of the printer (the normal,

power-up state).

MINIMUF DOCUMENTATION                     Page 3