FOIL SIDE               Rear Panel      COMPONENT SIDE

Ground          B1                      A1      ~I/O Check

Reset (powerup) B2                      A2      Data bit 7

+5 V DC         B3                      A3      Date bit 6

IRQ 2           B4                      A4      Data bit 5

-5 V DC         B5                      A5      Data bit 4

D REQ 2         B6                      A6      Data bit 3

-12 V DC        B7                      A7      Data bit 2

Reserved        B8                      A8      Data bit 1

+12 V DC        B9                      A9      Data bit 0

Ground          B10                     A10     I/O Channel Ready

~MEMW           B11                     A11     AEN

~MEMR           B12                     A12     Address bit 19

~IOW            B13                     A13     Address bit 18

~IOR            B14                     A14     Address bit 17

~D ACK 3        B15                     A15     Address bit 16

D REQ 3         B16                     A16     Address bit 15

~D ACK 1        B17                     A17     Address bit 14

D REQ 1         B18                     A18     Address bit 13

~ D ACK 0       B19                     A19     Address bit 12

Clock 4.77MHz   B20                     A20     Address bit 11

IRQ 7           B21                     A21     Address bit 10

IRQ 6           B22                     A22     Address bit 9

IRQ 5           B23                     A23     Address bit 8

IRQ 4           B24                     A24     Address bit 7

IRQ 3           B25                     A25     Address bit 6

~D ACK 2        B26                     A26     Address bit 5

T/C             B27                     A27     Address bit 4

ALE             B28                     A28     Address bit 3

+5 V DC         B29                     A29     Address bit 2

OSC 14.31818MHz B30                     A30     Address bit 1

Ground          B31                     A31     Address bit 0

ALE     Address Latch Enable

T/C     Terminal Count (DMA)

AEN     Address Enable

MEMR/W  Memory Read/Write

IOR/W   IO Read/Write