November 7, 1988

TNC-2 Software Release 1.1.6 Notes

This document describes the differences between the latest release

1.1.6 and the previous (1.1.5) release of TAPR software for the TNC 2

Terminal Node Controller.

The reported checksum on powerup is $2E.

This release includes "KISS" capability from K3MC.

NEW COMMANDS

KISS ON|OFF                            default:  OFF

Parameters

ON     The TNC will use the KISS command set and "SLIP" protocol

for TNC-to-host communications.

OFF    The TNC will use the standard user interface.

This command is used to enter the "KISS" mode for operation with

various experimental protocols.  KISS is used with intelligent host

computers.  The most popular use has been in conjunction with the

KA9Q TCP/IP networking software, available from TAPR and elsewhere.

To operate KISS with this release, enter the command KISS ON, then

the command RESTART (NOT RESET!).  The CON and STA LEDs will flash

three times when intially activated.  This command will then have

effect for subsequent power on/off cycles.  To return to normal

operation, enter the command PARAM <device> 255 at the

NET>

prompt if you are running the KA91 TCP/IP software package.  If you

cannot send this parameter with the software you intend to use, you

must power the TNC off, then disconnect the bbRAM battery for several

minutes, then reconnect the battery and power up again.

NOTE:     While the TNC is in the "KISS" mode, normal TNC operation

is suspended and the familiar command set is inoperative.

TXUIFRAM ON|OFF                      default:  OFF

Parameters

ON     The TNC will send the contents of the outgoing transmit

buffer as UI frames to the "UNPROTO" destination when it disconnects.

OFF    The TNC will send only BEACON TEXT and periodic ID as UI

frames, and will discard the contents of the transmit

buffer when it disconnects.

This command determines whether the TNC will transmit most

unconnected information packets.

Setting TXUIFRAMe OFF will prevent all but BEACON and ID unconnected

("UI") frames from being originated and transmitted.  This is most

useful for BBS and other stations which tend to leave unacknowledged

data in the TNC transmit buffer when a connection is lost.  The

normal behavior of a TNC would send the buffered data as "UI" frames,

adding to channel congestion.

The behavior with TXUIframe OFF serves to reduce channel congestion

becasue the now useless data is never sent.

NOTE:     TXIFRAM has NO effect on the digipeating of UI frames.  It

only affects frames being originated at this station.

CHANGES

o - The TNC now requires that the Data Carrier Detect (DCD) signal

from the internal or extern modem be asserted (true) for a packed to

be received.  This change is intended to ensure that a modem with a

"threshold" control is set properly.

The result is that the station operator will set the DCD control in

order to receive data.  This then ensures that the TNC will have the

best chance of detecting other station activity, reducing collisions

that would otherwise result from jumping on a channel believed to be

open when it is really in use by another station.

This protection feature will be invisible to most operators.

NOTE:     To test with digital loopback, FULLDUP must be ON.  For

normal operation, FULLDUP should be set to OFF.