To use Terminate with windows we provide a TERMINAT.PIF and a TERMINAT.ICO.

But you must also make some adjustments to the windows setup first.

Edit SYSTEM.INI

Under [386Enh]:

Com1AutoAssign=0

Under the windows menu: Main / Control Panel / 386 Enhanced

You should read NEVER WARN at the comport you use

The comport used for modem must not be autoassigned be windows. If you

use COM4, just enter Com4AutoAssign=0

Change the values to match the correct address for the comports.

COM1BASE=3F8h

COM2BASE=2F8h

COM3BASE=3E8h

COM4BASE=2E8h

Other values that can be changed, try reading the docs for windows

COMIrqSharing=true (This has only effect on EISA og Microchannel machines)

COMBoostTime=10

COM1Buffer=1024

COM2Buffer=1024

COM3Buffer=1024

COM4Buffer=1024

If your modem has an 16550 UART you also have to install another

communication driver to support 16550 UART FIFO in DOS-windows.

Use a driver like CHCOMB.386 (CHCOMB.ARJ) (from readme file):

1) Copy the driver to the WINDOWS\SYSTEM directory.

2) Edit SYSTEM.INI:

a) Remove the line (or comment it out) that reads : device=*combuff

b) In its place, add this line                    : device=chcomb.386

To use the soundblaster card enter under [386Enh]:

DMABUFFERSIZE=059

If you have many problems using the Windows coms-driver you could use the

communication ports directly by changing WIN.INI to:

COM1:=COM1.DOS or

COM2:=COM2.DOS or

COM3:=COM3.DOS or

COM4:=COM4.DOS

Information from Microsoft:

Windows cannot use a serial port if the address is to the right of the

missing port. An example of this problem could be that the modem is on

COM4 and COM3 is not physical present.

Without a physical COM3 the BIOS will find COM4 on address 02E8 and will

call it COM3 in the BIOS field. This confuses Windows so it cannot find

COM4 and thinks that COM3 is on 02E8/IRQ4.

This behaviour is normal for many BIOS's og kan be fixed with COMxIRQ=-1

in SYSTEM.INI under the [386Enh] section. For this example should the

following line be inserted in SYSTEM.INI:

COM3IRQ=-1