DigiWARE Release Notes P/N 93000114H

for

X* DOS Driver V 4.0.9

Operating System Release  2.0.0 - 6.0

Software Package   P/N 75000114H

Diskettes          P/N 40000673H

40000674H

40000908

Software Manual    P/N 92000114C

Date 09-01-93

1.  Enhancements

-  Expanded EBIOS support in INT 14h Function 0h, Initialize a

Port. See Manual Errata.

2.  Limitations

-  The driver's INT 14h interface is not 100% compatable with the

the following DOS drivers and their supported boards:

xapcm232.sys - PC/X*, xim232mc.sys - MC/Xi, and xim232.sys -

COM/Xi. If you are using an INT14h application for an earlier

driver, compare the driver's INT 14h specifications to

determine if any of the incompatabilities may affect your

application.

3.  Bug Fixes

-  Fixed bug in xidoscfg.exe. On the MC/2e board, when the IRQ

was disabled using the POS configuration utility, xidoscfg.exe

would display an erroneous IRQ number on the driver config-

uration screen.

4.   Manual Errata

-  Please note that the DOS driver files are no longer found in

the root directory of the release diskette, but located in

the DOS directory. Any manual references to the files being

located in the root directory is in error.

- Expanded EBIOS Support

Under the EBIOS specification, the highest baud rate

accessible is 19200. The function was enhanced to provide

access to 38400, 57600 and 115200 baud rates. For the

programming interface see INT14.TXT. This enhancement was

made to permit access to higher baud rates from INT 14h

based communication programs that only provided selections

of up to 19200. The driver was changed so that when EBIOS

support is selected from the XIDOSCFG.EXE configuration

utility, the following baud rate substitutions are made :

4.   Manual Errata (continued)

110      ->     19200

150      ->     19200

300      ->     38400

600      ->     57600

1200      ->    115200

2400      ->      2400

4800      ->      4800

9600      ->      9600

So, for example, to access 57600 support, select EBIOS

support in XIDOSCFG.EXE and select 600 baud in your

communication program.