�����������������������������������������������������������������������������Ŀ

� ���� Super textmodes ������������������������������������������������������ �

�������������������������������������������������������������������������������

Terminate will allow you to set up your own videomode at startup.

When leaving Terminate you can reset to another mode if you like.

The normal thing is to reset to 80x25, but you can do whatever you like. If

the register values are all zeroes, nothing will change, so you don't have to

reset when leaving Terminate.

To set the videomode you must define at least one register like AX = 3, then

INT 10h will be called with AX,BX,CX,DX. After the call Terminate will call

INT 10h,15 (Get videomode) to determine ScreenColumns 80,132 etc. It will then

look in the BIOS area $0040:$0085 for the screenlength. If this does not work,

you can force Terminate to use a fixed screenlength by using a command line

parameter called /SCRLEN=xxx /SCRCOL=xxx.   xxx are the lines/columns you want

Terminate to think you are using.

In the following sets of registers, the first set is normally used as the

reset registers, when leaving Terminate, to set a normal 80x25 screen.

All number are in HEX 0-F format. AX=AH*256+AL, AX=FFFE (AH=FF,AL=FE).

Please send configurations if you have set up another graphics card.

If you have another SVGA card, you should look in your manual how to set

up these modes.

SCANMODE in UTILS should find all these videomodes for you, but some

videocards may need some extra information, they will be mentioned here:

Video Seven FastWrite VGA & 1024i VGA

Col/Rows  AX   BX

80x25   6F05 0003   (Reset)

80x43   6F05 0040   (Set)

80x60   6F05 0043   (Set)

100x60   6F05 0044   (Set)

132x25   6F05 0041   (Set)

132x28   6F05 0045   (Set)

132x43   6F05 0042   (Set)

TRIDENT TVGA BIOS C.2.1 1024K VGA MODE

Col/Rows AX+BX+CX+DX must be the same

80x25   0003 (Reset)

80x30   0050 (Set)

80x43   0051 (Set)

80x60   0052 (Set)

132x25   0053 (Set)

132x30   0058 (Set)

132x43   0059 (Set)

132x60   0056 (Set)

TSENG ET-4000

Col/Rows  AX

80x25   0003  (Reset)

80x60   0026  (Set)

100x40   002A  (Set)

132x25   0023  (Set)

132x28   0024  (Set)

132x44   0022  (Set)

ATI VGA Wonder, ATI VGA Wonder+

Col/Rows  AX

80x25   0003  (Reset)

132x25   0023  (Set)

132x44   0033  (Set)

OAK OTI-037, 800 x 600

Col/Rows  AX

80x25   0003  (Reset)

132x25   0050  (Set)

132x43   0051  (Set)

132x60   004F  (Set)

XGA

Col/Rows  AX

80x25   0003  (Reset)

132x25   0014  (Set)

Orchid Prodesigner VGA

Col/Rows  AX

80x25   0003 (Reset)

80x60   0026 (Set)

100x40   002A (Set)

132x25   0024 (Set)

132x28   0023 (Set)

132x44   0022 (Set)

Tecmar VGA/AD

Col/Rows  AX

80x25   0003 (Reset)

132x25   0017 (Set)

Paradise VGA & S3

Col/Rows  AX

80x25   0003 (Reset)

132x25   0055 (Set)

132x43   0054 (Set)