LPR, LPQ, LPRM

Network Printing Utilities

(for MS-DOS Computers)

Paul Hilchey

University Computing Services

The University of British Columbia

September 18, 1989

Copyright (C) 1989

The University of British Columbia

Introduction

These utilities  allow access to printers attached to remote

computer  systems   via  a   TCP/IP  network.     They   are

implementations of the like named Unix utilities.

Things You Will Need

You will  need to have access to a computer that will accept

print jobs from you.  It must be accessible from the network

and support the Berkeley Line Printer protocol (available on

most Unix systems).

Your computer  must  have  a  suitable  network  connection.

Either

1 An Ethernet connection with one of the supported

adapter boards.  For an IBM PC, XT, AT, PS/2 model 30,

or compatible, the supported ethernet adapter boards

are:

y  3COM 3C501 Etherlink

y  MICOM NI5210

y  Ungermann-Bass PC-NIC

y  Western Digital WD8003E

For IBM PS/2 models, 50, 60, 70, or 80, or compatible,

the supported boards are:

y  Ungermann-Bass NICps/2

y  3COM 3C523 Etherlink/MC

2 A hard-wired serial port supporting the SLIP protocol.

A serial connection is slower than an Ethernet

connection, but may still be adequate.

Getting Started

The software is available by anonymous FTP from mtsg.ubc.ca.

Use binary  mode to  get  the  file  LP.EXE  from  the  PC1:

directory.

Instructions on  how to  install these  utilities are in the

appendix.

Problem Reports

Problem reports may be sent to Paul_Hilchey@mtsg.ubc.ca.

Acknowledgements

Portions of  this software  were developed  by the  National

Centre for  Supercomputer Applications  at the University of

Illinois at Urbana-Champaign.

Portions of  this software were developed by The Educational

Resources Center at Clarkson University.

PostScript is a registered trademark of Adobe Systems Inc.

Unix is a registered trademark of AT&T.

IBM PC, XT, AT, and PS/2 are registered trademarks of

International Business Machines Corporation.

MS-DOS is a registered trademark of Microsoft Corporation.

NIC is a trademark of Ungermann-Bass.

EtherCard PLUS is a trademark of Western Digital

Corporation.

Ethernet is a trademark of Xerox Corporation.

3COM and Etherlink are trademarks of 3COM Corporation.

Printing a File

To print  a file, enter the lpr command followed by the name

of the  file.   For  example,  if  you  had  a  file  called

readme.txt that you wanted to print, you would use

lpr readme.txt

If all goes well, the file will be sent over the network and

then printed.   When  lpr has  finished sending the file, it

reports done and returns to the DOS prompt.

Printing multiple copies

You can print more than one copy of a file by specifying the

-# option.  For example:

lpr -#3 readme.txt

will produce  three copies  of readme.txt.   Note  that  the

remote computer  you are  sending your  print files  to  may

impose limits on how many copies you can produce.

Printing files with page breaks and page titles

To have  your document broken up into pages and printed with

page titles, use the -p option.  For example:

lpr -p readme.txt

In this  example, each  page would  have readme.txt  as  the

title.   If you  want a  title other than the file name, use

the -T option.  For example:

lpr -p -T top_secret sdi.doc

The pages  would be titled with top_secret.  Note that there

must not  be any spaces within your title.  The -T option is

only of use in combination with the -p option.

You can  also specify  the page  width to  use with  the  -w

option.  For example:

lpr -p -w72 readme.txt

will format the text into lines of up to 72 characters.  The

-w option only works in combination with the -p option.

Indenting

To have  your output  shifted away from the left margin, use

the -i option.  For example:

lpr -i readme.txt

The printout  will be  shifted to the right by eight spaces.

For different  indents, you can specify the number of spaces

to use:

lpr -i4 readme.txt

will indent  by four  spaces.   There must  not be  a  space

between the -i and the number.

Printing TeX

If you  use the TeX text processing package, you can use lpr

to print  your .DVI  files.   To do  so, you must use the -d

option.  For example:

lpr -d sample.dvi

The remote  computer that is doing your printing for you may

or may not be able to print DVI files; check with the system

administrator to  find out.  Note also that there will be no

opportunity for  you to  do a font substitution when the DVI

file is  interpreted, so you must not use any fonts that are

not available on the printer.

Printing PostScript

If the  printer you will be using understands PostScript, it

should automatically  recognize the  `%!' signature found at

the start of most PostScript files and print them correctly.

You don't need to specify any formatting options.

Printing several files

You can  print more  than one  file at  a time; just include

each file name with the lpr command.  For example:

lpr -p c:\autoexec.bat c:\config.sys -d -#2 thesis.dvi

As you  can see  in the example, you can intermix formatting

options in  with the  files to  be printed.  The options and

file names  are processed  from left  to right.    Whichever

formatting options  are currently in effect when lpr finds a

file name are the ones that are used for that file.  Once an

option has  been set,  it remains in effect until changed or

over-ridden by another option.

Using a different printer

Your output  is normally  sent to  the printer identified by

the PRINTER  and SERVER environment variables (see part 4 of

the appendix).  You can use a different printer and/or print

server by  giving the  -P and/or  -S options.  The -P option

changes the  name of  the printer  to use.   The  -S  option

changes the  address of  the print  server computer  to use.

For example:

lpr -P LW-407 -S havarti lp.doc

output will be sent to the printer called LW-407 attached to

the computer called havarti.

When you  use the -P and -S options, they apply only to that

command.     They  don't   change  the  PRINTER  and  SERVER

environment  variables.     If   the  PRINTER   and   SERVER

environment variables have not been set, you must use the -P

and -S options.

Miscellaneous options

Print jobs  are normally  separated by  a `burst page' which

identifies the  owner of  the output.   Printed on the burst

page are  the job  classification and the job name.  The job

classification defaults to your machine name unless set with

the -C  option.   The job  name defaults  to the name of the

file being  printed unless  set with  the -J  option.    For

example:

lpr -C my_output -J beaujolais wine.lst

The job  classification and  job name  must not  contain any

spaces.   If a  burst page is not necessary, you can use the

-h option to prevent it from being printed.

The -f  option is  used if  the your  file is formatted with

FORTRAN  carriage  control  characters.    FORTRAN  carriage

control is seldom used on micro-computers.

The -l  option is  used if  your file  contains non-printing

control characters that the printer can understand.

Checking the Status of Your Print Jobs

Since several people may want to use the same printer at the

same time,  and each job may take a length of time to print,

a  job  that  you  send  may  not  be  printed  immediately.

Instead, it may have to wait in a queue until the printer is

available.   To find  out if  your output  has been printed,

enter the  lpq command.  The remote computer will be queried

and the  status  of  the  jobs  it  is  processing  will  be

displayed.  For example,

lpq

Owner  Files      Job#   Job Name  Status         Size

kent   champagne  34702  caviar    Printing       19192

sue    smongo.c   31241  smongo.c  Printed        27182

paul   games.txt  34789  games     Awaiting Print 84456

As you  can see  in the example, the status display includes

other peoples'  jobs. To check the status of your jobs only,

add your name to the lpq command:

lpq paul

Only jobs owned by the user paul would be displayed.

Periodic update

To have  the status  display updated  every few seconds, use

the + option:

lpq +

The status  will be  displayed and  then refreshed every few

seconds.   To stop  the  display,  press  any  key  on  your

keyboard.  It will stop automatically when there are no jobs

to report on.  You can also set a particular delay interval:

lpq +5

will wait five seconds between display refreshes.

Using a different printer

As with  the lpr  command, lpq  normally  uses  the  printer

identified by  the PRINTER  and SERVER environment variables

(see part  4 of the appendix), but you can use the -P and -S

options to  select a different printer.  lpq only checks the

queue for the printer selected.

Cancelling Output

If you  change your  mind after sending a file for printing,

but before  it has  actually been printed, you can cancel it

with the lprm command.  For example:

lprm 34789

would cancel  job 34789.   The  job number  (34789  in  this

example) is  assigned when you submit the file for printing,

and can  be obtained with the lpq command (described above).

You can only cancel your own jobs.

If you  want to  cancel all  of your  jobs that are awaiting

print, use the - option in place of the job number:

lprm -

would cancel any of your jobs that had not yet been printed.

lprm will report the names of any files it removes.

Using a different printer

As with  the lpr  and lpq  commands, lprm  normally uses the

printer identified  by the  PRINTER and  SERVER  environment

variables, but you can use the -P and -S options to select a

different printer.

Appendix A - Installation

These installation  instructions assume  you have a moderate

amount of  technical knowledge  of PC's,  MS-DOS, and  local

area networks.   You  should have  correctly configured  and

installed your  Ethernet adapter before beginning with these

instructions.

1. Unpack archive file

LP.EXE is  a self-extracting  archive of  the files you will

need.  Simply enter LP to extract the files:

lpr.exe      - program for printing files

lpq.exe      - program to check on print status

lprm.exe     - program to cancel a print job

config.tel   - network configuration file

slip8250.com - packet driver for serial port

manual.doc   - this manual

manual.ps    - this manual, in PostScript

After unpacking  the archive,  you will  want  to  move  the

lpr.exe, lpq.exe,  and lprm.exe  files to  a directory along

your search path.

2. Install SLIP packet driver

If you  do not  have an  Ethernet connection, but do have an

asynchronous serial  port connection that supports SLIP (the

serial line  internet protocol),  you  can  still  use  this

software.   You will,  however, have  to  install  an  extra

program called  a packet  driver.    If  you  are  using  an

Ethernet connection, skip this section.

The packet  driver you  need  to  install  is  in  the  file

slip8250.com.   It is  a Terminate  and Stay  Resident (TSR)

type program; running the program will install it.  You will

want to  run the  program from within your autoexec.bat file

so it  is loaded every time you re-boot your machine.  To do

so,

1 Copy the slip8250.com file to a directory in your

search path.

2 Choose an available interrupt number in the range 0x60

to 0x7f.  On most machines, interrupt 0x60 is probably

available and can be used.

3 Determine which serial port (COM1 or COM2) you are

going to use and at what baud rate.

4 The command to install the packet driver is of the form

slip8250 <interrupt number> slip <irq number>

<port address> <baud rate>

If you are using serial port COM1, the correct values

for <irq number> and <port address> are 4 and 0x3F8.

For COM2, the correct values are 3 and 0x2F8.  For

example, to install the packet driver using COM1 at

9600 baud, you would add the line

slip8250 0x60 slip 4 0x3f8 9600

to your autoexec.bat file.

After you  have added  this line  to your autoexec.bat file,

you must re-boot your machine to have it take effect.

3. Modify configuration file

If you  use the  NCSA Telnet program on your PC, you already

have a  config.tel file  and should  use that instead of the

one unpacked from the archive.  The only change you may need

to make is to define the `myname' entry, which specifies the

name that  your machine  is known  by.   Once you  have done

that, skip to section 4.

If you  do not  already have  them, obtain  an IP address, a

subnet mask,   and  a name  for your machine from your local

network administrator.

Next,  edit   the  config.tel  file  to  reflect  the  above

information, and  also the  type and  configuration of  your

network connection.   Only  seven  of  the  entries  in  the

config.tel file are essential:

myname=fred.phys.ubc.ca

Your machine  must have  a name  that it  is to be

known  by.     Check   with  your   local  network

administrator to find a suitable name.

myip=128.189.123.456

Your machine must have a unique address.  Using an

incorrect address  can cause  problems  for  other

people on  the network,  so be  careful.  See your

local network administrator if you need help.

netmask=255.255.0.0

Again, see your network administrator.

hardware=wd8003

This entry identifies the type of Ethernet adapter

you have, or if you are using a packet driver.

Value     Ethernet Board           Requires

3C501     3COM 3C501 Etherlink     interrupt,

ioaddr

PCNIC     Ungermann-Bass PC-NIC    address, ioaddr

NI5210    Micom NI5210             address, ioaddr

WD8003    Western Digital WD8003E  address, ioaddr

NICps2    Ungermann-Bass NICps/2   address

3C523     3COM 3C523 Etherlink/MC  address, ioaddr

packet    FTP packet driver        address, ioaddr

Depending on  the Ethernet  adapter type,  you may

need to  set the  interrupt, ioaddr,  and  address

entries as indicated above.

interrupt=5

The hardware  interrupt vector  number  that  your

Ethernet adapter was configured for (Only required

for 3C501 adapter).

address=d000

The segment address of the Ethernet board's shared

memory, entered  as four  hexadecimal digits.  Set

to  the   value  that  your  Ethernet  adapter  is

configured for.   If you are using a packet driver

interface, set to 0.

ioaddr=360

The I/O  base address  of the  Ethernet  board  in

hexadecimal.   Set to the value that your Ethernet

adapter is  configured for.   If  you are  using a

packet driver  interface,  set  to  the  interrupt

number (do not include the 0x prefix).

Edit your  autoexec.bat file  to  the  set  the  environment

variable CONFIG.TEL  to be  the location  of your  config.tel

file.  For example:

set configtel=c:\utility\config.tel

Complete documentation  on the  configuration  file  can  be

found in  chapter 6 of NCSA Telnet for the PC, Version 2.2TN

and Version 2.2D.

4. Set up a default printer and server

In order to use these utilities you need to specify the name

or address  of a  print server,  the computer  that receives

files from  you and  prints them.   Since a print server may

have several  printers attached  to it,  you  also  need  to

specify the  name of  the printer that you want to use.  You

obtain this information from the system administrator of the

machine that is to be your print server.

You could give the printer name and server address each time

you use  one of  the printing  utilities, but  it is usually

more convenient  to put  them  in  the  PRINTER  and  SERVER

environment variables.   The SERVER environment variable, if

defined, gives  the name  or Internet  address of  the print

server to  use.   The PRINTER environment variable gives the

name of  the printer  on the  server machine  that is  to be

used.   You will probably want to set the PRINTER and SERVER

variables to suitable values in your autoexec.bat file.  For

example:

set printer=cntr

set server=128.189.103.1

If you  later have  a  file  that  you  want  printed  on  a

different printer,  you can either set PRINTER and/or SERVER

to that  printer, or  use the  -P and/or -S options with the

lpr, lpq, and lprm commands.

Appendix B - Notes for System Administrators

These notes  are directed  to the  administrator of  a  Unix

system who  wishes to  allow use  of the  system as  a print

server.   They briefly  describe the  steps needed under Sun

OS, but  other Unix  systems should  be similar.  You should

also refer  to sections  12.3 (etc/hosts),  19.4 (hosts.lpd)

and chapter  22 (name  server) of the SunOS System & Network

Administration manual.

The first  step is  to add  entries in  the LPD access file,

/etc/hosts.lpd, for  the machines  that are  to have access.

Edit the  file and insert the names of the machines, one per

line.   Note that  an IP  address is  not acceptable, so all

microcomputers that  are to  have access  must be assigned a

name.

The second  step is  to add entries to either the /etc/hosts

file (if  you are  not using a domain name server) or to the

name server  database (if  you are  using  a  name  server).

Entries in  the /etc/hosts  file consist of IP addresses and

names, one entry per line.

If you are using a name server, list your named.boot file to

find out  what the  relevant hosts  and rev.hosts  files are

called.   Next, edit  those files,  being sure to update the

serial numbers  at the  start of  them.  To make the changes

take effect, use the Unix command:

kill -HUP `cat /etc/named.pid`

This will cause the name server daemon to re-read the files.