ZIP'R Professional For DOS Version 1.11

Copyright 1993

by

Stevenson Technical Services, Inc. (STSI)

108 Second Avenue West

Bertha MN 56437-0297

Phone: (218) 924-2050

FAX: (218) 924-2050 (Ext. 11)

The Bertha BBS: (218) 924-2060, 8,N,1

������������������������������������������

�                Welcome!                 �

������������������������������������������

Thank you for trying ZIP'R Professional For DOS Version 1.11!  ZIP'R Pro is a

program designed to use an existing compression program, such as PKZIP /

PKUNZIP, ARJ, or LHA, to automatically compress entire programs (including

sub-directories) when programs are exited, and decompress them prior to

execution.  This process saves valuable hard drive space by storing programs

in compressed form when they are not in use.  ZIP'R Pro even achieves a

substantial amount of compression on hard drives that already use an on-the-

fly compression program, such as SuperStor or Stacker!  ZIP'R Pro is unique

because it not only compresses and decompresses entire programs, but it

launches them as well!  This feature makes it possible to fully integrate

ZIP'R Pro into most, if not all menuing systems.

If you appreciate ZIP'R Pro For DOS, you are encouraged to call The Bertha BBS

at (218) 924-2060, and let us know!  Comments, suggestions, and enhancement

requests are always welcome!

������������������������������������������

�              Requirements               �

������������������������������������������

To run ZIP'R Pro reliably, you will need the following:

� DOS 3.3 or Higher

� IBM PC, PC-Compatible or MS-DOS Computer with a Hard Drive and 640K RAM

� Any one of the following compression programs: ARJ, LHA, PKZIP/PKUNZIP

� EGA or VGA Color Monitor Recommended

Page 1

������������������������������������������

�           Table of Contents:            �

������������������������������������������

Title Page & Welcome . . . . . . . . . . . . . . . . . . . . . . . .    Page 1

Requirements . . . . . . . . . . . . . . . . . . . . . . . . . . . .    Page 1

Quick-Start Instructions . . . . . . . . . . . . . . . . . . . . . .    Page 3

Disclaimer . . . . . . . . . . . . . . . . . . . . . . . . . . . . .    Page 3

Registration . . . . . . . . . . . . . . . . . . . . . . . . . . . .    Page 4

Technical Support  . . . . . . . . . . . . . . . . . . . . . . . . .    Page 4

What Is ZIP'R Pro? . . . . . . . . . . . . . . . . . . . . . . . . .    Page 5

ZIP'R Pro Feature List . . . . . . . . . . . . . . . . . . . . .        Page 5

10 Reasons To Use ZIP'R Pro  . . . . . . . . . . . . . . . . . . . .    Page 6

How Does ZIP'R Pro Work? . . . . . . . . . . . . . . . . . . . . . .    Page 7

Installing ZIP'R Pro . . . . . . . . . . . . . . . . . . . . . . . .    Page 7

Using The ZIF Editor Pro . . . . . . . . . . . . . . . . . . . . . .    Page 8

Executing The ZIF Editor Pro . . . . . . . . . . . . . . . . . . . .    Page 9

Editing .ZPD Files . . . . . . . . . . . . . . . . . . . . . . . . .    Page 9

Opening and Deleting Files . . . . . . . . . . . . . . . . . . . . .   Page 17

ZIP'R Pro Default Settings . . . . . . . . . . . . . . . . . . . . .   Page 17

Using ZIP'R Pro  . . . . . . . . . . . . . . . . . . . . . . . . . .   Page 21

Using ZIP'R Pro With A Menuing System  . . . . . . . . . . . . . . .   Page 21

What Happens When I Execute ZIP'R Pro? . . . . . . . . . . . . . . .   Page 22

Networks / Multitaskers  . . . . . . . . . . . . . . . . . . . . . .   Page 25

DESQview Tips & Techniques . . . . . . . . . . . . . . . . . . . . .   Page 25

Using ZIP'R Pro With Windows . . . . . . . . . . . . . . . . . . . .   Page 27

How Effective Is ZIP'R Pro?  . . . . . . . . . . . . . . . . . . . .   Page 27

Cautions When Using ZIP'R Pro  . . . . . . . . . . . . . . . . . . .   Page 29

Troubleshooting / Common Problems  . . . . . . . . . . . . . . . . .   Page 30

Questions And Answers  . . . . . . . . . . . . . . . . . . . . . . .   Page 32

Comments From ZIP'R Users  . . . . . . . . . . . . . . . . . . . . .   Page 33

Programs Mentioned . . . . . . . . . . . . . . . . . . . . . . . . .   Page 34

ZIP'R Pro Registration Form  . . . . . . . . . . . . . . . . . . . .   Page 35

Page 2

������������������������������������������

�        Quick-Start Instructions:        �

������������������������������������������

To get ZIP'R Pro up and running in a hurry, follow these Quick-Start

instructions:

1.  Run the Install program by typing "INSTALL" at the DOS prompt.

2.  Reboot your machine to ensure that any changes made by the Install program

to your AUTOEXEC.BAT file have taken effect.

3.  Execute the ZIF Editor Pro from the \ZIPRPRO sub-directory (or wherever

you installed ZIP'R Pro) by typing "ZIFPRO", and create a new .ZPD file

for the program you want to compress and launch through ZIP'R Pro.

4.  Once you have created a .ZPD file, exit the ZIF Editor Pro, make sure you

are still located in the ZIP'R Pro program directory, and type "ZIPRPRO

PROGRAM".

Examples of potential command lines could be:

ZIPRPRO PCSHELL

ZIPRPRO PSDELUXE

5.  If the fields of the .ZPD file have been entered correctly, ZIP'R Pro will

display the prelog screen, and then execute the program you designated in

the .ZPD file.  When you exit the program, ZIP'R Pro will use the

compression type you selected to compress the program directory (including

all sub-directories).  When it is completed, unregistered users will see a

shareware nag screen before ZIP'R Pro returns to a DOS prompt or the menu

system being used.

6.  The next time you want to run this program, use the same command line.

ZIP'R Pro will display the prelog screen, decompress the program, and

execute it normally.  Once again, after exiting the program, ZIP'R Pro

will compress the program and exit.

7.  To calculate how much disk space you are gaining by using ZIP'R Pro, check

the free hard drive space prior to compressing a program, and again after

compressing the program.  Subtract the two numbers, and the total will be

the disk space saved through using ZIP'R Pro.  The more programs you

choose to compress, the greater the savings will be!

������������������������������������������

�               Disclaimer:               �

������������������������������������������

By using ZIP'R Professional For DOS and/or the ZIF Editor Professional, users

indicate their acceptance of the following terms:

"ZIP'R Professional For DOS and the ZIF Editor Professional are supplied as

is.  The author disclaims all warranties, expressed or implied, including,

Page 3

without limitation, the warranties of merchantability and of fitness for any

purpose.  Neither STSI, The Bertha BBS, or any STSI employees will be held

responsible for any damages, direct or consequential, which may result from

the use of ZIP'R Professional For DOS and/or the ZIF Editor."

"Information in this documentation is subject to change without notice and

does not represent a commitment on the part of STSI.  STSI may make

improvements and/or changes in this documentation or in the products and/or

programs described in this documentation at any time."

������������������������������������������

�              Registration:              �

������������������������������������������

ZIP'R Pro is distributed under the shareware concept.  You may try ZIP'R Pro

for a period of two weeks.  After this period, you must either discontinue

using it, or register it.  A single-user license is only $49.95 (+$5.00 S&H in

the U.S., or $12.50 S&H on Foreign Orders).  This price includes a printed

manual, 3 1/2" and 5 1/4" diskettes, along with a Registration Name and Number

allowing you to register your copy of ZIP'R Pro.

To receive a temporary registration number over the telephone, call STSI at

(218) 924-2050, and charge it to your Visa, MasterCard, American Express, or

Discover credit card.  Upon request, you will receive a temporary registration

name and number over the telephone.  This will allow the modification and/or

removal of the prelog delay time, along with any nag screens.

Note: For registered users of the original ZIP'R For DOS, there is an upgrade

path available.  A $20.00 upgrade fee will cover the difference in price

between the two products, as well as the cost to produce a new manual and ship

the complete package to you.

For further registration information and pricing, please read the REGISTER.DOC

file included with ZIP'R Pro.  There is also a registration form contained at

the end of this documentation file.

������������������������������������������

�           Technical Support:            �

������������������������������������������

Technical support is available to ZIP'R Pro users through the following means:

The Bertha BBS:    New versions of ZIP'R Pro will be posted here as soon as

they are available.  The Bertha BBS uses a U.S. Robotics

Dual-Standard Modem, which is capable of V.32bis, V.32,

HST, and 2400 baud connect speeds.  The access number for

The Bertha BBS is (218)-924-2060.  Technical support

questions should be addressed to the SYSOP.

STSI FAX:          STSI has a FAX machine on-line 24 hours-a-day to accept FAX

transmissions.  Technical support questions / problems may

be faxed to (218) 924-2050.  Most FAX transmissions will be

automatically routed to the FAX machine.  If you have

problems reaching our FAX machine, wait until after the

first ring, press 11, and then begin transmitting.

Telephone:         Technical support calls may be placed at (218) 924-2050.

Page 4

������������������������������������������

�           What Is ZIP'R Pro?            �

������������������������������������������

ZIP'R (pronounced "zipper") Pro is a program designed to use an existing

compression program, such as PKZIP / PKUNZIP, ARJ, or LHA, to automatically

compress entire programs (including sub-directories) when programs are exited,

and decompress them prior to execution.  This process saves valuable hard

drive space by compressing program when they are not in use.  ZIP'R Pro even

achieves a substantial amount of compression on hard drives that already use

an on-the-fly compression program, such as SuperStor, Stacker or DoubleSpace!

ZIP'R Pro is unique because it not only compresses and decompresses entire

programs, but it launches them as well!  This feature makes it possible to

fully integrate ZIP'R Pro into most, if not all menuing systems.

������������������������������������������

�         ZIP'R Pro Feature List          �

������������������������������������������

ZIP'R Pro is a spin-off of the popular ZIP'R For DOS product.  ZIP'R Pro

contains most or all of the features found in ZIP'R, and adds other powerful

features as well.  If you like all the extra bells and whistles, you'll love

ZIP'R Pro!  The following list shows a comparison of the two products, and

details some of the highlights of ZIP'R Pro:

Feature                                          ZIP'R          ZIP'R Pro

Automatic Compression                             Yes             Yes

Automatic Decompression                           Yes             Yes

Automatic Program Launching                       Yes             Yes

Query Option (Optional Compression)               Yes             Yes

Update Option (Fast Recompression)                Yes             Yes

Sorted Extraction (PKUNZIP Only)                  Yes             Yes

Compression to Floppy Disks (PKZIP Only)          No              Yes

Store Compression Files On Any Drive/Directory    No              Yes

Compress/Decompress Only Option                   No              Yes

Automatic Network/Multitasker Detection           No              Yes

Windowed Compression/Decompression                No              Yes

Heap Compaction (Eliminates Unnecessary Swapping) No              Yes

On-Line Help Available In ZIF Editor              No              Yes

IN/OUT Commands Handled In ZIF Files              No              Yes

Atomic Lock Files Used                            No              Yes

Automatic Cluster Size Detection                  No              Yes

Swap To EMS Or Disk Only When Necessary           No              Yes

Option To Test Archive Integrity                  No              Yes

Improvements From ZIP'R To ZIP'R Pro:

* Significant speed enhancements in Update mode

* ZIP'R Pro is smaller and faster than ZIP'R

* Swapping routine frees about 5 - 10K more memory

* Improved (more extensive) error trapping in ZIP'R Pro

* AFSC (Accurate File Size Calculation) now has 4 levels, for specific

customization

Page 5

������������������������������������������

�       10 Reasons To Use ZIP'R Pro       �

������������������������������������������

1.  Saves Valuable Hard Drive Space!

Even though hard drive prices have dropped substantially in the past

year, the prices are still out of range for many computer users.  In

addition, computer programs have become much larger, requiring far more

space than they used to.  ZIP'R Pro can double, and in some cases

triple, the amount of data that can be stored on a hard drive.  Programs

like SuperStor, Stacker, and DoubleSpace typically achieve compression

ratios around 30 - 35%.  Even hard drives that use these on-the-fly

compression programs can usuallly save an extra 20% or more.

2.  Does Not Require 40 - 50K Of Valuable Memory!

Programs like SuperStor, Stacker, and DoubleSpace require that you

surrender 40 - 50K of valuable conventional memory to use their

services.  ZIP'R Pro does not!  For users who are already cramped for

conventional memory, ZIP'R Pro is clearly the way to go.

3.  Automatically Executes Programs After Decompressing Them.

ZIP'R Pro automatically launches programs after they are decompressed.

This allows you to continue with your work while ZIP'R Pro takes care of

the rest.  Rather than manually decompressing, launching, and

recompressing your program, ZIP'R Pro makes the entire process

automatic.

4.  Works On Networks And Under Multitaskers.

ZIP'R Pro has the built-in ability to detect when program execution is

occurring in a network environment and under a multitasker.  ZIP'R Pro

takes extra steps to protect your data, such as making sure that no one

else can access your program at the same time you are using it.

Potentially dangerous situations are monitored and handled elegantly by

ZIP'R Pro.

5.  Safe and Reliable!

One comment we receive regularly from our users is that ZIP'R is "safe

and reliable".  We have carried that same reliability into ZIP'R Pro.

Our company has never had data corrupted when using ZIP'R or ZIP'R Pro!

6.  Store Compressed Programs On Any Drive And In Any Subdirectory.

One new feature of ZIP'R Pro is the ability to store compressed files on

any drive and in any subdirectory.  This is a powerful feature, giving

you complete control of the compression and decompression processes.

7.  Compress Programs To Floppy Diskettes (PKZIP only)

Having the ability to store compressed files on floppy diskettes gives

you more flexibility, allowing you to completely remove programs from

your hard drive, if necessary.  A special switch within the ZIF Editor

Pro allows you to turn on DOS Verify, ensuring the integrity of your

data.

8.  Much Cheaper Than Buying A New Hard Drive!

ZIP'R Pro is a great way to double your hard drive capacity without

breaking your budget.  Priced at just $49.95 (+$5.00 S&H, USA) (see

REGISTER.DOC for more information), ZIP'R Pro gives you the extra space

you need to delay the purchase of that next hard drive.

9.  Compress programs at will using Query option.

Using ZIP'R Pro's Query option, users can decide on-the-fly whether or

not to compress a program.  With a simple keypress, ZIP'R Pro either

recompresses the program, or leaves it uncompressed during times when

Page 6

you're in-and-out more than usual.

10. Update option makes program recompression fast and easy!

Using the Update option, (available only with PKZIP), ZIP'R Pro can

significantly speed up the recompression process.  Rather than waiting

for a completely new compressed file to be created, the old file is

updated with any changes that have occurred since the file was

decompressed.  You'll be amazed at the dramatic difference in speed!

������������������������������������������

�        How Does ZIP'R Pro Work?         �

������������������������������������������

ZIP'R Pro works in combination with existing compression programs to compress

directories, sub-directories, and the files they contain into one smaller,

compressed file on the hard drive.  This allows an entire directory filled

with files to be stored in compressed form when not in use, saving a

substantial amount of hard drive space.

When a program is executed, ZIP'R Pro invokes the proper compression program

to decompress the desired directory, and executes the requested program.

During this time ZIP'R Pro swaps itself to disk or EMS memory, leaving behind

only about 2.5K plus the size of the command processor and its environment.

This usually is less than 10K.  When the program is exited, ZIP'R Pro invokes

the chosen compression program to compress the entire directory.

������������������������������������������

�          Installing ZIP'R Pro           �

������������������������������������������

The first step in testing a new piece of software is to perform a backup and

test its integrity.  Since ZIP'R Pro will compress entire directories into a

single file, your potential losses are much greater if this file becomes lost

or corrupted.

Log to the drive where you are installing ZIP'R Pro from, and type "INSTALL".

The ZIP'R Pro For DOS installation program will ask you where you are

installing ZIP'R Pro from (usually, the current directory), and the location

where the program files should be stored.  (The default destination directory

is C:\ZIPRPRO.) If you are upgrading from a previous version, ZIP'R Pro will

issue a warning before overwriting the ZIPRPRO.INI file.  If you have already

set up your registration information and user preferences in the ZIPRPRO.INI

file, answer "N" (No) to instruct ZIP'R Pro not to overwrite this file.  When

the installation is complete, ZIP'R Pro will display a completion message and

return to the operating system.

ZIP'R Pro can be installed on any drive in your system, including network

drives.

Important!   Be sure to include a SET ZIPRPRO=N:\ZIPRPRO statement (or

wherever you put ZIP'R Pro), in your AUTOEXEC.BAT file.  If this

environment variable is not found, ZIP'R Pro defaults to using

the C:\ZIPRPRO sub-directory.  (If either of those directories

cannot be found on your system, the current directory is used.)

Page 7

������������������������������������������

�        Using The ZIF Editor Pro         �

������������������������������������������

The ZIF (ZIP'R Information File) Editor Pro is a user-friendly interface that

was created to assist ZIP'R Pro users in the maintenance of .ZPD files, and

ZIP'R Pro configuration files.  The ZIF Editor Pro allows the direct editing

of the following files:

.ZPD Files         ZIP'R Pro For DOS Information Files (.ZPD Files) contain

program specific information that instruct ZIP'R Pro how to

execute a particular program, and what archiving utility

should be used.  This information is used to decompress,

execute, and re-compress program files and their sub-

directories.  These .ZPD files are referenced from the

ZIP'R Pro command line, and make ZIP'R Pro an easy-to-use

utility for novice and experienced users alike.

ZIPRPRO.INI File   This file contains program defaults for ZIP'R Pro.  From

within the ZIF Editor Pro, users can enter their

registration name and number, and alter several program

settings described later in this documentation.

The following are a few technical notes about specific editing features within

the ZIF Editor Pro.

Editing Keys:           The following keys are available to edit or type new

text into most text fields.

HOME            Move to beginning of field

CTRL+HOME       Move to beginning of field

END             Move to end of field

CTRL+END        Move to end of field

INSERT          Insert Mode / Overstrike Mode

DELETE          Remove character over the cursor

BACKSPACE       Remove character left of the cursor

LEFT ARROW      Move cursor one character to the left

RIGHT ARROW     Move cursor one character to the right

TAB             Move cursor to next field

SHIFT+TAB       Move cursor to previous field

ALT+DOWN ARROW  Show list on drop-down list box

SPACE BAR       Highlight file in file list box

Upper Case Characters:  When new field entries are added to the ZIF Editor

Pro's editing screens, the ZIF Editor Pro will

automatically convert them to UPPER CASE characters.

This speeds up the typing process and provides

consistency within the ZIF Editor Pro.  The only field

that serves as an exception to this rule is the

"Parameters:" field.  Some programs are case sensitive

when accepting passed parameters, so the ZIF Editor

Pro does not convert the characters in this field to

upper case.

Page 8

������������������������������������������

�      Executing The ZIF Editor Pro       �

������������������������������������������

To invoke the ZIF Editor Pro, type "ZIFPRO" from the \ZIPRPRO sub-directory

(or wherever you store ZIP'R Pro), and press [ENTER].  The ZIF Editor Pro's

desktop space will display the main menu, along with a logo containing version

and copyright information.

������������������������������������������

�           Editing .ZPD Files            �

������������������������������������������

The "File" menu allows users to create a "New" .ZPD file, "Open" an existing

.ZPD file, or "Delete" an existing .ZPD file.  By selecting "New" from the

file menu, users have access to the following fields.

ZIF (ZIP'R Information File) Filename

The entry typed in this field will be the name of the .ZPD file used to

contain the program preferences entered on this screen.  This file will be

referenced on the ZIP'R Pro command line to execute this program.  This

filename should reflect the name of the program being executed.  A full

path is required.

Executable File

This field should contain the full path and filename of the file to be

executed.  You must add the correct extension of the file you wish to

execute.  If you specify an incorrect extension, ZIP'R Pro will not be

able to find the correct file.

Parameters:

This field can optionally contain any necessary parameters that should be

passed to the program being executed.  These parameters are passed as one

variable, and since some programs require case sensitive program

parameters, ZIP'R Pro does not automatically convert the text in this

field to upper case.

Maximum Users

If you are using ZIP'R Pro in a network environment or under a multitasker

such as Windows or DESQview, specify the maximum number of concurrent

users (or accesses) that should be allowed for this program.  For example,

if a network program is only licensed for five users, you will want to

enter 5 in this field.  This tells ZIP'R Pro not to allow more than 5

users into this particular program at the same time.  If you are running a

multitasker, you may run the risk of opening multiple sessions of the same

program at the same time.  It would be to your benefit to enter 1 in this

field.  That way, ZIP'R Pro will only allow you to execute one copy of

this program at a time.  The default for this field is 1.  (Special Note:

The unregistered version of ZIP'R Pro only supports a maximum of 5

concurrent users in one program at the same time.  In order to have access

for more than 5 users, you must register ZIP'R Pro.)

Compressed File:

Enter the full path and filename of the compressed file that will be

created and used by ZIP'R Pro in this field.  After you exit from the

Page 9

chosen program, ZIP'R Pro creates the compressed file in the location

specified in this field, along with the filename you specify.  Do not add

an extension to the filename you type.  When ZIP'R Pro creates the

compressed file, it will append a .ZIP extension for files created with

PKZIP, a .ARJ extension for files created with ARJ, or a .LZH extension

for files created with LHA.

Note:  The compressed file can be created on any drive and in any sub-

directory, including floppy diskettes.  Common locations for the

compressed file include the directory where the program is executed from,

a special archive directory on your hard drive, a floppy diskette, or

network drive.

Compression Type

This field should contain the name of the compression program you want to

use to compress the specified program and its sub-directories.  The

following table shows the compression utilities currently supported by

ZIP'R Pro, and the corresponding entry for this field.

Compression Utility                       Field Entry

ARJ Version 2.30                          ARJ

LHA Version 2.13                          LHA

No Compression                            NONE

PKZIP Version 1.10                        ZIP

PKZIP Versions 2.04c, 2.04e, and 2.04g    ZIP

If "NONE" is selected as the Compression Type, ZIP'R Pro will decompress

an existing compressed file prior to executing the desired program, but

will not compress the files in the directory after the program is exited.

(Advanced compression options are described in detail in the "Advanced

Compression Options" section of this documentation.)

The default entry for this field is "ZIP".  To change this entry, use the

mouse to click on the down arrow located to the right of the field, or

press [ALT] + Down Arrow.  Click on the compression program you want to

use, or highlight it with the arrow keys and press [ENTER].

Test Integrity

When Test Integrity is checked, ZIP'R Pro will tell the compression

program being used to perform an integrity check on the archived files.

Query Option

The "Query" option is a ZIP'R Pro feature that allows users to choose,

after the program is exited, whether or not it should be compressed.

If this box is checked, ZIP'R Pro will query (ask) the user whether or not

the program should be compressed each time it is exited.  This allows the

user to have more control over the compression process, instead of having

to decide in advance whether a program should be compressed.

Update Option (PKZIP Only)

The "Update" option is a ZIP'R Pro feature which can significantly reduce

the time required to recompress a program.  Normally, when ZIP'R Pro

decompresses a program, it extracts all of the program files and sub-

directories, and deletes the old compressed file.  During the compression

Page 10

process, ZIP'R Pro creates a new compressed file, and moves all of the

program files and sub-directories into it.  This method makes the most

efficient use of hard disk space, but can require a significant amount of

time to recompress the program, depending on the number and size of files

that need to be compressed.

The "Update" option of ZIP'R Pro can save a tremendous amount of time in

this process.  Instead of deleting the old compressed file when the

program files and sub-directories are extracted, ZIP'R Pro leaves it in

its original location.  When the compression process starts, ZIP'R Pro

uses PKZIP to update any files and/or sub-directories that have been added

or changed, and deletes any files and/or sub-directories that have been

deleted since the program was extracted.  In essence, if no files or sub-

directories were added, changed, or deleted since the program was

decompressed, PKZIP does not need to update any files.  The old compressed

file is left in the directory while PKZIP deletes the program's files and

sub-directories.

Placing a check mark in this box instructs ZIP'R Pro to use the "Update"

method when compressing this program.  (Note: The "Update" option is only

available for PKZIP / PKUNZIP.)

The amount of time that can be saved when using the "Update" option varies

between programs.  I encourage you to try this option on a few of your own

programs.  Here are a few hints that I've figured out from experimenting

with this option.

1.  The greatest difference in speed will be noticed on programs that have

large files and/or files that have not changed.  It takes a very long time

to compress large files using the normal compression method.  With the

update method, if PKZIP detects that these large files are identical to

the ones already maintained in the compressed file, all it needs to do is

delete these files out of the directory, and the recompression process is

complete!

2.  You may notice that any lower-level sub-directories within the program

are always updated when the update method is enabled.  Since directories

are deleted when a program is compressed, PKZIP must create these

directories before decompressing the program.  When PKZIP attempts to

update the compressed file, it detects that the directories have

"changed", and therefore, is forced to update them in the compressed file.

3.  This option is useful to speed up the recompression process on most

programs, but it may actually increase the time of compression on others.

The programs that may not compress as quickly with the update method are

ones that have large numbers of small files and/or ones that have many

files added, changed, or deleted during each session.  If you're not sure,

use a watch and time each method to see which way would save the most

time.

Advanced (Advanced .ZPD Options)

Program's Parent Directory

Advanced users will want to use this optional field for programs with

unique requirements.  While most programs maintain their primary

executable file in the parent program directory, some programs maintain

their primary executable file in a lower-level sub-directory.  A good

Page 11

example of this problem is found with Borland Pascal 7.0.

C:\

��BP       <---- Most executable files are located

��PASCAL       in this sub-directory.

��DOC

��BIN

��BGI

��EXAMPLES

��RTL

��UNITS

��PENWIN

C:\

��BP

��PASCAL

��DOC

��BIN    <---- Borland Pascal places the file BP.EXE

��BGI          in the C:\BP\BIN sub-directory.

��EXAMPLES

��RTL

��UNITS

��PENWIN

In order to compensate for situations like this, ZIP'R Pro (along with

the ZIF Editor Pro), allows users to specify a parent directory for the

program.  In the case of Borland Pascal, the Executable File field given

on the front screen would contain "C:\BP\BIN\BP".  To compress all files

and directories from the main subdirectory (C:\BP and below), the

Program's Parent Directory field would contain "C:\BP".

Compress / Decompress Program Only (Do Not Execute)

This advanced feature of ZIP'R Pro allows you to specify whether or not

ZIP'R Pro should attempt to execute the program file once the program

has been decompressed.  If this box is checked, ZIP'R Pro will only

decompress the program if it is in compressed format, and only compress

the program if it is in non-compressed format.  This option is

potentially dangerous to the novice user.  Caution is advised.

Swapping

The three fields, labeled "Program Swapping", "IN File Swapping", and

"OUT File Swapping" allow you to specify when ZIP'R Pro should swap

itself to EMS or disk.  If ZIP'R Pro does not have to swap, execution of

the Program, IN Commands, and/or OUT Commands occurs much more rapidly

than if swapping is necessary.  There are three options available for

these fields.

Automatic Swap

This instructs ZIP'R Pro to swap itself to EMS or disk ONLY if there

is not enough memory available, as specified by the corresponding Trip

Point.

Always Swap

This option instructs ZIP'R Pro to always swap itself to EMS or disk,

no matter how much memory is available.  This is the safest method for

executing programs and/or DOS commands, although it can consume more

Page 12

time.

Never Swap

This instructs ZIP'R Pro that it should never swap to EMS or disk, no

matter how much (or how little) memory is actually available.  Use

this option carefully, as it could cause programs and DOS commands to

issue out of memory error messages.

Trip Point

This option allows users to specify the trip point (or memory

availability requirement) that should be used for ZIP'R Pro to decide

when it should swap itself to EMS or disk.  The trip point only works

when "Automatic Swap" is selected as the swapping type.

Compression Options

These PKZIP / PKUNZIP options may be used to choose less compression in

exchange for more speed, or more compression in exchange for less speed.

ZIP -e0 switch.  (PKZIP 2.0 Only)

ZIP -es switch.  (PKZIP 1.1 or 2.0)

ZIP -ef switch.  (PKZIP 2.0 Only)

ZIP -en switch.  (PKZIP 2.0 Only)

ZIP -ex switch.  (PKZIP 1.1 or 2.0)

These figures were generated using PKZIP 2.04c.

No Compression       ZIP Results   Compression

ZIP -e0        2,980,853           2,985,751        0%

ZIP -es        2,980,853           1,535,935       48%

ZIP -ef        2,980,853           1,432,248       52%

ZIP -en        2,980,853           1,401,026       53%

ZIP -ex        2,980,853           1,392,625       53%

In addition to these options, additional parameters may be added to the

PKZIP 2.04c, 2.04e, and 2.04g command lines to solve specific hardware and

software compatibility problems.  These extra parameters are as follows:

3  ->  Disable 32-bit Instruction Usage.

)  ->  Disable DPMI Support.

(  ->  Use "Slow" MemCopy.

+  ->  Disable EMS Usage.

-  ->  Disable XMS/UMB Usage.

~  ->  Disable Network Detection.

Each of these parameters is explained in the troubleshooting section of

the PKZIP documentation.  To use any of these options, place a check mark

beside the corresponding entry on the Compression Options screen.

The following compression options are available for people using ARJ.EXE

to compress / decompress programs.  The default ARJ compression setting is

to use the -m1 switch.

ARJ -m0 switch            ARJ -m1 -jm switches

ARJ -m1 switch            ARJ -m2 -jm switches

ARJ -m2 switch            ARJ -m1 -jm1 switches

ARJ -m3 switch            ARJ -m2 -jm1 switches.

ARJ -m4 switch

Page 13

These figures were generated with ARJ 2.30.

Switches      No Compression       ARJ Results   Compression

ARJ -m0          2,980,853           2,983,500        0%

ARJ -m1          2,980,853           1,406,625       53%

ARJ -m2          2,980,853           1,414,245       53%

ARJ -m3          2,980,853           1,444,400       52%

ARJ -m4          2,980,853           1,593,074       47%

ARJ -m1 -jm      2,980,853           1,403,524       53%

ARJ -m2 -jm      2,980,853           1,403,524       53%

ARJ -m1 -jm1     2,980,853           1,404,949       53%

ARJ -m2 -jm1     2,980,853           1,404,949       53%

Test Temporary Archive

In addition to these options, ARJ allows users to test the temporary

archive that is created when compressing files.  To do this, place a

check mark in the "Test Temporary Archive" check box.

Selective (Extraction / Exclusion)

Use Selective Extraction (PKUNZIP Only)

When this box is checked, Selective Extraction is turned on, and only

the files listed in the Selective Extraction File List will be extracted

from the .ZIP file when this program is decompressed.  The original .ZIP

file is preserved (similar to the process used when Update Option is

enabled).  After the program is exited, the original .ZIP file is

updated with any changes that were made during the current session.

When Selective Extraction is enabled, Query Option is ignored, and the

Update Option is always used.  All multiple-user access features are

disabled when using Selective Extraction.  Only one user is allowed into

a program at a time when Selective Extraction is enabled.

Selective Extraction File List

This list box contains a list of the files that should be extracted from

the .ZIP archive when the program is decompressed.  Rather than

extracting an entire archive, you can choose to decompress only the

files that are needed for program execution.  This can dramatically

speed up the compression and decompression process.  For example, to run

the Norton Utilities SFORMAT.EXE, the only files needed are SFORMAT.EXE

and NORTON.INI.  By entering these two files, PKUNZIP will only extract

the two files specified before executing SFORMAT.EXE.

When entering filenames into this file list, wild cards may be used,

such as *.EXE, *.DOC, ???.CFG, etc.  If a filename is entered that does

not exist in the .ZIP file, PKUNZIP.EXE will not attempt to extract it,

and it will not issue an error message.

Be very careful when using this option!  If you forget to extract a file

necessary for proper program execution, your program could terminate

unexpectedly or issue error messages.

Use Selective Exclusion (PKUNZIP Only)

When this box is checked, Selective Exclusion is turned on, and all

files listed in the Selective Extraction File List will be excluded from

the extraction process when the .ZIP file is decompressed.  When

Selective Exclusion is enabled, Query Option is ignored, and the Update

Option is always used.  All multiple-user access features are disabled

when using Selective Exclusion.  Only one user is allowed into a program

Page 14

at a time when Selective Exclusion is enabled.

Selective Exclusion File List

This list box contains a list of the files that should be excluded from

the extraction process when the .ZIP archive is decompressed.  Rather

than extracting an entire archive, you can choose to exclude certain

files that are not needed for program execution or operation.  This can

dramatically speed up the compression and decompression process.  For

example, most programs do not require *.DOC files to be present in order

to run successfully.  By entering "*.DOC" into the Selective Exclusion

File List, PKUNZIP will not extract these files, saving time during the

compression and decompression processes, and disk space while the

program is running.

When entering filenames into this file list, wild cards may be used,

such as *.EXE, *.DOC, ???.CFG, etc.

Be very careful when using this option!  Excluding a file that is

necessary for proper program execution could cause your program to

terminate unexpectedly or issue error messages.  Also remember that if

Selective Extraction AND Selective Exclusion are both enabled, Selective

Exclusion files take precedence.  That means that if PROGRAM.EXE is

listed in both file lists, it will always be excluded.

Floppy Options

This screen contains allows users to select options to be invoked when

creating a compressed file on a floppy diskette.

Important Note: Options for floppy diskette compression will only work

when PKZIP / PKUNZIP is used as the compression program.  Also note that

the Update option can NOT be used when compression is occurring to floppy

diskettes.

One of the following options can be selected as the default when creating

a compressed file on a floppy diskette.

Erase All Files and Sub-Directories

Format Destination Diskette (Quick Format)

Format Destination Diskette (Full Format)

Format Low-Density Disk in High-Density Drive (Quick Format)

Format Low-Density Disk in High-Density Drive (Full Format)

For an explanation of these options, see the Backup Guide supplied with

PKZIP 2.0.

Use DOS Verify

If you want DOS Verify to be turned on when compressing files to a

floppy diskette, place a check mark in the "Use DOS Verify" check box.

This adds an extra level of security to make sure that files are created

error-free.

Browse  (Browse for Executable File)

One powerful feature of the ZIF Editor Pro is the ability to browse for

files using file and directory list boxes.  Selecting the "Browse" button

will display a file browsing screen to allow you to search for the

executable file for the designated program.

Users can select any combination of file types from the File Filter list.

Page 15

When a box is checked, the pattern of files associated with that file type

will be displayed.  For example, if the only file type checked is "*.COM",

only files with an extension of "*.COM" will be displayed.  If "*.BAT" and

"*.EXE" are checked, files with either extension ("*.BAT" and "*.EXE")

will be displayed.  Once a file is selected from this screen, the drive,

directory, and executable file will be automatically pasted into the

"Executable File" field.

IN/OUT  (IN/OUT DOS Commands)

IN and OUT Commands are DOS commands that can be executed prior to running

a selected program, or after exiting a selected program.  An example of

this would be to swap a printer port prior to executing a particular

program, and swap it back to the default after exiting the same program.

Another example would be to flush and disable write caching before running

a disk utility program, and re-enabling it after the program is

terminated.

To use this feature, enter any necessary DOS commands that should be

executed when going "IN" or coming "OUT" of the program in the

corresponding text boxes.

Important!  Do NOT attempt to invoke or remove TSR (Terminate and Stay-

Resident) programs from memory with IN and OUT DOS commands.  This is

virtually guaranteed to cause conflicts with ZIP'R Pro.  If you need to

invoke a TSR before running a program, or remove one after exiting a

program, see the following section on TSR's.

TSR's

Just like IN and OUT Commands, TSR Commands are DOS commands that invoke

a TSR before executing a program, or remove it from memory after the

program is exited.  TSR's can be a bit tricky to handle due to the fact

that ZIP'R Professional can not tell what memory has been used by any

TSR's that load later.  But ZIP'R Professional makes TSR's easier to

manage by creating a "TSR Container File".  This file is simply a

temporary batch file that contains commands that surround the command line

used to execute the program with the TSR (Pre) and TSR (Post) commands.

When the program is executed, this batch file is executed instead of the

program, allowing TSR's to load and unload while ZIP'R Professional is

dormant.  No user intervention is required.  ZIP'R Professional

automatically creates, and then deletes this temporary batch file.  TSR's

that can be loaded and unloaded via command line parameters can usually be

invoked successfully with this method.  The key point to remember is to

unload TSR's in the REVERSE order that they were loaded.  The following

example illustrates this concept more clearly.

TSR (Pre) Commands :  TSR1 (Load)

TSR2 (Load)

TSR (Post) Commands:  TSR2 (Unload)

TSR1 (Unload)

TSR2 must be unloaded first, since it was the last TSR loaded, and is

loaded last in memory.  Once TSR2 is unloaded, TSR1 may be safely

unloaded, because it is now the last TSR in memory.

The important point to remember is that if you load a TSR before executing

the program, you MUST unload it after exiting the program.  Otherwise,

control will not be returned to ZIP'R Pro.  When this happens, ZIP'R Pro

Page 16

will not perform the compression process, resulting in several problems.

If the Update option was enabled, the compressed file will still be in the

compressed file directory, and the program files will still be in the

program directory.  This could be dangerous, if you don't know what you're

doing.  It is recommended that you delete the compressed file before

attempting to execute the program again.  You may also find some files in

the \ZIPRPRO sub-directory with a .U00 or .P00 extension.  If no one else

is accessing the program at the time, you should delete them.  To avoid

such problems as these, either don't load TSR's at all, or make sure that

you unload them using the TSR (Post) Commands.

������������������������������������������

�       Opening and Deleting Files        �

������������������������������������������

Also available from the File Menu is the ability to open or delete a

previously created .ZPD file.  Use the Open File screen to search through

available drives and directories for the .ZPD file you want to edit.  Once

you've located the file you want to open, double-click on it with the mouse,

or press [ENTER] when it is selected with the highlight bar, and the ZIF

Editor Pro will read the information and import it into the editing screen to

be viewed or modified.

If you choose "Delete" from the File Menu, a similar dialog box will appear,

allowing you to search through available drives and directories for the .ZPD

file you want to delete.  When you find it, double-clicking on it, or pressing

[ENTER] when it is selected with the highlight bar will display a confirmation

dialog box to confirm that you want to delete the selected file.

������������������������������������������

�       ZIP'R Pro Default Settings        �

������������������������������������������

From the Settings Menu, users can have access to the registration and default

settings for ZIP'R Pro.

Advanced Options

Network/Multitasker Execution  (Yes, No, Auto)

For users who intend on using ZIP'R Pro under a multitasker or in a

network environment, ZIP'R Pro has automatic network/multitasker

detection.  (For a list of current networks/multitaskers supported, see

Question #2 at the end of this documentation.)  When this option is set to

Yes, ZIP'R Pro always assumes that the advanced network/multitasker

features should be used.  If this option is set to No, ZIP'R Pro will

never use the advanced network multitasker features.  The default setting

is Auto, which instructs ZIP'R Pro to automatically detect whether or not

execution is occurring in a network environment or under a multitasker.

Use Atomic Lock Files  (Yes, No)

When the advanced network/multitasker features of ZIP'R Pro are being

used, users can specify whether or not atomic lock files are used.  For a

discussion on atomic lock files, see the "Networks / Multitaskers" section

of this documentation.

Page 17

Sorted Extraction (PKUNZIP Only) (Yes, No)

When Sorted Extraction is set to "Yes", ZIP'R Pro will extract files from

the .ZIP archive in alphabetical order.  This feature is especially useful

on network drives that generally prevent utilities of this nature from

working.  If this option is set to "No", files are extracted in the order

in which they appear in the file.

Windowed Compression (Yes, No)

When Windowed Compression is set to "Yes", ZIP'R Pro will attempt to

execute the program compression process within the limits of a window.

Otherwise, program compression will occur on a full DOS screen.

Windowed Decompression (Yes, No)

When Windowed Decompression is set to "Yes", ZIP'R Pro will attempt to

execute the program decompression process within the limits of a window.

Otherwise, program decompression will occur on a full DOS screen.

Accurate File Size Calculation (Level 1, Level 2, Level 3, Level 4)

Before ZIP'R Pro is able to decompress a program, it must calculate the

amount of disk space that will be required to hold the decompressed

program.  There are 4 levels of accuracy/speed available.  The more

accuracy that is desired, the slower the calculation process becomes.

Level 1

If Level 1 is selected, ZIP'R Pro "guesses" at the amount of space

required to hold the decompressed program files.

Level 2

If Level 2 is selected, ZIP'R Pro performs a more accurate calculation

for the space needed to successfully store the decompressed files.  This

level is recommended for most users, since the calculation used is

fairly accurate, and usually errs on the conservative side.

Level 3

Level 3 uses the same calculation as level 2, but with one difference in

the video display.  Level 2 views the contents and total size of the

archived file by redirecting the output to the screen.  Level 3

redirects this output to a file, which takes longer.  If you use Level

3, you won't see the archive file contents "flying-by" on the screen,

but it will take a bit longer to process it.

Level 4

This is the most complicated (and most time-consuming) method of

calculating the space needed for the decompressed files.  But, it is

EXACT.  ZIP'R Pro reads the size of each file from the archive file

contents, and uses the Cluster Size to determine exactly how much space

a file will consume once it is extracted.  This method takes a longer

time to complete, but there should be NO DOUBT about whether enough

space exists on the hard drive to complete the decompression process

successfully.

Cluster Size (Auto Detect, Manual Entry)

When ZIP'R Pro uses the Accurate File Size Calculation, described above,

it needs to know the size of the allocation unit used on the program's

disk drive.  By selecting "Auto Detect", you can instruct ZIP'R Pro to

automatically detect the true cluster size.  If you select "Manual Entry",

you can enter the cluster size you want ZIP'R Pro to use when making these

calculations.  When in doubt, use Auto Detect.

Page 18

Compression / Decompression Swapping (Automatic, Always, Never)

This option allows you to specify whether or not ZIP'R Pro swaps to EMS or

disk when it attempts to compress or decompress a program.  There are

three options available for this field:

Automatic Swap

This instructs ZIP'R Pro to swap itself to EMS or disk ONLY if there is

not enough memory available, as specified by the corresponding Trip

Point.

Always Swap

This option instructs ZIP'R Pro to always swap itself to EMS or disk, no

matter how much memory is available.  This is the safest method for

executing programs and/or DOS commands, although it can consume more

time.

Never Swap

This instructs ZIP'R Pro that it should never swap to EMS or disk, no

matter how much (or how little) memory is actually available.  Use this

option carefully, as it could cause programs and DOS commands to issue

out of memory error messages.

Since swapping generally slows a program down, it is best to let ZIP'R Pro

decide when to swap by using the "Automatic Swap" feature.

Trip Point:

This option allows users to specify the trip point (or memory availability

requirement) that should be used for ZIP'R Pro to decide when it should

swap itself to EMS or disk.  The trip point only works when "Automatic

Swap" is selected as the swapping type.

Compression Paths

In order for ZIP'R Pro to be able to compress and decompress programs, it

needs to know the path to the compression program(s) that you are using.  If

you want to use ARJ, LHA, or PKZIP/PKUNZIP, enter the appropriate path in

the ARJ Path, LHA Path, or ZIP Path field on this screen.  If ZIP'R Pro

cannot find the selected compression program in the directory specified on

this screen, it will search the ZIP'R Pro sub-directory.  If the compression

/ decompression program cannot be found there, it issues an error message.

Each of the compression paths must be in the format "C:\XXX\", as shown in

the following examples.

C:\ARJ\

C:\PKZIP\

C:\ARCHIVE\LHA\

Register

Registration Name

If you are a registered user of ZIP'R Pro, enter your registration name in

the Registration Name field.  The Registration Name cannot be longer than

36 characters.  If you are not a registered user, please read the

"Registration Terms" section below.

Registration Number

If you are a registered user of ZIP'R Pro, enter your registration number

Page 19

in the Registration Number field.  If you are not a registered user,

please read the "Registration Terms" section below.

Network Users

When you register ZIP'R Pro for a network license, you will receive a

registration code that supports a specific number of network users.  In

order for your registration code to work properly, you must enter the

exact number of stations your network license is registered for.  If you

are not using ZIP'R Pro on a network, simply leave this field blank.

Registration Terms

ZIP'R Pro For DOS is shareware.  You may try ZIP'R Pro for a period of two

weeks.  If you continue using ZIP'R Pro after the initial two week period,

you must register it.  Standard registration is only $49.95 (+ $5.00 S&H).

Registered users will receive the newest version of ZIP'R Pro on diskette,

along with a printed manual, and a registration number.  To register, call

(218) 924-2050.  We accept Visa, MasterCard, American Express, and

Discover, as well as personal checks and money orders.  (Checks / money

orders must be in U.S. Funds drawn on a U.S.  Bank!)  If you wish to

register by mail, print out the order form entitled REGISTER.DOC, and send

it to:

Stevenson Technical Services, Inc.

108 Second Avenue West

Bertha MN 56437-0297

TAME Options

Use Custom TAME Batch File

If this option is selected, ZIP'R Pro allows you to create a customized

ZPROTAME.BAT batch file that can be used under Windows, OS/2, or DESQview

to invoke TAME prior to running a program.

TAME Path

This field should contain the drive and path where TAME is located on your

hard drive.  This path you enter here will be hard-coded into the

ZPROTAME.BAT batch file.

Create Custom ZPROTAME.BAT Batch File

When this button is pressed, ZIP'R Pro creates a ZPROTAME.BAT batch file

in the ZIP'R Pro subdirectory.  This can be used to execute programs under

Windows, OS/2, and DESQview.

Video Options

Scan Activity Indicator (File Count, Twirling Bars, None)

The scan activity indicator is a small display that occurs on the top of

the compression screen when ZIP'R Pro is scanning for added or deleted

files.  If File Count is selected, ZIP'R Pro will display the number of

the current file that it is checking as it proceeds through the file list.

If Twirling Bars is selected, a pair of twirling bars will spin, letting

you know that there is continued activity on the part of ZIP'R Pro.

Select None if you do not want either of these two options displayed.

(Note: This process has been speeded up significantly from the original

ZIP'R, and in most cases, you probably won't notice this indicator.  It is

most useful on extremely large archives with many files.)

Page 20

ZIP'R Pro Delays

Non-registered copies of ZIP'R Pro will ignore any changes on this screen

and use the default settings.  Registering ZIP'R Pro with the author will

allow you to completely remove the prelog pause and delay.

Pause during Prelog

Place a check mark in this box if you want ZIP'R Pro to pause with a

"Press Any Key" message when the prelog screen is displayed.

Delay during Prelog

Place a check mark in this box if you want ZIP'R Pro to delay when the

prelog screen is displayed.  Enter the amount of the delay in the Prelog

Delay field.

Prelog Delay

Enter the number of seconds ZIP'R Pro should pause on the prelog screen

before ZIP'R Pro attempts to execute the selected program.  This field is

only valid if the "Delay during Prelog" box has a check mark in it.

Experienced ZIP'R Pro users can set this field to 0, which will prevent

ZIP'R Pro from displaying the prelog screen.  Novice users may want to set

this field to 4, to allow a short time to view the information displayed

on the prelog screen.

������������������������������������������

�             Using ZIP'R Pro             �

������������������������������������������

Once ZIP'R Pro system preferences have been set up in the ZIF Editor Pro, and

.ZPD files have been created for each program, you are ready to use ZIP'R Pro.

Log to the ZIP'R Pro program sub-directory, type "ZIPRPRO", append a space,

and add the name of the .ZPD file to be referenced.  The command line

structure should look similar to the following examples:

ZIPRPRO PCSHELL

ZIPRPRO PW

ZIPRPRO PSDELUXE

ZIPRPRO LOTUS123

ZIPRPRO THEDRAW

������������������������������������������

� Using ZIP'R Pro With A Menuing System:  �

������������������������������������������

To use ZIP'R Pro with a menuing system, you must modify the location and

program name specified for each program that will be run through ZIP'R Pro.

QuikMenu Example:

OLD     Location:  C:\PFS

Command:   PW

NEW     Location:  C:\ZIPRPRO

Command:   ZIPRPRO PW

Page 21

AutoMenu Example:

OLD     C:

CD\PFS

PW

CD\

AUTO

NEW     C:

CD\ZIPRPRO

ZIPRPRO PW

CD\

AUTO

All programs that are executed through ZIP'R Pro must have C:\ZIPRPRO (or the

actual location of the ZIP'R Pro files) specified as the path.  The location

of the program to be executed is specified in the .ZPD file that was created

with the ZIF Editor.  ZIP'R Pro automatically changes to the proper directory

before executing the desired program.

������������������������������������������

� What Happens When I Execute ZIP'R Pro?  �

������������������������������������������

Several things take place through the course of using ZIP'R Pro.  Here is a

brief summary of what ZIP'R Pro does, and what you can expect to see on the

screen:

When using the following variable on the ZIF Editor's .ZPD editing screen:

ZIF Filename....:  C:\ZIPRPRO\PCSHELL.ZPD

Executable File.:  C:\PCTOOLS\PCSHELL.EXE

Parameters......:

Maximum Users...:  1

Compressed File.:  C:\PCTOOLS\PCTOOLS

Compression Type:  ZIP

and entering the command line:    ZIPRPRO PCSHELL

the following events would take place:

1.  ZIP'R Pro checks for the existence of the .ZPD file.  If the .ZPD file

cannot be found, ZIP'R Pro issues an error message.  If ZIP'R Pro was able

to locate the .ZPD file, ZIP'R Pro proceeds to step #2.

2.  ZIP'R Pro displays the prelog screen.  The unregistered version of ZIP'R

Pro will pause for 10 seconds while this screen displays.  Registered

users may reduce the delay period, or remove it entirely by using the

ZIP'R Pro Defaults screen within the ZIF Editor.  (For more information,

see "ZIP'R Pro Default Settings".)

This screen contains a wealth of information for users with different

system configurations.  The following is a summary of this information,

Page 22

and what it means.

Conventional

Displays the amount of available conventional memory.

EMS

Displays the amount of available EMS memory.

Virtual

Displays the amount of free disk space on the drive where ZIP'R Pro is

being executed from.

Environment

Displays the amount of available environment space.

Free Space

Displays the amount of free disk space on the drive where the program is

being executed from.

Cluster Size

Detects the Cluster Size being used.  If Auto Detect was selected in the

ZIF Editor, ZIP'R Pro will display the size of the allocation unit that

it detects on the target drive.  If Manual Entry was selected in the ZIF

Editor, ZIP'R Pro will display the size that was manually entered.

Query

Displays a check mark if Query mode is enabled in the specified .ZPD

file.

Update

Displays a check mark if Update mode is enabled in the specified .ZPD

file.

Sorted

Displays a check mark if Sorted Extraction is being used.  (Applies only

for PKZIP / PKUNZIP.)

AFSC

Displays the Accurate File Size Calculation level selected for ZIP'R

Pro.

Integrity

Displays whether or not Test Integrity is enabled in the specified .ZPD

file.

Net/Multi

Displays whether or not ZIP'R Pro is using the advanced Network /

Multitasker features.  If "Auto" was selected in the ZIF Editor, this

option displays whether or not it detected an environment such as

Windows, DESQview, OS/2, Novell, etc.  If "Yes" was selected, this

option will always display "YES", no matter what environment may be

present, and if "No" was selected, this option will always display "NO".

Max Users

Displays the maximum number of concurrent users allowed for this

program, as set up in the specified .ZPD file.

INCommands

Displays the total number of "IN" DOS Commands read from the specified

.ZPD file.

OUTCommands

Page 23

Displays the total number of "OUT" DOS Commands read from the specified

.ZPD file.

Program Path/Name

Displays the program path and executable filename as set up in the

specified .ZPD file.

Parameters

Displays any parameters that will be passed to the program being

executed, as set up in the specified .ZPD file.

Compressed Filename

Displays the path and filename of the compressed file that will be

created when ZIP'R Pro compresses the program.  The extension that

appears on this field should reflect the name of the compression program

being used.  (PKZIP will use .ZIP, ARJ will use .ARJ, LHA will use .LZH,

and NONE will not display an extension.)

3.  After displaying the prelog screen, ZIP'R Pro checks the validity of the

drive and path that was entered in the .ZPD file.  If ZIP'R Pro cannot

find the drive and/or path, it issues an error message.  If ZIP'R Pro is

able to locate the drive and path specified, it then checks for the

existence of a compressed file, bearing the name entered in the "ZipName"

field of the .ZPD editing screen.  One of the many benefits of ZIP'R Pro

is that it allows the ability to change compression programs at any time.

ZIP'R Pro checks for the existence of any one of three different

compressed files.  In this case, ZIP'R Pro checks for PCTOOLS.ARJ,

PCTOOLS.LHA, and PCTOOLS.ZIP.  The compressed file that is found first by

ZIP'R Pro is decompressed using the corresponding decompression utility

(step #4).  If ZIP'R Pro does not find a compressed file, it skips the

decompression process and begins to execute the program (step #5).

4.  ZIP'R Pro begins the decompression process by viewing the compressed file

and calculating the disk space required to decompress the entire file.

The accuracy level of this calculation depends on the setting of the

Accurate File Size Calculation (Level 1 - 4) and the Cluster Size setting.

If ZIP'R Pro determines that there is not enough disk space available to

decompress the entire file, a warning message will be issued.  If ZIP'R

Pro determines that sufficient disk space is available to decompress the

entire file, it executes the necessary decompression program to decompress

the file.

5.  The next step performed by ZIP'R Pro is automatic execution of the

specified program.  This step makes ZIP'R Pro unique from most other

compression / decompression utilities.  Rather than forcing users to

compress and decompress programs separately, ZIP'R Pro does it all

automatically.  If ZIP'R Pro is not able to locate the executable file, it

issues an error message stating the problem.  If ZIP'R Pro is able to

locate the executable file, it executes the program normally, along with

any parameters set up in the .ZPD file.

6.  If Query Mode is enabled, ZIP'R Pro pops up a screen prompting you whether

or not you want to compress the program you just exited.  If you answer

Yes, ZIP'R Pro continues with step #7.  If you answer No, ZIP'R Pro skips

to step #8.

Page 24

7.  Once the program is exited, ZIP'R Pro begins to compress (or-recompress)

the program directory using the compression utility specified in the .ZPD

file.  The amount of time this takes depends on the number and size of the

files being compressed.  If Update Mode is enabled, ZIP'R Pro simply

updates the archive file with any changes that have occurred since the

file was decompressed.  New files are added to the compressed file,

modified files are updated in the compressed file, and deleted files are

removed from the compressed file.

8.  After the compressed file has been successfully created, ZIP'R Pro

displays a delay screen for unregistered users.  To remove this delay,

please read the file REGISTER.DOC to find out how to register ZIP'R Pro.

Once this is complete, ZIP'R Pro returns control of the computer back to a

DOS prompt or the menuing system you are using.

������������������������������������������

�         Networks / Multitaskers         �

������������������������������������������

ZIP'R Pro has advanced features available to deal with program execution,

compression, and decompression under multitaskers and in network environments.

Currently, ZIP'R Pro recognizes Windows, DESQview, OS/2, and Novell operating

systems / environments.  ZIP'R Pro has an additional feature that the original

ZIP'R did not have.  It's called atomic lock files.

When the Network / Multitasker option of ZIP'R Pro is enabled, ZIP'R Pro

creates special token files to make sure that two users do not attempt to

compress or decompress a program at the same time.  Normally, ZIP'R Pro would

check for a token file, and create it only if it did not exist already.

However, it is possible that another user could create the file between the

time that ZIP'R Pro checked for it and the time that it actually created it.

This situation is rare, but could happen, and could be dangerous.  That's

where atomic lock files are useful.  Atomic lock files ensure that a file by

the same name cannot be in existence AS IT IS CREATING IT.

If you are using ZIP'R Pro under a multitasker or in a network environment,

make sure that "Network/Multitasker Execution" is set to "Yes" (or "Auto", for

Auto-Detection), under Advanced Options on the Settings menu of the ZIF

Editor.  Also make sure that "Use Atomic Lock Files" is set to "Yes".  This

will provide the maximum amount of security against potential problems.

������������������������������������������

�       DESQview Tips & Techniques:       �

������������������������������������������

To set up ZIP'R Pro under DESQview, use the following examples:

DESQview .DVP Example:

OLD   Program...: PW

Parameters:

Directory.: C:\PFS

Page 25

NEW   Program...: ZIPRPRO

Parameters: PW

Directory.: C:\ZIPRPRO

If you are a Tame user, use the ZIF Editor to specify the correct path for the

TAME directory on your hard drive, and create a custom ZPROTAME.BAT batch

file.  This file invokes TAME-RES.COM prior to executing ZIP'R Pro.  To run

the previous example program, the .DVP command lines would be changed as shown

below:

DESQview .DVP Example (Invoking Tame):

OLD   Program...: ZIPRPRO

Parameters: PW

Directory.: C:\ZIPRPRO

NEW   Program...: ZPROTAME

Parameters: ZIPRPRO PW

Directory.: C:\ZIPRPRO

Since ZPROTAME.BAT is a batch file containing an EXIT statement, you should

also make the following change to your .DVP file:

Change a Program Advanced Options

OLD     Close on exit (Y,N,blank)......: [Y]

NEW     Close on exit (Y,N,blank)......: [N]

This change allows control to return to the ZPROTAME.BAT file after ZIP'R Pro

completes its operation.

Allocating about 300K of EMS memory to programs executed through ZIP'R Pro

will allow ZIP'R Pro to swap itself to EMS memory, rather than disk.  This

speeds up the swapping process somewhat.  If you are already allocating EMS

memory to the program, increasing the amount by 300K will provide extra room

for the swap file.

The opening ZIP'R Pro screen will indicate how much EMS memory is available to

ZIP'R Pro.  If this figure is less than 300K, increase the amount of EMS

memory allocated.

Since PKZIP 2.0 can also use EMS memory, additional memory should be set aside

as swap space for PKZIP.

Page 26

When using ZIPRPRO (without TAME), you can save about 8K of memory by

specifying the .EXE extension for ZIPRPRO as shown in the following example:

DESQview .DVP Example (Saves about 8K of memory.):

OLD   Program...: ZIPRPRO

Parameters: PW

Directory.: C:\ZIPRPRO

NEW   Program...: ZIPRPRO.EXE

Parameters: PW

Directory.: C:\ZIPRPRO

This saves about 8K of memory by allowing DESQview to execute ZIP'R Pro

directly, rather than loading a secondary command processor.

������������������������������������������

�      Using ZIP'R Pro with Windows:      �

������������������������������������������

ZIP'R Pro is only effective when running DOS programs under Windows.  It will

NOT work with Windows specific programs!

Windows Program Item Properties Example:

OLD   Description:        PFS Pro-Write

Command Line:       PW

Working Directory:  C:\PFS

NEW   Description:        PFS Pro-Write

Command Line:       ZIPRPRO PW

Working Directory:  C:\ZIPRPRO

Windows .PIF Example:

OLD   Program Filename:     PW.COM

Window Title:         PFS Pro-Write

Optional Parameters:

Start-up Directory:   C:\PFS

NEW   Program Filename:     ZIPRPRO.EXE

Window Title:         PFS Pro-Write

Optional Parameters:  PW

Start-up Directory:   C:\ZIPRPRO

TAME may also be used in combination with ZIP'R Pro under Windows.  If you are

a Tame user, use the ZIF Editor to specify the correct path for the TAME

directory on your hard drive, and create a custom ZPROTAME.BAT batch file.

This file invokes TAME-RES.COM prior to executing ZIP'R Pro.  To run the

previous example program, the Windows command lines would be changed as shown

below:

Page 27

Windows Program Item Properties Example (Invoking Tame):

OLD   Description:        PFS Pro-Write

Command Line:       ZIPRPRO PW

Working Directory:  C:\ZIPRPRO

NEW   Description:        PFS Pro-Write

Command Line:       ZPROTAME ZIPRPRO PW

Working Directory:  C:\ZIPRPRO

Windows .PIF Example (Invoking Tame):

OLD   Program Filename:     ZIPRPRO.EXE

Window Title:         PFS Pro-Write

Optional Parameters:  PW

Start-up Directory:   C:\ZIPRPRO

NEW   Program Filename:     ZPROTAME.BAT

Window Title:         PFS Pro-Write

Optional Parameters:  ZIPRPRO PW

Start-up Directory:   C:\ZIPRPRO

������������������������������������������

�       How Effective Is ZIP'R Pro?       �

������������������������������������������

ZIP'R Pro can save a typical computer user a substantial amount of hard drive

space.  Most users have several programs on their hard drives that are seldom

used, but take up a tremendous amount of hard disk real-estate.  Rather than

deleting these programs completely, ZIP'R Pro allows these programs to be

accessed when needed, storing them in a compressed form when they are not in

use.  The amount of hard drive space saved depends totally on the

effectiveness of the compression program used by ZIP'R Pro.  As compression

programs improve, so will the ratios that ZIP'R Pro is able to achieve.

The compression programs used by ZIP'R Pro do not provide "on-the-fly" data

compression.  As a result, there is a time delay during the compression and

decompression processes.  However, ZIP'R Pro does not require 50K of valuable

memory, either.  In addition, ZIP'R Pro does not degrade the performance of

the computer system as the "on-the-fly" data compression programs do.

A performance comparison of the three compression programs currently supported

by ZIP'R Pro is shown in the following table.  The test directory consisted of

33 files, plus a sub-directory which contained an additional 17 files.

Test Directory Size Using Microsoft MS-DOS 5.0

OVERALL

COMPRESSION TYPE           BYTES       COMPRESSION    COMPRESSION

------------------------------------------------------------------

Original Baseline            2,980,853         -              -

MS-DOS                       2,980,853         0%             0%

New Baseline                 2,980,853         -              0%

Additional compression obtained using ZIP'R Pro:

No Compression (NONE)        2,980,853         0%             0%

Page 28

PKZIP 1.1 Compression (ZIP)  1,536,688        48%            48%

PKZIP 2.0 Compression (ZIP)  1,401,026        53%            53%

LHA Compression (LHA)        1,428,393        52%            52%

ARJ Compression (ARJ)        1,406,625        53%            53%

Test Directory Size Using DR DOS 6.0 (with SuperStor)

OVERALL

COMPRESSION TYPE           BYTES       COMPRESSION    COMPRESSION

------------------------------------------------------------------

Original Baseline            2,980,853         -              -

DRDOS w/SuperStor            1,992,704        33%            33%

New Baseline                 1,992,704         -             33%

Additional compression obtained using ZIP'R Pro:

No Compression (NONE)        1,992,704         0%            33%

PKZIP 1.1 Compression (ZIP)  1,551,360        22%            48%

PKZIP 2.0 Compression (ZIP)  1,408,512        29%            53%

LHA Compression (LHA)        1,444,864        27%            52%

ARJ Compression (ARJ)        1,423,872        29%            52%

You can see by this table that SuperStor was only achieving an overall

compression ratio of 33%.  Using ZIP'R Pro with PKZIP 2.0 allowed an extra 20%

savings on hard drive space!

������������������������������������������

�      Cautions When Using ZIP'R Pro      �

������������������������������������������

Do NOT attempt to use ZIP'R Pro to compress any directories containing files

that are referenced in your CONFIG.SYS or AUTOEXEC.BAT.  If you wish to

compress the DOS sub-directory, copy any such files to the root directory of

the hard drive, and update the paths in your CONFIG.SYS and AUTOEXEC.BAT.

Also be cautious when compressing directories that contain memory managers,

caches, and utilities that may be needed to boot your machine.  These

directories may be compressed, but any files needed for booting the machine

must be copied to the root directory, and the CONFIG.SYS and AUTOEXEC.BAT

paths updated.

Keep in mind that in order to properly decompress a program, there must be

enough free space on the hard drive to contain all the extracted files in

addition to the original compressed file.  After the files have been

successfully extracted, the compressed file is deleted.  Likewise, when

compressing a program, sufficient space must be available for the original

program files and the newly created compressed file.  After the compressed

file has been successfully created, the original program files are deleted.

Multitaskers such as DESQview and Windows put additional stress on the disk

I/O systems of the computer.  If you have any doubts about the stability of

your system, use the Test Integrity option in the .ZPD file to force the

compression program to verify the integrity of the archive.  When running on a

network or under a multitasker, make sure that "Network/Multitasker Execution"

is set to "Yes" or "Auto", and "Use Atomic Lock Files" is set to "Yes".

Page 29

When upgrading to PKZIP 2.0, test the reliability of the compression and

decompression processes before trusting it with your data!  PKZIP 2.0 has some

documented problems with fast machines (486/25 and up) using external CPU

memory caches.  Read the troubleshooting section of the PKZIP 2.0

documentation for more information.

������������������������������������������

�   Troubleshooting / Common Problems:    �

������������������������������������������

Problem #1:

The most common problem among ZIP'R Pro users is incorrect setup of the .ZPD

file being used.  The following list gives some hints and tips for filling in

each field when editing a .ZPD file through the ZIF Editor.

Field Name         Hints / Tips

ZIP'R Pro Filename

Make sure that this field contains only characters that can be used as part

of a filename.  To find out which characters may or may not be used as part

of a DOS filename, see your DOS reference manual.

Executable File

You must place the correct extension of the program to be executed on the

filename.  If ZIP'R Pro issues an error message that it contain find the

program, make sure you have check the extension on this field.

Parameters

Since some programs require case sensitive passed parameters, the ZIF Editor

does not automatically convert this field to UPPER CASE.  Enter any

parameters exactly as you would enter them after the program name, if you

were using a DOS command line.

Compressed Filename

Do not place an extension for the compressed file in this field.  ZIP'R Pro

automatically adds the correct extension at run time.  If the compression

program is PKZIP, the extension will be .ZIP.  If the compression program in

ARJ, the extension will be .ARJ, and likewise, if the compression program is

LHA, the extension will be .LZH.

Maximum Users

Make sure that the maximum number of concurrent users does not exceed 256.

ZIP'R Pro will only allow 256 users in one program at the same time.

Compression Type

If you want to decompress a previously compressed program without

recompressing it, use the "NONE" option.

Test Integrity

Check this box ONLY if you want the compression program to perform an

integrity check on the archived files once they have been compressed.

Query Option

Check this field when you want ZIP'R Pro to prompt you before compressing a

program.  This allows you to specify whether a program should be compressed

"on-the-fly".

Update Option

Page 30

This option is only available for use when ZIP (PKZIP / PKUNZIP) is selected

as the Compression Type.  If any other option is selected, the Update Option

field will be grayed and disabled.

Problem #2:

If you receive the error message, "Unable to run COMMAND /C ...", you are

probably running DR DOS, and do not have COMMAND.COM available on your path.

For some reason, DR DOS does not copy COMMAND.COM into the \DRDOS sub-

directory at installation time, making it unavailable on the path.  The

solution here is simply to copy COMMAND.COM from the root directory into the

\DRDOS sub-directory.  Another alternative is to add the root directory to the

PATH= statement in the AUTOEXEC.BAT file.

ZIP'R Pro is able to detect if COMMAND.COM cannot be found, and therefore

issues a warning message.

Problem #3:

If ZIP'R Pro won't compress the program's files and sub-directories after it

is exited, check the .ZPD file.  If the fields are all correct, check the

\ZIPRPRO sub-directory for any files with a .U00 or .U01 extension.  These are

token files created by ZIP'R Pro to track concurrent access to a compressed

file.  Normally, these files are deleted when the program is exited.  However,

if the computer hangs, these files will not be deleted, and will prevent ZIP'R

Pro from compressing the program's files and sub-directories.  The filename

should match the name of the .ZPD file specified.  To make ZIP'R Pro operate

correctly, make sure that the program is not being accessed, and delete either

or both of these files from the \ZIPRPRO sub-directory.

In the same way, if you are receiving an error message that there are no more

program numbers available, it either means that the selected program is being

used by the maximum number of users, or there are left-over program token

files in the \ZIPRPRO sub-directory.  These files will have the extension

.P00, .P01, and so on.  If you find these files in the \ZIPRPRO sub-directory,

and you can confirm that no one else is using the program at the time, go

ahead and delete them.

The last set of token files that are created are ZIPRPRO.N00, ZIPRPRO.N01, and

so on.  These files track the number of concurrent users for network versions

of ZIP'R Pro.  ZIP'R Pro creates one of these files for each active ZIP'R Pro

user.  In the event that your \ZIPRPRO sub-directory contains any of these

files while there are NO OTHER USERS ACCESSING PROGRAMS THROUGH ZIP'R PRO, go

ahead and delete them.

The most common reason that any of these types of files would be left behind

is usually due to a computer hang.  When the computer hangs, control is not

returned to ZIP'R Pro, so ZIP'R Pro cannot delete them as part of the cleanup

process.  They can also be left behind due to a TSR not be unloaded in the TSR

(Post) Command process if it was loaded in the TSR (Pre) Command process.  As

mentioned earlier, this can also cause control to not be returned to ZIP'R

Pro, in which case, these files would not be deleted.

Problem #4

If ZIP'R Pro will not recognize your registration name and number and

Page 31

continues to issue an "UNREGISTERED" message on the prelog screen, check the

file ZIPRPRO.INI.  If you manually edited this file without using the ZIF

Editor, it is possible the text editor you were using allowed additional

spaces to be saved at the end of both the registration name and number.  To

solve this problem, use the ZIF Editor to modify these entries.  The ZIF

Editor makes sure that all extra spaces are trimmed before saving these fields

to the .ZPD file.  If you cannot use the ZIF Editor, use a text editor that

allows you to view and trim any extra spaces, or use a text editor that

automatically trims extraneous spaces from the end of a line.

Problem #5

If you are having trouble getting ZIP'R Pro to run properly, there is a

special "TEST" parameter that can aid you in diagnosing your problem.  When

typing the command line to run ZIP'R Pro, instead of typing "ZIPRPRO

FILENAME", you can type "ZIPRPRO FILENAME TEST".  This extra parameter will

allow you to view several screens of parameters.  These parameters contain

information that ZIP'R Pro is reading from the system, the ZIPRPRO.INI file,

and the individual .ZPD file.  This can be used to find out if ZIP'R Pro is

reading the necessary files accurately.  If you detect a problem when using

the TEST parameter, please contact STSI at (218) 924-2050.

Note: The keys (CTRL + Break) can be used on the ZIP'R Pro prelog screen to

exit from ZIP'R Pro.  Although this is not recommended, it will enable

users to stop execution in an emergency situation.

Problem #6

There is one known problem with the ZIF Editor Pro that we are currently

looking into.  Although we cannot explain why, when the ZIF Editor Pro is

launched under Windows and the context-sensitive help is accessed one or more

times, the program will crash with an error message that the database must be

repaired or an index cannot be found.  The file ZIFPRO.HLP is actually a

database of help topics, and should be fine, even after this problem occurs.

The best we can say right now, is that users should try not to access online

help in the ZIF Editor Pro when running under Windows.

������������������������������������������

�          Questions And Answers          �

������������������������������������������

Question #1:    Can ZIP'R Pro leave a program uncompressed after it has been

previously compressed with ZIP'R Pro?

Answer:         Yes!  Simply specify "NONE" as the Compression Type.  When

ZIP'R Pro attempts to execute the program, it will decompress

it as normal, but once it is exited, it will not attempt to

recompress it.

Another alternative for leaving a program uncompressed is to

use the "Query" option.  This allows you to decide if a

program should be compressed "on-the-fly".

Question #2:    What networks / multitaskers does ZIP'R Pro recognize?

Answer:         ZIP'R Pro currently recognizes/supports the following

Page 32

operating systems / environments:

Windows

OS/2

DESQView / DESQview/X

Novell

LANtastic

10Net

������������������������������������������

�        Comments From ZIP'R Users        �

������������������������������������������

The following list shows samples of some of the comments we have received on

the original ZIP'R product:

"Works as advertised. Seamless integration.... I think you have a superb

product."

- Toronto, Ontario

"ZIP'R is going to save you money, and more important hard disk storage

space.  It will save you money AND headaches by not having to purchase a disk

compression program such as Stacker or SuperStor.... Support shareware and

give this program a try.  I know you will use it, NO ONE ever has enough hard

disk space."

- Throughput (The S.W.I.P.C.C. Inc. Journal)

March 1993

"Really neat program."

- Dresden, Tennessee

"There is an excellent interface for setting up each program within ZIP'R...

Once a program has been set up, it's operation within ZIP'R is transparent to

the user.  One advantage to those who can't afford the 40 to 50 K of RAM

required by other compression programs is that ZIP'R only requires 2.5K RAM

when it has finished loading your program.  When you have become familiar with

the program you will wonder how you ever managed without it."

- read.me (The Personal Computer Club of Toronto)

October 1993

"...thanks for a great product."

- Edgewood, New Mexico

Page 33

������������������������������������������

�           Programs Mentioned            �

������������������������������������������

386MAX is copyrighted by Qualitas.

ARJ is copyrighted by Robert K Jung.

AutoMenu is copyrighted by Magee Enterprises.

BLAST FAX PC is copyrighted by U.S. Robotics, Inc.

DESQview is copyrighted by Quarterdeck Office Systems.

DoorWay is copyrighted by Marshall Dudley.

DR DOS is copyrighted by Digital Research Inc.

LANtastic is copyrighted by Artisoft, Inc.

LHA is copyrighted by Haruyasu Yoshizaki.

MS-DOS is copyrighted by Microsoft.

Novell is copyrighted by Novell.

OS/2 is copyrighted by IBM.

PKZIP is copyrighted by PKWARE, Inc.

PKUNZIP is copyrighted by PKWARE, Inc.

QEMM is copyrighted by Quarterdeck Office Systems.

QuikMenu is copyrighted by OSCS Software Development, Inc.

Stacker is copyrighted by Stac Electronics, Inc.

SuperStor is copyrighted by AddStor, Inc.

Tame is copyrighted by PowerSoft, Inc. and David G. Thomas.

Windows is copyrighted by Microsoft.

ZIP'R is copyrighted by Stevenson Technical Services, Inc.

Page 34

Stevenson Technical Services, Inc. (STSI)

108 Second Avenue West

Bertha, MN 56437-0297

Phone: (218) 924-2050

FAX: (218) 924-2050 (Ext. 11)

The Bertha BBS: (218) 924-2060, N,8,1

ZIP'R Pro 1.11 Registration Form

Qty    Description                                   Price Each     Total

_____   ZIP'R Pro 1.11 Single-User Registration           $49.95   ________

Includes Printed Manual, 3 1/2" & 5 1/4" Diskettes

_____   ZIP'R Pro 1.11 Network Registration (Users? ____) _______  ________

(Same price breaks apply for site-licensing.)

��������������������������Ŀ    ��������������������������Ŀ

� # of Users �    Price    �    � # of Users �    Price    �

��������������������������Ĵ    ��������������������������Ĵ

�      5     �   $100.00   �    �     50     �   $400.00   �

�     10     �   $200.00   �    �    100     �   $500.00   �

�     20     �   $300.00   �    �   More?    �    CALL     �

����������������������������    ����������������������������

_____   Additional Copies of Printed Manual               $10.00   ________

Sub-Total:   ________

Minnesota Residents Add 6.5% Sales Tax:   ________

Shipping/Handling (USA/Canada = $5.00, Foreign = $12.50):   ________

Upgrade From ZIP'R (USA/Canada = $20.00, Foreign = $27.50):   ________

(No other charges or fees are required for upgrades.)

Total:   ________

Name: ___________________________________  Title: ______________________

Company: __________________________________________________________________

Address: ______________________________  City: ____________________________

State: ___________  Zip Code: _______________  Phone: ___________________

Please Register ZIP'R Pro To This Name: ___________________________________

Method of Payment: ___ Visa/MC   ___ American Express   ___ Discover

___ Check/Money Order Enclosed

(Checks/MO's must be in U.S. Funds drawn on a U.S. Bank!)

Credit Card #: __________________________________  Expiration Date: ___/___

Name on Card: _______________________  Signature: _________________________

Comments / Suggestions: ___________________________________________________

___________________________________________________________________________

___________________________________________________________________________

Page 35

Please take a minute to answer the following questions.  Your answers

will assist us in improving and marketing ZIP'R Pro.

(Check all that apply.)

Where did you hear about ZIP'R Pro?

[ ] BBS             [ ] Catalog/Magazine    [ ] Friend/Relative

[ ] Rack Vendor     [ ] User's Group        [ ] Other _______________

What operating systems / environments do you use?

[ ] DOS    [ ] Windows   [ ] DESQview/DESQview/X     [ ] OS/2

What do you use ZIP'R Pro for?

[ ] Business Applications    [ ] Games    [ ] Other _________________

What compression program(s) are you using?

[ ] ARJ    [ ] LHA    [ ] ZIP

Would you be interested in seeing ZIP'R Pro work on Windows-specific

programs?

[ ] Yes    [ ] No     [ ] Doesn't Matter

Why are you registering ZIP'R Pro?

[ ]  I'm honest by nature!

[ ]  The prelog and nag delays are so annoying!

[ ]  Other  ______________________________________________________________

What feature(s) do you like the most about ZIP'R Pro?  ___________________

__________________________________________________________________________

__________________________________________________________________________

What feature(s) would you like to see added to ZIP'R Pro?  _______________

__________________________________________________________________________

__________________________________________________________________________

Additional Comments:  ____________________________________________________

__________________________________________________________________________

__________________________________________________________________________

May We Quote You?    [ ] Yes    [ ] No    [ ] Doesn't Matter

Page 36