A LOW COST PLD PROGRAMMER

This programmer can program the GAL16V8A, GAL16V8B, GAL20V8A and

GAL20V8B from Lattice, SGS and National Semiconductor.  This may seem

limited but it is not because the 16V8 directly replaces the 16R6, 16R6,

16R4, 16RP8, 16RP6, 16RP4, 16L8, 16H8, 16P8, 10L8, 12L6, 14L4, 16L2,

10H8, 12H6, 14H4, 16H2, 10P8, 12P6, 14P4 and 16P2 PALs.  The 20V8

directly replaces the 20R6, 20R6, 20R4, 20RP8, 20RP6, 20RP4, 20L8, 20H8,

20P8, 14L8, 16L6, 18L4, 20L2, 14H8, 16H6, 18H4, 20H2, 14P8, 16P6, 18P4

and 20P2 PALs.  GALs can handle logic that can not be handled by the

standard PALs. GALs unlike standard PALs can be reprogrammed in seconds

making them idea for the developer. You can build this programmer for

only $150.00 far below the $500 - $10,000 that a universal programmer

typically costs.

DESIGN AND THEORY OF OPERATION

The programmer is made up of two separate modules, an interface board

that resides in a slot in your PC and a main programmer module external

to the PC.  They are connected by a 50 pin ribbon cable.  The same

interface board is also used with a lost cost logic analyzer.  By

keeping things modular you only need to use one PC slot and can connect

either the PLD unit or logic analyzer unit to the interface card as

needed.

The interface board is a very standard two chip parallel interface.  The

8255 has three 8 bit parallel ports that can be configured by software.

This provides for a flexible interface.  In normal operation of the

programmer port A and port B will be configure to be output ports and

port C is configured with bit 0 to 3 as outputs and bits 4 to 7 as

inputs.  The second chip is a 74LS682 8 bit comparator.  It is used as

an address decoder for the 8255.  The 74LS682 asserts pin 19 low when

it's P inputs are the same as it's Q inputs.  The eight Q inputs have an

internal 10 K pull up resistor so they will stay high unless forced low.

The 8255 address is selected with the DIP switch, each switch that is

turned on will pull the corresponding Q input low.  When the address on

the PC bus (the P inputs) matches the address selected by the DIP switch

(Q inputs) a chip select is generated for the 8255.  If it is an I/O

cycle then either IOR* or IOW* will also be asserted and data will be

read from or written to the 8255 register selected by A0 and A1.  The

8255 base address is specified to the software in the configuration

file.

50 pin connector P1 is the bridge to the main programmer board.  Each of

the three 8 bit ports from the 8255 are connected to P1.  Of the

remaining 26 lines, 12 are connected to PC ground, 12 to PC +5 volts

supply, 1 to the PC +12 volts and one to -12 volts.  The +/- 12 volt

supplies are not used by the programmer but are available for other

projects.

GALs are TTL level devices, during programming however, they need a

super voltage of either 12.0, 14.5 or 16.5 volts on pin 2 of the GAL.

U2 in combination with C1, C2, D1 and D2 form a voltage doubler circuit

that takes the 12 Volts from the interface and converts it into about

22.6 Volts (12 Volts doubled to 24 Volts minus the two diode voltage

drops).  The 22.6 Volt output is feed into adjustable voltage regulator

U1.  The output voltage of U1 is control by the resistance between pin1

and ground.  The maximum output voltage of U1 is adjusted with R8 to be

16.5 Volts.  The output voltage can be lowered one step by bring signal

VSEL0 high, this turns T2 on and places resistors R6 and R5 in parallel

with R8 and R9, this lowers the resistance between U1 pin1 and ground,

lowering the output voltage.  R5 is adjusted so that the output voltage

is 14.5 Volts.  An additional output voltage is obtained by setting

VSEL1 high to turn on T1.  R4 is adjusted to produce an output of 12.0

Volts.

The super voltage can be enabled to either pin 2 or pin 4 of the ZIF

socket.  (When a 20 pin 16V8 is placed in the socket is is positioned in

the lower portion of the socket so that pin 2 of the chip actually sits

in pin 4 of the socket.) Transistor pairs T3 and T4 form a switch that

enables the super voltage to appear on pin 2 of the ZIF when the signal

into the base of of T3 is high. T5 and T6 form a similiar switch for pin

4 of the ZIF.

The +5 Volt power to the chip being programmed in the ZIF is switched by

the signals VCC22* and VCC24* from U3. U3 also turns on the LED when

either VCC22* or VCC24* are active.

Data is read in from the chip being programmed through either PS14

(16V8) or PS15 (20V8). The GAL output is open drain during programming

so it is pulled up through either R10 or R11 as appropriate. The input

signal appears at the interface on PC7.

The remaining pins are driven by U4 and U5.  These are 8 bit addressable

latches.  The bit address is placed on A0, A1 and A2 (pins 5,6 and 7).

The data to be latched is put on pin 3.  When pin 4 is high the data

from pin 3 is placed on the addressed output pin (if a0=0, a1=0 and a2=0

and data=1 then q0=1).  When pin 4 is brought back low the data is

latched.

BUILDING THE PROGRAMMER

Before building the programmer get a copy of the software from one

of the sources in the parts list.  You will need the software for

testing the programmer.

The interface board is best built on a purchased PC board (see parts

list for details).  If you elect to hand wire on a PC/XT prototype board

use a heavy gauge wire for the +5 volt and ground connections from the

PC bus to P1.

The programmer board is best built on a purchased PC board (see parts

list for details). If you wish to mount the PCB in a case make sure to

solder the LED so that it is higher then the ZIF socket, that way it can

be seen through a hole in the case. You can use sockets for all the ICs

if desired. No other special procedures are needed.

The interface board should be tested before hooking up the programmer

board, it will be easier to locate and fix interface problems isolated

from the main board.  Before plugging the interface into the PC

carefully check for shorts and other errors.  Set the I/O address using

the DIP switch.  I've found that 390H is usually free, this corresponds

to A9-OFF, A8-OFF, A7-OFF, A6-ON, A5-ON, A4-OFF, A3-ON (remember ON

corresponds to a low because it grounds the input).  Once set plug in

the board and power up the PC, if it does not boot up normally, power

down and recheck everything.  The configuration file RGBPLD.CFG should

be edited to match the logic programmer.  RGBLOGIC.CFG is a two line

ASCII text file, the first line contains the base I/O address in hex

(i.e.  390), the second line should always have 1.  Any characters after

the numeric value on each line are ignored.  Edit the file to match your

configuration.  Check the voltages at the end of the 50 conductor ribbon

cable for proper power and ground supplys.  Start up the interface test

program RGBINT.  The interface test allows you to read and write the

8255 registers.  Write an 80 (all values are expressed in hex) to the

CNTRL register, this will program ports A, B and C as output ports.

Check each bit of the three ports at the end of the cable.  You can

write different values to the ports from the diagnostic program.  If the

interface doesn't respond at all make sure the configuration file and

DIP switch settings match, if they do then check the interface board for

errors.  If everything is working, power down.

Double check the programmer board for shorts or other errors.  Connect the

50 conductor ribbon cable to the programmer board and power up the PC.  If

the PC does not boot up normally the power supply is probably shorted -

power down and correct the problem. If everything powers up fine, then run the

program RGBSETUP. Run through each of the tests in the menu. F1 will

allow you the adjust the super voltages. If the the voltages are not

present check the section of the board with U1, U2, T1, T2, T3, T4, T5

and T6. F2 and F3 allow you to check the power to the ZIF socket. F4

checks the pullup resistors and the ability to read data from ZIF pins

14 and 15. F5 will allow you to check the functioning of each active pin

on the ZIF socket - be sure all pins can be set high or low.

To use the programmer start the program RGBPLD.  Before placing a chip

in the ZIF socket select the device type using F1 (not having the

correcting device selected could burn out the chip).  The remaining menu

selections should be self explainatory - they allow you to read/write

JEDEC files and load, program and verify GAL devices.  The User

Electronic Signature (UES) for both the main and verify buffer is

displayed in ASCII format.

WRAPPING UP

That covers the details of a low cost logic programmer.  National

Semiconductor offers a free Logic Compiler called OPAL Jr. Get a copy

and you can start to design your circuits with programmable logic.

RESOURCES

National Semiconductor (OPAL Jr.) - (408) 721-2092