DSwitch              Copyright 1992 Ryan Brooks

-----------------------------------------------

NAME

DSwitch version 1.04 Gamma - first public release

SYNOPSIS

A tool to select DCTV video and pass-thru modes.

FUNCTION

Click on the Install icon to copy DSwitch into your C:

directory.

'DSwitch <mode>' will run the program and invoke the

appropriate mode.  If no parameters are given or an improper

selection is made, a help template will be shown.  DSwitch

will return control after switching.  DCTV *must* be connected

to the parallel port for the command to function correctly.

This can be used from Shell, incorporated into scripts or called

by other software that can issue AmigaDOS commands.

INPUTS

The options for DSwitch are as follows:

(only the first letter, case-insensitive, is required, all

modes are mutually exclusive)

Normal   Restores normal behavior in the DCTV unit.

Video    Makes DCTV always pass whatever is connected to

the 'Video Input' straight through to 'Video

Output'.  This overrides the presence of a DCTV

picture on the current screen.

Rgbi     Turns on an "unusual" DCTV mode, where it appears

DCTV encodes pseudo-RGBI Amiga video into a

composite output.  Normal DCTV picture encoding is

still active.  This also may be a different DCTV

mode.  For experimental purposes only.

Bw       With this option, all encoded DCTV pictures appear

in black and white.  For experimental purposes

only.

Strobe   After enabling this option, areas of DCTV pictures

with high chrominance values will appear to

"flash" at about 10 Hz.  For experimental purposes

only.

RESULTS

No return codes are generated in any situation.  The

shell regains control immediately after the switching takes

place.

WARNING

This product is in no way affiliated with Digital

Creations or Progressive Image Technology.

Just a reminder: DON'T disconnect or connect DCTV video

or parallel connections while the Amiga is on!

NOTES

This software was developed by experimentation with the

DCTV unit.  I never disassembled one line of code or opened

the case, so my command sequences may only be approximations

of the appropriate protocol.  Because of this, I cannot

guarantee that it will work to your expectations for your

exact application.  However, I have used this command on my

own system and several friends' systems with no problem

whatsoever.

Personally, I use it to switch between the output of my

A520 (color encoder) and DCTV encoded pictures.  That way I

can have both photographic-type pictures/anims and high-res

Amiga pictures in the same presentation.  Just use your

favorite authoring program to execute the DSwitch command to

put DCTV in the appropriate mode.  Also, it's handy for

checking your video input without always changing cables.  I

suppose it could also be used to switch between DCTV and

LaserDisc or VCR during presentations.

SEE ALSO

< Insert favorite Michael Sinz quote here >

Please see DSwitch.readme.

BUGS

Not really a bug, but an "inconvenience that requires

planning", a 'DSwitch v' followed by a 'DSwitch v' will

produce a slight glitch on the DCTV video output.

Also, DSwitch has no way of knowing if DCTV is attached

to your computer, and will happily think it is working.

FUTURE ENHANCEMENTS

Send me comments, bugs, info on your application, etc.:

2575brooksr@vms.csd.mu.edu   	(internet)

or for non-net people:

Ryan Brooks

12886 Bristlecone Lane

Muskego, WI   53150  USA

SOURCE

Available upon request.