1. About TRAP-D

TRAP-D is caused by invalid memory access operations. The following operations

cause TRAP-D.

(1) The segment register points to an invalid segment descriptor.

(2) The offset of the address goes beyond the size of the segment descriptor.

(When the stack pointer goes beyond the size of the stack segment,

TRAP-C(stack overflow) occurs.)

(3) The privilege rules have been violated. (A program accesses the data

which belongs to the different privilege level.)

(4) Code segment is loaded with a value of the data segment descriptor.

(except alias segment)

(5) A program tries to read from an execute only code segment or to write

into a read only data segment.

Generally, most of TRAP-Ds are caused by case (1) or (2).

The following C sample programs illustrate case (1) and (2).

Case (1).

void main(int argc, char **argv, char **envp)

{

/* p has an invalid selector value 0x1234 */

int far *p = (int far *)0x12345678L;

/* TRAP-D will occur on the following code  because it tries */

/* to access an invalid address.                             */

*p = 1;

:

:

}

Case (2).

void main(int argc, char **argv, char **envp)

{

char far *p;

unsigned selector;

/* Allocate a memory whose size is 10 bytes */

DosAllocSeg(10, &selector, 0);

/* Make p point to the top of the memory area allocated by the */

/* above DosAllocSeg                                           */

FP_SEG(p) = selector;

FP_OFF(p) = 0;

p += 100;

/* TRAP-D will occur on the following code because the offset */

/* of p goes beyond the size of the memory area (i.e. 10).     */

*p = 1;

:

:

}

2. About TRAP-D popup panel

When a program causes a protection violation (TRAP-D), you will see the

following popup panel.

+-----------------------------------------------------------------+

|                                                                 |

1 |    Session Title:                                               |

2 |    XXXXXXXX                                                     |

3 |                                                                 |

4 |          SYS1943: A program caused a protection violation.      |

5 |                                                                 |

6 |          TRAP 000D                                              |

7 |          AX=0000 BX=0000 CX=000B DX=0047 BP=0FF0                |

8 |          SI=02FF DI=1349 DS=0067 ES=0000 FLG=2246               |

9 |          CS=0047 IP=0017 SS=0067 SP=0FEC MSW=FFFB               |

10 |          CSLIM=0624 SSLIM=134F DSLIM=134F ESLIM=****            |

11 |          CSACC=FB SSACC=F3 DSACC=F3 ESACC=**                    |

12 |          ERRCD=1234 ERLIM=**** ERACC=**                         |

|                                                                 |

+-----------------------------------------------------------------+

|              End the program                                    |

|                                                                 |

|                                                                 |

+-----------------------------------------------------------------+

Line 2  : Session name to which the TRAP-D program belongs.

Line 4  : Cause of the error. When a program causes a TRAP-C, this line will

be

SYS1942: A program attempted to reference storage outside the

limits of a stack segment.

Line 6  : Kind of traps

Line 7-9: Dump of registers.

Line 10 : Size of the segment that segment register points to.

When a value of the segment register is invalid, this field is

filled with ****.

Line 11 : (80286 architecture) Access rights byte of the segment descriptor.

This access rights byte of the general segment descriptor is as

follows.

bit 0: Access bit (used by OS for memory management)

bit 1: Access type

Code segment     Data segment

0  Execute only     Read only

1  Execute/Read     Read/Write

bit 2: Expansion direction (DATA segment)

0  Downward

1  Upward

Conforming type

0  Non-conforming segment

1  Conforming segment

bit 3: Segment type

0  Non-executable segment (DATA segment)

1  Executable segment (CODE segment)

bit 4: Always 1.

bit 5-6: Descriptor privilege level (from 0 to 3)

bit 7: Present bit (used by OS for memory management)

If A program runs in the descriptor privilege level 3, Access

rights byte of a CODE segment is 0xFB and DATA segment is 0xF3.

When a value of the segment register is invalid, this field is

filled with **.

Line 12 : Value of the segment descriptor which is the cause of the TRAP-D,

its size, and its access rights byte.

In the above sample code of the case (1), ERRCD=1234, ERLIM=****,

and ERACC=** because 0x1234 is an invalid segment descriptor value.

3. How to use FINDSEG to find out TRAP-D module.

To search TRAP-D module, you had better set CSLIM value in TRAP-D panel as

segment length(-l switch) rather than DSLIM, ESLIM or SSLIM because data

segments(DS, ES, SS) might be allocated or reallocated after the process

started.

You had better set IP value(by -I switch). When two or more modules are found,

you can determine TRAP-D module from disassembled code. For example, TRAP-D

may not occur on the following instructions

mov   ax, 0

inc   cx

:

etc.

and the following instructions have potential of TRAP-D (or TRAP-C).

TRAP-D occurs on the following condition.

pop   ax                <--- sp exceeds SSLIM (In this case TRAP-C will

occur).

les   bx, es:�bx+si�    <--- bx+si exceeds ESLIM or ES is invalid

rep   movs              <--- si exceeds DSLIM or di exceeds ESLIM

:

etc.

So I recommend you the following command to find out TRAP-D module from the

TRAP-D popup panel mentioned in section 3.

findseg -c -lx624 -Ix17 -s C:\

(CODE segment only, CSLIM=0624, IP=0017)

If doubtful EXEs and DLLs are in other drives too (i.e. LIBPATH and PATH

environment includes other drives), you have to check out each drives.

4. How to interpret outputs of FINDSEG

The following depicts outputs of findseg.

C:\OS2\DLL\PMGPI.DLL <--- File name

265110bytes    90-03-30    01:20:00   <--- File size and update time

001 05FCC CODE MOVABLE NONSHARED LOADONCALL E/R RELOC   PL2 NONDISCARDABLE

(1)  (2)   (3)   (4)     (5)       (6)      (7)   (8)   (9)       (10)

Instruction: push     es:�bx+si�  <-- Disassembled code

(1) : Entry id in segment table

(2) : Minimum allocation size for the segment

(3) : Segment type (CODE/DATA)

(4) : Fixed or Movable segment

(5) : Shared or Nonshared segment

(6) : Preload or Load on call segment

(7) : Execute/read or Execute-only if code, Read/Write or Read-only if data

(8) : Relocation table present or not

(9) : Privilege level

(10): Discardable or Nondiscardable segment

File name, Minimum allocation size, segment type and disassembled code are

important in above information. Other information is not used usually.

If two or more segments are reported, you have to check disassembled code.

If "IP is invalid value" is printed, the code segment is not what you want.

If a disassembled code is printed, you have to check whether the instruction

caused a TRAP-D(or TRAP-C) or not from segment register values in a popup panel.

Example 1.

Instruction:   mov ax, es:�bx+si�

ESLIM=0020, BX=0010, SI=0016

|

|

V

Because offset(bx+si=0026) exceeds ESLIM, this instruction will cause a

TRAP-D.

Example 2.

Instruction:   mov ax, es:�bx+si�

ESLIM=0020, BX=0010, SI=0006

|

|

V

Because offset(bx+si=0016) is within ESLIM, this instruction will not

cause a TRAP-D.

Example 3.

Instruction:   mov es, bx

ERRCD=1234, ERLIM=****, BX=1234

|

|

V

Because this instruction tries to load an invalid segment descriptor value

(BX=1234) into ES, it will cause a TRAP-D.

Example 4.

Instruction:   rep movs

DSLIM=1000, ESLIM=8000, SI=1002, DI=2002

|

|

V

Because SI(=1002) exceeds DSLIM, this instruction will cause a TRAP-D.

Example 5.

Instruction:   pop ax

SSLIM=0BFF, SP=0C00

|

|

V

Because SP(=0C00) exceeds SSLIM, this instruction will cause a TRAP-C.

In case that two or more segments are left after the above considerations,

it might help you that search DSLIM, ESLIM or SSLIM from DATA segments unless

DS, ES and SS were allocated or reallocated after a process started.

5. Other Information

(1) Sometimes CSLIM is one byte different from the segment size written in

an executable file. So the default search range is form CSLIM-1 to

CSLIM+1.

(2) The module reported by FINDSEG is not always a perpetrator of TRAP-D.

There is a case that the callee of the module passes an invalid parameter

to the module.

(3) In case that the TRAP-D is reproducible, Code view(a debugger) will help

you. For example, If TRAP-D occurs while EXCEL.EXE is running, type

CVP EXCEL.EXE from OS/2 command prompt. After code view starts, just type

g and ENTER-KEY, and replay operations that cause TRAP-D. When TRAP-D

occurs, the code view shows you instructions where TRAP-D occurs and more

information.