NotePlayer

Standard description

Copyright 1992 Bryan Ford

Version 1.10

Distribution

~~~~~~~~~~~~

This package is provided "as is" without warranty of any kind, either

expressed or implied, including, but not limited to, the implied warranty

of fitness for a particular purpose.  All risks associated with the use of

this package are assumed by you.

This package is freely redistributable as long as none of its contents

are modified, added, or removed.  You may not charge more for distributing

it than a small fee to cover the disk copying expenses.  It may not be used

in or distributed with any commercial program without my written

permission.  (If you want to use anything in a commercial program, just ask

me - I probably won't charge anything; I mainly just want to know who is

using it for what.)  The distribution must contain the following files:

NotePlayer.doc      This document

note.i              Include file used to accessing a NotePlayer

notesys.asm         System-friendly audio.device NotePlayer

notehard.asm        Simple hardware-banging NotePlayer

note.lib            Link library containing assembled versions of

notesys.asm and notehard.asm

Introduction

~~~~~~~~~~~~

This document describes the NotePlayer standard.  The NotePlayer

standard is an application interface specification designed to be

compatible across all 680x0-based computers.  Its purpose is to allow

hardware-independent music players to be written which can be used in a

variety of environments, operating systems, and even on completely

different computers.  It also makes it easier to simply write music players

for the Amiga.

Note that while this document defines note players rather than music

players, one of the goals is NOT to make NotePlayers interchangeable with

each other.  (This would be pretty much impossible.)  The goal is to make

the interface to a note player standardized, so music players can be

made interchangeable.

A NotePlayer module is a module written to this standard, which

provides facilities to music players for playing notes.  These calls, along

with some timing facilities provided in some other way (such as through the

GMOD standard), provide everything a music player needs to play music. The

interface is very powerful, but at the same time very easy to use for a

music player.

Two example NotePlayer modules for the Amiga have been provided already

(described lager).  For an example of a music player that uses the

NotePlayer standard, see the file "ptsplay.asm" in the MultiPlayer 1.30 (or

later) distribution.  This is a standard Protracker player modified to use

a NotePlayer instead of banging on the hardware directly.  (As a result,

MultiPlayer's Protracker player is one of the friendliest around.)

Client interface

~~~~~~~~~~~~~~~~

The only part of a NotePlayer module visible to the music player is a

short jump table.  A particular NotePlayer module is referred to by a

pointer to its jump table.  For example, in a portable GMOD module, a

pointer to the NotePlayer's jump table is given to the module.  The module

then performs all of its low-level note playing through entrypoints in this

jump table.

NotePlayer entrypoints are intended to be called from assembly

language.  Calling them from C or another high-level language may require

some kind of stub routines written in assembly language.  (But who wants to

write music players in C, anyway?)

The parameters to each of the functions in the jump table entries

listed below are passed in registers, as indicated on the second line of

each entry description.  The suffix '.b', '.w', and '.l' indicates which

part of that register the NotePlayer looks at.  For '.b' and '.w', the

NotePlayer ignores the top part of the register - it does not have to

contain zero.

The NotePlayer routines have pretty much standard calling conventions.

They may destroy registers D0-D1/A0-A1, but will save all other registers.

Return codes are returned in D0 where appropriate.

The entries in the jump table are each four bytes long.  The number of

entries in the jump table depends on the version number of the NotePlayer.

The version number of a particular NotePlayer corresponds to the version

number of the NotePlayer.doc (this document) according to which the

NotePlayer was written.  The first eight entrypoints are present in all

NotePlayers.  The currently defined entrypoints are:

NotePlayer+$00: info = NoteInfo()

D0.l

Returns a pointer to an information structure in D0.  The structure's

size depends on the version number in the structure.  The part of the

structure defined in this version of the standard is:

info+$00 (byte) Version number of NotePlayer standard used

info+$01 (byte) Revision number of this particular NotePlayer

info+$02 (byte) General-purpose flags:

bit 0: All sound samples must be in MEMF_CHIP

bit 1: Amiga "period" extension supported (described later)

info+$03 (byte) Maximum number of channels this NotePlayer supports

info+$04 (long) Pointer to name of this particular NotePlayer module

NotePlayer+$04: errmsg = NoteInit(channels,stereoarray)

D0.l               D0.b       A0.l

Gets the NotePlayer ready to play notes.  This must be called before

any other entrypoints are called.  (Exceptions:  NoteInfo and NoteFinish

may be called before NoteInit.)

The client passes the number of (monophonic) channels it will need in

D0, and an array describing the preferred stereo orientation of each

channel in A0.  The client can ask for between 1 and 128 channels, but if

it asks for more channels than the NotePlayer supports, NoteInit will fail

and the client will not be able to use this NotePlayer to play its music.

The stereoarray, if supplied, must contain as many bytes as the number

of channels requested. The first byte describes channel 0, the second byte

channel 1, and so on.  Each byte contains a signed value from -127 to 127

(-128 is reserved and should not be used) describing "where" this channel

is supposed to play.  A value of -127 indicates far left; 127 indicates far

right; 0 indicates center (no stereo).  Other values in this range may be

used as a balance control, indicating the position on a gradient between

left and right, although most NotePlayers will simply look at whether the

byte is negative, positive, or zero.  (Future hot-shot NotePlayers running

on advanced sound hardware can use two hardware channels per client-visible

channel to achieve smooth stereo panning; a future version of this standard

will provide for this situation when it arises.)  If the client passes NULL

for the stereoarray, an array full of zeros is assumed.

NoteInit returns zero if the NotePlayer was successfully initialized;

it returns a pointer to a null-terminated error message if initialization

failed for some reason or another.  If the call is successful, the client

may commence playing notes with the NotePlayer, and must call NoteFinish

when it is done.  If the call fails, the client must not call any other

NotePlayer entrypoints except the first three, and it does not need to

(though it may) call NoteFinish.

NotePlayer+$08: NoteFinish()

This call stops all currently playing notes and performs whatever

internal cleanup is necessary when note playing is finished.  If the

NoteInit call succeeded, the client must call this entrypoint when it is

finished with the NotePlayer.

NotePlyaer+$0c: NoteStart(chan,oneshotdata,oneshotlen,repdata,replen,freq,vol)

D2.b      A0.l       D0.l     A1.l   D1.l  D3.w D4.w

This entrypoint is the workhorse of the NotePlayer system: it plays

notes.  Any note previously playing in the channel is stopped, and a new

note is started.

The channel number must be between 0 and n-1, where n is the number of

channels requested in (and granted by) the NoteInit call.  If bit 7 in the

channel number is set, the NotePlayer takes the logical NOT of the

parameter as the real channel number, and modifies the behavior of the call

slightly.  In this case, instead of stopping any currently playing note and

starting the new note immediately, the change is made after the currently

playing note finishes its current cycle: when it is about to start the

repeat part, either directly after the one-shot part finishes, or when the

current repeat cycle is finished.  The new note will effectively be

"joined" to the old note, making it possible to implement "release" phases

of notes, among other things.

If oneshotlen is nonzero, then it contains the number of 8-bit samples

in the one-shot part of the note, and oneshotdata points to the sample

data.  If oneshotlen is zero, the note has no one-shot part, and the repeat

part (if it exists) is started immediately.

The replen and repdata parameters work exactly like the one-shot

parameters.  If repdata is zero, the note has no repeat part, and the

channel will become silent as soon as the one-shot part is finished.

If both oneshotlen and replen are zero, then any currently playing note

is stopped without starting a new note.  This is equivalent in effect to

the NoteStop entrypoint.

The freq parameter contains the sampling frequency at which to play the

note, in Hz.  The vol parameter contains the volume at which to play the

note, ranging from $000 (silent) to $100 (maximum).

If flag bit 1 in the Info structure returned by NoteInfo is set, then a

special extension to the frequency specification is supported, to allow

Amiga-based "tracker" players to be more easily accomodated.  If the client

calls NoteStart with D3.w (freq) set to zero, then the frequency is defined

by an Amiga period value in the high word of D3.  To calculate frequency

from period, or vice versa, simply divide the known value into 3579545.

NotePlayer+$10: NoteStop(chan)

D2.b

Stops any note currently playing in the specified channel, either

immediately (if D2.b contains a normal channel number from 0-127), or after

the current cycle of the currently playing note is finished (if D2.b

contains a NOTed channel number).  This call is equivalent to calling

NoteStart with zero in both oneshotlen and replen.

NotePlayer+$14: NoteFreqVol(chan,freq,vol)

D2.b D3.w D4.w

Changes the frequency and volume of the currently playing note, either

immediately (if D2.b contains a normal channel number from 0-127), or after

the current cycle is finished (if D2.b contains a NOTed channel number).

This entrypoint is used to implement slides and fades and such.  The freq

and vol parameters work exactly as they do in the NoteStart entrypoint.

NotePlayer+$18: NoteFreq(chan,freq)

D2.b D3.w

Changes the frequency of the currently playing note, without changing the

volume.  Otherwise works exactly the same way as NoteFreqVol.

NotePlayer+$1c: NoteVol(chan,vol)

D2.b D4.w

Changes the volume of the currently playing note, without changing the

frequency.  Otherwise works exactly the same way as NoteFreqVol.

Amiga system-friendly NotePlayer (notesys.asm)

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The file "notesys.asm" contains a sample NotePlayer module for the

Amiga which uses the audio.device to play notes.  To use it, simply link

"note.lib" with your program.  The symbol "NoteSys" then points to the

NotePlayer jump table.  (The symbol "_NoteSys" also points to the jump

table, for high-level language users.)

Since this NotePlayer uses the audio.device for everything it does, it

may be somewhat slower than a player that accesses the hardware directly.

However, as the programmers at Commodore have pointed out many times, using

the audio.device means there is no need for ugly DMA-delay loops and such,

which often compensates for the slightly lower efficiency of the

audio.device.  Moreover, this NotePlayer can give up audio channels as

needed, and it automatically re-allocates them when they again become

available, thus allowing beeps and such to be heard through the music.

This NotePlayer includes a master volume/balance control which is

completely independent of the main NotePlayer interface.  (In other words,

the music player can't use it - instead, the main program that's

responsible for putting the NotePlayer and the music player together uses

it.)  To set the master volume, call the NoteSysMasterVol function:

NoteSysMasterVol(volume)

D0.w

This function is available in both assembly ("NoteSysMasterVol") and C

("_NoteSysMasterVol" for stack args, "@NoteSysMasterVol" for register

args).  The volume parameter must be between 0 and $100 (256).  If any

music is currently playing, the volume changes immediately.  The master

volume cannot be changed in any way through calls to the entrypoints in the

normal (music player-visible) jump table.

If you want separate control of the volume of the left and right

channels (i.e. for a balance control), use the NoteSysMasterVolBal

function:

NoteSysMasterVolBal(leftvolume,rightvolume)

D0.w       D1.w

This function is also available in assembly and C forms.

"notesys.asm" can be assembled with A68k version 2.71.

You can specify the priority level for audio channel allocation with

the NoteSysAudioPri function:

NoteSysAudioPri(newpri)

D0.b

This priority will be applied to all future audio channel allocations.

Amiga hardware-banging NotePlayer (notehard.asm)

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

For situations where the audio.device isn't available, is too slow, or

simply isn't in style, "notehard.asm" can be used instead for 4-channel

playing.  This module is much smaller and (if you arrange it so DMA delay

loops aren't needed) faster, although not exactly system-friendly.

The NotePlayer jump table in "notehard.asm" works exactly the same way

as in "notesys.asm", except it's externally defined as "NoteHard" (and

"_NoteHard") instead of "NoteSys".

For simplicity, the NoteInit call in this NotePlayer does no error

checking.  In particular, it doesn't check the number of channels requested

by the music player.  If the music player requests more than four channels,

NoteInit will succeed, but the first time the music player tries to use

channel number 4 or higher, Bad Things will happen.

This NotePlayer imposes a few restrictions, for the sake of simplicity.

First, the sync-cycle feature (activated by NOTing the channel number) is

not supported.  Do not hook a music player that uses it to this NotePlayer,

or your results will be Not Good.

Second, this NotePlayer doesn't handle samples more than 64K in size.

Third, it assumes that the first four bytes in chip memory (at absolute

address 0) contain zero.  This is normally the case anyway, and it is easy

to enforce in a game or demo that takes over the machine either temporarily

or permanently.

Finally, unlike "notesys.asm" (and any other full-featured NotePlayer,

"notehard.asm" does no mapping of channels; it does not attempt to match

hardware channels with client channels according to the stereo preferences

specified in the NoteInit call.  The client's channel 0 goes to hardware

channel 0, etc.

This NotePlayer does not need any of the audio interrupts, and you

should have them all turned off (or redirected to a harmless routine) when

you use it.

The DMA delay, of course, is the bane of all Amiga players.

"notehard.asm" requires that the client (not the music player, but the

program that hooks the music player and the NotePlayer together) handle DMA

waiting, for maximum flexibility.  After the music player finishes all its

activity for a certain cycle (probably including calls to NoteStart) and

returns to the client, the client must check the longword named

"NoteHardDMACall" (or "_NoteHardDMACall"), which is defined in

"notehard.asm".  If this longword is nonzero, it is a pointer to a routine

which must be called after waiting an appropriate amount of time (usually

about 7 scanlines).  The routine obeys standard Amiga calling conventions:

it can munch d0-d1/a0-a1, but saves all other registers.  After the routine

returns, the client must again check "NoteHardDMACall" and if it's still

nonzero, wait again and call the new address, and so on until it returns

zero.

As an example, if you can't implement full interrupt-driven DMA

waiting, this scrap of code could do the trick:

bsr     musciplayer

1$      move.l  NoteHardDMACall,d0

bz      9$

move.l  d0,a0

moveq   #7-1,d1

2$      move.b  $dff006,d0

3$      cmp.b   $dff006,d0

beq.s   3$

dbra    d1,2$

jsr     (a0)

bra     1$

9$

Of course, playing music will hog a lot less CPU time if you use a CIA

timer interrupt for this purpose.

As a final reminder, this NotePlayer never accesses the operating

system or anything.  The only things it accesses (outside of some internal

variables) are the DMACON register and the audio registers ($dff0a0 to

$dff0d8).

If you (heaven forbid!) want to use this routine while the OS is

around, you'd better open the audio.device yourself and lock all the

channels so things don't go haywire if someone else tries to access the

audio hardware at the same.

"notehard.asm" can be assembled with A68k version 2.71.

Future plans

~~~~~~~~~~~~

In the future I hope to write more NotePlayers to be distributed in

this package.  Among them:

* A better hardware NotePlayer that supports sound effects and such.

* An 8-channel NotePlayer.

* NotePlayers for the new sound boards for the Amiga.

It would also be nice to see NotePlayers written for other 680x0-based

computers besides the Amiga.  This could be very handy for games and other

programs which are often written for several computers at once.

Version History

~~~~~~~~~~~~~~~

1.10 (R3, 14-Sep-92)

Added NoteSysAudioPri function to notesys.asm, to allow the client

to specify the audio channel allocation priority.

1.01 (R2, 25-May-92)

Removed 'ptplay.asm' from the NotePlayer distribution, since it

can now be gotten in its "native" environment with MultiPlayer.

1.00 (R1, 5-May-92)

First release.

Contact Address

~~~~~~~~~~~~~~~

I tend to move around a great deal, so mail sent directly to me

sometimes has a hard time catching up.  If you want mail to reach me (it

may take a while, but it WILL reach me), send it to this address:

Bryan Ford

8749 Alta Hills Circle

Sandy, UT 84093

I can be reached more quickly (for the time being anyway) on the phone

or through one of the electronic mail addresses below:

(801) 944-1990

baford@peruvian.utah.edu